3GPP TSG-RAN WG3 Meeting #130	R3-258545

# Dallas, Texas, U.S., 17th - 21st November 2025

Agenda Item:	14.2

Source:	Ericsson

Title:	Topology 2 - Continuing on starting points for discussions

Document for:	Discusson and Agreement

# 1	Introduction

RAN#108 agreed on an "AIoT Phase 2" WI which was revised at RAN#109 in [1].

Relevant content for discussions to start after RAN#109 is as follows:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769.

  - No specification support for CW control

- According to D2T2-C for active Device(s).

  - Start after March 2026

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s)).

  - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command).

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum.

- Specify RRC-based architecture, in coordination with SA2.

- The following are not supported:

  - Coordination between UEs operating as UE readers

  - New Uu protocol layer(s)

  - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band

This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1.

The work is subject to the following constraints:

RAN1

- The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS.

- No physical layer impact for signaling A-IoT resources to the intermediate UE.

- No additional specification change on R2D and D2R for Device 1 in Rel-20

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

RAN3

- Specify signaling for Topology 2

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection.

  - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

At RAN3#129bis, the following agreements were achieved (quoting Notes and Agenda):

General Architecture:

For Rel-20 T2 discussion, based on the RRC-based solution, and RAN3 focus is indirect connectivity is applied.

NGAP procedures specified for Topology 1 are baseline for Topology 2.

NGAP signaling procedures (Inventory and Command):

The AIOT procedures (Topology 2, RRC-based solution) in TR38.769 can be as baseline.

UE Reader Authorization:

UE Reader Authorization is performed at 5GC

The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB.

FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

# 2	Discussion

## 2.1	Applicability of solutions specified for Topology 1

Given the expectation that the RAN3 part should minimize impact on respective interfaces, and following also expectations discussed during the study for the RRC-based solution (see [2]), we would like to extend the already established principle that NGAP procedures specified for T1 are baseline for T2 with a principle concerning the architecture work for Rel-20:

Proposal 1:	If functional feasible, architectural principles specified for Topology 1 will be specified to be applicable for Topology 2.

Applicability of solutions developed for Topology 1 to Topology 2  is dependent on the expectations in which way Topology 2 is used. Main differentiating factor is whether we expect from AIoT RAN solutions using Topology 2 to provide AIoT services in a contiguous area, which would require a well-planned network setup throughout the projected AIoT service area by means of fixed-installed UE readers ("fixed UE readers"), or whether UE readers are used as handheld devices ("mobile UE readers"). In case of well-planned network setup we expect that a mixed deployment of topology 1 and topology 2 for type 1 devices is possible by using Rel-19 NGAP protocol functions and applying the same NG-RAN architectural principles

Observation 2:	It is likely that discussions on Topology 2 would need to take into account different deployment scenarios, like "fixed UE readers" and "mobile UE readers".

Observation 3:	We expect that mixed deployment scenarios consisting of topology 1 and fixed-installed UE readers for topology 2 are possible and supported by applying the same Rel-19 NGAP protocol functions and the same NG-RAN architectural principles.

Further input to discussions can be found in the WI description [1]:

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command).

Observation 4:	Only indoor deployment scenarios are considered for T2

- The following are not supported:

  - Coordination between UEs operating as UE readers

Observation 5:	Like for Topology 1, the gNB needs to coordinate allocation of AIoT radio resources among readers in case of Topology 2.

- Inter-gNB resource coordination is not specified.

Observation 6:	No XnAP protocol functions needed for coordination of A-IoT radio resources.

Proposal 7:	Acknowledge the deployment scenarios of (1) "fixed UE reader" assuming a well-planned network setup throughout the projected AIoT service area and (2) "mobile UE reader" assuming handheld reading devices.

## 2.2	Support of "fixed UE reader" deployments

Following discussions on the "RRC-solution" for Topology 2, we can see that from an AIOTF point of view, there is no difference in requesting inventory or inventory&command in a certain area with either Topology 1 or Topology 2 deployment (or even in mixed scenarios). We believe that it is common understanding that all AIoT-operation specific NGAP protocol solutions specified in NGAP can be re-used in case of "fixed UE reader " deployments.

One aspect on "fixed UE reader" deployments to be discussed is related to operational states of UE readers and applicability of OAM requirements made for Topology 1:

-	 In Topology 1 we have assumed that OAM at RAN and AIOTF is configured with the gNB's served readers, their identity and location and mapping to AIoT Area IDs. If there is a common understanding, that "fixed UE reader" deployments would need well planned network setup as for topology 1, then we can further agree on the following:

Proposal 8:	Agree that for "fixed UE reader" deployments, the gNB and the AIOTF are configured with the UE readers' IDs, their location and mapping to AIOT Area IDs.

-	Further, regarding the operational state of "fixed UE Readers", the above mentioned OAM assumptions would require UE readers to be in RRC_CONNECTED during AIOT operation. In general, whenever the AIOTF assumes the availability of AIoT service provision in a certain area, the "fixed UE readers" would need to be "ready" to provide such service.
In our view, such "ready" state of "fixed UE readers", that is known to the network  would require an implementation specific solution that "switches on" a network constituted of "fixed UE readers" which is outside 3GPP specification work.

Proposal 9:	How "fixed UE readers" enter an operational state that allow AIOTF- and RAN-OAM to be aware of their availability for AIoT service provision is outside 3GPP specification work.

-	If we assume that OAM is able to configure NG-RAN and the AIOTF with fixed readers' IDs, their location, and, if applicable, their association with A-IoT Area IDs, we can also assume that a fixed reader is pre-provisioned with a Reader ID, which the UE reader can communicate to the serving gNB at part of RRC connection establishment. 
The communicated Reader ID of a "fixed" UE reader can be used to bind an A-IoT operation (functionally initiated by the AIOTF) to the RRC connection of the UE reader.

Proposal 10:	Acknowledge the possibility of pre-provisioning a Reader ID to a "fixed" UE Reader which would need to be communicated to the gNB at RRC connection establishment and used for binding an A-IoT operation to an RRC connection. RAN2 should be informed about that assumption.

## 2.3	Support of "mobile UE reader" deployments

We acknowledge the use-case of "mobile UE reader" deployments under the following assumptions, which can be derived from principles assumed for topology 1 and constraints captured in the WID [1]:

-	how the gNB co-ordinates the usage of AIoT radio resources among different types of readers is a matter of deplyoment/implementation

-	A "mobile UE reader", assumed to be a handheld, probably human operated device, enters an operational state by means of UE reader initiated activity on application level i.e. NGAP protocol functions can be applicable for such readers as well, as they are ultimately triggered by the Application Function (AF) which triggers the AIOTF.

Proposal 11:	"Mobile UE readers" allow use of Topology 1 AIoT device related NGAP protocol functions.

-	Binding an AIoT operation triggered via NG to a UE-associated/RRC signalling connection is necessary and is not assumed to be possible by means of pre-provisioned identifiers, as the UE reader's location may change during operation and pre-configuration within NG-RAN and AIOTF can be ruled out. 
We expect a per session allocation of a binding identifier, with the allocating entity being probably outside NG-RAN, dependent on SA2 discussions.

Proposal 12:	For "mobile" UE readers, wait for input from SA2 regarding the binding of an AIoT operation to a UE-associated/RRC signalling connection.

One aspect is on the level of support of mobility for "mobile UE readers". Given the assumption that

- A-IoT radio resource of the UE reader is only valid within the serving cell.

continuation of an A-IoT operation would need to stop once the mobile UE reader moves into another cell's area. For intra-gNB scenarios, the gNB could re-issue A-IoT resource provision, if the UE reader is still in the area where it is allowed to operate. However, for indoor-scenarios we rule out inter-gNB mobility (and the nice playground that would open widely for possible though not necessarily essential optimisations).

Proposal 13:	Rel-20 assumptions for Topology 2 regarding inter-cell mobility do not seem to result in the necessity for introducing AIoT protocol elements on Xn.

## 2.3	Authorisation of "UE readers"

It is assumed that the gNB would need to know whether a UE is allowed to consume AIoT radio resources. As a first guess, we would assume that this follows "authorisation" signalling as introduced for other features on NG. We have discussed that we do not see any need on the Xn interface. We do not expect any F1 impact regarding UE reader authorisation.

Proposal 14:	Introduce "Authorisation" information only within NGAP UE Context related procedures following equivalent protocol functions introduced for other features, no need for authorisation related Xn or F1 protocol additions.

# 3	Conclusion and Proposals

We have discussed basic principles for work on Topology 2 in RAN3 along the following observations and proposals:

Proposal 1:	If functional feasible, architectural principles specified for Topology 1 will be specified to be applicable for Topology 2.

Observation 2:	It is likely that discussions on Topology 2 would need to take into account different deployment scenarios, like "fixed UE readers" and "mobile UE readers".

Observation 3:	We expect that mixed deployment scenarios consisting of topology 1 and fixed-installed UE readers for topology 2 are possible and supported by applying the same Rel-19 NGAP protocol functions and the same NG-RAN architectural principles.

Observation 4:	Only indoor deployment scenarios are considered for T2.

Observation 5:	Like for Topology 1, the gNB needs to coordinate allocation of AIoT radio resources among readers in case of Topology 2.

Observation 6:	No XnAP protocol functions needed for coordination of A-IoT radio resources.

Proposal 7:	Acknowledge the deployment scenarios of (1) "fixed UE reader" assuming a well-planned network setup throughout the projected AIoT service area and (2) "mobile UE reader" assuming handheld reading devices.

Proposal 8:	Agree that for "fixed UE reader" deployments, the gNB and the AIOTF are configured with the UE readers' IDs, their location and mapping to AIOT Area IDs.

Proposal 9:	How "fixed UE readers" enter an operational state that allow AIOTF- and RAN-OAM to be aware of their availability for AIoT service provision is outside 3GPP specification work.

Proposal 10:	Acknowledge the possibility of pre-provisioning a Reader ID to a "fixed" UE Reader which would need to be communicated to the gNB at RRC connection establishment and used for binding an A-IoT operation to an RRC connection. RAN2 should be informed about that assumption.

Proposal 11:	"Mobile UE readers" allow use of Topology 1 AIoT device related NGAP protocol functions.

Proposal 12:	For "mobile" UE readers, wait for input from SA2 regarding the binding of an AIoT operation to a UE-associated/RRC signalling connection.

Proposal 13:	Rel-20 assumptions for Topology 2 regarding inter-cell mobility do not seem to result in the necessity for introducing AIoT protocol elements on Xn.

Proposal 14:	Introduce "Authorisation" information only within NGAP UE Context related procedures following equivalent protocol functions introduced for other features, no need for authorisation related Xn or F1 protocol additions.

# 4	References

[1]	RP-252894 "Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2", agreed at RAN#109

[2]	TR 38.769 "Study on solutions for Ambient IoT (Internet of Things) in NR", latest version