3GPP TSG-RAN WG3 #130																		R3-258558

Dallas, USA, 17 - 21 November 2025

Title: 	Support for Modification of LTM Configurations

Source: 	Ericsson, Ofinno, Jio Platforms

Agenda item:	9.2.5

Document Type:	Discussion and Decision

# Introduction

In RAN3#129-bis meeting, the discussion on supporting modification of LTM configurations resulted in the following open questions for continued consideration.

Whether the modification of LTM configurations is allowed over network signalings?

If yes, which configuration needs to be modified?

When the LTM modification can be triggered? For example, during or after LTM preparation? Is it possible to modify during LTM execution or in subsequent LTM?

If there is any configuration that can be modified, which way is preferred?

a. Existing LTM Cancel procedure, and the source re-initiate the configuration procedure

b. New procedure

This contribution discusses the ways to support modification of LTM configurations over the network interface.

# Discussion

Currently, the specification supports add, release, and modify LTM configurations between the source gNB and the UE, but such modification is not available between the source gNB and the candidate gNB. Without this support, the RRC-level configuration update for LTM cannot function properly, resulting in increased signaling overhead.

Taking an example from TS 38.331, there are always possibilities to add/mod and to release the LTM candidate configuration, for example, CSI-RS resource configurations.

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...,

[[

ltm-NoSecurityChangeID-r19                     LTM-NoSecurityChangeId-r19                            OPTIONAL,    -- Need M

ltm-ExecutionCondition-r19                     SetupRelease {LTM-ExecutionConditionList-r19}         OPTIONAL,    -- Need M

ltm-NZP-CSI-RS-ResourceToAddModList-r19        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r19       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r19     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

ltm-CSI-ReportConfig-r19                       LTM-CSI-ReportConfig-r18                              OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceToAddModList-r19            SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource

OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceToReleaseList-r19           SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId

OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceSetToAddModList-r19         SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet

OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceSetToReleaseList-r19        SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId

OPTIONAL     -- Need N

]]

}

Therefore, modification of LTM configurations is essential to handle dynamic changes such as:

- CSI-RS resource configuration updates for L1 measurements and CSI acquisition

- Early UL synchronization configuration

- CFRA resource configuration

- Modification of LTM configurations shall be supported over network interfaces to ensure LTM functions properly.

Two possible solutions were discussed.

(a) Use the existing LTM Cancel procedure followed by re-initiation.

(b) Define a new dedicated modification procedure.

When considering solution (a), i.e., in the absence of direct modification function over network interfaces, the source gNB must cancel the ongoing LTM procedure and re-initiate configuration from the beginning. This would add significant delay and signaling overhead, particularly in the scenarios of more than one candidate cell, which are typical in LTM deployments.

- Cancelling LTM and re-initiating the procedures, as well as performing a UE reconfiguration from scratch, results in unnecessary signaling overhead and unintended UE behaviours.

Thus, initiating a new procedure from the candidate gNB is the most straightforward approach, as it avoids unnecessary signaling associated with cancelling and restarting LTM or performing a UE reconfiguration from the beginning. By allowing the candidate gNB to directly trigger the procedure, the network can ensure efficient resource allocation, minimize UE disruption, and keep consistent state synchronization between the involved network nodes.

- RAN3 agrees to introduce a new procedure, initiated by the candidate gNB, to update the LTM-related configurations.

- Updated configurations would include CSI-RS resource configurations, early UL sync and CFRA resource configurations.

Furthermore, consideration is required regarding the timing of triggering configuration modification. Based on offline feedback, two relevant scenarios have been identified. The first scenario occurs during or immediately after LTM preparation, but before the first LTM cell switch, where the initial configurations may need to be updated. The second scenario may happen during subsequent LTM, when ongoing configuration updates are necessary. The proposed procedure can address both scenarios, to make sure timely and efficient updates.

- The candidate gNB may trigger configuration updates during both LTM preparation and subsequent LTM phases.

Regarding the specification impacts, since RRC spec already supports such add/mod/release, no change is required in TS 38.331. For network interfaces, we need to consider XnAP changes. An example of specification updates is given below.

#### 9.1.5.x	CANDIDATE-INITIATED LTM CONFIGURATION MODIFICATION

This message is sent by NG-RAN node1 to NG-RAN node2 is to inform the NG-RAN node2 about the updates of LTM configurations for the UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| LTM Configurations to Modify List |  | 1 |  |  | YES | reject |
| >LTM Configurations to Modify Item |  | 1.. <maxnoofLTMCells> |  |  | – |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Early Sync Configuration | O |  | 9.2.3.218 |  | – |  |
| >>LTM CFRA Resource Information | O |  | 9.2.3.232 |  | – |  |
| >>CSI-RS Resource Configuration for L1 Measurements | O |  | CSI-RS Resource Configuration 9.2.3.224 |  | – |  |
| >>CSI-RS Resource Configuration for Early CSI Acquisition | O |  | CSI-RS Resource Configuration 9.2.3.224 |  | – |  |

Regarding the F1AP change, given that the CU is able to decide whether to include the corresponding IEs when transferring the message to the source DU, candidate-DU initiated modifications would not be required. Therefore, we propose to focus, at this stage, on the feasibility of supporting such modifications over Xn.

- Agree the corresponding CR to TS 38.423 in [1].

Another point raised during offline discussions concerns the support for PRACH resource release. This can be considered as part of LTM configuration update, as it involves previously allocated preamble index resources. Once the new procedure is agreed upon in RAN3, it can also be applied to handle PRACH resource release. Relevant papers can be found in [2][3][4][5][6].

- Once the new procedure for modifying LTM configurations is agreed, it can also be reused to release PRACH resources as part of configuration updates.

# Conclusion

There is strong agreement that modification of LTM configurations should be supported to ensure flexible and efficient operation of LTM. This support is considered a correction rather than a new feature. Companies are invited to contribute further analysis and proposals for detailed procedures and signaling support.

Observation 1	Modification of LTM configurations shall be supported over network interfaces to ensure LTM functions properly.

Observation 2	Cancelling LTM and re-initiating the procedures, as well as performing a UE reconfiguration from scratch, results in unnecessary signaling overhead and unintended UE behaviours.

Proposal 1	RAN3 agrees to introduce a new procedure, initiated by the candidate gNB, to update the LTM-related configurations.

Proposal 2	Updated configurations would include CSI-RS resource configurations, early UL sync and CFRA resource configurations.

Proposal 3	The candidate gNB may trigger configuration updates during both LTM preparation and subsequent LTM phases.

Proposal 4	Agree the corresponding CR to TS 38.423 in [1].

Proposal 5	Once the new procedure for modifying LTM configurations is agreed, it can also be reused to release PRACH resources as part of configuration updates.

# Reference

[1] R3-258559, Support for Modification of LTM Configurations, Ericsson, Ofinno, Jio Platforms

[2] R3-258555, PRACH resources for Inter-CU LTM, Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO, Ofinno

[3] R3-258551, PRACH resources for Inter-CU LTM, TS 38.423, Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO, Ofinno

[4] R3-258552, PRACH resources for Inter-CU LTM, TS 38.473, Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO, Ofinno

[5] R3-258553, PRACH resources for Inter-CU LTM, TS 38.401, Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO, Ofinno

[6] R3-258554, PRACH resources for Inter-CU LTM, TS 38.300, Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO, Ofinno