3GPP TSG RAN3 meeting #130        	         	                               R3-258579

Dallas, US, 17 – 21 November 2025

Agenda item: 	    11.2.1

Source: 	China Unicom

Title: 	Discussion on MRO enhancements for inter-CU LTM

Document for:	Discussion

1	Introduction

At RAN#108, the new Work Item on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5 was agreed and the WID was revised at RAN#109 in[1] introducing the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] If needed, co-operate with RAN1, SA5.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-20.

2	Discussion

2.1	MRO for inter-CU LTM

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as base line.

For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

For the connection failure cases:

-	After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis.

In the last meeting, we have the above agreements for the MRO solutions for intra-CU LTM.

Looking back what we have for R19 MRO solution below in TS 38.300.

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows: - Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell. - Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell. - Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell. In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure. In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM. In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

The definition of connection failure cases doesnot differentiate between the LTM type, thus for inter-CU LTM, the definition of connection failure cases in 38.300 can be reused directly in R20.

Proposal 1: The definition of connection failure cases in 38.300 can be reused directly in R20 for inter-CU LTM.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure: - Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM or CHO with candidate SCG(s) is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt). - Intra-system Too Early Handover: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO. - Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward. The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM execution until connection failure.

The detection mechanism of connection failure cases also can be reused directly in R20.

Proposal 2: The detection mechanism of connection failure cases in 38.300 can be reused directly in R20 for inter-CU LTM.

MRO for intra-CU LTM failure, we defined the failure cases of Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell. The agreed LTM failure scenarios are listed below.

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure. - - Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell. - - Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell. For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell. - - Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure. - - Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell. LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the cell which is different from the source or target one. - - Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure. - - Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

We can takes Rel-19 MRO for LTM scenarios as baseline for further study on inter-CU LTM failure scenarios.

Proposal 3: Rel-19 MRO for LTM scenarios can be considered as baseline for further study on inter-CU LTM failure scenarios.

After the LTM connection failure, UE sends the RLF report about the LTM failure issue to the gNB-CU.

The last serving gNB-CU performs initial analysis.

In case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch,  the last serving gNB-DU should be resposible for its execution. Then the gNB-CU should forward the RLF report to the last serving gNB-DU.

Proposal 4: the gNB-CU should forward the RLF report to the last serving gNB-DU in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch.

In case of too early LTM cell switch or LTM cell switch to wrong cell, the source gNB-DU should be resposible for its execution. Then the last serving gNB-CU should forward the RLF report to the source gNB-DU.

Proposal 5: The last serving gNB-CU should forward the RLF report to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell.

For inter-CU LTM cell switch, the RLF report transferring should be supported over Xn while Handover Report procedure over Xn can be reused.

Proposal 6: Handover Report procedure over Xn can be reused to tranfer RLF report to the source gNB.

3	Conclusion

Proposal 1: The definition of connection failure cases in 38.300 can be reused directly in R20 for inter-CU LTM.

Proposal 2: The detection mechanism of connection failure cases in 38.300 can be reused directly in R20 for inter-CU LTM.

Proposal 3: Rel-19 MRO for LTM scenarios can be considered as baseline for further study on inter-CU LTM failure scenarios.

Proposal 4: the gNB-CU should forward the RLF report to the last serving gNB-DU in case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch.

Proposal 5: The last serving gNB-CU should forward the RLF report to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell.

Proposal 6: Handover Report procedure over Xn can be reused to tranfer RLF report to the source gNB.

4   References

- RP-234038: Revised WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5; TSG RAN Meeting #109, Beijing, China, Sep 15-18, 2025