3GPP TSG RAN3 meeting #130   	                      R3-258592

17th – 21st Nov 2025  Dallas , US

Agenda Item:	13.3

Source:	China Unicom

Title:	Standard impacts of RAN-CN procedures for ISAC

Document for:	Discussion

- Introduction

The Study Item on Study on Integrated Sensing And Communication (ISAC) for NR has been agreed in RAN#108 (RP-251861) with the following objectives:

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

For RAN3, the signaling between RAN and CN to support ISAC should be studied, and the standard impacts of gNB-based mono-static sensing for UAV sensing target use cases should be considered as the first priority. This paper discusses the potential standard impacts for the RAN-CN procedures and signaling of ISAC.

- Solutions and standard impacts

The procedures, signaling between RAN and CN to support ISAC should be studied in Rel-20 5G-A, and the gNB-based mono-static sensing for UAV should be the prioritized use case, and the gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts.

Figure 1: gNB-based mono-static sensing and gNB bistatic sensing

The procedure and sinalling for gNB-based mono-static sensing only include the procedure and sinalling between SF and one gNB. Considering to not include any inter-gNB coordination procedure in this Rel-20 5G-A, the gNB bistatic sensing will only include the procedure and sinalling between SF and the Tx/Rx gNB, but not involve any procedure and signalling between Tx gNB and Rx gNB.

Proposal 1: The procedure and signalling for gNB-based mono-static sensing only include the procedure and sinalling between SF and one gNB, and these procedures and signalling should be studied as the baseline.

Proposal 2: The procedure and signalling for gNB bistatic sensing only include the procedure and sinalling between SF and Tx/Rx gNB, but not involve any procedure and signalling between Tx gNB and Rx gNB.

The following procedures are designed all for NG-RAN Node and SF, since in Rel-20 5G-A, only gNB mono-static sensing and bistatic sensing are supported, and the message may send between NG-RAN Node and SF directly or via AMF.

Sensing Entity/Entities Registration

Figure 2: Sensing Entity/Entities Registration Procedure

A Sensing Entity/Entities needs to register with the SF to get authorized to sensing, sensing entity/entities registration procedure is to initiate this procedure from sensing entity/entities, or from SF if needed. In 5G-A Rel-20, the sensing entity/entities is/are the NG-RAN Node, not include the UE, and sensing entity/entities registration only include the NG-RAN Node sensing registration.

The NG-RAN Node can initiate this procedure when it is activated with sensing ability, if it is not register with the SF, and the SF want to trigger the sensing procedure, the SF also can trigger the sensing entity/entities registration procedure. The sensing entity/entities information can be take to the SF through sensing entity/entities registration message, just like the basic information of the sensing entity/entities, the role of the sensing entity/entities(e.g. act as Tx and/or Rx entity, etc) and the sensing capability of the  sensing entity/entities, etc.

Proposal 3: The Sensing Entity/Entities Registration shall be considered as the basic procedure for sensing.

Proposal 4: The Sensing Entity/Entities Registration procedure can be initiated by Sensing Entity/Entities or SF.

Proposal 5: The Sensing gNB ID, sensing role of the sensing entity and the sensing capability can be reported to the sensing function via the Sensing Entity/Entities Registration procedure.

Sensing Capability Report

Figure 3: Sensing Capability Report Procedure

Figure 4: Sensing Capability Update Procedure

The SF can performs a selection of the Sensing Entity/Entities, and the should get the capability of the Sensing Entity/Entities first, and then select the Sensing Entity/Entities according to the capability and the sensing requirement from the customer. The Sensing Capability Report Procedure can be triggered by the SF, and if the Sensing Entity/Entities’s capability is changed, the NG-RAN node can initiate the sensing capability update procedure to notify the SF about it’s latest capability. The sensing Entity/Entities’s capability can include the sensing capability and the data processing capability of the Sensing Entity/Entities.

The SF can configure whether the sensing Entity/Entities’s to process the sensing result itself according to the data processing capability of the Sensing Entity/Entities, and the data processing capability may include the data processing time and data processing precision.

Proposal 6: The Sensing Capability Report procedure and Sensing Capability Update procedure shall be considered as the basic procedure for sensing.

Proposal 7: The Sensing capability can include the basic sensing capability and the data processing capability of the Sensing Entity/Entities.

Proposal 8: The data processing capability of the Sensing Entity/Entities can include the the data processing time and data processing precision.

Sensing Procedure Control for gNB-based Mono-static Sensing

Figure 5: Sensing Activation Procedure(Success) for Mono-static Sensing

The sensing activation/deactivation procedure for gNB-based Mono-static Sensing only involves one NG-RAN node . When sensing service is required by the sensing custom, the sensing function need to be activated by the sensing activation procedure, and when the sensing service is not still needed by the sensing custom, a sensing deactivation message shall be trigger to deactivate the sensing function. The sensing activation procedure should include the service requirement of the customer, e.g. service area, service indicator requirements, etc.

In the last meeting, it is agreed that the sensing request and sensing response message can be included in the TR, and it can be used to configure the sensing Entity/Entities. The sensing role need to be indicated by the SF, and the sensing resource can be indicated by the SF according to the sensing requirements.

Proposal 9: The Sensing role of the sensing Entity/Entities and the sensing resource can be considered to include in the sensing request message.

Sensing Mobility Management

When activating the sensing services, all cells within the sensing area can be activated to provide services to the sensing target. However, if the sensing area is very large, it will consume a lot of sensing resources. In scenarios such as intrusion detection and motion tracking, the sensing boundary cell can be activated first to provide sensing services. When a sensing target is detected, sensing services for other cells can be activated as needed based on the mobility of the sensing target.

The source sensing entity can predict the trajectory of the sensing target, negotiate with the target sensing nodes in the sensing area, and ultimately select the target sensing node. This method can avoid unnecessary sensing resource waste and ensure the continuity of sensing services more accurately.

Figure 6: Sensing Mobility Control for the Architecture not Involved AMF

Sensing Mobility Control for the Architecture not Involved AMF mainly applicable to the  architecture that there are direct connections between SF and Sending Entity/Entities. The source NG-RAN Node can get the sensing target information and predict the sensing trajectory of the sensing target, then the source NG-RAN Node can send the sensing handover request to the SF, and send the predicted information to the SF to help SF to select target NG-RAN Node.

Figure 7: Sensing Mobility Control for the Architecture Involved AMF

Sensing Mobility Control for the Architecture Involved AMF mainly applicable to the  architecture that the connections between SF and Sending Entity/Entities are via AMF. The source NG-RAN Node can get the sensing target information and predict the sensing trajectory of the sensing target, then the source NG-RAN Node can send the sensing handover request to the target via AMF, and send the predicted information to the target to help target node to decide whether it can take the sensing task. In this cases, the source NG-RAN node will select one or more target NG-RAN node itself to avoid the excessive interaction with the SF, when it decide to choose the target NG-RAN node, it should notify the SF.

Proposal 10: The Sensing Mobility Management should be studied in Rel-20 5G-A to avoid unnecessary sensing resource waste.

Proposal 11: The Sensing Mobility Management can be studied according to the predicted sensing trajectory information of the sensing target.

Sensing Objects Identity Recognition and Association

Figure 8: Sensing Objects Identity Recognition and Association

The sensing target may be an unknown object or a network terminal for the communication base station. In order to identify whether the sensing target is an unknown object or a network terminal device, the sensing target can be recognized by matching the sensing trajectory of the sensing target with the network terminal device. This can make the network better manage the sensing target. For example, online terminals flying into the no fly zone can be notified and driven away, and other break-in objects will be reported and take further action.

In the sensing detection and tracking area, the NG-RAN node configures the terminal to record the trajectory and indicating the reporting period and other information if it enters the sensing area. The trajectory information will be reported to the SF via the NG-RAN Node. SF then compares the trajectory information reported by the UE with the sensing trajectory information to determine whether the sensing object is a known target in the network or an unknown target.

Proposal 12: Sensing Objects Identity Recognition and Association is important for the network to identify the known target or an unknown target, and it will benefit for the next step action.

Proposal 13: Sensing Objects Identity Recognition and Association procedure should be studied in RAN3 and RAN2.

Proposal 14: Sensing assistant information should be reported from UE to the network to help the SF to determine whether the sensing object is a known target in the network or an unknown target.

Sensing Service Evoked and Exposed to The Customers

Figure 9: Sensing Service Evoked and Exposure to UE

In the SA2 TR 23700-14, it studied about the sensing result requested from UE an the network, and then exposed to the UE and the network. RAN should also study about the sensing service evoked and exposure.

The sensing results need to be exposed  to the application customs, there may have two types of customers, one is the sensing service request from UE and sensing result exposure to UE, and the other is the sensing service request from network(AF) and sensing result exposure to the network.  For both types of customs, the sensing request is from the customers to the SF, and the sensing result is processed by the SF and exposed to the customers.

Proposal 15: Both the sensing service evoked and exposed to the UE and to the network need to be considered.

Proposal 16: For sensing service evoked and exposed to UE, the sensing procedure is triggered by the UE through NG-RAN Node, and the sensing result is exposed to UE through NG-RAN Node.

- Conclusion

Proposal 1: The procedure and signalling for gNB-based mono-static sensing only include the procedure and sinalling between SF and one gNB, and these procedures and signalling should be studied as the baseline.

Proposal 2: The procedure and signalling for gNB bistatic sensing only include the procedure and sinalling between SF and Tx/Rx gNB, but not involve any procedure and signalling between Tx gNB and Rx gNB.

Proposal 3: The Sensing Entity/Entities Registration shall be considered as the basic procedure for sensing.

Proposal 4: The Sensing Entity/Entities Registration procedure can be initiated by Sensing Entity/Entities or SF.

Proposal 5: The Sensing gNB ID, sensing role of the sensing entity and the sensing capability can be reported to the sensing function via the Sensing Entity/Entities Registration procedure.

Proposal 6: The Sensing Capability Report procedure and Sensing Capability Update procedure shall be considered as the basic procedure for sensing.

Proposal 7: The Sensing capability can include the basic sensing capability and the data processing capability of the Sensing Entity/Entities.

Proposal 8: The data processing capability of the Sensing Entity/Entities can include the the data processing time and data processing precision.

Proposal 9: The Sensing role of the sensing Entity/Entities and the sensing resource can be considered to include in the sensing request message.

Proposal 10: The Sensing Mobility Management should be studied in Rel-20 5G-A to avoid unnecessary sensing resource waste.

Proposal 11: The Sensing Mobility Management can be studied according to the predicted sensing trajectory information of the sensing target.

Proposal 12: Sensing Objects Identity Recognition and Association is important for the network to identify the known target or an unknown target, and it will benefit for the next step action.

Proposal 13: Sensing Objects Identity Recognition and Association procedure should be studied in RAN3 and RAN2.

Proposal 14: Sensing assistant information should be reported from UE to the network to help the SF to determine whether the sensing object is a known target in the network or an unknown target.

Proposal 15: Both the sensing service evoked and exposed to the UE and to the network need to be considered.

Proposal 16: For sensing service evoked and exposed to UE, the sensing procedure is triggered by the UE through NG-RAN Node, and the sensing result is exposed to UE through NG-RAN Node.