3GPP TSG-RAN WG3 Meeting #130	R3-258595

Dallas, US, 17-21 November, 2025

Agenda item:	9.2.7

Source: 	Samsung, Huawei, CMCC

Title: 	Discussion on the encoding of AIOTF identifier

Document for:	Agreement

Introduction

In last meeting, the correction to TS 38.413 on AIoT leftovers on IE details has been endorsed in R3-257251 [1], wherein the IE type of AIOTF Identifier IE is defined as OCTET STRING (SIZE(32)).

In this contribution, we further check the details of the encoding of the AIOTF Identifier IE.

Discussion

In the endorsed CR in R3-257251, the AIOTF Identifier is corrected as follows, i.e. adding SIZE(32) to the OCTET STRING,

9.3.3.70	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identifier | M |  | OCTET STRING (SIZE(32)) | Coded as the NfInstanceId IE defined in TS 29.571 [35]. |

In general, it is a common understanding that the AIOTF Identifier IE refers to the NfInstanceId IE which is defined in TS 29.571 [2], sub-clause 5.3.2 as follows,

| NfInstanceId | string | String uniquely representing a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 9562 [15]. The hexadecimal letters should be formatted as lower-case characters by the sender, and they shall be handled as case-insensitive by the receiver. Example: "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" (NOTE 3) |
| --- | --- | --- |

It can be observed from TS 29.571 that the NfInstanceId is encoded in terms of hexadecimal letters and the hexadecimal letters should be formatted as lower-case characters by the sender.

Observation 1: TS 29.571 has indicated that the NfInstanceId is encoded in terms of hexadecimal letters and the hexadecimal letters should be formatted as lower-case characters by the sender.

Observation 2: TS 29.571 has indicated that the format of the Nf Instance ID shall be a UUID version 4.

TS 29.571 also mentions that NfInstanceId shall be in the format of UUID version 4, so we further check IETF RFC 9562 [3], and finds the following information related to the UUID format,

From the above quoted text in RFC 9562, we can clearly observe that,

- Firstly, RFC 9562 has clearly indicated that the UUID format is 16 octets in size.

- Secondly, there are multiple ways to represent a UUID format, such as String UUID, Binary UUID, Unsigned Integer UUID, etc.

In our understanding, no matter which way is represented, the basic principle is that the UUID format is 16 octets in size.

Observation 3: IETF RFC 9562 has indicated that the UUID format is 16 octets in size.

As a consequence, the example given in the description of the NfInstanceId in TS 29.571 which contains hyphens in between the hexadecimal letters is just a representation way of UUID, i.e. represented by the ‘hex-and-dash’ string as defined in RFC 9562.

Observation 4: The example given in the description of the NfInstanceId in TS 29.571 is just a representation way of UUID, other than the encoding way of UUID.

By having the above observations, we can conclude that the NfInstanceId is encoded in the UUID format, and the UUID format is 16 octets in size. As a result, the AIOTF Identifier IE referring to NfInstanceId should also be 16 octets in size.

Proposal 1: The size of AIOTF Identifier IE is 16 octets.

Conclusion

In this paper, we provide discussion on the encoding of AIOTF identifier. The following proposal is provided,

Observation 1: TS 29.571 has indicated that the NfInstanceId is encoded in terms of hexadecimal letters and the hexadecimal letters should be formatted as lower-case characters by the sender.

Observation 2: TS 29.571 has indicated that the format of the Nf Instance ID shall be a UUID version 4.

Observation 3: IETF RFC 9562 has indicated that the UUID format is 16 octets in size.

Observation 4: The example given in the description of the NfInstanceId in TS 29.571 is just a representation way of UUID, other than the encoding way of UUID.

Proposal 1: The size of AIOTF Identifier IE is 16 octets.

Reference

[1] RP-257251

[2] TS 29.571 v19.4.0

[3] IETF RFC 9562