3GPP TSG-RAN WG3 Meeting #130														R3-258601

Dallas, US, 17th-21th Nov, 2025

Agenda item:		12.2.3

Source:				CMCC

Title:					Discussion on AI/ML based handover enhancement

Document for:		Discussion

Introduction

During RAN#109, a Release 20 new study item “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” was approved [1] with the following objectives:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

In RAN3#129bis, the following was captured for inter-CU LTM:

Study inter-CU LTM in R20 SI?

In this paper, we provide our understanding on what can be discussed for AI/ML assisted inter-CU LTM.

Discussion

According to the R20 SID, AI/ML assisted inter-CU LTM was in the scope of the discussion scenarios. And in RAN3#129bis, RAN3 has discussed whether to study inter-CU LTM in R20, companies support to have this in scope but propose to first discuss intra-CU LTM. In our view, AI/ML assisted intra-CU LTM only implements mobility management within a single gNB, which does not require any signaling exchange between gNBs. Therefore, it’s a relative basic starting point for introducing AI/ML function to LTM feature. However, when considering AI/ML assisted mobility enhancement, only study AI/ML assisted intra-CU LTM within a single gNB has some limitations for UE mobility, and may not gain much benefits from introducing AI/ML. To have a wider support of mobility enhancement, study AI/ML assisted inter-CU LTM should not be excluded in R20.

Observation 1: AI/ML assisted intra-CU LTM does not require any signaling exchange between gNBs, which is a relative basic starting point for introducing AI/ML function to LTM feature.

Observation 2: Only study AI/ML assisted intra-CU LTM in R20 will limit UE mobility in one gNB, which may not gain so much benefits from introducing AI/ML to LTM mobility.

From our understanding, intra-CU LTM and inter-CU LTM has similar procedures, but intra-CU LTM only focus on F1 interface enhancement, while inter-CU LTM requires additional Xn interface enhancements. Currently, the inter-CU LTM procedure is shown in Figure 9.2.3.5.2-2 in TS 38.300.

Figure 9.2.3.5.2-2. Signalling procedure for inter-gNB LTM

From the signalling procedure, inter-CU LTM including 4 phases, which is the similar to intra-CU LTM:

- LTM preparation (steps 0-9)

- Early synchronisation (steps 10-11)

- LTM cell switch (steps 12-18)

- LTM switch completion (steps 19-22)

With the similar LTM procedure phases, the solution for AI/ML assisted intra-CU LTM can be taken as baseline for the AI/ML assisted inter-CU LTM. However, as intra-CU LTM only focuses on F1 interface, although the solution can be considered as starting point, the standard impact and what signalling message should be used to support the inter-CU LTM related information exchanges over Xn should be further discussed. This is where the AI/ML assisted inter-CU LTM solution different from the intra-CU LTM. Therefore, for AI/ML assisted inter-CU LTM, if the AI/ML assisted intra-CU LTM solution is determined, inter-CU LTM can take the solution of intra-CU LTM as starting point and discuss the detailed Xn enhancement.

Observation 3: The solution of AI/ML assisted intra-CU LTM can also be applicable to AI/ML assisted inter-CU LTM, but the standard impact and signalling messages exchanged over Xn interface should be further discussed for inter-CU LTM.

Proposal 1: It’s proposed RAN3 to study AI/ML-assisted inter-CU LTM in R20 which take the solution of AI/ML assisted intra-CU LTM as the starting point. Detailed Xn enhancements need further discussion.

Based on the procedures of inter-CU LTM, some potential enhancements by utilizing AI/ML are listed as:

- LTM preparation phase: The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and may contain the CSI resource configuration for subsequent LTM. The candidate prepares and provides the LTM configuration(s) to the source gNB. During this procedure, the candidate cells/beams prediction/determination can be optimized using AI/ML.

- Early sync phase: The network may request the UE to perform early TA acquisition of a candidate cell before a cell switch. Depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch. For RACH-less LTM, invalid TA value is possible to cause LTM failure. By introducing AI/ML function, valid TA value can be predicted, which can improve the robustness of LTM handover and ensure user experience.

- LTM cell switch phase: In this phase, LTM cell switch is triggered by cell switch command. From our understanding, the AI/ML function can be used to generate L1/L3 measurement prediction or LTM cell switch time prediction, which can assist the network making LTM cell switch decision and reserve resources in advance to improve the handover accuracy and further reduce handover latency.

- LTM cell switch completion phase: In this phase, if AI/ML is utilized for handover enhancement, the feedback information, such as measured information and UE performance after handover, should be collected to evaluate prediction accuracy, ensure the user experience and update the AI/ML model.

Observation 4: The following potential enhancements for AI/ML based inter-CU LTM including:

- Candidate cell/beam prediction

- TA prediction

- L1/L3 measurement prediction or LTM cell switch time prediction

- Feedback information collection

In RAN3#129bis, RAN3 has the following agreements in AI/ML assisted intra-CU LTM:

AI/ML assisted candidate cell selection for LTM Handover Preparation Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation. Further discuss whether to study addition of beam prediction in UE Trajectory prediction. Further discuss predicting TA value in the next meeting. Further discuss prediction of validity time of a measured TA value

Obviously, RAN3 has already agreed to study candidate cell/beam prediction for AI/ML assisted intra-CU LTM, and whether TA prediction, prediction of validity time of a measured TA value and beam level UE trajectory prediction are supported need further discussion. Thus, for AI/ML assisted inter-CU LTM, candidate cell/beam prediction can be first studied and supported.

Observation 5: Candidate cell/beam prediction is already agreed to be studied for AI/ML assisted intra-CU LTM in R20, it can also be supported for AI/ML assisted inter-CU LTM with similar signalling procedure between F1 and Xn interface.

Proposal 2: It’s proposed RAN3 to study candidate cell/beam prediction for AI/ML assisted inter-CU LTM.

Conclusion

In this contribution, we propose the following proposals:

Observation 1: AI/ML assisted intra-CU LTM does not require any signaling exchange between gNBs, which is a relative basic starting point for introducing AI/ML function to LTM feature.

Observation 2: Only study AI/ML assisted intra-CU LTM in R20 will limit UE mobility in one gNB, which may not gain so much benefits from introducing AI/ML to LTM mobility.

Observation 3: The solution of AI/ML assisted intra-CU LTM can also be applicable to AI/ML assisted inter-CU LTM, but the standard impact and signalling messages exchanged over Xn interface should be further discussed for inter-CU LTM.

Proposal 1: It’s proposed RAN3 to study AI/ML-assisted inter-CU LTM in R20 which take the solution of AI/ML assisted intra-CU LTM as the starting point. Detailed Xn enhancements need further discussion.

Observation 4: The following potential enhancements for AI/ML based inter-CU LTM including:

- Candidate cell/beam prediction

- TA prediction

- L1/L3 measurement prediction or LTM cell switch time prediction

- Feedback information collection

- Observation 5: Candidate cell/beam prediction is already agreed to be studied for AI/ML assisted intra-CU LTM in R20, it can also be supported for AI/ML assisted inter-CU LTM with similar signalling procedure between F1 and Xn interface.

- Proposal 2: It’s proposed RAN3 to study candidate cell/beam prediction for AI/ML assisted inter-CU LTM.

- References

[1] RP-252867, “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3”, ZTE, NEC