3GPP TSG-RAN WG3 Meeting #130														R3-258602

Dallas, US, 17th-21th Nov, 2025

Agenda item:		10.5.1

Source:				CMCC

Title:					Discussion on 6G AIML use cases

Document for:		Discussion

Introduction

In the RAN3#129bis, the following agreements are captured:

The design of AI/ML algorithms and models for RAN3 led use cases are implementation specific and out of RAN3 scope. The following use cases will be studied in RAN3: - AI/ML based Network energy saving - AI/ML based mobility optimization Other use cases can be discussed in future meetings based on contributions.

In this paper, we provide our views on other AI/ML related use cases and the details of AI/ML based network energy saving and mobility optimization.

Discussion

After the discussion in RAN3#129bis, two aspects can be considered in 6G AI/ML use cases: one is whether other use cases need to be supported in 6G Day 1, another one is the details to be studied in AI/ML based network energy saving and mobility optimization.

AI/ML new use cases

In RAN3#129bis, network energy saving and mobility optimization have been agreed to be studied as 6G AI/ML use cases. Some new use cases are also provided to be considered, including:

- AI/ML assisted QoE

- Cross-domain collaboration for AI/ML

- Agentic AI

- AI computing resource management

- AI/ML-enabled RAN Digital Twin for leveraging environmental data

- ......

The above listed use cases can be divided into two categories, i.e. AI for network and network for AI.

Table 1: 6G use cases category

| Category | 6G AI/ML use cases |
| --- | --- |
| AI for network | AI/ML assisted QoE, cross-domian collaboration for AI/ML, etc. |
| Network for AI | Agentic AI, AI computing resource management, Digital Twin, Network for AI, etc. |

AI for network use cases

Except the AI/ML based energy saving and mobility optimization, several use cases are also studied in 5G, such as load balance, slice, CCO.

For AI/ML based load balancing, we believe that in 6G initial deployment phase, the number of 6G users will not be large enough to create significant congestion, i.e. the load may not be particularly heavy. Traditional load balancing mechanism is likely to suffice for the early 6G network. Given this, introducing AI/ML based load balancing in a later phases when user density and traffic patterns justify additional complexity seems reasonable. Therefore, we propose RAN3 discuss whether this use case can be postponed on 6G Day 1.

Observation 1: The number of 6G users or load will not be large enough to create significant congestion in 6G initial deployment phase. Traditional load balancing mechanism is likely to suffice for the early 6G network. AI/ML based load balancing can be discussed when user density and traffic patterns justify additional complexity.

For AI/ML based network slicing, although network slicing is explicitly supported in 6G, the working mechanism and technical details of network slicing still unclear. Thus, it’s proposed to support AI/ML assisted network slicing after the working mechanism and technical details of 6G network slicing are determined.

Observation 2: Network slicing is explicitly supported in 6G, but the working mechanism and technical details of network slicing still unclear. AI/ML solution can be discussed once the working mechanism and technical details of 6G network slicing are determined.

For AI/ML based CCO, two CCO issues are discussed in 5G, i.e. coverage and cell edge capacity. In the early phase of 6G, coverage issues may be considered during the initial network deployment. The optimization of coverage issue by AI/ML based CCO may be useful for 6G network deployment, such as using AI to analyze network performance, adjust coverage parameters, etc. It seems that RAN3 can consider to support this use case in 6G Day 1.

Observation 3: AI/ML based CCO, e.g. resolve coverage issue, may be useful for 6G network deployment, such as using AI to analyze network performance, adjust coverage parameters, etc.

Proposal 1: It’s proposed RAN3 consider AI/ML based CCO in 6G Day 1.

Except the use cases supported in 5G, several other use cases also mentioned to be studied in 6G, such as QoE and cross-domain collaboration.

For AI/ML assisted QoE, as 6G has not determined whether QoE will be supported yet, AI/ML based QoE optimization proposes to be considered with secondary priority, and discusses only after 6G explicitly supports QoE features and determines the QoE working mechanisms.

Observation 4: 6G has not determined whether QoE feature will be supported, AI/ML assisted QoE can be discussed after 6G explicitly supports QoE features.

For AI/ML cross-domain collaboration, we believe that in the early stage of 6G, the separate functions of RAN and CN are not clear enough to support cross-domain collaboration. It is necessary to have some time to discuss the supported functions separately in RAN and CN, and then further consider cross-domain collaboration. Therefore, it is proposed that 6G Day 1 postpones the study of cross-domain collaboration and discusses whether to support it after RAN and CN determines their responsibilities or supported functions.

Observation 5: In early stage of 6G, RAN and CN need some time to determine their own supported functions or separate responsibilities. AI/ML cross-domain collaboration can be postponed in 6G Day 1 until RAN and CN determines their responsibilities or supported functions separately.

Proposal 2: It’s proposed RAN3 postpone the study of cross-domain collaboration to reserve some time for CN and RAN to determine their responsibilities or supported functions separately.

Network for AI use cases

Network for AI is mentioned as new services in 6G Radio SID [1].

(8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In our view, 6G network for AI have different optimization objectives from AI for network. AI for network aiming to optimize the network performance by utilizing AI/ML function, such as dynamic resource allocation, energy efficiency improvement, handover enhancement, etc. While network for AI may mainly focus on providing computing resources and exposing RAN related information/AI data to the UEs or other entities. AI computing resource management can be seen as one of the function for 6G network for AI. As 6G already support to cover NW for AI as new services, RAN3 can follow the 6G SID to discuss network for AI use cases. Potential use cases may include AI Agent, immersive communication service, digital twin, etc.

Observation 6: NW for AI is covered by new services in 6G Radio SID. NW for AI use cases are worth to be studied in RAN3 with different optimization objectives from AI for network use cases, i.e. AI for NW aiming to optimize the network performance and NW for AI focus on providing computing resources and exposing RAN related information/AI data.

Proposal 3: It’s proposed RAN3 to discuss 6G network for AI use cases, including AI Agent, immersive communication service, digital twin, etc.

However, as 5G AI/ML framework mainly considered the use cases of AI for Network, if network for AI use cases are supported, 6G AI/ML framework needs to be further discussed based on the 5G AI/ML framework to support 6G network for AI use cases.

Observation 7: 5G AI/ML framework mainly considered the use cases of AI for Network, which may insufficient to support network for AI use cases.

Proposal 4: If 6G supports network for AI use cases, RAN3 discusses 6G AI/ML framework based on 5G AI/ML framework.

Details of AI/ML based energy saving and mobility optimization

AI/ML based mobility optimization:

Mobility Optimization is one of the typical scenarios optimized by means of AI/ML. The solutions and procedures for AI/ML based mobility optimization has been discussed in both R18 and R19, and R20 will continuous discussing AI/ML based mobility use cases, such as multiple-hop UE trajectory prediction and LTM.

In 5G, the AI/ML based mobility optimization mainly focused on two aspects: UE trajectory prediction and NR-DC scenario. Considering that 6G has not been determined whether DC scenario will be supported, it’s proposed that for AI/ML based mobility optimization in 6G, SA scenario should be prioritized and other scenarios can be discussed based on the progress of 6G migration option(s) discussion.

From RP-252912: TSG#113 (September/2026): RAN plenary to make a decision on additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G). This includes decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

Observation 8: 6G has not determined whether additional migration option(s), e.g. 6G-6G DC, will be supported other than standalone.

For UE trajectory prediction, R18 only supported single-hop UE trajectory prediction which created a dependency chain: if the previous hop lacks prediction capabilities, subsequent hop may lose the critical UE trajectory prediction information. The multi-hop UE trajectory prediction will be discussed in R20, which enables multiple hops obtaining trajectory prediction in advance no matter whether the previous hop has the prediction capability or not, thereby facilitating proactive resource reservation and significantly improving the robustness of mobility. Given the above advantages, the multi-hop UE trajectory prediction should be supported in 6G Day 1.

Observation 9: R18 only supported single-hop UE trajectory prediction, where the availability of prediction information for subsequent hops strictly depends on the prediction capability of previous hop. Multi-hop UE trajectory prediction will be discussed in R20 to enable the subsequent hops obtain the prediction in advance and reserve resource accordingly.

Therefore, based on the above analysis, for 6G AI/ML based mobility optimization, multi-hop UE trajectory prediction and SA scenario should be supported with high priority.

Proposal 5: AI/ML based mobility optimization in 6G proposes to support SA scenario first and support multi-hop UE trajectory prediction with high priority.

Besides, in RAN2#131bis, RAN2 has the following agreement:

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

It seems that LTM (e.g. early DL/UL synchronization) and conditional handover are included in the study aspects of 6G. Therefore, the following aspects can also be studied in RAN3.

- L3 handover: CMCC has deployed R15 L3 measurement based HO in the commercial network, and based on the real network data, the handover success probability is almost 99%, which can ensure UE’s experience very well. It seems that the L3 handover can be used as baseline mobility mechanism in 6G. For 6G, as handover performance already has good performance, AI/ML can be used to enhance the other aspects, such as how to save measurement resources, etc.

- L1/L2-triggered mobility (LTM): LTM was first introduced in 5G to reduce the handover latency. It is applicable to high frequency scenarios with frequently handover, like FR2. The the potential enhancements by utilizing AI/ML may include target cell/beam prediction, TA prediction, etc.

- Conditional handover (CHO): CHO is defined as a handover that is executed by the UE when one or more handover execution conditions are met. It is applicable to high speed scenario. Some potential enhancements may include candidate cell prediction, etc.

Proposal 6: For AI/ML based mobility optimization, L3 handover, LTM and CHO can be considered to be supported in 6G Day 1.

AI/ML based energy saving

In R18, the solution for AI/ML based energy saving has been discussed with limited progress, i.e. the measured energy cost is only limited to node level and there is no consensus on prediction information. Then, during the study item phase of R19, energy saving use case also been discussed with no consensus reached and, as a result, dropped for the R19 WI phase. Based on the discussion in R18 and R19, 6G should first support the 5G AI/ML based energy saving enhancements, i.e. node level measured energy cost.

Proposal 7: It’s proposed 6G AI/ML based energy saving first support node level measured energy cost as it’s supported in 5G.

During the discussion in 5G, the predicted energy cost had been discussed for several meetings with no consensus achieved. Also, for the cell level energy cost, although it’s beneficial to have this information, it remains challenges to be obtained as many cells share the same hardware infrastructure which is difficult to calculate the energy cost per cell. Observation 10: 5G has discussed some AI/ML based energy saving enhancements with no consensus achieved, such as energy cost prediction and finer granularity of energy cost.

From our perspective, based on the experience of 5G discussion, when discussing 6G AI/ML based energy saving, the scope of the discussion should be clarified first. Two potential aspects may be evaluated:

- Whether the 6G network remains almost similar to that of 5G. If so, the finer granularity of energy cost and other enhancements with the same issue may still not be achievable.

- Whether the features supported in 5G will still be supported in 6G. For the 5G NES features, such as Cell DTX/DRX, Cell (de)Activation, OD-SSB, and OD-SIB1, if 6G will not support these features, the discussion of the related enhancements will be meaningless.

Therefore, it is proposed that RAN3 first analyze the above aspects to determine a clear discussion scope for AI/ML based energy saving.

Proposal 8: For AI/ML based energy saving, except the enhancements that already supported in 5G, RAN3 analyzes the following aspects to determine a clear further discussion scope:

- Whether finer granularity of energy cost can be supported in 6G RAN architecture

- Whether 5G NES feature can be studied in 6G AI/ML based energy saving

Conclusion

In this contribution, we propose the following proposals:

Observation 1: The number of 6G users or load will not be large enough to create significant congestion in 6G initial deployment phase. Traditional load balancing mechanism is likely to suffice for the early 6G network. AI/ML based load balancing can be discussed when user density and traffic patterns justify additional complexity.

Observation 2: Network slicing is explicitly supported in 6G, but the working mechanism and technical details of network slicing still unclear. AI/ML solution can be discussed once the working mechanism and technical details of 6G network slicing are determined.

Observation 3: AI/ML based CCO, e.g. resolve coverage issue, may be useful for 6G network deployment, such as using AI to analyze network performance, adjust coverage parameters, etc.

Proposal 1: It’s proposed RAN3 consider AI/ML based CCO in 6G Day 1.

Observation 4: 6G has not determined whether QoE feature will be supported, AI/ML assisted QoE can be discussed after 6G explicitly supports QoE features.

Observation 5: In early stage of 6G, RAN and CN need some time to determine their own supported functions or separate responsibilities. AI/ML cross-domain collaboration can be postponed in 6G Day 1 until RAN and CN determines their responsibilities or supported functions separately.

Proposal 2: It’s proposed RAN3 postpone the study of cross-domain collaboration to reserve some time for CN and RAN to determine their responsibilities or supported functions separately.

Observation 6: NW for AI is covered by new services in 6G Radio SID. NW for AI use cases are worth to be studied in RAN3 with different optimization objectives from AI for network use cases, i.e. AI for NW aiming to optimize the network performance and NW for AI focus on providing computing resources and exposing RAN related information/AI data.

Proposal 3: It’s proposed RAN3 to discuss 6G network for AI use cases, including AI Agent, immersive communication service, digital twin, etc.

Observation 7: 5G AI/ML framework mainly considered the use cases of AI for Network, which may insufficient to support network for AI use cases.

Proposal 4: If 6G supports network for AI use cases, RAN3 discusses 6G AI/ML framework based on 5G AI/ML framework.

Observation 8: 6G has not determined whether additional migration option(s), e.g. 6G-6G DC, will be supported other than standalone.

Observation 9: R18 only supported single-hop UE trajectory prediction, where the availability of prediction information for subsequent hops strictly depends on the prediction capability of previous hop. Multi-hop UE trajectory prediction will be discussed in R20 to enable the subsequent hops obtain the prediction in advance and reserve resource accordingly.

Proposal 5: AI/ML based mobility optimization in 6G proposes to support SA scenario first and support multi-hop UE trajectory prediction with high priority.

Proposal 6: For AI/ML based mobility optimization, L3 handover, LTM and CHO can be considered to be supported in 6G Day 1.

Proposal 7: It’s proposed 6G AI/ML based energy saving first support node level measured energy cost as it’s supported in 5G.

Observation 10: 5G has discussed some AI/ML based energy saving enhancements with no consensus achieved, such as energy cost prediction and finer granularity of energy cost.

Proposal 8: For AI/ML based energy saving, except the enhancements that already supported in 5G, RAN3 analyzes the following aspects to determine a clear further discussion scope:

- Whether finer granularity of energy cost can be supported in 6G RAN architecture

- Whether 5G NES feature can be studied in 6G AI/ML based energy saving

- References

- RP-251881, “New SID: Study on 6G Radio”, RAN#108, June 2025.

- 3GPP TR 38.914 V0.2.0, Study on 6G Scenarios and Requirements