3GPP TSG-RAN WG3 Meeting #130	             R3-258612

Dallas, USA, 17th – 21st Nov 2025

Agenda Item:	13.2

Source:	CMCC

Title:	Discussion on Network Architecture for Sensing

Document for:	Discussion

# Introduction

This contribution will continue to discuss the study of the network architecture for gNB-based mono-static sensing targeting UAV detection as described in SID [1].

In RAN3#129bis meeting, the following texts are agreed in TR 38.765 [2]:

# 7 Network architecture Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied. This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture. Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF. Figure 7.1 Logical architecture for ISAC This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF. Editor’s Note: The details of "Nx" interface need further discussion and decision.

# Discussions

## General

During last meeting, companies reached the common understanding that the SF introduced in 5G-A is deployed in CN which requires the coordination with SA2 to keep align with their study item on ISAC. In addition, the study on sensing as a critical service in 6G will be started soon under the scope of 6G SID. The NR only limits the scope of sensing towards gNB-based mono-static sensing for UAV sensing target use cases without UE involvement. By contrast, the broad sensing scenarios and multiple sensing mode are provided by companies to solve various use cases. From our perspective, all the companies in RAN3 shall followed the principle that the network architecture to support the sensing in the overall 5G system architecture should not impact the study on the network architecture to support the sensing in 6G. This principle also should be captured in the current TR which can be considered as a baseline to be taken into account when designing the network architecture for sensing in NR.

Proposal 1: The network architecture to support the sensing in the overall 5G system architecture should not impact the study on the network architecture to support the sensing in 6G.

Meanwhile, companies spent a lot of time to discuss the interface and protocol stack for sensing in last meeting. The current logical architecture for ISAC is independent of the transport and the whole system architecture for sensing is based on the progress from SA2. When checking the latest progress in SA2, the following agreements on system architecture are captured in their TR:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary.

It can be observed that the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). The names of these two sensing functions in the SA2 reflect the sensing function needs to support control plane function and user plane function. Besides, based on feedback from SA2 delegates, SA2 is still discussing whether Sensing Function may have different endpoints for control messages and for sensing data without solid conclusion. From our point of view, we maintain a positive attitude on this sentence where both control plane and user plane for sensing service should be supported. The control plane handles control signalling for sensing service, while the user plane carries sensing reports encapsulated in GTP-U packets.

Proposal 2: Both control plane and user plane for sensing service should be supported.

## 2.2	Control Plane Protocol Stack

From the last meeting, several options of control plane protocol stacks are summarized below:

### Option 1: New interface solution[R3-256558] In this solution, a new interface between the ISAC RAN and the Sensing Function and the corresponding new protocol stack should be defined. Option 1: Control-plane Protocol Stack for sensing

### Option 2: Ambient-IoT like solution [R3-256676] In this solution, sensing specific information is transported between the gNB and the Sensing CN node via the NG-C interface, the protocol stack should apply the NG-C protocol stack. In case of indirect connectivity with the SF, sensing specific information is transferred transparently via the AMF. Option 2: Control-plane Protocol Stack for sensing

### Option 3: Positioning like solution [R3-256558] In this solution, a new application protocol like NRPPa should be defined. NLX is the new interface between AMF and SF, NRxPa is the new application protocol, and the ISAC signalling is transported over the NRXPa on the top of NGAP. Option 3: Control-plane Protocol Stack for sensing

### Option 1: New interface solution

This solution establishes a direct logical and transport connection between the gNB and the Sensing Function, requiring a completely new interface and control plane protocol stack, designed similarly to the NG interface and NG-C protocol stack without introducing too much standardization effort, all the basic signalling procedure can be referred the NGAP protocol. The primary advantage of this approach is architectural clarity and provides maximum flexibility, allowing the protocol to be specifically tailored to the unique needs of ISAC without being constrained by the design choices of other interfaces. From our view, the new interface solution where a new control interface is established between the gNB and the Sensing Function can be supported for 5G-A Sensing Control Plane Protocol Stack.

Proposal 3: The new interface solution where a new control interface is established between the gNB and the Sensing Function can be supported for 5G-A Sensing Control Plane Protocol Stack.

### Option 2: Ambient-IoT like solution

This solution maximizes reuse by treating ISAC signaling as a type of non-UE associated signalling, similar to the interface we designed in Ambient IoT. The principal benefit of this option is the minimal standardization effort and maximum reuse. It leverages the entire existing NG-C protocol stack and procedures, offering the fastest path to implementation. Deployment is simplified from a connectivity perspective, as it copies the existing NG interface between gNB and AMF. One of the leftover issues is which connectivity mode should be supported, i.e., direct connectivity or indirect connectivity depends on the discussion in SA2. Anyway, this solution is also a feasible choice where NG-C interface is reused between gNB and Sensing CN for 5G-A Sensing Control Plane Protocol Stack.

Proposal 4: The Ambient-IoT like solution where NG-C interface is reused between gNB and Sensing CN can be supported for 5G-A Sensing Control Plane Protocol Stack.

### Option 3: Positioning-like solution

This solution reuses the architectural pattern of positioning, where a new application protocol (NRSPa) is carried transparently over the existing NG interface. The AMF acts as a relay, forwarding the NRSPa messages between the gNB and SF without interpreting the content—functioning as a "sealed envelope". This option will have an impact on AMF although AMF is transparently transmission between the SF and gNB. From the operator’s perspective, the deployment of 5G-A sensing is primarily targeted at ToB business and will remain limited in scale and be deployed independently from the commercial network. As stated in the SID, the study of ISAC in NR does not involve using UEs as sensing entities, and operators prefer to avoid any impact on the AMF in commercial networks.

Proposal 5: The Positioning-like solution is not pursued for 5G-A Sensing Control Plane Protocol Stack.

## 2.3	 Protocol Stack for Sensing Report

Last meeting, there is an FFS for the protocol stack for sensing reporting. In our point of view, the sensing report is related to the discussion on sensing measurement in RAN1. The objective in RAN1 clearly states that metrics, measurements, and relevant measurement quantization for UAV use case should be identified and studied. During last RAN1 meeting discussion [3], levels of sensing measurements were discussed as below:

[FL3][H] Proposal 7.1-1-rev2 (all records from offline) - The following measurements are identified in the study of NR ISAC. - Level 1: raw data, which is the subcarrier-domain samples of the channel response from each antenna port, or anything equivalent - E.g., data size: Msubcarrier*Ncpi*Mport*Nport*Nquantization, e.g., 3276*(80*4)*(4*8*2)*32 = 2.1Gbits - Level 2: (partial raw data) - Amplitude and phase profile of delay, and/or Doppler, and/or angle - E.g., Amplitude and phase profile of delay-angle - E.g., data size: Mcp*Ncpi*Mport*Nport* Nquantization, e.g., 288*(80*4)*(4*8*2)*32 = 188.7Mbits - Level 3 per path (subset of Level 2): QC - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles - A path can correspond to one or multiple detected points - A path can correspond to one or multiple detected objects - Supported by: QC - Level 4 per point: LG, OPPO, Xiaomi, - Option 1: Range, velocity, angle, and power per detected point for a given time stamp - Definition: A point is defined by one range/delay, one Doppler and one angle - vivo, CATT, Xiaomi, OPPO, Sony, LGE, HW, QC, IDCC - Supported by: - Option 2: Position, velocity, and power per detected point for a given time stamp - Definition: A point is defined by one location and one velocity - ZTE, Nokia, - Supported by: - E.g., data size: Npoint*5* Nquantization - ‘5’ means 5 parameters, i.e., delay, Doppler, horizontal angle, vertical angle, power - E.g., 100*5*32 = 16Kbits - Level 5: e.g., {position, velocity, no target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported - Level 6: e.g., {position, velocity, target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported - E.g., data size: (Npoint=1)*5* Nquantization - ‘5’ means 5 parameters, i.e., 3D location, velocity, power - E.g., 15*5*32 = 2400bits - Note: Trajectory report is separate discussion - SINR, RSRP from perspective of sensing - Time stamp - Study the following levels of measurements for potential down-selection within Rel-20 SI - Level 2 - Level 3 - Level 4 - Level 5 - Level 6 - SINR, RSRP from perspective of sensing

From our view, control plane protocol is designed for configuration messages, and the practical implementation for control plane limits typically in the range of 64 KB to 256 KB per message. While there is no strict standardized maximum, several factors constrain control plane message sizes:

- SCTP buffer limitations (typically 64 KB - 1 MB)

- ASN.1 encoding/decoding processing overhead for large messages

- AMF/gNB processing delays for large message parsing

- If DTLS over SCTP is used for security, the current standard (RFC 6083) [4] imposes a 16 KB per message limitation. Although in the recent IETF draft [5], some company suggests removing this limitation, the draft did not reach consensus.

Considered above limitations, the control plane has size limitation on message transmission and not suitable for transmit quite large sensing report.

Observation 1: The control plane has size limitation on message transmission and not suitable for transmit quite large sensing report.

During RAN1 discussion, our position is supporting the Level 2 information which can be considered as Delay/angular/Doppler spectrums, this sensing measurements may benefit for target classification, improving the performance of detection and false alarm, compatibility with more sensing use cases and algorithms, sensing verification and data consistency. It can be seen that the data size of Level 2 information is quite large which exceeds reasonable control plane message size, the user plane shall be used for Level 2 information transmission. Anyway, RAN3 is not the proper group who decides the sensing measurement and the final results needs to waited for the conclusion made from RAN1.

Proposal 6: The user plane for sensing service should be used for sensing data reporting.

## 2.4 User Plane Protocol Stack

In the user plane design, the new interface solution is provided with a direct GTP-U tunnel established between the gNB and the SF, similar to the N3 interface between gNB and UPF. The architecture introduces a new Nx-u interface where the sensing data is encapsulated in GTP-U over UDP/IP.

The direct GTP-U tunnel establishes a point-to-point user plane connection between the gNB and Sensing Function, similar to the N3 interface design but bypassing the 5G Core user plane entirely. This results in minimal latency since sensing data flows directly from the gNB to the SF without intermediate processing, making it ideal for latency-sensitive sensing applications such as real-time object tracking, vehicular safety systems, and industrial automation. The protocol stack is streamlined with GTP-U encapsulation providing tunnel identification via TEID, sequence numbering for packet ordering, and optional extension headers for sensing-specific data.

Proposal 7: The new interface solution where a new Nx-u interface is introduced with direct GTP-U tunnel established between the gNB and the SF is supported for 5G-A Sensing User Plane Protocol Stack.

# Conclusion

This contribution focuses the foundation for RAN3's gNB mono-static sensing architecture study. The following observations and proposals are provided:

Observation 1: The control plane has size limitation on message transmission and not suitable for transmit quite large sensing report.

Proposal 1: The network architecture to support the sensing in the overall 5G system architecture should not impact the study on the network architecture to support the sensing in 6G.

Proposal 2: Both control plane and user plane for sensing service should be supported.

Proposal 3: The new interface solution where a new control interface is established between the gNB and the Sensing Function can be supported for 5G-A Sensing Control Plane Protocol Stack.

Proposal 4: The Ambient-IoT like solution where NG-C interface is reused between gNB and Sensing CN can be supported for 5G-A Sensing Control Plane Protocol Stack.

Proposal 5: The Positioning-like solution is not pursued for 5G-A Sensing Control Plane Protocol Stack.

Proposal 6: The user plane for sensing service should be used for sensing data reporting.

Proposal 7: The new interface solution where a new Nx-u interface is introduced with direct GTP-U tunnel established between the gNB and the SF is supported for 5G-A Sensing User Plane Protocol Stack.

# References

RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR

R3-257332, (pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR

R1-2507427 Summary #5 on NR ISAC_v041_Moderator

RFC 6083, https://datatracker.ietf.org/doc/html/rfc6083

https://www.ietf.org/archive/id/draft-ietf-tsvwg-dtls-over-sctp-bis-08.html