3GPP TSG-RAN WG3 Meeting #130	             R3-258613

Dallas, USA, 17th – 21st Nov 2025

Agenda Item:	13.3

Source:	CMCC

Title:	Discussion on RAN-CN Procedures Supporting ISAC

Document for:	Discussion

# Introduction

This contribution discusses on the necessary procedures and signaling enhancements for the RAN-CN interface to support Integrated Sensing and Communication (ISAC) services [1]. With SA2 defining the overall ISAC architecture framework [2], RAN3 must ensure the NG interface can efficiently support the sensing-related procedures and data flows required by the core network functions.

# Discussion

## Sensing Capability Transfer Procedure

This procedure enables the gNB to report its sensing capabilities to the SF during initial setup or when capabilities change. The SF uses this information for sensing entity selection and optimal sensing service configuration.

The SENSING CAPABILITY INDICATION message serves the purpose of enabling the gNB to advertise its sensing capabilities to the Sensing Function (SF) during initial network setup or when capability changes occur. This message contains comprehensive information about the gNB's sensing capacities to facilitate proper sensing entity selection and optimal sensing service configuration by the SF. The contents include supported sensing geographic areas defined by lists of geographic coordinates, various supported sensing service types such as presence detection, motion tracking, and gesture recognition, and the sensing functionality capabilities including Tx-only, Rx-only, or Tx&Rx operation modes. Additionally, the message specifies the supported frequency bands available for sensing operations, sensing resolution capabilities covering range, velocity, and angular resolution parameters, the maximum number of concurrent sensing sessions the gNB can handle, the supported sensing signal types including PRS, SRS, CSI-RS variants, and the sensing update rate capabilities defining the minimum time intervals between consecutive sensing measurements.

The SENSING CAPABILITY ACK message provides the SF's acknowledgment of the received sensing capability information from the gNB. This message serves to confirm the successful reception and processing of the capability indication or to indicate any issues encountered during the capability exchange procedure. The contents include a simple result field indicating whether the capability reception was successful or failed, and in case of failure, an error cause field that specifies the reason for the failure, enabling the gNB to understand and potentially rectify the issue in subsequent capability reporting attempts.

Introduce SENSING CAPABILITY INDICATION and SENSING CAPABILITY ACK message for gNB to interact with SF to report  its sensing capabilities, including:

    - Supported sensing geographic areas

    - Supported sensing service types

    - Sensing functionality

    - Supported frequency bands for sensing

    - Sensing resolution capabilities

    - Maximum concurrent sensing sessions

    - Supported sensing signals

    - Sensing update rate capabilities

## Sensing Reporting

This procedure allows the SF to request sensing services from the gNB and establish a sensing session with specific configuration parameters, which was identified and agreed in the last RAN3 meeting [3].

The SENSING REQUEST message serves as the initiation point for sensing operations, containing configuration parameters that define the sensing service requirements. The message includes a unique Sensing ID that identifies the specific sensing session throughout its lifecycle. Target area information precisely defines the geographical scope of sensing operations through geographic coordinates and area shape descriptors, ensuring the sensing covers the intended physical space. Sensing KPIs establish the performance requirements for the service, including accuracy specifications for detection and measurement, along with latency constraints that define the maximum allowable delays in sensing processing and reporting. The sensing Tx configuration details the transmission parameters for sensing signals, encompassing waveform characteristics, power level settings, and beamforming configurations to optimize sensing performance. Complementary to this, the sensing Rx configuration specifies the reception and processing requirements, including the types of measurements to be performed and the signal processing algorithms to be applied. Finally, the reporting configuration defines how and when sensing results should be delivered, supporting both periodic reporting at regular intervals and event-triggered reporting based on specific detection thresholds or conditions.

The SENSING RESPONSE message provides the gNB's acknowledgment and response to a sensing request, confirming the acceptance and configuration of the proposed sensing session. It includes the same Sensing ID from the corresponding request to maintain session correlation and tracking. The accepted parameters section indicates which of the requested configurations have been approved, potentially including modifications or adjustments made by the gNB based on its current capabilities and resource availability. A result field clearly states whether the sensing session establishment was successful or failed, providing immediate feedback to the SF. In cases of failure, an error cause field elaborates on the specific reason for rejection, which could include resource constraints, capability mismatches, invalid parameters, or other operational limitations that prevented session establishment.

The SENSING REPORT message delivers the actual sensing results from the gNB to the SF. The message is correlated to a specific sensing session through the included Sensing ID, ensuring proper association with the ongoing sensing operation. Sensing measurements form the core of the report, presenting either raw data for further processing by the SF or pre-processed data that has undergone initial analysis by the gNB. Detected objects information provides detailed characteristics of any identified targets, including their positional coordinates, velocity vectors, classification types, and confidence levels associated with each detection. Quality metrics offer crucial context about the reliability of the reported data, featuring confidence levels for different aspects of the detection, accuracy estimates for positional and velocity measurements, and signal-to-noise ratio (SNR) values that indicate measurement quality. Finally, timestamp information precisely records when the sensing measurements were taken, enabling temporal analysis and synchronization with other data sources.

- SENSING REQUEST message could include:

    - Target area information,

    - Sensing KPIs

    - Sensing Tx configuration

    - Sensing Rx configuration

    - Reporting configuration (periodic, event-triggered, thresholds)

- SENSING RESPONSE message could include:

    - Sensing ID

    - Accepted parameters (modified from request if necessary)

    - Result (success/failure)

    - Error cause (if failure)

- SENSING REPORT message could include:

    - Sensing ID

    - Sensing measurements (raw or processed data)

    - Detected objects information (position, velocity, type, confidence)

    - Quality metrics (confidence levels, accuracy estimates, SNR)

    - Timestamp information

Regard to the sensing measurement, the objective in RAN1 clearly states that metrics, measurements, and relevant measurement quantization for UAV use case should be identified and studied. During last RAN1 meeting discussion [4], levels of sensing measurements were discussed as below:

[FL3][H] Proposal 7.1-1-rev2 (all records from offline) - The following measurements are identified in the study of NR ISAC. - Level 1: raw data, which is the subcarrier-domain samples of the channel response from each antenna port, or anything equivalent - E.g., data size: Msubcarrier*Ncpi*Mport*Nport*Nquantization, e.g., 3276*(80*4)*(4*8*2)*32 = 2.1Gbits - Level 2: (partial raw data) - Amplitude and phase profile of delay, and/or Doppler, and/or angle - E.g., Amplitude and phase profile of delay-angle - E.g., data size: Mcp*Ncpi*Mport*Nport* Nquantization, e.g., 288*(80*4)*(4*8*2)*32 = 188.7Mbits - Level 3 per path (subset of Level 2): QC - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles - A path can correspond to one or multiple detected points - A path can correspond to one or multiple detected objects - Supported by: QC - Level 4 per point: LG, OPPO, Xiaomi, - Option 1: Range, velocity, angle, and power per detected point for a given time stamp - Definition: A point is defined by one range/delay, one Doppler and one angle - vivo, CATT, Xiaomi, OPPO, Sony, LGE, HW, QC, IDCC - Supported by: - Option 2: Position, velocity, and power per detected point for a given time stamp - Definition: A point is defined by one location and one velocity - ZTE, Nokia, - Supported by: - E.g., data size: Npoint*5* Nquantization - ‘5’ means 5 parameters, i.e., delay, Doppler, horizontal angle, vertical angle, power - E.g., 100*5*32 = 16Kbits - Level 5: e.g., {position, velocity, no target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported - Level 6: e.g., {position, velocity, target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported - E.g., data size: (Npoint=1)*5* Nquantization - ‘5’ means 5 parameters, i.e., 3D location, velocity, power - E.g., 15*5*32 = 2400bits - Note: Trajectory report is separate discussion - SINR, RSRP from perspective of sensing - Time stamp - Study the following levels of measurements for potential down-selection within Rel-20 SI - Level 2 - Level 3 - Level 4 - Level 5 - Level 6 - SINR, RSRP from perspective of sensing

For the sensing measurement, RAN3 needs to wait for the conclusion made from RAN1.

## Sensing Update and Cancel

This procedure allows the SF to modify ongoing sensing sessions or cancel them based on changing requirements or network conditions.

The SENSING UPDATE REQUEST message serves the critical function of enabling the Sensing Function (SF) to dynamically modify an ongoing sensing session at the gNB. This message allows for real-time adjustments to sensing operations in response to changing requirements, environmental conditions, or optimization needs. The message contains the specific Sensing ID that uniquely identifies the target session to be modified, ensuring that the updates are applied to the correct sensing operation. The updated parameters section encompasses all the modifiable aspects of the sensing session, including adjustments to the target geographic area, revisions to the Key Performance Indicators (KPIs) such as accuracy thresholds or latency requirements, and modifications to the sensing configuration parameters. This flexibility allows the SF to adapt sensing operations based on emerging needs, such as tracking moving targets, responding to changing environmental conditions, or optimizing resource utilization without needing to terminate and re-establish the entire sensing session.

The SENSING UPDATE RESPONSE message provides the gNB's formal reply to a sensing update request, confirming the acceptance or rejection of the proposed modifications. This message serves as an acknowledgment mechanism that ensures both entities maintain synchronized understanding of the current sensing session parameters. The result field clearly indicates whether the update request was successfully implemented or if it failed, providing immediate feedback to the SF. For successfully processed requests, the accepted modifications section details exactly which parameters were updated and may include any adjustments the gNB made to the requested changes based on its current capabilities or resource constraints. In cases where the update cannot be fully or partially implemented, the error cause field specifies the reason for failure, which could include resource unavailability, capability limitations, invalid parameter values, or conflicts with other ongoing sensing sessions, enabling the SF to make appropriate follow-up decisions.

The SENSING CANCEL REQUEST message facilitates the controlled termination of an active sensing session by the SF. This message is essential for proper resource management and clean session teardown when sensing services are no longer required or need to be reassigned. The message includes the specific Sensing ID that identifies the session to be terminated, ensuring precise targeting of the cancellation action. Additionally, the cancellation reason field provides important context for the termination, which could include normal completion of sensing objectives, resource reallocation needs, quality of service issues, mobility events requiring session transfer, or administrative interventions. This reason information helps the gNB with logging, analytics, and potential optimization of future sensing sessions, while also providing transparency about the session lifecycle management decisions made by the SF.

The SENSING CANCEL RESPONSE message completes the session termination handshake by providing the gNB's confirmation of the cancellation request processing. This message ensures reliable session teardown and prevents ambiguous session states between the SF and gNB. The result field unambiguously states whether the cancellation was successfully executed or if the operation failed, providing definitive closure to the termination process. In successful cases, this confirmation allows the SF to reliably release any associated resources and update its session management records. If the cancellation encounters issues, the error cause field explains the reason for failure, which might include references to non-existent sessions, ongoing processing that cannot be immediately interrupted, or communication problems. This error information enables the SF to take appropriate corrective actions, such as retrying the cancellation or escalating the issue through management interfaces.

Introduce the SENSING UPDATE REQUEST and SENSING UPDATE RESPONSE messages to enable modification of ongoing sensing sessions.

Introduce the SENSING CANCEL REQUEST and SENSING CANCEL RESPONSE messages to provide controlled termination of sensing sessions.

# Conclusion

The successful deployment of ISAC services requires comprehensive RAN-CN interface enhancements to support sensing-specific procedures and data flows. This contribution proposes that RAN3 develops the necessary signaling procedures to enable efficient sensing service delivery while maintaining backward compatibility with existing communication services.

We therefore have the following proposals:

- Introduce SENSING CAPABILITY INDICATION and SENSING CAPABILITY ACK message for gNB to interact with SF to report  its sensing capabilities, including:

    - Supported sensing geographic areas

    - Supported sensing service types

    - Sensing functionality

    - Supported frequency bands for sensing

    - Sensing resolution capabilities

    - Maximum concurrent sensing sessions

    - Supported sensing signals

    - Sensing update rate capabilities

- SENSING REQUEST message could include:

    - Target area information,

    - Sensing KPIs

    - Sensing Tx configuration

    - Sensing Rx configuration

    - Reporting configuration (periodic, event-triggered, thresholds)

- SENSING RESPONSE message could include:

    - Sensing ID

    - Accepted parameters (modified from request if necessary)

    - Result (success/failure)

    - Error cause (if failure)

- SENSING REPORT message could include:

    - Sensing ID

    - Sensing measurements (raw or processed data)

    - Detected objects information (position, velocity, type, confidence)

    - Quality metrics (confidence levels, accuracy estimates, SNR)

    - Timestamp information

- For the sensing measurement, RAN3 needs to wait for the conclusion made from RAN1.

Introduce the SENSING UPDATE REQUEST and SENSING UPDATE RESPONSE messages to enable modification of ongoing sensing sessions.

Introduce the SENSING CANCEL REQUEST and SENSING CANCEL RESPONSE messages to provide controlled termination of sensing sessions.

# References

RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR

SP-250401, New SID on Study on Stage 2 for Integrated Sensing and Communication

Draft_R3-257332 RAN3 BL pCR for study on ISAC for NR v1

R1-2507427 Summary #5 on NR ISAC_v041_Moderator