3GPP TSG-RAN WG3 Meeting #130	             R3-258619

Dallas, US, Nov 17th - Nov 21th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on A-IoT security aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS 33.369, the authentication procedure for Ambient IoT is specified by SA3. In the authentication procedure, the AIOTF sends inventory request message including RANDAIOT_n to the NG-RAN, and then included in the A-IoT paging message. Once receiving the RANDAIOT_n, the A-IoT device generates the RESAIOT, and transmits the RESAIOT and RANDAIOT_d in the A-IoT NAS PDU to the AIOTF for device authentication. Firstly, the calculation of security parameters introduces additional complexity to A-IoT devices and increases their power consumption. In addition, the security parameter introduced in Rel-19 are unlikely to be reused in Rel-20. It is most likely that the same NGAP message is used to transmit Rel-19 and Rel-20 security parameters, so it wouldn’t be appropriate for RAN3 to make RANDAIOT_n mandatory. Moreover, in real deployment, it is unnecessary for the operator/network to repeatedly re-authenticating a specific known device once it has successfully completed the authentication procedure. In sum, it is proposed to optionally include the RANDAIOT_n in the Inventory Request message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedural text of inventory procedure to optionally include RANDAIOT_n in the Inventory Request message. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, the impact can be considered isolated because the change affects the Ambient IoT service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The authentication procedure for Ambient IoT defined by SA3 is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.23.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 16.23.6	Inventory Procedure

Figure 16.23.6-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.23.6-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list). If the Requested Service Area Information contains:

-	Neither the Requested AIoT Area List nor the Requested Reader List, the gNB shall select all readers it serves;

-	The Requested AIoT Area List, the gNB shall select readers within the indicated AIoT Area(s);

-	The Requested Reader List, the gNB shall take the reader list into account to perform inventory.

The Inventory Request message also includes the Inventory Assistance Information. The Inventory Assistance Information includes the expected D2R message size and may also include e.g., approximate number of Target A-IoT devices, time interval.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

The Inventory Request message may also include the RANDAIOT_n for device authentication.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 1:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The Inventory procedure over the A-IoT radio interface is performed, i.e., A-IoT paging, A-IoT Access Procedure and D2R data transmission, as specified in section 16.23.5.

5./6.	Upon receiving the response(s) from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

Upon completion of the inventory procedure the gNB sends an Inventory Complete Indication in the Inventory Report message.

NOTE 2:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

7.	If there is no follow-on command, the A-IOT CN triggers the A-IoT CN node initiated A-IoT Session Release procedure as defined in clause 16.23.8.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>