3GPP TSG-RAN WG3 Meeting #130	R3-258642

Dallas, US, 17th ~21st Nov, 2025

Title:	Further discussions on RAN functions and logical architecture

Agenda Item:	10.2.2

Source:	Huawei

Document for:	Discussion and decision

1	Introduction

The definition of new system, aka 6G [1], like previous system, requires the 6G RAN functions and logical architecture definitions. Following the requirements on 6G [2] and the progress of RAN discussion [3], this contribution discussed the 6G RAN functions and logical architecture. The 6G RAN functions and logical architecture, should take a solid basis in 5G-A that why we start from the same principle as 5G study [4] with considering the 5G-A evolutions and all updated by the 6G requirements.

We should also acknowledge that the RAN functions are pending to the RAN-CN function split and the 6G logical architecture is still pending to the RAN decision particularly related to the non-standalone deployments, if any.

The contribution reviews the RAN functions, the legacy on from 5G (Transfer of User Data, Paging, etc.), the functions introduced later in 5G (Direct services support, etc.) and then the Functions specific for New RAN.

Then the contributions will focus on RAN Standalone architecture as common initial understanding for the deployment.

2	6G overall RAN architecture and RAN functions

2.1 Consideration on 6G overall RAN architecture

Basically, the 5G RAN architecture follows the principle in 4G, to continue with a flat IP architecture, which could better control the resource in a more efficient way, comparing the split architecture adopted in 3G. In the meaning time, operators are more and more considering different deployment scenarios where RAN functions based on software handling could enjoy the cloud resource sharing, thus the introduction of a central unit was proposed and agreed. As for whether to keep CU/DU split or not, due to the fact that CU and DU may be able to be deployed in a whole cloud environment, this could be further discussed, see another discussion paper in [2].

Observation 1: The current 5G RAN overall architecture already allowed for efficient resource usage and different deployment scenarios.

Later on, during 5G-A phase, new services were introduced, including XR, AI/ML and is now studying sensing, we have seen initial standardized mechanism for some of them, while some are still under study phase.

Observation 2: The current 5G RAN overall architecture already supported or will support new services, e.g. AI/ML, sensing, etc., which were introduced in 5G-A phase.

With the above observations, we could see that, for current 5G RAN overall RAN architecture, a solid foundation has been laid for subsequent evolution.

For 6G discussions, on the other hand, it is still worth noting that more new service types may pop out, or new requirements might be raised. For example, AI service is not only for the case of AI for Net, but also may serve the case of Net for AI; in addition, new types of immersive communication/sensing/data management and transfer/RAN digital twin/token or agent communication may also impose new requirements.

However, whether such new services or new requirements would require fundamental changes to the current RAN overall architecture or not, this for sure needs further discussions in RAN3, but at least we think RAN3 could take current RAN architecture as starting point for further discussion of 6G RAN architecture.

Proposal 1: RAN3 to discuss and agree to take current 5G-A RAN architecture as starting point for further discussion of 6G RAN architecture, to accommodate new services or new requirements, such as AI and sensing.

Taking a further step, when referring to Figure 2.1-1 above, in our understanding, the basic RAN architecture at least contains RAN base stations with standardized interface between two neighbour ones, i.e. an Xn-like interface, and another standardized interface between RAN and core network, i.e. an NG-like interface. This 6G base station will, at least, also provide 6G radio U-plane and C-plane protocol terminations towards the UE, which is business as usual. Taking proposal 1 into account together, we could have a basic and initial 6G RAN architecture, see the following Figure 2.1-1. Hence, the following proposal should be discussed and agreed in RAN3.

Proposal 2: RAN3 to discuss and agree that, 6G base station with Xn-like and NG-like interfaces should be kept as base line for 6G RAN overall architecture, and capture the Figure 2.1-1 in the TR.

Figure 2.1-1 6G RAN Overall Architecture

Corresponding TP to overall architecture could be seen in Annex part.

2.2 6G RAN Functions

By reference to the 5G Study [5] and the 5G specification, the agreed requirements in last plenary [3], and also the general discussions and the basic functions proposed to be supported in [6], we can continue to consider the rest of the listed functions, what functions need to be discussed and confirmed now, what to be discussed and decided during earlier meetings, what functions needs to be postponed till clear guidance received from plenary.

2.1.1	Functions to be discussed and confirmed

The following requirements, which are actually kind of functions closely related with RAN3, were agreed in RAN#109 in [3]:

-	The 6G RAN architecture shall allow non-public networks.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

Taking the above agreed function into account, and comparing the functions which were supported in 5G day 1, in our understanding, RAN3 should first discuss, during earlier meetings, if the following functions should be supported or not, most of which are also linked to other working groups.

-	Selection of an CN node at UE attachment when no routing to an AMF can be determined from the information provided by the UE;

This function is linked with SA2 (assuming there will be similar function as AMF in 6G core network), but RAN3 could still assume that from RAN perspective, this function is needed from day 1.

-	Transport level packet marking in the uplink;

This function is a pure RAN behaviour, RAN3 could discuss if it should be supported from day 1.

-	QoS Flow management and mapping to data radio bearers;

Technically, the above function is related with service awareness which was already agreed as port of requirement [3], it is also related to SA2, but RAN3 should discuss this in parallel with SA2 and coordinate with SA2 when needed.

- Self-Configuration and Self-Optimisation;

This function is normally led by RAN3, so RAN3 should be able to decide when this function should be discussed in order to be supported in day 1.

-	Selection of an CN node at UE attachment when no routing can be determined from the information provided by the UE;

-	Transport level packet marking in the uplink;

-	QoS Flow management and mapping to data radio bearers;

-	Self-Configuration and Self-Optimisation.

2.2.3	Functions which are postponed

As clearly guided by RAN#109 plenary, the following two functions will be studied from Mar. 2026 to Sep. 2026, thus there is no need for RAN3 to discuss them from now on, till clear guidance being received from plenary.

- Dual Connectivity;

- Tight interworking between NR and 6G;

Observation 3: RAN3 will not discuss the following two functions until clear guidance being received from plenary, including DC and tight interworking between NR and 6G.

2.2.4	RAN Functions inherited from 5G-A (i.e., RAN side of RAN-CN functional split)

As could be seen in 38.300 [4], the 5G base station hosts the following functions:

The gNB and ng-eNB host the following functions:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling);

-	IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data;

-	Selection of an AMF at UE attachment when no routing to an AMF can be determined from the information provided by the UE;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state;

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Dual Connectivity;

-	Tight interworking between NR and E-UTRA;

-	Maintain security and radio configuration for User Plane CIoT 5GS Optimisation, as defined in TS 23.501 [3] (ng-eNB only).

NOTE 1:	BL UE or UE in enhanced coverage is only supported by ng-eNB, see TS 36.300 [2].

NOTE 2:	NB-IoT UE is only supported by ng-eNB, see TS 36.300 [2].

In our understanding, some basic RAN functions shall be kept as starting point, since they are fundamental functions which are necessary to basic operations for a RAN base station, no matter what types of new services may be introduced in the future. These functions shall at least include:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and side link (scheduling);

-	Measurement and measurement reporting configuration for mobility and scheduling;

RRM function is a fundamental and core function hosted by a base station since day 1 of 3GPP wireless network, while the measurement and reporting are basic parts of RRM function. Note that side link itself is a function should be discussed later.

-	Routing of User Plane data (towards user plane handling entity in core network);

-	Routing of Control Plane information (towards control plane handling entity in core network);

As agreed in [1], “The 6G RAN architecture shall allow for control plane and user plane separation”, so the above two functions are the natural consequence of this agreement.

-	Connection setup and release;

-	Session Management;

The conception of session and connection will anyway be there, since any service or feature will finally be based on the session and connection to bear the communication.

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the CN or OAM);

-	Distribution function for NAS messages;

In our understanding, the basic architecture a wireless network should still be composed of RAN and CN (and corresponding OAM), thus there will be paging messages coming from CN system information from CN or OAM, and NAS messages between CN and UE.

-	Radio access network sharing;

This should be a basic deployment requirement from operators, we think this should be supported from day 1, this is also in line with the agreed requirements in RAN#109 [1]: “-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.”.

With these basic functions as starting point, RAN3 could continue to discuss if new logical functions are needed for new services or new requirements.

Taking all the discussions in sections above, we think at least the following RAN functions could be taken as starting point:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink;

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Routing of User Plane data (towards user plane handling entity in core network);

-	Routing of Control Plane information (towards control plane handling entity in core network);

-	Connection setup and release;

-	Session Management;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the CN or OAM);

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Self-Configuration and Self-Optimisation.

-	Selection of an CN node at UE attachment when no routing to an AMF can be determined from the information provided by the UE;

-	Transport level packet marking in the uplink;

-	QoS Flow management and mapping to data radio bearers;

Proposal 3: RAN3 to discuss and agree the basic RAN functions listed above as starting point, and capture them into TR.

Corresponding TP could be seen in the Annex part.

3	Conclusion

This document made the following proposals and observations:

Observation 1: Observation 1: The current 5G RAN overall architecture already allowed for efficient resource usage and different deployment scenarios

Observation 2: The current 5G RAN overall architecture already support new services, e.g. AI/ML, sensing etc., which was introduced in 5G-A phase.

Observation 3: RAN3 will not discuss the following two functions until clear guidance being received from plenary, including DC and tight interworking between NR and 6G.

Proposal 1: RAN3 to discuss and agree to take current 5G-A RAN architecture as starting point for further discussion of 6G RAN architecture, to accommodate new services or new requirements, such as AI and sensing.

Proposal 2: RAN3 to discuss and agree that, 6G base station with Xn-like and NG-like interfaces should be kept as base line for 6G RAN overall architecture, and capture the Figure 2.1-1 in the TR.

Proposal 3: RAN3 to discuss and agree the basic RAN functions listed above as starting point, and capture them into TR.

4	Reference

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] RP-252960, pCR for 38.914 on Requirements of new and existing services for 6G, CMCC (moderator)

[3] RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G, NTT DOCOMO, INC. (Moderator)

[4] TS 38.300

[5]TR 38.801

[6] R3-257179, Consideration on General requirement and principles for RAN architecture,	Huawei

[6] R3-256700, Consideration on RAN internal function split and interfaces, Huawei

Annex1	Text Proposal (TR skeleton based on TR 38.760-3 V0.1.0)

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

3	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP RP-252912: “New SID: Study on 6G Radio”

[3]	3GPP TR 38.914: “Study on 6G Scenarios and Requirements”

[x]	3GPP TR 23.801-01: “Study on Architecture for 6G System; Stage 2”

5.3	Overall RAN Architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

Note: this overall architecture will be used as a base line, names for interface and base stations are FFS.

The 6G RAN consists of the following logical nodes:

-	6G base station providing the 6G radio U-plane and C-plane protocol terminations towards the UE; and

-	NG-like interface connected to 6G core network; and

-	Xn-like interface connected to neighbour 6G base stations

Figure 5.3-1: 6G RAN overall architecture

5.3.1	RAN-CN Functional Split

Editor’s note: The aim of this section is to describe functions split between RAN and CN.

The general aspects of RAN-CN functional split are captured in TR 23.801-01 [x].

The functions hosted by the 6G RAN node are elaborated in clause 5.3.2. The functions hosted by the CN subject to SA2 decision.

5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN.

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink;

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Routing of User Plane data (towards user plane handling entity in core network);

-	Routing of Control Plane information (towards control plane handling entity in core network);

-	Connection setup and release;

-	Session Management;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the CN or OAM);

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Self-Configuration and Self-Optimisation;

-	Selection of an CN node at UE attachment when no routing can be determined from the information provided by the UE;

Editor’s note: the final conclusion is also pending to SA2

-	Transport level packet marking in the uplink;

-	QoS Flow management and mapping to data radio bearers;

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>