3GPP TSG-RAN WG3 Meeting #130	R3-258657

Dallas, US, 17 – 21 November 2025

Agenda item:	10.5.1

Source: 	Samsung

Title: 	Discussion on AI/ML use cases for 6G

Document for:	Discussion and decision

# Introduction

RP-252912 describes the 6G study item scope, where the AI/ML related part contains the use case and framework as

AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

Since the 6G study has officially started, RAN3 is supposed to study on further AI/ML use cases according to the requirements and services in 6G. This discussion section provides our detailed discussion on the potential 6G AI/ML use cases.

# Discussion

## 2.1 Principles

3GPP started the AI/ML for NG-RAN from R17 in 5G era. A set of principles was set to give the basic rules for the design of enabling the RAN intelligence. The mechanisms were adopted by specification to insert the AI/ML function in the nearly mature 5G system. The main difference for 6G is to involve the AI/ML for Day1. Hence, the principles should be re-defined to give the instruction for future study.

RAN3 129bis meeting agrees the fundamental one as the design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope.

SID includes two parts: use case and LCM framework. There is a voice that the framework should be unified across different working groups. The LCM framework aims to provide the common understanding of the AI/ML and lay the foundation of detailed mechanism design. There is no direct impact on the scheme or standard impact. Besides, the use cases to study are different for different working group. There is no need to align the LCM framework. To progress the study efficient, the framework is unified for RAN3 use cases.

Observation 1: There is no need for unified framework across the multiple working groups.

Proposal 1: LCM framework is unified for RAN3 use cases.

## 2.2 Potential use cases

## 2.2.1 AI enabled service awareness network

In 6G, it is expected to support various services classes, which includes the existing types, e.g. immersive multimedia, and emerging ones, e.g. integrated communication and sensing. With growing demands of extreme performance, flexibility and scalability, the network design needs to be more elastic to adapt the different service requirements. As the appearance of diverse automatic equipment and dependency of devices in our daily life and work, the number of UE connections is in trend of going up. Under the fact that insufficient radio resource, the question should continue to be solved with a concrete answer: How to meet the requirements of high volume of UEs in disparate services with the limited radio resources. The QoS is the main factor to determine the transmission strategy such as resource allocation, scheduling, etc. In legacy 5G networks, core network is the entity to provide the QoS and alternative QoS. RAN follows or selects the QoS to do the transmission between base station and UE. How to select the QoS is left for implementation. And the QoS options are limited to the defined scope, which may not be the best one to be adopted when considering multiple factors in the network. Besides, multi-IP flows with different traffic type could be mapped to one single QoS flow by UPF. But the actual QoS requirements for different services are not same. The potential way forward for RAN is to select or adjust the QoS based on the service type. One possible method is: the resource can be prioritised for high-requirement service (e.g. immersive multimedia), comparing with the low-requirement service (e.g. IoT). Different service has different data arrival features. For example, the data arrival interval for audio services is larger than that of video services. It can be the basis for AI/ML to recognise the service type. In summary, the AI/ML can be the potential tools for the network to identify and recognize different services and the corresponding requirements; after that, the network can adjust its resource management strategy to adapt to the different services.

Figure 1: Example of service recognition

In terms of RAN3 impact, the traffic feature identification via interfaces and the mechanism of how recognised service type affect the network operation should be studied.

Proposal 2: Service-awareness network can be one of the potential use cases for RAN3 to study.

## 2.2.2 AI for network resilience

To guarantee the network performance, the network reliability is one of the most significant factors. If the network broken in one area, the UEs would lose connections. After that, the back-up base station would continue to serve the UE. To some extent, it can solve the issue. But the drawback is that the connection is interrupted and user experience has already lower down. The conventional way can not handle it well due to the limitation of view of what is happened or happening. AI/ML is acknowledged to give the status of future and what will be. With the assistance of AI/ML, the resilience can be enhanced to avoid the gap from network broken to recovery.

The main goal of AI/ML for resilience is to ensure the network operating in the correct way and mitigate the risk of network crash. AI/ML predicts issue via the trend of collected data such as network performance, and then wake-up the back-up base station or other method to avoid the hidden danger.

As for the RAN3 impact, the data of network performance as well as the new configuration should be transferred via the network interfaces. For example, the network performance collects from the different components of the base station and/or from the neighbour nodes.

Proposal 3: AI for network resilience can be one of the potential use cases for RAN3 to study.

## 2.3 Agreed use cases

In RAN3 129bis meeting, the network energy saving and mobility optimization have been agreed as the use case to be discussed in 6G AI.

## 2.3.1 AI-based mobility optimization

As for mobility scenarios in 5G, there are quite a few mobility types including L3 HO, LTM, CHO, DAPS, etc. When it comes to the mobility evolution in 6G, it is highly possible that the mobility related procedures could be further enhanced or simplified somehow, and also for the purpose of reducing the latency or interruption during mobility. It might seem a bit too early to make an assumption, but we tend to believe that the mobility framework would be reshaped in the 6G phase, along with the evolution of network architecture and new requirements and services in 6G. The discussion mobility for 6GR still not open. The scheme outlines for mobility mechanism in 6G, the foundation for AI/ML discussion, is unclear now, so the detailed design needs to hold on. RAN3 can conclude the general instruction or common understanding to lay the basics for future study.

6G mobility may need to explore the mechanisms for following scenarios:

- Mobility between 6G and 6G

- Mobility between 6G and 5G

5G has been commercialised for several years, and it can not expect all the 5G devices have the capability of AI/ML. To minimise the impact for existing network, the mobility from 6G to 6G can be prioritised for AI/ML for mobility optimization.

Proposal 4: AI/ML based mobility optimization prioritise the mobility between 6G and 6G.

Proposal 5: Agree the use case description for AI/ML based mobility optimization in section 5.

## 2.3.2 AI-based network energy saving

In 6G, energy consumption would remain a crucial factor that would be concerned by the operators, as energy consumption always matters to the operational cost and the environment. As 6G aims to deliver higher data rates, ultra-low latency, and massive connectivity, and considering some new services like sensing which would require constant sensing signal transmission and measurements, the energy consumption of network infrastructure is expected to increase significantly. Although, there are quite some network energy saving mechanisms designed in previous releases, the involvement of AI/ML is not sufficient, if we consider the potential benefit that AI/ML could bring in monitoring energy consumption and making network energy saving optimization decisions. There is actually not much discussion about how the AI/ML could assist with the mechanisms of Network Energy Saving, since the AI/ML-base Network Energy Saving was only discussed in Rel-18 for 5G standalone architecture, and in Rel-19 for split architecture. In other words, Network Energy Saving has been a significant concern for quite a few releases, however the AI/ML assistance for the network energy saving was only introduced for two releases, with some basic discussion of energy consumption, not touching various energy saving mechanisms and scenarios.

Given the effect that the increasing network dynamic factors, the energy saving scheme should be flexible to adapt any change with the high energy efficiency (best performance with minimum energy consumption). As a start point, for example, the mechanism of cell activation and deactivation, which was a basic mechanism supported since Rel-15 for network energy saving, could be considered in 6G early stage. If a RAN providing coverage collects enough information related to a RAN node providing capacity, it can use the AI/ML function to generate an intelligent cell activation/deactivation strategy for network energy saving, and at the same time make sure the coverage and capacity provided by the serving cells are not impacted. With the assistance of AI/ML, more intelligent and efficient decisions and actions could be taken by the RAN node, to monitor the network energy, and also to provide guaranteed services to the users without degrading network performance.

Proposal 6: AI/ML-based network energy saving takes the optimization of cell activation/deactivation as the starting point.

Proposal 7: Agree the use case description for AI/ML based network energy saving in section 5.

# Conclusion

In this paper, the following proposals are provided:

Observation 1: There is no need for unified framework across the multiple working groups.

Proposal 1: LCM framework is unified for RAN3 use cases.

Proposal 2: Service-awareness network can be one of the potential use cases for RAN3 to study.

Proposal 3: AI for network resilience can be one of the potential use cases for RAN3 to study.

Proposal 4: AI/ML based mobility optimization prioritise the mobility between 6G and 6G.

Proposal 5: Agree the use case description for AI/ML based mobility optimization in section 5.

Proposal 6: AI/ML-based network energy saving takes the optimization of cell activation/deactivation as the starting point.

Proposal 7: Agree the use case description for AI/ML based network energy saving in section 5.

# References

[1] TR 37.817 Study on enhancement for Data Collection for NR and EN-DC

[2] TR 38.743 Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[3] TS 38.300 NR; NR and NG-RAN Overall Description; Stage 2

[4] TS 38.423 Xn application protocol (XnAP)

# TP to TR 38.760-3

<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>

# 7	AI/ML for RAN

### 7.1 	General

Support of AI/ML for RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in RAN.

The objective of AI/ML for RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the RAN, which can yield further insights.

The design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope.

### 7.2 	AI/ML use cases

### 7.2.1 AI/ML-based Mobility Optimisation

#### 7.2.1.1 Use case description

Mobility management is the scheme to guarantee the service-continuity during the mobility by minimizing the call drops, RLFs, unnecessary handovers, and ping-pong. Considering the potential evolution of 6G mobility framework, the AI/ML-based mobility optimisation would still play a significant role for the mobility enhancement. The optimization mainly focuses on the mobility between 6G and 6G.

### 7.2.2 AI/ML-based Network Energy Saving

#### 7.2.2.1 Use case description

Network energy saving is an important use case which may involve different layers of the network, with mechanisms operating at different time scales. As 6G aims to deliver higher data rates, ultra-low latency, and massive connectivity, and considering some new services like sensing which would require constant sensing signal transmission and measurements, the energy consumption of network infrastructure is expected to increase significantly. The AI/ML-based Network Energy Saving in 6G could further assist with the network to generate intelligent energy saving strategies, while providing guaranteed services to the users without degrading network performance.

<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>