3GPP TSG-RAN3 #130                                                                           R3-258681

# Dallas, USA, November 17-21, 2025

Agenda item:	10.3.2

Source:	Rakuten

Title:	Discussion on RAN-CN interface

Document for:	Discussion

# Introduction

In RAN#108 meeting [1], the 6G WG SID was approved with the following objectives on RAN architecture:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we will share our initial views on SBA architecture for the RAN-CN interface.

# Overview

The evolution of mobile networks from 4G to 5G and beyond has brought significant changes in architecture and interface design. One of the pivotal shifts is the movement from traditional point-to-point (P2P) interfaces toward a Service-Based Architecture (SBA) for communication between the Radio Access Network (RAN) and the core network. This migration reflects a broader industry trend of adopting cloud-native, flexible, and modular network functions to meet the increasing demand for scalability, agility, and new service capabilities.

The service-based architecture allows for 5G Core (5GC) network functions (e.g. AMF, SMF, PCF, etc.) to be implemented as a set of software-defined services. Each service is provided by a service producer and can be consumed by one or more service consumers. The service consumers can discover available services and start using them without prior knowledge of the specific details or access points of those services. Whereas in point-to-point or peer-to-peer (P2P) model, each pair of NFs communicated with each other using a pre-established peer-to-peer signaling interface.

All intra-5GC interfaces for control-plane functionality have been defined as service-based interfaces. The only 5GC interfaces that are implemented as p2p interfaces are those connecting with NG-RAN. In legacy 5G, the NG-C (i.e., N2) interface between RAN and the Access and Mobility Management Function (AMF) of the Core Network (CN) uses NG Application Protocol (NGAP) over SCTP, while the NG-U (i.e., N3) interface carries user plane traffic encapsulated in GTP-U tunnels to the UPF (User Plane Function). The NG-C includes all control functions towards the UE, e.g., mobility management, PDU session management, security, etc, and it is realized by P2P based interface. The NG-U is mainly used to handle the data transmission between UE and DN.  The only 5GC interfaces that are still implemented as legacy (p2p) interfaces are those connecting with NG-RAN (N2), with the UPF (N4).

The dedicated P2P connections ensure low-latency, reliable communication optimized for mobility management, session establishment, and data forwarding. The predictability and stability of the point-to-point connections stem from their static and tightly coupled nature, where endpoints are pre-configured, and protocols are optimized for real-time telecom requirements.

Service Based Architecture (SBA) interface adopts a radically different approach. Instead of point-to-point links, SBA utilizes service APIs exposed via HTTP/2 and RESTful protocols over TCP, with network functions interacting as loosely coupled services. This design introduces flexibility, allowing network functions to be developed, deployed, and scaled independently in a cloud-native environment, leveraging container orchestration and microservices patterns. In 5G system, the Service-Based Architecture (SBA) was natively introduced successfully in the Core Network since Rel-15. This paradigm has proven to enable modularity, loosely coupled design, easier integration of new functionalities, smoother lifecycle management of services, and enhanced interaction across domains.

Figure 1: P2P and SBA in RAN and Core

2.1 3GPP Standardization history

The possibility of introducing SBA principles in the RAN has already been considered in the past. In SA2#146E it was proposed for Rel-18 to study a service-based interface equivalent for N2 [2] and N4 [3] that would provide a complete N2 and N4 functionality that is currently provided for all the other interfaces in the core network based on the service-based architecture. However, there was a lack of agreement during the discussion and hence, the study item phase was not pursued.

Figure 2: Proposed SBA study for N2 and N4 interfaces

Observation 1: The introduction of a service-based interface in RAN for communication between Radio Access Network (RAN) and Core Network (CN) has already been considered in the past.

With the addition of new features in 6G, we consider it important for RAN3 to conduct a feasibility study aimed at evaluating the feasibility, architectural impacts, and potential benefits of migrating from P2P to SBA in the RAN-CN interface as part of the 6G system design.

Proposal 1: We propose that RAN3 conduct a feasibility study aimed at evaluating the feasibility, architectural and performance impacts, and potential benefits of migrating from P2P to SBA in the RAN-CN interface as part of the 6G system design.

# Critical evaluation of migrating from P2P to SBA

The evolution of mobile networks toward 6G is prompting a transition from traditional Point-to-Point (P2P) interfaces to a Service-Based Architecture (SBA) for the RAN-Core connection. P2P interfaces are highly optimized for low latency and minimal jitter, benefiting real-time control and user plane data forwarding. They leverage binary protocols with minimal overhead and have mature, well-understood security mechanisms. However, the static nature of P2P interfaces limits scalability and flexibility. Adding new functions or evolving services often requires significant reconfiguration or new physical/logical interfaces, leading to slower innovation cycles. SBA, in contrast, excels in agility and extensibility by leveraging cloud-native principles to support containerized and microservice-based deployments. By exposing network functions as services with defined APIs, SBA supports rapid introduction of new capabilities such as AI/ML-based analytics, integrated sensing, and network slicing, without overhauling the entire system.

However, the shift from P2P to SBA is not without its difficulties, and several operators have valid concerns. SBA introduces additional protocol overhead, including HTTP header fields and security tokens, which can increase signaling latency. In an SBA environment, due to the statelessness, the service consumers, like the Central Unit-Control Plane (CU-CP), must first locate the correct service instance through a service registry or dictionary, adding lookup time before an API call is even made The service discovery process inherent in SBA also adds extra steps and hence, unpredictable latency, before service invocation, potentially impacting real-time performance. Unlike P2P protocols that maintain session state within dedicated tunnels for faster and more efficient communication, typical SBA APIs use stateless REST calls that require separate request and response cycles for each interaction, which can increase round-trip times.

Furthermore, the larger attack surface due to multiple exposed APIs is another vital consideration. While SBA's API exposure increases flexibility, it presents a larger attack surface by exposing core functions as separate, API-accessible services. This contrasts with P2P's fewer, more defined tunnels, which inherently limit exposure via dedicated and fixed communication links. Consequently, robust security measures, such as stringent authentication, authorization, and continuous monitoring, become essential.

Why Stick with P2P?

P2P interfaces like NGAP (N2) and GTP-U (N3) are well understood, optimized for low latency, and stable. Operators have invested heavily in these, and they meet stringent telecom requirements. Migrating core functions fully to SBA involves operational complexity and integration challenges. Sticking with P2P for legacy functions reduces risk during the critical transition. Many UEs and network elements currently deployed expect the traditional interface behavior, so maintaining P2P ensures backward compatibility. Dedicated tunnels in P2P provide predictable performance and a smaller attack surface, which is crucial for mobility management, session management, and user plane traffic.

Why Adopt SBA?

SBA enables rapid development and deployment of new services, especially complex functions like AI/ML or sensing without modifying core legacy protocols. If the RAN-CN interface is migrated to adopt SBA, then the core functions leveraging microservices, containers, and orchestration, will fit naturally with SBA’s loosely coupled API-driven design. The workloads may be distributed, requiring scalable service invocation rather than fixed tunnels.

Potential way forward: Hybrid approach?

A hybrid architecture, where legacy core functions continue to utilize P2P interfaces while newer functions and services leverage SBA, can combine the strengths of both paradigms. This approach minimizes risk and disruption by preserving the stability of proven P2P signaling for mobility and session management, while enabling innovation and scalability through SBA for AI/ML-based analytics, network slicing control, and novel 6G services. Hybrid deployments also allow operators to phase the migration gradually, managing interoperability challenges and operational learning curves. In this model, interworking gateways translate and bridge between P2P and SBA domains, maintaining session continuity and security. While this adds architectural complexity, it provides a practical migration path that aligns with business priorities and technical readiness.

Observation 2: The migration to SBA can be a complex undertaking, and many operators may choose a hybrid strategy, implementing SBA for new functions while retaining P2P for legacy systems to minimize disruption.

# Conclusion

We have the following observations:

Observation 1: The introduction of a service-based interface in RAN for communication between Radio Access Network (RAN) and Core Network (CN) has already been considered in the past.

Observation 2: The migration to SBA can be a complex undertaking, and many operators may choose a hybrid strategy, implementing SBA for new functions while retaining P2P for legacy systems to minimize disruption.

We have the following proposals:

Proposal 1: We propose that RAN3 conduct a feasibility study aimed at evaluating the feasibility, architectural and performance impacts, and potential benefits of migrating from P2P to SBA in the RAN-CN interface as part of the 6G system design.

# 5		Reference

[1] Chair Notes RAN#108

[2] 3GPP S2-2105493

[3] 3GPP S2-2106286