3GPP TSG-RAN WG3 Meeting #130	R3-258689

# Dallas, USA, 17th – 21st Nov, 2025

Agenda item:	12.2.2

Source: 	Samsung

Title: 	Discussion on AI/ML-based intra-CU LTM

Document for:	Discussion and decision

Introduction

This paper provides further discussion on AI/ML-based intra-CU LTM, based on the progress of RAN3#129bis meeting:

Prioritize normal intra-CU LTM scenarios.

Cover both L1 and L3 Measurement Report based Intra-CU LTM?

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

RAN3 to study AI/ML assisted candidate cell selection for LTM Handover Preparation

RAN3 to study TA value and validity predication for Intra-CU LTM in spatial and/or temporal domain

Beam prediction should also be considered?

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM.

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

Further discuss whether to study addition of beam prediction in UE Trajectory prediction.

Further discuss predicting TA value in the next meeting.

Further discuss prediction of validity time of a measured TA value.

Discussion

After the discussion of RAN3#129bis meeting, it has been confirmed that normal intra-CU LTM scenarios is prioritized in the study of this intra-CU LTM, but whether both L1 and L3 measurement based LTM shall be supported remains an open issue:

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM.

In our understanding, the L1 measurement based LTM and L3 measurement based compose the whole solution of LTM, including the case that L1 measurements are available, and also the case when only L3 measurements are available. During Rel-19, necessary enhancements were standardized to support CU providing L3 measurement based assistance information to DU for early sync and cell switch. We believe this scenario can also be considered in AI/ML-based intra-CU LTM, especially when CU has been deployed with AI model, which could be used to generate assistance information to DU based on L3 measurements.

Proposal 1: Consider both L1 and L3 measurement based intra-CU LTM for Rel-20 AI/ML study.

On the basis of the above proposal, we provide further discussion on our study of intra-CU LTM in the following part of this paper. The consideration is focused on the different procedures of intra-CU LTM, e.g., candidate cell selection, early UL sync, and cell switch command.

2.1 LTM Candidate Preparation

It has been agreed to study AI/ML assisted candidate cell selection, as well as candidate beams, according to the agreements of RAN3#129bis:

RAN3 to study AI/ML assisted candidate cell selection for LTM Handover Preparation

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

With the purpose of selecting suitable candidate cells and beams for LTM Handover preparation, information from RAN node and UE is needed. Firstly, the L3 measurement results from UE shall be taken as the input, considering in normal LTM cases L3 measurements are used for the gNB to select candidate cells. It should be noted that the beam level measurements in L3 measurement reports can be used for generating candidate beams of the selected candidate cells. Besides, UE location information, as per UE trajectory prediction, can also be one of the input information collected from UE.

Other information related to the UE trajectory, which is available at RAN node, can also be counted as the input information, for example, UE mobility history, predicted UE trajectory. Another type of information which could be of use, in our mind, is the MRO for LTM Information IE in the DU to CU ACCESS AND MOBILITY INDICATION message. This IE was introduced in Rel-19 in the topic of MRO for LTM in SON/MDT WI, and it enables DU to provide information related to the network performance after HO for the purpose of optimizing mobility robustness. The beam specific IE in MRO for LTM Information IE, e.g., BFR SSB Index, Target SSB Index after Cell Switch Failure, could be useful for training the AI/ML model to derive the accurate beams for candidate beam selection, with the beam failure cases taken as input.

To summarize, the following information could be used as the input information for AI/ML-based candidate cell and beam selection in intra-CU LTM:

From the UE:

-	UE location information

-	L3 measurement results, including beam level measurements

From the RAN node:

-	UE mobility history

-	UE trajectory prediction

-	MRO for LTM Information

Proposal 2: The following information can be taken as the input information or AI/ML-based candidate cell and beam selection in intra-CU LTM:

From the UE:

-	UE location information

-	L3 measurement results, including beam level measurements

From the RAN node:

-	UE mobility history

-	UE trajectory prediction

-	MRO for LTM Information

As for the feedback information, aside from legacy UE performance metrics, such as packet delay, UE throughput, packet loss rate, we believe the MRO for LTM Information can also be considered as feedback information, used for AI/ML model monitoring.

Proposal 3: Aside from existing UE performance metrics, the MRO for LTM Information can also be used as the feedback information.

2.2 Early UL synchronization

When it comes to the early UL synchronization procedure, there were some discussions regarding the TA value prediction. However, due to some concerns on the motivation of predicting value and the consequences of predicting inaccurate TA, the prediction of predicting TA value and its validity time was not agreed as an item to be studied for AI/ML-based intra-CU LTM. The corresponding open issues are listed below:

Further discuss predicting TA value in the next meeting.

Further discuss prediction of validity time of a measured TA value.

As a proponent of TA value prediction, we believe the prediction of TA value and its validity time is of much benefit to the overall procedures of LTM. If we focus on the intra-CU LTM in this sub-agenda, there are a few procedures defined in Rel-18 to allow the early TA acquisition for the purpose of early UL synchronization. Before the cell switch command is executed, UE may perform UL synchronization with LTM candidate cell(s), by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the gNB [1]. No matter it is in early sync or RACH procedure, the TA acquisition is a main reason which brings the interruption and raises the latency during handover.

Observation 1: The TA acquisition is one of the main factors that lead to the interruption during handover procedures.

By receiving the predicted TA value of candidate cells from the NG-RAN node. If the NG-RAN node, whether it’s a gNB-CU or source gNB-DU or candidate/target gNB-DU, as long as the the network node is able to provide the TA value(s) of candidate cell(s) to the UE, the TA acquisition procedure could be omitted. With this understanding, since the UE doesn’t need to send preamble to the target RAN node, there would be no need for the candidate cells to reserve early sync resources, and this would simplify the implementation of early synchronization. Last but not least, the prediction of TA value is not only applicable to intra-CU LTM, other handover procedures, generally speaking, would be optimized by using the predicted TA value to replace early TA acquisition or RACH process, so as to minimize the interruption during handover.

Based on the above discussion, the predicted TA has the following benefits:

-	Early TA acquisition procedure could be saved, by using the predicted TA value from RAN node to UE.

-	There is no need for the candidate DU to reserve early sync resources, since the UE does not need to send the preamble to acquire TA.

-	The predicted TA value is generally applicable to handover procedures, not only LTM.

Observation 2: The prediction of TA value can save procedures for early TA acquisition and reduce the interruption during HO procedures.

Some voice of objection last meeting showed the concern on the consequence of predicting an inaccurate TA, which could result in a prompt RLF without any fallback mechanism. However, we don’t think this could be a reason to avoid the TA prediction from being selected as a potential item to study. In fact, the receiving if TA value is not restricted to a hard value, according to the receiver implementation, there is always a window at the receiver, if the predicted TA value is fits in the time window, the early RACH can be successfully performed. The window size is usually around the length of CP. With the allowed offset within the length of CP, a small deviation of TA value could be tolerated. Even in the legacy RACH, a strict TA without any offset is not guaranteed, and that’s the reason why the CP is used in actual cases to give more fault-tolerance space at the receiver.

Observation 3: A predicted TA value with acceptable offset to the actual value is acceptable at the receiver, and will not result in a prompt failure.

With the above discussion and observations, we would still propose to study the prediction of TA value and its validity time.

Proposal 4: It is proposed to study the prediction of TA value for AI/ML-based intra-CU LTM.

It has been agreed last meeting that AI model inference at gNB-CU is the baseline, but in our mind, there is also some value for the source gNB-DU or candidate gNB-DU to predict TA value(s).

If the source gNB-DU predicts the TA value(s), it can directly send the TA value of the selected target gNB-DU to the UE via the cell switch command, while some part of the input information may need to be transferred via F1AP to collect essential information from the gNB-CU or candidate gNB-DU(s).

Besides, for a candidate DU, there is also justification for the candidate DU(s) to become the node who generates prediction. First of all, in the legacy mechanism for early TA acquisition, it is the candidate/target DU to measure the TA value and provides it to the UE (other than the UE itself is configured to measure the TA value), then it should be natural for the candidate DU to generate the predicted TA value using the assistance of AI/ML. Last but not least, the candidate DU or target DU itself knows the most about the cell information of itself, and could perform an accurate model inference to the most extent. In the case of candidate/target gNB-DU makes the prediction, some information about the UE may need to be transferred from the gNB-CU to the gNB-DU, which can be further studied.

It should be noted that, this paper mainly focuses on the case of intra-CU inter-DU LTM, because for intra-DU LTM, it can be understood that the source DU and the candidate DU is the same one.

Proposal 5: It can also be further studied how the source gNB-DU or candidate/target DU generate the inference of TA value(s), which means the gNB-DU model inference can also be considered.

2.3 Cell Switch Command

Regarding cell switch command, there was some discussion regarding UE trajectory. In Rel-18 and Rel-19, the UE trajectory is focused on L3 HO cases, where the predicted UE trajectory is informed to the target gNB within the Handover Request message.

For intra-CU LTM, there is no impact on normal HO request message —— the target cell and beam information is selected by the source gNB-DU according to L1/L3 measurements, or informed by the gNB-CU according to L3 measurements. With the assistance of AI/ML, we believe the selected cell and beam information for cell switch command can also be generated. Which node to generate the target cell and beam information, i.e., CU or DU could be further discussed.

Aside from the prediction of beam information for cell switch command, there was also some proposal to introduce beam prediction for UE trajectory prediction. In our mind, the so-called UE trajectory prediction is focused on the Xn HO, and shall not be discussed in the item of intra-CU LTM. Such proposals can be discussed in the sub-agenda for other HO enhancements.

Proposal 6: The AI-assisted target cell and beams for cell switch command can be studied for intra-CU LTM.

Proposal 7: Whether to introduce beam prediction for UE trajectory prediction shall be discussed in HO enhancement.

Conclusion

In this paper, the following proposals are provided:

Proposal 1: Consider both L1 and L3 measurement based intra-CU LTM for Rel-20 AI/ML study.

LTM Candidate Preparation:

Proposal 2: The following information can be taken as the input information or AI/ML-based candidate cell and beam selection in intra-CU LTM:

From the UE:

-	UE location information

-	L3 measurement results, including beam level measurements

From the RAN node:

-	UE mobility history

-	UE trajectory prediction

-	MRO for LTM Information

Proposal 3: Aside from existing UE performance metrics, the MRO for LTM Information can also be used as the feedback information.

Early UL Synchronization:

Observation 1: The TA acquisition is one of the main factors that lead to the interruption during handover procedures.

Observation 2: The prediction of TA value can save procedures for early TA acquisition and reduce the interruption during HO procedures.

Observation 3: A predicted TA value with acceptable offset to the actual value is acceptable at the receiver, and will not result in a prompt failure.

Proposal 4: It is proposed to study the prediction of TA value for AI/ML-based intra-CU LTM.

Proposal 5: It can also be further studied how the source gNB-DU or candidate/target DU generate the inference of TA value(s), which means the gNB-DU model inference can also be considered.

Cell Switch Command:

Proposal 6: The AI-assisted target cell and beams for cell switch command can be studied for intra-CU LTM.

Proposal 7: Whether to introduce beam prediction for UE trajectory prediction shall be discussed in HO enhancement.

References

[1] TS 38.300 v19.0.0

Text proposals to pCR of TR 38.745

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

4.2	AI/ML assisted Intra-CU LTM

4.2.1	Use case description

Editor’s Note: Capture the description of use case

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [2].

Intra-CU LTM is specified in TS38.401[3].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

For CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

-	AI/ML Model Training is located in the gNB-CU and AI/ML Model Inference is located in the gNB-DU

4.1.2.2		Input data of AI/ML based Intra-CU LTM

From the UE:

-	UE location information

-	L3 measurement results, including beam level measurements

From the RAN node:

-	UE mobility history

-	UE trajectory prediction

-	MRO for LTM Information

Editor’s Note: To be updated

4.1.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

- Candidate cell and beam for LTM HO Preparation

- TA value(s) for early UL synchronization

- Validity time of TA value(s)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>