3GPP TSG-RAN WG3 Meeting #130	R3-258702

Dallas, USA, 17th – 21st November, 2025

Agenda Item:	10.2.1

Source: 	OPPO

Title:  	Response to R3-258140, R3-258641, R3-258652

Document for:	Response

- Introduction

In the last meeting, the following open issue on enhanced RAN-based service awareness was captured in the chairman notes.

Study the impact of enhanced RAN-based service awareness on 6G RAN architecture.

Based on the guidance above, there are various contributions [1-7] in this meeting to discuss the enhanced RAN-based service awareness in 6G. We here provide our response to some related contributions, including R3-258140 [1], R3-258641 [2], R3-258652 [3].

In this paper, we would like to highlight the related discussion and progress which has already made in other WGs, and propose our consideration on which service require enhanced RAN-based service awareness in 6G, mainly for tokenized AI traffic. Furthermore, some examples of the tokenized AI traffic are provided, so that RAN3 can have a better understanding on the traffic characteristics of tokenized AI.

- Discussion on traffic characteristics of tokenized AI

  - Related discussion and progress in RAN1/2

In other WGs, there are already similar discussion and progress on service-awareness in RAN.

In RAN2#131bis meeting, RAN2 discussed the applications/service-awareness in the RAN and made the following agreement:

- Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework.

RAN2 agreement has already touched specific service for RAN-based service awareness in 6G, with two examples of the service, i.e., immersive communications and AI mobile traffic.

On the other hand, an LS as quoted below is also sent from RAN1 to SA4 (CC RAN2, SA1 and SA2) regarding the RAN1 studying progress on the “traffic model(s) for 6GR AI/ML services”. We understand it would also be helpful for RAN3, but unfortunately RAN3 is not involved as CC in this LS.

RAN1 is studying the traffic model and reached the following agreements in RAN1#122bis meeting: Agreement For the study traffic model(s) for 6GR AI/ML services: - A representative AI/ML service is the generative AI, e.g., as defined in TR 22.870. Send LS to SA4 (cc RAN2, SA1, SA2) requesting input if any on traffic characteristics for AI/ML services.. Note: RAN1 is discussing the following options for the model: - Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget. - Token is the minimum unit of data generated in the application layer. - How to associate Tokens to PHY layer packets. - How to reflect the variable importance of tokens. - Whether other parameters are additionally needed when tokens are encapsulated together into a packet, e.g., packet arrival rate, packet success rate, and packet delay. - Option-1b: The model is characterized by the parameters of PHY layer packet, including e.g., packet size, arrival rates, latency requirement, reliability requirement, etc. - Option-1c: reusing or extending the FTP-3/XR traffic model. - FFS other models/options need to be defined for other AI/ML services.

Therefore, we observe that AI mobile traffic is one kinds of important service for RAN-based service awareness in 6G, which has already been discussed in RAN1/2.

- AI mobile traffic is one kinds of important service for RAN-based service awareness in 6G, which has already been discussed in RAN1/2.

In our understanding, the AI mobile traffic includes both tokenized AI traffic and non-tokenized AI traffic, and the two types of AI mobile traffic have different traffic patterns and transmission requirements.

- AI mobile traffic includes both tokenized AI traffic and non-tokenized AI traffic, which have different traffic patterns and transmission requirements.

We provide some examples of the tokenized AI traffic, so that RAN3 can have a better understanding on the traffic characteristics of tokenized AI.

  - Tokenized AI

Figure 1: An example for Token-based data transmission

The above figure illustrates a typical token-based data transmission flow:

- UE Tokenizes the source information (e.g., an image) and then transmits the Token sequence to the NW side.

- The receiver de-Tokenizes the Tokens to recover the information, which is then fed into an AI service (e.g., a classification task).

Regarding the 6G new services, there is a significant trend for the fast growth of token consumption [9]. AI traffic has some special characteristics and token is the most popularly used “data” for application AI traffic. As reported recently, in some countries, daily Token consumption has exceeded 30 trillion and continues to grow rapidly [10]. In 6G, with the proliferation of mobile AI, token-based data transmission is destined to become a typical requirement and a new service modality that 6G needs to support.

- The token consumption is expected to grow rapidly.

In addition, 3GPP SA1 has begun discussing the service requirements for different services in 3GPP TR 22.870 [14], e.g., for error tolerant communication. In TR 22.870, some use cases (e.g., in clause 6.7, 6.28, 9.11, 9.14) have been agreed on how to reduce QoS (e.g., reliability) while maintaining deterministic QoE for some real time service, e.g., potentially falling under the 99.99% to 50% reliability range proposed in SA1 TR 22.870 clause 9.11. Furthermore, SA2 is discussing whether and how the adaptive QoS mechanisms for AI characteristics traffic e.g., under work task 3.2.

- In the 6G study phase, RAN1, SA1 and SA2 are studying the traffic characteristics of the tokenized AI and the potential standard impacts in each working group.

According to the token communication traffic model given in the annex, compared with “Original source data size (image)” (i.e., 256*256*3*8 bit), the “Tokenized source data size (tokenized image)” (256*12 bit) is reduced to less than 0.2% of the source file. Compared with the regular packet error rate (e.g., 5%) of real-time video or voice, the token error rate which is tolerable for user can be 20% or 50% (i.e., 10 times more resilient to the packet loss). Compared with the TN deployment, the tokenized AI service is an even more promising service for the NTN deployment scenario which has relatively limited bandwidth and the higher packet loss rate caused by mobility or link budget limitation.

- Tokenized AI is a promising service for NTN network due to the reduced transmission bit rate and the resilience to the packet loss for voice and video.

For AI services, the essential requirement is service performance (e.g., classification accuracy), not 100% correct data transmission. Note that: 100% data correctness ≠ High classification accuracy, and also Non-100% data correctness ≠ Failure to meet service requirements. Thus, the AI token is a typical scenario of error tolerant communication.

The following description shows some simulations on token-based data transmission, in which the ImageNet dataset with 3000 images belonging to 1000 categories (https://www.image-net.org) and a classification task [11] is used. Each image is converted into 256 Tokens (each Token is 12 bits), and the impact of token-based transmission error rate on the classification accuracy (Top-1/Top-5) is evaluated. As a certain TER (Token Error Rate), Top-5 shows better classification accuracy than Top-1. The classification accuracy shows significant tolerance to token errors. When errors are randomly distributed across all 256 Tokens, different TER significantly impact classification accuracy. For instance, if the requirement is only to ensure Top-5 accuracy > 90%, the system can tolerate a TER as high as 20% (TER=0.2). The detailed parameters of the token communication traffic model can be found in the Annex.

Some advanced tokenizer can generate Token representations differentiated by “importance”. For example, the 256 Tokens can be numbered from 0 to 255 in descending order of importance; a lower number indicates higher criticality for information expression, while a higher number corresponds to more detailed information. In this case, as shown in the following figures, there is a content recovery in two situations:

- only transmit a number of most important tokens without error while abandoning the rest of tokens, and

- transmit a number of most important tokens without error and transmit rest of tokens with error.

Figure 2: Exmple-1 of token-based data transmission

Figure 3: Exmple-2 of token-based data transmission

The two figures below show the roles played by token importance. Compared to transmitting all 256 tokens with randomly distributed token error rate in Figure-2, the performance can be obviously better in Figure-3, i.e., selecting and guaranteeing lossless transmission for certain number of important Tokens, while applying lossy transmission (allowing a certain error rate) to the rest of Tokens. This approach both ensures basic performance through the critical Tokens and allows flexibly enhancing accuracy through supplementary transmission, adapting to constraints like channel conditions and limited transmission resources.

Figure 4: 256 tokens are transmitted with randomly distributed token error rate

Figure 5: 256 tokens are transmitted with the 32 most important tokens transmitted losslessly

According to the above evaluations, some KPIs of token are provided in the table-1 below with considering relevant references.

Table-1 example token size

| Token type | Payload Size | Error rate | Scenario |
| --- | --- | --- | --- |
| Video | 400 bits per token, 500 tokens per video frame [13] | 0.01% for important tokens. [20%] for other tokens | Realtime video processing |
| Image | 12 bit per token, 256 tokens per image [11] |  | The picture in ImageNet data set |
| Text | 8-24 bits per token [12] | 0.01% for all tokens | Human-chatbot communication |
| NOTE: The token size in the table is for example. the token size may vary from specific scenarios. |  |  |  |

Based on the simulation given above, for some scenarios, guaranteeing the most important Tokens (e.g., top 25%) can achieve a high-quality recovery. This active selection based on Token importance, which involves discarding or downgrading QoS for non-important Tokens as needed, also helps build efficient transmission schemes, reducing the amount of data transmitted and improving efficiency. Thus, more adaptive/dynamic QoS control for AI token-based data transmission should be considered in 6G user plane design. From our understanding, the tokens generated from a source file has the following two transmission requirements:

- Error-tolerance: Not all tokens need to be transmitted correctly.

- Importance: Different tokens have different transmission reliability requirement (e.g., token error rate)

- The tokenized AI has the following traffic pattern:
1/ One source file (e.g., video frame/ picture/ text) can be used to generate a number of tokens;
2/ The source files for tokenization include at least non-real-time and real-time audio/video, picture and text;
3/ The traffic pattern (e.g., arrival rate/interval) of token has dependency on the traffic pattern of the source file.

- The tokenized AI has the following transmission requirements:
1/ Error-tolerance: Not all tokens need to be transmitted correctly;
2/ Importance: Different tokens have different transmission reliability requirement (e.g., token error rate).

In conclusion, we think that RAN3 can firstly study the traffic characteristics (including both the traffic pattern and the transmission requirements) of the tokenized AI application. And for the tokenized AI traffic, the impact on 6G RAN architecture can be based on the traffic characteristics of the token communication.

- For the service in enhanced RAN-based service awareness, RAN3 study the traffic characteristics of tokenized AI service.

Besides, we also think an LS to SA4 and SA2 is needed to ask the application-service-awareness in RAN for mobile AI traffic including tokenized AI. With this LS, we may collect more information from other WGs on the services require to be aware in RAN, as well as also to share RAN3 progress with other WGs if we have something in this meeting.

- RAN3 send an LS to SA4 and SA2 to ask the application-service-awareness in RAN for mobile AI traffic including tokenized AI.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- AI mobile traffic is one kinds of important service for RAN-based service awareness in 6G, which has already been discussed in RAN1/2.

- AI mobile traffic includes both tokenized AI traffic and non-tokenized AI traffic, which have different traffic patterns and transmission requirements.

- The token consumption is expected to grow rapidly.

- In the 6G study phase, RAN1, SA1 and SA2 are studying the traffic characteristics of the tokenized AI and the potential standard impacts in each working group.

- Tokenized AI is a promising service for NTN network due to the reduced transmission bit rate and the resilience to the packet loss for voice and video.

- The tokenized AI has the following traffic pattern:
1/ One source file (e.g., video frame/ picture/ text) can be used to generate a number of tokens;
2/ The source files for tokenization include at least non-real-time and real-time audio/video, picture and text;
3/ The traffic pattern (e.g., arrival rate/interval) of token has dependency on the traffic pattern of the source file.

- The tokenized AI has the following transmission requirements:
1/ Error-tolerance: Not all tokens need to be transmitted correctly;
2/ Importance: Different tokens have different transmission reliability requirement (e.g., token error rate).

- For the service in enhanced RAN-based service awareness, RAN3 study the traffic characteristics of tokenized AI service.

- RAN3 send an LS to SA4 and SA2 to ask the application-service-awareness in RAN for mobile AI traffic including tokenized AI.

4. Reference

[1] R3-258140, Service Aware RAN Requirements for 6G, T-Mobile USA Inc.

[2] R3-258641, Further discussions on general requirement and principles for RAN architecture, Huawei

[3] R3-258652, Discussion on RAN architecture principle and requirement, Samsung

[4] R3-258268, Discussion on 6G RAN general principles and requirements, Nokia

[5] R3-258444, Discussion on general principles and requirements for 6G RAN, CATT

[6] R3-258567, General Principles and Requirements for the 6G RAN architecture, Ericsson, Jio Platforms

[7] R3-258616, Discussion on the principle for 6G RAN architecture, CMCC

[8] R1-2508184, LS from RAN1, to SA4, (CC RAN2, SA1 and SA2), “LS on traffic model study in RAN1”

[9] Explaining Tokens — the Language and Currency of AI (https://blogs.nvidia.com/blog/ai-tokens-explained/)

[10] China Reports Surge in Token Consumption as AI Applications Expand (https://technode.com/2025/08/19/china-reports-surge-in-token-consumption-as-ai-applications-expand/).

[11] K. Miwa, et al. One-D-Piece: Image Tokenizer Meets Quality-Controllable Compression, ICML Tokenization Workshop, 2025.

[12] Wu, et al, DeepSeek-VL2: Mixture-of-Experts Vision-Language Models for Advanced Multimodal Understanding, arXiv, Dec. 2024.

[13] Cheng, Yihua, et al. "{GRACE}:{Loss-Resilient}{Real-Time} video through neural codecs." 21st USENIX Symposium on Networked Systems Design and Implementation (NSDI 24). 2024.

[14] 3GPP TR 22.870, “Study on 6G Use Cases and Service Requirements; Stage 1”

5. Annex: Token communication traffic model

Table 1: A typical Token communication traffic model

| New traffic type |  | Token communication, e.g., tokenized image |
| --- | --- | --- |
| Token based Packet (Tokenized image) | Original source data size (image) | 256*256*3*8 bit |
|  | Tokenized source data size (tokenized image) | 256*12 bit |
|  | Number of Token per image | 256 |
|  | Number of bits per token | 12 |
|  | Tokenizer and Detokenizer model | Up to implementation, [e.g., by AI models like One-D-Piece] |
|  | Packet size Tokens to be transmitted in one packet | N image, N*256 tokens to be transmitted, i.e., N*256*12/8 Bytes FFS the value of N |
|  | Packet arrival | Case1: Packet arrival in Poisson distribution Case2: Packet arrival in Periodic distribution |
|  | Packet delay budget | The value of Packet delay budget may vary for different services. |
|  | Assumptions on Token Importance | Case 1: tokens are treated without different importance Case 2: tokens are treated with different importance, e.g., with assumptions as below: - 256 Tokens can be numbered from 0 to 255 in descending order of importance; - a lower number indicates higher criticality for information expression, while a higher number corresponds to detail information. |
| Success rate requirement for token transmission | Requirements for Token Transmission | Consider the error tolerance characteristics for token communication, follow token transmission requirement should be evaluated: Case1: All tokens are transmitted with different token error rate (TER) requirements, e.g., TER=10%,20%,50% Case2: Partial tokens with high importance are transmitted, TER=0 for the selected and transmitted high importance tokens, e.g., 10%,20%,50% tokens with high importance are selected and transmitted Case3: Partial tokens with high importance are transmitted, TER=0 for the selected and transmitted high importance tokens, 10%,20%,50% tokens with high importance are selected and transmitted; and the rest tokens are transmitted with different token error rate requirements, e.g., TER=10%,20%,50% Note: The value of Success rate requirement for token transmission (based on TER) may vary for different services. |
|  | Methodology for Token error identification | Methods for Token level error identification are needed |
| Service requirement for token transmission | Assumptions on downstream service | Case 1: a classification task, [ e.g., ImageNet dataset, with N images, belonging to M categories, FFS N, M, e.g., N=3000, M=1000] - FFS whether/how to define an aligned source data set - FFS whether/how to define an aligned classification algorithm/model Case 2: FFS others |
|  | Downstream service requirement | Top-5 accuracy > 90% |
|  | Transmission requirement to guarantee the service requirement | X TER (token error rate) / BER (bit error rate) / BLER (block error rate) that could guarantee corresponded service requirement. FFS the value of X, e.g., 20% TER, or 5% BER. Y important tokens are selected for transmission that guarantee corresponded service requirement. FFS the value of Y, e.g., 25% tokens are selected based on their importance. |