3GPP TSG-RAN WG3 Meeting #130	R3-258729

# Dallas, US, 17th - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1604 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction (SSB) on the CU-DU Mobility Initiation Request message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, China Telecom, Jio Platforms, LG Electronics, Ericsson, Qualcomm, Huawei, Lenovo, CATT, ZTE Corporation, Google, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With respect to the CU-DU Mobility Initiation Request message, the tabular and ASN.1 need to be aligned. - In the message, the list of “SSB Index with measurement quantities” is transferred to the DU included in the Assistance Information IE and this is reflected in the tabular. However, in the ASN.1, single SSB Index and single CSI-RS with measurement quantities item is used instead of the list, which is clear misalignment with the tabular. - In the Candidate Cell with Beam Information List IE, the SSB Index List is optional in the tabular. It is not aligned to the ASN.1 where the list is defined as a mandatory field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - ASN.1 correction - SSBIndexwithMeasurements-Item SSBIndexwithMeasurementsList - Rename the variable for better readability: sSBIndexwithMeasurements sSBIndexwithMeasurementsList SSBIndexList-Item SSBIndex-Item - Correct the tabular - Optional Mandatory: SSB list IE in the Candidate Cell with Beam Information List - Clean up the tabular Impact assessment This CR is non-backward compatible (NBC). This CR has an impact on the structure of the CU-DU Mobility Initiation message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The CU cannot provide the list of measurement quantities with the corresponding SSB to the DU when supporting L3 measurement based LTM. - Candidate Cell with Beam Information List in the tabular remains not aligned to the ASN.1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.17; 9.3.1.348; 9.3.1.349; 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Re-submitted to RAN3#130, add co-source |  |  |  |  |  |  |  |  |

////////////////////////////////////////////////////////////////////////// Start of changes //////////////////////////////////////////////////////////////////////////

#### 9.2.2.17	CU-DU MOBILITY INITIATION REQUEST

This message is sent by the gNB-CU to the gNB-DU to trigger cell switch command and/or early synchronization for the UE.

Direction: gNB-CU ® gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE Mobility Initiation | M |  |  |  | YES | reject |
| >Mobility Trigger |  |  |  |  |  |  |
| >>Triggering Indication | M |  | BIT STRING (SIZE(8)) | Indicates the triggering of the CU-DU Mobility Initiation procedure. First bit = early UL synchronization, second bit = early DL synchronization, third bit = cell switch, other bits reserved for future use. | - |  |
| >>Cell Switch Information | C-ifTrigger IndicationCellSwitch |  |  |  | YES | ignore |
| >>>Candidate Cell with Beam Information | M |  | 9.3.1.348 |  | - |  |
| >>Early UL Sync Information | C-ifTrigger IndicationEarlyULSync |  |  |  | YES | ignore |
| >>>Candidate Cell with Beam Information List | M |  | 9.3.1.349 |  | - |  |
| >>Early DL Sync Information | C-ifTrigger IndicationEarlyDLSync |  |  |  | YES | ignore |
| >>>Candidate Cell with Beam Information List | M |  | 9.3.1.349 |  | - |  |
| >Assistance Information |  |  |  |  |  |  |
| >>Serving Cell Measurements |  | 1 |  |  | - |  |
| >>>Serving Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>>SSB Index with Measurements List |  | 1 |  |  |  |  |
| >>>>SSB Index with Measurements Item |  | 1..<maxnoofSSBIndices> |  |  |  |  |
| >>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |
| >>>>CSI-RS List |  | 0..<maxnoofCSI-RSs> |  |  | YES | ignore |
| >>>>>CSI-RS Resource ID | M |  | INTEGER (0..192) |  | - |  |
| >>>>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |
| >>Candidate Cell Measurements List |  | 1 |  |  | - |  |
| >>>Candidate Cell Measurements Item |  | 1..<maxnoofCandidateCells> |  |  |  |  |
| >>>>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>>>SSB Index with Measurements List |  | 1 |  |  |  |  |
| >>>>>SSB Index with Measurements Item |  | 1..<maxnoofSSBIndices> |  |  |  |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |
| >>>>CSI-RS List |  | 0..<maxnoofCSI-RSs> |  |  | YES | ignore |
| >>>>>CSI-RS Resource ID | M |  | INTEGER (0.. 192) |  | - |  |
| >>>>>Selected Measurement Quantities | M |  | Measurement Quantities 9.3.1.350 |  | - |  |

| Condition | Explanation |
| --- | --- |
| ifTriggeringIndicationCellSwitch | This IE shall be present if the Triggering Indication IE is set to the value "cell switch". |
| ifTriggeringIndicationEarlyULSync | This IE shall be present if the Triggering Indication IE is set to the value "early UL synchronization ". |
| ifTriggeringIndicationEarlyDLSync | This IE shall be present if the Triggering Indication IE is set to the value "early DL synchronization ". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of Cells configured towards one UE, the maximum value is 8. |
| maxnoofSSBIndices | Maximum no. of SSBs that can be served by a NG-RAN node cell. Value is 64. |
| MaxnoofCSI-RSs | Maximum no. of CSI-RSs that can be served by a NG-RAN node cell. Value is 192. |

//////////////////////////////////////////////////////////////////////////// Next change ////////////////////////////////////////////////////////////////////////////

#### 9.3.1.348	Candidate Cell with Beam Information

This IE includes a candidate cell and associated SSB index information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| SSB Index | M |  | INTEGER (0..63) |  |

#### 9.3.1.349	Candidate Cell with Beam Information List

This IE includes a list of candidate cells and associated SSB indexes information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Candidate Cell with Beam Information Item |  | 1 .. <maxnoofCandidateCells> |  |  |
| >Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >SSB Index List |  | 1 |  |  |
| >>SSB Index Item |  | 1..<maxnoofSSBIndices> |  |  |
| >>>SSB Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of Cells configured as candidates allowed towards one UE, the maximum value is 8. |
| maxnoofSSBIndices | Maximum no. of SSBs that can be served by a NG-RAN node cell. Value is 64. |

//////////////////////////////////////////////////////////////////////////// Next change ////////////////////////////////////////////////////////////////////////////

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

//////////////////////////////////////////////////////////////////////// skip irrelevant part ////////////////////////////////////////////////////////////////////////

CandidateCellwithBeamInfo	::= SEQUENCE {

candidateCellID				NRCGI,

sSBIndex			SSBIndex,

iE-Extensions		ProtocolExtensionContainer { { CandidateCellwithBeamInfo-ExtIEs } }	OPTIONAL

}

CandidateCellwithBeamInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CandidateCellwithBeamInfoList ::= SEQUENCE (SIZE(1..maxnoofCandidateCells)) OF CandidateCellwithBeamInfo-Item

CandidateCellwithBeamInfo-Item ::= SEQUENCE {

candidateCellID				NRCGI,

sSBIndexList		SSBIndexList,

iE-Extensions		ProtocolExtensionContainer { { CandidateCellwithBeamInfo-Item-ExtIEs } }	OPTIONAL

}

CandidateCellwithBeamInfo-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CandidateCellwithMeasurementsList ::= SEQUENCE (SIZE(1..maxnoofCandidateCells)) OF CandidateCellwithMeasurements-Item

CandidateCellwithMeasurements-Item	::= SEQUENCE {

candidateCellID				NRCGI,

sSBIndexwithMeasurementsList		SSBIndexwithMeasurementsList,

iE-Extensions		ProtocolExtensionContainer { { CandidateCellwithMeasurements-Item-ExtIEs } }	OPTIONAL

}

CandidateCellwithMeasurements-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CSI-RSMeasurementsList		CRITICALITY ignore		EXTENSION CSI-RSMeasurementsList				PRESENCE optional	},

...

}

//////////////////////////////////////////////////////////////////////// skip irrelevant part ////////////////////////////////////////////////////////////////////////

ServingCellMeasurements ::= SEQUENCE {

servingCellID								NRCGI,

sSBIndexwithMeasurementsList		SSBIndexwithMeasurementsList,

iE-Extensions						ProtocolExtensionContainer { { ServingCellMeasurements-ExtIEs } }	OPTIONAL,

...

}

ServingCellMeasurements-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CSI-RSMeasurementsList			CRITICALITY ignore	EXTENSION CSI-RSMeasurements-Item		PRESENCE optional },

...

}

SSBIndex  ::= INTEGER(0..63)

SSBIndexList	::=  SEQUENCE (SIZE(1..maxnoofSSBIndices)) OF SSBIndex-Item

SSBIndex-Item	::= SEQUENCE {

sSBIndex			SSBIndex,

iE-Extensions		ProtocolExtensionContainer { { SSBIndex-Item-ExtIEs } }	OPTIONAL}

SSBIndex-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBIndexwithMeasurementsList	::=  SEQUENCE (SIZE(1..maxnoofSSBIndices)) OF SSBIndexwithMeasurements-Item

SSBIndexwithMeasurements-Item	::= SEQUENCE {

sSBIndex			SSBIndex,

selectedMeasurementQuantities	SelectedMeasurementQuantities,

iE-Extensions		ProtocolExtensionContainer { { SSBIndexwithMeasurements-Item-ExtIEs } }	OPTIONAL}

SSBIndexwithMeasurements-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////////////////// End of changes ////////////////////////////////////////////////////////////////////////