3GPP TSG-RAN WG3 Meeting #130	R3-258779

Dallas, US, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1606 | Rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the CSI-RS Coordination procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, LG Electronics, ZTE, Ofinno, NTT Docomo, Jio Platforms, CMCC, Lenovo, Nokia, Ericsson, Samsung, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the CSI-RS Coordination procedure, it is not necessary to distinguish CSI-RS resources for L1 measurements and early CSI acquisition, from function perspective as the CSI resource configuration indicated by the CSI resource configuration ID can already distinguish the purpose. Likewise, when the source gNB selects an SP CSI-RS for activation, it also needs to determine the appropriate TCI State and indicate it to the candidate gNB, this ensures that the SP CSI-RS can be properly activated with the correct TCI State. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made to the XnAP for inter-CU LTM. - revise the structure and remove the differentiation of L1 measurements and early CSI acquisition. - Add the TCI state information List IE in the CSI-RS COORDINATION REQUEST message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignments exist between XnAP and F1AP. Also, candidate gNB is unable to know the TCI State for SP CSI-RS activation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.5, 9.1.5.7, 9.1.5.8, 9.3 (ASN.1) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: merged with previously endorsed CR#1540 and update based on comments during the meeting. |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 8.6.5	CSI-RS Coordination

#### 8.6.5.1	General

The purpose of the CSI-RS Coordination procedure is to enable coordination of CSI-RS transmission. The procedure uses UE-associated signalling.

#### 8.6.5.2	Successful Operation

Figure 8.6.5.2-1: CSI-RS Coordination procedure, successful operation

The NG-RAN node1 initiates the procedure by sending the CSI-RS COORDINATION REQUEST message to NG-RAN node2.

If the TCI State Information List IE is included in the CSI-RS COORDINATION REQUEST message, the NG-RAN node2 shall, if supported, use it for Semi-Persistent CSI-RS activation.

<< Next Change >>

#### 9.1.5.7	CSI-RS COORDINATION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to coordinate the activation and deactivation of CSI-RS transmission for a UE at NG-RAN node2.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| CSI-RS Coordination Request List |  | 1 |  |  | YES | reject |
| >CSI-RS Coordination Request Item |  | 1 .. <maxnoofCSIResourceConfigurations> |  |  | – |  |
| >>Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the LTM-CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |
| >>TCI State Information List |  | 0..1 |  | Indicates the TCI states where the semi persistent CSI-RS resource is transmitted. The mapping between the CSI-RS Resource indicated by the LTM CSI Resource Configuration ID IE and the TCI state is defined in TS 38.321 [35]. | - | - |
| >>>TCI state information Item |  | 1 .. < maxnoofLTM-CSI-ResourcesPerSet> |  |  | - |  |
| >>>>Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [10]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIResourceConfigurations | Maximum number of CSI Resource Configurations. Value is 112. |
| maxnoofLTM-CSI-ResourcesPerSet | Maximum number of LTM CSI-RS resource per set. Value is 512. |

<< Next Change >>

#### 9.1.5.8	CSI-RS COORDINATION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to coordinate the activation and deactivation of CSI-RS transmission for a UE at NG-RAN node2.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| CSI-RS Coordination Result List |  | 1 |  |  | YES | reject |
| >CSI-RS Coordination Result Item |  | 1 .. <maxnoofCSIResourceConfigurations> |  |  | – |  |
| >>Transmission Status | M |  | ENUMERATED(activated, deactivated, …) |  | – |  |
| >>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIResourceConfigurations | Maximum number of CSI Resource Configurations. Value is 112. |

<< Next Change >>

-- **************************************************************

--

#### -- CSI-RS COORDINATION REQUEST

--

-- **************************************************************

CSIRSCoordinationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CSIRSCoordinationRequest-IEs}},

...

}

CSIRSCoordinationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-ID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-NGRAN-Node2-ID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-CSI-RSCoordinationRequest	CRITICALITY reject	TYPE CSI-RSCoordinationRequestList				PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- CSI-RS COORDINATION RESPONSE

--

-- **************************************************************

CSIRSCoordinationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CSIRSCoordinationResponse-IEs}},

...

}

CSIRSCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NGRAN-Node1-ID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-NGRAN-Node2-ID				CRITICALITY reject	TYPE NG-RANnodeUEXnAPID						PRESENCE mandatory}|

{ ID id-CSI-RSCoordinationResponse	CRITICALITY reject	TYPE CSI-RSCoordinationResultList				PRESENCE mandatory},

...

}

<< Next Change >>

TAInformation-List,

LTMUpdatesToCandidateNodeInformation,

LTMUpdatesToCandidateCellInformation-List,

LTMUESecurityInformation,

LTMUpdatesFromCandidateCellInformation-List,

LTMCandidateCellsToBeCancelled-List,

CSI-RSCoordinationRequestList,

CSI-RSCoordinationResultList,

CLI-MeasurementResult-List,

SRS-Resource-Indication,

WAB-MT-ID,

LPWUSPSassistanceInformation,

FurtherExtended-UEIdentityIndexValue,

Future-Coverage-Modification-List,

TimeSCG-Failure,

FailureType,

LTMInformationSCG-AddReq,

LTMInformationSCG-AddReqAck,

LTMInformationSCG-ModReq,

LTMInterSNExecutionNotification,

LTMPSCellInformation-AddReq,

LTMPSCellInformation-AddReqAck,

LTMPSCellInformation-UpdateReq,

LTMPSCellInformation-UpdateReqAck,

LTMPSCellInformation-UpdateReqired,

LTMPSCellInformation-UpdateConfirm,

LTMPSCellInformation-ChangeRequired,

LTMPSCellInformation-ChangeConfirm,

LTM-DC-DataForwarding-Indicator

<< Next Change >>

maxnoofThresholds,

maxnoofEarlyRACHResourcesID,

maxnoofLTMCells,

maxNoSSBs,

maxnoofTAList,

maxnoofPreambleIndexes,

maxnoofCSIResourceConfigurations,

maxnoofNZP-CSI-RS-ResourcesPerSet,

maxnoofSRS-Resource,

maxnoofFailedSliceMeasObjects,

maxnoofSliceItemsforMDT,

maxnoofAreaNTNforMDT,

maxnoofLTMCellsPlusOne,

maxnoofSCGSecurityConfigurations,

maxnoofLTM-CSI-ResourcesPerSet

<< Next Change >>

CSI-RSCoordinationRequestList ::= SEQUENCE (SIZE(1.. maxnoofCSIResourceConfigurations)) OF CSI-RSCoordinationRequest-Item

CSI-RSCoordinationRequest-Item	::= SEQUENCE {

transmissionRequest					ENUMERATED{activate, deactivate},

cSIResourceConfigurationID			INTEGER (0..111),

tci-State-InformationList			Tci-State-InformationList 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CSI-RSCoordinationRequest-Item-ExtIEs} }	OPTIONAL,

...

}

CSI-RSCoordinationRequest-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

CSI-RSCoordinationResultList ::= SEQUENCE (SIZE(1.. maxnoofCSIResourceConfigurations)) OF CSI-RSCoordinationResult-Item

CSI-RSCoordinationResult-Item	::= SEQUENCE {

transmissionStatus					ENUMERATED{activated, deactivated},

cSIResourceConfigurationID			INTEGER (0..111),

iE-Extensions						ProtocolExtensionContainer { { CSI-RSCoordinationResult-Item-ExtIEs} }	OPTIONAL,

...

}

CSI-RSCoordinationResult-Item-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}

<< Next Change >>

<< Next Change >>

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Tci-State-InformationList	::=  SEQUENCE (SIZE(1.. maxnoofLTM-CSI-ResourcesPerSet)) OF Tci-State-Information-Item

Tci-State-Information-Item	::= SEQUENCE {

jointorDLTCIStateID			JointorDLTCIStateID,

iE-Extensions				ProtocolExtensionContainer { { Tci-State-Information-Item-ExtIEs } }	OPTIONAL,

...

}

Tci-State-Information-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<< Next Change >>

maxnoofPreambleIndexes						INTEGER ::= 64

maxnoofCSIResourceConfigurations		INTEGER ::= 112

maxnoofNZP-CSI-RS-ResourcesPerSet			INTEGER ::= 64

maxnoofSRS-Resource							INTEGER ::= 64

maxnoofFailedSliceMeasObjects				INTEGER ::= 124

maxnoofSliceItemsforMDT						INTEGER ::= 1024

maxnoofAreaNTNforMDT						INTEGER ::= 32

maxnoofLTMCellsPlusOne						INTEGER ::= 9

maxnoofSCGSecurityConfigurations			INTEGER ::= 8

maxnoofLTM-CSI-ResourcesPerSet				INTEGER ::= 512

<< End of Changes >>