3GPP TSG-RAN WG3 Meeting #130	R3-258791

Dallas, Texas, USA, 17-21 November 2025

Agenda Item:		12.2.1

Source:		NEC, Qualcomm, Huawei, Samsung

Title:			(pCR for TR38.745) Multi-hop UE trajectory across gNBs

Document for:		Agreement

# 1	Text Proposal

<Start of change>

## 4.1	Multi-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Input data of multi-hop UE trajectory

To predict the multi-hop UE trajectory, a gNB may need the following information as input data:

From the UE:

-	UE measurement report related to serving cell and neighbouring cells, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

From the neighbouring RAN nodes:

-	UE Mobility History Information

From the local node:

-	Multi-hop predicted UE trajectory

#### 4.1.2.3	Output data of multi-hop UE trajectory

The following information can be generated as output:

-	Multi-hop predicted UE trajectory, including a list of cells that UE is expected to connect to in chronological order and the associated expected time UE stays in the cell

#### 4.1.2.4	Feedback of multi-hop UE trajectory

To optimize the performance of multi-hop UE trajectory prediction, the following feedback can be considered to be collected from gNBs:

#### -	Measured UE Trajectory collected at each individual gNB

#### 4.1.2.5	Standard impact

Multi-hop UE trajectory prediction is transferred to the target NG-RAN nodes over the Xn interface via the Handover Preparation procedure. For the subsequent handovers, multi-hop predicted UE trajectory includes a list of cells where the UE is expected to connect to in chronological order.

For the subsequent handovers, Data Collection Reporting Initiation procedure needs to be in place between the source gNB and a subsequent gNB to request the reporting of the measured UE trajectory. Each target gNB directly transmits the collected measured UE trajectory back to the source gNB.

<End of change>