3GPP TSG-RAN WG3 Meeting #130	R3-258797

# Dallas, US, 17 – 21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | draft | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on multi-modality, uplink rate control and available bit rate monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Qualcomm, Nokia, Nokia Shanghai Bell, Ofinno, ZTE, Lenovo, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, RAN3 captured in XnAP specification that MN can send the the MMSID IE, Monitoring Request on Available Bitrate IE and the the Indication of Bitrate Adaptation IE to the SN, for dual-connected UE. The three IEs are used by SN for multi-modality service, available bitrate monitoring and uplink rate control. But the stage 2 description on the three features in DC case are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 13.5.2, add a paragraph about the multi-modality service support. - Add clause 13.5.X, to capture the content about uplink rate control. - Add clause 13.5.y, to capture the content about the available bitrate monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The important stage 2 description about the multi-modality service, available bitrate monitoring and uplink rate control are missing for DC case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5.2, 13.5.x, 13.5.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

13.5	eXtended Reality Services

13.5.1	Overview

The eXtended Reality Services as described in TS 38.300 [3] is extended to address the NR-DC operation.

13.5.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling.

The UE may report uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow via UE Assistance Information to the MN or the SN as configured. If the SN receives the burst arrival time, the SN may use it by considering the SFN offset of the MN.

When the PDCP hosting gNB receives the End of Data Burst Indication from the UPF, it always provides the End of Data Burst Indication to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

When the PDCP hosting gNB receives the Burst Size and/or the Time To Next Burst from the UPF, it always provides the Burst Size and/or the Time To Next Burst to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the Multi-modal Service ID (MMSID) to the SN, as part of the QoS profile for the corresponding QoS flow. It is up to the SN’s implementation to use this information for multi-modality service.

13.5.3	PSI based Discarding

For MN terminated SCG bearer and split bearer, the MN notifies SN whether the UL/DL PSI based SDU discarding is (re)configured/released for uplink/downlink discarding via XnAP signaling.

For SN terminated MCG bearer and split bearer, the SN notifies MN whether the UL/DL PSI based SDU discarding is (re)configured/released for uplink/downlink discarding via XnAP signaling.

13.5.x	Uplink Rate Control

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the information indicating that the QoS Flow allows bit rate adaptation in the uplink direction to the SN, as part of the QoS profile for the corresponding QoS flow. Then, the SN can perform uplink bit rate control for the QoS flow.

13.5.y	Available bitrate monitoring

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the request for available bitrate monitoring including the reporting thresholds to the SN, for the corresponding GBR QoS flow. Then, the SN can perform available bitrate monitoring as specified in TS 23.501[11].

End of Change