3GPP TSG-RAN WG3 Meeting # 130	R3-258810

# Dallas, US, 17 – 21, November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0490 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to stage-2 specification for support for on-demand SIB1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, Ericsson, CMCC, Nokia, Samsung, Qualcomm, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of the Rel-19 WI on enhancements of NES for NR, stage-2 details for support for on-demand SIB1 were introduced in TS 38.401. However, a few errors and improvements have been identified in the CR approved at RAN plenary: - The abbreviation for OD-SIB1 is missing. - NES Cell and Cell A are defined in TS 38.423 rather than TS 38.213 - Missing the including the F1 SETUP REQUEST message for OD-SIB1 configuration provisioning message flow when set to “start”. - Incorrect characterization of the contents of the XnAP OD-SIB1 CONFIGURATION PROVISION REQUEST message when set to “stop” - Missing notes describing alternative sequence of messages - The OD-SIB1 Configuration Provision Status Update procedure only support one Cell A Additionally, a few editorial modifications have also been identified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add abbreviation of OD-SIB1 - Change the NES Cell and Cell A references to TS 38.423 - Add the F1 SETUP REQUEST message for OD-SIB1 configuration provisioning message flow when set to “start” - Correct characterization of the contents of the XnAP OD-SIB1 CONFIGURATION PROVISION REQUEST message when set to “stop” - New notes describing alternative sequence of messages - Correct the OD-SIB1 Configuration Provision Update procedure to support multiple Cell As - Some editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lead to a defective Stage-2 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 8.25.1.1, 8.25.1.2, 8.25.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: changes the cover page and removes the editorial changes. Rev#2: add several changes and update the cover page. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

[35]	3GPP TS 37.460: "Iuant interface: General aspects and principles".

[36]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[37]	3GPP TS 38.412: "NG-RAN; NG signalling transport".

[38]	3GPP TS 38.422: "NG-RAN; Xn signalling transport".

[39]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[40]	3GPP TS 38.423: "NG-RAN; Xn application protocol (XnAP)".

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

3.1	Definitions

For the purpose of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery.

Cell A: NR cell requested to broadcast the on-demand SIB1 configuration to UE as defined in TS 38.423 [40].

Conditional Handover: as defined in TS 38.300 [2].

Conditional PSCell Addition: as defined in TS 37.340 [12].

Conditional PSCell Change: as defined in TS 37.340 [12].

DAPS Handover: as defined in TS 38.300 [2].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MP Relay UE: as defined in TS 38.300 [2].

MP Remote UE: as defined in TS 38.300 [2].

Multi-path: as defined in TS 38.300 [2].

NCR-MT: as defined in TS 38.300 [7].

NES cell: NR cell requesting on-demand SIB1 operation as defined in TS 38.423 [40].

ng-eNB: as defined in TS 38.300 [2].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

AI	Artificial Intelligence

BAP	Backhaul Adaptation Protocol

BH	Backhaul

BSR	Buffer Status Report

CAG	Closed Access Group

CHO	Conditional Handover

CLI	Cross-Link Interference

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

ML	Machine Learning

MT-SDT	Mobile Terminated Small Data Transmission

N3C	Non-3GPP Connection

NAS	Non-Access Stratum

NES	Network Energy Savings

NCI	NR Cell Identity

NDS	Network Domain Security

NID	Network identifier

NPN	Non-Public Network

NSA	Non Standalone

OAM	Operation, Administration and Maintenance

OD-SIB1	On-demand SIB1

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated Non-Public Network

PTP	Point to Point

PTM				Point to Multipoint

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.25	Procedures for network energy savings

8.25.1	Procedures for on-demand SIB1

8.25.1.1	Inter-gNB coordination for OD-SIB1 configuration provisioning

The signalling flow for the inter-gNB coordination of the OD-SIB1 configuration provisioning and start of OD-SIB1 operation is shown in Figure 8.25.1.1-1.

Figure 8.25.1.1-1: Inter-gNB coordination for the provisioning of the OD-SIB1 configuration

The procedure has the following steps:

1. gNB1-DU generates the OD-SIB1 configuration for one or more of its NES cells.

2a.	gNB1-DU sends an F1 SETUP REQUEST message or a GNB-DU CONFIGURATION UPDATE message to gNB1-CU, which includes the OD-SIB1 configuration(s) for the NES cell(s).

2b.	gNB1-CU responds to gNB1-DU with an F1 SETUP RESPONSE message or a GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message.

3a.	gNB1-CU requests gNB2-CU for the transmission of an OD-SIB1 configuration of a NES cell. It sends an OD-SIB1 CONFIGURATION PROVISION REQUEST message to gNB2-CU including the NES cell ID and its associated OD-SIB1 configuration, and it may include a Cell A ID of gNB2 for the transmission of this OD-SIB1 configuration.

3b.	In case gNB2-CU accepts the transmission of the OD-SIB1 configuration as requested, it responds to gNB1-CU with an OD-SIB1 CONFIGURATION PROVISION RESPONSE message.

NOTE 1:	In case gNB2-CU cannot accept the transmission of the OD-SIB1 configuration as requested, it responds to gNB1-CU with an OD-SIB1 CONFIGURATION PROVISION FAILURE message and the flow stops at this step.

NOTE 2:	Step 3b may occur after step 4b.

4a.	gNB2-CU sends a GNB-CU CONFIGURATION UPDATE message to gNB2-DU with an updated SIB26 for one or multiple cells, where the SIB26 includes the received OD-SIB1 configuration from gNB1-CU.

4b.	gNB2-DU responds to gNB2-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

5.	gNB2-DU cell(s) transmit(s) the updated SIB26.

NOTE 3:	Steps 6a, 6b, and 7 are a consequence of step 3b, and thus may happen before any of steps 4a, 4b, and 5.

6a.	gNB1-CU sends a GNB-CU CONFIGURATION UPDATE message to gNB1-DU, including a start indicator for OD-SIB1 operation for the NES cell, where assistance for OD-SIB1 operation is assured.

6b.	gNB1-DU responds to gNB1-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

7.	gNB1-DU may start OD-SIB1 operation for the NES cell, where a start of OD-SIB1 operation was indicated by gNB1-CU.

8.25.1.2	Inter-gNB coordination when terminating OD-SIB1 operation

The signalling flow for inter-gNB coordination for termination of OD-SIB1 operation initiated from the gNB that requests the assistance of OD-SIB1 operation is shown in Figure 8.25.1.2-1.

Figure 8.25.1.2-1: Inter-gNB coordination when terminating OD-SIB1 operation

The procedure has the following steps:

1.	gNB1-DU determines that it will no longer operate one or more of its NES cells in OD-SIB1 transmission mode.

2a.	gNB1-DU sends a GNB-DU CONFIGURATION UPDATE message to gNB1-CU, which includes a stop indicator for the NES cell(s).

2b.	gNB1-CU responds to gNB1-DU with a GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message.

3a.	gNB1-CU sends an OD-SIB1 CONFIGURATION PROVISION REQUEST message to gNB2-CU indicating that OD-SIB1 operation has stopped for a NES cell. It includes the NES cell ID for which OD-SIB1 operation has stopped.

3b.	gNB2-CU responds to gNB1-CU with an OD-SIB1 CONFIGURATION PROVISION RESPONSE message.

4a.	gNB2-CU sends a GNB-CU CONFIGURATION UPDATE message to gNB2-DU with an updated SIB26 for the Cell A(s) which have transmitted the OD-SIB1 configuration for the NES cell. The updated SIB26 excludes the OD-SIB1 configuration of this NES cell.

4b.	gNB2-DU responds to gNB2-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

5.	gNB2-DU Cell A(s) transmit(s) the updated SIB26.

8.25.1.3	Inter-gNB coordination when terminating assistance for OD-SIB1 operation

The signalling flow for inter-gNB coordination when assistance for OD-SIB1 operation is terminated by the assisting gNB is shown in Figure 8.25.1.3-1.

Figure 8.25.1.3-1: Inter-gNB coordination when terminating assistance for OD-SIB1 operation

The procedure has the following steps:

1. gNB2-CU determines that one or more Cell As should discontinue the transmission of an OD-SIB1 configuration for a NES cell of gNB1-CU.

2. gNB2-CU sends the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message to gNB1-CU including the NES cell ID of gNB1 to report that the transmissions of the OD-SIB1 configuration for the NES cell is being stopped. It may include the Cell A ID that needs to discontinue the transmission of the NES cell’s OD-SIB1 configuration in SIB26.

NOTE 1:	It is up to implementation whether step 2 occurs before or after any of steps 5a, 5b, and 6.

NOTE 2:	Steps 3a, 3b, and 4 are a consequence of step 2, and thus may happen after any of steps 5a, 5b, and 6.

3a.	gNB1-CU sends a GNB-CU CONFIGURATION UPDATE message to gNB1-DU, including a stop indicator of OD-SIB1 operation for the NES cell, where assistance for OD-SIB1 operation has been discontinued.

3b.	gNB1-DU responds to gNB1-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

4.	gNB1-DU stops OD-SIB1 operation for the NES cell, where a stop of OD-SIB1 operation was indicated by gNB1-CU.

5a.	gNB2-CU sends a GNB-CU CONFIGURATION UPDATE message to gNB2-DU with an updated SIB26 for the cell A(s) that should stop the transmission of the OD-SIB1 configuration for the NES cell.

5b.	gNB2-DU responds to gNB2-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

6.	gNB2-DU Cell A(s) transmit(s) the updated SIB26.

<<<<<<<<<<<<<<<<<<<< End of changes >>>>>>>>>>>>>>>>>>>>