3GPP TSG-RAN WG3 Meeting #130	R3-258822

# Dallas, US, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0206 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections for supporting for AI/ML-based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT, Nokia, NEC, Ericsson, CEWiT, ZTE, Samsung, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the context of the Rel-19 “Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” RAN3 introduced support for AI/ML based positioning Case 3a (AI/ML model hosted in the gNB) and Case 3b (AIML model hosted in the LMF). This CR provides clarifications on the procedural text of the new Positioning Data Collection Report procedure and some of the new IEs introduced in support of AI/ML-based positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In this CR: - the procedural text of the Inferred Measurement IE is improved to clarify that it refers only to applicable measurements requested in the MEASUREMENT REQUEST message; - the procedural text of the Positioning Data Collection Needed IE is improved to clarify that positioning data may be sent by the LMF not only if requested by the gNB but if data is actually available at the LMF side; - the general description of the Positioning Data Collection Report procedure is improved to clarify that it applies only if the NG-RAN node is a gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification for Measurement Information Transfer and Positioning Data Collection Information Tranfer procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.1.2, 8.5.2.2, 8.6.1.1, 9.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1 – To capture changes agreed in RAN3#129bis Rev.2 – Resubmission to RAN3#130 Rev.3 – To revert the change in clause 8.6.1.2 (no needed based on the agreed CR in R3-258793) |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 8.5	Measurement Information Transfer

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

*** skip unchanged text ***

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider this as the time window(s) within which to perform the positioning measurements, as specified in TS 38.305 [18].

If the Inferred Measurement IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF shall, if supported, consider that the received positioning information is an inferred version of the measurements as requested in the MEASUREMENT REQUEST message.

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

Interaction with the Positioning Data Collection Report procedure:

If the Positioning Data Collection Needed IE is included in the MEASUREMENT RESPONSE message, the LMF shall, if supported, take it into account and provide, if available, the requested positioning data in the POSITIONING DATA COLLECTION REPORT message, as specified in TS 38.305 [18].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.5.2	Measurement Report

#### 8.5.2.1	General

The Measurement Report procedure allows the NG-RAN node to report positioning measurements to the LMF. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.2.2	Successful Operation

Figure 8.5.2.2.1: Measurement Report procedure. Successful operation.

The NG-RAN node initiates the procedure by sending a MEASUREMENT REPORT message to the LMF. The MEASUREMENT REPORT message contains the measurement results according to the associated measurement configuration.

Interaction with the Positioning Data Collection Report procedure:

If the Positioning Data Collection Needed IE is included in the MEASUREMENT REPORT message, the LMF shall, if supported, take it into account and provide, if available, the requested positioning data in the POSITIONING DATA COLLECTION REPORT message, as specified in TS 38.305 [18].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.6	Positioning Data Collection Information Transfer

### 8.6.1	Positioning Data Collection Report

#### 8.6.1.1	General

The Positioning Data Collection Report procedure allows the LMF to provide positioning information to the NG-RAN node for data collection. This procedure applies only if the NG-RAN node is a gNB.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.1.5	Messages for Positioning Data Collection Information Transfer Procedures

#### 9.1.5.1	POSITIONING DATA COLLECTION REPORT

This message is sent by the LMF to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Data Information | O |  | 9.2.107 |  | YES | ignore |
| Positioning Data Unavailable | O |  | ENUMERATED(not supported, not available, …) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>