| 3GPP TR 38.745 V0.2.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group RAN; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Use cases and Solutions	6

4.1	Multi-hop UE trajectory	6

4.1.1	Use case description	6

4.1.2	Solutions and standard impacts	6

4.2	AI/ML assisted Intra-CU LTM	7

4.2.1	Use case description	7

4.2.2	Solutions and standard impacts	7

4.3	Other handover enhancements	8

4.3.1 	AI/ML assisted inter-CU LTM	8

5	Conclusion	8

Annex <A> (informative): Change history	9

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document provides the description and investigation of new AI/ML based use cases, i.e., multi-hop UE trajectory, AI/ML based intra-CU LTM, and other handover enhancements.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: " NR; NR and NG-RAN Overall Description".

[3]	3GPP TS 38.401: "NG-RAN; Architecture description".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Use cases and Solutions

4.1	Multi-hop UE trajectory

4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Input data of multi-hop UE trajectory

To predict the multi-hop UE trajectory, a gNB may need the following information as input data:

From the UE:

-	UE measurement report related to serving cell and neighbouring cells, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

From the neighbouring RAN nodes:

-	UE Mobility History Information

From the local node:

-	Multi-hop predicted UE trajectory

#### 4.1.2.3	Output data of multi-hop UE trajectory

The following information can be generated as output:

-	Multi-hop predicted UE trajectory, including a list of cells that UE is expected to connect to in chronological order and the associated expected time UE stays in the cell

#### 4.1.2.4	Feedback of multi-hop UE trajectory

To optimize the performance of multi-hop UE trajectory prediction, the following feedback can be considered to be collected from gNBs:

#### -	Measured UE Trajectory collected at each individual gNB

#### 4.1.2.5	Standard impact

Multi-hop UE trajectory prediction is transferred to the target NG-RAN nodes over the Xn interface via the Handover Preparation procedure. For the subsequent handovers, multi-hop predicted UE trajectory includes a list of cells where the UE is expected to connect to in chronological order.

For the subsequent handovers, Data Collection Reporting Initiation procedure needs to be in place between the source gNB and a subsequent gNB to request the reporting of the measured UE trajectory. Each target gNB directly transmits the collected measured UE trajectory back to the source gNB.

4.2	AI/ML assisted Intra-CU LTM

4.2.1	Use case description

Editor’s Note: Capture the description of use case

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [2].

Intra-CU LTM is specified in TS38.401[3].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

AI/ML assisted L3 and L1 measurements based intra-CU LTM with inference in gNB-CU are studied.

4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

4.2.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

For CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

4.2.2.2	Input data of AI/ML based Intra-CU LTM

Editor’s Note: To be updated

For AI/ML optimization of intra-CU LTM the following information can be considered as input data:

-	L3 measurement results

-	UE mobility history

-	Measured/Predicted radio resource status per cell/SSB area

-	Measured/Predicted cell-based UE trajectory

-	Historical UE’s candidate LTM cell and beam list(s)

-	Measured TA values

4.2.2.3	Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

-	Candidate cell and beam for LTM HO Preparation

-	Target cell and beam selection for cell switch command

-	Cell(s) and beam(s) for early synchronization

-	Predicted TA value(s) for early UL synchronization

4.2.2.4		Feedback of AI/ML based Intra-CU LTM

The following data is required as feedback data for intra-CU LTM:

-	LTM target cell

-	Measured TA value

4.3	Other handover enhancements

Editor’s Note: Identify other handover enhancements via AI/ML, e.g., inter-CU LTM

4.3.1 	AI/ML assisted inter-CU LTM

Inter-CU LTM is specified in TS 38.300 [2].

AI/ML may be used to optimise inter-CU LTM procedures, e.g., for candidate cell selection.

Editor’s Note: If applicable, use agreements for Intra-CU LTM as baseline for inter-CU LTM．

5	Conclusion

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-10 | RAN3#129-bis | R3-257331 |  |  |  |  | 0.1.0 |
| 2025-11 | RAN3#130 | R3-258086 |  |  |  |  | 0.1.1 |