3GPP TSG RAN WG3 Meeting # 130
R3-258802
Dallas, US, 17th – 21th November, 2025 
Agenda Item:
9.2.10
Source:
CATT (moderator)

Title:
Summary of the offline discussion on CB # 24_R19 SBFD
Document for:
Approval

Introduction

CB: # 24_R19

- Further discuss 8647, e.g. whether SBFD Additional RACH Configuration is needed

- Further discuss benefits of 8686

- Check Stage 2 (38.420, 38.300, 38.401), whether agreeable

(CATT - moderator)

For the Chairman’s Notes

Propose the following CRs:

R3-258647 rev in R3-25xxxx – agreed

R3-258686 rev in R3-25xxxx – agreed

R3-258607 rev in R3-25xxxx – agreed

R3-258639 rev in R3-25xxxx – endorsed

R3-258640 rev in R3-25xxxx – agreed
Discussion 

 Stage 3 CRs
 R3-258647
	R3-258647
	Correction on SBFD RACH configuration for F1AP (CATT)
	CR1667r, TS 38.473 v19.0.0, Rel-19, Cat. F

E///: SBFD Additional RACH Configuration is not needed, and “RACH configuration option 2” should be “dual configuration”

SS, QC: SBFD Additional RACH Configuration is needed


- Further discuss 8647, e.g. whether SBFD Additional RACH Configuration is needed
The change as below

9.3.1.140
NR PRACH Configuration List

Unchanged part is not copied
	>SBFD Additional RACH Configuration 
	O
	
	ENUMERATED (true, …)
	Indicate whether PRACH Configuration is for SBFD RACH option 2.
	YES
	reject

	>SBFD Across Symbol Type
	O
	
	ENUMERATED (true, …)
	Indicate whether a configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots is valid for RACH configuration option 2.
	YES
	reject


Conclusion:
  R3-258686
	R3-258686
	Discussion on the open issues regarding L1 UE-to-UE CLI configuration (Samsung, Qualcomm, Charter)
	discussion

It should be DU that generates and compiles the CellGroupConfig, and CU only compiles the RRC message with the CellGroupConfig included as OCTET STRING.

Include an indication for L1 UE-to-UE CLI configuration from DU to CU in the DU to CU RRC Information IE over F1.

HW: It’s optimization, not essential

QC: L1 and L3 should not be configured simultaneously and needed to avoid it

ZTE, Nokia, CATT: CR is beneficial

E///: solution does not ensure that L3 measurements are not configured, e.g. due to F1 delay. OAM solution is most reliable

	R3-258687
	Correction on the indication of L1 UE-to-UE CLI (Samsung, Qualcomm, Charter)
	CR1598r1, TS 38.473 v19.0.0, Rel-19, Cat. F


- Further discuss benefits of 8686
The change as below

9.3.1.26
DU to CU RRC Information

Unchanged part is not copied
	L1 CLI Measurement Configuration Indication
	O
	
	ENUMERATED (true, ...)
	Indicates that L1 UE-to-UE CLI measurement configuration has been included in CellGroupConfig.
	YES
	ignore


Conclusion:
 Stage 2 CRs
R3-258607
	R3-258607
	Correction on Evolution of NR duplex operation for 38.420 (CMCC, CATT)
	CR0053r, TS 38.420 v19.0.0, Rel-19, Cat. F


The change as below

5.2.16
CLI indication function
This function allows the transfer of gNB-to-gNB CLI measurement results, gNB-to-gNB CLI mitigation request and SRS Resource indication between NG-RAN nodes.

Conclusion:
R3-258639
	R3-258639
	Clarification to UE-to-UE CLI mitigation in SBFD operation (Huawei, CATT, China Unicom)
	draftCR


The change as below

17.2
Cross-Link Interference Management

Unchanged part is not copied
A gNB serving victim UEs may request neighbour gNBs to report SRS resources. The neighbour gNB that receives the request may signal the information concerning SRS resources potentially causing UE-to-UE CLI back to the neighbour gNB(s) that initiated the request.
Conclusion:
R3-258640
	R3-258640
	Clarification to CLI mitigation in SBFD operation (Huawei)
	CR0503r1, TS 38.401 v19.0.0, Rel-19, Cat. F


The change as below

7.3
Cross-Link Interference Management
Unchanged part is not copied
For the case of detection of UE-to-UE CLI, a gNB-DU that has activated SBFD operation may provide SRS resource configuration to the gNB-CU. A gNB-DU serving potential victim UEs may indicate to a gNB-CU that SRS resource configurations of neighbour cells are needed. The gNB-CU may signal to the gNB-DU information received from neighbour gNBs concerning SRS resources potentially causing UE-to-UE CLI. For inter-UE CLI for SBFD operation in split architecture, simultaneous configuration of L1 and L3 measurements shall be avoided.
 Conclusion, Recommendations [if needed]

If needed

 References

