3GPP TSG-RAN WG3 #131	R3-260061

# Gothenburg, Sweden, 9 – 13th February, 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Ephemeris orbital parameters and epoch time provided by O&M for NR TN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES, Apple, Iridium, CATT, Airbus, ESA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2026-01-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For EphemerisInfo expressed as orbital parameters the coordinate system used is ECI (Earth-Centered Inertial) frame. The reference to NIMA TR 8350.2 is not relevant here and might be confusing. The reference to NIMA TR 8350.2 implies the frame is WGS-84 ECEF(Earth-Centered, Earth-Fixed). To avoid ambiguity, a should remove the reference to NIMA TR 3850.2 when using parameters in orbital parameter ephemeris format. the explicit epoch time associated with the ephemeris data provided by O&M to the gNB providing NTN access should be an absolute and unambiguous time reference, it would be beneficial to specify that the J2000,0 reference epoch is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the reference to NIMA TR 3850.2 when Ephemeris Informations are using the orbital parameter ephemeris format. Clarify also that Epoch time provided by O&M to the gNB providing NTN access is using the J2000,0 standard reference epoch. Impact analysis Impacted functionality: NTN orbital parameter data Inter-operability: If the network is implemented according to the CR but the UE is not, or vice versa, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion regarding the applicable coordinate system. Users may incorrectly interpret the orbital parameter’s frame as WGS-84 ECEF instead of ECI, leading to misimplementation or misinterpretation of orbital parameter data. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 16.14	Non-Terrestrial Networks

<Unchanged parts are omitted>>

### 16.14.7	O&M Requirements

The following NTN related parameters shall be provided by O&M to the gNB providing NTN access:

-	Ephemeris information describing the orbital trajectory information or coordinates for the NTN payload of the serving cell, and optionally for the NTN payload(s) of neighbour cell(s). This information is provided on a regular basis or upon demand to the gNB;

-	Two different sets of ephemeris format shall be supported:

-	Set 1: NTN payload position and velocity state vectors:

-	Position;

-	Velocity.

-	Set 2: At least the following parameters in orbital parameter ephemeris format:

-	Semi-major axis;

-	Eccentricity;

-	Argument of periapsis;

-	Longitude of ascending node;

-	Inclination;

-	Mean anomaly at epoch time.

-	The explicit epoch time associated to ephemeris data, with respect to the J2000,0 reference epoch;

-	The location of the NTN Gateways;

NOTE 1:	The ephemeris of the NTN payloads and the location of the NTN Gateways, are used at least for the Uplink timing and frequency synchronization. It may also be used for the random access and the mobility management purposes.

-	Additional information to enable gNB operation for feeder/service link switch overs.

NOTE 2:	The NTN related parameters provided by O&M to the gNB may depend on the type of supported service links, i.e., Earth-fixed, quasi-Earth-fixed, or Earth-moving.

END OF CHANGES