# 3GPP TSG-RAN WG2 Meeting #131    	   			               R3-260259

# Goteborg, Sweden, 9 – 13, February 2026

Agenda item:	14.2

Source: 	Samsung

Title: 	Discussion on Topology 2 for AIoT

Document for:	Discussion and decision

Introduction

Last meeting discussed the support of Topology 2 for AIoT, and the following agreements are captured,

NGAP: Include UE Reader authorization status (authorized, not authorized) in the NGAP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST.

WA: XnAP: The source/old gNB informs the target/new gNB about the UE Reader authorization status (authorized, not authorized), via XnAP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE messages.

No XnAP protocol functions needed for inter-gNB coordination of A-IoT radio resources.

UE Reader authorization status in PATH SWITCH REQUEST ACKNOWLEDGE message.

Whether and how for the gNB-CU to inform gNB-DU about the UE Reader authorization status.

UE Reader Selection.

Reader ID to represent UE Reader. (NOTE: by taking SA2 progress, if any, into account)

How to support Inter-gNB RRC-Connected UE Reader Mobility.

In this contribution, we further discuss the support of Topology 2 for AIoT from RAN3 perspective.

Discussion

Last meeting continued to discuss the T2 for AIoT.

The first set of open issues is about the authorization, and the following open issues are captured,

WA: XnAP: The source/old gNB informs the target/new gNB about the UE Reader authorization status (authorized, not authorized), via XnAP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE messages.

UE Reader authorization status in PATH SWITCH REQUEST ACKNOWLEDGE message.

Whether and how for the gNB-CU to inform gNB-DU about the UE Reader authorization status.

According to the discussion, some companies had concerns on whether the authorization information is needed over F1AP.

In our understanding, the authorization information transfer over XnAP is needed because of the AIoT session continuity. It is beneficial for the UE reader to maintain the AIoT session during mobility, and one of the major benefits is that the UE reader does not need to start over with the inventory procedure to communicate with an AIoT device which is still under the coverage of the UE reader, which will 1) save the effort of duplicate inventory; 2) reduce the chance of collisions caused by unnecessary random accesses; and 3) continue with the Command procedures which reduces the latency of the end-to-end communication between AF and AIoT device.

Observation 1: It is beneficial for the UE reader to maintain the AIoT session during mobility.

In addition, there is possibility for the operators to deploy multiple AIoT RAN nodes in case of T2 to perform AIoT services in a specific area. Therefore, the authorization information is needed to be transferred over XnAP when the UE reader mobility happens.

Proposal 1: Turn WA into agreement: XnAP: The source/old gNB informs the target/new gNB about the UE Reader authorization status (authorized, not authorized), via XnAP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE messages.

Proposal 2: The target node makes decision on whether the UE reader keeps the ongoing AIoT session during mobility.

On whether the authorization information is needed over NGAP PATH SWITCH REQUEST ACKNOWLEDGE message, since the CN decides whether the UE reader is authorized or not, there’s no harm to also include it in the path switch ack message to update the authorization status as soon as possible.

Proposal 3: Include UE Reader authorization status in NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

On whether to transfer UE reader authorized information over F1AP, in our understanding, the answer depends on whether gNB-DU will perform any AIoT related operation, such as providing lower layer AIoT configuration (i.e. AIoT MAC/PHY configuration) for UE reader, or performing AIoT dynamic resource allocation.

Although currently little progress has been achieved on how the AIoT radio resources will be provided from RAN node to UE reader, there is a tendency in RAN1 to follow Sidelink Mode 1 like mechanism for AIoT radio resource allocation, which means that the AIoT RAN node is able to dynamically allocate AIoT radio resources to the UE for AIoT radio. Thus, if such Sidelink Mode 1 like mechanism is followed for AIoT radio resource allocation, the authorization information is needed to be transferred from CU to DU.

So the problem now is whether companies can achieve common understanding on whether Sidelink Mode 1 like mechanism is used by AIoT radio resource allocation (at least seems so for us). If RAN3 cannot achieve common understanding, maybe an LS to RAN1 is helpful.

Proposal 4: Include UE Reader Authorized information in the following messages: F1AP UE CONTEXT SETUP REQUEST message, F1AP UE CONTEXT MODIFICATION REQUEST message.

For UE reader selection, recall that for BS reader selection in R19, several descriptions have been captured in our TR or TS,

- In the latest TS 38.300, the gNB shall select BS readers if the Requested Reader List is absent.

- In the latest TS 38.300, the gNB shall take the list into account if the Requested Reader List is present.

- In the latest TR 38.769, the sentence stating that in RRC based solution, UE Reader selection may need coordination between AIoT RAN and AIoT CN is captured.

It can be observed from current spec that the gNB has the final say on which BS reader is selected.

Observation 2: In current spec, the gNB can make the final decision on which BS reader is selected.

While for the UE reader, different from the BS reader which is fixed in the location, the UE reader may move so the location of the UE reader can change from time to time. If we reuse similar design logic as BS reader selection, and still allow the gNB to have the final say on which UE reader to be selected, the first thing needed to be checked is whether the gNB is able to obtain the location of the UE reader with enough precision.

- The gNB is able to obtain the location info of a UE by means of MDT, which is also dependent on User consent. So if the UE reader does not consent to perform MDT related measurement, the gNB is unable to obtain the location info of the UE.

- For current UE positioning method (including GNSS method or NR RAT dependent positioning methods), the calculation of the UE location is performed either by UE or LMF for all of the positioning method, for which the UE location info is agnostic to the gNB. So at least the gNB is unable to obtain the UE location info by means of current UE positioning method.

- The gNB is able to obtain the Cell ID, Timing Advance, SS/CSI-RSRP/RSRQ and AoA of a UE by means of E-CID positioning method. Whether the gNB is allowed to use such information to perform location estimate by implementation, and whether the positioning accuracy is good enough is still unclear.

As a result, for UE reader selection, if the Requested Service Area Information contained in the Inventory Request message contains,

- Neither the Requested AIoT Area List nor the Requested Reader List, the gNB shall select all UE readers it serves

- The Requested AIoT Area List, whether the gNB is able to select UE readers within the indicated AIoT Area is unclear

- The Requested Reader List, the gNB shall take the reader list into account, but whether the gNB is able to select UE readers out of the Requested Reader List is unclear

In our understanding, the LMF is able to obtain the UE location info, but whether the LMF always exists in AIoT CN, and whether such UE location info can be used by AIOTF when generating the Requested Service Area Information needs further discussion in SA2.

On the other hand, another open issue we need to discuss is whether the UE reader can move in a pre-planned path. In current spec, the UAV path info can be obtained from UE by the gNB. In out understanding, in some cases e.g. Smart factory, the UE reader can be mounted on unmanned vehicle which follows a fixed path in motion, so it is beneficial to introduce the path info for a UE reader, and if the UE reader path info can be obtained by the gNB, the gNB is also able to decide the UE readers to be used for a service request.

In order to move forward, firstly when neither the Requested AIoT Area List nor the Requested Reader List is contained, the gNB shall select all UE readers it serves, similar to legacy behaviour.

While for other cases, there are two alternatives,

- Alternative 1: RAN3 confirms that the gNB is able to obtain the UE location related info with enough accuracy by implementation. For Alternative 1, UE reader selection is performed by the gNB.

- Alternative 2: RAN3 assumes the AIOTF is able to obtain the UE location related info with potential enhancement in AIoT CN, and sends LS to SA2 to confirm such assumption. For Alternative 2, UE reader selection is performed by the AIoT CN. For Alternative 2, when only the Requested AIoT Area List is provided, the gNB shall select all UE readers served by all the cells within the AIoT Area List.

Proposal 5: RAN3 is kindly asked to discuss whether it is possible that a UE Reader will follow a pre-planned moving path, and whether it is beneficial to adopt the concept of UE reader path info in the spec.

Proposal 6: When neither the Requested AIoT Area List nor the Requested Reader List is contained in Inventory Request message, the gNB shall select all UE readers it serves.

Proposal 7: RAN3 is kindly asked to discuss whether the gNB needs to obtain the UE location related info with enough accuracy, in order to perform reader selection.

Another issue is that whether and how the UE Reader Index is updated. And as a basis, the following open issue is captured,

Reader ID to represent UE Reader. (NOTE: by taking SA2 progress, if any, into account)

By looking into the latest version of TR 23.700-30, it seems that SA2 has not achieved progress on how the UE reader ID is allocated.

Recall that in R19, the BS Reader Index is configured to AIoT CN by OAM; while for UE Reader, since the UE Reader mobility may happen, it is no longer proper to also configure UE Reader Index by OAM; instead, the UE Reader Index may change when the UE Reader changes the serving gNB.

As a result, we need to figure out a solution on how to change the UE Reader Index when the UE Reader changes the serving gNB.

Basically there are two options,

- Option 1: The UE Reader Index is allocated by AIoT CN, and the updated UE Reader Index can be contained in existing UE-associated NGAP message, such as NGAP UE CONTEXT MODIFICATION REQUEST message.

- Option 2: The UE Reader Index is allocated by AIoT RAN node, and the updated UE Reader Index can be contained in existing UE-associated NGAP message, such as NGAP PATH SWITCH REQUEST message.

In fact, we do not see strong need to introduce new message to convey the updated UE Reader Index. Actually in our understanding, if there is UE reader, the AIoT CN should contain AMF to handle the mobility of the UE reader; when the mobility happens by UE reader, the AMF which is responsible of Mobility Management anyway needs to update the UE reader mobility related information with AIOTF. So reusing the existing NGAP UE-associated message is the simplest and the most straight-forward way to update the UE Reader Index.

Compared to Option 2, in our opinion, Option 1 is more proper since AIoT CN has more knowledge on UE Reader such as the UE reader location, the UE Reader Index assignment for all UE Readers in the network, so AIoT CN may have a better decision on the UE Reader Index selection. Therefore, we slightly prefer the UE Reader Index is allocated by AIoT CN.

Proposal 8: When inter-gNB mobility happens, the UE Reader Index is allocated and signalled to AIoT RAN node by AIoT CN via existing UE-associated NGAP message.

The last open issue is as follows,

How to support Inter-gNB RRC-Connected UE Reader Mobility.

In our understanding, the above open issue indicates the discussion on what additional information is needed to be introduced over XnAP HANDOVER REQUEST message.

Firstly, in order to maintain AIoT service continuity for the UE reader, basically the AIOTF ID and Correlation ID should be signalled over XnAP HANDOVER REQUEST message.

In addition, in order to support AIoT inventory continuity for the UE reader, the following information can be considered,

- Requested AIoT area information: In general, this information is needed by the target node in order for the target node to decide when to trigger AIOT session release request when the UE reader moves out of the service area. It should be noted that the requested reader list is not needed by the target node because such information will be outdated if the UE reader is moving.

- Inventory assistance information: Such information includes Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval. And such information is needed by the target node to continue to perform inventory operations.

- Follow on command indication: This information is needed by the target node to decide when to trigger AIOT session release request.

And in order to support AIoT command continuity for the UE reader, the following information can be considered,

- RAN A-IoT Device NGAP ID list: This information is used to associate specific devices between RAN node and AIOTF, and such ID is unique on a per AIoT service (i.e. Correlation ID) basis. So such information is needed by the target node when AIOTF sends Command related messages to the target node.

- ID list between UE reader and gNB to associate with specific devices: This ID is agreed by RAN2 and the details is FFS in RAN2. Such ID is needed by the target node when RAN node sends Command related messages to the UE reader.

Proposal 9: The following information is needed to be signalled over XnAP HANDOVER REQUEST message in order to support inter-gNB RRC-Connected UE Reader Mobility:

- AIOTF identifier

- A-IoT Correlation Identifier

- Requested A-IoT Area List

- Inventory Assistance Information including Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval

- Follow on Command Indication

- RAN A-IoT Device NGAP ID list

- ID list between UE reader and gNB to associate with specific devices

Conclusion

In this paper, we provide discussion on Topology 2 for Ambient IoT. The following proposals are provided,

Observation 1: It is beneficial for the UE reader to maintain the AIoT session during mobility.

Proposal 1: Turn WA into agreement: XnAP: The source/old gNB informs the target/new gNB about the UE Reader authorization status (authorized, not authorized), via XnAP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE messages.

Proposal 2: The target node makes decision on whether the UE reader keeps the ongoing AIoT session during mobility.

Proposal 3: Include UE Reader authorization status in NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

Proposal 4: Include UE Reader Authorized information in the following messages: F1AP UE CONTEXT SETUP REQUEST message, F1AP UE CONTEXT MODIFICATION REQUEST message.

Observation 2: In current spec, the gNB can make the final decision on which BS reader is selected.

Proposal 5: RAN3 is kindly asked to discuss whether it is possible that a UE Reader will follow a pre-planned moving path, and whether it is beneficial to adopt the concept of UE reader path info in the spec.

Proposal 6: When neither the Requested AIoT Area List nor the Requested Reader List is contained in Inventory Request message, the gNB shall select all UE readers it serves.

Proposal 7: RAN3 is kindly asked to discuss whether the gNB needs to obtain the UE location related info with enough accuracy, in order to perform reader selection.

Proposal 8: When inter-gNB mobility happens, the UE Reader Index is allocated and signalled to AIoT RAN node by AIoT CN via existing UE-associated NGAP message.

Proposal 9: The following information is needed to be signalled over XnAP HANDOVER REQUEST message in order to support inter-gNB RRC-Connected UE Reader Mobility:

- AIOTF identifier

- A-IoT Correlation Identifier

- Requested A-IoT Area List

- Inventory Assistance Information including Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval

- Follow on Command Indication

- RAN A-IoT Device NGAP ID list

- ID list between UE reader and gNB to associate with specific devices

Reference