3GPP TSG-RAN WG3 Meeting #131	R3-260270

Goteborg, Sweden, February 9 – 13, 2026

Agenda Item:	10.3.2.2

Source:	Pengcheng Laboratory

Title:	Discussion on Service Based Interface (SBI) of RAN-CN control plane

Document for:	Discussion and Decision

Introduction

The following were captured in the 6G SI which was approved in RAN plenary:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

Base on the the agreement captured in RAN3 #131 meeting for RAN-CN control plane interface[2], in this contribution, we further provide our view on the 6G RAN-CN interface, and provides the related pCR to the TR 38.760-3.

Discussion

The interfaces in 5G between the RAN and CN are separated into the user plane and control plane, a design that will also be applicable to 6G architecture,  For the control plane and it’s functional split the between RAN and CN. Building on this, the interface between RAN node to AMF is P2P, the interface between AMF and other NF in CN are service based.

Figure 4.2.3-1: Non-Roaming 5G System Architecture

There have been extensive discussions across various companies on the functional and service split of P2P and SBI interfaces, as well as whether legacy and enhanced interfaces are needed to support legacy services and functions.

P2P interfaces have advantages focus on mobility, low latency, heterogeneous link adaptation, and the reliability of multi-stream signaling/data bearing, while SBI service-based interfaces focus on high concurrency, elastic scaling of NFs, service invocation efficiency, and lightweight adaptation to cloud-native environments.

If RAN3 differentiates between legacy and new types at the service or functional level, such a distinction can also be implemented through interfaces. For instance, the existing services and functions in 4G and 5G can be clearly defined as legacy services, and as the S1 and NG interfaces in 4G and 5G are both point-to-point interfaces, these legacy services can be supported by well-defined legacy P2P interfaces.

Regarding the introduction of QUIC, its benefits specifically when compared to TCP+TLS or SCTP can be summarized as follows:

- QUIC vs SCTP:

- Generally, QUIC has advantages in weak radio environments. However, compared to SCTP/TCP+HTTP2, it is a relatively new protocol, and some legacy equipment may not support it. Therefore, completely replacing traditional methods with QUIC does not necessarily offer advantages in terms of operations and maintenance (O&M) or performance. However, in some scenarios such as Non-Terrestrial Networks (NTN), or when the link environment between the RAN and CN is poor with high jitter, features like FEC (Forward Error Correction) in QUIC will yield significant benefits. Therefore, even for traditional functions, the design of QUIC should not be excluded.

Observation 1: QUIC can provide advantages in NTN or low-quality environment than SCTP. Features in QUIC can offer significant benefits. The design of QUIC should not be excluded for legacy functions/services.

- QUIC VS TCP+TLS:

- The comparison between QUIC VS TCP+TLS follows similar selection logic with their usage in other system,and also need to consider the adapt to the SBA architecture in the CN, including potential CN support for HTTP3. The following table outlines the scenarios where each protocol holds an advantage:

| Dimension | TCP + TLS + HTTP/2 | QUIC + HTTP/3 |
| --- | --- | --- |
| Concurrency Capability | Disadvantage: Suffers from TCP Head-of-Line (HOL) Blocking. A single packet loss can block all requests on the connection. | Advantage: No HOL blocking. Streams are independent, offering stable performance under extremely high concurrency (e.g., massive UE access). |
| Connection Latency | Disadvantage: Slow handshake (TCP + TLS), typically requiring 3 RTTs to send data. | Advantage: Supports 1-RTT or 0-RTT. Huge advantage in scenarios requiring frequent short connections or rapid responses. |
| Resource Consumption | Advantage: The OS kernel optimizes TCP efficiently; memory and CPU usage are relatively low. | Disadvantage: UDP packet handling and user-space stack processing consume significantly more server CPU resources. |
| Adaptation | Disadvantage:requiring application-level re-connection for TCP and potentially causing service interruptions. | Advantage: Supports connection migration， transparent to upper-layer services. |
| Hardware or Middleware Support | Advantage: Existing support is extremely mature. | Disadvantage: Support for HTTP/3 (UDP) load balancing in some legacy gateways is still being perfected. |

- Observation 2: HTTP/2 is suitable for stable long-connection scenarios; QUIC is suitable for scenarios with frequent switching.

Proposal 1: For both P2P and SBI interfaces, legacy solutions such as SCTP/TCP+HTTP/2 have clear advantages in stable internal networks or scenarios with limited hardware resources; QUIC+HTTP/3 has better  performance in scenarios with poor network stability, high latency, high packet loss or frequent node address changes.

Proposal 2: Agree to capture the pCR in the Annex。

Conclusion

The following observations and proposals are made.

Observation 1: QUIC can provide advantages in NTN or low-quality environment than SCTP. Features in QUIC can offer significant benefits. The design of QUIC should not be excluded for legacy functions/services.

- Observation 2: HTTP/2 is suitable for long-connection scenarios; QUIC is suitable for scenarios with frequent switching.

Proposal 1: For both P2P and SBI interfaces, legacy solutions such as SCTP/TCP+HTTP/2 have clear advantages in stable internal networks or scenarios with limited hardware resources; QUIC+HTTP/3 has better  performance in scenarios with poor network stability, high latency, high packet loss or frequent node address changes.

Proposal 2: Agree to capture the pCR in the Annex。

- References

- RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

- R3-258859 pCR to TR 38.760-3 on 6G RAN-CN interface options

pCR to the TR 38.760-3

--------------Start of the change---------------

### 6.1.3	RAN-CN Interface Options

This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

#### 6.1.3.1 RAN-CN Control Plane

Editor’s Note: The encoding (e.g., ASN.1) for control plane interface options below is FFS.

##### 6.1.3.1.1 Point to Point (P2P)

A RAN-CN P2P interface refers to application layer communication between the 6G RAN node and the CN entity for 6G by means of elementary procedures, either triggered by the 6G RAN node or by the CN entity for 6G.

Editor’s Note 1: FFS whether multiple CN entities can be involved.

Potential options for the 6G P2P protocol stack are as follows:

Editor's Note 2:	Other options are not precluded.

-	SCTP based

-	QUIC based

##### 6.1.3.1.2 Service Based Interface (SBI)

A RAN-CN SBI (service-based interface) refers to application layer communication between the 6G RAN node and the CN entity for 6G by means of services provided/exposed by either the 6G RAN node or the CN entity for 6G.

Editor’s Note 1: FFS whether multiple CN entities can be involved.

Potential options for the 6G SBI protocol stack are as follows:

Editor's Note 2:	Other options are not precluded.

-	TCP+ HTTP/2 based

Figure 6.1.3.1.2-1：SBI Protocol Stack based on TCP+HTTP/2

The legacy NGAP or enhanced NG+AP stack is used in SCTP based protocol stack.

NOTE: The description of NG+ will be updated according to the discussion

-	QUIC+HTTP/3 based

Figure 6.1.3.1.2-2：SBI Protocol Stack based on QUIC+HTTP/3

The legacy NGAP or enhanced NG+AP stack is used in SCTP based protocol stack.

NOTE: The description of NG+ will be updated according to the discussion

NOTE:	The application layer serialization protocol is FFS.

The usage of above two options can depend on the specific scenarios and network requirements, which either chosen or by the 6G RAN node or by the CN entity for 6G;

---------------End of the Change------------------