3GPP TSG-RAN WG3 #131	R3-260611

Goteborg, Sweden, 9 – 13 Feb. 2026

Agenda Item:	10.3.2.3 Evaluation criteria

Source:	NTT DOCOMO INC.

Title:	Necessary considerations for 6G RAN-CN interface options

Document for:	Discussion

Introduction

In the previous meeting, RAN3 discussed the transport protocol of RAN-CN interface and captured candidate solutions in TR [1]. As stated in 6G SID [2], we need to analyze the advantages and challenges of each solution by June 2026. This contribution provides the necessary perspective from the operator's point of view to perform these analyses.

Discussion

Analysis of current protocol design for C-plane

In current specifications, C-plane RAN interfaces including RAN-CN interfaces adopt a protocol stack using SCTP [3]. This assumes an end-to-end connection, and the IP address of the endpoint is signaled as a TNL address in NGAP and other RAN IFs’ protocols. SCTP has an integrated heart-beat function, and connection life management using this function is widely used by operators. Regarding security, 3GPP supports DTLS and IPsec [4], and these security functions can be used depending on the deployment environment of RAN nodes and transport networks. From the architecture perspective, the architecture in which RAN-CN interfaces are terminated by AMF enables simple procedures and management.

On the other hand, in the architecture where AMF is a single endpoint of RAN-CN interface, if signaling between RAN and NF other than AMF increases, the load may be concentrated on AMF which has fundamental functions related to access and mobility. From the security point of view, when signaling through SecGW for IPsec, additional latency of around tens of ms may occur because SecGW is usually placed centralized. In NG interface, this effect is relatively small because AMF is also placed centralized as well as SecGW, but it is especially noticeable for interfaces between RAN nodes such as Xn interface.

Observation 1:	Current protocol design for C-plane RAN-CN interface (i.e. SCTP) has following benefits:

- Integrated heart-beat function to monitor the connection health

- Flexible security options depending on deployments

- Simple architecture and procedures due to single endpoint of RAN-CN connection

Observation 2:	Current protocol design for C-plane RAN-CN interface (i.e. SCTP) has following issues:

- Not virtualization/cloud friendly due to end-to-end connection

- Load concentration on AMF

Analysis of potential solution for C-plane protocol

Among the existing protocol designs mentioned in the previous chapter, SBI can be a candidate solution, especially considering the compatibility for virtualization. Direct connectivity to NF other than RAN node and AMF can also be considered. This may avoid the architecture in which AMF is a single endpoint and separate the load of AMF from the new services that require signaling between RAN and NF.

On the other hand, possible drawbacks brought by the new design of transport protocol should be studied carefully at this stage. From operator perspective, user experience has first priority on option selection, therefore even if it contributes CAPEX/OPEX reduction, we cannot employ it without knowing the drawbacks would be experienced by customers.

Comparing to the existing protocol design, when a certain NW node is attacked, there are more NW nodes that can be accessed from that node. From a transport network point of view, it may be possible to access other nodes even in legacy, but from a security point of view, it is necessary to coordinate with SA3 when the protocol in the 3GPP specification allows access to more NW nodes. In addition, when considering the integration with the SBI chain of CN, it may be difficult to simply connect all RAN nodes directly to the SBI chain of CN because the existing 5GC typically assumes the connection of ~100 order NFs. Especially, the increased number of NFs managed by NRF and the load balancing of the transport network would be issues. Those issues need to be addressed in the study. Finally, from the learning in 5GC, the introduction of SBI may cause an increase of NFs and the complexity of procedures. Longer procedures may also potentially lead to slower signaling, lower performance, or IOT issues.

When it comes to the protocol stack, there are several options for adopting SBI. One of them is QUIC + HTTP/3 [5], which is one of the most modern and cloud-friendly protocols and might be a good candidate solution for virtualization. However, there is a concern about the use of QUIC [6], regardless of the use of HTTP/3 (i.e.SBI or not). Unlike SCTP, QUIC integrates TLS 1.3 and has the advantage of supporting multi-streams without restriction by endpoint IP addresses. However, this integration, i.e. mandating security protection might reduce the flexibility of the security protection option for operators. According to the existing specification [4], the use of security protection (DTLS and/or IPsec) is up to operator decision, and in secure environments, it may not be used. In some deployments, based on operator’s security policy and/or legal regulation, IPsec is preferred because it allows us to place security endpoint, i.e. SecGW, in a secure place under the operator’s control, and security protection by DTLS may not be needed. In that sense IPsec has an advantage over DTLS, and whether operators should have flexibility on this point needs to be discussed.

Regarding message encoding, we think that the existing ASN.1 should be a baseline even when introducing SBI. Text format is used in 5GC, but at the same time, we experience increased message size, and essentially, increasing message size increases load on the transport network and can incur additional infrastructure costs. Changing the encoding to the more readable text format should consider the benefits, as well as increased load on the transport network and in performance.

Observation 3:	SBI for C-plane RAN-CN interface transport protocol (e.g. QUIC + HTTP/3) has following potential benefits:

- Optimal for RAN virtualization

- Allow RAN nodes connection to other NFs in CN than AMF

- Flexible/dynamic TNL endpoint handling contributes enhanced feeder link switch over for NTN or resiliency functionalities, etc.

Observation 4:	SBI for C-plane RAN-CN interface transport protocol has following potential issues:

- Security threats due to more flexible connection between RAN nodes and NFs

- Some issues on integration with SBI chain in CN

- More complex architecture and longer procedures

Observation 5:	QUIC for C-plane RAN-CN interface transport protocol has following potential issue:

- No flexibility on the use of security protection even in case the operator can make it sure that the RAN nodes and transport network are deployed in secure environment

Proposal 1:	In study of the new transport protocol for C-plane RAN-CN interface in 6G, following perspectives need to be addressed:

- Performance: procedure complexity should be reasonable to avoid slower C-plane signaling and degraded user experience.

- Security risks: flexible connectivity among NW entities may increase security risk; security implications should be assessed upon coordination with SA3.

- Architecture option: direct RAN-to-NF connectivity (beyond AMF) should be evaluated to avoid AMF single-endpoint and offload new-service signaling.

- SBI/NRF scaling: feasibility of connecting many RAN nodes to CN SBI chain and the scalability impact on NRF (discovery/registration load, stability) should be considered.

- Transport impact: load balancing and capacity impact on transport network should be evaluated for increased endpoints/flows.

- Operator security flexibility: impact of QUIC mandating TLS 1.3 should be assessed against existing operator options (DTLS/IPsec, SecGW placement).

- Message encoding: ASN.1 should be baseline; any move to text encoding should justify added message size and transport network burden.

- Interoperability risk: the complexity of the procedure and the connectivity between the RAN nodes and more NFs should be considered for a realistic IOTD effort.

Conclusions and proposals

Our proposals are summarized below.

Observation 1:	Current protocol design for C-plane RAN-CN interface (i.e. SCTP) has following benefits:

- Integrated heart-beat function to monitor the connection health

- Flexible security options depending on deployments

- Simple architecture and procedures due to single endpoint of RAN-CN connection

Observation 2:	Current protocol design for C-plane RAN-CN interface (i.e. SCTP) has following issues:

- Not virtualization/cloud friendly due to end-to-end connection

- Load concentration on AMF

Observation 3:	SBI for C-plane RAN-CN interface transport protocol (e.g. QUIC + HTTP/3) has following potential benefits:

- Optimal for RAN virtualization

- Allow RAN nodes connection to other NFs in CN than AMF

- Flexible/dynamic TNL endpoint handling contributes enhanced feeder link switch over for NTN or resiliency functionalities, etc.

Observation 4:	SBI for C-plane RAN-CN interface transport protocol has following potential issues:

- Security threats due to more flexible connection between RAN nodes and NFs

- Some issues on integration with SBI chain in CN

- More complex architecture and longer procedures

Observation 5:	QUIC for C-plane RAN-CN interface transport protocol has following potential issue:

- No flexibility on the use of security protection even in case the operator can make it sure that the RAN nodes and transport network are deployed in secure environment

Proposal 1:	In study of the new transport protocol for C-plane RAN-CN interface in 6G, following perspectives need to be addressed:

- Performance: procedure complexity should be reasonable to avoid slower C-plane signaling and degraded user experience.

- Security risks: flexible connectivity among NW entities may increase security risk; security implications should be assessed upon coordination with SA3.

- Architecture option: direct RAN-to-NF connectivity (beyond AMF) should be evaluated to avoid AMF single-endpoint and offload new-service signaling.

- SBI/NRF scaling: feasibility of connecting many RAN nodes to CN SBI chain and the scalability impact on NRF (discovery/registration load, stability) should be considered.

- Transport impact: load balancing and capacity impact on transport network should be evaluated for increased endpoints/flows.

- Operator security flexibility: impact of QUIC mandating TLS 1.3 should be assessed against existing operator options (DTLS/IPsec, SecGW placement).

- Message encoding: ASN.1 should be baseline; any move to text encoding should justify added message size and transport network burden.

- Interoperability risk: the complexity of the procedure and the connectivity between the RAN nodes and more NFs should be considered for a realistic IOTD effort.

References

- R3-258859, “pCR to TR 38.760-3 on 6G RAN-CN interface options”, ZTE

- RP-253876, “Revised SID: Study on 6G Radio”, NTT Docomo.

- TS 38.412, “NG-RAN; NG signalling transport”

- TS 33.501, “Security architecture and procedures for 5G system”

- RFC 9114, “HTTP/3”

- RFC 9000, “QUIC: A UDP-Based Multiplexed and Secure Transport”

Annex: pCR for TR 38.760-3

### 6.1.4	Evaluation of RAN-CN Interface Options

This chapter includes evaluation and comparison of RAN-CN interface options described in clause 6.1.3.1

The following perspectives should be considered upon evaluation and comparison of RAN-CN interface options:

-	Performance: procedure complexity should be reasonable to avoid slower C-plane signaling and degraded user experience.

-	Security risks: flexible connectivity among NW entities may increase security risk; security implications should be assessed based on coordination with SA3.

-	Architecture option: direct RAN-to-NF connectivity (beyond AMF) should be evaluated to avoid AMF single-endpoint and offload new-service signaling.

-	SBI/NRF scaling: feasibility of connecting many RAN nodes to CN SBI chain and the scalability impact on NRF (discovery/registration load, stability) should be considered.

-	Transport impact: load balancing and capacity impact on transport network should be evaluated for increased endpoints/flows.

-	Operator security flexibility: impact of QUIC mandating TLS 1.3 should be assessed against existing operator options (DTLS/IPsec, SecGW placement).

-	Message encoding: ASN.1 should be baseline; any move to text encoding should justify added message size and transport network burden.

-	Interoperability risk: the complexity of the procedure and the connectivity between the RAN nodes and more NFs should be considered for a realistic IOTD effort.