3GPP TSG-RAN3 Meeting #18 	Tdoc R3-010207

Lidingo, Sweden, 15th-19th January, 2001

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25.433 |  |  |  | CR | 328 |  |  |  rev |  |  |  | 1 |  |  Current version: |  |  |  |  | 3.4.1 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  | X |  | Core Network |  |  |
| Title:  |  | AUDIT RESPONSE tabular format modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | January 2001 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | R99 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | In AUDIT RESPONSE the range table is incomplete, more over the AICH Information should not have its range bound to the number of RACHs, but to the number of PRACHs instead. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The range table is made complete and the range of AICH information is changed to go up to “MaxPRACHCell” This change is backwards compatible. R1: category was changed from D to F. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | The NBAP specification will have an incomplete tabular format and will contain a logical error with regards to the AICH information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 9.1.17, 9.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

### 9.1.17	AUDIT RESPONSE

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Discriminator | M |  | 9.2.1.45 |  | – |  |
| Message Type | M |  | 9.2.1.46 |  | YES | reject |
| Transaction ID | M |  | 9.2.1.62 |  | – |  |
| End Of Audit Sequence Indicator | M |  | 9.2.1.29A |  | YES | ignore |
| Cell Information |  | 0.. <maxCellinNodeB > |  |  | EACH | ignore |
| >C-ID | M |  | 9.2.1.9 |  | – |  |
| >Configuration Generation ID | M |  | 9.2.1.16 |  | – |  |
| >Resource Operational State | M |  | 9.2.1.52 |  | – |  |
| >Availability Status | M |  | 9.2.1.2 |  | – |  |
| >Local Cell ID | M |  | 9.2.1.38 | The local cell that the cell is configured on | – |  |
| >Primary SCH Information | O |  | Common Physical Channel Status Information 9.2.1.13A |  | YES | ignore |
| >Secondary SCH Information | O |  | Common Physical Channel Status Information 9.2.1.13A |  | YES | ignore |
| >Primary CPICH Information | O |  | Common Physical Channel Status Information 9.2.1.13A |  | YES | ignore |
| >Secondary CPICH Information |  | 0..<maxSCPICHCell> |  |  | EACH | ignore |
| >>Secondary CPICH Individual Information | M |  | Common Physical Channel Status Information 9.2.1.13A |  | – |  |
| >Primary CCPCH Information | O |  | Common Physical Channel Status Information 9.2.1.13A |  | YES | ignore |
| >BCH Information | O |  | Common Transport Channel Status Information 9.2.1.13A |  | YES | ignore |
| >Secondary CCPCH Information |  | 0..<maxSCCPCHCell> |  |  | EACH | ignore |
| >>Secondary CCPCH Individual Information | M |  | Common Physical Channel Status Information 9.2.1.13A |  | – |  |
| >PCH Information | O |  | Common Transport Channel Status Information 9.2.1.14B |  | YES | ignore |
| >PICH Information | O |  | Common Physical Channel Status Information 9.2.1.13A |  | YES | ignore |
| >FACH Information |  | 0..<maxFACHCell> |  |  | EACH | ignore |
| >>FACH Individual Information | M |  | Common Transport Channel Status Information 9.2.1.14B |  | – |  |
| >PRACH Information |  | 0..<maxPRACHCell> |  |  | EACH | ignore |
| >>PRACH Individual Information | M |  | Common Physical Channel Status Information 9.2.1.13A |  | – |  |
| >RACH Information |  | 0..<maxRACHCell> |  |  | EACH | ignore |
| >>RACH Individual Information | M |  | Common Transport Channel Status Information 9.2.1.14B |  | – |  |
| >AICH Information |  | 0..<maxPRACHCell> |  |  | EACH | ignore |
| >>AICH Individual Information | M |  | Common Physical Channel Status Information 9.2.1.13A |  | – |  |
| >PCPCH Information |  | 0..<maxPCPCHCell> |  |  | EACH | ignore |
| >>PCPCH Individual Information | M |  | Common Physical Channel Status Information 9.2.1.13A |  | – |  |
| >CPCH Information |  | 0..<maxCPCHCell> |  |  | EACH | ignore |
| >>CPCH Individual Information | M |  | Common Transport Channel Status Information 9.2.1.14B |  | – |  |
| >AP-AICH Information |  | 0..<maxCPCHCell> |  |  | EACH | ignore |
| >>AP-AICH Individual Information | M |  | Common Physical Channel Status Information 9.2.1.13A |  | – |  |
| >CD/CA-ICH Information |  | 0..<maxCPCHCell> |  |  | EACH | ignore |
| >>CD/CA-ICH Individual Information | M |  | Common Physical Channel Status Information 9.2.1.13A |  | – |  |
| >SCH Information | O |  | Common Physical Channel Status Information 9.2.1.13A | TDD Sync Channel | YES | ignore |
| Communication Control Port Information |  | 0.. <maxCCPinNodeB> |  |  | EACH | ignore |
| >Communication Control Port ID | M |  | 9.2.1.15 |  | – |  |
| >Resource Operational State | M |  | 9.2.1.52 |  | – |  |
| >Availability Status | M |  | 9.2.1.2 |  | – |  |
| Local Cell Information |  | 0.. <maxLocalCellinNodeB> |  |  | EACH | ignore |
| >Local Cell ID | M |  | 9.2.1.38 |  | – |  |
| >DL or Global Capacity Credit | M |  | 9.2.1.20B |  | – |  |
| >UL Capacity Credit | O |  | 9.2.1.65A |  | – |  |
| >Common Channels Capacity Consumption Law | M |  | 9.2.1.9A |  | – |  |
| >Dedicated Channels Capacity Consumption Law | M |  | 9.2.1.20A |  | – |  |
| >Maximum DL Power Capability | O |  | 9.2.1.39 |  | – |  |
| >Minimum Spreading Factor | O |  | 9.2.1.47 |  | – |  |
| >Minimum DL Power Capability | O |  | 9.2.1.46A |  | – |  |
| >Local Cell Group ID | O |  | 9.2.1.37A |  | – |  |
| Local Cell Group Information |  | 0.. <maxLocalCellinNodeB> |  |  | EACH | ignore |
| >Local Cell Group ID | M |  | 9.2.1.37A |  | – |  |
| >DL or Global Capacity Credit | M |  | 9.2.1.20B |  | – |  |
| >UL Capacity Credit | O |  | 9.2.1.65A |  | – |  |
| >Common Channels Capacity Consumption Law | M |  | 9.2.1.9A |  | – |  |
| >Dedicated Channels Capacity Consumption Law | M |  | 9.2.1.20A |  | – |  |
| Criticality Diagnostics | O |  | 9.2.1.17 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| MaxCellinNodeB | Maximum number of Cell that can be configured in Node B |
| MaxCCPinNodeB | Maximum number of communication control ports that can exist in the Node B |
| MaxCPCHCell | Maximum number of CPCHes that can be defined in a Cell |
| MaxLocalCellinNodeB | Maximum number of Local Cells that can exist in the Node B |
| MaxPCPCHCell | Maximum number of PCPCHes that can be defined in a Cell |
| MaxSCPICHCell | Maximum number of Secondary CPICH that can be defined in a Cell. |
| MaxSCCPCHCell | Maximum number of Secondary CCPCH that can be defined in a Cell. |
| MaxFACHCell | Maximum number of FACHes that can be defined in a Cell |
| MaxPRACHCell | Maximum number of PRACHes that can be defined in a Cell |
| MaxRACHCell | Maximum number of RACHes that can be defined in a Cell |

### PDU Definitions

****** UNCHANGED PARTS ARE SKIPPED******

-- **************************************************************

--

-- AUDIT RESPONSE

--

-- **************************************************************

AuditResponse ::= SEQUENCE {

protocolIEs					ProtocolIE-Container	{{AuditResponse-IEs}},

protocolExtensions			ProtocolExtensionContainer	{{AuditResponse-Extensions}}		OPTIONAL,

...

}

AuditResponse-IEs NBAP-PROTOCOL-IES ::= {

{ ID	id-End-Of-Audit-Sequence-Indicator			CRITICALITY		ignore	TYPE 	End-Of-Audit-Sequence-Indicator	 	PRESENCE mandatory }|

{ ID	id-Cell-InformationList-AuditRsp			CRITICALITY		ignore			TYPE	Cell-InformationList-AuditRsp			PRESENCE	optional	}|

{ ID	id-CCP-InformationList-AuditRsp				CRITICALITY		ignore			TYPE	CCP-InformationList-AuditRsp			PRESENCE	optional	}|

-- CCP (Communication Control Port) --

{ ID	id-Local-Cell-InformationList-AuditRsp		CRITICALITY		ignore			TYPE	Local-Cell-InformationList-AuditRsp		PRESENCE	optional	}|

{ ID	id-Local-Cell-Group-InformationList-AuditRsp	CRITICALITY		ignore		TYPE	Local-Cell-Group-InformationList-AuditRsp	PRESENCE	optional	}|

{ ID	id-CriticalityDiagnostics					CRITICALITY		ignore			TYPE	CriticalityDiagnostics					PRESENCE	optional	},

...

}

AuditResponse-Extensions NBAP-PROTOCOL-EXTENSION ::= {

...

}

Cell-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxCellinNodeB)) OF ProtocolIE-Single-Container {{ Cell-InformationItemIE-AuditRsp}}

Cell-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID	id-Cell-InformationItem-AuditRsp		CRITICALITY		ignore		TYPE	Cell-InformationItem-AuditRsp			PRESENCE 	optional	}

}

Cell-InformationItem-AuditRsp ::= SEQUENCE {

c-ID									C-ID,

configurationGenerationID				ConfigurationGenerationID,

resourceOperationalState				ResourceOperationalState,

availabilityStatus						AvailabilityStatus,

local-Cell-ID							Local-Cell-ID,

primary-SCH-Information					P-SCH-Information-AuditRsp					OPTIONAL,

secondary-SCH-Information				S-SCH-Information-AuditRsp					OPTIONAL,

primary-CPICH-Information				P-CPICH-Information-AuditRsp				OPTIONAL,

secondary-CPICH-InformationList			S-CPICH-InformationList-AuditRsp			OPTIONAL,

primary-CCPCH-Information				P-CCPCH-Information-AuditRsp				OPTIONAL,

bCH-Information							BCH-Information-AuditRsp					OPTIONAL,

secondary-CCPCH-InformationList			S-CCPCH-InformationList-AuditRsp			OPTIONAL,

pCH-Information							PCH-Information-AuditRsp					OPTIONAL,

pICH-Information						PICH-Information-AuditRsp					OPTIONAL,

fACH-InformationList					FACH-InformationList-AuditRsp				OPTIONAL,

pRACH-InformationList					PRACH-InformationList-AuditRsp				OPTIONAL,

rACH-InformationList					RACH-InformationList-AuditRsp				OPTIONAL,

aICH-InformationList					AICH-InformationList-AuditRsp				OPTIONAL,

pCPCH-InformationList					PCPCH-InformationList-AuditRsp				OPTIONAL,

cPCH-InformationList					CPCH-InformationList-AuditRsp				OPTIONAL,

aP-AICH-InformationList					AP-AICH-InformationList-AuditRsp			OPTIONAL,

cDCA-ICH-InformationList				CDCA-ICH-InformationList-AuditRsp			OPTIONAL,

sCH-Information							SCH-Information-AuditRsp					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer  { { Cell-InformationItem-AuditRsp-ExtIEs} }		OPTIONAL,

...

}

Cell-InformationItem-AuditRsp-ExtIEs NBAP-PROTOCOL-EXTENSION ::= {

...

}

P-SCH-Information-AuditRsp ::= ProtocolIE-Single-Container {{ P-SCH-InformationIE-AuditRsp }}

P-SCH-InformationIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-P-SCH-Information	CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information 	PRESENCE	mandatory }

}

S-SCH-Information-AuditRsp ::= ProtocolIE-Single-Container {{ S-SCH-InformationIE-AuditRsp }}

S-SCH-InformationIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-S-SCH-Information	CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information 	PRESENCE	mandatory }

}

P-CPICH-Information-AuditRsp ::= ProtocolIE-Single-Container {{ P-CPICH-InformationIE-AuditRsp }}

P-CPICH-InformationIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-P-CPICH-Information	CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information 	PRESENCE	mandatory }

}

S-CPICH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxSCPICHCell)) OF ProtocolIE-Single-Container {{ S-CPICH-InformationItemIE-AuditRsp }}

S-CPICH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-S-CPICH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE mandatory }

}

P-CCPCH-Information-AuditRsp ::= ProtocolIE-Single-Container {{ P-CCPCH-InformationIE-AuditRsp }}

P-CCPCH-InformationIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-P-CCPCH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE mandatory }

}

BCH-Information-AuditRsp ::= ProtocolIE-Single-Container {{ BCH-InformationIE-AuditRsp }}

BCH-InformationIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-BCH-Information   CRITICALITY ignore	TYPE Common-TransportChannel-Status-Information		PRESENCE mandatory }

}

S-CCPCH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxSCCPCHCell)) OF ProtocolIE-Single-Container {{ S-CCPCH-InformationItemIE-AuditRsp }}

S-CCPCH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-S-CCPCH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE mandatory }

}

PCH-Information-AuditRsp ::= ProtocolIE-Single-Container {{ PCH-InformationIE-AuditRsp }}

PCH-InformationIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-PCH-Information   CRITICALITY ignore	TYPE Common-TransportChannel-Status-Information		PRESENCE mandatory }

}

PICH-Information-AuditRsp ::= ProtocolIE-Single-Container {{ PICH-InformationIE-AuditRsp }}

PICH-InformationIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-PICH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE mandatory }

}

FACH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxFACHCell)) OF ProtocolIE-Single-Container {{ FACH-InformationItemIE-AuditRsp }}

FACH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-FACH-Information   CRITICALITY ignore	TYPE Common-TransportChannel-Status-Information		PRESENCE mandatory }

}

PRACH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxPRACHCell)) OF ProtocolIE-Single-Container {{ PRACH-InformationItemIE-AuditRsp }}

PRACH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-PRACH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE mandatory }

}

RACH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxRACHCell)) OF ProtocolIE-Single-Container {{ RACH-InformationItemIE-AuditRsp }}

RACH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-RACH-Information   CRITICALITY ignore	TYPE Common-TransportChannel-Status-Information		PRESENCE mandatory }

}

AICH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxPRACHCell)) OF ProtocolIE-Single-Container {{ AICH-InformationItemIE-AuditRsp }}

AICH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-AICH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE mandatory }

}

PCPCH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxPCPCHCell)) OF ProtocolIE-Single-Container {{ PCPCH-InformationItemIE-AuditRsp }}

PCPCH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-PCPCH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE optional }

}

CPCH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxCPCHCell)) OF ProtocolIE-Single-Container {{ CPCH-InformationItemIE-AuditRsp }}

CPCH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-CPCH-Information   CRITICALITY ignore	TYPE Common-TransportChannel-Status-Information		PRESENCE optional }

}

AP-AICH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxCPCHCell)) OF ProtocolIE-Single-Container {{ AP-AICH-InformationItemIE-AuditRsp }}

AP-AICH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-AP-AICH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE mandatory }

}

CDCA-ICH-InformationList-AuditRsp ::= SEQUENCE (SIZE (1..maxCPCHCell)) OF ProtocolIE-Single-Container {{ CDCA-ICH-InformationItemIE-AuditRsp }}

CDCA-ICH-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-CDCA-ICH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE mandatory }

}

SCH-Information-AuditRsp ::= ProtocolIE-Single-Container {{ SCH-InformationIE-AuditRsp }}

SCH-InformationIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID id-SCH-Information   CRITICALITY ignore	TYPE Common-PhysicalChannel-Status-Information		PRESENCE mandatory }

}

CCP-InformationList-AuditRsp ::=SEQUENCE (SIZE (1..maxCCPinNodeB)) OF ProtocolIE-Single-Container {{ CCP-InformationItemIE-AuditRsp }}

CCP-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ID	id-CCP-InformationItem-AuditRsp			CRITICALITY		ignore			TYPE	CCP-InformationItem-AuditRsp			PRESENCE	mandatory}

}

CCP-InformationItem-AuditRsp ::= SEQUENCE {

communicationControlPortID			CommunicationControlPortID,

resourceOperationalState			ResourceOperationalState,

availabilityStatus					AvailabilityStatus,

iE-Extensions						ProtocolExtensionContainer  {{ CCP-InformationItem-AuditRsp-ExtIEs }}		OPTIONAL,

...

}

CCP-InformationItem-AuditRsp-ExtIEs NBAP-PROTOCOL-EXTENSION ::= {

...

}

Local-Cell-InformationList-AuditRsp	 ::=SEQUENCE (SIZE (1..maxLocalCellinNodeB)) OF ProtocolIE-Single-Container {{ Local-Cell-InformationItemIE-AuditRsp }}

Local-Cell-InformationItemIE-AuditRsp NBAP-PROTOCOL-IES ::= {

{ ID	id-Local-Cell-InformationItem-AuditRsp			CRITICALITY		ignore			TYPE	Local-Cell-InformationItem-AuditRsp		PRESENCE	mandatory}

}

Local-Cell-InformationItem-AuditRsp ::= SEQUENCE {

local-Cell-ID								Local-Cell-ID,

dl-or-global-capacityCredit					DL-or-Global-CapacityCredit,

ul-capacityCredit							UL-CapacityCredit		OPTIONAL,

commnonChannelsCapacityConsumptionLaw 		CommonChannelsCapacityConsumptionLaw,

dedicatedChannelsCapacityConsumptionLaw 	DedicatedChannelsCapacityConsumptionLaw,

maximumDL-PowerCapability					MaximumDL-PowerCapability		OPTIONAL,

minSpreadingFactor							MinSpreadingFactor				OPTIONAL,

minimumDL-PowerCapability					MinimumDL-PowerCapability		OPTIONAL,

local-Cell-Group-ID							Local-Cell-ID					OPTIONAL,

iE-Extensions								ProtocolExtensionContainer  {{ Local-Cell-InformationItem-AuditRsp-ExtIEs}}		OPTIONAL,

...

}