3GPP TSG-RAN WG3 #97-bis	R3-173499

Prague, Czechia, EU, 9 - 13 August 2017

Agenda item:	10.8.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	S-RLF handling for MR-DC

Document for:	Decision

# 1	Introduction

During the RAN2 NR AdHoc meeting in Qingdao and the following RAN2 meeting #99, it was decided the SCG RLF report received from the UE shall be forwarded to the SgNB. This is further analysed in [1].

In this document, we are providing the stage-3 TP for XnAP needed for the solution.

# References

- R3-173497, RAN3 #97-bis

# Text proposal for baseline CR for TS 38.423

First change, ommited text not changed

### 8.X.Y	S-RLF Indication

#### 8.X.Y.1	General

The purpose of the S-RLF Indication procedure is to report an SCG RLF to the S-NG-RAN-NODE and to provide the NR measurements included in the UE report.

The procedure uses UE-associated signalling.

#### 8.X.Y.2	Successful Operation

Figure 8.X.Y.2-1: S-RLF Indication procedure, successful operation.

The M-NG-RAN-NODE initiates the procedure by sending the S-RLF INDICATION message to the S-NG-RAN-NODE.

#### 8.X.Y.3	Abnormal Conditions

None.

Next change, ommited text not changed

#### 9.1.Z.1	S-RLF INDICATION

This message is sent by the M-NG-RAN-NODE to the S-NG-RAN-NODE to transfer an SCG RLF report.

Direction: M-NG-RAN-NODE  S-NG-RAN-NODE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| M-NG-RAN node UE X2AP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE X2AP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| RRC Container | M |  | OCTET STRING | NR measurements received from the UE | YES | reject |

Remaining text not changed