3GPP TSG RAN WG3 NR AdHoc 1801	Tdoc R3-180596

Sophia Antipolis, France, 22 - 26 January 2018

Agenda Item:	10.10.7

Source:	Ericsson

Title:	UE Reject Indication

Document for:	approval (TP TS 38.473)

# Introduction

At the previous RAN3 meeting, there was a discussion on how the gNB-CU and gNB-DU can exchange information in case the UE is not admitted during the initial access procedure. In this contribution, we propose some enhancements to such procedures and to introduce a new UE Reject Indication IE in the DL RRC Message Transfer for this purpose.

# Annex 1: TP for the BL CR to TS 38.473

Start of Text Proposal 1 for TS 38.473

### 8.4.2 	DL RRC Message Transfer

#### 8.4.2.1	General

The purpose of the DL RRC Message Transfer procedure is to transfer an RRC message as a DL PDCP-PDU to the gNB-DU.

#### 8.4.2.2	Successful operation

Figure 8.4.2.2-1: DL RRC Message Transfer procedure.

If a UE-associated logical F1-connection exists, the DL RRC MESSAGE TRANSFER message shall contain the gNB-DU UE F1AP ID IE, which should be used by gNB-DU to lookup the stored UE context.

The DL RRC MESSAGE TRANSFER message shall include, if available, the old gNB-DU UE F1AP ID IE so that the gNB-DU can retrieve the existing UE context in RRC connection reestablishment procedure.

If the UE Reject Indication IE is included in the DL RRC MESSAGE TRANSFER, the gNB-DU shall understand that the gNB-CU has rejected the UE.

#### 8.4.2.3	Abnormal Conditions

Not applicable.

Start of Text Proposal 2 for TS 38473

#### 9.2.3.2	DL RRC MESSAGE TRANSFER

This message is sent by the gNB-CU to transfer the layer 3 message to the gNB-DU over the F1 interface.

Direction: gNB-CU gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| old gNB-DU UE F1AP ID | O |  | 9.3.1.5 | Include it if RRCConnectionReestablishment is included in RRC-Container | YES | reject |
| SRB ID | M |  | 9.3.1.7 |  | YES | reject |
| RRC-Container | M |  | 9.3.1.6 |  | YES | reject |
| UE Reject Indication | O |  | 9.3.1.x |  | YES | reject |

Start of Text Proposal 3 for TS 38.473

#### 9.3.1.x 	UE Reject Indication

The UE Reject indication is used to inform the gNB-DU that gNB-CU has rejected the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Indication | M | ENUMERATED(Rejected, …) |  |  |
| Cause | M |  | 9.3.1.2 |  |

End Text Proposal for TS 38.473