| 3GPP TS 36.133 V19.3.0 (2025-12) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	107

1	Scope	109

2	References	109

3	Definitions, symbols and abbreviations	111

3.1	Definitions	111

3.2	Symbols	113

3.3	Abbreviations	114

3.4	Test tolerances	117

3.5	Additional notation	117

3.5.1	Groups of bands	117

3.5.1A	Groups of bands for satellite access	120

3.6	General	121

3.6.1	Applicability of requirements in this specification version	121

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	127

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	129

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	130

3.6.2	Applicability of requirements for EN-DC operation	131

3.6.3	Applicability of requirements for NE-DC operation	132

3.6.4	Applicability of requirements for NGEN-DC operation	133

3.6.5	Applicability of 2-step RA and 4-step RA in RRM requirements	133

3.6.6 Applicability of requirements for UE category NB-IoT for frame structure type 1 for NTN-TDD	133

4	E-UTRAN RRC_IDLE state mobility	133

4.1	Cell Selection	133

4.2	Cell Re-selection	133

4.2.1	Introduction	133

4.2.2	Requirements	134

4.2.2.1	Measurement and evaluation of serving cell	135

4.2.2.2	Void	136

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	136

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	138

4.2.2.5	Measurements of inter-RAT cells	140

4.2.2.5.1	Measurements of UTRAN FDD cells	140

4.2.2.5.2	Measurements of UTRAN TDD cells	142

4.2.2.5.3	Measurements of GSM cells	143

4.2.2.5.4	Measurements of HRPD cells	144

4.2.2.5.5	Measurements of cdma2000 1X	145

4.2.2.5.6	Measurements of NR cells	146

4.2.2.5.7	Measurements of NR cells subject to CCA	149

4.2.2.5.8	Measurements of NR cells for RedCap	151

4.2.2.6	Evaluation of cell re-selection criteria	153

4.2.2.7	Maximum interruption in paging reception	153

4.2.2.8	void	154

4.2.2.9	UE measurement capability	154

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	155

4.2.2.10	Reselection to CSG cells	155

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	156

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	156

4.2.2.11	Void	157

4.2.2.12	Void	157

4.2.2.13	Void	157

4.2.2.14	UE measurement capability for RedCap	157

4.3	Minimization of Drive Tests (MDT)	158

4.3.1	Introduction	158

4.3.2	Measurements	158

4.3.2.1	Requirements	158

4.3.3	Relative Time Stamp Accuracy	158

4.3.3.1	Requirements	158

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	159

4.3.4.1	 Requirements	159

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	159

4.3.5.1	Requirements for timeSinceFailure	159

4.4	MBSFN Measurements	159

4.4.1	Introduction	159

4.4.2	MBSFN RSRP measurements	159

4.4.3	MBSFN RSRQ measurements	160

4.4.4	MCH BLER measurements	160

4.5	Proximity-based Services	160

4.5.1	Introduction	160

4.5.2	Requirements	160

4.5.2.1	Interruptions with ProSe Direct Discovery	160

4.5.2.2	Interruptions with ProSe Direct Communication	160

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	161

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	161

4.6	Cell Selection and Re-selection Requirements for UE category NB1	162

4.6.1	Cell Selection	162

4.6.2	Cell Re-selection	162

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	162

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	163

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	164

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	165

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	167

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	168

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	169

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	170

4.6.2.7	Maximum interruption in paging reception in normal coverage	172

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	172

4.6.2.8	UE measurement capability	172

4.6.2.9	WUS receptions for NB1	172

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	173

4.6.3.1	Introduction	173

4.6.3.2	Requirements on UE synchronization for transmission using PUR	173

4.6.3.3	Requirements on TA validation for transmission using PUR	173

4.6A	Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access	174

4.6A.1	Cell Selection	174

4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access	174

4.6A.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	175

4.6A.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	176

4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	177

4.6A.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	179

4.6A.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	181

4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	182

4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	184

4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	186

4.6A.2.7	Maximum interruption in paging reception in normal coverage	188

4.6A.2.7A	Maximum interruption in paging reception in enhanced coverage	188

4.6A.2.8	UE measurement capability	189

4.6A.2.9	WUS receptions for NB1	189

4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access	190

4.6A.3.1	Introduction	190

4.6A.3.2	Requirements on UE synchronization for transmission using PUR	190

4.6A.3.3	Requirements on TA validation for transmission using PUR	190

4.6B	Cell Selection and Re-selection Requirements for UE category NB-IoT for frame structure type 1 for NTN-TDD	190

4.6B.1	Cell Selection	190

4.6B.2	Cell Re-selection for UE category NB-IoT for Satellite Access	190

4.6B.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	191

4.6B.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	192

4.6B.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	193

4.6B.2.7	Maximum interruption in paging reception in normal coverage	194

4.6B.2.8	UE measurement capability	194

4.7	Cell Selection and Re-selection Requirements for UE category M1	195

4.7.1	Cell Selection	195

4.7.2	Cell Re-selection	195

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	195

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	195

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	196

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	197

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	198

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	200

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	200

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	201

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	201

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	202

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	203

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	206

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	207

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	208

4.7.2.3	WUS receptions for UE category M1	208

4.7.3	Channel quality report for UE Category M1 in idle mode	208

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	209

4.7.4.1	Introduction	209

4.7.4.2	Requirements on UE synchronization for transmission using PUR	209

4.7.4.3	Requirements on TA validation for transmission using PUR	209

4.7A	Cell Selection and Re-selection Requirements for UE category M1 for Satellite Access	210

4.7A.1	Cell Selection	210

4.7A.2	Cell Re-selection for UE category M1 for Satellite Access	210

4.7A.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	211

4.7A.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	211

4.7A.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	213

4.7A.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	214

4.7A.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	217

4.7A.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	219

4.7A.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	219

4.7A.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	220

4.7A.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	220

4.7A.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	221

4.7A.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	223

4.7A.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	225

4.7A.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	228

4.7A.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	228

4.7A.2.3	WUS receptions for UE category M1	229

4.7A.3	Channel quality report for UE Category M1 in idle mode for Satellite Access	229

4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access	230

4.7A.4.1	Introduction	230

4.7A.4.2	Requirements on UE synchronization for transmission using PUR	230

4.7A.4.3	Requirements on TA validation for transmission using PUR	230

4.8	Idle State Positioning Measurement Requirements for UE category NB1	230

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	230

4.8.1.1	RSTD Measurement Reporting Delay	232

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	232

4.8.2.1	RSTD Measurement Reporting Delay	233

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	234

4.8.3.1	RSTD Measurement Reporting Delay	235

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	236

4.8.4.1	RSTD Measurement Reporting Delay	237

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	238

4.8.5.1	Measurement Reporting Delay	239

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	239

4.8.6.1	Measurement Reporting Delay	240

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	241

4.8.7.1	Measurement Reporting Delay	242

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	242

4.8.8.1	Measurement Reporting Delay	243

4.9	Idle Mode CA Measurement	244

4.9.1	Introduction	244

4.9.2	Requirement	244

4.9.2.1	Detected cell requirement during state transition and Idle mode	244

4.9.2.2	Measurements of inter-frequency CA candidate cells	245

4.9.2.3	Measurements on serving cell	245

4A	E-UTRAN RRC_INACTIVE state mobility	246

4A.1	Cell Re-selection	246

4A.1.1	Introduction	247

4A.1.2	Requirements	247

4A.1.2.1	UE measurement capability	247

4A.1.2.2	Measurement and evaluation of serving cell	247

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	247

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	247

4A.1.2.5	Evaluation of cell re-selection criteria	247

4A.1.2.6	Maximum interruption in paging reception	247

4A.1.2.7	Measurements of inter-RAT NR cells	247

4A.1.2.8	UE measurement capability for RedCap	247

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	247

4A.2	Requirements for UE Category M1	248

4A.2.1	Introduction	248

4A.2.2	Cell Selection	248

4A.2.3	Cell Reselection	248

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	248

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	248

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	249

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	249

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	249

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	249

4A.2.4	Channel quality report for UE Category M1 in idle mode	251

5	E-UTRAN RRC_CONNECTED state mobility	251

5.1	E-UTRAN Handover	252

5.1.1	Introduction	252

5.1.2	Requirements	252

5.1.2.1	E-UTRAN FDD – FDD	252

5.1.2.1.1	Handover delay	252

5.1.2.1.2	Interruption time	252

5.1.2.2	E-UTRAN FDD – TDD	253

5.1.2.2.1	(Void)	254

5.1.2.2.2	(Void)	254

5.1.2.3	E-UTRAN TDD – FDD	254

5.1.2.3.1	(Void)	254

5.1.2.3.2	(Void)	254

5.1.2.4	E-UTRAN TDD – TDD	254

5.1.2.4.1	Handover delay	254

5.1.2.4.2	Interruption time	254

5.1.2.5	E-UTRAN HD–FDD	256

5.1.2.5.1	Handover delay	256

5.1.2.5.2	Interruption time	256

5.1.2.6	E-UTRAN FDD – FDD conditional handover	257

5.1.2.6.1	Handover delay	257

5.1.2.6.2	Measurement time	258

5.1.2.6.3	Preparation time	258

5.1.2.6.4	Interruption time	258

5.1.2.7	E-UTRAN FDD – TDD conditional handover	258

5.1.2.8	E-UTRAN TDD – FDD conditional handover	259

5.1.2.9	E-UTRAN TDD – TDD conditional handover	259

5.2	Void	259

5.3	Handover to other RATs	259

5.3.1	E-UTRAN - UTRAN FDD Handover	259

5.3.1.1	Introduction	259

5.3.1.1.1	Handover delay	259

5.3.1.1.2	Interruption time	259

5.3.2	E-UTRAN - UTRAN TDD Handover	260

5.3.2.1	Introduction	260

5.3.2.2	Requirements	260

5.3.2.2.1	Handover delay	260

5.3.2.2.2	Interruption time	260

5.3.3	E-UTRAN - GSM Handover	261

5.3.3.1	Introduction	261

5.3.3.2	Requirements	261

5.3.3.2.1	Handover delay	261

5.3.3.2.2	Interruption time	261

5.3.4	E-UTRAN - NR FR1 Handover	261

5.3.4.1	Introduction	261

5.3.4.2	Handover delay	262

5.3.4.3	Interruption time	262

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	262

5.3.4A.1	Introduction	262

5.3.4A.2	Handover delay	263

5.3.4A.3	Interruption time	263

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	264

5.3.4B.1	Introduction	264

5.3.4B.2	Requirements	264

5.3.5	E-UTRAN - NR FR2 Handover	264

5.3.5.1	Introduction	264

5.3.5.2	Handover delay	264

5.3.5.3	Interruption time	264

5.4	Handover to Non-3GPP RATs	265

5.4.1	E-UTRAN – HRPD Handover	265

5.4.1.1	Introduction	265

5.4.1.1.1	Handover delay	265

5.4.1.1.2	Interruption time	265

5.4.2	E-UTRAN – cdma2000 1X Handover	266

5.4.2.1	Introduction	266

5.4.2.1.1	Handover delay	266

5.4.2.1.2	Interruption time	266

5.5	E-UTRAN Handover for Cat-M1 UEs	266

5.5.1	Introduction	266

5.5.2	Requirements in CEModeA	267

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	267

5.5.2.1.1	Handover delay	267

5.5.2.1.2	Interruption time	267

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	267

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	268

5.5.2.3.1	Void	268

5.5.2.3.2	Void	268

5.5.3	Requirements in CEModeB	268

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	268

5.5.3.1.1	Handover delay	268

5.5.3.1.2	Interruption time	268

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	269

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	269

5.5A	E-UTRAN Handover for Cat-M1 UEs for Satellite Access	269

5.5A.1	Introduction	269

5.5A.2	Requirements in CEModeA	269

5.5A.2.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	269

5.5A.2.1.1	Handover delay	269

5.5A.2.1.2	Interruption time	269

5.5A.2.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	270

5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	270

5.5A.2.3.1	Handover delay	270

5.5A.2.3.2	Measurement time	271

5.5A.2.3.3	Preparation time	272

5.5A.2.3.4	Interruption time	272

5.5A.2.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	273

5.5A.3	Requirements in CEModeB	273

5.5A.3.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	273

5.5A.3.1.1	Handover delay	273

5.5A.3.1.2	Interruption time	273

5.5A.3.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	274

5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	274

5.5A.3.3.1	Handover delay	274

5.5A.3.3.2	Measurement time	275

5.5A.3.3.3	Preparation time	276

5.5A.3.3.4	Interruption time	276

5.5A.3.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	277

5.6	Void	277

5.7	E-UTRAN DAPS Handover	277

5.7.1	Introduction	277

5.7.2	Requirements	278

5.7.2.1	E-UTRAN FDD – FDD	278

5.7.2.1.1	DAPS Handover delay	278

5.7.2.1.2	Interruption time	278

5.7.2.2	E-UTRAN FDD – TDD	279

5.7.2.3	E-UTRAN TDD – FDD	279

5.7.2.4	E-UTRAN TDD – TDD	279

5.8	EN-DC Handover with PSCell	279

5.8.1	Introduction	279

5.8.1.1	Handover with PSCell Interruption time	279

5.8.1.2	Handover with PSCell - NR PSCell Change Delay requirements	280

5.9	EN-DC Handover with PSCell using CCA	281

5.9.1	Introduction	281

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	281

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	281

6	RRC Connection Mobility Control	282

6.1	RRC Re-establishment	282

6.1.1	Introduction	282

6.1.2	Requirements	282

6.1.2.1	UE Re-establishment delay requirement	283

6.2	Random Access	283

6.2.1	Introduction	283

6.2.2	Requirements	283

6.2.2.1	Contention based random access	283

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	283

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	284

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	284

6.2.2.1.4	Void	284

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	284

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	284

6.2.2.2	Non-Contention based random access	284

6.2.2.2.1	Correct behaviour when receiving Random Access Response	284

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	284

6.2.3	Requirements for Cat-M1 UEs	284

6.2.3A	Random Access Requirements for Cat-M1 UEs for Satellite Access	285

6.2.4A	Random Access Requirements for Cat-M1 UEs with CB-Msg3 EDT for Satellite Access	285

6.2.4A.1	Correct behaviour when transmitting CB-Msg3	285

6.2.4A.2	Correct behaviour when receiving CB-Msg4	285

6.2.4A.3	Correct behaviour when not receiving CB-Msg4	285

6.3	RRC Connection Release with Redirection	285

6.3.1	Introduction	285

6.3.2	Requirements	285

6.3.2.1	RRC connection release with redirection to UTRAN FDD	285

6.3.2.2	RRC connection release with redirection to GERAN	286

6.3.2.3	RRC connection release with redirection to UTRAN TDD	286

6.3.2.4	RRC connection release with redirection to NR	287

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	288

6.3.2.6	RRC connection release with redirection to NR Redcap	289

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	290

6.4.1	Introduction	290

6.4.2	Requirements	290

6.5	RRC Re-establishment for NB-IoT UEs	290

6.5.1	Introduction	290

6.5.2	Requirements	290

6.5.2.1	UE Re-establishment delay requirement in normal coverage	291

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	291

6.5A	RRC Re-establishment for NB-IoT UEs for Satellite Access	292

6.5A.1	Introduction	292

6.5A.2	Requirements	292

6.5A.2.1	UE Re-establishment delay requirement in normal coverage	292

6.5A.2.2	UE Re-establishment delay requirement in enhanced coverage	293

6.5B	RRC Re-establishment for NB-IoT UEs for frame structure type 1 for NTN-TDD	293

6.5B.1	Introduction	293

6.5B.2	Requirements	293

6.5B.2.1	UE Re-establishment delay requirement in normal coverage	294

6.6	Random Access for UE category NB1	294

6.6.1	Introduction	294

6.6.2	Requirements	294

6.6.2.1	Correct behaviour when receiving Random Access Response reception	295

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	295

6.6.2.3	Correct behaviour when receiving a NACK on msg3	295

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	295

6.6.2.5	Correct behaviour when contention Resolution timer expires	295

6.6.2.6	MSG3-based channel quality report for UE Category NB1	295

6.6.3	Requirements for NPRACH configuration	296

6.6A	Random Access for UE category NB-IoT for Satellite Access	296

6.6A.1	Introduction	296

6.6A.2	Requirements	296

6.6A.2.1	Correct behaviour when receiving Random Access Response reception	297

6.6A.2.2	Correct behaviour when not receiving Random Access Response reception	297

6.6A.2.3	Correct behaviour when receiving a NACK on msg3	297

6.6A.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	297

6.6A.2.5	Correct behaviour when contention Resolution timer expires	297

6.6A.2.6	MSG3-based channel quality report for UE Category NB1	297

6.6A.3	Requirements for NPRACH configuration	298

6.6A.4	Requirements for CB-Msg3-EDT procedure	298

6.6A.4.1 Correct behaviour when transmitting CB-Msg3	298

6.6A.4.2	Correct behaviour when receiving a CB-Msg4 over CB-RNTI	298

6.6A.4.3	Correct behaviour when detecting CB-Msg3-EDT failure	298

6.6B	Random Access for UE category NB-IoT for frame structure type 1 for NTN-TDD	299

6.6B.1	Introduction	299

6.6B.2	Requirements	299

6.6B.2.1	Correct behaviour when receiving Random Access Response reception	299

6.6B.2.2	Correct behaviour when not receiving Random Access Response reception	299

6.6B.2.3	Correct behaviour when receiving a NACK on msg3	299

6.6B.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	299

6.6B.2.5	Correct behaviour when contention Resolution timer expires	299

6.6B.2.6	MSG3-based channel quality report for UE Category NB1	299

6.6B.3	Requirements for NPRACH configuration	300

6.7	RRC Re-establishment for Cat-M1 UEs	300

6.7.1	Introduction	300

6.7.2	Requirements	300

6.7.2.1	UE Re-establishment delay requirement for CEModeA	301

6.7.2.2	UE Re-establishment delay requirement for CEModeB	301

6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access	302

6.7A.1	Introduction	302

6.7A.2	Requirements	302

6.7A.2.1	UE Re-establishment delay requirement for CEModeA	302

6.7A.2.2	UE Re-establishment delay requirement for CEModeB	303

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	304

6.8.1	Introduction	304

6.8.2	Requirements	304

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	304

6.8A	RRC Connection Release with Redirection for UE Category M1 for Satellite Access	305

6.8A.1	Introduction	305

6.8A.2	Requirements	305

6.8A.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	305

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	306

6.9.1	Introduction	306

6.9.2	Requirements	306

6.9A	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access	306

6.9A.1	Introduction	306

6.9B	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for frame structure type 1 for NTN-TDD	307

6.9B.1	Introduction	307

6.9B.2	Requirements	307

7	Timing and signalling characteristics	308

7.1	UE transmit timing	308

7.1.1	Introduction	308

7.1.2	Requirements	309

7.2	UE timer accuracy	310

7.2.1	Introduction	310

7.2.2	Requirements	310

7.3	Timing Advance	310

7.3.1	Introduction	310

7.3.2	Requirements	310

7.3.2.1	Timing Advance adjustment delay	310

7.3.2.2	Timing Advance adjustment accuracy	311

7.4	Cell phase synchronization accuracy (TDD)	311

7.4.1	Definition	311

7.4.2	Minimum requirements	311

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	312

7.5.1	Introduction	312

7.5.2	eNodeB Synchronization Requirements	312

7.5.2.1	Synchronized E-UTRAN	312

7.5.2.2	Non-Synchronized E-UTRAN	312

7.6	Radio Link Monitoring	312

7.6.1	Introduction	312

7.6.2	Requirements	314

7.6.2.1	Minimum requirement when no DRX is used	314

7.6.2.2	Minimum requirement when DRX is used	314

7.6.2.3	Minimum requirement at transitions	316

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	316

7.6.2.5	Minimum requirement under IDC Interference	316

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	316

7.7.1	Introduction	316

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	317

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	318

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	319

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	320

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	321

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	322

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	323

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	323

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	323

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	324

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	325

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	326

7.7.14	SCell Activation Delay Requirement for Dormant SCell	326

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	328

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	328

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	330

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	330

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	332

7.8	Interruptions with Carrier Aggregation	334

7.8.1	Introduction	334

7.8.2	Requirements	334

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	334

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	334

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	334

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	335

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	335

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	335

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	335

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	335

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	336

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	337

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	337

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	337

7.8.2.13	Interruptions at SRS carrier based switching	338

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	339

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	339

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	339

7.8.2.17	Interruptions during CQI measurement on dormant SCell	339

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	340

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	340

7.8.2.20	Interruptions at SCell hibernation	340

7.8.2.21	Interruptions at direct SCell activation and hibernation	340

7.8.2.22	Interruptions during inter-RAT NR measurements without measurement gap	341

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	342

7.9.1	Introduction	342

7.9.2	Minimum Requirements for Interband Carrier Aggregation	342

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	342

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	342

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	343

7.10.1	Introduction	343

7.10.2	Requirements	343

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	343

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	343

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	343

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	344

7.11	Radio Link Monitoring for UE Category 0	344

7.11.1	Introduction	344

7.11.2	Requirements for FD-FDD and TDD	345

7.11.2.1	Minimum requirement when no DRX is used	345

7.11.2.2	Minimum requirement when DRX is used	346

7.11.2.3	Minimum requirement at transitions	346

7.11.3	Requirements for HD-FDD	347

7.11.3.1	Minimum requirement when no DRX is used	347

7.11.3.2	Minimum requirement when DRX is used	347

7.11.3.3	Minimum requirement at transitions	348

7.12	Interruptions with Dual Connectivity	348

7.12.1	Introduction	348

7.12.2	Requirements	348

7.12.2.1	Interruptions at PSCell addition/release	348

7.12.2.2	Interruptions at transitions between active and non-active during DRX	348

7.12.2.3	Interruptions at transitions from non-DRX to DRX	349

7.12.2.4	Interruptions at SCell addition/release	349

7.12.2.5	Interruptions at SCell activation/deactivation	349

7.12.2.6	Interruptions during measurements on SCC	350

7.12.2.7	Interruptions at SRS carrier based switching	350

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	351

7.13.1	Definition	351

7.13.2	Minimum requirements	351

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	351

7.14.1	Introduction	351

7.14.2	PSCell Addition Delay Requirement	351

7.14.3	PSCell Release Delay Requirement	352

7.15	Maximum Receive Timing Difference in Dual Connectivity	352

7.15.1	Introduction	352

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	352

7.16	Proximity-based Services	352

7.16.1	Introduction	352

7.16.2	Requirements	352

7.16.2.1	ProSe UE transmission timing	352

7.16.2.1.1	Serving cell or PCell as timing reference	353

7.16.2.1.2	SCell or non-serving cell as timing reference	353

7.16.3	Interruptions with ProSe	353

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	353

7.16.3.2	Interruptions at ProSe Direct Communication configuration	354

7.16.3.3	Interruptions during ProSe Direct Discovery	354

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	354

7.16.3.5	Interruptions during ProSe Direct Communication	355

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	355

7.16.4.1	Measurement and evaluation of selected cell	355

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	355

7.16.5	Selection / Reselection of ProSe relay UE	356

7.16.6	ProSe operation under deactivated SCell	356

7.17	Maximum Transmission Timing Difference in Dual Connectivity	356

7.17.1	Introduction	356

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	357

7.18.1	Introduction	357

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	357

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	357

7.19	Radio Link Monitoring for UE Category M1	357

7.19.1	Introduction	357

7.19.2	Requirements for FD-FDD and TDD CE mode A	358

7.19.2.1	Minimum requirement when no DRX is used	359

7.19.2.2	Minimum requirement when DRX is used	360

7.19.2.3	Minimum requirement at transitions	361

7.19.3	Requirements for HD-FDD with CE mode A	361

7.19.3.1	Minimum requirement when no DRX is used	362

7.19.3.2	Minimum requirement when DRX is used	362

7.19.3.3	Minimum requirement at transitions	363

7.19.4	Requirements for FD-FDD and TDD with CE mode B	363

7.19.4.1	Minimum requirement when no DRX is used	365

7.19.4.2	Minimum requirement when DRX is used	366

7.19.4.3	Minimum requirement at transitions	367

7.19.5	Requirements for HD-FDD with CE mode B	367

7.19.5.1	Minimum requirement when no DRX is used	367

7.19.5.2	Minimum requirement when DRX is used	368

7.19.5.3	Minimum requirement at transitions	369

7.19A	Radio Link Monitoring for UE Category M1 for Satellite Access	369

7.19A.1	Introduction	369

7.19A.2	Requirements for FD-FDD and CE mode A	369

7.19A.2.1	Minimum requirement when no DRX is used	371

7.19A.2.2	Minimum requirement when DRX is used	372

7.19A.2.3	Minimum requirement at transitions	373

7.19A.3	Requirements for HD-FDD with CE mode A	373

7.19A.3.1	Minimum requirement when no DRX is used	373

7.19A.3.2	Minimum requirement when DRX is used	374

7.19A.3.3	Minimum requirement at transitions	375

7.19A.4	Requirements for HD-FDD with CE mode B	375

7.19A.4.1	Minimum requirement when no DRX is used	375

7.19A.4.2	Minimum requirement when DRX is used	375

7.19A.4.3	Minimum requirement at transitions	376

7.20	UE transmit timing for NB-IoT	377

7.20.1	Introduction	377

7.20.2	Requirements	377

7.20A	UE transmit timing for NB-IoT for Satellite Access	377

7.20A.1	Introduction	377

7.20A.2	Requirements	378

7.20B	UE transmit timing for NB-IoT for frame structure type 1 for NTN-TDD	378

7.20B.1	Introduction	378

7.20B.2	Requirements	379

7.21	 UE timer accuracy for NB-IoT	379

7.21.1	Introduction	379

7.21.2	Requirements	379

7.21A	UE timer accuracy for NB-IoT for Satellite Access	380

7.21A.1	Introduction	380

7.21A.2	Requirements	380

7.21B	UE timer accuracy for NB-IoT for frame structure type 1 for NTN-TDD	380

7.21B.1	Introduction	380

7.21B.2	Requirements	380

7.22	Timing Advance for NB-IoT	381

7.22.1	Introduction	381

7.22.2	Requirements	381

7.22.2.1	Timing Advance adjustment delay	381

7.22.2.2	Timing Advance adjustment accuracy	381

7.22A	Timing Advance for NB-IoT for Satellite Access	381

7.22A.1	Introduction	381

7.22A.2	Requirements	381

7.22A.2.1	Timing Advance adjustment delay	381

7.22A.2.2	Timing Advance adjustment accuracy	381

7.22B	Timing Advance for NB-IoT for frame structure type 1 for NTN-TDD	381

7.22B.1	Introduction	381

7.22B.2	Requirements	381

7.22B.2.1	Timing Advance adjustment delay	381

7.22B.2.2	Timing Advance adjustment accuracy	382

7.23	Radio Link Monitoring for Category NB1 UE	382

7.23.1	Introduction	382

7.23.2	Requirements for Category NB1 UE	382

7.23.2.1	Minimum requirement when no DRX is used	382

7.23.2.2	Minimum requirement when DRX is used	383

7.23.2.3	Minimum requirement at transitions	383

7.23A	Radio Link Monitoring for Category NB-IoT UE for Satellite Access	384

7.23A.1	Introduction	384

7.23A.2	Requirements for Category NB1 UE	384

7.23A.2.1	Minimum requirement when no DRX is used	384

7.23A.2.2	Minimum requirement when DRX is used	385

7.23A.2.3	Minimum requirement at transitions	385

7.23B	Radio Link Monitoring for Category NB-IoT UE for frame structure type 1 for NTN-TDD	385

7.23B.1	Introduction	385

7.23B.2	Requirements for Category NB1 UE	386

7.23B.2.1	Minimum requirement when no DRX is used	386

7.23B.2.2	Minimum requirement when DRX is used	387

7.23B.2.3	Minimum requirement at transitions	387

7.24	UE transmit timing for Category M1	387

7.24.1	Introduction	387

7.24.2	Requirements	388

7.24A	UE transmit timing for Category M1 for Satellite Access	388

7.24A.1	Introduction	388

7.24A.2	Requirements	389

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	390

7.25.1	Definition	390

7.25.2	Minimum requirements	390

7.26	UE transmit timing for Category M2	390

7.26.1	Introduction	390

7.26.2	Requirements	390

7.27	UE timer accuracy for category M1	391

7.27.1	Introduction	391

7.27.2	Requirements	391

7.27A	UE timer accuracy for category M1 for Satellite Access	391

7.27A.1	Introduction	391

7.27A.2	Requirements	391

7.28	Timing Advance for Category M1	391

7.28.1	Introduction	391

7.28.2	Requirements	391

7.28A	Timing Advance for Category M1 for Satellite Access	391

7.28A.1	Introduction	391

7.28A.2	Requirements	391

7.28A.2.1	Timing Advance adjustment delay	391

7.28A.2.2	Timing Advance adjustment accuracy	392

7.29	Interruptions requirements with FeMBMS	392

7.29.1	Introduction	392

7.29.2	Requirements	392

7.30	Numerology switching delay requirements with FeMBMS	392

7.30.1	Introduction	392

7.30.2	Requirements	392

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	392

7.31.1	Introduction	392

7.31.2	NR PSCell Addition Delay Requirement	393

7.31.3	NR PSCell Release Delay Requirement	393

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	394

7.31A.1	Introduction	394

7.31A.2	NR PSCell Addition Delay Requirement	394

7.31A.3	NR PSCell Release Delay Requirement	395

7.32	Interruptions with EN-DC	395

7.32.1	Introduction	395

7.32.2	Requirements	396

7.32.2.1	Interruptions at PSCell addition/release	396

7.32.2.2	Interruptions at transitions between active and non-active during DRX	396

7.32.2.3	Interruptions at transitions from non-DRX to DRX	396

7.32.2.4	Interruptions at SCell addition/release	396

7.32.2.5	Interruptions at SCell activation/deactivation	396

7.32.2.6	Interruptions during measurements on SCC	397

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	397

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	397

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	397

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	398

7.32.2.7	Interruptions at active BWP switching	398

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	399

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	399

7.32.2.10	Interruptions at SCell hibernation	399

7.32.2.11	Interruptions at direct SCell activation and hibernation	399

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	399

7.32.2.13	Interruptions at NR SRS carrier based switching	399

7.32.2.14	Interruptions at NR SCell dormancy	400

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	400

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	401

7.32.2.15	Interruption during NR measurement with autonomous gaps	401

7.32.2.16	Interruptions at SRS carrier based switching	401

7.32.2.17	Interruptions at SCG activation/deactivation	402

7.32.2.18	Interruptions due to NR SRS antenna port switching	402

7.32.2.19	Interruptions at fast SCell activation/deactivation	402

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	403

7.32.2.21	Interruptions during RLM/BFD measurements on deactivated PScell	403

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	403

7.33.1	Introduction	403

7.33.2	Requirements	403

7.34	Void	404

7.35	Interruptions with SFTD measurements	404

7.35.1	Introduction	404

7.35.2	Requirements	404

7.36	Interruptions with NE-DC	404

7.36.1	Introduction	404

7.36.2	Requirements	405

7.36.2.1	Interruptions at transitions between active and non-active during DRX	405

7.36.2.2	Interruptions at transitions from non-DRX to DRX	405

7.36.2.3	Interruptions at SCell addition/release	405

7.36.2.4	Interruptions at SCell activation/deactivation	405

7.36.2.5	Interruptions during measurements on SCC	406

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	406

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	406

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	406

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	406

7.36.2.6	Interruptions at active BWP switching	406

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	407

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	407

7.36.2.9	Interruptions at SCell hibernation	407

7.36.2.10	Interruptions at direct SCell activation and hibernation	407

7.36.2.11	Interruptions at NR SRS carrier based switching	408

7.36.2.12	Interruptions at NR SCell dormancy	408

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	408

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	409

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	409

7.36.2.14	Interruption during NR measurement with autonomous gaps	409

7.36.2.15	Interruptions at SRS carrier based switching	410

7.36.2.16	Interruptions due to NR SRS antenna port switching	410

7.37	Interruptions during NR measurement with autonomous gaps	411

7.37.1	Introduction	411

7.37.2	Requirements	411

7.38 SCG Activation and Deactivation Delay	411

7.38.1	Introduction	411

7.38.2	SCG Activation Delay Requirement	411

7.38.3	SCG Deactivation Delay Requirement	412

8	UE Measurements Procedures in RRC_CONNECTED State	413

8.1	General Measurement Requirements	413

8.1.1	Introduction	413

8.1.2	Requirements	413

8.1.2.1	UE measurement capability	413

8.1.2.1.1	Monitoring of multiple layers using gaps	421

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	422

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	423

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	425

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	426

8.1.2.1.2	Network controlled small gap	426

8.1.2.2	E-UTRAN intra frequency measurements	428

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	428

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	433

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	437

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	438

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	439

8.1.2.3	E-UTRAN inter frequency measurements	439

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	440

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	445

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	452

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	452

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	452

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	453

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	455

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	456

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	457

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	464

8.1.2.4	Inter RAT measurements	464

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	464

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	469

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	469

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	473

8.1.2.4.5	E-UTRAN FDD – GSM measurements	473

8.1.2.4.6	E-UTRAN TDD – GSM measurements	478

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	478

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	480

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	480

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	480

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	481

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	481

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	481

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	481

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	483

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	483

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	483

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	483

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	484

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	484

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	486

8.1.2.4.21	E-UTRAN FDD – NR measurements	486

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	490

8.1.2.4.22	E-UTRAN TDD – NR measurements	494

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	494

8.1.2.4.23	Void	494

8.1.2.4.24	Void	494

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	494

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	496

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	496

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	497

8.1.2.4.29	E-UTRAN FDD – NR measurements without measurement gap	497

8.1.2.4.30	E-UTRAN TDD – NR measurements without measurement gap	502

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	502

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	502

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	504

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	505

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	507

8.1.2.6.5	Void	509

8.1.2.6.6	Void	509

8.1.2.6.7	Void	509

8.1.2.6.8	Void	509

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	509

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	509

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	511

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	513

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	515

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	516

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	518

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	520

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	521

8.1.2.7	E-UTRAN E-CID Measurements	523

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	523

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	524

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	526

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	526

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	527

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	527

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	530

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	533

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	536

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	540

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	540

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	540

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	541

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	541

8.1.2.10	Void	542

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	542

8.2.1	Introduction	542

8.2.2	Requirements	542

8.3	Measurements for E-UTRA carrier aggregation	546

8.3.1	Introduction	546

8.3.2	Measurements of the primary component carrier	546

8.3.3	Measurements of a secondary component carrier	546

8.3.3.1	Measurements of a secondary component carrier with active SCell	546

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	547

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	547

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	548

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	550

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	550

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	550

8.4.1	Introduction	550

8.4.2	Measurements on the primary component carrier	551

8.4.3	Measurements on a secondary component carrier	552

8.4.4	Measurements on both primary component carrier and a secondary component carrier	552

8.4.5	Measurements on different secondary component carriers	553

8.5	Measurements for UE category 0	554

8.5.1	Introduction	554

8.5.2 	Requirements	555

8.5.2.1 	E-UTRAN intra frequency measurements	555

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	555

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	558

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	560

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	564

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	564

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	565

8.6	Discovery signal measurements	566

8.6.1	Introduction	566

8.6.2	Requirements for CRS based discovery signal measurements	566

8.6.2.1	E-UTRAN intra frequency measurements	566

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	566

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	569

8.6.2.2	E-UTRAN inter frequency measurements	571

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	572

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	574

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	577

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	577

8.6.3	Requirements for CSI-RS based discovery signal measurements	577

8.6.3.1	E-UTRAN intra frequency measurements	577

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	578

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	580

8.6.3.2	E-UTRAN inter frequency measurements	582

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	583

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	585

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	588

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	588

8.7	Discovery signal measurements for E-UTRA carrier aggregation	588

8.7.1	Introduction	588

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	589

8.7.2.1	Measurements of the primary component carrier	589

8.7.2.2	Measurements of a secondary component carrier	589

8.7.2.3	Measurements of a secondary component carrier with active SCell	589

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	589

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	589

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	590

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	592

8.7.3.1	Measurements of the primary component carrier	592

8.7.3.2	Measurements of a secondary component carrier	592

8.7.3.3	Measurements of a secondary component carrier with active SCell	592

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	592

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	592

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	594

8.8	Measurements for E-UTRA dual connectivity	595

8.8.1	Introduction	595

8.8.2	Intra-frequency measurements requirements on PCell	595

8.8.3	Intra-frequency measurements requirements on PSCell	596

8.8.4	Inter-frequency and inter-RAT measurement requirements	596

8.8.5	Intra-frequency measurements with autonomous gaps	596

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	596

8.8.5.2	ECGI reporting delay	597

8.8.6	Inter-frequency measurements with autonomous gaps	597

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	597

8.8.6.2	ECGI reporting delay	598

8.8.7	SSTD Measurements	598

8.8.7.1	Introduction	598

8.8.7.2	SSTD Measurement requirements	598

8.8.7.3	SSTD Measurement Reporting Delay	599

8.8.8	Intra-frequency measurements requirements on SCell	599

8.9	MBSFN Measurements	599

8.9.1	Introduction	599

8.9.2	MBSFN RSRP Measurements	599

8.9.3	MBSFN RSRQ Measurements	600

8.9.4	MCH BLER Measurements	600

8.10	Proximity-based Services	600

8.10.1	Introduction	600

8.10.2	Requirements	600

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	600

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	601

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	602

8.11.1	Introduction	602

8.11.2	CRS based discovery signal measurements	602

8.11.2.1	E-UTRAN intra-frequency measurements	602

8.11.2.1.1	Requirements	602

8.11.2.1.1.1	Requirements when no DRX is used	602

8.11.2.1.1.1.1	Measurement Reporting Requirements	604

8.11.2.1.1.2	Requirements when DRX is used	605

8.11.2.1.1.2.1	Measurement Reporting Requirements	607

8.11.2.2	E-UTRAN inter-frequency measurements	608

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	608

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	612

8.11.3	CSI-RS based discovery signal measurements	612

8.11.3.1	E-UTRAN intra-frequency measurements	612

8.11.3.1.1	Requirements	612

8.11.3.1.1.1	Requirements when no DRX is used	612

8.11.3.1.1.1.1	Measurement Reporting Requirements	614

8.11.3.1.1.2	Requirements when DRX is used	614

8.11.3.1.1.2.1	Measurement Reporting Requirements	615

8.11.3.2	E-UTRAN inter-frequency measurements	616

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	616

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	620

8.11.4	RSSI measurements	620

8.11.4.1	E-UTRAN intra-frequency measurements	620

8.11.4.2	E-UTRAN inter-frequency measurements	620

8.11.5	Channel occupancy measurements	621

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	621

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	621

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	621

8.12.1	Introduction	621

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	621

8.12.2.1	Introduction	621

8.12.2.2	Measurements of a secondary component carrier	621

8.12.2.3	Measurements of a secondary component carrier with active SCell	621

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	622

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	622

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	624

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	627

8.12.3.1	Introduction	627

8.12.3.2	Measurements of a secondary component carrier	627

8.12.3.3	Measurements of a secondary component carrier with active SCell	627

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	627

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	627

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	629

8.13	Measurements for UE Category M1	631

8.13.1	Introduction	631

8.13.2	Requirements for UE category M1 with CE mode A	631

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	632

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	632

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	636

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	639

8.13.2.2	Void	643

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	643

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	643

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	646

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	648

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	649

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	649

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	651

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	653

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	654

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	654

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	654

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	655

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	655

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	655

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	656

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	656

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	657

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	658

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	658

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	659

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	663

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	665

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	669

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	669

8.13.3	Requirements for UE category M1 with CE mode B	670

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	670

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	671

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	675

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	678

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	683

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	684

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	684

8.13.3.2	Void	685

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	685

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	685

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	688

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	690

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	691

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	691

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	691

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	691

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	692

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	692

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	693

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	693

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	693

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	698

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	700

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	705

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	705

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	705

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	707

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	710

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	710

8.13A	Measurements for UE Category M1 for Satellite Access	711

8.13A.1	Introduction	711

8.13A.2	Requirements for UE category M1 with CE mode A	711

8.13A.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	712

8.13A.2.1.1	E-UTRAN FDD intra frequency measurements	712

8.13A.2.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	712

8.13A.2.1.1.2	E-UTRAN intra frequency measurements when DRX is used	714

8.13A.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	716

8.13A.2.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	716

8.13A.2.1.2.2	E-UTRAN intra frequency measurements when DRX is used	716

8.13A.2.2	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	718

8.13A.2.2.1	E-UTRAN FDD - FDD inter frequency measurements	718

8.13A.2.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	718

8.13A.2.2.1.2	E-UTRAN inter frequency measurements when DRX is used	720

8.13A.2.2.2	E-UTRAN inter-frequency measurements for HD-FDD	722

8.13A.2.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	722

8.13A.2.2.2.2	E-UTRAN inter frequency measurements when DRX is used	722

8.13A.2.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	724

8.13A.2.4	Channel quality report for UE Category M1 in connected mode with CE mode A	725

8.13A.3	Requirements for UE category M1 with CE mode B	725

8.13A.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	726

8.13A.3.1.1	E-UTRAN FDD intra frequency measurements	726

8.13A.3.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	726

8.13A.3.1.1.2	E-UTRAN intra frequency measurements when DRX is used	728

8.13A.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	730

8.13A.3.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	730

8.13A.3.1.2.2	E-UTRAN intra frequency measurements when DRX is used	731

8.13A.3.2	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	733

8.13A.3.2.1	E-UTRAN FDD - FDD inter frequency measurements	733

8.13A.3.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	733

8.13A.3.2.1.2	E-UTRAN inter frequency measurements when DRX is used	735

8.13A.3.2.2	E-UTRAN inter-frequency measurements for HD-FDD	737

8.13A.3.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	737

8.13A.3.2.2.2	E-UTRAN inter frequency measurements when DRX is used	737

8.13A.3.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	739

8.13A.3.4	Channel quality report for UE Category M1 in connected mode with CE mode B	740

8.14	Measurements for UE category NB1	740

8.14.1	Introduction	740

8.14.2	NB-IoT intra frequency measurements under normal coverage	740

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	740

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	740

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	741

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	741

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	741

8.14.4	Connected mode channel quality report for UE Category NB1	741

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	741

8.14.6	NB-IoT neighbour cell measurements	742

8.14.6.1	Introduction	742

8.14.6.2	Requirements	742

8.14.6.3	Intra-frequency neighbour cell measurements	742

8.14.6.4	Inter-frequency neighbour cell measurements	743

8.14.6.5	 Requirements for monitoring multiple carriers	744

8.14A	Measurements for UE category NB-IoT for Satellite Access	744

8.14A.1	Introduction	744

8.14A.2	NB-IoT intra frequency measurements under normal coverage	745

8.14A.2.1	NB-IoT intra frequency measurements when no DRX is used	745

8.14A.2.2	NB-IoT intra frequency measurements when DRX is used	745

8.14A.3	NB-IoT intra frequency measurements under enhanced coverage	745

8.14A.3.1	NB-IoT intra frequency measurements when no DRX is used	745

8.14A.3.2	NB-IoT intra frequency measurements when DRX is used	745

8.14A.4	Connected mode channel quality report for UE Category NB1	745

8.14B	Measurements for UE category NB-IoT for frame structure type 1 for NTN-TDD	748

8.14B.1	Introduction	748

8.14B.2	NB-IoT intra frequency measurements under normal coverage	749

8.14B.2.1	NB-IoT intra frequency measurements when no DRX is used	749

8.14B.2.2	NB-IoT intra frequency measurements when DRX is used	749

8.14B.4	Connected mode channel quality report for UE Category NB1	749

8.14B.6	NB-IoT neighbour cell measurements	750

8.14B.6.1	Introduction	750

8.14B.6.2	Requirements	750

8.14B.6.3	Intra-frequency neighbour cell measurements	750

8.14B.6.4	Inter-frequency neighbour cell measurements	751

8.14B.6.5	Requirements for monitoring multiple carriers	752

8.15	Void	752

8.16	Measurements for UE Category M2	752

8.16.1	Introduction	752

8.16.2	Requirements for UE category M2 with CE mode A	753

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	753

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	753

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	754

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	754

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	755

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	755

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	755

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	756

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	758

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	761

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	761

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	761

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	763

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	766

8.16.3	Requirements for UE category M2 with CE mode B	766

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	766

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	766

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	769

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	772

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	772

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	772

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	774

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	777

8.17	Measurements for E-UTRA – NR Dual Connectivity	777

8.17.1	Introduction	777

8.17.1.1	Measurement Gap Sharing	777

8.17.1A	Intrafrequency Measurements	778

8.17.2	SFTD Measurements	778

8.17.2.1	Introduction	778

8.17.2.2	SFTD Measurement requirements	778

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	779

8.17.2.3	SFTD Measurement Reporting Delay	780

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	780

8.17.3.1	Introduction	780

8.17.3.2	E-UTRAN FDD inter frequency measurements	781

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	781

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	782

8.17.3.3	E-UTRAN TDD inter frequency measurements	784

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	784

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	786

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	788

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	788

8.17.4.1.1	NR Inter-RAT cell identification	788

8.17.4.1.2	NR Inter-RAT measurement	790

8.17.4.1.3	NR Inter-RAT measurement reporting	790

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	791

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	791

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	791

8.17.4A.1.1	NR Inter-RAT cell identification	792

8.17.4A.1.2	NR Inter-RAT measurement	793

8.17.4A.1.3	NR Inter-RAT measurement reporting	794

8.17.4A.1.4	NR inter-RAT RSSI measurements	795

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	795

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	796

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	796

8.17.5.1	Introduction	796

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	796

8.17.5.2.1	Identification of a new UTRA FDD cell	796

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	796

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	797

8.17.5.2.4	Periodic Reporting	797

8.17.5.2.5	Event Triggered Reporting	797

8.17.5.2.6	Event-triggered Periodic Reporting	798

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	798

8.17.5.3.1	Periodic Reporting	799

8.17.5.3.2	Event Triggered Reporting	799

8.17.5.3.3	Event-triggered Periodic Reporting	800

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	800

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	800

8.17.7.1	Introduction	800

8.17.7.2	Identification of a new UTRA FDD cell for SON	800

8.17.7.2.1	Requirements when no DRX is used	800

8.17.7.2.2	Requirements when DRX is used	801

8.17.7.2.3	Reporting Delay	801

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	802

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	802

8.17.9.1	Introduction	802

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	802

8.17.9.2.1	Identification of a new UTRA FDD cell	802

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	802

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	803

8.17.9.2.4	Periodic Reporting	803

8.17.9.2.5	Event Triggered Reporting	803

8.17.9.2.6	Event-triggered Periodic Reporting	804

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	804

8.17.9.3.1	Periodic Reporting	805

8.17.9.3.2	Event Triggered Reporting	805

8.17.9.3.3	Event-triggered Periodic Reporting	805

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	806

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	806

8.17.11.1	Introduction	806

8.17.11.2	Identification of a new UTRA TDD cell for SON	806

8.17.11.2.1	Requirements when no DRX is used	806

8.17.11.2.2	Requirements when DRX is used	806

8.17.11.2.3	Reporting Delay	807

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	807

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	808

8.17.13.1	Introduction	808

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	808

8.17.13.2.1	GSM carrier RSSI	808

8.17.13.2.2	BSIC verification	808

8.17.13.2.3	Enhanced BSIC verification	810

8.17.13.2.4	Periodic Reporting	810

8.17.13.2.5	Event Triggered Reporting	810

8.17.13.2.6	Event-triggered Periodic Reporting	811

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	811

8.17.13.3.1	GSM carrier RSSI	811

8.17.13.3.2	BSIC verification	811

8.17.13.3.3	Periodic Reporting	813

8.17.13.3.4	Event Triggered Reporting	813

8.17.13.3.5	Event-triggered Periodic Reporting	813

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	813

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	813

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	813

8.17.15.1.1	RSTD Measurement Reporting Delay	814

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	814

8.17.15.2.1	RSTD Measurement Reporting Delay	814

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	814

8.17.15.3.1	RSTD Measurement Reporting Delay	815

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	815

8.17.15.4.1	RSTD Measurement Reporting Delay	815

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	816

8.17.16.1	Introduction	816

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	816

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	816

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	816

8.17.17.1	Introduction	816

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	816

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	816

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	816

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	816

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	816

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	816

8.18	Measurements for non-BL/CE UE	816

8.18.1	Introduction	816

8.18.2	Requirements for non-BL/CE UE with CE Mode B	817

8.18.2.1	E-UTRAN intra frequency measurements	817

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	817

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	817

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	818

8.19	Measurements for NR – E-UTRA Dual Connectivity	819

8.19.1	Introduction	819

8.19.2	Intra-frequency Measurements	819

8.19.3	Inter-frequency Measurements	819

8.19.4	Void	819

8.19.5	Intra-frequency E-CID Measurements	819

8.19.6	Intra-frequency measurements with autonomous gaps	820

8.19.6.1	Introduction	820

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	820

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	820

8.19.7	Inter-frequency measurements with autonomous gaps	820

8.19.7.1	Introduction	820

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	820

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	820

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	820

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	820

8.20	Inter-RAT NR Measurements for RedCap UE	820

8.20.1	Introduction	820

8.20.2	E-UTRAN FDD – NR measurements	820

8.20.2.1	Identification of a new NR cell	820

8.20.2.2	Periodic Reporting	824

8.20.2.3	Event Triggered Reporting	824

8.20.2.4	Event-triggered Periodic Reporting	824

8.20.3	E-UTRAN TDD – NR measurements	824

9	Measurements performance requirements for UE	825

9.1	E-UTRAN measurements	825

9.1.1	Introduction	825

9.1.2	Intra-frequency RSRP Accuracy Requirements	825

9.1.2.1	Absolute RSRP Accuracy	825

9.1.2.2	Relative Accuracy of RSRP	826

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	827

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	827

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	828

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	829

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	830

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	831

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	832

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	832

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	832

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	833

9.1.2B.1	Introduction	833

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	833

9.1.3	Inter-frequency RSRP Accuracy Requirements	834

9.1.3.1	Absolute RSRP Accuracy	834

9.1.3.2	Relative Accuracy of RSRP	835

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	836

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	836

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	837

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	837

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	838

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	839

9.1.3B.1	Introduction	839

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	839

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	839

9.1.4	RSRP Measurement Report Mapping	840

9.1.5	Intra-frequency RSRQ Accuracy Requirements	840

9.1.5.1	Absolute RSRQ Accuracy	840

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	841

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	842

9.1.5.4	Absolute WB-RSRQ Accuracy	843

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	844

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	845

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	845

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	845

9.1.5B.1	Introduction	845

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	845

9.1.6	Inter-frequency RSRQ Accuracy Requirements	846

9.1.6.1	Absolute RSRQ Accuracy	846

9.1.6.2	Relative Accuracy of RSRQ	847

9.1.6.3	Absolute WB-RSRQ Accuracy	848

9.1.6.4	Relative WB-RSRQ Accuracy	848

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	849

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	850

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	850

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	850

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	851

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	852

9.1.6B.1	Introduction	852

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	852

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	852

9.1.7	RSRQ Measurement Report Mapping	853

9.1.8	Power Headroom	853

9.1.8.1	Period	854

9.1.8.2	Reporting Delay	854

9.1.8.3	Void	854

9.1.8.4	Report Mapping	854

9.1.8A	Power Headroom for UE category M1 for satellite access	854

9.1.8A.1	Period	855

9.1.8A.2	Reporting Delay	855

9.1.8A.3	Void	855

9.1.8A.4	Report Mapping	855

9.1.9	UE Rx – Tx time difference	855

9.1.9.1	Measurement Requirement	855

9.1.9.2	Measurement Report mapping	856

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	857

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	858

9.1.10	Reference Signal Time Difference (RSTD)	859

9.1.10.1	Intra-Frequency Accuracy Requirement	859

9.1.10.2	Inter-Frequency Accuracy Requirement	860

9.1.10.3	RSTD Measurement Report Mapping	862

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	862

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	863

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	864

9.1.11	Carrier aggregation measurement accuracy	865

9.1.11.1	Primary component carrier accuracy requirement	866

9.1.11.2	Secondary component carrier accuracy requirement	866

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	866

9.1.11.4	Secondary component carrier relative accuracy requirement	866

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	866

9.1.13	Measurement accuracy for UE category 0	867

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	867

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	867

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	868

9.1.14	Accuracy requirements for Discovery Signal Measurements	869

9.1.14.1	Introduction	869

9.1.14.2	RSRP measurements in discovery signal occasions	869

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	869

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	869

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	869

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	870

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	871

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	871

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	871

9.1.14.3.3	CSI-RSRP measurement report mapping	872

9.1.14.4	RSRQ measurements in discovery signal occasions	872

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	872

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	873

9.1.15.1.1	Primary component carrier accuracy requirement	873

9.1.15.1.2	Secondary component carrier accuracy requirement	873

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	873

9.1.15.1.4	Secondary component carrier relative accuracy requirement	873

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	873

9.1.15.2.1	Primary component carrier accuracy requirement	873

9.1.15.2.2	Secondary component carrier accuracy requirement	873

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	873

9.1.15.2.4	Secondary component carrier relative accuracy requirement	873

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	874

9.1.17	RS-SINR Measurements	874

9.1.17.1	Measurement Report Mapping	874

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	875

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	875

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	875

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	875

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	876

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	877

9.1.18.1	Introduction	877

9.1.18.2	RSRP measurements	877

9.1.18.2.1	RSRP measurement report mapping	877

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	877

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	878

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	878

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	879

9.1.18.3	RSRQ measurements	879

9.1.18.3.1	RSRQ measurement report mapping	879

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	879

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	880

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	880

9.1.18.4	CSI-RSRP measurements	881

9.1.18.4.1	CSI-RSRP measurement report mapping	881

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	881

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	881

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	882

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	882

9.1.18.5	RSSI measurements	883

9.1.18.5.1	RSSI measurement report mapping	883

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	883

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	884

9.1.18.6	Channel occupancy measurements	884

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	884

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	884

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	885

9.1.19.1	Introduction	885

9.1.19.2	Accuracy requirements for measurements on SCC	885

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	885

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	885

9.1.20	SFN and Subframe Time Difference (SSTD)	886

9.1.20.1	SSTD Accuracy Requirement	886

9.1.20.2	SSTD Measurement Report Mapping	886

9.1.21	Measurement accuracy for UE category M1	887

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	887

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	889

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	890

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	892

9.1.21.5	RSRP Measurement Report Mapping	893

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	893

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	894

9.1.21.8	RSRQ Measurement Report Mapping	895

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	895

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	896

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	897

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	898

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	899

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	900

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	901

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	902

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	903

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	904

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	905

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	906

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	909

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	912

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	912

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	915

9.1.21A	Measurement accuracy for UE category M1 for satellite access	916

9.1.21A.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	917

9.1.21A.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	918

9.1.21A.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	919

9.1.21A.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	921

9.1.21A.5	RSRP Measurement Report Mapping	922

9.1.21A.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	922

9.1.21A.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	923

9.1.21A.8	RSRQ Measurement Report Mapping	924

9.1.21A.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	924

9.1.21A.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	925

9.1.21A.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	926

9.1.21A.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	927

9.1.21A.13	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	928

9.1.21A.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	929

9.1.21A.15	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	930

9.1.21A.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	931

9.1.21A.17	Downlink Channel Report Mapping for UE Category M1	932

9.1.21A.18	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	932

9.1.21A.19	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	934

9.1.22	Measurement accuracy for UE Category NB1	935

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	935

9.1.22.2	Void	936

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	936

9.1.22.4	Void	937

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	937

9.1.22.6	Void	938

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	938

9.1.22.8	Void	939

9.1.22.9	NRSRP Measurement Report Mapping	939

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	939

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	940

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	941

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	942

9.1.22.14	NRSRQ Measurement Report Mapping	943

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	944

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	944

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	945

9.1.22A	Measurement accuracy for UE Category NB1 for satellite access	945

9.1.22A.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	945

9.1.22A.2	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	946

9.1.22A.3	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	947

9.1.22A.4	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	948

9.1.22A.5	NRSRP Measurement Report Mapping	949

9.1.22A.6	NRSRQ Measurement Report Mapping	949

9.1.22A.7	MSG3-based Measurement Report Mapping for UE Category NB1	949

9.1.22A.8	Downlink Channel Quality Measurement Accuracy for UE Category NB1	949

9.1.23	Power Headroom for UE Category NB1	950

9.1.23.1	Period	950

9.1.23.2	Reporting Delay	950

9.1.23.3	Report Mapping for UE Category NB1	950

9.1.23.3.1	Void	952

9.1.23.3.2	Void	952

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	952

9.1.23A	Power Headroom for UE Category NB1 for Satellite Access	953

9.1.23A.1	Period	953

9.1.23A.2	Reporting Delay	953

9.1.23A.3	Report Mapping for UE Category NB1 for Satellite Access	954

9.1.23A.3.1	Void	954

9.1.23A.3.2	Void	954

9.1.23A.4	Report Mapping for UE Category NB1 for UE Power Class 6 for Satellite Access	954

9.1.24	Void	954

9.1.25	Measurement accuracy for UE category M2	954

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	954

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	955

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	956

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	957

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	959

9.1.26	Measurement Accuracy for non-BL CE UE	960

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	961

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	963

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	963

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	965

9.1.26.5	RSRP Measurement Report Mapping	966

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	966

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	966

9.1.26.8	RSRQ Measurement Report Mapping	967

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	967

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	968

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	969

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	970

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	971

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	971

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	971

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	972

9.1.27	SFN and frame Timing Difference (SFTD)	973

9.1.27.1	SFTD Accuracy Requirement	973

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	975

9.1.28.1	SFTD Accuracy Requirement under CCA	975

9.2	UTRAN FDD Measurements	976

9.2.1	UTRAN FDD CPICH RSCP	976

9.2.2	Void	977

9.2.3	UTRAN FDD CPICH Ec/No	977

9.3	UTRAN TDD Measurements	977

9.3.1	UTRAN TDD P-CCPCH RSCP	978

9.3.2	Void	978

9.3.3	Void	978

9.4	GSM Measurements	978

9.4.1	GSM carrier RSSI	978

9.5	CDMA2000 1x RTT Measurements	978

9.5.1	CDMA2000 1x RTT Pilot Strength	978

9.6	PCMAX,c	979

9.6.1	Report Mapping	979

9.6.2 	Estimation Period	979

9.6.3	Reporting Delay	979

9.7	IEEE802.11 Measurements	979

9.7.1	WLAN RSSI	979

9.7.2	WLAN RSSI Measurement Report Mapping	979

9.8	MBSFN Measurements	980

9.8.1	Introduction	980

9.8.2	MBSFN RSRP	980

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	980

9.8.2.2	MBSFN RSRP measurement report mapping	981

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	981

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	982

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	982

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	982

9.8.3	MBSFN RSRQ	983

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	983

9.8.3.2	MBSFN RSRQ measurement report mapping	983

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	984

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	984

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	984

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	985

9.8.4	MCH BLER	985

9.8.4.1	Measurement report mapping for MCH BLER	985

9.8.4.2	Measurement report mapping for MCH Block Number	986

9.9	ProSe Measurements	987

9.9.1	Introduction	987

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	987

9.9.2.1	Absolute S-RSRP Accuracy	987

9.9.2.2	Relative Accuracy of S-RSRP	988

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	989

9.9.3.1	Absolute SD-RSRP Accuracy	989

9.9.3.2	Relative Accuracy of SD-RSRP	989

9.10	V2X Measurements	990

9.10.1	Introduction	990

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	990

9.10.2.1	Absolute S-RSRP Accuracy	990

9.10.2.2	Relative Accuracy of S-RSRP	991

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	991

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	991

9.10.4	S-RSSI Measurement Accuracy Requirements	992

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	992

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	992

9.11	NR Measurements	993

9.11.1	NR SS-RSRP Measurements	993

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	993

9.11.2	NR SS-RSRQ Measurements	994

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	994

9.11.3	NR SS-SINR Measurements	994

9.11.4	NR SS-RSRP Measurements under CCA	995

9.11.5	NR SS-RSRQ Measurements under CCA	995

9.11.6	NR SS-SINR Measurements under CCA	995

9.11.7	NR RSSI Measurements under CCA	995

9.11.8	NR Channel Occupancy Measurements under CCA	996

10	Measurements Performance Requirements for E-UTRAN	996

10.1	Received Interference Power	996

10.1.1	Absolute accuracy requirement	996

10.1.2	Relative accuracy requirement	996

10.1.3	Received Interference Power measurement report mapping	997

10.2	Angle of Arrival (AOA)	997

10.2.1	Range/mapping	997

10.3	Timing Advance (TADV)	997

10.3.1	Report mapping	997

11	ProSe Requirements in Any Cell Selection state	998

11.1	Introduction	998

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	998

11.2.1	Introduction	998

11.2.2	ProSe UE transmission timing	998

11.3	Initiation/Cease of SLSS Transmissions	999

11.3.1	Introduction	999

11.3.2	Requirements	999

11.4	Measurements for ProSe in Any Cell Selection State	999

11.4.1	Introduction	999

11.4.2	Requirements	999

11.4.2.1	E-UTRA FDD	999

11.4.2.2	E-UTRA TDD	1000

11.5	Selection / Reselection of ProSe Synchronization Reference	1000

11.5.1	Introduction	1000

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	1000

11.5.2.1	Introduction	1000

11.5.2.2	Requirements	1000

11.6	Void	1001

11.7	Selection / Reselection of ProSe relay UE	1001

11.7.1	Introduction	1001

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	1001

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	1001

12.1	Introduction	1001

12.2	Transmit Timing	1002

12.2.1	GNSS as timing reference	1002

12.3	Interruption	1002

12.4	Reliability of GNSS signal	1002

13	V2X Requirements	1002

13.1	Introduction	1002

13.2	UE Transmit Timing	1003

13.2.1	Introduction	1003

13.2.2	GNSS as synchronization reference source	1003

13.2.3	Serving cell/PCell as synchronization reference source	1003

13.2.4	SyncRef UE as synchronization reference source	1003

13.3	Initiation/Cease of SLSS Transmissions	1003

13.3.1	Introduction	1003

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	1003

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1004

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1004

13.4	Selection / Reselection of V2X Synchronization Reference Source	1005

13.5	Autonomous Resource Selection/Reselection measurements	1006

13.5.1	Introduction	1006

13.5.2	PSSCH-RSRP measurements	1006

13.5.3	S-RSSI measurements	1006

13.6	Congestion Control measurements	1006

13.7	Interruption	1006

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	1006

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	1006

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	1007

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	1007

13.8	Reliability of GNSS signal	1007

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	1008

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	1008

Annex A (normative):	Test Cases	1009

A.1	Purpose of annex	1009

A.2	Requirement classification for statistical testing	1009

A.2.1	Types of requirements in TS 36.133	1009

A.2.1.1	Time and delay requirements on UE higher layer actions	1009

A.2.1.2	Measurements of power levels, relative powers and time	1009

A.2.1.3	Implementation requirements	1010

A.2.1.4	Physical layer timing requirements	1010

A.3	RRM test configurations	1011

A.3.1	Reference Measurement Channels	1011

A.3.1.1	PDSCH	1011

A.3.1.1.1	FDD	1011

A.3.1.1.2	TDD	1016

A.3.1.1.3	FDD for UE category 0	1019

A.3.1.1.4	HD-FDD for UE category 0	1020

A.3.1.1.5	TDD for UE category 0	1021

A.3.1.1.6	Frame Structure 3	1022

A.3.1.2	PCFICH/PDCCH/PHICH	1023

A.3.1.2.1	FDD	1023

A.3.1.2.2	TDD	1023

A.3.1.2.3	HD-FDD for UE category 0	1024

A.3.1.2.4	FS 3	1024

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	1024

A.3.1.3.1	FDD in CEModeA	1025

A.3.1.3.2	HD-FDD in CEModeA	1025

A.3.1.3.3	TDD in CEModeA	1026

A.3.1.3.4	FDD in CEModeB	1026

A.3.1.3.5	HD-FDD in CEModeB	1027

A.3.1.3.6	TDD in CEModeB	1027

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	1028

A.3.1.4.1	FDD in CEModeA	1028

A.3.1.4.2	HD-FDD in CEModeA	1029

A.3.1.4.3	TDD in CEModeA	1030

A.3.1.4.4	FDD in CEModeB	1031

A.3.1.4.5	HD-FDD in CEModeB	1032

A.3.1.4.6	TDD in CEModeB	1032

A.3.1.5	NPDSCH Reference Channel for UE category NB1	1033

A.3.1.5.1	HD-FDD in-band operation	1033

A.3.1.5.2	Void	1034

A.3.1.5.3	HD-FDD standalone operation	1034

A.3.1.5.4	Void	1035

A.3.1.5.5	HD-FDD guard band operation	1035

A.3.1.5.6	Void	1036

A.3.1.5.7	TDD in-band operation	1036

A.3.1.5.8	TDD standalone operation	1036

A.3.1.5.9	TDD guard band operation	1037

A.3.1.6	NPDCCH Reference Channel for UE category NB1	1037

A.3.1.6.1	In-band operation	1037

A.3.1.6.2	Void	1038

A.3.1.6.3	Standalone operation	1038

A.3.1.6.4	Void	1038

A.3.1.6.5	Guard band operation	1038

A.3.1.6.6	Void	1039

A.3.2	OFDMA Channel Noise Generator (OCNG)	1039

A.3.2.1	OCNG Patterns for FDD	1039

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	1040

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	1040

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	1041

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	1042

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	1042

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	1043

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	1043

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	1043

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	1044

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	1045

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	1045

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	1046

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	1046

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	1047

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	1047

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	1048

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	1048

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	1049

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	1049

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	1050

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	1050

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	1051

A.3.2.2	OCNG Patterns for TDD	1051

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1052

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	1052

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	1053

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	1053

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	1054

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	1055

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	1056

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	1057

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	1057

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	1058

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	1059

A.3.2.3	OCNG Patterns for Narrowband IoT	1059

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1061

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1062

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	1062

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	1063

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	1064

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1065

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1066

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	1067

A.3.2.4	OCNG Patterns for V2X sidelink	1068

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1068

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	1069

A.3.3	Reference DRX Configurations	1069

A.3.4	ABS Transmission Configurations	1070

A.3.4.1	Non-MBSFN ABS Transmission Configurations	1070

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	1070

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	1070

A.3.4.2	MBSFN ABS Transmission Configurations	1071

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	1071

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	1072

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	1072

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	1072

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	1073

A.3.6.1	Introduction	1073

A.3.7	Test Cases with Different Channel Bandwidths	1073

A.3.7.1	Introduction	1073

A.3.7.2	Principle of testing	1073

A.3.8	Antenna Configuration	1073

A.3.8.1	Antenna connection for 4 Rx capable UEs	1073

A.3.8.1.1 Introduction	1073

A.3.8.1.2 Principle of testing	1073

A.3.8.1.2.1 Single carrier tests	1073

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	1074

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	1075

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	1075

A.3.8.2	Antenna connection for 8 Rx capable UEs	1075

A.3.8.2.1	Introduction	1075

A.3.8.2.2	Principle of testing	1075

A.3.8.2.2.1	Single carrier tests	1075

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	1076

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	1076

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	1076

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	1076

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	1076

A.3.9.1	Introduction	1076

A.3.9.2	Principle of testing	1076

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	1077

A.3.10.1	Introduction	1077

A.3.10.2	Principle of testing	1077

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	1077

A.3.11.1	Introduction	1077

A.3.11.2	Principle of Testing	1077

A.3.12	Proximity-based Services	1077

A.3.12.1	Introduction	1077

A.3.12.2	Reference DRX configurations for ProSe tests	1077

A.3.12.3	Test Cases with Different Channel Bandwidths	1078

A.3.12.3.1	Introduction	1078

A.3.12.3.2	Principle of testing	1078

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	1078

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	1085

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	1088

A.3.12.6.1	FDD	1088

A.3.12.7	Reference measurement channels for ProSe Direct Communication	1088

A.3.12.7.1	FDD	1088

A.3.12.8	ProSe Receive Traffic Generator	1089

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	1089

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	1089

A.3.13	Time Offset between Cells	1090

A.3.13.1	Introduction	1090

A.3.13.2	Definition	1090

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	1090

A.3.14.1	Introduction	1090

A.3.14.2	Principle of testing	1090

A.3.15	Dual connectivity test cases with different combination of duplex mode	1090

A.3.15.1	Introduction	1090

A.3.15.2	Principle of testing	1090

A.3.16	Reference PRACH Configurations	1091

A.3.17	Listen before talk model	1091

A.3.17.1	Introduction	1091

A.3.17.2	Definition	1091

A.3.18	Reference NPRACH Configurations	1092

A.3.19	Dual connectivity test cases with different bandwidth combinations	1093

A.3.19.1	Introduction	1093

A.3.19.2	Principle of testing	1093

A.3.20	Category M1 UE Test Cases	1094

A.3.20.1	Introduction	1094

A.3.20.2	Principle of Cat-M1 UE Testing	1094

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1095

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	1096

A.3.21.1	Introduction	1096

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	1096

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	1097

A.3.22	Category 1bis UE Test Cases	1098

A.3.22.1	Introduction	1098

A.3.22.2	Principle of Category 1bis UE Testing	1098

A.3.23	Category NB2 UE Test Cases	1102

A.3.23.1	Introduction	1102

A.3.23.2	Principle of Category NB2 UE Testing	1102

A.3.24	V2X sidelink communication	1105

A.3.24.1	Introduction	1105

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	1106

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	1110

A.3.25	Category M2 UE Test Cases	1111

A.3.25.1	Introduction	1111

A.3.25.2	Principle of Cat-M2 UE Testing	1111

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1112

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	1113

A.3.26.1	Introduction	1113

A.3.26.2	Principle of testing	1113

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	1113

A.3.27.1	Introduction	1113

A.3.27.2	Principle of INACTIVE cell re-selection Testing	1113

A.3.28	Testing related to Satellite access	1113

A.3.28.1	Introduction	1113

A.3.28.2	Principle of testing GSO and NGSO scenarios	1113

A.3.28.2	Principle of testing different RRM requirements	1114

A.3.28.3	Principle of testing different ephemeris formats	1118

A.3.28.4	General setup for SIB31/SIB-31-NB	1118

A.3.28.5	Satellite specific parameters configuration	1118

A.3.28.5.1	Satellite specific configuration for serving cell	1118

A.3.28.5.2	Satellite specific configuration for neighbour cell	1118

A.4	E-UTRAN RRC_IDLE state	1119

A.4.2	Cell Re-Selection	1119

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	1119

A.4.2.1.1	Test Purpose and Environment	1119

A.4.2.1.2	Test Requirements	1121

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	1122

A.4.2.2.1	Test Purpose and Environment	1122

A.4.2.2.2	Test Requirements	1123

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	1124

A.4.2.3.1	Test Purpose and Environment	1124

A.4.2.3.2	Test Requirements	1125

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	1126

A.4.2.4.1	Test Purpose and Environment	1126

A.4.2.4.2	Test Requirements	1127

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	1128

A.4.2.5.1	Test Purpose and Environment	1128

A.4.2.5.2	Test Requirements	1129

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	1130

A.4.2.6.1	Test Purpose and Environment	1130

A.4.2.6.2	Test Requirements	1131

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	1132

A.4.2.7.1	Test Purpose and Environment	1132

A.4.2.7.2	Test Requirements	1133

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	1134

A.4.2.8.1	Test Purpose and Environment	1134

A.4.2.8.2	Test Requirements	1135

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	1136

A.4.2.9.1	Test Purpose and Environment	1136

A.4.2.9.2	Test Requirements	1136

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	1136

A.4.2.10.1	Test Purpose and Environment	1136

A.4.2.10.2	Test Requirements	1140

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	1140

A.4.2.11.1	Test Purpose and Environment	1140

A.4.2.11.2	Test Requirements	1144

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1144

A.4.2.12.1	Test Purpose and Environment	1144

A.4.2.12.2	Test Requirements	1146

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1147

A.4.2.13.1	Test Purpose and Environment	1147

A.4.2.13.2	Test Requirements	1148

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1149

A.4.2.14.1	Test Purpose and Environment	1149

A.4.2.14.2	Test Requirements	1150

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1151

A.4.2.15.1	Test Purpose and Environment	1151

A.4.2.15.2	Test Requirements	1152

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1153

A.4.2.16.1	Test Purpose and Environment	1153

A.4.2.16.2	Test Requirements	1154

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	1155

A.4.2.17.1	Test Purpose and Environment	1155

A.4.2.17.2	Test Requirements	1156

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1157

A.4.2.18.1	Test Purpose and Environment	1157

A.4.2.18.2	Test Requirements	1159

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1160

A.4.2.19.1	Test Purpose and Environment	1160

A.4.2.19.2	Test Requirements	1162

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	1163

A.4.2.20.1	Test Purpose and Environment	1163

A.4.2.20.2	Test Requirements	1164

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	1165

A.4.2.21.1	Test Purpose and Environment	1165

A.4.2.21.2	Test Requirements	1166

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1167

A.4.2.22.1	Test Purpose and Environment	1167

A.4.2.22.2	Test Requirements	1168

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1169

A.4.2.23.1	Test Purpose and Environment	1169

A.4.2.23.2	Test Requirements	1170

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1171

A.4.2.24.1	Test Purpose and Environment	1171

A.4.2.24.2	Test Requirements	1174

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1175

A.4.2.25.1	Test Purpose and Environment	1175

A.4.2.25.2	Test Requirements	1176

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1177

A.4.2.26.1	Test Purpose and Environment	1177

A.4.2.26.2	Test Requirements	1178

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1179

A.4.2.27.1	Test Purpose and Environment	1179

A.4.2.27.2	Test Requirements	1180

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1181

A.4.2.28.1	Test Purpose and Environment	1181

A.4.2.28.2	Test Requirements	1182

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1183

A.4.2.29.1	Test Purpose and Environment	1183

A.4.2.29.2	Test Requirements	1184

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	1185

A.4.2.30.1	Test Purpose and Environment	1185

A.4.2.30.2	Test Requirements	1186

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	1187

A.4.2.31.1	Test Purpose and Environment	1187

A.4.2.31.2	Test Requirements	1188

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	1189

A.4.2.32.1	Test Purpose and Environment	1189

A.4.2.32.2	Test Requirements	1190

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	1191

A.4.2.33.1	Test Purpose and Environment	1191

A.4.2.33.2	Test Requirements	1192

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	1193

A.4.2.34.1	Test Purpose and Environment	1193

A.4.2.34.2	Test Requirements	1194

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1195

A.4.2.35.1	Test Purpose and Environment	1195

A.4.2.35.2	Test Requirements	1197

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1198

A.4.2.36.1	Test Purpose and Environment	1198

A.4.2.36.2	Test Requirements	1200

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1201

A.4.2.37.1	Test Purpose and Environment	1201

A.4.2.37.2	Test Requirements	1203

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1204

A.4.2.38.1	Test Purpose and Environment	1204

A.4.2.38.2	Test Requirements	1206

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1207

A.4.2.39.1	Test Purpose and Environment	1207

A.4.2.39.2	Test Requirements	1208

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1209

A.4.2.40.1	Test Purpose and Environment	1209

A.4.2.40.2	Test Requirements	1210

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1211

A.4.2.41.1	Test Purpose and Environment	1211

A.4.2.41.2	Test Requirements	1214

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1215

A.4.2.42.1	Test Purpose and Environment	1215

A.4.2.42.2	Test Requirements	1217

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1218

A.4.2.43.1	Test Purpose and Environment	1218

A.4.2.43.2	Test Requirements	1220

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1221

A.4.2.44.1	Test Purpose and Environment	1221

A.4.2.44.2	Test Requirements	1223

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1224

A.4.2.45.1	Test Purpose and Environment	1224

A.4.2.45.2	Test Requirements	1226

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1227

A.4.2.46.1	Test Purpose and Environment	1227

A.4.2.46.2	Test Requirements	1229

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1230

A.4.2.47.1	Test Purpose and Environment	1230

A.4.2.47.2	Test Requirements	1232

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1233

A.4.2.48.1	Test Purpose and Environment	1233

A.4.2.48.2	Test Requirements	1234

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1235

A.4.2.49.1	Test Purpose and Environment	1235

A.4.2.49.2	Test Requirements	1237

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1238

A.4.2.50.1	Test Purpose and Environment	1238

A.4.2.50.2	Test Requirements	1240

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1241

A.4.2.51.1	Test Purpose and Environment	1241

A.4.2.51.2	Test Requirements	1243

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1244

A.4.2.52.1	Test Purpose and Environment	1244

A.4.2.52.2	Test Requirements	1246

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1247

A.4.2.53.1	Test Purpose and Environment	1247

A.4.2.53.2	Test Requirements	1248

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1249

A.4.2.54.1	Test Purpose and Environment	1249

A.4.2.54.2	Test Requirements	1251

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1252

A.4.2.55.1	Test Purpose and Environment	1252

A.4.2.55.2	Test Requirements	1254

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1255

A.4.2.56.1	Test Purpose and Environment	1255

A.4.2.56.2	Test Requirements	1256

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1257

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1257

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1257

A.4.3.1.1.1	Test Purpose and Environment	1257

A.4.3.1.1.2	Test Requirements	1259

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1259

A.4.3.1.2.1	Test Purpose and Environment	1259

A.4.3.1.2.2	Test Requirements	1262

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1262

A.4.3.1.3.1	Test Purpose and Environment	1262

A.4.3.1.3.2	Test Requirements	1265

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1265

A.4.3.1.4.1	Test Purpose and Environment	1265

A.4.3.1.4.2	Test Requirements	1266

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1266

A.4.3.1.5.1	Test Purpose and Environment	1266

A.4.3.1.5.2	Test Requirements	1269

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1270

A.4.3.2.1	Test Purpose and Environment	1270

A.4.3.2.1.1	Void	1270

A.4.3.2.1.2	1.28Mcps TDD option	1270

A.4.3.2.1.3	Void	1272

A.4.3.2.2	Test Requirements	1272

A.4.3.2.2.1	1.28Mcps TDD option	1272

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1272

A.4.3.2A.1	Test Purpose and Environment	1272

A.4.3.2A.2	Test Requirements	1276

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1276

A.4.3.3.1	Test Purpose and Environment	1276

A.4.3.3.2	Test Requirements	1279

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1279

A.4.3.3A.1	Test Purpose and Environment	1279

A.4.3.3A.2	Test Requirements	1284

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1285

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1285

A.4.3.4.1.1	Test Purpose and Environment	1285

A.4.3.4.1.2	Test Requirements	1287

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1287

A.4.3.4.2.1	Test Purpose and Environment	1287

A.4.3.4.2.2	Test Requirements	1289

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1289

A.4.3.4.3.1	Test Purpose and Environment	1289

A.4.3.4.3.2	Test Requirements	1292

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1292

A.4.3.4.4.1	Test Purpose and Environment	1292

A.4.3.4.4.2	Test Requirements	1296

A.4.4	E-UTRAN to GSM Cell Re-Selection	1296

A.4.4.1	E-UTRAN FDD – GSM:	1296

A.4.4.1.1	Test Purpose and Environment	1296

A.4.4.1.2	Test Requirements	1298

A.4.4.2	E-UTRAN TDD – GSM:	1298

A.4.4.2.1	Test Purpose and Environment	1298

A.4.4.2.2	Test Requirements	1300

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1301

A.4.5.1	E-UTRAN FDD – HRPD	1301

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1301

A.4.5.1.1.1	Test Purpose and Environment	1301

A.4.5.1.1.2	Test Requirements	1303

A.4.5.2	E-UTRAN TDD – HRPD	1303

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1303

A.4.5.2.1.1	Test Purpose and Environment	1303

A.4.5.2.1.2	Test Requirements	1306

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1306

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1306

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1306

A.4.6.1.1.1	Test Purpose and Environment	1306

A.4.6.1.1.2	Test Requirements	1309

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1309

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1309

A.4.6.2.1.1	Test Purpose and Environment	1309

A.4.6.2.1.2	Test Requirements	1312

A.4.7	Idle State Positioning Measurement for UE category NB1	1312

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1312

A.4.7.1.1	Test Purpose and Environment	1312

A.4.7.1.2	Test Requirements	1317

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1318

A.4.7.2.1	Test Purpose and Environment	1318

A.4.7.4.2	Test Requirements	1322

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1323

A.4.7.3.1	Test Purpose and Environment	1323

A.4.7.3.2	Test Requirements	1327

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1328

A.4.7.4.1	Test Purpose and Environment	1328

A.4.7.4.2	Test Requirements	1332

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1333

A.5.1	E-UTRAN Handover	1333

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1333

A.5.1.1.1	Test Purpose and Environment	1333

A.5.1.1.2	Test Requirements	1335

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1335

A.5.1.2.1	Test Purpose and Environment	1335

A.5.1.2.2	Test Requirements	1337

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1337

A.5.1.3.1	Test Purpose and Environment	1337

A.5.1.3.2	Test Requirements	1339

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1339

A.5.1.4.1	Test Purpose and Environment	1339

A.5.1.4.2	Test Requirements	1341

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1341

A.5.1.5.1	Test Purpose and Environment	1341

A.5.1.5.2	Test Requirements	1343

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1343

A.5.1.6.1	Test Purpose and Environment	1343

A.5.1.6.2	Test Requirements	1345

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1345

A.5.1.7.1	Test Purpose and Environment	1345

A.5.1.7.2	Test Requirements	1348

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1348

A.5.1.8.1	Test Purpose and Environment	1348

A.5.1.8.2	Test Requirements	1351

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1351

A.5.1.9.1	Test Purpose and Environment	1351

A.5.1.9.2	Test Requirements	1352

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1352

A.5.1.10.1	Test Purpose and Environment	1352

A.5.1.10.2	Test Requirements	1354

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1354

A.5.1.11.1	Test Purpose and Environment	1354

A.5.1.11.2	Test Requirements	1356

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1356

A.5.1.12.1	Test Purpose and Environment	1356

A.5.1.12.2	Test Requirements	1358

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1358

A.5.1.13.1	Test Purpose and Environment	1358

A.5.1.13.2	Test Requirements	1360

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1361

A.5.1.14.1	Test Purpose and Environment	1361

A.5.1.14.2	Test Requirements	1362

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1363

A.5.1.15.1	Test Purpose and Environment	1363

A.5.1.15.2	Test Requirements	1364

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1365

A.5.1.16.1	Test Purpose and Environment	1365

A.5.1.16.2	Test Requirements	1366

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1367

A.5.1.17.1	Test Purpose and Environment	1367

A.5.1.17.2	Test Requirements	1368

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1369

A.5.1.18.1	Test Purpose and Environment	1369

A.5.1.18.2	Test Requirements	1370

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1371

A.5.1.19.1	Test Purpose and Environment	1371

A.5.1.19.2	Test Requirements	1372

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1373

A.5.1.20.1	Test Purpose and Environment	1373

A.5.1.20.2	Test Requirements	1374

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1375

A.5.1.21.1	Test Purpose and Environment	1375

A.5.1.21.2	Test Requirements	1376

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1376

A.5.1.22.1	Test Purpose and Environment	1376

A.5.1.22.2	Test Requirements	1378

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1378

A.5.1.23.1	Test Purpose and Environment	1378

A.5.1.23.2	Test Requirements	1380

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1380

A.5.1.24.1	Test Purpose and Environment	1380

A.5.1.24.2	Test Requirements	1382

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1382

A.5.1.25.1	Test Purpose and Environment	1382

A.5.1.25.2	Test Requirements	1384

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1385

A.5.1.26.1	Test Purpose and Environment	1385

A.5.1.26.2	Test Requirements	1386

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1387

A.5.1.27.1	Test Purpose and Environment	1387

A.5.1.27.2	Test Requirements	1388

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1389

A.5.1.28.1	Test Purpose and Environment	1389

A.5.1.28.2	Test Requirements	1390

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1391

A.5.1.29.1	Test Purpose and Environment	1391

A.5.1.29.2	Test Requirements	1392

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1393

A.5.1.30.1	Test Purpose and Environment	1393

A.5.1.30.2	Test Requirements	1394

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1395

A.5.1.31.1	Test Purpose and Environment	1395

A.5.1.31.2	Test Requirements	1396

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1397

A.5.1.32.1	Test Purpose and Environment	1397

A.5.1.32.2	Test Requirements	1398

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1399

A.5.1.33.1	Test Purpose and Environment	1399

A.5.1.13.2	Test Requirements	1400

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1401

A.5.1.34.1	Test Purpose and Environment	1401

A.5.1.34.2	Test Requirements	1402

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1403

A.5.1.35.1	Test Purpose and Environment	1403

A.5.1.35.2	Test Requirements	1404

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1405

A.5.1.36.1	Test Purpose and Environment	1405

A.5.1.36.2	Test Requirements	1406

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1407

A.5.1.37.1	Test Purpose and Environment	1407

A.5.1.37.2	Test Requirements	1408

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1409

A.5.1.38.1	Test Purpose and Environment	1409

A.5.1.38.2	Test Requirements	1410

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1411

A.5.1.39.1	Test Purpose and Environment	1411

A.5.1.39.2	Test Requirements	1412

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1413

A.5.1.40.1	Test Purpose and Environment	1413

A.5.1.40.2	Test Requirements	1414

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1415

A.5.1.41.1	Test Purpose and Environment	1415

A.5.1.41.2	Test Requirements	1417

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1418

A.5.1.42.1	Test Purpose and Environment	1418

A.5.1.42.2	Test Requirements	1419

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1420

A.5.1.43.1	Test Purpose and Environment	1420

A.5.1.43.2	Test Requirements	1421

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1422

A.5.1.44.1	Test Purpose and Environment	1422

A.5.1.44.2	Test Requirements	1423

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1424

A.5.1.45.1	Test Purpose and Environment	1424

A.5.1.45.1	Test Requirements	1425

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1426

A.5.1.46.1	Test Purpose and Environment	1426

A.5.1.46.2	Test Requirements	1427

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1428

A.5.1.47.1	Test Purpose and Environment	1428

A.5.1.47.2	Test Requirements	1429

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1430

A.5.1.48.1	Test Purpose and Environment	1430

A.5.1.48.2	Test Requirements	1431

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1432

A.5.1.49.1	Test Purpose and Environment	1432

A.5.1.49.2	Test Requirements	1433

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1434

A.5.1.50.1	Test Purpose and Environment	1434

A.5.1.50.2	Test Requirements	1435

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1436

A.5.1.51.1	Test Purpose and Environment	1436

A.5.1.51.2	Test Requirements	1438

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1438

A.5.152.1	Test Purpose and Environment	1438

A.5.1.52.2	Test Requirements	1441

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1442

A.5.1.53.1	Test Purpose and Environment	1442

A.5.1.53.2	Test Requirements	1444

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1445

A.5.1.54.1	Test Purpose and Environment	1445

A.5.1.54.2	Test Requirements	1447

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1448

A.5.1.55.1	Test Purpose and Environment	1448

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1452

A.5.1.56.1	Test Purpose and Environment	1452

A.5.1.56.2	Test Requirements	1455

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1456

A.5.1.57.1	Test Purpose and Environment	1456

A.5.1.57.2	Test Requirements	1458

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1458

A.5.1.58.1	Test Purpose and Environment	1458

A.5.1.58.2	Test Requirements	1461

A.5.2	E-UTRAN Handover to other RATs	1461

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1461

A.5.2.1.1	Test Purpose and Environment	1461

A.5.2.1.2	Test Requirements	1463

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1464

A.5.2.2.1	Test Purpose and Environment	1464

A.5.2.2.2	Test Requirements	1467

A.5.2.3 E-UTRAN FDD- GSM Handover	1467

A.5.2.3.1	Test Purpose and Environment	1467

A.5.2.3.2	Test Requirements	1468

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1469

A.5.2.4.1	Test Purpose and Environment	1469

A.5.2.4.1.1	Void	1469

A.5.2.4.1.2	1.28 Mcps TDD option	1469

A.5.2.4.1.3	Void	1471

A.5.2.4.2	Test Requirements	1471

A.5.2.4.2.1	Void	1471

A.5.2.4.2.2	1.28 Mcps TDD option	1471

A.5.2.4.2.3	Void	1471

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1471

A.5.2.5.1	Test Purpose and Environment	1471

A.5.2.5.1.3	Void	1474

A.5.2.5.2	Test Requirements	1474

A.5.2.5.2.1	Void	1474

A.5.2.5.2.2	1.28 Mcps TDD option	1474

A.5.2.5.2.3	Void	1474

A.5.2.6	E-UTRAN TDD - GSM Handover	1474

A.5.2.6.1	Test Purpose and Environment	1474

A.5.2.6.2	Test Requirements	1476

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1477

A.5.2.7.1	Test Purpose and Environment	1477

A.5.2.7.2	Test Requirements	1479

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1479

A.5.2.8.1	Test Purpose and Environment	1479

A.5.2.8.2	Test Requirements	1480

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1481

A.5.2.9.1	Test Purpose and Environment	1481

A.5.2.9.2	Test Requirements	1482

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1483

A.5.2.10.1	Test Purpose and Environment	1483

A.5.2.10.2	Test Requirements	1485

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1485

A.5.2.10A.1	Test Purpose and Environment	1485

A.5.2.10A.2	Test Requirements	1488

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1488

A.5.2.10B.1	Test Purpose and Environment	1488

A.5.2.10B.2	Test Requirements	1491

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1491

A.5.2.11.1	Test Purpose and Environment	1491

A.5.2.11.2	Test Requirements	1491

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1492

A.5.3.1	E-UTRAN FDD – HRPD Handover	1492

A.5.3.1.1	Test Purpose and Environment	1492

A.5.3.1.2	Test Requirements	1494

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1494

A.5.3.2.1	Test Purpose and Environment	1494

A.5.3.2.2	Test Requirements	1497

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1497

A.5.3.3.1	Test Purpose and Environment	1497

A.5.3.3.2	Test Requirements	1500

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1500

A.5.3.4.1	Test Purpose and Environment	1500

A.5.3.4.2	Test Requirements	1502

A.5.3.5	E-UTRAN TDD – HRPD Handover	1502

A.5.3.5.1	Test Purpose and Environment	1502

A.5.3.5.2	Test Requirements	1505

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1505

A.5.3.6.1	Test Purpose and Environment	1505

A.5.3.6.2	Test Requirements	1508

A.6	RRC Connection Control	1508

A.6.1	RRC Re-establishment	1508

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1508

A.6.1.1.1	Test Purpose and Environment	1508

A.6.1.1.2	Test Requirements	1509

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1510

A.6.1.2.1	Test Purpose and Environment	1510

A.6.1.2.2	Test Requirements	1511

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1512

A.6.1.3.1	Test Purpose and Environment	1512

A.6.1.3.2	Test Requirements	1513

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1514

A.6.1.4.1	Test Purpose and Environment	1514

A.6.1.4.2	Test Requirements	1515

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1516

A.6.1.5.1	Test Purpose and Environment	1516

A.6.1.5.2	Test Requirements	1516

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1516

A.6.1.6.1	Test Purpose and Environment	1516

A.6.1.6.2	Test Requirements	1518

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1519

A.6.1.7.1	Test Purpose and Environment	1519

A.6.1.7.2	Test Requirements	1520

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1521

A.6.1.8.1	Test Purpose and Environment	1521

A.6.1.8.2	Test Requirements	1522

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1523

A.6.1.9.1	Test Purpose and Environment	1523

A.6.1.9.2	Test Requirements	1524

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1525

A.6.1.10.1	Test Purpose and Environment	1525

A.6.1.10.2	Test Requirements	1526

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1527

A.6.1.11.1	Test Purpose and Environment	1527

A.6.1.11.2	Test Requirements	1528

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1529

A.6.1.12.1	Test Purpose and Environment	1529

A.6.1.12.2	Test Requirements	1530

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1531

A.6.1.13.1	Test Purpose and Environment	1531

A.6.1.13.2	Test Requirements	1532

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1533

A.6.1.14.1	Test Purpose and Environment	1533

A.6.1.14.2	Test Requirements	1534

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1535

A.6.1.15.1	Test Purpose and Environment	1535

A.6.1.15.2	Test Requirements	1537

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1538

A.6.1.16.1	Test Purpose and Environment	1538

A.6.1.16.2	Test Requirements	1540

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1541

A.6.1.17.1	Test Purpose and Environment	1541

A.6.1.17.2	Test Requirements	1542

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1543

A.6.1.18.1	Test Purpose and Environment	1543

A.6.1.18.2	Test Requirements	1544

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1545

A.6.1.19.1	Test Purpose and Environment	1545

A.6.1.19.2	Test Requirements	1546

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1547

A.6.1.20.1	Test Purpose and Environment	1547

A.6.1.20.2	Test Requirements	1548

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1549

A.6.1.21.1	Test Purpose and Environment	1549

A.6.1.21.2	Test Requirements	1550

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1551

A.6.1.22.1	Test Purpose and Environment	1551

A.6.1.22.2	Test Requirements	1552

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1553

A.6.1.23.1	Test Purpose and Environment	1553

A.6.1.23.2	Test Requirements	1555

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1556

A.6.1.24.1	Test Purpose and Environment	1556

A.6.1.24.2	Test Requirements	1558

A.6.2	Random Access	1559

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1559

A.6.2.1.1	Test Purpose and Environment	1559

A.6.2.1.2	Test Requirements	1560

A.6.2.1.2.1	Random Access Response Reception	1560

A.6.2.1.2.2	No Random Access Response Reception	1560

A.6.2.1.2.3	Receiving a NACK on msg3	1560

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1561

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1561

A.6.2.1.2.6	Contention Resolution Timer expiry	1561

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1561

A.6.2.2.1	Test Purpose and Environment	1561

A.6.2.2.2	Test Requirements	1562

A.6.2.2.2.1	Random Access Response Reception	1563

A.6.2.2.2.2	No Random Access Response Reception	1563

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1563

A.6.2.3.1	Test Purpose and Environment	1563

A.6.2.3.2	Test Requirements	1565

A.6.2.3.2.1	Random Access Response Reception	1565

A.6.2.3.2.2	No Random Access Response reception	1565

A.6.2.3.2.3	Receiving a NACK on msg3	1565

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1566

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1566

A.6.2.3.2.6	Contention Resolution Timer expiry	1566

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1566

A.6.2.4.1	Test Purpose and Environment	1566

A.6.2.4.2	Test Requirements	1568

A.6.2.4.2.1	Random Access Response Reception	1568

A.6.2.4.2.2	No Random Access Response Reception	1568

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1568

A.6.2.5.1	Test Purpose and Environment	1568

A.6.2.5.2	Test Requirements	1569

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1569

A.6.2.6.1	Test Purpose and Environment	1569

A.6.2.6.2	Test Requirements	1569

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1570

A.6.2.7.1	Test Purpose and Environment	1570

A.6.2.7.2	Test Requirements	1572

A.6.2.7.2.1	Random Access Response Reception	1572

A.6.2.7.2.2	No Random Access Response Reception	1572

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1572

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1573

A.6.2.8.1	Test Purpose and Environment	1573

A.6.2.8.2	Test Requirements	1575

A.6.2.8.2.1	Random Access Response Reception	1575

A.6.2.8.2.2	No Random Access Response Reception	1575

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1575

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1576

A.6.2.9.1	Test Purpose and Environment	1576

A.6.2.9.2	Test Requirements	1579

A.6.2.9.2.1	Random Access Response Reception	1579

A.6.2.9.2.2	No Random Access Response Reception	1580

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1580

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1581

A.6.2.10.1	Test Purpose and Environment	1581

A.6.2.10.2	Test Requirements	1583

A.6.2.10.2.1	Random Access Response Reception	1583

A.6.2.10.2.2	No Random Access Response Reception	1584

A.6.2.10.2.3	Receiving a NACK on msg3	1584

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1584

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1584

A.6.2.10.2.6	Contention Resolution Timer expiry	1584

A.6.2.10.2.7	PRACH Resource Selection	1585

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1585

A.6.2.11.1	Test Purpose and Environment	1585

A.6.2.11.2	Test Requirements	1587

A.6.2.11.2.1	Random Access Response Reception	1587

A.6.2.11.2.2	No Random Access Response Reception	1588

A.6.2.11.2.3	Receiving a NACK on msg3	1588

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1588

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1588

A.6.2.11.2.6	Contention Resolution Timer expiry	1588

A.6.2.11.2.7	PRACH Resource Selection	1589

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1589

A.6.2.12.1	Test Purpose and Environment	1589

A.6.2.12.2	Test Requirements	1591

A.6.2.12.2.1	Random Access Response Reception	1591

A.6.2.12.2.2	No Random Access Response Reception	1592

A.6.2.12.2.3	Receiving a NACK on msg3	1592

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1592

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1592

A.6.2.12.2.6	Contention Resolution Timer expiry	1592

A.6.2.12.2.7	PRACH Resource Selection	1593

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1593

A.6.2.13.1	Test Purpose and Environment	1593

A.6.2.13.2	Test Requirements	1595

A.6.2.13.2.1	Random Access Response Reception	1595

A.6.2.13.2.2	No Random Access Response Reception	1596

A.6.2.13.2.3	Receiving a NACK on msg3	1596

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1596

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1596

A.6.2.13.2.6	Contention Resolution Timer expiry	1596

A.6.2.13.2.7	PRACH Resource Selection	1597

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1597

A.6.2.14.1	Test Purpose and Environment	1597

A.6.2.14.2	Test Requirements	1599

A.6.2.14.2.1	Random Access Response Reception	1599

A.6.2.14.2.2	No Random Access Response Reception	1600

A.6.2.14.2.3	Receiving a NACK on msg3	1600

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1600

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1600

A.6.2.14.2.6	Contention Resolution Timer expiry	1600

A.6.2.14.2.7	PRACH Resource Selection	1601

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1601

A.6.2.15.1	Test Purpose and Environment	1601

A.6.2.15.2	Test Requirements	1603

A.6.2.15.2.1	Random Access Response Reception	1603

A.6.2.15.2.2	No Random Access Response Reception	1604

A.6.2.15.2.3	Receiving a NACK on msg3	1604

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1604

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1604

A.6.2.15.2.6	Contention Resolution Timer expiry	1604

A.6.2.15.2.7	PRACH Resource Selection	1605

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1605

A.6.2.16.1	Test Purpose and Environment	1605

A.6.2.16.2	Test Requirements	1608

A.6.2.16.2.1	Random Access Response Reception	1608

A.6.2.16.2.2	No Random Access Response Reception	1609

A.6.2.16.2.3	Receiving a NACK on msg3	1609

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1609

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1609

A.6.2.16.2.6	Contention Resolution Timer expiry	1609

A.6.2.16.2.7	NPRACH Resource Selection	1609

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1610

A.6.2.17.1	Test Purpose and Environment	1610

A.6.2.17.2	Test Requirements	1613

A.6.2.17.2.1	Random Access Response Reception	1613

A.6.2.17.2.2	No Random Access Response Reception	1614

A.6.2.17.2.3	Receiving a NACK on msg3	1614

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1614

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1614

A.6.2.17.2.6	Contention Resolution Timer expiry	1614

A.6.2.17.2.7	NPRACH Resource Selection	1614

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1615

A.6.2.18.1	Test Purpose and Environment	1615

A.6.2.18.2	Test Requirements	1618

A.6.2.18.2.1	Random Access Response Reception	1618

A.6.2.18.2.2	No Random Access Response Reception	1619

A.6.2.18.2.3	Receiving a NACK on msg3	1619

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1619

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1619

A.6.2.18.2.6	Contention Resolution Timer expiry	1619

A.6.2.18.2.7	NPRACH Resource Selection	1619

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1620

A.6.2.19.1	Test Purpose and Environment	1620

A.6.2.19.2	Test Requirements	1622

A.6.2.19.2.1	Random Access Response Reception	1622

A.6.2.19.2.2	No Random Access Response Reception	1623

A.6.2.19.2.3	Receiving a NACK on msg3	1623

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1623

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1623

A.6.2.19.2.6	Contention Resolution Timer expiry	1623

A.6.2.19.2.7	NPRACH Resource Selection	1623

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1624

A.6.2.20.1	Test Purpose and Environment	1624

A.6.2.20.2	Test Requirements	1626

A.6.2.20.2.1	Random Access Response Reception	1626

A.6.2.20.2.2	No Random Access Response Reception	1627

A.6.2.20.2.3	Receiving a NACK on msg3	1627

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1627

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1627

A.6.2.20.2.6	Contention Resolution Timer expiry	1627

A.6.2.20.2.7	NPRACH Resource Selection	1627

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1628

A.6.2.21.1	Test Purpose and Environment	1628

A.6.2.21.2	Test Requirements	1630

A.6.2.21.2.1	Random Access Response Reception	1630

A.6.2.21.2.2	No Random Access Response Reception	1631

A.6.2.21.2.3	Receiving a NACK on msg3	1631

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1631

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1631

A.6.2.21.2.6	Contention Resolution Timer expiry	1631

A.6.2.21.2.7	NPRACH Resource Selection	1631

A.6.3	RRC Connection Release with Redirection	1632

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1632

A.6.3.1.1	Test Purpose and Environment	1632

A.6.3.1.2	Test Requirements	1634

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1634

A.6.3.2.1	Test Purpose and Environment	1634

A.6.3.2.2	Test Requirements	1636

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1636

A.6.3.3.1	Test Purpose and Environment	1636

A.6.3.3.2	Test Requirements	1637

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1638

A.6.3.4.1	Test Purpose and Environment	1638

A.6.3.4.2	Test Requirements	1639

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1640

A.6.3.5.1	Test Purpose and Environment	1640

A.6.3.5.2	Test Requirements	1642

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1642

A.6.3.6.1	Test Purpose and Environment	1642

A. 6.3.6.2	Test Requirements	1645

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1645

A.6.3.7.1	Test Purpose and Environment	1645

A.6.3.7.2	Test Requirements	1648

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1648

A.6.3.8.1	Test Purpose and Environment	1648

A.6.3.8.2	Test Requirements	1651

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1651

A.6.3.9.1	Test Purpose and Environment	1651

A.6.3.9.2	Test Requirements	1653

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1653

A.6.3.10.1	Test Purpose and Environment	1653

A.6.3.10.2	Test Requirements	1655

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1655

A.6.3.11.1	Test Purpose and Environment	1655

A.6.3.11.2	Test Requirements	1657

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1657

A.6.3.12.1	Test Purpose and Environment	1657

A.6.3.12.2	Test Requirements	1660

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1660

A.6.3.13.1	Test Purpose and Environment	1660

A.6.3.13.2	Test Parameters	1660

A.6.3.13.3	Test Requirements	1664

A.7	Timing and Signalling Characteristics	1665

A.7.1	UE Transmit Timing	1665

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1665

A.7.1.1.1	Test Purpose and Environment	1665

A.7.1.1.2	Test Requirements	1666

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1667

A.7.1.2.1	Test Purpose and Environment	1667

A.7.1.2.2	Test Requirements	1669

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1670

A.7.1.3.1	Test Purpose and Environment	1670

A.7.1.3.2	Test Requirements	1672

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1673

A.7.1.4.1	Test Purpose and Environment	1673

A.7.1.4.2	Test Requirements	1674

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1675

A.7.1.4A.1	Test Purpose and Environment	1675

A.7.1.4A.2	Test Requirements	1675

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1675

A.7.1.5.1	Test Purpose and Environment	1675

A.7.1.5.2	Test Requirements	1676

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1676

A.7.1.6.1	Test Purpose and Environment	1676

A.7.1.6.2	Test Requirements	1678

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1679

A.7.1.7.1	Test Purpose and Environment	1679

A.7.1.7.2	Test Requirements	1680

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1681

A.7.1.7A.1	Test Purpose and Environment	1681

A.7.1.7A.2	Test Requirements	1681

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1681

A.7.1.7B.1	Test Purpose and Environment	1681

A.7.1.7B.2	Test Requirements	1681

A.7.1.8	Void	1682

A.7.1.8.1	Void	1682

A.7.1.8.2	Void	1682

A.7.1.9	Void	1682

A.7.1.9.1	Void	1682

A.7.1.9.2	Void	1682

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1682

A.7.1.10.1	Test Purpose and Environment	1682

A.7.1.10.2	Test Requirements	1684

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1684

A.7.1.11.1	Test Purpose and Environment	1684

A.7.1.11.2	Test Requirements	1686

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1686

A.7.1.12.1	Test Purpose and Environment	1686

A.7.1.12.2	Test Requirements	1688

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1688

A.7.1.13.1	Test Purpose and Environment	1688

A.7.1.13.2	Test Requirements	1690

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1691

A.7.1.14.1	Test Purpose and Environment	1691

A.7.1.14.2	Test Requirements	1692

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1692

A.7.1.15.1	Test Purpose and Environment	1692

A.7.1.15.2	Test Requirements	1693

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1694

A.7.1.16.1	Test Purpose and Environment	1694

A.7.1.16.2	Test Requirements	1695

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1696

A.7.1.17.1	Test Purpose and Environment	1696

A.7.1.17.2	Test Requirements	1698

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1699

A.7.1.18.1	Test Purpose and Environment	1699

A.7.1.18.2	Test Requirements	1701

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1702

A.7.1.19.1	Test Purpose and Environment	1702

A.7.1.19.2	Test Requirements	1703

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1703

A.7.1.20.1	Test Purpose and Environment	1703

A.7.1.20.2	Test Requirements	1705

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1705

A.7.1.21.1	Test Purpose and Environment	1705

A.7.1.21.2	Test Requirements	1707

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1707

A.7.1.22.1	Test Purpose and Environment	1707

A.7.1.22.2	Test Requirements	1709

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1709

A.7.1.23.1	Test Purpose and Environment	1709

A.7.1.23.2	Test Requirements	1711

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1711

A.7.1.24.1	Test Purpose and Environment	1711

A.7.1.24.2	Test Requirements	1712

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1713

A.7.1.25.1	Test Purpose and Environment	1713

A.7.1.25.2	Test Requirements	1714

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1715

A.7.1.26.1	Test Purpose and Environment	1715

A.7.1.26.2	Test Requirements	1716

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1717

A.7.1.27.1	Test Purpose and Environment	1717

A.7.1.27.2	Test Requirements	1719

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1719

A.7.1.28.1	Test Purpose and Environment	1719

A.7.1.28.2	Test Requirements	1721

A.7.2	UE Timing Advance	1722

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1722

A.7.2.1.1	Test Purpose and Environment	1722

A.7.2.1.2	Test Requirements	1723

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1724

A.7.2.2.1	Test Purpose and Environment	1724

A.7.2.2.2	Test Requirements	1726

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1726

A.7.2.3.1	Test Purpose and Environment	1726

A.7.2.3.2	Test Requirements	1726

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1726

A.7.2.4.1	Test Purpose and Environment	1726

A.7.2.4.2	Test Requirements	1729

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1729

A.7.2.5.1	Test Purpose and Environment	1729

A.7.2.5.2	Test Requirements	1731

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1731

A.7.2.5A.1	Test Purpose and Environment	1731

A.7.2.5A.2	Test Requirements	1732

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1732

A.7.2.5B.1	Test Purpose and Environment	1732

A.7.2.5B.2	Test Requirements	1732

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1732

A.7.2.6.1	Test Purpose and Environment	1732

A.7.2.6.2	Test Requirements	1735

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1735

A.7.2.7.1	Test Purpose and Environment	1735

A.7.2.7.2	Test Requirements	1737

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1737

A.7.2.8.1	Test Purpose and Environment	1737

A.7.2.8.2	Test Requirements	1739

A.7.2.9.2	Test Requirements	1740

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1741

A.7.2.10.1	Test Purpose and Environment	1741

A.7.2.10.2	Test Requirements	1742

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1742

A.7.2.11.1	Test Purpose and Environment	1742

A.7.2.11.2	Test Requirements	1744

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1744

A.7.2.12.1	Test Purpose and Environment	1744

A.7.2.12.2	Test Requirements	1746

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1747

A.7.2.13.1	Test Purpose and Environment	1747

A.7.2.13.2	Test Requirements	1748

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1749

A.7.2.14.1	Test Purpose and Environment	1749

A.7.2.14.2	Test Requirements	1751

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1751

A.7.2.15.1	Test Purpose and Environment	1751

A.7.2.15.2	Test Requirements	1753

A.7.3	Radio Link Monitoring	1753

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1753

A.7.3.1.1	Test Purpose and Environment	1753

A.7.3.1.2	Test Requirements	1757

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1757

A.7.3.2.1	Test Purpose and Environment	1757

A.7.3.2.2	Test Requirements	1761

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1761

A.7.3.3.1	Test Purpose and Environment	1761

A.7.3.3.2	Test Requirements	1765

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1765

A.7.3.4.1	Test Purpose and Environment	1765

A.7.3.4.2	Test Requirements	1769

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1769

A.7.3.5.1	Test Purpose and Environment	1769

A.7.3.5.2	Test Requirements	1772

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1773

A.7.3.6.1	Test Purpose and Environment	1773

A.7.3.6.2	Test Requirements	1776

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1776

A.7.3.7.1	Test Purpose and Environment	1776

A.7.3.7.2	Test Requirements	1779

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1779

A.7.3.8.1	Test Purpose and Environment	1779

A.7.3.8.2	Test Requirements	1782

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1782

A.7.3.9.2	Test Requirements	1785

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1785

A.7.3.10.1	Test Purpose and Environment	1785

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1789

A.7.3.11.1	Test Purpose and Environment	1789

A.7.3.11.2	Test Requirements	1794

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1794

A.7.3.12.1	Test Purpose and Environment	1794

A.7.3.12.2	Test Requirements	1799

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1799

A.7.3.13.1	Test Purpose and Environment	1799

A.7.3.13.2	Test Requirements	1802

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1802

A.7.3.14.1	Test Purpose and Environment	1802

A.7.3.14.2	Test Requirements	1805

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1805

A.7.3.15.1	Test Purpose and Environment	1805

A.7.3.15.2	Test Requirements	1809

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1809

A.7.3.16.1	Test Purpose and Environment	1809

A.7.3.16.2	Test Requirements	1814

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1814

A.7.3.17.1	Test Purpose and Environment	1814

A.7.3.17.2	Test Requirements	1818

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1818

A.7.3.18.1	Test Purpose and Environment	1818

A.7.3.18.2	Test Requirements	1822

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1822

A.7.3.19.1	Test Purpose and Environment	1822

A.7.3.19.2	Test Requirements	1827

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1827

A.7.3.20.1	Test Purpose and Environment	1827

A.7.3.20.2	Test Requirements	1832

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1832

A.7.3.21.1	Test Purpose and Environment	1832

A.7.3.21.2	Test Requirements	1837

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1837

A.7.3.22.1	Test Purpose and Environment	1837

A.7.3.22.2	Test Requirements	1842

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1842

A.7.3.23.1	Test Purpose and Environment	1842

A.7.3.23.2	Test Requirements	1843

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1843

A.7.3.24.1	Test Purpose and Environment	1843

A.7.3.24.2	Test Requirements	1844

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1844

A.7.3.25.1	Test Purpose and Environment	1844

A.7.3.25.2	Test Requirements	1845

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1845

A.7.3.26.1	Test Purpose and Environment	1845

A.7.3.26.2	Test Requirements	1848

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1848

A.7.3.27.1	Test Purpose and Environment	1848

A.7.3.27.2	Test Requirements	1851

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1851

A.7.3.28.1	Test Purpose and Environment	1851

A.7.3.28.2	Test Requirements	1854

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1854

A.7.3.29.1	Test Purpose and Environment	1854

A.7.3.29.2	Test Requirements	1857

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1857

A.7.3.30.1	Test Purpose and Environment	1857

A.7.3.30.2	Test Requirements	1860

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1860

A.7.3.31.1	Test Purpose and Environment	1860

A.7.3.31.2	Test Requirements	1863

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1863

A.7.3.32.1	Test Purpose and Environment	1863

A.7.3.32.2	Test Requirements	1866

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1866

A.7.3.33.1	Test Purpose and Environment	1866

A.7.3.33.2	Test Requirements	1869

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1869

A.7.3.34.1	Test Purpose and Environment	1869

A.7.3.34.2	Test Requirements	1872

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1872

A.7.3.35.1	Test Purpose and Environment	1872

A.7.3.35.2	Test Requirements	1875

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1875

A.7.3.36.1	Test Purpose and Environment	1875

A.7.3.36.2	Test Requirements	1878

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1878

A.7.3.37.1	Test Purpose and Environment	1878

A.7.3.37.2	Test Requirements	1881

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1881

A.7.3.38.1	Test Purpose and Environment	1881

A.7.3.38.2	Test Requirements	1884

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1884

A.7.3.39.1	Test Purpose and Environment	1884

A.7.3.39.2	Test Requirements	1887

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1888

A.7.3.40.1	Test Purpose and Environment	1888

A.7.3.40.2	Test Requirements	1890

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1891

A.7.3.41.1	Test Purpose and Environment	1891

A.7.3.41.2	Test Requirements	1894

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1894

A.7.3.42.1	Test Purpose and Environment	1894

A.7.3.42.2	Test Requirements	1897

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1897

A.7.3.43.1	Test Purpose and Environment	1897

A.7.3.43.2	Test Requirements	1900

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1900

A.7.3.44.1	Test Purpose and Environment	1900

A.7.3.44.2	Test Requirements	1903

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1904

A.7.3.45.1	Test Purpose and Environment	1904

A.7.3.45.2	Test Requirements	1906

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1907

A.7.3.46.1	Test Purpose and Environment	1907

A.7.3.46.2	Test Requirements	1910

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1910

A.7.3.47.1	Test Purpose and Environment	1910

A.7.3.47.2	Test Requirements	1913

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1913

A.7.3.48.1	Test Purpose and Environment	1913

A.7.3.48.2	Test Requirements	1916

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1916

A.7.3.49.1	Test Purpose and Environment	1916

A.7.3.49.2	Test Requirements	1919

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1919

A.7.3.50.1	Test Purpose and Environment	1919

A.7.3.50.2	Test Requirements	1922

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1922

A.7.3.51.1	Test Purpose and Environment	1922

A.7.3.51.2	Test Requirements	1925

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1925

A.7.3.52.1	Test Purpose and Environment	1925

A.7.3.52.2	Test Requirements	1928

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1928

A.7.3.53.1	Test Purpose and Environment	1928

A.7.3.53.2	Test Requirements	1931

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1931

A.7.3.54.1	Test Purpose and Environment	1931

A.7.3.54.2	Test Requirements	1934

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1934

A.7.3.55.1	Test Purpose and Environment	1934

A.7.3.55.2	Test Requirements	1937

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1937

A.7.3.56.1	Test Purpose and Environment	1937

A.7.3.56.2	Test Requirements	1940

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1940

A.7.3.57.1	Test Purpose and Environment	1940

A.7.3.57.2	Test Requirements	1943

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1943

A.7.3.58.1	Test Purpose and Environment	1943

A.7.3.58.2	Test Requirements	1946

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1946

A.7.3.59.1	Test Purpose and Environment	1946

A.7.3.59.2	Test Requirements	1949

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1949

A.7.3.60.1	Test Purpose and Environment	1949

A.7.3.60.2	Test Requirements	1953

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1953

A.7.3.61.1	Test Purpose and Environment	1953

A.7.3.61.2	Test Requirements	1957

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1957

A.7.3.62.1	Test Purpose and Environment	1957

A.7.3.62.2	Test Requirements	1961

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1961

A.7.3.63.1	Test Purpose and Environment	1961

A.7.3.63.2	Test Requirements	1965

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1965

A.7.3.64.1	Test Purpose and Environment	1965

A.7.3.64.2	Test Requirements	1968

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1969

A.7.3.65.1	Test Purpose and Environment	1969

A.7.3.65.2	Test Requirements	1972

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1973

A.7.3.66.1	Test Purpose and Environment	1973

A.7.3.66.2	Test Requirements	1975

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1976

A.7.3.67.1	Test Purpose and Environment	1976

A.7.3.67.2	Test Requirements	1979

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1980

A.7.3.68.1	Test Purpose and Environment	1980

A.7.3.68.2	Test Requirements	1982

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1982

A.7.3.69.1	Test Purpose and Environment	1982

A.7.3.69.2	Test Requirements	1984

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1984

A.7.3.70.1	Test Purpose and Environment	1984

A.7.3.70.2	Test Requirements	1986

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1986

A.7.3.71.1	Test Purpose and Environment	1986

A.7.3.71.2	Test Requirements	1989

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1989

A.7.3.72.1	Test Purpose and Environment	1989

A.7.3.72.2	Test Requirements	1991

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1991

A.7.3.73.1	Test Purpose and Environment	1991

A.7.3.73.2	Test Requirements	1993

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1993

A.7.3.74.1	Test Purpose and Environment	1993

A.7.3.74.2	Test Requirements	1996

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1996

A.7.3.75.1	Test Purpose and Environment	1996

A.7.3.75.2	Test Requirements	1998

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1999

A.7.3.76.1	Test Purpose and Environment	1999

A.7.3.76.2	Test Requirements	2002

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	2002

A.7.3.77.1	Test Purpose and Environment	2002

A.7.3.77.2	Test Requirements	2005

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	2005

A.7.3.78.1	Test Purpose and Environment	2005

A.7.3.78.2	Test Requirements	2008

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	2008

A.7.3.79.1	Test Purpose and Environment	2008

A.7.3.79.2	Test Requirements	2011

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	2011

A.7.3.80.1	Test Purpose and Environment	2011

A.7.3.80.2	Test Requirements	2014

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	2014

A.7.3.81.1	Test Purpose and Environment	2014

A.7.3.81.2	Test Requirements	2017

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2017

A.7.3.82.1	Test Purpose and Environment	2017

A.7.3.82.2	Test Requirements	2020

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2020

A.7.3.83.1	Test Purpose and Environment	2020

A.7.3.83.2	Test Requirements	2021

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2022

A.7.3.84.1	Test Purpose and Environment	2022

A.7.3.84.2	Test Requirements	2024

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2024

A.7.3.85.1	Test Purpose and Environment	2024

A.7.3.85.2	Test Requirements	2025

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2026

A.7.3.86.1	Test Purpose and Environment	2026

A.7.3.86.2	Test Requirements	2028

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2028

A.7.3.87.1	Test Purpose and Environment	2028

A.7.3.87.2	Test Requirements	2030

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	2031

A.7.3.88.1	Test Purpose and Environment	2031

A.7.3.88.2	Test Requirements	2035

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	2035

A.7.3.89.1	Test Purpose and Environment	2035

A.7.3.89.2	Test Requirements	2039

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	2039

A.7.3.90.1	Test Purpose and Environment	2039

A.7.3.90.2	Test Requirements	2043

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2043

A.7.3.91.1	Test Purpose and Environment	2043

A.7.3.91.2	Test Requirements	2047

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	2047

A.7.3.92.1	Test Purpose and Environment	2047

A.7.3.92.2	Test Requirements	2050

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2051

A.7.3.93.1	Test Purpose and Environment	2051

A.7.3.93.2	Test Requirements	2054

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	2055

A.7.3.94.1	Test Purpose and Environment	2055

A.7.3.94.2	Test Requirements	2057

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	2058

A.7.3.95.1	Test Purpose and Environment	2058

A.7.3.95.2	Test Requirements	2061

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2062

A.7.3.96.1	Test Purpose and Environment	2062

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2063

A.7.3.97.1	Test Purpose and Environment	2063

A.7.3.97.2	Test Requirements	2066

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2066

A.7.3.98.1	Test Purpose and Environment	2066

A.7.3.98.2	Test Requirements	2069

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2069

A.7.3.99.1	Test Purpose and Environment	2069

A.7.3.99.2	Test Requirements	2072

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2072

A.7.3.100.1	Test Purpose and Environment	2072

A.7.3.100.2	Test Requirements	2074

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2074

A.7.3.101.1	Test Purpose and Environment	2074

A.7.3.101.2	Test Requirements	2076

A.7.4	Interruption for Dual Connectivity	2076

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2076

A.7.4.1.1	Test Purpose and Environment	2076

A.7.4.1.2	Test Requirements	2079

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2079

A.7.4.2.1	Test Purpose and Environment	2079

A.7.4.2.2	Test Requirements	2081

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	2081

A.7.4.3.1	Test Purpose and Environment	2081

A.7.4.3.2	Test Requirements	2083

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2083

A.7.4.4.1	Test Purpose and Environment	2083

A.7.4.4.2	Test Requirements	2086

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2086

A.7.4.5.1	Test Purpose and Environment	2086

A.7.4.5.2	Test Requirements	2088

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	2088

A.7.4.6.1	Test Purpose and Environment	2088

A.7.4.6.2	Test Requirements	2090

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	2090

A.7.4.7.1	Test Purpose and Environment	2090

A.7.4.7.2	Test Requirements	2094

A.7.5	Proximity-based Services	2094

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2094

A.7.5.1.1	Test Purpose and Environment	2094

A.7.5.1.2	Test Requirements	2095

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2096

A.7.5.2.1	Test Purpose and Environment	2096

A.7.5.1.2	Test Requirements	2096

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	2097

A.7.5.3.1	Test Purpose and Environment	2097

A.7.5.3.2	Test Requirements	2098

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2099

A.7.5.4.1	Test Purpose and Environment	2099

A.7.5.4.2	Test Requirements	2100

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	2101

A.7.5.5.1	Test Purpose and Environment	2101

A.7.5.5.2	Test Requirements	2103

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	2104

A.7.5.6.1	Test Purpose and Environment	2104

A.7.5.6.2	Test Requirements	2105

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	2106

A.7.5.7.1	Test Purpose and Environment	2106

A.7.5.7.2	Test Requirements	2108

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	2108

A.7.5.8.1	Test Purpose and Environment	2108

A.7.5.8.2	Test Requirements	2110

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	2111

A.7.5.9.1	Test Purpose and Environment	2111

A.7.5.9.2	Test Requirements	2113

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	2114

A.7.5.10.1	Test Purpose and Environment	2114

A.7.5.10.2	Test Requirements	2116

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	2117

A.7.5.11.1	Test Purpose and Environment	2117

A.7.5.11.2	Test Requirements	2119

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	2119

A.7.5.12.1	Test Purpose and Environment	2119

A.7.5.12.2	Test Requirements	2123

A.7.6	Interruption for carrier aggregation	2124

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	2124

A.7.6.1.1	Test Purpose and Environment	2124

A.7.6.1.2	Test Requirements	2127

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	2127

A.7.6.2.1	Test Purpose and Environment	2127

A.7.6.2.2	Test Requirements	2130

A.8	UE Measurements Procedures	2131

A.8.1	E-UTRAN FDD Intra-frequency Measurements	2131

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2131

A.8.1.1.1	Test Purpose and Environment	2131

A.8.1.1.2	Test Requirements	2132

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2133

A.8.1.2.1	Test Purpose and Environment	2133

A.8.1.2.2	Test Requirements	2134

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2135

A.8.1.3.1	Test Purpose and Environment	2135

A.8.1.3.2	Test Requirements	2137

A.8.1.4	Void	2137

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2137

A.8.1.5.1	Test Purpose and Environment	2137

A.8.1.5.2	Test Requirements	2139

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2139

A.8.1.6.1	Test Purpose and Environment	2139

A.8.1.6.2	Test Requirements	2141

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2141

A.8.1.7.1	Test Purpose and Environment	2141

A.8.1.7.2	Test Requirements	2143

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2144

A.8.1.8.1	Test Purpose and Environment	2144

A.8.1.8.2	Test Requirements	2146

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	2147

A.8.1.9.1	Test Purpose and Environment	2147

A.8.1.9.2	Test Requirements	2147

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	2147

A.8.1.10.1	Test Purpose and Environment	2147

A.8.1.10.2	Test Requirements	2148

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2148

A.8.1.11.1	Test Purpose and Environment	2148

A.8.1.11.2	Test Requirements	2150

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2151

A.8.1.12.1	Test Purpose and Environment	2151

A.8.1.12.2	Test Requirements	2152

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2153

A.8.1.13.1	Test Purpose and Environment	2153

A.8.1.13.2	Test Requirements	2155

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2155

A.8.1.14.1	Test Purpose and Environment	2155

A.8.1.14.2	Test Requirements	2157

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2158

A.8.1.15.1	Test Purpose and Environment	2158

A.8.1.15.2	Test Requirements	2159

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2160

A.8.1.16.1	Test Purpose and Environment	2160

A.8.1.16.2	Test Requirements	2162

A.8.1.17	Void	2162

A.8.1.18	Void	2162

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2162

A.8.1.19.1	Test Purpose and Environment	2162

A.8.1.19.2	Test Requirements	2164

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2165

A.8.1.20.1	Test Purpose and Environment	2165

A.8.1.20.2	Test Requirements	2167

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2167

A.8.1.21.1	Test Purpose and Environment	2167

A.8.1.21.2	Test Requirements	2169

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2170

A.8.1.22.1	Test Purpose and Environment	2170

A.8.1.22.2	Test Requirements	2172

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2172

A.8.1.23.1	Test Purpose and Environment	2172

A.8.1.23.2	Test Requirements	2173

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2174

A.8.1.24.1	Test Purpose and Environment	2174

A.8.1.24.2	Test Requirements	2176

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2176

A.8.1.25.1	Test Purpose and Environment	2176

A.8.1.25.2	Test Requirements	2178

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2178

A.8.1.26.1	Test Purpose and Environment	2178

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2181

A.8.1.27.1	Test Purpose and Environment	2181

A.8.1.27.2	Test Requirements	2183

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2183

A.8.1.28.1	Test Purpose and Environment	2183

A.8.1.28.2	Test Requirements	2185

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2185

A.8.1.29.1	Test Purpose and Environment	2185

A.8.1.29.2	Test Requirements	2187

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2188

A.8.1.30.1	Test Purpose and Environment	2188

A.8.1.30.2	Test Requirements	2190

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2190

A.8.1.31.1	Test Purpose and Environment	2190

A.8.1.31.2	Test Requirements	2192

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2193

A.8.1.32.1	Test Purpose and Environment	2193

A.8.1.32.2	Test Requirements	2194

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2195

A.8.1.33.1	Test Purpose and Environment	2195

A.8.1.33.2	Test Requirements	2196

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2197

A.8.1.34.1	Test Purpose and Environment	2197

A.8.1.34.2	Test Requirements	2198

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2199

A.8.1.35.1	Test Purpose and Environment	2199

A.8.1.35.2	Test Requirements	2200

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2201

A.8.1.36.1	Test Purpose and Environment	2201

A.8.1.36.2	Test Requirements	2202

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2203

A.8.1.37.1	Test Purpose and Environment	2203

A.8.1.37.2	Test Requirements	2205

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2205

A.8.1.38.1	Test Purpose and Environment	2205

A.8.1.38.2	Test Requirements	2206

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2207

A.8.1.39.1	Test Purpose and Environment	2207

A.8.1.39.2	Test Requirements	2209

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2209

A.8.1.40.1	Test Purpose and Environment	2209

A.8.1.40.2	Test Requirements	2211

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2211

A.8.1.41.1	Test Purpose and Environment	2211

A.8.1.41.2	Test Requirement	2213

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2213

A.8.1.42.1	Test Purpose and Environment	2213

A.8.1.42.2	Test Requirement	2214

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2214

A.8.1.43.1	Test Purpose and Environment	2214

A.8.1.43.2	Test Requirements	2217

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2217

A.8.1.44.1	Test Purpose and Environment	2217

A.8.1.44.2	Test Requirements	2220

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2221

A.8.1.45.1	Test Purpose and Environment	2221

A.8.1.45.2	Test Requirements	2223

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2224

A.8.1.46.1	Test Purpose and Environment	2224

A.8.1.46.2	Test Requirements	2225

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2226

A.8.1.47.1	Test Purpose and Environment	2226

A.8.1.47.2	Test Requirements	2228

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2229

A.8.1.48.1	Test Purpose and Environment	2229

A.8.1.48.2	Test Requirements	2231

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2232

A.8.1.49.1	Test Purpose and Environment	2232

A.8.1.49.2	Test Requirements	2233

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2234

A.8.1.50.1	Test Purpose and Environment	2234

A.8.1.50.2	Test Requirements	2236

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2237

A.8.1.51.1	Test Purpose and Environment	2237

A.8.1.51.2	Test Requirements	2239

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2240

A.8.1.52.1	Test Purpose and Environment	2240

A.8.1.52.2	Test Requirements	2241

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2242

A.8.1.53.1	Test Purpose and Environment	2242

A.8.1.53.2	Test Requirements	2244

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2245

A.8.1.54.1	Test Purpose and Environment	2245

A.8.1.54.2	Test Requirements	2247

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2248

A.8.1.55.1	Test Purpose and Environment	2248

A.8.1.55.2	Test Requirements	2249

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2250

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2250

A.8.2.1.1	Test Purpose and Environment	2250

A.8.2.1.2	Test Requirements	2251

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2251

A.8.2.2.1	Test Purpose and Environment	2251

A.8.2.2.2	Test Requirements	2254

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2254

A.8.2.3.1	Test Purpose and Environment	2254

A.8.2.3.2	Test Requirements	2256

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2257

A.8.2.4.1	Test Purpose and Environment	2257

A.8.2.4.2	Test Requirements	2259

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2259

A.8.2.5.1	Test Purpose and Environment	2259

A.8.2.5.2	Test Requirements	2261

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2262

A.8.2.6.1	Test Purpose and Environment	2262

A.8.2.6.2	Test Requirements	2265

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2266

A.8.2.7.1	Test Purpose and Environment	2266

A.8.2.7.2	Test Requirements	2267

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2268

A.8.2.8.1	Test Purpose and Environment	2268

A.8.2.8.2	Test Requirements	2270

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2270

A.8.2.9.1	Test Purpose and Environment	2270

A.8.2.9.2	Test Requirements	2272

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2273

A.8.2.10.1	Test Purpose and Environment	2273

A.8.2.10.2	Test Requirements	2275

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2275

A.8.2.11.1	Test Purpose and Environment	2275

A.8.2.11.2	Test Requirements	2277

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2277

A.8.2.12.1	Test Purpose and Environment	2277

A.8.2.12.2	Test Requirements	2279

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2280

A.8.2.13.1	Test Purpose and Environment	2280

A.8.2.13.2	Test Requirements	2282

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2282

A.8.2.14.1	Test Purpose and Environment	2282

A.8.2.14.2	Test Requirement	2284

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2284

A.8.2.15.1	Test Purpose and Environment	2284

A.8.2.15.2	Test Requirements	2286

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2286

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2286

A.8.3.1.1	Test Purpose and Environment	2286

A.8.3.1.2	Test Requirements	2288

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2288

A.8.3.2.1	Test Purpose and Environment	2288

A.8.3.2.2	Test Requirements	2291

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2291

A.8.3.3.1	Test Purpose and Environment	2291

A.8.3.3.2	Test Requirements	2294

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2294

A.8.3.4.1	Test Purpose and Environment	2294

A.8.3.4.2	Test Requirements	2296

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2296

A.8.3.5.2	Test Requirements	2298

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2298

A.8.3.6.1	Test Purpose and Environment	2298

A.8.3.6.2	Test Requirements	2300

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2301

A.8.3.7.1	Test Purpose and Environment	2301

A.8.3.7.2	Test Requirements	2303

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2304

A.8.3.8.1	Test Purpose and Environment	2304

A.8.3.8.2	Test Requirements	2307

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2307

A.8.3.9.1	Test Purpose and Environment	2307

A.8.3.9.2	Test Requirements	2311

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2312

A.8.3.10.1	Test Purpose and Environment	2312

A.8.3.10.2	Test Requirements	2313

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2313

A.8.3.11.1	Test Purpose and Environment	2313

A.8.3.11.2	Test Requirement	2315

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2316

A.8.3.12.1	Test Purpose and Environment	2316

A.8.3.12.2	Test Requirement	2317

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2318

A.8.3.13.1	Test Purpose and Environment	2318

A.8.3.13.2	Test Requirement	2319

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2320

A.8.3.14.1	Test Purpose and Environment	2320

A.8.3.14.2	Test Requirement	2321

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2322

A.8.3.15.1	Test Purpose and Environment	2322

A.8.3.15.2	Test Requirement	2323

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2324

A.8.3.16.1	Test Purpose and Environment	2324

A.8.3.16.2	Test Requirement	2326

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2326

A.8.3.17.1	Test Purpose and Environment	2326

A.8.3.17.2	Test Requirement	2329

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2329

A.8.3.18.1	Test Purpose and Environment	2329

A.8.3.18.2	Test Requirement	2332

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2332

A.8.3.19.1	Test Purpose and Environment	2332

A.8.3.19.2	Test Requirement	2335

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2335

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2335

A.8.4.1.1	Test Purpose and Environment	2335

A.8.4.1.2	Test Requirements	2337

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2337

A.8.4.2.1	Test Purpose and Environment	2337

A.8.4.2.2	Test Requirements	2340

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2340

A.8.4.3.1	Test Purpose and Environment	2340

A.8.4.3.2	Test Requirements	2342

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2343

A.8.4.4.1	Test Purpose and Environment	2343

A.8.4.4.2	Test Requirements	2345

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2346

A.8.4.5.1	Test Purpose and Environment	2346

A.8.4.5.2	Test Requirements	2348

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2348

A.8.4.6.1	Test Purpose and Environment	2348

A.8.4.6.2	Test Requirements	2349

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2349

A.8.4.7.1	Test Purpose and Environment	2349

A.8.4.7.2	Test Requirements	2353

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2353

A.8.4.8.1	Test Purpose and Environment	2353

A.8.4.8.2	Test Requirements	2356

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2356

A.8.4.9.1	Test Purpose and Environment	2356

A.8.4.9.2	Test Requirements	2361

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2361

A.8.4.10.1	Test Purpose and Environment	2361

A.8.4.10.2	Test Requirements	2363

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2363

A.8.4.11.1	Test Purpose and Environment	2363

A.8.4.11.2	Test Requirement	2365

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2365

A.8.4.12.1	Test Purpose and Environment	2365

A.8.4.12.2	Test Requirement	2367

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2368

A.8.4.13.1	Test Purpose and Environment	2368

A.8.4.13.2	Test Requirement	2369

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2370

A.8.4.14.1	Test Purpose and Environment	2370

A.8.4.14.2	Test Requirement	2372

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2372

A.8.4.15.1	Test Purpose and Environment	2372

A.8.4.15.2	Test Requirement	2375

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2375

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2375

A.8.5.1.1	Test Purpose and Environment	2375

A.8.5.1.2	Test Requirements	2377

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2377

A.8.5.2.1	Test Purpose and Environment	2377

A.8.5.2.2	Test Requirements	2379

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2379

A.8.5.3.1	Test Purpose and Environment	2379

A.8.5.3.2	Test Requirements	2382

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2382

A.8.5.4.1	Test Purpose and Environment	2382

A.8.5.4.2	Test Requirements	2384

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2384

A.8.5.5.1	Test Purpose and Environment	2384

A.8.5.5.2	Test Requirements	2387

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2387

A.8.5.6.1	Test Purpose and Environment	2387

A.8.5.6.2	Test Requirements	2388

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2389

A.8.5.7.1	Test Purpose and Environment	2389

A.8.5.7.2	Test Requirements	2389

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2389

A.8.5.8.1	Test Purpose and Environment	2389

A.8.5.8.2	Test Requirements	2392

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2392

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2392

A.8.6.1.1	Test Purpose and Environment	2392

A.8.6.1.2	Test Requirements	2394

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2394

A.8.6.2.1	Test Purpose and Environment	2394

A.8.6.2.2	Test Requirements	2397

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2397

A.8.6.3.1	Test Purpose and Environment	2397

A.8.6.3.2	Test Requirements	2400

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2400

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2400

A.8.7.1.1	Test Purpose and Environment	2400

A.8.7.1.1.1	Void	2400

A.8.7.1.1.2	1.28 Mcps TDD option	2400

A.8.7.1.1.3	Void	2402

A.8.7.1.2	Test Requirements	2402

A.8.7.1.2.1	Void	2402

A.8.7.1.2.2	1.28 Mcps TDD option	2402

A.8.7.1.2.3	Void	2402

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2402

A.8.7.2.1	Test Purpose and Environment	2402

A.8.7.2.2	Test Requirements	2405

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2406

A.8.7.3.1	Test Purpose and Environment	2406

A.8.7.3.2	Test Parameters	2406

A.8.7.3.3	Test Requirements	2407

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2408

A.8.7.4.1	Test Purpose and Environment	2408

A.8.7.4.2	Test Requirements	2410

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2410

A.8.7.5.1	Test Purpose and Environment	2410

A.8.7.5.2	Test Requirements	2412

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2412

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2412

A.8.7A.1.1	Test Purpose and Environment	2412

A.8.7A.1.2	Test Requirements	2415

A.8.8	E-UTRAN FDD – GSM Measurements	2415

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2415

A.8.8.1.1	Test Purpose and Environment	2415

A.8.8.1.2	Test Requirements	2417

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2417

A.8.8.2.1	Test Purpose and Environment	2417

A.8.8.2.2	Test Requirements	2419

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2420

A.8.8.3.1	Test Purpose and Environment	2420

A.8.8.3.2	Test Requirements	2421

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2422

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2422

A.8.9.1.1	Test Purpose and Environment	2422

A.8.9.1.2	Test Requirements	2423

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2424

A.8.9.2.1	Test Purpose and Environment	2424

A.8.9.2.2	Test Requirements	2426

A.8.10	E-UTRAN TDD – GSM Measurements	2426

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2426

A.8.10.1.1	Test Purpose and Environment	2426

A.8.10.1.2	Test Requirements	2427

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2428

A.8.10.2.1	Test Purpose and Environment	2428

A.8.10.2.2	Test Requirements	2430

A.8.11	Monitoring of Multiple Layers	2430

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2430

A.8.11.1.1	Test Purpose and Environment	2430

A.8.11.1.2	Test Requirements	2432

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2433

A.8.11.2.1	Test Purpose and Environment	2433

A.8.11.2.2	Test Requirements	2434

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2435

A.8.11.3.1	Test Purpose and Environment	2435

A.8.11.3.2	Test Requirements	2437

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2437

A.8.11.4.1	Test Purpose and Environment	2437

A.8.11.4.2	Test Requirements	2440

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2440

A.8.11.5.1	Test Purpose and Environment	2440

A.8.11.5.2	Test Requirements	2442

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2443

A.8.11.6.1	Test Purpose and Environment	2443

A.8.11.6.2	Test Requirements	2445

A.8.12	RSTD Intra-frequency Measurements	2446

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2446

A.8.12.1.1	Test Purpose and Environment	2446

A.8.12.1.2	Test Requirements	2450

A.8.12.1.2A	Test Requirements for UE Category 1bis	2450

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2450

A.8.12.2.1	Test Purpose and Environment	2450

A.8.12.2.2	Test Requirements	2455

A.8.12.2.2A	Test Requirements for UE Category 1bis	2455

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2455

A.8.12.3.1	Test Purpose and Environment	2455

A.8.12.3.2	Test Requirements	2461

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2461

A.8.12.4.1	Test Purpose and Environment	2461

A.8.12.4.2	Test Requirements	2466

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2466

A.8.12.5.1	Test Purpose and Environment	2466

A.8.12.5.2	Test Requirements	2471

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2471

A.8.12.6.1	Test Purpose and Environment	2471

A.8.12.6.2	Test Requirements	2476

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2476

A.8.12.7.1	Test Purpose and Environment	2476

A.8.12.7.2	Test Requirements	2481

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2481

A.8.12.8.1	Test Purpose and Environment	2481

A.8.12.8.2	Test Requirements	2486

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2486

A.8.12.9.1	Test Purpose and Environment	2486

A.8.12.9.2	Test Requirements	2491

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2491

A.8.12.10.1	Test Purpose and Environment	2491

A.8.12.10.2	Test Requirements	2496

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2496

A.8.12.11.1	Test Purpose and Environment	2496

A.8.12.11.2	Test Requirements	2501

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2501

A.8.12.12.1	Test Purpose and Environment	2501

A.8.12.12.2	Test Requirements	2506

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2506

A.8.12.13.1	Test Purpose and Environment	2506

A.8.12.13.2	Test Requirements	2511

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2511

A.8.12.14.1	Test Purpose and Environment	2511

A.8.12.14.2	Test Requirements	2516

A.8.13	RSTD Inter-frequency Measurements	2516

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2516

A.8.13.1.1	Test Purpose and Environment	2516

A.8.13.1.2	Test Requirements	2521

A.8.13.1.2A	Test Requirements for UE Category 1bis	2521

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2521

A.8.13.2.1	Test Purpose and Environment	2521

A.8.13.2.2	Test Requirements	2527

A.8.13.2.2A	Test Requirements for UE Category 1bis	2527

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2527

A.8.13.3.1	Test Purpose and Environment	2527

A.8.13.3.2	Test Requirements	2534

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2534

A.8.13.4.1	Test Purpose and Environment	2534

A.8.13.4.2	Test Requirements	2540

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2540

A.8.13.5.1	Test Purpose and Environment	2540

A.8.13.5.2	Test Requirements	2546

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2546

A.8.13.6.1	Test Purpose and Environment	2546

A.8.13.6.2	Test Requirements	2552

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2552

A.8.13.7.1	Test Purpose and Environment	2552

A.8.13.7.2	Test Requirements	2558

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2558

A.8.13.8.1	Test Purpose and Environment	2558

A.8.13.8.2	Test Requirements	2564

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2564

A.8.13.9.1	Test Purpose and Environment	2564

A.8.13.9.2	Test Requirements	2570

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2570

A.8.13.10.1	Test Purpose and Environment	2570

A.8.13.10.2	Test Requirements	2576

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2576

A.8.13.11.1	Test Purpose and Environment	2576

A.8.13.11.2	Test Requirements	2582

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2582

A.8.13.12.1	Test Purpose and Environment	2582

A.8.13.12.2	Test Requirements	2588

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2588

A.8.13.13.1	Test Purpose and Environment	2588

A.8.13.13.2	Test Requirements	2594

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2594

A.8.13.14.1	Test Purpose and Environment	2594

A.8.13.14.2	Test Requirements	2600

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2600

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2600

A.8.14.1.1	Test Purpose and Environment	2600

A.8.14.1.2	Test Requirements	2602

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2602

A.8.14.2.1	Test Purpose and Environment	2602

A.8.14.2.2	Test Requirements	2605

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2605

A.8.14.3.1	Test Purpose and Environment	2605

A.8.14.3.2	Test Requirements	2607

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2608

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2608

A.8.15.1.1	Test Purpose and Environment	2608

A.8.15.1.2	Test Requirements	2609

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2609

A.8.15.2.1	Test Purpose and Environment	2609

A.8.15.2.2	Test Requirements	2612

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2612

A.8.15.3.1	Test Purpose and Environment	2612

A.8.15.3.2	Test Requirements	2614

A.8.16	E-UTRAN Carrier Aggregation Measurements	2615

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2615

A.8.16.1.1	Test Purpose and Environment	2615

A.8.16.1.2	Test Requirements	2617

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2617

A.8.16.2.1	Test Purpose and Environment	2617

A.8.16.2.2	Test Requirements	2619

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2619

A.8.16.3.1	Test Purpose and Environment	2619

A.8.16.3.2	Test Requirements	2621

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2622

A.8.16.3A.1	Test Purpose and Environment	2622

A.8.16.3A.2	Test Requirements	2623

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2624

A.8.16.4.1	Test Purpose and Environment	2624

A.8.16.4.2	Test Requirements	2625

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2626

A.8.16.4A.1	Test Purpose and Environment	2626

A.8.16.4A.2	Test Requirements	2628

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2629

A.8.16.5.1	Test Purpose and Environment	2629

A.8.16.5.2	Test Requirements	2629

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2629

A.8.16.6.1	Test Purpose and Environment	2629

A.8.16.6.2	Test Requirements	2630

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2630

A.8.16.7.1	 Test Purpose and Environment	2630

A.8.16.7.2	 Test Requirements	2631

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2631

A.8.16.8.1	 Test Purpose and Environment	2631

A.8.16.8.2	 Test Requirements	2632

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2632

A.8.16.9.1	Test Purpose and Environment	2632

A.8.16.9.2	Test Requirements	2633

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2633

A.8.16.10.1	Test Purpose and Environment	2633

A.8.16.10.2	Test Requirements	2634

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2634

A.8.16.11.1	 Test Purpose and Environment	2634

A.8.16.11.2	 Test Requirements	2635

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2635

A.8.16.12.1	 Test Purpose and Environment	2635

A.8.16.12.2	 Test Requirements	2635

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2635

A.8.16.13.1	Test Purpose and Environment	2635

A.8.16.13.2	Test Requirements	2636

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2636

A.8.16.14.1	Test Purpose and Environment	2636

A.8.16.14.2	Test Requirements	2637

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2637

A.8.16.15.1	 Test Purpose and Environment	2637

A.8.16.7.2	 Test Requirements	2637

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2637

A.8.16.16.1	 Test Purpose and Environment	2637

A.8.16.16.2	 Test Requirements	2638

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2638

A.8.16.17.1	Test Purpose and Environment	2638

A.8.16.17.2	Test Requirements	2640

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2641

A.8.16.17A.1	Test Purpose and Environment	2641

A.8.16.17A.2	Test Requirements	2641

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2641

A.8.16.17B.1	Test Purpose and Environment	2641

A.8.16.17B.2	Test Requirements	2642

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2642

A.8.16.17C.1	Test Purpose and Environment	2642

A.8.16.17C.2	Test Requirements	2642

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2642

A.8.16.18.1	Test Purpose and Environment	2642

A.8.16.18.2	Test Requirements	2644

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2645

A.8.16.18A.1	Test Purpose and Environment	2645

A.8.16.18A.2	Test Requirements	2645

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2645

A.8.16.18B.1	Test Purpose and Environment	2645

A.8.16.18B.2	Test Requirements	2646

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2646

A.8.16.18C.1	Test Purpose and Environment	2646

A.8.16.18C.2	Test Requirements	2646

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2646

A.8.16.18D.1	Test Purpose and Environment	2646

A.8.16.18D.2	Test Requirements	2647

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2647

A.8.16.19.1	Test Purpose and Environment	2647

A.8.16.19.2	Test Requirements	2649

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2649

A.8.16.19A.1	Test Purpose and Environment	2649

A.8.16.19A.2	Test Requirements	2650

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2650

A.8.16.19B.1	Test Purpose and Environment	2650

A.8.16.19B.2	Test Requirements	2650

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2650

A.8.16.19C.1	Test Purpose and Environment	2650

A.8.16.19C.2	Test Requirements	2651

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2651

A.8.16.20.1	Test Purpose and Environment	2651

A.8.16.20.2	Test Requirements	2653

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2654

A.8.16.20A.1	Test Purpose and Environment	2654

A.8.16.20A.2	Test Requirements	2654

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2654

A.8.16.20B.1	Test Purpose and Environment	2654

A.8.16.20B.2	Test Requirements	2655

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2655

A.8.16.20C.1	Test Purpose and Environment	2655

A.8.16.20C.2	Test Requirements	2655

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2655

A.8.16.20D.1	Test Purpose and Environment	2655

A.8.16.20D.2	Test Requirements	2656

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2656

A.8.16.21.1	Test Purpose and Environment	2656

A.8.16.21.2	Test Requirements	2658

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2658

A.8.16.22.1	Test Purpose and Environment	2658

A.8.16.22.2	Test Requirements	2660

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2660

A.8.16.23.1	Test Purpose and Environment	2660

A.8.16.23.2	Test Requirements	2663

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2663

A.8.16.24.1	Test Purpose and Environment	2663

A.8.16.24.2	Test Requirements	2666

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2666

A.8.16.25.1	Test Purpose and Environment	2666

A.8.16.25.2	Test Requirements	2669

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2669

A.8.16.26.1	Test Purpose and Environment	2669

A.8.16.26.2	Test Requirements	2672

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2672

A.8.16.27.1	Test Purpose and Environment	2672

A.8.16.27.2	Test Requirements	2677

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2677

A.8.16.28.1	Test Purpose and Environment	2677

A.8.16.28.2	Test Requirements	2682

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2682

A.8.16.29.1	Test Purpose and Environment	2682

A.8.16.29.2	Test Requirements	2687

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2687

A.8.16.30.1	Test Purpose and Environment	2687

A.8.16.30.2	Test Requirements	2692

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2692

A.8.16.31.1	Test Purpose and Environment	2692

A.8.16.31.2	Test Requirements	2697

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2697

A.8.16.32.1	Test Purpose and Environment	2697

A.8.16.32.2	Test Requirements	2702

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2702

A.8.16.33.1	Test Purpose and Environment	2702

A.8.16.33.2	Test Requirements	2707

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2707

A.8.16.34.1	Test Purpose and Environment	2707

A.8.16.34.2	Test Requirements	2712

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2712

A.8.16.35.1	Test Purpose and Environment	2712

A.8.16.35.2	Test Requirements	2714

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2715

A.8.16.36.1	Test Purpose and Environment	2715

A.8.16.36.2	Test Requirements	2717

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2718

A.8.16.37.1	Test Purpose and Environment	2718

A.8.16.37.2	Test Requirements	2720

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2721

A.8.16.38.1	Test Purpose and Environment	2721

A.8.16.38.2	Test Requirements	2723

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2724

A.8.16.39.1	Test Purpose and Environment	2724

A.8.16.39.2	Test Requirements	2727

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2727

A.8.16.40.1	Test Purpose and Environment	2727

A.8.16.40.2	Test Requirements	2730

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2730

A.8.16.41.1	Test Purpose and Environment	2730

A.8.16.41.2	Test Requirements	2732

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2733

A.8.16.42.1	Test Purpose and Environment	2733

A.8.16.42.2	Test Requirements	2735

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2736

A.8.16.43.1	Test Purpose and Environment	2736

A.8.16.43.2	Test Requirements	2739

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2739

A.8.16.44.1	Test Purpose and Environment	2739

A.8.16.44.2	Test Requirements	2742

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2742

A.8.16.45.1	Test Purpose and Environment	2742

A.8.16.45.2	Test Requirements	2745

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2745

A.8.16.46.1	Test Purpose and Environment	2745

A.8.16.46.2	Test Requirements	2747

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2748

A.8.16.47.1	Test Purpose and Environment	2748

A.8.16.47.2	Test Requirements	2750

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2751

A.8.16.48.1	Test Purpose and Environment	2751

A.8.16.48.2	Test Requirements	2753

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2754

A.8.16.49.1	Test Purpose and Environment	2754

A.8.16.49.2	Test Requirements	2757

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2757

A.8.16.50.1	Test Purpose and Environment	2757

A.8.16.50.2	Test Requirements	2759

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2760

A.8.16.51.1	Test Purpose and Environment	2760

A.8.16.51.2	Test Requirements	2765

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2765

A.8.16.52.1	Test Purpose and Environment	2765

A.8.16.52.2	Test Requirements	2770

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2770

A.8.16.53.1	Test Purpose and Environment	2770

A.8.16.53.2	Test Requirements	2774

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2774

A.8.16.54.1	Test Purpose and Environment	2774

A.8.16.54.2	Test Requirements	2778

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2778

A.8.16.55.1	Test Purpose and Environment	2778

A.8.16.55.2	Test Requirements	2784

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2784

A.8.16.56.1	Test Purpose and Environment	2784

A.8.16.56.2	Test Requirements	2790

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2790

A.8.16.57.1	Test Purpose and Environment	2790

A.8.16.57.2	Test Requirements	2792

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2793

A.8.16.58.1	Test Purpose and Environment	2793

A.8.16.58.2	Test Requirements	2795

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2796

A.8.16.59.1	Test Purpose and Environment	2796

A.8.16.59.2	Test Requirements	2800

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2801

A.8.16.60.1	Test Purpose and Environment	2801

A.8.16.60.2	Test Requirements	2804

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2805

A.8.16.61.1	Test Purpose and Environment	2805

A.8.16.61.2	Test Requirements	2808

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2808

A.8.16.62.1	Test Purpose and Environment	2808

A.8.16.62.2	Test Requirements	2810

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2811

A.8.16.63.1	Test Purpose and Environment	2811

A.8.16.63.2	Test Requirements	2814

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2815

A.8.16.64.1	Test Purpose and Environment	2815

A.8.16.64.2	Test Requirements	2819

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2819

A.8.16.65.1	Test Purpose and Environment	2819

A.8.16.65.2	Test Requirements	2824

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2824

A.8.16.66.1	Test Purpose and Environment	2824

A.8.16.66.2	Test Requirements	2829

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2829

A.8.16.67.1	Test Purpose and Environment	2829

A.8.16.67.2	Test Requirements	2834

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2834

A.8.16.68.1	Test Purpose and Environment	2834

A.8.16.68.2	Test Requirements	2839

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2839

A.8.16.69.1	Test Purpose and Environment	2839

A.8.16.69.2	Test Requirements	2843

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2843

A.8.16.70.1	Test Purpose and Environment	2843

A.8.16.70.2	Test Requirements	2847

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2847

A.8.16.71.1	Test Purpose and Environment	2847

A.8.16.71.2	Test Requirements	2852

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2852

A.8.16.72.1	Test Purpose and Environment	2852

A.8.16.72.2	Test Requirements	2857

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2857

A.8.16.73.1	Test Purpose and Environment	2857

A.8.16.73.2	Test Requirements	2864

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2864

A.8.16.74.1	Test Purpose and Environment	2864

A.8.16.74.2	Test Requirements	2870

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2870

A.8.16.75.1	Test Purpose and Environment	2870

A.8.16.75.2	Test Requirements	2876

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2876

A.8.16.76.1	Test Purpose and Environment	2876

A.8.16.76.2	Test Requirements	2882

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2882

A.8.16.77.1	Test Purpose and Environment	2882

A.8.16.77.2	Test Requirements	2886

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2886

A.8.16.78.1	Test Purpose and Environment	2886

A.8.16.78.2	Test Requirements	2888

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2889

A.8.16.79.1	Test Purpose and Environment	2889

A.8.16.79.2	Test Requirements	2896

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2896

A.8.16.80.1	Test Purpose and Environment	2896

A.8.16.80.2	Test Requirements	2903

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2903

A.8.16.81.1	Test Purpose and Environment	2903

A.8.16.81.2	Test Requirements	2909

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2909

A.8.16.82.1	Test Purpose and Environment	2909

A.8.16.82.2	Test Requirements	2915

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2915

A.8.16.83.1	Test Purpose and Environment	2915

A.8.16.83.2	Test Requirements	2920

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2920

A.8.16.84.1	Test Purpose and Environment	2920

A.8.16.84.2	Test Requirements	2926

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2926

A.8.16.85.1	Test Purpose and Environment	2926

A.8.16.85.2	Test Requirements	2930

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2930

A.8.16.86.1	Test Purpose and Environment	2930

A.8.16.86.2	Test Requirements	2933

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2933

A.8.16.87.1	Test Purpose and Environment	2933

A.8.16.87.2	Test Requirements	2939

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2939

A.8.16.88.1	Test Purpose and Environment	2939

A.8.16.88.2	Test Requirements	2947

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2947

A.8.16.89.1	Test Purpose and Environment	2947

A.8.16.89.2	Test Requirements	2954

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2954

A.8.16.90.1	Test Purpose and Environment	2954

A.8.16.90.2	Test Requirements	2961

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2961

A.8.16.91.1	Test Purpose and Environment	2961

A.8.16.91.2	Test Requirements	2967

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2967

A.8.16.92.1	Test Purpose and Environment	2967

A.8.16.92.2	Test Requirements	2975

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2975

A.8.16.93.1	Test Purpose and Environment	2975

A.8.16.93.2	Test Requirements	2982

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2982

A.8.16.94.1	Test Purpose and Environment	2982

A.8.16.94.2	Test Requirements	2989

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2989

A.8.16.95.1	Test Purpose and Environment	2989

A.8.16.95.2	Test Requirements	2997

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2997

A.8.16.96.1	Test Purpose and Environment	2997

A.8.16.96.2	Test Requirements	3006

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	3006

A.8.16.97.1	Test Purpose and Environment	3006

A.8.16.97.2	Test Requirements	3013

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	3013

A.8.16.98.1	Test Purpose and Environment	3013

A.8.16.98.2	Test Requirements	3021

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	3021

A.8.16.99.1	Test Purpose and Environment	3021

A.8.16.99.2	Test Requirements	3031

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	3031

A.8.16.100.1	Test Purpose and Environment	3031

A.8.16.100.2	Test Requirements	3041

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	3041

A.8.16.101.1	Test Purpose and Environment	3041

A.8.16.101.2	Test Requirements	3051

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	3051

A.8.16.102.1	Test Purpose and Environment	3051

A.8.16.102.2	Test Requirements	3062

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	3062

A.8.16.103.1	Test Purpose and Environment	3062

A.8.16.103.2	Test Requirements	3067

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	3068

A.8.16.104.1	Test Purpose and Environment	3068

A.8.16.104.2	Test Requirements	3072

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	3073

A.8.16.105.1	Test Purpose and Environment	3073

A.8.16.105.2	Test Requirements	3077

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	3078

A.8.16.106.1	Test Purpose and Environment	3078

A.8.16.106.2	Test Requirements	3082

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3082

A.8.16.107.1	Test Purpose and Environment	3082

A.8.16.107.2	Test Requirements	3084

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3085

A.8.16.108.1	Test Purpose and Environment	3085

A.8.16.108.2	Test Requirements	3087

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	3088

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	3088

A.8.17.1.1	Test Purpose and Environment	3088

A.8.17.1.2	Test Requirements	3094

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	3094

A.8.17.2.1	Test Purpose and Environment	3094

A.8.17.2.2	Test Requirements	3101

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	3101

A.8.17.3.1	Test Purpose and Environment	3101

A.8.17.3.2	Test Requirements	3102

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	3102

A.8.17.4.1	Test Purpose and Environment	3102

A.8.17.4.2	Test Requirements	3103

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3103

A.8.17.5.1	Test Purpose and Environment	3103

A.8.17.5.2	Test Requirements	3104

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3104

A.8.17.6.1	Test Purpose and Environment	3104

A.8.17.6.2	Test Requirements	3105

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	3106

A.8.17.7.1	Test Purpose and Environment	3106

A.8.17.7.2	Test Requirements	3106

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	3107

A.8.17.8.1	Test Purpose and Environment	3107

A.8.17.8.2	Test Requirements	3107

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	3108

A.8.17.9.1	Test Purpose and Environment	3108

A.8.17.9.2	Test Requirements	3109

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	3109

A.8.17.10.1	Test Purpose and Environment	3109

A.8.17.10.2	Test Requirements	3116

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	3117

A.8.17.11.1	Test Purpose and Environment	3117

A.8.17.11.2	Test Requirements	3123

A.8.18	E-UTRAN TDD – HRPD Measurements	3124

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	3124

A.8.18.1.1	Test Purpose and Environment	3124

A.8.18.1.2	Test Requirements	3126

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	3126

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	3126

A.8.19.1.1	Test Purpose and Environment	3126

A.8.19.1.2	Test Requirements	3127

A.8.20	Inter-frequency/RAT Measurements in CA mode	3128

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	3128

A.8.20.1.1	Test Purpose and Environment	3128

A.8.20.1.2	Test Requirements	3129

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3129

A.8.20.2.1	Test Purpose and Environment	3130

A.8.20.2.2	Test Requirements	3131

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	3132

A.8.20.2A.1	Test Purpose and Environment	3132

A.8.20.2A.2	Test Requirements	3132

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	3132

A.8.20.2B.1	Test Purpose and Environment	3132

A.8.20.2B.2	Test Requirements	3135

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	3135

A.8.20.3.1	Test Purpose and Environment	3135

A.8.20.3.2	Test Requirements	3137

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	3137

A.8.20.4.1	Test Purpose and Environment	3137

A.8.20.4.1.1	1.28 Mcps TDD option	3137

A.8.20.4.2	Test Requirements	3139

A.8.20.4.2.1	1.28 Mcps TDD option	3139

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3139

A.8.20.4A.1	Test Purpose and Environment	3139

A.8.20.4A.1.1	1.28 Mcps TDD option	3139

A.8.20.4A.2	Test Requirements	3139

A.8.20.4A.2.1	1.28 Mcps TDD option	3139

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3140

A.8.20.4B.1	Test Purpose and Environment	3140

A.8.20.4B.1.1	1.28 Mcps TDD option	3140

A.8.20.4B.2	Test Requirements	3142

A.8.20.4B.2.1	1.28 Mcps TDD option	3142

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	3142

A.8.21.1	Test Purpose and Environment	3142

A.8.21.2 Test Requirements	3146

A.8.22	E-UTRAN Discovery Signal Measurements	3146

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3146

A.8.22.1.1	Test Purpose and Environment	3146

A.8.22.1.2	Test Requirements	3149

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3149

A.8.22.2.1	Test Purpose and Environment	3149

A.8.22.2.2	Test Requirements	3152

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3152

A.8.22.3.1	Test Purpose and Environment	3152

A.8.22.3.2	Test Requirements	3155

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3155

A.8.22.4.1	Test Purpose and Environment	3155

A.8.22.4.2	Test Requirements	3158

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3158

A.8.22.5.1	Test Purpose and Environment	3158

A.8.22.5.2	Test Requirements	3162

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3162

A.8.22.6.1	Test Purpose and Environment	3162

A.8.22.6.2	Test Requirements	3166

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3166

A.8.22.7.1	Test Purpose and Environment	3166

A.8.22.7.2	Test Requirements	3170

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	3170

A.8.22.8.1	Test Purpose and Environment	3170

A.8.22.8.2	Test Requirements	3174

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3174

A.8.22.9.1	Test Purpose and Environment	3174

A.8.22.9.2	Test Requirements	3176

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3177

A.8.22.10.1	Test Purpose and Environment	3177

A.8.22.10.2	Test Requirements	3179

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3180

A.8.22.11.1	Test Purpose and Environment	3180

A.8.22.11.2	Test Requirements	3183

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3183

A.8.22.12.1	Test Purpose and Environment	3183

A.8.22.12.2	Test Requirements	3187

A.8.23	E-UTRAN Dual Connectivity Measurements	3187

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3187

A.8.23.1.1	Test Purpose and Environment	3187

A.8.23.1.2	Test Requirements	3190

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	3190

A.8.23.2.1	Test Purpose and Environment	3190

A.8.23.2.2	Test Requirements	3193

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3193

A.8.23.3.1	Test Purpose and Environment	3193

A.8.23.3.2	Test Requirements	3196

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3196

A.8.23.4.1	Test Purpose and Environment	3196

A.8.23.4.2	Test Requirements	3199

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	3199

A.8.23.5.1	Test Purpose and Environment	3199

A.8.23.5.2	Test Requirements	3202

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3202

A.8.23.6.1	Test Purpose and Environment	3202

A.8.23.6.2	Test Requirements	3205

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	3205

A.8.23.7.1	Test Purpose and Environment	3205

A.8.23.7.2	Test Requirements	3207

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	3208

A.8.23.8.1	Test Purpose and Environment	3208

A.8.23.8.2	Test Requirements	3210

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	3211

A.8.23.9.1	Test Purpose and Environment	3211

A.8.23.9.2	Test Requirements	3214

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3214

A.8.23.10.1	Test Purpose and Environment	3214

A.8.23.10.2	Test Requirements	3217

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3217

A.8.23.11.1	Test Purpose and Environment	3217

A.8.23.11.2	Test Requirements	3220

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3220

A.8.23.12.1	Test Purpose and Environment	3220

A.8.23.12.2	Test Requirements	3223

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3223

A.8.23.13.1	Test Purpose and Environment	3223

A.8.23.13.2	Test Requirements	3226

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3226

A.8.23.14.1	Test Purpose and Environment	3226

A.8.23.14.2	Test Requirements	3229

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3229

A.8.23.15.1	Test Purpose and Environment	3229

A.8.23.15.2	Test Requirements	3232

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3232

A.8.23.16.1	Test Purpose and Environment	3232

A.8.23.16.2	Test Requirements	3233

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3234

A.8.23.17.1	Test Purpose and Environment	3234

A.8.23.17.2	Test Requirements	3236

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3236

A.8.23.18.1	Test Purpose and Environment	3236

A.8.23.18.2	Test Requirements	3237

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3238

A.8.23.19.1	Test Purpose and Environment	3238

A.8.23.19.2	Test Requirements	3239

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3240

A.8.23.20.1	Test Purpose and Environment	3240

A.8.23.20.2	Test Requirements	3241

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3242

A.8.23.21.1	Test Purpose and Environment	3242

A.8.23.21.2	Test Requirements	3243

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3244

A.8.23.22.1	Test Purpose and Environment	3244

A.8.23.22.2	Test Requirements	3245

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3246

A.8.23.23.1	Test Purpose and Environment	3246

A.8.23.23.2	Test Requirements	3247

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3248

A.8.23.24.1	Test Purpose and Environment	3248

A.8.23.24.2	Test Requirements	3250

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3251

A.8.23.25.1	Test Purpose and Environment	3251

A.8.23.25.2	Test Requirements	3253

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3254

A.8.23.26.1	Test Purpose and Environment	3254

A.8.23.26.2	Test Requirements	3256

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3257

A.8.23.27.1	Test Purpose and Environment	3257

A.8.23.27.2	Test Requirements	3260

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3260

A.8.23.28.1	Test Purpose and Environment	3260

A.8.23.28.2	Test Requirements	3264

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3264

A.8.23.29.1	Test Purpose and Environment	3264

A.8.23.29.2	Test Requirements	3268

A.8.24	Proximity-based Services	3268

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3268

A.8.24.1.1	Test Purpose and Environment	3268

A.8.24.1.2	Test Requirements	3269

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3270

A.8.24.2.1	Test Purpose and Environment	3270

A.8.24.2.2	Test Requirements	3271

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3271

A.8.24.3.1	Test Purpose and Environment	3272

A.8.24.3.2	Test Requirements	3273

A.8.25	E-UTRAN-WLAN Measurements	3274

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3274

A.8.25.1.1	Test Purpose and Environment	3274

A.8.25.1.2	Test Requirements	3276

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3276

A.8.25.2.1	Test Purpose and Environment	3276

A.8.25.2.2	Test Requirements	3278

A.8.26 Frame Structure 3 (FS3)	3278

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3278

A.8.26.1.1	Test Purpose and Environment	3278

A.8.26.1.2	Test Requirements	3280

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3281

A.8.26.2.1	Test Purpose and Environment	3281

A.8.26.2.2	Test Requirements	3283

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3284

A.8.26.3.1	Test Purpose and Environment	3284

A.8.26.3.2	Test Requirements	3287

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3287

A.8.26.3A.1	Test Purpose and Environment	3287

A.8.26.3A.2	Test Requirements	3290

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3290

A.8.26.4.1	Test Purpose and Environment	3290

A.8.26.4.2	Test Requirements	3294

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3294

A.8.26.4A.1	Test Purpose and Environment	3294

A.8.26.4A.2	Test Requirements	3298

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3298

A.8.26.5.1	Test Purpose and Environment	3298

A.8.26.5.2	Test Requirements	3301

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3301

A.8.26.5A.1	Test Purpose and Environment	3301

A.8.26.5A.2	Test Requirements	3305

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3305

A.8.26.6.1	Test Purpose and Environment	3305

A.8.26.6.2	Test Requirements	3309

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3309

A.8.26.6A.1	Test Purpose and Environment	3309

A.8.26.6A.2	Test Requirements	3313

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3313

A.8.26.7.1	Test Purpose and Environment	3313

A.8.26.7.2	Test Requirements	3316

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3316

A.8.26.8.1	Test Purpose and Environment	3316

A.8.26.8.2	Test Requirements	3320

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3320

A.8.26.9.1	Test Purpose and Environment	3320

A.8.26.9.2	Test Requirements	3323

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3323

A.8.26.10.1	Test Purpose and Environment	3323

A.8.26.10.2	Test Requirements	3326

A.9	Measurement Performance Requirements	3328

A.9.1	RSRP	3328

A.9.1.1	FDD Intra frequency case	3328

A.9.1.1.1	Test Purpose and Environment	3328

A.9.1.1.2	Test parameters	3328

A.9.1.1.3	Test Requirements	3331

A.9.1.2	TDD Intra frequency case	3331

A.9.1.2.1	Test Purpose and Environment	3331

A.9.1.2.2	Test parameters	3331

A.9.1.2.3	Test Requirements	3332

A.9.1.3	FDD—FDD Inter frequency case	3333

A.9.1.3.1	Test Purpose and Environment	3333

A.9.1.3.2	Test parameters	3333

A.9.1.3.3	Test Requirements	3336

A.9.1.4	TDD—TDD Inter frequency case	3336

A.9.1.4.1	Test Purpose and Environment	3336

A.9.1.4.2	Test parameters	3336

A.9.1.4.3	Test Requirements	3338

A.9.1.5	FDD—TDD Inter frequency case	3339

A.9.1.5.1	Test Purpose and Environment	3339

A.9.1.5.2	Test parameters	3339

A.9.1.5.3	Test Requirements	3340

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3341

A.9.1.6.1	Test Purpose and Environment	3341

A.9.1.6.2	Test parameters	3341

A.9.1.6.3	Test Requirements	3344

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3344

A.9.1.7.1	Test Purpose and Environment	3344

A.9.1.7.2	Test parameters	3344

A.9.1.7.3	Test Requirements	3347

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3347

A.9.1.8.1	Test Purpose and Environment	3347

A.9.1.8.2	Test parameters	3347

A.9.1.8.3	Test Requirements	3351

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3351

A.9.1.9.1	Test Purpose and Environment	3351

A.9.1.9.2	Test parameters	3351

A.9.1.9.3	Test Requirements	3353

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3354

A.9.1.10.1	Test Purpose and Environment	3354

A.9.1.10.2	Test parameters	3354

A.9.1.10.3	Test Requirements	3357

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3357

A.9.1.11.1	Test Purpose and Environment	3357

A.9.1.11.2	Test parameters	3357

A.9.1.11.3	Test Requirements	3361

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3361

A.9.1.12.1	Test Purpose and Environment	3361

A.9.1.12.2	Test parameters	3361

A.9.1.12.3	Test Requirements	3362

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3362

A.9.1.13.1	Test Purpose and Environment	3362

A.9.1.13.2	Test parameters	3362

A.9.1.13.3	Test Requirements	3363

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3363

A.9.1.14.1	Test Purpose and Environment	3363

A.9.1.14.2	Test parameters	3363

A.9.1.14.3	Test Requirements	3367

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3367

A.9.1.15.1	Test Purpose and Environment	3367

A.9.1.15.2	Test parameters	3367

A.9.1.15.3	Test Requirements	3369

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3370

A.9.1.16.1	Test Purpose and Environment	3370

A.9.1.16.2	Test parameters	3370

A.9.1.16.3	Test Requirements	3371

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3371

A.9.1.17.1	Test Purpose and Environment	3371

A.9.1.17.2	Test parameters	3372

A.9.1.17.3	Test Requirements	3373

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3373

A.9.1.18.1	Test Purpose and Environment	3373

A.9.1.18.2	Test parameters	3373

A.9.1.18.3	Test Requirements	3374

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3375

A.9.1.19.1	Test Purpose and Environment	3375

A.9.1.19.2	Test parameters	3375

A.9.1.19.3	Test Requirements	3375

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3375

A.9.1.20.1	Test Purpose and Environment	3375

A.9.1.20.2	Test parameters	3375

A.9.1.20.3	Test Requirements	3377

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3377

A.9.1.21.1	Test Purpose and Environment	3377

A.9.1.21.2	Test parameters	3377

A.9.1.21.3	Test Requirements	3377

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3377

A.9.1.22.1	Test Purpose and Environment	3377

A.9.1.22.2	Test parameters	3378

A.9.1.22.3	Test Requirements	3382

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3382

A.9.1.23.1	Test Purpose and Environment	3382

A.9.1.23.2	Test parameters	3382

A.9.1.23.3	Test Requirements	3386

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3386

A.9.1.24.1	Test Purpose and Environment	3386

A.9.1.24.2	Test parameters	3386

A.9.1.24.3	Test Requirements	3387

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3387

A.9.1.25.1	Test Purpose and Environment	3387

A.9.1.25.2	Test parameters	3387

A.9.1.25.3	Test Requirements	3390

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3390

A.9.1.26.1	Test Purpose and Environment	3390

A.9.1.26.2	Test parameters	3390

A.9.1.26.3	Test Requirements	3393

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3393

A.9.1.27.1	Test Purpose and Environment	3393

A.9.1.27.2	Test parameters	3393

A.9.1.27.3	Test Requirements	3396

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3396

A.9.1.28.1	Test Purpose and Environment	3396

A.9.1.28.2	Test parameters	3396

A.9.1.28.3	Test Requirements	3399

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3399

A.9.1.29.1	Test Purpose and Environment	3399

A.9.1.29.2	Test parameters	3399

A.9.1.29.3	Test Requirements	3402

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3402

A.9.1.30.1	Test Purpose and Environment	3402

A.9.1.30.2	Test parameters	3403

A.9.1.30.3	Test Requirements	3406

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3406

A.9.1.31.1	Test Purpose and Environment	3406

A.9.1.31.2	Test parameters	3406

A.9.1.31.3	Test Requirements	3409

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3410

A.9.1.32.1	Test Purpose and Environment	3410

A.9.1.32.2	Test parameters	3410

A.9.1.32.3	Test Requirements	3413

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3413

A.9.1.33.1	Test Purpose and Environment	3413

A.9.1.33.2	Test parameters	3413

A.9.1.33.3	Test Requirements	3416

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3416

A.9.1.34.1	Test Purpose and Environment	3416

A.9.1.34.2	Test parameters	3416

A.9.1.34.3	Test Requirements	3419

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3419

A.9.1.35.1	Test Purpose and Environment	3419

A.9.1.35.2	Test parameters	3419

A.9.1.35.3	Test Requirements	3422

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3423

A.9.1.36.1	Test Purpose and Environment	3423

A.9.1.36.2	Test parameters	3423

A.9.1.36.3	Test Requirements	3426

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3426

A.9.1.37.1	Test Purpose and Environment	3426

A.9.1.37.2	Test parameters	3426

A.9.1.37.3	Test Requirements	3429

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3430

A.9.1.38.1	Test Purpose and Environment	3430

A.9.1.38.2	Test parameters	3430

A.9.1.38.3	Test Requirements	3433

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3434

A.9.1.39.1	Test Purpose and Environment	3434

A.9.1.39.2	Test parameters	3434

A.9.1.39.3	Test Requirements	3440

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3440

A.9.1.40.1	Test Purpose and Environment	3440

A.9.1.40.2	Test parameters	3441

A.9.1.40.3	Test Requirements	3447

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3447

A.9.1.41.1	Test Purpose and Environment	3447

A.9.1.41.2	Test parameters	3447

A.9.1.41.3	Test Requirements	3450

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3450

A.9.1.42.1	Test Purpose and Environment	3450

A.9.1.42.2	Test parameters	3450

A.9.1.42.3	Test Requirements	3453

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3453

A.9.1.43.1	Test Purpose and Environment	3453

A.9.1.43.2	Test parameters	3453

A.9.1.43.3	Test Requirements	3454

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3455

A.9.1.44.1	Test Purpose and Environment	3455

A.9.1.44.2	Test parameters	3455

A.9.1.44.3	Test Requirements	3460

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3460

A.9.1.45.1	Test Purpose and Environment	3460

A.9.1.45.2	Test parameters	3460

A.9.1.45.3	Test Requirements	3432

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3432

A.9.1.46.1	Test Purpose and Environment	3432

A.9.1.46.2	Test parameters	3432

A.9.1.46.3	Test Requirements	3441

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3442

A.9.1.47.1	Test Purpose and Environment	3442

A.9.1.47.2	Test parameters	3442

A.9.1.47.3	Test Requirements	3451

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3451

A.9.1.48.1	Test Purpose and Environment	3451

A.9.1.48.2	Test parameters	3452

A.9.1.48.3	Test Requirements	3462

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3463

A.9.1.49.1	Test Purpose and Environment	3463

A.9.1.49.2	Test parameters	3463

A.9.1.49.3	Test Requirements	3473

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3474

A.9.1.50.1	Test Purpose and Environment	3474

A.9.1.50.2	Test parameters	3474

A.9.1.50.3	Test Requirements	3481

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3482

A.9.1.51.1	Test Purpose and Environment	3482

A.9.1.51.2	Test parameters	3482

A.9.1.51.3	Test Requirements	3488

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3489

A.9.1.52.1	Test Purpose and Environment	3489

A.9.1.52.2	Test parameters	3489

A.9.1.52.3	Test Requirements	3492

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3492

A.9.1.52A.1	Test Purpose and Environment	3492

A.9.1.52A.2	Test parameters	3492

A.9.1.52A.3	Test Requirements	3493

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3493

A.9.1.53.1	Test Purpose and Environment	3493

A.9.1.53.2	Test parameters	3494

A.9.1.53.3	Test Requirements	3497

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3497

A.9.1.53A.1	Test Purpose and Environment	3497

A.9.1.53A.2	Test parameters	3497

A.9.1.53A.3	Test Requirements	3498

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3499

A.9.1.54.1	Test Purpose and Environment	3499

A.9.1.54.2	Test parameters	3499

A.9.1.54.3	Test Requirements	3501

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	3501

A.9.1.55.1	Test Purpose and Environment	3501

A.9.1.55.2	Test parameters	3501

A.9.1.55.3	Test Requirements	3505

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	3505

A.9.1.56.1	Test Purpose and Environment	3505

A.9.1.56.2	Test parameters	3505

A.9.1.56.3	Test Requirements	3508

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3509

A.9.1.57.1	Test Purpose and Environment	3509

A.9.1.57.2	Test parameters	3509

A.9.1.57.3	Test Requirements	3512

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3512

A.9.1.57A.1	Test Purpose and Environment	3512

A.9.1.57A.2	Test parameters	3512

A.9.1.57A.3	Test Requirements	3513

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3513

A.9.1.58.1	Test Purpose and Environment	3513

A.9.1.58.2	Test parameters	3514

A.9.1.58.3	Test Requirements	3517

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3517

A.9.1.58A.1	Test Purpose and Environment	3517

A.9.1.58A.2	Test parameters	3517

A.9.1.58A.3	Test Requirements	3518

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3518

A.9.1.59.1	Test Purpose and Environment	3518

A.9.1.59.2	Test parameters	3519

A.9.1.59.3	Test Requirements	3521

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	3521

A.9.1.60.1	Test Purpose and Environment	3521

A.9.1.60.2	Test parameters	3521

A.9.1.60.3	Test Requirements	3524

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	3525

A.9.1.61.1	Test Purpose and Environment	3525

A.9.1.61.2	Test parameters	3525

A.9.1.61.3	Test Requirements	3528

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3528

A.9.1.62.1	Test Purpose and Environment	3528

A.9.1.62.2	Test parameters	3528

A.9.1.62.3	Test Requirements	3531

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3531

A.9.1.53.1	Test Purpose and Environment	3531

A.9.1.53.2	Test parameters	3531

A.9.1.63.3	Test Requirements	3534

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3534

A.9.1.64.1	Test Purpose and Environment	3534

A.9.1.64.2	Test parameters	3534

A.9.1.64.3	Test Requirements	3535

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3536

A.9.1.65.1	Test Purpose and Environment	3536

A.9.1.65.2	Test parameters	3536

A.9.1.65.3	Test Requirements	3542

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3542

A.9.1.66.1	Test Purpose and Environment	3542

A.9.1.66.2	Test parameters	3542

A.9.1.66.3	Test Requirements	3548

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3548

A.9.1.67.1	Test Purpose and Environment	3548

A.9.1.67.2	Test parameters	3548

A.9.1.67.3	Test Requirements	3549

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3550

A.9.1.68.1	Test Purpose and Environment	3550

A.9.1.68.2	Test parameters	3550

A.9.1.68.3	Test Requirements	3554

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3554

A.9.1.69.1	Test Purpose and Environment	3554

A.9.1.69.2	Test parameters	3554

A.9.1.69.3	Test Requirements	3558

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3559

A.9.1.70.1	Test Purpose and Environment	3559

A.9.1.70.2	Test parameters	3559

A.9.1.70.3	Test Requirements	3571

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3572

A.9.1.71.1	Test Purpose and Environment	3572

A.9.1.71.2	Test parameters	3572

A.9.1.71.3	Test Requirements	3584

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3585

A.9.1.72.1	Test Purpose and Environment	3585

A.9.1.72.2	Test parameters	3585

A.9.1.72.3	Test Requirements	3597

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	3598

A.9.1.73.1	Test Purpose and Environment	3598

A.9.1.73.2	Test parameters	3598

A.9.1.73.3	Test Requirements	3601

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	3601

A.9.1.74.1	Test Purpose and Environment	3601

A.9.1.74.2	Test parameters	3601

A.9.1.74.3	Test Requirements	3602

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	3602

A.9.1.75.1	Test Purpose and Environment	3602

A.9.1.75.2	Test parameters	3603

A.9.1.75.3	Test Requirements	3604

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3604

A.9.1.76.1	Test Purpose and Environment	3604

A.9.1.76.2	Test parameters	3604

A.9.1.76.3	Test Requirements	3607

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3607

A.9.1.77.1	Test Purpose and Environment	3607

A.9.1.77.2	Test parameters	3607

A.9.1.77.3	Test Requirements	3610

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3610

A.9.1.78.1	Test Purpose and Environment	3610

A.9.1.78.2	Test parameters	3610

A.9.1.78.3	Test Requirements	3613

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3613

A.9.1.79.1	Test Purpose and Environment	3613

A.9.1.79.2	Test parameters	3613

A.9.1.79.3	Test Requirements	3616

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3616

A.9.1.80.1	Test Purpose and Environment	3616

A.9.1.80.2	Test parameters	3616

A.9.1.80.3	Test Requirements	3619

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3619

A.9.1.81.1	Test Purpose and Environment	3619

A.9.1.78.2	Test parameters	3619

A.9.1.81.3	Test Requirements	3622

A.9.2	RSRQ	3622

A.9.2.1	FDD Intra frequency case	3622

A.9.2.1.1	Test Purpose and Environment	3622

A.9.2.1.2	Test parameters	3622

A.9.2.1.3	Test Requirements	3625

A.9.2.2	TDD Intra frequency case	3625

A.9.2.2.1	Test Purpose and Environment	3625

A.9.2.2.2	Test parameters	3625

A.9.2.2.3	Test Requirements	3627

A.9.2.3	FDD—FDD Inter frequency case	3627

A.9.2.3.1	Test Purpose and Environment	3627

A.9.2.3.2	Test parameters	3627

A.9.2.3.3	Test Requirements	3630

A.9.2.4	TDD—TDD Inter frequency case	3630

A.9.2.4.1	Test Purpose and Environment	3630

A.9.2.4.2	Test parameters	3630

A.9.2.4.3	Test Requirements	3632

A.9.2.4A	FDD—TDD Inter frequency case	3633

A.9.2.4A.1	Test Purpose and Environment	3633

A.9.2.4A.2	Test parameters	3633

A.9.2.4A.3	Test Requirements	3635

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	3635

A.9.2.5.1	Test Purpose and Environment	3635

A.9.2.5.2	Test parameters	3636

A.9.2.5.3	Test Requirements	3639

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	3639

A.9.2.6.1	Test Purpose and Environment	3639

A.9.2.6.2	Test parameters	3639

A.9.2.6.3	Test Requirements	3642

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3642

A.9.2.7.1	Test Purpose and Environment	3642

A.9.2.7.2	Test parameters	3642

A.9.2.7.3	Test Requirements	3646

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3646

A.9.2.8.1	Test Purpose and Environment	3646

A.9.2.8.2	Test parameters	3646

A.9.2.8.3	Test Requirements	3650

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	3650

A.9.2.9.1	Test Purpose and Environment	3650

A.9.2.9.2	Test parameters	3650

A.9.2.9.3	Test Requirements	3654

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	3654

A.9.2.10.1	Test Purpose and Environment	3654

A.9.2.10.2	Test parameters	3654

A.9.2.10.3	Test Requirements	3658

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3658

A.9.2.11.1	Test Purpose and Environment	3658

A.9.2.11.2	Test parameters	3658

A.9.2.11.3	Test Requirements	3659

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3659

A.9.2.12.1	Test Purpose and Environment	3659

A.9.2.12.2	Test parameters	3660

A.9.2.12.3	Test Requirements	3660

A.9.2.13	Void	3661

A.9.2.13.1	Void	3661

A.9.2.13.2	Void	3661

A.9.2.13.3	Void	3661

A.9.2.14	Void	3661

A.9.2.14.1	Void	3661

A.9.2.14.2	Void	3661

A.9.2.14.3	Void	3661

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3661

A.9.2.15.1	Test Purpose and Environment	3661

A.9.2.15.2	Test parameters	3661

A.9.2.15.3	Test Requirements	3665

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3665

A.9.2.16.1	Test Purpose and Environment	3665

A.9.2.16.2	Test parameters	3665

A.9.2.16.3	Test Requirements	3670

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	3670

A.9.2.17.1	Test Purpose and Environment	3670

A.9.2.17.2	Test parameters	3670

A.9.2.17.3	Test Requirements	3672

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	3672

A.9.2.18.1	Test Purpose and Environment	3672

A.9.2.18.2	Test parameters	3672

A.9.2.18.3	Test Requirements	3675

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	3675

A.9.2.19.1	Test Purpose and Environment	3675

A.9.2.19.2	Test parameters	3675

A.9.2.19.3	Test Requirements	3677

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	3677

A.9.2.20.1	Test Purpose and Environment	3677

A.9.2.20.2	Test parameters	3677

A.9.2.20.3	Test Requirements	3680

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3680

A.9.2.21.1	Test Purpose and Environment	3680

A.9.2.21.2	Test parameters	3680

A.9.2.21.3	Test Requirements	3683

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3683

A.9.2.22.1	Test Purpose and Environment	3684

A.9.2.22.2	Test parameters	3684

A.9.2.22.3	Test Requirements	3684

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3684

A.9.2.23.1	Test Purpose and Environment	3684

A.9.2.23.2	Test parameters	3684

A.9.2.23.3	Test Requirements	3687

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3687

A.9.2.24.1	Test Purpose and Environment	3687

A.9.2.24.2	Test parameters	3687

A.9.2.24.3	Test Requirements	3688

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3688

A.9.2.25.1	Test Purpose and Environment	3688

A.9.2.25.2	Test parameters	3688

A.9.2.25.3	Test Requirements	3693

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3693

A.9.2.26.1	Test Purpose and Environment	3693

A.9.2.26.2	Test parameters	3693

A.9.2.26.3	Test Requirements	3697

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	3697

A.9.2.27.1	Test Purpose and Environment	3697

A.9.2.27.2	Test parameters	3697

A.9.2.27.3	Test Requirements	3698

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3698

A.9.2.28.1	Test Purpose and Environment	3698

A.9.2.28.2	Test parameters	3698

A.9.2.28.3	Test Requirements	3701

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3701

A.9.2.29.1	Test Purpose and Environment	3701

A.9.2.29.2	Test parameters	3701

A.9.2.29.3	Test Requirements	3704

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3704

A.9.2.30.1	Test Purpose and Environment	3704

A.9.2.30.2	Test parameters	3704

A.9.2.30.3	Test Requirements	3707

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3707

A.9.2.31.1	Test Purpose and Environment	3707

A.9.2.31.2	Test parameters	3707

A.9.2.31.3	Test Requirements	3710

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3710

A.9.2.32.1	Test Purpose and Environment	3710

A.9.2.32.2	Test parameters	3710

A.9.2.32.3	Test Requirements	3713

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3713

A.9.2.33.1	Test Purpose and Environment	3713

A.9.2.33.2	Test parameters	3713

A.9.2.33.3	Test Requirements	3717

A.9.2.34	FDD—FDD Inter frequency new RSRQ	3717

A.9.2.34.1	Test Purpose and Environment	3717

A.9.2.34.2	Test parameters	3717

A.9.2.34.3	Test Requirements	3720

A.9.2.35	TDD—TDD Inter frequency new RSRQ	3720

A.9.2.35.1	Test Purpose and Environment	3720

A.9.2.35.2	Test parameters	3720

A.9.2.35.3	Test Requirements	3723

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	3723

A.9.2.36.1	Test Purpose and Environment	3723

A.9.2.36.2	Test parameters	3723

A.9.2.36.3	Test Requirements	3724

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	3725

A.9.2.37.1	Test Purpose and Environment	3725

A.9.2.37.2	Test parameters	3725

A.9.2.37.3	Test Requirements	3726

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3727

A.9.2.38.1	Test Purpose and Environment	3727

A.9.2.38.2	Test parameters	3727

A.9.2.38.3	Test Requirements	3731

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3731

A.9.2.39.1	Test Purpose and Environment	3731

A.9.2.39.2	Test parameters	3731

A.9.2.39.3	Test Requirements	3735

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3735

A.9.2.40.1	Test Purpose and Environment	3735

A.9.2.40.2	Test parameters	3735

A.9.2.40.3	Test Requirements	3738

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3739

A.9.2.41.1	Test Purpose and Environment	3739

A.9.2.41.2	Test parameters	3739

A.9.2.41.3	Test Requirements	3742

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	3742

A.9.2.42.1	Test Purpose and Environment	3742

A.9.2.42.2	Test parameters	3742

A.9.2.42.3	Test Requirements	3745

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	3745

A.9.2.43.1	Test Purpose and Environment	3745

A.9.2.43.2	Test parameters	3745

A.9.2.43.3	Test Requirements	3748

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	3748

A.9.2.44.1	Test Purpose and Environment	3748

A.9.2.44.2	Test parameters	3748

A.9.2.44.3	Test Requirements	3750

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	3750

A.9.2.45.1	Test Purpose and Environment	3750

A.9.2.45.2	Test parameters	3750

A.9.2.45.3	Test Requirements	3754

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	3755

A.9.2.46.1	Test Purpose and Environment	3755

A.9.2.46.2	Test parameters	3755

A.9.2.46.3	Test Requirements	3759

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3760

A.9.2.47.1	Test Purpose and Environment	3760

A.9.2.47.2	Test parameters	3760

A.9.2.47.3	Test Requirements	3764

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3764

A.9.2.48.1	Test Purpose and Environment	3764

A.9.2.48.2	Test parameters	3765

A.9.2.48.3	Test Requirements	3769

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3770

A.9.2.49.1	Test Purpose and Environment	3770

A.9.2.49.2	Test parameters	3770

A.9.2.49.3	Test Requirements	3776

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3777

A.9.2.50.1	Test Purpose and Environment	3777

A.9.2.50.2	Test parameters	3777

A.9.2.50.3	Test Requirements	3783

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	3783

A.9.2.51.1	Test Purpose and Environment	3783

A.9.2.51.2	Test parameters	3784

A.9.2.51.3	Test Requirements	3788

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	3788

A.9.2.52.1	Test Purpose and Environment	3788

A.9.2.52.2	Test parameters	3788

A.9.2.52.3	Test Requirements	3791

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3792

A.9.2.53.1	Test Purpose and Environment	3792

A.9.2.53.2	Test parameters	3792

A.9.2.53.3	Test Requirements	3798

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3799

A.9.2.54.1	Test Purpose and Environment	3799

A.9.2.54.2	Test parameters	3799

A.9.2.54.3	Test Requirements	3805

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3805

A.9.2.55.1	Test Purpose and Environment	3805

A.9.2.55.2	Test parameters	3806

A.9.2.55.3	Test Requirements	3811

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3811

A.9.2.56.1	Test Purpose and Environment	3811

A.9.2.56.2	Test parameters	3811

A.9.2.56.3	Test Requirements	3816

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3816

A.9.2.57.1	Test Purpose and Environment	3816

A.9.2.57.2	Test parameters	3817

A.9.2.57.3	Test Requirements	3821

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3822

A.9.2.58.1	Test Purpose and Environment	3822

A.9.2.58.2	Test parameters	3822

A.9.2.58.3	Test Requirements	3826

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3827

A.9.2.59.1	Test Purpose and Environment	3827

A.9.2.59.2	Test parameters	3827

A.9.2.59.3	Test Requirements	3832

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	3833

A.9.2.60.1	Test Purpose and Environment	3833

A.9.2.60.2	Test parameters	3833

A.9.2.60.3	Test Requirements	3836

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	3836

A.9.2.61.1	Test Purpose and Environment	3836

A.9.2.61.2	Test parameters	3836

A.9.2.61.3	Test Requirements	3839

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	3839

A.9.2.62.1	Test Purpose and Environment	3839

A.9.2.62.2	Test parameters	3839

A.9.2.62.3	Test Requirements	3842

A.9.3	UTRAN FDD CPICH RSCP	3842

A.9.3.1	E-UTRAN FDD	3842

A.9.3.1.1	Test Purpose and Environment	3842

A.9.3.1.2	Parameters	3842

A.9.3.1.3	Test Requirements	3845

A.9.3.2 E-UTRAN TDD	3845

A.9.3.2.1	Test Purpose and Environment	3845

A.9.3.2.2	Parameters	3845

A.9.3.2.3	Test Requirements	3848

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	3848

A.9.3.3.1	Test Purpose and Environment	3848

A.9.3.3.2	Parameters	3848

A.9.3.3.3	Test Requirements	3849

A.9.4	UTRAN FDD CPICH Ec/No	3850

A.9.4.1	E-UTRAN FDD	3850

A.9.4.1.1	Test Purpose and Environment	3850

A.9.4.1.2	Parameters	3850

A.9.4.1.3	Test Requirements	3852

A.9.4.2	E-UTRAN TDD	3853

A.9.4.2.1	Test Purpose and Environment	3853

A.9.4.2.2	Parameters	3853

A.9.4.2.3	Test Requirements	3855

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	3856

A.9.4.3.1	Test Purpose and Environment	3856

A.9.4.3.2	Parameters	3856

A.9.4.3.3	Test Requirements	3857

A.9.5	UTRAN TDD measurement	3857

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	3857

A.9.5.1.1	Test Purpose and Environment	3857

A.9.5.1.2	Test parameters	3857

A.9.5.1.3	Test Requirements	3859

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	3859

A.9.5.2.1	Test Purpose and Environment	3859

A.9.5.2.2	Test parameters	3859

A.9.5.2.3	Test Requirements	3861

A.9.6	GSM Carrier RSSI	3862

A.9.6.1	E-UTRAN FDD	3862

A.9.6.1.1	Test Purpose and Environment	3862

A.9.6.1.2	Test Requirements	3863

A.9.6.2	E-UTRAN TDD	3863

A.9.6.2.1	Test Purpose and Environment	3863

A.9.6.2.2	Test Requirements	3865

A.9.7	UE Rx – Tx Time Difference	3866

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	3866

A.9.7.1.1	Test Purpose and Environment	3866

A.9.7.1.2	Test parameters	3866

A.9.7.1.3	Test Requirements	3867

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	3867

A.9.7.2.1	Test Purpose and Environment	3867

A.9.7.2.2	Test parameters	3867

A.9.7.2.3	Test Requirements	3869

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3869

A.9.7.3.1	Test Purpose and Environment	3869

A.9.7.3.2	Test parameters	3869

A.9.7.3.3	Test Requirements	3872

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3872

A.9.7.4.1	Test Purpose and Environment	3872

A.9.7.4.2	Test Parameters	3872

A.9.7.4.3	Test Requirements	3875

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3875

A.9.7.5.1	Test Purpose and Environment	3875

A.9.7.5.2	Test parameters	3875

A.9.7.5.3	Test Requirements	3878

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3878

A.9.7.6.1	Test Purpose and Environment	3878

A.9.7.6.2	Test Parameters	3878

A.9.7.6.3	Test Requirements	3881

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3881

A.9.7.7.1	Test Purpose and Environment	3881

A.9.7.7.2	Test parameters	3881

A.9.7.7.3	Test Requirements	3882

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3883

A.9.7.8.1	Test Purpose and Environment	3883

A.9.7.8.2	Test parameters	3883

A.9.7.8.3	Test Requirements	3884

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3884

A.9.7.9.1	Test Purpose and Environment	3884

A.9.7.9.2	Test parameters	3884

A.9.7.9.3	Test Requirements	3885

A.9.8	RSTD	3886

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	3886

A.9.8.1.1	Test Purpose and Environment	3886

A.9.8.1.2	Test Requirements	3888

A.9.8.1.2A	Test Requirements for UE Category 1bis	3888

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	3889

A.9.8.2.1	Test Purpose and Environment	3889

A.9.8.2.2	Test Requirements	3892

A.9.8.2.2A	Test Requirements for UE Category 1bis	3892

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	3893

A.9.8.3.1	Test Purpose and Environment	3893

A.9.8.3.2	Test Requirements	3895

A.9.8.3.2A	Test Requirements for UE Category 1bis	3895

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	3896

A.9.8.4.1	Test Purpose and Environment	3896

A.9.8.4.2	Test Requirements	3898

A.9.8.4.2A	Test Requirements for UE Category 1bis	3898

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	3899

A.9.8.5.1	Test Purpose and Environment	3899

A.9.8.5.2	Test Requirements	3901

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	3901

A.9.8.6.1	Test Purpose and Environment	3901

A.9.8.6.2	Test Requirements	3904

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3904

A.9.8.7.1	Test Purpose and Environment	3904

A.9.8.7.2	Test Requirements	3905

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3905

A.9.8.8.1	Test Purpose and Environment	3905

A.9.8.8.2	Test Requirements	3906

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3906

A.9.8.9.1	Test Purpose and Environment	3906

A.9.8.9.2	Test Requirements	3907

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3907

A.9.8.10.1	Test Purpose and Environment	3907

A.9.8.10.2	Test Requirements	3908

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	3908

A.9.8.11.1	Test Purpose and Environment	3908

A.9.8.11.2	Test Requirements	3909

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	3909

A.9.8.12.1	Test Purpose and Environment	3909

A.9.8.12.2	Test Requirements	3910

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	3910

A.9.8.13.1	Test Purpose and Environment	3910

A.9.8.13.2	Test Requirements	3911

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3911

A.9.8.14.1	Test Purpose and Environment	3911

A.9.8.14.2	Test Requirements	3918

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3918

A.9.8.15.1	Test Purpose and Environment	3918

A.9.8.15.2	Test Requirements	3924

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3924

A.9.8.16.1	Test Purpose and Environment	3924

A.9.8.16.2	Test Requirements	3927

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3927

A.9.8.17.1	Test Purpose and Environment	3927

A.9.8.17.2	Test Requirements	3930

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3930

A.9.8.18.1	Test Purpose and Environment	3930

A.9.8.18.2	Test Requirements	3933

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3933

A.9.8.19.1	Test Purpose and Environment	3933

A.9.8.19.2	Test Requirements	3936

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	3936

A.9.8.20.1	Test Purpose and Environment	3936

A.9.8.20.2	Test Requirements	3939

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	3940

A.9.8.21.1	Test Purpose and Environment	3940

A.9.8.21.2	Test Requirements	3943

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	3944

A.9.8.22.1	Test Purpose and Environment	3944

A.9.8.22.2	Test Requirements	3947

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	3948

A.9.8.23.1	Test Purpose and Environment	3948

A.9.8.23.2	Test Requirements	3951

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3951

A.9.8.24.1	Test Purpose and Environment	3951

A.9.8.24.2	Test Requirements	3955

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3956

A.9.8.25.1	Test Purpose and Environment	3956

A.9.8.25.2	Test Requirements	3959

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3960

A.9.8.26.1	Test Purpose and Environment	3960

A.9.8.26.2	Test Requirements	3962

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3962

A.9.8.27.1	Test Purpose and Environment	3962

A.9.8.27.2	Test Requirements	3965

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3965

A.9.8.28.1	Test Purpose and Environment	3965

A.9.8.28.2	Test Requirements	3968

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3968

A.9.8.29.1	Test Purpose and Environment	3968

A.9.8.29.2	Test Requirements	3972

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3973

A.9.8.30.1	Test Purpose and Environment	3973

A.9.8.30.2	Test Requirements	3976

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3977

A.9.8.31.1	Test Purpose and Environment	3977

A.9.8.31.2	Test Requirements	3981

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3981

A.9.8.32.1	Test Purpose and Environment	3981

A.9.8.32.2	Test Requirements	3984

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3984

A.9.8.33.1	Test Purpose and Environment	3984

A.9.8.33.2	Test Requirements	3987

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3987

A.9.8.34.1	Test Purpose and Environment	3987

A.9.8.34.2	Test Requirements	3990

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3990

A.9.8.35.1	Test Purpose and Environment	3990

A.9.8.35.2	Test Requirements	3993

A.9.9	RSRP and RSRQ on the serving cell	3993

A.9.9.1	FDD Intra frequency serving cell case	3993

A.9.9.1.1	Test Purpose and Environment	3993

A.9.9.1.2	Test parameters	3993

A.9.9.1.3	Test Requirements	3996

A.9.9.2	TDD Intra frequency serving cell case	3996

A.9.9.2.1	Test Purpose and Environment	3996

A.9.9.2.2	Test parameters	3996

A.9.9.2.3	Test Requirements	3998

A.9.10	SSTD	3998

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	3998

A.9.10.1.1	Test Purpose and Environment	3998

A.9.10.1.2	Test parameters	3998

A.9.10.1.3	Test Requirements	4000

A.9.10.2	Void	4000

A.9.10.3	Void	4000

A.9.10.4	Void	4000

A.9.11 RSSI	4000

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	4000

A.9.11.1.1	Test Purpose and Environment	4000

A.9.11.1.2	Test parameters	4000

A.9.11.1.3	Test Requirements	4003

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	4003

A.9.11.2.1	Test Purpose and Environment	4003

A.9.11.2.2	Test parameters	4003

A.9.12	Channel occupancy	4006

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	4006

A.9.12.1.1	Test Purpose and Environment	4006

A.9.12.1.2	Test parameters	4006

A.9.12.1.3	Test Requirements	4009

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	4009

A.9.12.2.1	Test Purpose and Environment	4009

A.9.12.2.2	Test parameters	4009

A.9.12.2.3	Test Requirements	4012

A.9.13	RS-SINR	4012

A.9.13.1	FDD Intra-Frequency Case	4012

A.9.13.1.1	Test Purpose and Environment	4012

A.9.13.1.2	Test parameters	4012

A.9.13.1.3	Test Requirements	4015

A.9.13.2	TDD Intra-Frequency Case	4015

A.9.13.2.1	Test Purpose and Environment	4015

A.9.13.2.2	Test parameters	4015

A.9.13.2.3	Test Requirements	4018

A.9.13.3	FDD—FDD Inter frequency case	4018

A.9.13.3.1	Test Purpose and Environment	4018

A.9.13.3.2	Test parameters	4018

A.9.13.3.3	Test Requirements	4021

A.9.13.4	TDD—TDD Inter frequency case	4021

A.9.13.4.1	Test Purpose and Environment	4021

A.9.13.4.2	Test parameters	4021

A.9.13.4.3	Test Requirements	4025

A.9.13.5	FDD—TDD Inter frequency case	4026

A.9.13.5.1	Test Purpose and Environment	4026

A.9.13.5.2	Test parameters	4026

A.9.13.5.3	Test Requirements	4032

A.9.13.6	TDD—FDD Inter frequency case	4032

A.9.13.6.1	Test Purpose and Environment	4032

A.9.13.6.2	Test parameters	4032

A.9.13.6.3	Test Requirements	4036

A.9.14	Channel quality reporting accuracy	4036

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4036

A.9.14.1.1	Test Purpose and Environment	4036

A.9.14.1.2	Test parameters	4037

A.9.14.1.3	Test Requirements	4037

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4038

A.9.14.2.1	Test Purpose and Environment	4038

A.9.14.2.2	Test parameters	4038

A.9.14.2.3	Test Requirements	4039

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4039

A.9.14.3.1	Test Purpose and Environment	4039

A.9.14.3.2	Test parameters	4039

A.9.14.3.3	Test Requirements	4040

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4040

A.9.14.4.1	Test Purpose and Environment	4040

A.9.14.4.2	Test parameters	4040

A.9.14.4.3	Test Requirements	4041

A.10	Proximity-based Services in Any Cell Selection State	4064

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	4064

A.10.1.1	Test Purpose and Environment	4064

A.10.1.2	Test Requirements	4065

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	4066

A.10.2.1	Test Purpose and Environment	4066

A.10.2.2	Test Requirements	4067

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	4067

A.10.3.1	Test Purpose and Environment	4067

A.10.3.2	Test Requirements	4069

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	4070

A.10.4.1	Test Purpose and Environment	4070

A.10.4.2	Test Requirements	4072

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	4072

A.11.1	V2V UE Transmission Timing Accuracy Test	4072

A.11.1.1	Test Purpose and Environment	4072

A.11.1.2	Test requirements	4073

A.11.2	Interruptions due to V2V sidelink communication	4073

A.11.2.1	Test Purpose and Environment	4073

A.11.2.2	Test Requirements	4075

A.12	4075

A.12.1	V2X UE Transmission Timing Accuracy Test	4075

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	4075

A.12.1.1.1	Test Purpose and Environment	4075

A.12.1.1.2	Test requirements	4076

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	4077

A.12.1.2.1	Test Purpose and Environment	4077

A.12.1.2.2	Test Requirements	4077

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	4078

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	4078

A.12.2.1.1	Test Purpose and Environment	4078

A.12.2.1.2	Test Requirements	4079

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	4080

A.12.2.2.1	Test Purpose and Environment	4080

A.12.2.2.2	Test Requirements	4081

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	4081

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	4081

A.12.3.1.1	Test Purpose and Environment	4081

A.12.3.1.2	Test Requirements	4084

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	4085

A.12.3.2.1	Test Purpose and Environment	4085

A.12.3.1.2	Test Requirements	4087

A.12.4	Congestion Control Measurement Test for V2X UE	4087

A.12.4.1	Test Purpose and Environment	4087

A.12.4.2	Test Requirements	4089

A.12.5	Interruptions due to V2X Sidelink Communication	4089

A.12.5.1	Test Purpose and Environment	4089

A.12.5.2	Test Requirements	4091

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	4091

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	4091

A.12.6.1.1	Test Purpose and Environment	4091

A.12.6.1.2	Test Requirements	4093

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	4093

A.12.6.2.1	Test Purpose and Environment	4093

A.12.6.1.2	Test Requirements	4095

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	4095

A.12.7.1	Test Purpose and Environment	4095

A.12.7.2	Test Requirements	4098

A.12.8	Interruptions due to V2X Carrier Aggregation	4099

A.12.8.1	Interruptions on a FDD PCell	4099

A.12.8.1.1	Test Purpose and Environment	4099

A.12.8.1.2	Test Requirements	4100

A.12.8.2	Interruptions on a TDD PCell	4100

A.12.8.2.1	Test Purpose and Environment	4100

A.12.8.2.2	Test Requirements	4102

A.13	E-UTRAN Standalone Tests for UE Category NB for Satellite Access	4103

A.13.1	RRC_IDLE state for satellite access	4103

A.13.1.1	Cell re-selection for satellite access	4103

A.13.1.1.1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage	4103

A.13.1.1.1.1	Test Purpose and Environment	4103

A.13.1.1.1.2	Test Requirements	4105

A.13.1.1.2	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation	4106

A.13.1.1.2.1	Test Purpose and Environment	4106

A.13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX	4109

A.13.1.1.3.1	Test Purpose and Environment	4109

A.13.1.1.3.2	Test Requirements	4111

A.13.1.1.4	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage	4112

A.13.1.1.4.1	Test Purpose and Environment	4112

A.13.1.1.4.2	Test Requirements	4113

A.13.1.1.5	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO	4114

A.13.1.1.5.1	Test Purpose and Environment	4114

A.13.1.1.5.2	Test Requirements	4115

A.13.1.1.6	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO	4116

A.13.1.1.6.1	Test Purpose and Environment	4116

A.13.1.1.6.2	Test Requirements	4117

A.13.2	Void	4118

A.13.3	RRC connection mobility control for satellite access	4118

A.13.3.1	RRC re-establishment for satellite access	4118

A.13.3.1.1	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage	4118

A.13.3.1.1.1	Test Purpose and Environment	4118

A.13.3.1.1.2	Test Requirements	4119

A.13.3.1.2	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4120

A.13.3.1.2.1	Test Purpose and Environment	4120

A.13.3.1.2.2	Test Requirements	4121

A.13.3.1.3	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4122

A.13.3.1.3.1	Test Purpose and Environment	4122

A.13.3.1.3.2	Test Requirements	4123

A.13.3.2	Random Access for Satellite Access	4123

A.13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage	4124

A.13.3.2.1.1	Test Purpose and Environment	4124

A.13.3.2.1.2	Test Requirements	4126

A.13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage	4128

A.13.3.2.2.1	Test Purpose and Environment	4128

A.13.3.2.2.2	Test Requirements	4129

A.13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage	4131

A.13.3.2.3.1	Test Purpose and Environment	4131

A.13.3.2.3.2	Test Requirements	4133

A.13.4	Timing and signalling characteristics for satellite access	4135

A.13.4.1	UE transmit timing for satellite access	4135

A.13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access	4135

A.13.4.1.1.1	Test Purpose and Environment	4135

A.13.4.1.1.2	Test Requirements	4136

A.13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access	4137

A.13.4.1.2.1	Test Purpose and Environment	4137

A.13.4.1.2.2	Test Requirements	4138

A.13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access	4139

A.13.4.1.3.1	Test Purpose and Environment	4139

A.13.4.1.3.2	Test Requirements	4140

A.13.4.2	UE timing advance for satellite access	4141

A.13.4.2.1	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access	4141

A.13.4.2.1.1	Test Purpose and Environment	4141

A.13.4.2.1.2	Test Requirements	4143

A.13.4.2.2	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access	4143

A.13.4.2.2.1	Test Purpose and Environment	4143

A.13.4.2.2.2	Test Requirements	4145

A.13.4.3	Radio Link Monitoring for satellite access	4145

A.13.4.3.1	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage	4145

A.13.4.3.1.1	Test Purpose and Environment	4145

A.13.4.3.1.2	Test Requirements	4148

A.13.4.3.2	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage	4148

A.13.4.3.2.1	Test Purpose and Environment	4148

A.13.4.3.2.2	Test Requirements	4151

A.13.4.3.3	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4151

A.13.4.3.3.1	Test Purpose and Environment	4151

A.13.4.3.3.2	Test Requirements	4154

A.13.4.3.4	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage	4154

A.13.4.3.4.1	Test Purpose and Environment	4154

A.13.4.3.4.2	Test Requirements	4157

A.13.4.3.5	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4157

A.13.4.3.5.1	Test Purpose and Environment	4157

A.13.4.3.5.2	Test Requirements	4160

A.13.4.3.6	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4160

A.13.4.3.6.1	Test Purpose and Environment	4160

A.13.4.3.6.2	Test Requirements	4163

A.13.4.3.7	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4163

A.13.4.3.7.1	Test Purpose and Environment	4163

A.13.4.3.7.2	Test Requirements	4165

A.13.4.3.8	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4166

A.13.4.3.8.1	Test Purpose and Environment	4166

A.13.4.3.8.2	Test Requirements	4168

A.13.5	UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access	4169

A.13.6	Measurement performance requirements for UE for satellite access	4176

A.13.6.1	Void	4176

A.13.6.2	Channel quality reporting accuracy for satellite access	4176

A.13.6.2.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4176

A.13.6.2.1.1	Test Purpose and Environment	4176

A.13.6.2.1.2	Test parameters	4176

A.13.6.2.1.3	Test Requirements	4177

A.13.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4177

A.13.6.2.2.1	Test Purpose and Environment	4177

A.13.6.2.2.2	Test parameters	4177

A.13.6.2.2.3	Test Requirements	4178

A.13.6.2.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4178

A.13.6.2.3.1	Test Purpose and Environment	4178

A.13.6.2.3.2	Test parameters	4179

A.13.6.2.3.3	Test Requirements	4179

A.13.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4180

A.13.6.2.4.1	Test Purpose and Environment	4180

A.13.6.2.4.2	Test parameters	4180

A.13.6.2.4.3	Test Requirements	4181

A.13.6.2.5	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage	4181

A.13.6.2.5.1	Test Purpose and Environment	4181

A.13.6.2.5.2	Test parameters	4181

A.13.6.2.5.3	Test Requirements	4182

A.13.6.2.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage	4183

A.13.6.2.6.1	Test Purpose and Environment	4183

A.13.6.2.6.2	Test parameters	4183

A.13.6.2.6.3	Test Requirements	4184

A.14	E-UTRAN Standalone Tests for UE Category M1 for Satellite Access	4184

A.14.1	RRC_IDLE state for satellite access	4184

A.14.1.1	Cell re-selection for satellite access	4184

A.14.1.1.1	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4184

A.14.1.1.1.1	Test Purpose and Environment	4184

A.14.1.1.1.2	Test Requirements	4186

A.14.1.1.2	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4187

A.14.1.1.2.1	Test Purpose and Environment	4187

A.14.1.1.2.2	Test Requirements	4189

A.14.1.1.3	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4190

A.14.1.1.3.1	Test Purpose and Environment	4190

A.14.1.1.3.2	Test Requirements	4192

A.14.1.1.4	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4193

A.14.1.1.4.1	Test Purpose and Environment	4193

A.14.1.1.4.2	Test Requirements	4195

A.14.1.1.5	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4196

A.14.1.1.5.1	Test Purpose and Environment	4196

A.14.1.1.5.2	Test Requirements	4198

A.14.1.1.6	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4199

A.14.1.1.6.1	Test Purpose and Environment	4199

A.14.1.1.6.2	Test Requirements	4201

A.14.1.1.7	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering	4202

A.14.1.1.7.1	Test Purpose and Environment	4202

A.14.1.1.7.2	Test Requirements	4203

A.14.1.1.8 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage, time-based triggering	4204

A.14.1.1.8.1	Test Purpose and Environment	4204

A.14.1.1.8.2	Test Requirements	4205

A.14.1.1.9	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering	4206

A.14.1.1.9.1	Test Purpose and Environment	4206

A.14.1.1.9.2	Test Requirements	4207

A.14.1.1.10	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering	4208

A.14.1.1.10.1	Test Purpose and Environment	4208

A.14.1.1.10.2	Test Requirements	4209

A.14.2	RRC_CONNECTED state mobility for satellite access	4210

A.14.2.1	E-UTRAN handover for satellite access	4210

A.14.2.1.1	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4210

A.14.2.1.1.1	Test Purpose and Environment	4210

A.14.2.1.1.2	Test Requirements	4212

A.14.2.1.2	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4213

A.14.2.1.2.1	Test Purpose and Environment	4213

A.14.2.1.2.2	Test Requirements	4214

A.14.2.1.3	E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4215

A.14.2.1.3.1	Test Purpose and Environment	4215

A.14.2.1.3.2	Test Requirements	4216

A.14.2.1.4	E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4217

A.14.2.1.4.1	Test Purpose and Environment	4217

A.14.2.1.4.2	Test Requirements	4218

A.14.2.1.5	E-UTRAN FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4219

A.14.2.1.5.1	Test Purpose and Environment	4219

A.14.2.1.5.2	Test Requirements	4220

14.2.1.6	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4221

A.14.2.1.6.1	Test Purpose and Environment	4221

A.14.2.1.6.2	Test Requirements	4222

A.14.2.1.7	E-UTRAN FD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4223

A.14.2.1.7.1	Test Purpose and Environment	4223

A.14.2.1.7.2	Test Requirements	4224

A.14.2.1.8	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4225

A.14.2.1.8.1	Test Purpose and Environment	4225

A.14.2.1.8.2	Test Requirements	4226

A.14.2.1.9	E-UTRAN FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4227

A.14.2.1.9.1	Test Purpose and Environment	4227

A.14.2.1.9.2	Test Requirements	4228

A.14.2.1.10	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4229

A.14.2.1.10.1	Test Purpose and Environment	4229

A.14.2.1.10.2	Test Requirements	4230

A.14.2.1.11	E-UTRAN FDD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4231

A.14.2.1.11.1	Test Purpose and Environment	4231

A.14.2.1.12.2	Test Requirements	4232

A.14.2.1.12	E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4233

A.14.2.1.12.1	Test Purpose and Environment	4233

A.14.2.1.12.2	Test Requirements	4234

A.14.2.1.15	E-UTRAN FDD-FDD Inter frequency location based conditional handover for Cat-M1 UEs in CEModeA	4239

A.14.2.1.15.1	Test Purpose and Environment	4239

A.14.2.1.16.2	Test Requirements	4240

A.14.2.1.16	E-UTRAN HD-FDD Inter frequency time based conditional handover for Cat-M1 UEs in CEModeA	4241

A.14.2.1.16.1	Test Purpose and Environment	4241

A.14.2.1.16.2	Test Requirements	4242

A.14.3	RRC connection mobility control for satellite access	4243

A.14.3.1	RRC re-establishment for satellite access	4243

A.14.3.1.1	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4243

A.14.3.1.1.1	Test Purpose and Environment	4243

A.14.3.1.1.2	Test Requirements	4244

A.14.3.1.2	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	4245

A.14.3.1.2.1	Test Purpose and Environment	4245

A.14.3.1.2.2	Test Requirements	4247

A.14.3.1.3	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4248

A.14.3.1.3.1	Test Purpose and Environment	4248

A.14.3.1.3.2	Test Requirements	4250

A.14.3.1.4	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4251

A.14.3.1.4.1	Test Purpose and Environment	4251

A.14.3.1.4.2	 Test Requirements	4253

A.14.3.2	Random access for satellite access	4254

A.14.3.2.1	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4254

A.14.3.2.1.1	Test Purpose and Environment	4254

A.14.3.2.1.2	Test Requirements	4256

A.14.3.2.2	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4257

A.14.3.2.2.1	Test Purpose and Environment	4257

A.14.3.2.2.2	Test Requirements	4259

A.14.3.2.3	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4261

A.14.3.2.3.1	Test Purpose and Environment	4261

A.14.3.2.3.2	Test Requirements	4263

A.14.3.2.4	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4264

A.14.3.2.4.1	Test Purpose and Environment	4264

A.14.3.2.4.2	Test Requirements	4266

A.14.4	Timing and signalling characteristics for satellite access	4268

A.14.4.1	UE transmit timing for satellite access	4268

A.14.4.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4268

A.14.4.1.1.1	Test Purpose and Environment	4268

A.14.4.1.1.2	Test Requirements	4270

A.14.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4271

A.14.4.1.2.1	Test Purpose and Environment	4271

A.14.4.1.2.2	Test Requirements	4272

A.14.4.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4273

A.14.4.1.3.1	Test Purpose and Environment	4273

A.14.4.1.3.2	Test Requirements	4274

A.14.4.1.4	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4275

A.14.4.1.4.1	Test Purpose and Environment	4275

A.14.4.1.4.2	Test Requirements	4276

A.14.4.1.5	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB with segment transmission in NGSO for Satellite Access	4277

A.14.4.1.5.1	Test Purpose and Environment	4277

A.14.4.1.5.2	Test Requirements	4278

A.14.4.2	UE timing advance for satellite access	4279

A.14.4.2.1	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4279

A.14.4.2.1.1	Test Purpose and Environment	4279

A.14.4.2.1.2	Test Requirements	4281

A.14.4.2.2	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4281

A.14.4.2.2.1	Test Purpose and Environment	4281

A.14.4.2.2.2	Test Requirements	4284

A.14.4.2.3	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4284

A.14.4.2.3.1	Test Purpose and Environment	4284

A.14.4.2.3.2	Test Requirements	4285

A.14.4.2.4	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4285

A.14.4.2.4.1	Test Purpose and Environment	4285

A.14.4.2.4.2	Test Requirements	4287

A.14.4.3	Radio Link Monitoring for satellite access	4288

A.14.4.3.1	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4288

A.14.4.3.1.1	Test Purpose and Environment	4288

A.14.4.3.1.2	Test Requirements	4291

A.14.4.3.2	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4291

A.14.4.3.2.1	Test Purpose and Environment	4291

A.14.4.3.2.2	Test Requirements	4294

A.14.4.3.3	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4294

A.14.4.3.3.1	Test Purpose and Environment	4294

A.14.4.3.3.2	Test Requirements	4297

A.14.4.3.4	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4297

A.14.4.3.4.1	Test Purpose and Environment	4297

A.14.4.3.4.2	Test Requirements	4300

A.14.4.3.5	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4300

A.14.4.3.5.1	Test Purpose and Environment	4300

A.14.4.3.5.2	 Test Requirements	4303

A.14.4.3.6	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4303

A.14.4.3.6.1	Test Purpose and Environment	4303

A.14.4.3.6.2	Test Requirements	4306

A.14.4.3.7	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4306

A.14.4.3.7.1	Test Purpose and Environment	4306

A.14.4.3.8	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4309

A.14.4.3.8.1 	Test Purpose and Environment	4309

A.14.4.3.8.2	Test Requirements	4312

A.14.5	UE measurement procedures in RRC_CONNECTED state for satellite access	4312

A.14.5.1 Intra-frequency measurements for satellite access	4312

A.14.5.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4312

A.14.5.1.1.1	Test Purpose and Environment	4312

A.14.5.1.1.2	Test Requirements	4314

A.14.5.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4315

A.14.5.1.2.1	Test Purpose and Environment	4315

A.14.5.1.2.2	Test Requirements	4317

A.14.5.1.3	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4317

A.14.5.1.3.1	Test Purpose and Environment	4317

A.14.5.1.3.2	Test Requirements	4319

A.14.5.1.4	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4320

A.14.5.1.4.1	Test Purpose and Environment	4320

A.14.5.1.4.2	Test Requirements	4322

A.14.5.1.5	E-UTRAN FD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4322

A.14.5.1.5.1	Test Purpose and Environment	4322

A.14.5.1.5.2	Test Requirements	4324

A.14.5.1.6 	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4325

A.14.5.1.6.1	Test Purpose and Environment	4325

A.14.5.1.6.2	Test Requirements	4326

A.14.5.1.7	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4327

A.14.5.1.7.1	Test Purpose and Environment	4327

A.14.5.1.7.2	Test Requirements	4328

A.14.5.2 Inter-frequency measurements for satellite access	4329

A.14.5.2.1	E-UTRAN FD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4329

A.14.5.2.1.1	Test Purpose and Environment	4329

A.14.5.2.1.2	Test Requirements	4330

A.14.5.2.2	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4331

A.14.5.2.2.1	Test Purpose and Environment	4331

A.14.5.2.2.2	Test Requirements	4332

A.14.5.2.3	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeB when DRX is used with time-based triggering	4333

A.14.5.2.3.1	Test Purpose and Environment	4333

A.14.5.2.3.2	Test Requirements	4334

A.14.5.2.4	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4335

A.14.5.2.4.1	Test Purpose and Environment	4335

A.14.5.2.4.2	Test Requirement	4336

A.14.5.2.5	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	4337

A.14.5.2.5.1	Test Purpose and Environment	4337

A.14.5.2.5.2	Test Requirement	4339

A.14.5.2.6	E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4339

A.14.5.2.6.1	Test Purpose and Environment	4339

A.14.5.2.6.2	Test Requirement	4341

A.14.5.2.7	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA in DRX	4341

A.14.5.2.7.1	Test Purpose and Environment	4341

A.14.5.2.7.2	Test Requirement	4343

A.14.5.2.8	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4343

A.14.5.2.8.1	Test Purpose and Environment	4343

A.14.5.2.8.2	Test Requirement	4345

A.14.5.2.9	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4346

A.14.5.2.9.1	Test Purpose and Environment	4346

A.14.5.2.9.2	Test Requirement	4347

A.14.6	Measurement performance requirements for UE for satellite access	4348

A.14.6.1	RSRP for satellite access	4348

A.14.6.1.1	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4348

A.14.6.1.1.1	Test Purpose and Environment	4348

A.14.6.1.1.2	Test parameters	4348

A.14.6.1.1.3	Test Requirements	4350

A.14.6.1.2	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4350

A.14.6.1.2.1	Test Purpose and Environment	4350

A.14.6.1.2.2	Test parameters	4350

A.14.6.1.2.3	Test Requirements	4352

A.14.6.1.3	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4352

A.14.6.1.3.1	Test Purpose and Environment	4352

A.14.6.1.3.2	Test parameters	4352

A.14.6.1.3.3	Test Requirements	4353

A.14.6.1.4	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4354

A.14.6.1.4.1	Test Purpose and Environment	4354

A.14.6.1.4.2	Test parameters	4354

A.14.6.1.4.3	Test Requirements	4355

A.14.6.1.5	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4355

A.14.6.1.5.1	Test Purpose and Environment	4355

A.14.6.1.5.2	Test parameters	4355

A.14.6.1.5.3	Test Requirements	4356

A.14.6.1.6	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4356

A.14.6.1.6.1	Test Purpose and Environment	4356

A.14.6.1.6.2	Test parameters	4357

A.14.6.1.6.3	Test Requirements	4358

A.14.6.2	Channel quality reporting accuracy for satellite access	4358

A.14.6.2.1	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4358

A.14.6.2.1.1	Test Purpose and Environment	4358

A.14.6.2.1.2	Test parameters	4358

A.14.6.2.1.3	Test Requirements	4359

A.14.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4360

A.14.6.2.2.1	Test Purpose and Environment	4360

A.14.6.2.2.2	Test parameters	4360

A.14.6.2.2.3	Test Requirements	4361

A.14.6.2.3.2	Test parameters	4361

A.14.6.2.3.3	Test Requirements	4362

A.14.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B for Satellite access	4363

A.14.6.2.4.1	Test Purpose and Environment	4363

A.14.6.2.4.2	Test parameters	4363

A.14.6.2.4.3	Test Requirements	4364

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	4365

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	4365

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	4365

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	4365

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	4365

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	4367

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	4368

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	4368

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	4368

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	4368

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	4370

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	4371

B.1.9	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4371

B.1.10	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1 and NB2 for satellite access	4372

B.1.11	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1 for satellite access	4373

B.1.12	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4373

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	4373

B.2.1	Conditions for E-UTRAN intra-frequency measurements	4373

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	4374

B.2.3	Conditions for E-UTRAN inter-frequency measurements	4374

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	4375

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	4375

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	4375

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	4376

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	4376

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4377

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	4377

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	4377

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	4377

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	4378

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4378

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4378

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	4379

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	4379

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4379

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4380

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	4380

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	4381

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	4382

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	4383

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	4384

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	4385

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	4385

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	4385

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	4385

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	4386

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	4386

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	4388

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4388

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4388

B.2.26	Conditions for E-UTRAN intra-frequency measurements by UE Category M1 for satellite access	4388

B.2.27	Conditions for NB-IoT intra-frequency measurements by UE Category NB1 and NB2 for satellite access	4389

B.2.28	Conditions for E-UTRAN inter-frequency measurements by UE Category M1 for satellite access	4390

B.2.29	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4390

B.2.30	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4391

B.3	Conditions for measurements performance requirements for UE	4391

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	4391

B.3.2	Void	4391

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	4392

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	4392

B.3.5	Conditions for UE Rx – Tx time difference	4392

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	4392

B.3.7	Conditions for inter-frequency RSTD measurements	4392

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	4392

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	4393

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	4393

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4393

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4393

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	4393

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4393

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	4393

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	4394

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4394

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	4394

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	4394

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4395

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	4395

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	4395

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4395

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	4395

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	4395

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	4396

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	4396

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	4396

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4396

B.3.21.1	Conditions for RSRP measurements	4396

B.3.21.2	Conditions for RSRQ measurements	4396

B.3.21.3	Conditions for CSI-RSRP measurements	4396

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4397

B.3.22.1	Conditions for RSRP measurements	4397

B.3.22.2	Void	4397

B.3.22.3	Conditions for CSI-RSRP measurements	4397

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4397

B.3.23.1	Conditions for RSRP measurements	4397

B.3.23.2	Conditions for RSRQ measurements	4398

B.3.23.3	Conditions for CSI-RSRP measurements	4398

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4398

B.3.24.1	Conditions for RSRP measurements	4398

B.3.24.2	Conditions for RSRQ measurements	4398

B.3.24.3	Conditions for CSI-RSRP measurements	4398

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4398

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4399

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	4399

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	4399

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4400

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4400

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4400

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4400

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4400

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4400

B.4	RRM Requirements Exceptions	4401

B.4.1	General	4401

B.4.2	Receiver sensitivity relaxation for UE supporting CA	4401

B.4.3	Receiver sensitivity relaxation for UE configured with CA	4401

B.4.3.1	Inter-band carrier aggregation	4401

B.4.3.2	Intra-band non-contiguous carrier aggregation	4401

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	4401

B.5	Conditions for Measurement Performance Requirements for ProSe UE	4402

B.5.1	Conditions for S-RSRP Accuracy Requirements	4402

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	4402

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4402

B.5.4	Conditions for SD-RSRP Accuracy Requirements	4403

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	4403

B.6	Conditions for V2X	4404

B.6.1	Test parameters for GNSS signals	4404

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	4404

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	4404

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4404

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	4405

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	4405

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	4405

B.8	High level test procedure for SAN RRM tests	4407

Annex C (informative):	Change history:	4407