| 3GPP TS 38.101-1 V19.4.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	24

1	Scope	26

2	References	26

3	Definitions, symbols and abbreviations	27

3.1	Definitions	27

3.2	Symbols	28

3.3	Abbreviations	31

4	General	32

4.1	Relationship between minimum requirements and test requirements	32

4.2	Applicability of minimum requirements	33

4.3	Specification suffix information	33

5	Operating bands and channel arrangement	34

5.1	General	34

5.2	Operating bands	35

5.2A	Operating bands for CA	37

5.2A.0	General	37

5.2A.1	Intra-band CA	37

5.2A.2	Inter-band CA	38

5.2A.2.1	Inter-band CA (two bands)	38

5.2A.2.2	Inter-band CA (three bands)	43

5.2A.2.3	Inter-band CA (four bands)	49

5.2A.2.4	Inter-band CA (five bands)	53

5.2A.2.5	Inter-band CA (six bands)	53

5.2B	Operating bands for DC	54

5.2C	Operating band combination for SUL	54

5.2D	Operating bands for UL MIMO	56

5.2E	Operating band for V2X	57

5.2E.1	V2X operating bands	57

5.2E.1A	Sidelink CA operating bands	58

5.2E.1F	Operating bands for Sidelink Unlicensed	58

5.2E.2	V2X operating bands for concurrent operation	58

5.2E.2F	Operating bands for SL-U concurrent operation	59

5.2J	Operating band for ATG	59

5.2J.1	General	59

5.2J.1A	Operating band for ATG CA	59

5.2J.1A.1	Operating band for ATG intra-band CA	59

5.2J.1A.2	Operating band for ATG inter-band CA	59

5.2J.1D	Operating band for ATG UL MIMO	60

5.2K	Operating bands for Aerial UE	60

5.2M	Operating bands for LP-WUS/WUR	60

5.3	UE channel bandwidth	60

5.3.1	General	60

5.3.2	Maximum transmission bandwidth configuration	62

5.3.3	Minimum guardband and transmission bandwidth configuration	62

5.3.4	RB alignment	64

5.3.5	UE channel bandwidth per operating band	64

5.3.6	Asymmetric channel bandwidths	71

5.3A	UE channel bandwidth for CA	72

5.3A.1	General	72

5.3A.2	Maximum transmission bandwidth configuration for CA	72

5.3A.3	Minimum guardband and transmission bandwidth configuration for CA	72

5.3A.4	Void	74

5.3A.5	UE channel bandwidth per operating band for CA	74

5.3E	Channel bandwidth for V2X	75

5.3E.1	General	75

5.3E.1A	Channel bandwidth for Sidelink CA	75

5.3E.1F	Channel bandwidth for Sidelink Unlicensed	76

5.3E.2	Channel bandwidth for V2X concurrent operation	77

5.3E.2F	Channel bandwidth for SL-U concurrent operation	78

5.3I	Channel bandwidth for (e)RedCap	79

5.3M	UE channel bandwidth for LP-WUS/WUR	79

5.3M.1	General	79

5.3M.2	Maximum transmission bandwidth	79

5.4	Channel arrangement	79

5.4.1	Channel spacing	79

5.4.1.1	Channel spacing for adjacent NR carriers	79

5.4.2	Channel raster	80

5.4.2.1	NR-ARFCN and channel raster	80

5.4.2.2	Channel raster to resource element mapping	80

5.4.2.3	Channel raster entries for each operating band	81

5.4.3	Synchronization raster	84

5.4.3.1	Synchronization raster and numbering	85

5.4.3.3	Synchronization raster entries for each operating band	86

5.4.4	TX–RX frequency separation	88

5.4A	Channel arrangement for CA	89

5.4A.1	Channel spacing for CA	89

5.4A.2	Channel raster for CA	89

5.4A.3	Synchronization raster for CA	89

5.4A.4	Tx-Rx frequency separation for CA	90

5.4B	Reserved	90

5.4C	Reserved	90

5.4D	Reserved	90

5.4E	Channel arrangement for V2X	90

5.4E.1	Channel spacing	90

5.4E.1A	Channel spacing for Sidelink CA	90

5.4E.1F	Channel spacing for Sidelink Unlicensed	90

5.4E.2	Channel raster	90

5.4E.2.1	NR-ARFCN and channel raster	90

5.4E.2.1A	Void	90

5.4E.2.1F	Void	90

5.4E.2.2	Channel raster to resource element mapping	90

5.4E.2.2A	Void	91

5.4E.2.2F	Void	91

5.4E.2.3	Channel raster entries for each operating band	91

5.4E.2.3A	Void	91

5.4E.2.3F	Void	91

5.4E.3	Synchronization raster for V2X	92

5.4E.3A	Synchronization raster for Sidelink CA	92

5.4E.3F	Synchronization raster for Sidelink Unlicensed	92

5.4I	Channel arrangement for (e)RedCap	92

5.4I.1	Channel spacing for (e)RedCap	92

5.4I.2	Channel raster for (e)RedCap	92

5.4I.2.1	NR-ARFCN and channel raster	92

5.4I.2.2	Channel raster to resource element mapping	92

5.4I.2.3	Channel raster entries for each operating band	92

5.4I.3	Synchronization raster for (e)RedCap	92

5.4I.4	Tx-Rx frequency separation for (e)RedCap	92

5.5	Void	90

5.5A	Configurations for CA	90

5.5A.0	General	90

5.5A.1	Configurations for intra-band contiguous CA	91

5.5A.2	Configurations for intra-band non-contiguous CA	95

5.5J	Configurations for ATG	100

5.5J.1A	Configurations for ATG CA	100

5.5J.1A.1	Configurations for ATG intra-band contiguous CA	100

5.5J.1A.2	Configurations for ATG inter-band CA	100

5.5A.3	Configurations for inter-band CA	99

5.5A.3.0	General	99

5.5A.3.1	Configurations for inter-band CA (two bands)	100

Table 5.5A.3.1-1a ~ Table 5.5A.3.1-1e	100

Table 5.5A.3.1-1f ~ Table 5.5A.3.1-1j	122

Table 5.5A.3.1-1k ~ Table 5.5A.3.1-1o	146

5.5A.3.2	Configurations for inter-band CA (three bands)	164

Table 5.5A.3.2-1a	164

Table 5.5A.3.2-1b	255

Table 5.5A.3.2-1c	308

5.5A.3.3	Configurations for inter-band CA (four bands)	334

Table 5.5A.3.3-1a	334

Table 5.5A.3.3-1b	421

5.5A.3.4	Configurations for inter-band CA (five bands)	481

5.5A.3.5	Configurations for inter-band CA (six bands)	517

5.5B	Configurations for DC	454

5.5C	Configurations for SUL	468

5.5D	Reserved	481

5.5E	Configurations for Sidelink	481

5.5E.1A	Configurations for Sidelink CA	481

5.5E.1A.1	Configurations for Sidelink intra-band contiguous CA	481

5.5E.1A.2	Configurations for Sidelink intra-band non-contiguous CA	481

6	Transmitter characteristics	480

6.1	General	480

6.1A	General	480

6.1F	General	480

6.1G	(Reserved)	480

6.1H	(Reserved)	480

6.1I	(Reserved)	480

6.1J	General	480

6.1K	(Reserved)	481

6.1L	(Reserved)	481

6.2	Transmitter power	481

6.2.1	UE maximum output power	481

6.2.1I	Void	483

6.2.2	UE maximum output power reduction	483

6.2.3	UE additional maximum output power reduction	487

6.2.3.1	General	487

6.2.3.2	A-MPR for NS_04	491

6.2.3.3	A-MPR for NS_10	494

6.2.3.4	A-MPR for NS_05 and NS_05U	495

6.2.3.5	A-MPR for NS_40	498

6.2.3.6	A-MPR for NS_43 and NS_43U	499

6.2.3.7	A-MPR for NS_03 and NS_03U	501

6.2.3.8	A-MPR for NS_37	502

6.2.3.9	A-MPR for NS_38	503

6.2.3.10	A-MPR for NS_39	503

6.2.3.11	A-MPR for NS_41	504

6.2.3.12	A-MPR for NS_42	504

6.2.3.13	A-MPR for NS_18	505

6.2.3.14	A-MPR for NS_21	507

6.2.3.15	A-MPR for NS_24	509

6.2.3.16	A-MPR for NS_27	510

6.2.3.17	A-MPR for NS_46	512

6.2.3.18	A-MPR for NS_47	517

6.2.3.19	A-MPR for NS_50	519

6.2.3.20	A-MPR for NS_44	522

6.2.3.21	A-MPR for NS_12	523

6.2.3.22	A-MPR for NS_13	524

6.2.3.23	A-MPR for NS_14	525

6.2.3.24	A-MPR for NS_15	526

6.2.3.25	A-MPR for NS_45	529

6.2.3.26	A-MPR for NS_48	530

6.2.3.27	A-MPR for NS_49	532

6.2.3.28	A-MPR for NS_51	536

6.2.3.29	A-MPR for NS_07	537

6.2.3.30	A-MPR for NS_56	538

6.2.3.31	A-MPR for NS_35	539

6.2.3.32	A-MPR for NS_06	539

6.2.3.33	A-MPR for NS_17	540

6.2.3.34	A-MPR for NS_26	541

6.2.3.35	A-MPR for NS_36	541

6.2.4	Configured transmitted power	542

6.2A	Transmitter power for CA	548

6.2A.0	General	548

6.2A.1	UE maximum output power for CA	549

6.2A.1.1	UE maximum output power for Intra-band contiguous CA	549

6.2A.1.2	UE maximum output power for Intra-band non-contiguous CA	550

6.2A.1.3	UE maximum output power for Inter-band CA	552

6.2A.1.4	Void	558

6.2A.1.5	Void	558

6.2A.2	UE maximum output power reduction for CA	558

6.2A.2.1	UE maximum output power reduction for Intra-band contiguous CA	558

6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA	564

6.2A.2.2.0	General	564

6.2A.2.2.1	MPR to meet -30dBm/MHz	565

6.2A.2.2.2	MPR to meet -13dBm/MHz	567

6.2A.2.3	UE maximum output power reduction for Inter-band CA	569

6.2A.2.4	Void	569

6.2A.3	UE additional maximum output power reduction for CA	569

6.2A.3.1	UE additional maximum output power reduction for Intra-band CA	569

6.2A.3.1.1	UE additional maximum output power reduction for Intra-band contiguous CA	569

6.2A.3.1.2	UE additional maximum output power reduction for Intra-band non-contiguous CA	577

6.2A.3.1.3	UE additional maximum output power reduction for Inter-band CA	580

6.2A.4	Configured output power for CA	583

6.2A.4.1	Configured transmitted power level	583

6.2A.4.1.1	Configured transmitted power for Intra-band contiguous CA	583

6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA	585

6.2A.4.1.3	Configured transmitted power for Inter-band CA	588

6.2A.4.1.4	Void	592

6.2A.4.2	ΔTIB,c for CA	592

6.2A.4.2.1	Void	592

6.2A.4.2.2	Void	592

6.2A.4.2.3	ΔTIB,c for Inter-band CA (two bands)	592

6.2A.4.2.4	ΔTIB,c for Inter-band CA (three bands)	597

6.2A.4.2.5	ΔTIB,c for Inter-band CA (four bands)	604

6.2A.4.2.6	ΔTIB,c for Inter-band CA (five bands)	608

6.2A.4.2.7	ΔTIB,c for Inter-band CA (six bands)	609

6.2B	Transmitter power for NR-DC	609

6.2B.0	General	609

6.2B.1	UE maximum output power for NR-DC	609

6.2B.2	UE maximum output power reduction for NR-DC	611

6.2B.3	UE additional maximum output power reduction for NR-DC	611

6.2B.4.1	Configured transmitted power level for NR-DC	612

6.2B.4.2	ΔTIB,c for NR-DC	616

6.2C	Transmitter power for SUL	616

6.2C.1	Configured transmitted power for SUL	616

6.2C.2	ΔTIB,c	617

6.2D	Transmitter power for UL MIMO	619

6.2D.1	UE maximum output power for UL MIMO	619

6.2D.2	UE maximum output power reduction for UL MIMO	621

6.2D.3	UE additional maximum output power reduction for UL MIMO	624

6.2D.4	Configured transmitted power for UL MIMO	624

6.2E	Transmitter power for V2X	626

6.2E.1	UE maximum output power for V2X	626

6.2E.1.1	General	626

6.2E.1.1A	Void	627

6.2E.1.2	UE maximum output power for V2X concurrent operation	627

6.2E.1A	UE maximum output power for Sidelink CA	628

6.2E.1F	UE maximum output power for Sidelink Unlicensed	628

6.2E.1F.1	General	628

6.2E.1.2F	Void	630

6.2E.1F.2	UE Maximum output power for SL-U concurrent operation	630

6.2E.2	UE maximum output power reduction for V2X	630

6.2E.2.1	General	630

6.2E.2.1A	MPR for sidelink CA	630

6.2E.2.1A.1	MPR for sidelink intra-band contiguous CA	630

6.2E.2.1A.2	MPR for sidelink intra-band non-contiguous CA	633

6.2E.2.1A.2.0	General	633

6.2E.2.1A.2.1	MPR with indicating dualPA-Architecture supported	634

6.2E.2.1A.2.2	MPR without indicating dualPA-Architecture supported	635

6.2E.2.2	MPR for Power class 2 and Power class 3 V2X UE	636

6.2E.2.3	MPR for Power class 2 and Power class 3 V2X concurrent operation	638

6.2E.2.4	MPR for Power class 1 UE in Band n14	641

6.2E.2F	UE maximum output power reduction for Sidelink Unlicensed	641

6.2E.2F.1	General	641

6.2E.2F.2	MPR for NR SL-U UE	641

6.2E.2F.3	MPR for SL-U concurrent operation	643

6.2E.3	UE additional maximum output power reduction for V2X	643

6.2E.3.1	General	643

6.2E.3.2	A-MPR for V2X UE by NS_33	644

6.2E.3.2A	A-MPR for sidelink CA by NS_33	649

6.2E.3.2A.1	A-MPR for sidelink intra-band non-contiguous CA	649

6.2E.3.2A.1.1	A-MPR for SLCA_NC_NS_33 (SLCA_n47(2A))	649

6.2E.3.3	A-MPR for Power class 3 V2X UE by NS_52	653

6.2E.3.4	A-MPR for V2X concurrent operation	655

6.2E.3F	UE additional maximum output power reduction for Sidelink Unlicensed	655

6.2E.3F.1	General	655

6.2E.3F.2	A-MPR for NS_31	656

6.2E.3F.3	Void	657

6.2E.3F.4	Void	657

6.2E.3F.5	Void	657

6.2E.3F.6	A-MPR for NS_61	657

6.2E.3F.7	A-MPR for SL-U concurrent operation	658

6.2E.3F.8	A-MPR for NS_28	659

6.2E.3F.9	A-MPR for NS_29	660

6.2E.3F.10	A-MPR for NS_30	661

6.2E.3F.11	A-MPR for NS_54	663

6.2E.3F.12	A-MPR for NS_64	664

6.2E.3F.13	A-MPR for NS_65	666

6.2E.3F.14	A-MPR for NS_66	667

6.2E.3F.15	A-MPR for NS_67 or NS_71	668

6.2E.3F.16	A-MPR for NS_68	669

6.2E.3F.17	A-MPR for NS_69	670

6.2E.4	Configured transmitted power for V2X	671

6.2E.4.1	General	671

6.2E.4.2	Configured transmitted power for inter-band V2X concurrent operation	672

6.2E.4.3	Configured transmitted power for intra-band V2X concurrent operation	673

6.2E.4A	Configured transmitted power for Sidelink CA	675

6.2E.4F	Configured transmitted power for Sidelink Unlicensed	676

6.2E.4F.1	General	676

6.2E.4F.2	Configured transmitted power for inter-band concurrent operation	677

6.2F	Transmitter power for shared spectrum channel access	678

6.2F.1	UE maximum output power	678

6.2F.1A	UE maximum output power for CA	679

6.2F.1A.1	UE maximum output power for inter-band CA	679

6.2F.1A.2	UE maximum output power for intra-band contiguous CA	680

6.2F.1A.2.1	Additional requirements for transmit power density for intra-band contiguous CA for CA_NS_53	680

6.2F.1A.2.2	Additional requirements for transmit power density for intra-band contiguous CA for CA_NS_54	680

6.2F.1B	UE maximum output power for NR-DC	681

6.2F.1D	UE maximum output power for UL MIMO	681

6.2F.2	UE maximum output power reduction	681

6.2F.2A	UE maximum output power reduction for CA	684

6.2F.2A.1	UE maximum output power reduction for inter-band CA	684

6.2F.2A.2	UE maximum output power reduction for intra-band contiguous CA	684

6.2F.2D	UE maximum output power reduction for UL MIMO	687

6.2F.3	UE additional maximum output power reduction	687

6.2F.3.1	General	687

6.2F.3.2	A-MPR for NS_28	688

6.2F.3.3	A-MPR for NS_29	689

6.2F.3.4	A-MPR for NS_30	690

6.2F.3.5	A-MPR for NS_31	691

6.2F.3.6	A-MPR for NS_53	692

6.2F.3.7	A-MPR for NS_54	692

6.2F.3.8	A-MPR for NS_58	693

6.2F.3.9	A-MPR for NS_59	694

6.2F.3.10	A-MPR for NS_60	695

6.2F.3.11	A-MPR for NS_61	696

6.2F.3.12	A-MPR for NS_63	697

6.2F.3.13	A-MPR for NS_64	698

6.2F.3.14	A-MPR for NS_65	698

6.2F.3.15	A-MPR for NS_66	699

6.2F.3.16	A-MPR for "NS_67" or "NS_71"	699

6.2F.3.17	A-MPR for NS_68	700

6.2F.3.18	A-MPR for NS_69	700

6.2F.3A	UE additional maximum output power reduction for CA	701

6.2F.3A.1	UE additional maximum output power reduction for inter-band CA	701

6.2F.3A.2	UE additional maximum output power reduction for intra-band CA	701

6.2F.3A.2.0	General	701

6.2F.3A.2.1	UE additional maximum output power reduction for intra-band contiguous CA	701

6.2F.3A.2.2	A-MPR for CA_NS_53	703

6.2F.3A.2.3	A-MPR for CA_NS_54	704

6.2F.3D	UE additional maximum output power reduction for UL MIMO	704

6.2F.4	Configured transmitted power	704

6.2F.4D	Configured transmitted power UL MIMO	704

6.2G	Transmitter power for Tx Diversity	705

6.2G.1	UE maximum output power for Tx Diversity	705

6.2G.2	UE maximum output power reduction for Tx Diversity	705

6.2G.3	UE additional maximum output power reduction for Tx Diversity	705

6.2G.4	Configured transmitted power for Tx Diversity	705

6.2H	Transmitter power for CA with UL MIMO	706

6.2H.1	Transmitter power for intra-band UL contiguous CA with UL MIMO	706

6.2H.1.1	UE maximum output power for intra-band UL contiguous CA with UL MIMO	706

6.2H.1.2	UE maximum output power reduction for intra-band UL contiguous CA with UL MIMO	707

6.2H.1.3	UE additional maximum output power reduction for intra-band UL contiguous CA with UL MIMO	707

6.2H.1.4	Configured transmitted power for intra-band UL contiguous CA with UL MIMO	707

6.2H.2	Void	708

6.2H.3	Transmitter power for inter-band UL CA with UL MIMO	708

6.2H.3.1	UE maximum output power for inter-band UL CA with UL MIMO	708

6.2H.3.2	UE maximum output power reduction for inter-band UL CA with UL MIMO	711

6.2H.3.3	UE additional maximum output power reduction for inter-band UL CA with UL MIMO	711

6.2H.3.4	Configured transmitted power for inter-band UL CA with UL MIMO	711

6.2I	Transmitter power for (e)RedCap	711

6.2I.1	Maximum output power for RedCap	711

6.2J	Transmitter power for ATG	712

6.2J.0	Reserved	712

6.2J.0D	General	712

6.2J.1	UE maximum output power for ATG	712

6.2J.1A	UE maximum output power for ATG CA	712

6.2J.1A.1	UE maximum output power for ATG intra-band contiguous CA	712

6.2J.1A.2	UE maximum output power for ATG inter-band CA	712

6.2J.1A.3	ΔTIB,c for ATG CA	712

6.2J.1A.3.1	ΔTIB,c for Inter-band ATG CA (two bands)	712

6.2J.1D	UE maximum output power for ATG UL MIMO	712

6.2J.2	Configured transmitted power for ATG	713

6.2J.2D	Configured transmitted power for UL MIMO	714

6.2K	Transmitter power for Aerial UE	715

6.2K.1	Maximum output power for Aerial UE	715

6.2K.2	Maximum output power reduction for Aerial UE	715

6.2K.3	Additional maximum output power reduction for Aerial UE	715

6.2K.3.1	General	715

6.2K.3.2	A-MPR for NS_UAV_44	716

6.2K.3.3	A-MPR for NS_UAV_70	717

6.2K.4	Configured transmitted power for Aerial UE	718

6.2L	Transmitter power for CA with Tx Diversity	718

6.2L.1	Void	718

6.2L.2	Void	718

6.2L.3	Transmitter power for inter-band UL CA with Tx Diversity	718

6.2L.3.1	UE maximum output power for inter-band UL CA with Tx Diversity	718

6.2L.3.2	UE maximum output power reduction for inter-band UL CA with Tx Diversity	720

6.2L.3.3	UE additional maximum output power reduction for inter-band UL CA with Tx Diversity	720

6.2L.3.4	Configured transmitted power for inter-band UL CA with Tx Diversity	720

6.3	Output power dynamics	721

6.3.1	Minimum output power	721

6.3.2	Transmit OFF power	721

6.3.3	Transmit ON/OFF time mask	721

6.3.3.1	General	721

6.3.3.2	General ON/OFF time mask	722

6.3.3.3	Transmit power time mask for slot and short or long subslot boundaries	722

6.3.3.4	PRACH time mask	722

6.3.3.5	Void	723

6.3.3.6	SRS time mask	723

6.3.3.7	PUSCH-PUCCH and PUSCH-SRS time masks	725

6.3.3.8	Transmit power time mask for consecutive slot or long subslot transmission and short subslot transmission boundaries	726

6.3.3.9	Transmit power time mask for consecutive short subslot transmissions boundaries	727

6.3.4	Power control	727

6.3.4.1	General	727

6.3.4.2	Absolute power tolerance	727

6.3.4.3	Relative power tolerance	728

6.3.4.4	Aggregate power tolerance	728

6.3A	Output power dynamics for CA	729

6.3A.1	Minimum output power for CA	729

6.3A.1.1	Minimum output power for intra-band contiguous CA	729

6.3A.1.2	Minimum output power for intra-band non-contiguous CA	729

6.3A.1.3	Minimum output power for inter-band CA	729

6.3A.1.4	Void	729

6.3A.2	Transmit OFF power for CA	729

6.3A.2.1	Transmit OFF power for intra-band contiguous CA	729

6.3A.2.2	Transmit OFF power for intra-band non-contiguous CA	729

6.3A.2.3	Transmit OFF power for inter-band CA	729

6.3A.2.4	Void	730

6.3A.3	Transmit ON/OFF time mask for CA	730

6.3A.3.1	Transmit ON/OFF time mask for intra-band contiguous CA	730

6.3A.3.2	Transmit ON/OFF time mask for intra-band non-contiguous CA	730

6.3A.3.3	Transmit ON/OFF time mask for inter-band CA	730

6.3A.3.3.1	General	730

6.3A.3.3.2	Time mask for switching between two uplink carriers	731

6.3A.3.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors	732

6.3A.3.3.4	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	733

6.3A.3.3.5	Time mask for switching between two uplink bands with two transmit antenna connectors	735

6.3A.3.3.6	Time mask for switching across up to four uplink bands	736

6.3A.3.3.6a	Additional requirements for three-band switching with dual TAG	740

6.3A.3.3.7	Time mask for low NR band carrier aggregation via switching	740

6.3A.3.3.8	Time mask for switching between two uplink bands with three transmit antenna connectors and maximum two transmit antenna connectors for each band	740

6.3A.3.4	Void	742

6.3A.4	Power control for CA	742

6.3A.4.1	Power control for intra-band contiguous CA	742

6.3A.4.1.1	Absolute power tolerance	742

6.3A.4.1.2	Relative power tolerance	742

6.3A.4.1.3	Aggregate power control tolerance	742

6.3A.4.2	Power control for intra-band non-contiguous CA	743

6.3A.4.2.1	Absolute power tolerance	743

6.3A.4.2.1.1	 Minimum requirements	743

6.3A.4.2.2	Relative power tolerance	743

6.3A.4.2.2.1	Minimum requirements	743

6.3A.4.2.3	Aggregate power control tolerance	743

6.3A.4.3	Power control for inter-band CA	743

6.3A.4.4	Void	743

6.3B	Output power dynamics for NR-DC	743

6.3C	Output power dynamics for SUL	744

6.3C.1	Void	744

6.3C.2	Void	744

6.3C.3	Transmit ON/OFF time mask for SUL	744

6.3C.3.0	General	744

6.3C.3.1	Time mask for switching between two uplink carriers	744

6.3C.3.2	Time mask for switching between two uplink carriers with two transmit antenna connectors	745

6.3C.3.3	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	746

6.3C.3.4	Time mask for switching between two uplink bands with two transmit antenna connectors	748

6.3C.3.5	Time mask for switching across up to four uplink bands	749

6.3C.3.5a	Additional requirements for three-band switching with dual TAG	753

6.3D	Output power dynamics for UL MIMO	753

6.3D.1	Minimum output power for UL MIMO	753

6.3D.2	Transmit OFF power for UL MIMO	753

6.3D.3	Transmit ON/OFF time mask for UL MIMO	753

6.3D.4	Power control for UL MIMO	753

6.3E	Output power dynamics for V2X	754

6.3E.1	Minimum output power for V2X	754

6.3E.1.1	General	754

6.3E.1.1A	Minimum output power for sidelink CA	754

6.3E.1.2	Minimum output power for V2X concurrent operation	754

6.3E.1F	Minimum output power for Sidelink Unlicensed	754

6.3E.1F.1	Minimum output power for SL-U concurrent operation	755

6.3E.2	Transmit OFF power for V2X	755

6.3E.2.1	General	755

6.3E.2.1A	Transmit OFF power for sidelink CA	755

6.3E.2.2	Transmit OFF power for V2X concurrent operation	755

6.3E.2F	Transmit OFF power for Sidelink Unlicensed	755

6.3E.2F.1	Transmit OFF power for SL-U concurrent operation	756

6.3E.3	Transmit ON/OFF time mask for V2X	756

6.3E.3.1	General	756

6.3E.3.1A	Transmit ON/OFF time mask for sidelink CA	756

6.3E.3.2	General time mask	756

6.3E.3.3	S-SSB time mask	756

6.3E.3.4	Transmit ON/OFF time mask for V2X concurrent operation	757

6.3E.3F	Transmit ON/OFF time mask for Sidelink Unlicensed	758

6.3E.3F.1	General	758

6.3E.3F.2	General ON/OFF time mask	758

6.3E.3F.3	S-SSB time mask	759

6.3E.3F.4	Transmit ON/OFF time mask for NR SL-U concurrent operation	759

6.3E.4	Power control for V2X	759

6.3E.4.1	General	759

6.3E.4.1A	Power control for sidelink CA	759

6.3E.4.2	Absolute power tolerance	759

6.3E.4.3	Power control for V2X concurrent operation	760

6.3E.4F	Power control for Sidelink Unlicensed	760

6.3E.4F.1	General	760

6.3E.4F.2	Absolute power tolerance	760

6.3E.4F.3	Power control for SL-U concurrent operation	760

6.3F	Output power dynamics for shared spectrum channel access	760

6.3F.1	Minimum output power	760

6.3F.2	Transmit OFF power	760

6.3F.3	Transmit ON/OFF time mask	760

6.3F.3.1	General	760

6.3F.3.2	General ON/OFF time mask	760

6.3F.3A	General ON/OFF mask for CA	761

6.3F.3A.1	General ON/OFF mask for inter-band CA	761

6.3F.4	Power control	761

6.3F.4.1	General	761

6.3F.4.2	Absolute power tolerance	761

6.3F.4.3	Relative power tolerance	761

6.3F.4.4	Aggregate power tolerance	761

6.3F.4A	Power control for inter-band CA	761

6.3G	Output power dynamics for Tx Diversity	762

6.3G.1	Minimum output power for Tx Diversity	762

6.3G.2	Transmit OFF power for Tx Diversity	762

6.3G.3	Transmit ON/OFF time mask for Tx Diversity	762

6.3G.4	Power control for Tx Diversity	762

6.3H	Output power dynamics for CA with UL MIMO	762

6.3H.1	Output power dynamics for intra-band UL contiguous CA with UL MIMO	762

6.3H.1.1	Minimum output power for intra-band UL contiguous CA with UL MIMO	762

6.3H.1.2	Transmit OFF power for intra-band UL contiguous CA with UL MIMO	762

6.3H.1.3	Transmit ON/OFF time mask for intra-band UL contiguous CA with UL MIMO	762

6.3H.1.4	Power control for intra-band UL contiguous CA with UL MIMO	763

6.3H.3	Output power dynamics for inter-band UL CA with UL MIMO	763

6.3H.3.1	Minimum output power for inter-band UL CA with UL MIMO	763

6.3H.3.2	Transmit OFF power for inter-band UL CA with UL MIMO	763

6.3H.3.3	Transmit ON/OFF time mask for inter-band UL CA with UL MIMO	763

6.3H.3.4	Power control for inter-band UL CA with UL MIMO	763

6.3I	(Reserved)	763

6.3J	Output power dynamics for ATG	763

6.3J.1	Minimum output power for ATG	763

6.3J.1D	Minimum output power for ATG UL MIMO	764

6.3J.2	Transmit OFF power for ATG	764

6.3J.2D	Transmit OFF power for ATG UL MIMO	764

6.3J.3	Transmit ON/OFF time mask for ATG	764

6.3J.3D	Transmit ON/OFF time mask for ATG UL MIMO	764

6.3J.4	Power control for ATG	765

6.3J.4D	Power control for ATG UL MIMO	765

6.3K	(Reserved)	765

6.3L	Output power dynamics for CA with Tx Diversity	765

6.3L.1	Void	765

6.3L.2	Void	765

6.3L.3	Output power dynamics for inter-band UL CA with Tx Diversity	765

6.3L.3.1	Minimum output power for inter-band UL CA with Tx Diversity	765

6.3L.3.2	Transmit OFF power for inter-band UL CA with Tx Diversity	765

6.3L.3.3	Transmit ON/OFF time mask for inter-band UL CA with Tx Diversity	765

6.3L.3.4	Power control for inter-band UL CA with Tx Diversity	766

6.4	Transmit signal quality	766

6.4.1	Frequency error	766

6.4.2	Transmit modulation quality	766

6.4.2.0	General	766

6.4.2.1	Error Vector Magnitude	766

6.4.2.1a	Error Vector Magnitude including symbols with transient period	767

6.4.2.2	Carrier leakage	768

6.4.2.3	In-band emissions	768

6.4.2.4	EVM equalizer spectrum flatness	770

6.4.2.4.1	Requirements for Pi/2 BPSK modulation with powerBoosting-pi2BPSK capability	771

6.4.2.4.2	Requirements for Pi/2 BPSK and QPSK modulation with powerBoosting-pi2BPSK-QPSK-Modified-r18 capability	772

6.4.2.5	Phase continuity requirements for DMRS bundling	773

6.4A	Transmit signal quality for CA	774

6.4A.1	Frequency error for CA	774

6.4A.1.1	Frequency error for intra-band contiguous CA	774

6.4A.1.2	Frequency error for intra-band non-contiguous CA	774

6.4A.1.3	Frequency error for inter-band CA	774

6.4A.1.4	Void	774

6.4A.2	Transmit modulation quality for CA	774

6.4A.2.1	Transmit modulation quality for intra-band contiguous CA	774

6.4A.2.1.0	General	774

6.4A.2.1.1	Error Vector Magnitude	775

6.4A.2.1.2	In-band emissions	775

6.4A.2.1.3	Carrier leakage	778

6.4A.2.2	Transmit modulation quality for intra-band non-contiguous CA	778

6.4A.2.2.0	General	778

6.4A.2.2.1	Error Vector Magnitude	778

6.4A.2.2.2	In-band emissions	779

6.4A.2.2.3	Carrier leakage	779

6.4A.2.3	Transmit modulation quality for inter-band CA	779

6.4A.2.4	Void	780

6.4B	Transmit signal quality for NR-DC	780

6.4C	Transmit signal quality for SUL	780

6.4D	Transmit signal quality for UL MIMO	780

6.4D.0	General	780

6.4D.1	Frequency error for UL MIMO	780

6.4D.2	Transmit modulation quality for UL MIMO	781

6.4D.2.0	General	781

6.4D.2.1	Error Vector Magnitude	781

6.4D.2.2	Carrier leakage	781

6.4D.2.3	In-band emissions	781

6.4D.2.4	EVM equalizer spectrum flatness for UL MIMO	781

6.4D.3	Time alignment error for UL MIMO	781

6.4D.4	Requirements for coherent UL MIMO	782

6.4E	Transmit signal quality for V2X	782

6.4E.1	Frequency error for V2X	782

6.4E.1.1	General	782

6.4E.1.1A	Frequency error for sidelink CA	783

6.4E.1.2	Frequency error for V2X concurrent operation	783

6.4E.1F	Frequency error for Sidelink Unlicensed	783

6.4E.1F.1	Frequency error for SL-U concurrent operation	783

6.4E.2	Transmit modulation quality for V2X	783

6.4E.2.1	General	783

6.4E.2.2	Error Vector Magnitude for V2X	783

6.4E.2.2A	Error Vector Magnitude for sidelink CA	783

6.4E.2.3	Carrier leakage for V2X	783

6.4E.2.3A	Carrier leakage for sidelink CA	784

6.4E.2.4	In-band emissions for V2X	784

6.4E.2.4A	In-band emissions for sidelink CA	784

6.4E.2.5	EVM equalizer spectrum flatness for V2X	784

6.4E.2.6	Transmit modulation quality for V2X concurrent operation	784

6.4E.2F	Transmit modulation quality for Sidelink Unlicensed	784

6.4E.2F.0	General	784

6.4E.2F.1	Error Vector Magnitude	784

6.4E.2F.2	Carrier leakage	784

6.4E.2F.3	In-band emissions	785

6.4E.2F.4	EVM equalizer spectrum flatness	785

6.4E.2F.5	Transmit modulation quality for SL-U concurrent operation	785

6.4F	Transmit signal quality for shared spectrum channel access	785

6.4F.1	Frequency error	785

6.4F.2	Transmit modulation quality	785

6.4F.2.0	General	785

6.4F.2.1	Error Vector Magnitude	785

6.4F.2.2	Carrier leakage	785

6.4F.2.3	In-band emissions	785

6.4F.2.4	EVM equalizer spectrum flatness	786

6.4F.2A	Transmit modulation quality for CA	786

6.4F.2A.1	Transmit modulation quality for inter-band CA	786

6.4G	Transmit signal quality for Tx Diversity	787

6.4G.1	Frequency error for Tx Diversity	787

6.4G.2	 Transmit modulation quality for Tx Diversity	787

6.4G.2.0	General	787

6.4H	Transmit signal quality for CA with UL MIMO	788

6.4H.1	Transmit signal quality for intra-band UL contiguous CA with UL MIMO	788

6.4H.1.1	Frequency error for intra-band UL contiguous CA with UL MIMO	788

6.4H.1.2	Transmit modulation quality for intra-band UL contiguous CA with UL MIMO	788

6.4H.1.2.0	General	788

6.4H.1.2.1	Error Vector Magnitude	788

6.4H.1.2.2	Carrier leakage	788

6.4H.1.2.3	In-band emissions	788

6.4H.1.3	Time alignment error for intra-band UL contiguous CA with UL MIMO	789

6.4H.1.4	Coherent UL MIMO requirement for intra-band UL contiguous CA with UL MIMO	789

6.4H.2	Void	789

6.4H.3	Transmit signal quality for inter-band UL CA with UL MIMO	789

6.4H.3.1	Frequency error for inter-band UL CA with UL MIMO	789

6.4H.3.2	Transmit modulation quality for inter-band UL CA with UL MIMO	789

6.4I	(Reserved)	789

6.4J	Transmit signal quality for ATG	789

6.4J.0	Reserved	789

6.4J.0D	General	789

6.4J.1	Frequency error for ATG	790

6.4J.2	Transmit modulation quality for ATG	790

6.4J.2D	Transmit modulation quality for ATG UL MIMO	790

6.4J.2D.0	General	790

6.4J.2D.1	Error Vector Magnitude for ATG UL MIMO	790

6.4J.2D.2	Carrier leakage for ATG UL MIMO	790

6.4J.2D.3	In-band emissions for ATG UL MIMO	790

6.4J.2D.4	EVM equalizer spectrum flatness for ATG UL MIMO	790

6.4J.3D	Time alignment error for ATG	791

6.4J.3D.1	Time alignment error for ATG UL MIMO	791

6.4J.3D	Requirement for ATG coherent UL MIMO	791

6.4K	(Reserved)	791

6.4L	Transmit signal quality for CA with Tx Diversity	791

6.4L.1	Void	791

6.4L.2	Void	791

6.4L.3	Transmit signal quality for inter-band UL CA with Tx Diversity	791

6.4L.3.1	Frequency error for inter-band UL CA with Tx Diversity	791

6.4L.3.2	Transmit modulation quality for inter-band UL CA with Tx Diversity	791

6.5	Output RF spectrum emissions	791

6.5.1	Occupied bandwidth	791

6.5.2	Out of band emission	792

6.5.2.1	General	792

6.5.2.2	Spectrum emission mask	792

6.5.2.3	Additional spectrum emission mask	792

6.5.2.3.1	Requirements for network signalling value "NS_35"	792

6.5.2.3.2	Requirements for network signalling value "NS_04"	793

6.5.2.3.3	Requirements for network signalling values "NS_03" and “NS_03U”	794

6.5.2.3.4	Requirements for network signalling value "NS_06" or “NS_07”	794

6.5.2.3.5	Void	795

6.5.2.3.6	Void	795

6.5.2.3.7	Void	795

6.5.2.3.8	Requirements for network signalling value "NS_27"	795

6.5.2.3.9	Requirements for network signalling value "NS_21"	795

6.5.2.4	Adjacent channel leakage ratio	796

6.5.2.4.1	NR ACLR	796

6.5.2.4.2	UTRA ACLR	797

6.5.3	Spurious emissions	797

6.5.3.0	General	797

6.5.3.1	General spurious emissions	798

6.5.3.2	Spurious emissions for UE co-existence	798

6.5.3.3	Additional spurious emissions	806

6.5.3.3.1	Requirement for network signalling value "NS_04"	807

6.5.3.3.2	Requirement for network signalling value "NS_17"	807

6.5.3.3.3	Requirement for network signalling value "NS_18"	807

6.5.3.3.4	Requirement for network signalling values "NS_05" and “NS_05U”	807

6.5.3.3.5	Requirement for network signalling values "NS_43" and “NS_43U”	808

6.5.3.3.6	Requirement for network signalling value "NS_37"	808

6.5.3.3.7	Requirement for network signalling value "NS_38"	808

6.5.3.3.8	Requirement for network signalling value "NS_39"	809

6.5.3.3.9	Requirement for network signalling value "NS_40"	809

6.5.3.3.10	Requirement for network signalling value "NS_41"	809

6.5.3.3.11	Requirement for network signalling value "NS_42"	809

6.5.3.3.12	Requirement for network signalling value "NS_21"	810

6.5.3.3.13	Requirement for network signalling value "NS_24"	810

6.5.3.3.14	Requirement for network signalling value "NS_27"	810

6.5.3.3.15	Requirement for network signalling value "NS_47"	811

6.5.3.3.16	Requirement for network signalling value "NS_50"	811

6.5.3.3.17	Requirement for network signalling value "NS_12"	811

6.5.3.3.18	Requirement for network signalling value "NS_13"	812

6.5.3.3.19	Requirement for network signalling value "NS_14"	812

6.5.3.3.20	Requirement for network signalling value "NS_15"	812

6.5.3.3.21	Requirement for network signalling value "NS_45"	812

6.5.3.3.22	Requirement for network signalling values "NS_48" and "NS_51"	813

6.5.3.3.23	Requirement for network signalling value "NS_49"	813

6.5.3.3.24	Requirement for network signalling value "NS_44"	813

6.5.3.3.25	Requirement for network signalling value "NS_46"	814

6.5.3.3.26	Requirement for network signalling value "NS_07"	814

6.5.3.3.27	Requirement for network signalling value “NS_56”	814

6.5.3.3.28	Requirement for network signalling value “NS_62”	815

6.5.3.3.29	Requirement for network signalling value “NS_26”	815

6.5.3.3.30	Requirement for network signalling value “NS_36”	815

6.5.4	Transmit intermodulation	816

6.5A	Output RF spectrum emissions for CA	816

6.5A.0	General	816

6.5A.1	Occupied bandwidth for CA	816

6.5A.1.1	Void	816

6.5A.1.1a	Occupied bandwidth for Intra-band contiguous CA	816

6.5A.1.2	Occupied bandwidth for Intra-band non-contiguous CA	817

6.5A.1.3	Occupied bandwidth for Inter-band CA	817

6.5A.2	Out of band emission for CA	817

6.5A.2.1	General	817

6.5A.2.2	Spectrum emission mask	817

6.5A.2.2.1	Spectrum emission mask for intra-band contiguous CA	817

6.5A.2.2.2	Spectrum emission mask for intra-band non-contiguous CA	817

6.5A.2.2.3	Spectrum emission mask for Inter-band CA	818

6.5.A.2.2.4	Void	818

6.5A.2.3	Additional spectrum emission mask for CA	818

6.5A.2.3.1	Additional spectrum emission mask for intra-band contiguous CA	818

6.5A.2.3.2	Additional spectrum emission mask for Intra-band non-contiguous CA	819

6.5A.2.3.3	Additional spectrum emission mask for Inter-band CA	819

6.5A.2.4	Adjacent channel leakage ratio	819

6.5A.2.4.1	NR ACLR	819

6.5A.2.4.2	UTRA ACLR	821

6.5A.3	Spurious emission for CA	821

6.5A.3.1	General spurious emissions	821

6.5A.3.2	Spurious emissions for UE co-existence	822

6.5A.3.2.0	General	822

6.5A.3.2.1	Spurious emissions for UE co-existence for intra-band contiguous CA	823

6.5A.3.2.2	Spurious emissions for UE co-existence for intra-band non-contiguous CA	824

6.5A.3.2.3	Spurious emissions for UE co-existence for Inter-band CA	825

6.5A.3.2.4	Void	830

6.5A.3.2.5	Void	830

6.5A.3.2.6	Void	830

6.5A.3.3	Additional spurious emissions for CA	830

6.5A.3.3.1	Additional spurious emissions for intra-band contiguous  CA	830

6.5A.3.3.2	Additional spurious emissions for intra-band non-contiguous CA	831

6.5A.4	Transmit intermodulation for CA	831

6.5A.4.2.1	Transmit intermodulation for intra-band contiguous CA	831

6.5A.4.2.2	Void	832

6.5B	Output RF spectrum emissions for NR-DC	832

6.5D	Output RF spectrum emissions for UL MIMO	832

6.5D.1	Occupied bandwidth for UL MIMO	832

6.5D.2	Out of band emission for UL MIMO	832

6.5D.3	Spurious emission for UL MIMO	833

6.5D.4	Transmit intermodulation for UL MIMO	833

6.5E	Output RF spectrum emissions for V2X	833

6.5E.1	Occupied bandwidth for V2X	833

6.5E.1.1	General	833

6.5E.1.1A	Occupied bandwidth for sidelink CA	834

6.5E.1.2	Occupied bandwidth for V2X concurrent operation	834

6.5E.1F	Occupied bandwidth for Sidelink Unlicensed	834

6.5E.1F.1	Occupied bandwidth for SL-U concurrent operation	834

6.5E.2	Out of band emission for V2X	834

6.5E.2.1	General	834

6.5E.2.2	Spectrum emission mask	834

6.5E.2.2.1	General	834

6.5E.2.2.1A	Spectrum emission mask for sidelink CA	834

6.5E.2.2.2	Spectrum emission mask for V2X concurrent operation	835

6.5E.2.3	Additional Spectrum emission mask	835

6.5E.2.3.1	Requirements for network signalling value "NS_33"	835

6.5E.2.3.1A	Requirements for network or pre-configured signalling value “SLCA_NC_NS_33”	835

6.5E.2.3.2	Requirements for network signalling value "NS_52"	835

6.5E.2.3.3	Requirements for network signalling value "NS_06"	836

6.5E.2.4	Adjacent channel leakage ratio	836

6.5E.2.4.1	General	836

6.5E.2.4.1A	ACLR for sidelink CA	836

6.5E.2.4.2	ACLR for V2X concurrent operation	836

6.5E.2F	Out of band emission for Sidelink Unlicensed	837

6.5E.2F.1	General	837

6.5E.2F.2	Spectrum emission mask for operation with shared spectrum channel access	837

6.5E.2F.2.1	Spectrum emission mask for SL-U concurrent operation	837

6.5E.2F.3	Additional spectrum emission mask	837

6.5E.2F.4	Adjacent channel leakage ratio	837

6.5E.2F.4.1	Adjacent channel leakage ratio for SL-U concurrent operation	837

6.5E.3	Spurious emissions for V2X	837

6.5E.3.1	General spurious emissions	837

6.5E.3.1A	Spurious emissions for sidelink CA	837

6.5E.3.2	Spurious emissions for UE co-existence	837

6.5E.3.2A	Spurious emissions band UE co-existence for sidelink CA	838

6.5E.3.3	Spurious emissions for UE co-existence for V2X concurrent operation	838

6.5E.3.4	Additional spurious emissions requirements for V2X	839

6.5E.3.4.1	General	839

6.5E.3.4.2	Requirements for network signalling value "NS_33"	839

6.5E.3.4A	Additional spurious emissions requirements for sidelink CA	839

6.5E.3.4A.1	General	839

6.5E.3.4A.2	Requirements for network signalling value "SLCA_NC_NS_33"	840

6.5E.3.4.3	Void	840

6.5E.3F	Spurious emissions for Sidelink Unlicensed	840

6.5E.3F.0	General	840

6.5E.3F.1	General spurious emissions	840

6.5E.3F.2	Spurious emissions for UE co-existence	840

6.5E.3F.2.1	Spurious emissions for UE co-existence for SL-U concurrent operation	841

6.5E.3F.3	Additional spurious emissions	841

6.5E.3F.3.0	General	841

6.5E.4	Transmit intermodulation	841

6.5E.4.1	General	841

6.5E.4.1A	Transmit intermodulation for sidelink CA	841

6.5E.4.2	Transmit intermodulation for V2X concurrent operation	841

6.5F	Output RF spectrum emissions for shared spectrum channel access	841

6.5F.1	Occupied bandwidth	841

6.5F.2	Out of band emission	842

6.5F.2.1	General	842

6.5F.2.2	Spectrum emission mask for operation with shared spectrum channel access	842

6.5F.2.2.0	General	842

6.5F.2.2.1	Spectrum emission mask for non-transmitted channels	842

6.5F.2.3	Additional spectrum emission mask	843

6.5F.2.4	Adjacent channel leakage ratio	843

6.5F.2.4.0	General	843

6.5F.2.4.1	Shared spectrum channel access ACLR	843

6.5F.2.4.2	Additional requirement for network signalled value "NS_29"	843

6.5F.2A	Out of band emission for CA	844

6.5F.2A.1	Spectrum emission mask for CA	844

6.5F.2A.1.1	Spectrum emission mask for Inter-band CA	844

6.5F.2A.1.2	Spectrum emission mask for Intra-band contiguous CA	844

6.5F.2A.1.2.1	General	844

6.5F.2A.1.2.2	Intra-band contiguous CA spectrum emission mask for non-transmitted channels	845

6.5F.2A.2	Adjacent channel leakage ratio for CA	845

6.5F.2A.2.1	Adjacent channel leakage ratio for inter-band CA	845

6.5F.2A.2.2	Adjacent channel leakage ratio for intra-band contiguous CA	845

6.5F.3	Spurious emissions	845

6.5F.3.0	General	845

6.5F.3.1	General spurious emissions	846

6.5F.3.2	Spurious emissions for UE co-existence	846

6.5F.3.3	Additional spurious emissions	846

6.5F.3.3.0	General	846

6.5F.3.3.1	Requirement for network signalling value "NS_28"	846

6.5F.3.3.2	Requirement for network signalling value "NS_29"	846

6.5F.3.3.3	Requirement for network signalling value "NS_30"	848

6.5F.3.3.4	Requirement for network signalling value "NS_31"	849

6.5F.3.3.5	Requirements for network signalling value "NS_53" or "NS_54" or "NS_60" or "NS_66" or "NS_67" or "NS_71"	850

6.5F.3.3.6	Requirements for network signalling value "NS_58"	850

6.5F.3.3.7	Requirements for network signalling value "NS_61"	850

6.5F.3.3.8	Requirements for network signalling value “NS_63” or “NS_69”	850

6.5F.3.3.9	Requirements for network signalling value “NS_64”	851

6.5F.3A	Spurious emissions for CA	851

6.5F.3A.0	General	851

6.5F.3A.1	General spurious emissions	852

6.5F.3A.2	Spurious emissions for UE co-existence	852

6.5F.3A.3	Additional spurious emissions	852

6.5F.3A.3.0	General	852

6.5F.3A.3.1	Requirements for network signalling value "CA_NS_53" or "CA_NS_54"	852

6.5F.4	Transmit intermodulation	852

6.5G	Output RF spectrum emissions for Tx Diversity	852

6.5G.1	Occupied bandwidth for Tx Diversity	852

6.5G.2	Out of band emission for Tx Diversity	853

6.5G.3	Spurious emission for Tx Diversity	853

6.5G.4	Transmit intermodulation for Tx Diversity	853

6.5H	Output RF spectrum emissions for CA with UL MIMO	853

6.5H.1	Output RF spectrum emissions for intra-band UL contiguous CA with UL MIMO	853

6.5H.1.1	Occupied bandwidth for intra-band UL contiguous CA with UL MIMO	853

6.5H.1.2	Out of band emission for intra-band UL contiguous CA with UL MIMO	853

6.5H.1.3	 Spurious emission for intra-band UL contiguous CA with UL MIMO	853

6.5H.1.4	Transmit intermodulation for intra-band UL contiguous CA with UL MIMO	854

6.5H.2	Void	854

6.5H.3	Output RF spectrum emissions for inter-band UL CA with UL MIMO	854

6.5H.3.1	Occupied bandwidth for inter-band UL CA with UL MIMO	854

6.5H.3.2	Out of band emission for inter-band UL CA with UL MIMO	854

6.5H.3.3	Spurious emission for inter-band UL CA with UL MIMO	854

6.5H.3.4	Transmit intermodulation for inter-band UL CA with UL MIMO	854

6.5I	(Reserved)	854

6.5J	Output RF spectrum emissions for ATG	855

6.5J.1	Occupied bandwidth for ATG	855

6.5J.1D	Occupied bandwidth for ATG UL MIMO	855

6.5J.2	Out of band emission for ATG	855

6.5J.2.1	General	855

6.5J.2.2	Spectrum emission mask	855

6.5J.2.3	Adjacent channel leakage ratio	855

6.5J.2D	Out of band emission for ATG with UL MIMO	856

6.5J.3	Spurious emissions for ATG	856

6.5J.3D	Spurious emissions for ATG with UL MIMO	856

6.5K	Output RF spectrum emissions for Aerial UE	856

6.5K.1	Occupied bandwidth for Aerial UE	856

6.5K.2	Out of band emission for Aerial UE	857

6.5K.3	Spurious emissions for Aerial UE	857

6.5K.3.0	General	857

6.5K.3.1	General spurious emissions	857

6.5K.3.2	Spurious emissions for UE co-existence	857

6.5K.3.3	Additional spurious emissions	857

6.5K.3.3.1	Requirement for network signalling value "NS_UAV_44"	857

6.5K.3.3.2	Requirement for network signalling value "NS_UAV_46"	857

6.5K.3.3.3	Requirement for network signalling value “NS_UAV_70"	857

6.5L	Output RF spectrum emissions for CA with Tx Diversity	858

6.5L.1	Void	858

6.5L.2	Void	858

6.5L.3	Output RF spectrum emissions for inter-band UL CA with Tx Diversity	858

6.5L.3.1	Occupied bandwidth for inter-band UL CA with Tx Diversity	858

6.5L.3.2	Out of band emission for inter-band UL CA with Tx Diversity	858

6.5L.3.3	Spurious emission for inter-band UL CA with Tx Diversity	858

6.5L.3.4	Transmit intermodulation for inter-band UL CA with Tx Diversity	858

6.6	Void	858

6.6E	Time alignment error	858

7	Receiver characteristics	825

7.1	General	825

7.1A	General	826

7.1F	General	826

7.1G	(Reserved)	826

7.1H	(Reserved)	826

7.1I	General	826

7.1J	General for ATG	826

7.1K	(Reserved)	826

7.1L	(Reserved)	826

7.1M	General for LP-WUS/WUR	826

7.2	Diversity characteristics	827

7.2J	Diversity characteristics for ATG	827

7.2M	Diversity characteristics for WUS/WUR	827

7.3	Reference sensitivity	827

7.3.1	General	827

7.3.2	Reference sensitivity power level	828

7.3.3	ΔRIB,c	838

7.3A	Reference sensitivity for CA	838

7.3A.1	General	838

7.3A.2	Reference sensitivity power level for CA	839

7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA	839

7.3A.2.2	Reference sensitivity power level for Intra-band non-contiguous CA	840

7.3A.2.3	Reference sensitivity power level for Inter-band CA	842

7.3A.2.3.1	PC2 and PC1.5 MSD requirements with look-up tables for two-band DL CA with 1UL band single CC	842

7.3A.2.3.2	PC2 and PC1.5 MSD requirements with look-up tables for two-band or three-band DL CA with two-band UL CA	843

7.3A.2.4	Void	845

7.3A.3	ΔRIB,c for CA	845

7.3A.3.1	General	845

7.3A.3.2	ΔRIB,c for Inter-band CA	845

7.3A.3.2.1	ΔRIB,c for two bands	845

7.3A.3.2.2	Void	848

7.3A.3.2.3	ΔRIB,c for three bands	848

7.3A.3.2.4	ΔRIB,c for four bands	853

7.3A.3.2.5	ΔRIB,c for five bands	857

7.3A.3.2.6	ΔRIB,c for six bands	858

7.3A.3.3	ΔRIB,c for Intra-band CA	858

7.3A.4	Reference sensitivity exceptions due to harmonic interference for CA	858

7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	872

7.3A.6	Reference sensitivity exceptions due to cross band isolation for CA	936

7.3A.7	Lower-MSD requirements for inter-band CA	941

7.3B	Reference sensitivity for NR-DC	942

7.3C	Reference sensitivity for SUL	942

7.3C.1	General	942

7.3C.2	Reference sensitivity power level for SUL	942

7.3C.3	ΔRIB,c for SUL	945

7.3C.3.1	General	946

7.3C.3.2	SUL band combination	946

7.3C.3.2.1	ΔRIB,c  for two bands	946

7.3C.3.2.2	ΔRIB,c  for three bands	946

7.3C.3.2.3	ΔRIB,c  for four bands	947

7.3D	Reference sensitivity for UL MIMO	947

7.3E	Reference sensitivity for V2X	948

7.3E.1	General	948

7.3E.2	Minimum requirements	948

7.3E.2A	Minimum requirements for Sidelink CA	949

7.3E.2A.1	Reference sensitivity power level for Sidelink CA	949

7.3E.2F	Minimum requirements for Sidelink Unlicensed	949

7.3E.2F.1	General	949

7.3E.2F.2	Reference sensitivity power level	949

7.3E.3	Reference sensitivity power level for V2X concurrent operation	950

7.3E.3.1	General	950

7.3E.3F	Minimum requirements for SL-U concurrent operation	951

7.3E.3F.1	Reference sensitivity power level for SL-U concurrent operation	951

7.3F	Reference sensitivity for shared spectrum channel access	951

7.3F.1	General	951

7.3F.2	Reference sensitivity power level	951

7.3F.3	Void	952

7.3F.4	Void	952

7.3F.4A	Shared spectrum channel access CA	952

7.3F.4A.1	Intra-band contiguous shared spectrum channel access CA	952

7.3F.5	Void	952

7.3F.5.1	Void	953

7.3F.5.2	Void	953

7.3F.5.3	Void	953

7.3G	Reference sensitivity for Tx Diversity	953

7.3G.5	Void	953

7.3G.5.0	Void	953

7.3H	(Reserved)	953

7.3I	Reference sensitivity for (e)RedCap	953

7.3I.1	General	953

7.3I.2	Reference sensitivity power level for RedCap	953

7.3I.3	Reference sensitivity power level for eRedCap	957

7.3J	Reference sensitivity for ATG	958

7.3J.1	General	958

7.3J.2	Reference sensitivity power level	958

7.3J.2A	Reference sensitivity power level for ATG CA	958

7.3J.2A.1	Reference sensitivity power level for ATG intra-band contiguous CA	958

7.3J.2A.2	Reference sensitivity power level for ATG inter-band CA	959

7.3J.3A	ΔRIB,c for ATG CA	959

7.3J.4A	Reference sensitivity exceptions due to cross band isolation for ATG CA	959

7.3K	(Reserved)	960

7.3L	(Reserved)	960

7.3M	Reference sensitivity for LP-WUS/WUR	960

7.3M.1	General	960

7.3M.2	Reference sensitivity power level for LP-WUS/WUR	960

7.4	Maximum input level	960

7.4A	Maximum input level for CA	961

7.4A.1	Maximum input level for Intra-band contiguous CA	961

7.4A.2	Maximum input level for Intra-band non-contiguous CA	961

7.4A.3	Maximum input level for Inter-band CA	962

7.4B	Maximum input level for NR-DC	962

7.4D	Maximum input level for UL MIMO	962

7.4E	Maximum input level for V2X	962

7.4E.1	General	962

7.4E.1A	Maximum input level for Sidelink CA	962

7.4E.1F	General requirement for Sidelink Unlicensed	963

7.4E.2	Maximum input level for V2X concurrent operation	963

7.4E.2F	Maximum input level for SL-U concurrent operation	963

7.4F	(Reserved)	963

7.4G	(Reserved)	963

7.4H	(Reserved)	963

7.4I	(Reserved)	963

7.4J	Maximum input level for ATG	963

7.4J.1	General	963

7.4J.2A	Maximum input level for ATG CA	964

7.4J.2A.1	Minimum requirement for ATG intra-band contiguous CA	964

7.4J.2A.1	Minimum requirement for ATG inter-band CA	964

7.4K	(Reserved)	965

7.4L	(Reserved)	965

7.4M	Maximum input level for LP-WUS/WUR	965

7.5	Adjacent channel selectivity	965

7.5A	Adjacent channel selectivity for CA	967

7.5A.1	Adjacent channel selectivity for Intra-band contiguous CA	967

7.5A.2	Adjacent channel selectivity Intra-band non-contiguous CA	969

7.5A.3	Adjacent channel selectivity Inter-band CA	970

7.5B	Adjacent channel selectivity for NR-DC	970

7.5D	Adjacent channel selectivity for UL MIMO	970

7.5E	Adjacent channel selectivity for V2X	970

7.5E.1	General	970

7.5E.1A	Adjacent channel selectivity requirement for Sidelink CA	972

7.5E.1F	General requirement for Sidelink Unlicensed	973

7.5E.2	Adjacent channel selectivity for V2X concurrent operation	973

7.5E.2F	Adjacent channel selectivity for SL-U concurrent operation	973

7.5F	Adjacent channel selectivity for shared spectrum channel access	973

7.5F.1	General	973

7.5F.1A	Adjacent channel selectivity for shared spectrum channel access CA	974

7.5F.1A.1	Intra-band contiguous shared spectrum channel access CA	974

7.5F.2	Void	974

7.5G	(Reserved)	974

7.5H	(Reserved)	974

7.5I	(Reserved)	975

7.5J	Adjacent channel selectivity for ATG	975

7.5J.1	General	975

7.5J.2A	Adjacent channel selectivity for ATG CA	977

7.5J.2A.1	Minimum requirement for ATG intra-band contiguous CA	977

7.5J.2A.2	Minimum requirement for ATG inter-band contiguous CA	978

7.5K	(Reserved)	978

7.5L	(Reserved)	978

7.5M	Adjacent channel and subcarrier selectivity for LP-WUS/WUR	978

7.5M.1	General	978

7.5M.2	Adjacent channel selectivity for LP-WUS/WUR	979

7.5M.3	Adjacent subcarrier selectivity for LP-WUS/WUR	980

7.6	Blocking characteristics	980

7.6.1	General	980

7.6.2	In-band blocking	980

7.6.3	Out-of-band blocking	983

7.6.4	Narrow band blocking	986

7.6A	Blocking characteristics for CA	988

7.6A.1	General	988

7.6A.2	In-band blocking for CA	988

7.6A.2.1	In-band blocking for Intra-band contiguous CA	988

7.6A.2.2	In-band blocking for Intra-band non-contiguous CA	989

7.6A.2.3	In-band blocking for Inter-band CA	989

7.6A.3	Out-of-band blocking for CA	990

7.6A.3.1	Out-of-band blocking for Intra-band contiguous CA	990

7.6A.3.2	Out-of-band blocking for Intra-band non-contiguous CA	991

7.6A.3.3	Out-of-band blocking for Inter-band CA	991

7.6A.4	Narrow band blocking for CA	993

7.6A.4.1	Narrow band blocking for Intra-band contiguous CA	993

7.6A.4.2	Narrow band blocking for Intra-band non-contiguous CA	994

7.6A.4.3	Narrow band blocking for Inter-band CA	994

7.6B	Blocking characteristics for NR-DC	994

7.6C	Blocking characteristics for SUL	994

7.6C.1	General	994

7.6C.2	In-band blocking for SUL	994

7.6C.3	Out-of-band blocking for SUL	995

7.6C.4	Narrow band blocking for SUL	995

7.6D	Blocking characteristics for UL MIMO	995

7.6E	Blocking characteristics for V2X	995

7.6E.1	General	995

7.6E.2	In-band blocking	995

7.6E.2.1	General	996

7.6E.2.1A	In-band blocking for Sidelink CA	997

7.6E.2.2	In-band blocking for V2X concurrent operation	997

7.6E.2.2F	In-band blocking for SL-U concurrent operation	997

7.6E.3	Out-of-band blocking	997

7.6E.3.1	General	997

7.6E.3.1A	Out-of-band blocking for Sidelink CA	998

7.6E.3.2	Out-of-band blocking for V2X concurrent operation	999

7.6E.3.2F	Out-of-band blocking for SL-U concurrent operation	999

7.6E.3F	Out-of-band blocking for Sidelink Unlicensed	999

7.6F	Blocking characteristics for shared spectrum channel access	999

7.6F.1	General	999

7.6F.2	In-band blocking	1000

7.6F.2.1	General	1000

7.6F.2.2	Void	1000

7.6F.2A	In-band blocking for shared spectrum CA	1000

7.6F.2A.1	Intra-band contiguous shared spectrum channel access CA	1000

7.6F.3	Out-of-band blocking	1001

7.6F.3.1	General	1001

7.6F.3.2	Void	1002

7.6F.3A	Out-of-band blocking for shared spectrum CA	1002

7.6F.3A.1	Intra-band contiguous shared spectrum channel access CA	1002

7.6G	(Reserved)	1003

7.6H	(Reserved)	1003

7.6I	Blocking characteristics for RedCap	1003

7.6J	Blocking characteristics for ATG	1003

7.6J.1	General	1003

7.6J.2	In-band blocking for ATG	1003

7.6J.2A	In-band blocking for ATG CA	1003

7.6J.2A.1	In-band blocking for ATG Intra-band contiguous CA	1003

7.6J.2A.2	In-band blocking for ATG Inter-band CA	1003

7.6J.3	Out-of-band blocking for ATG	1003

7.6J.3A	Out-of-band blocking for ATG CA	1003

7.6J.3A.1	Out-of-band blocking for ATG Intra-band contiguous CA	1004

7.6J.3A.2	Out-band blocking for ATG Inter-band CA	1004

7.6K	(Reserved)	1004

7.6L	(Reserved)	1004

7.7	Spurious response	1004

7.7A	Spurious response for CA	1005

7.7A.1	Spurious response for Intra-band contiguous CA	1005

7.7A.2	Spurious response for Intra-band non-contiguous CA	1005

7.7A.3	Spurious response for Inter-band CA	1005

7.7B	Spurious response for NR-DC	1006

7.7D	Spurious response for UL MIMO	1006

7.7E	Spurious response for V2X	1006

7.7E.1	General	1006

7.7E.1A	Spurious response requirements for Sidelink CA	1006

7.7E.1F	General requirement for Sidelink Unlicensed	1007

7.7E.2	Spurious response for V2X concurrent operation	1007

7.7E.2F	Spurious response for SL-U concurrent operation	1007

7.7F	Spurious response for shared spectrum channel access	1007

7.7F.1	General	1007

7.7F.1A	Spurious response for shared spectrum channel access CA	1008

7.7F.1A.1	Intra-band contiguous shared spectrum channel access CA	1008

7.7F.2	Void	1008

7.7G	(Reserved)	1008

7.7H	(Reserved)	1008

7.7I	(Reserved)	1008

7.7J	Spurious response for ATG	1008

7.7J.1	General	1008

7.7J.1A	Spurious response for ATG CA	1008

7.7J.1A.1	Spurious response for ATG intra-band contiguous CA	1008

7.7J.1A.2	Spurious response for ATG inter-band CA	1008

7.7K	(Reserved)	1009

7.7L	(Reserved)	1009

7.8	Intermodulation characteristics	1009

7.8.1	General	1009

7.8.2	Wide band Intermodulation	1009

7.8A	Intermodulation characteristics for CA	1011

7.8A.1	General	1011

7.8A.2	Wide band intermodulation for CA	1011

7.8A.2.1	Wide band intermodulation for Intra-band contiguous CA	1011

7.8A.2.2	Wide band intermodulation for Intra-band non-contiguous CA	1012

7.8A.2.3	Wide band intermodulation for Inter-band CA	1012

7.8B	Intermodulation characteristics for NR-DC	1013

7.8D	Intermodulation characteristics for UL MIMO	1013

7.8E	Intermodulation characteristics for V2X	1013

7.8E.1	General	1013

7.8E.2	Wide band Intermodulation	1013

7.8E.2.1	General	1013

7.8E.2.2	Wide band Intermodulation for V2X concurrent operation	1014

7.8E.2.2A	Wide band intermodulation for Sidelink CA	1014

7.8E.2.2F	Wide band Intermodulation for SL-U concurrent operation	1014

7.8E.2F	Wide band Intermodulation for Sidelink Unlicensed	1015

7.8F	Intermodulation characteristics for shared spectrum channel access	1015

7.8F.1	General	1015

7.8F.2	Wide band Intermodulation	1015

7.8G	(Reserved)	1015

7.8H	(Reserved)	1016

7.8I	(Reserved)	1016

7.8J	Intermodulation characteristics for ATG	1016

7.8J.1	General	1016

7.8J.2	Wide band intermodulation for ATG	1016

7.8J.2A	Wide band intermodulation for ATG CA	1016

7.8J.2A.1	Wide band intermodulation for ATG intra-band contiguous CA	1016

7.8J.2A.2	Wide band intermodulation for ATG inter-band CA	1016

7.8K	(Reserved)	1016

7.8L	(Reserved)	1016

7.9	Spurious emissions	1016

7.9A	Spurious emissions for CA	1017

7.9A.1	Void	1017

7.9A.2	Void	1017

7.9A.3	Spurious emissions for Inter-band CA	1017

7.9B	Spurious emissions for NR-DC	1017

7.9J	Spurious emissions for ATG	1017

7.9J.1	General	1017

7.9J.1A	Spurious emissions for ATG CA	1017

7.9J.1A.1	Spurious emissions for ATG inter-band CA	1017

7.9M	Spurious emissions for LP-WUS/WUR	1017

7.10	Power imbalance	1017

7.10A	Power imbalance for CA	1017

7.10A.1	General	1017

7.10A.2	Minimum requirement	1018

Annex A (normative): Measurement channels	995

A.1	General	995

A.2	UL reference measurement channels	995

A.2.1	General	995

A.2.2	Reference measurement channels	997

A.2.2.1	DFT-s-OFDM Pi/2-BPSK	997

A.2.2.2	DFT-s-OFDM QPSK	998

A.2.2.3	DFT-s-OFDM 16QAM	1000

A.2.2.4	DFT-s-OFDM 64QAM	1001

A.2.2.5	DFT-s-OFDM 256QAM	1003

A.2.2.6	CP-OFDM QPSK	1004

A.2.2.7	CP-OFDM 16QAM	1006

A.2.2.8	CP-OFDM 64QAM	1008

A.2.2.9	CP-OFDM 256QAM	1010

A.2.3	Reference measurement channels for TDD	1013

A.2.3.1	DFT-s-OFDM Pi/2-BPSK	1013

A.2.3.2	DFT-s-OFDM QPSK	1013

A.2.3.3	DFT-s-OFDM 16QAM	1013

A.2.3.4	DFT-s-OFDM 64QAM	1013

A.2.3.5	DFT-s-OFDM 256QAM	1013

A.2.3.6	CP-OFDM QPSK	1014

A.2.3.7	CP-OFDM 16QAM	1014

A.2.3.8	CP-OFDM 64QAM	1014

A.2.3.9	CP-OFDM 256QAM	1014

A.3	DL reference measurement channels	1014

A.3.1	General	1014

A.3.2	DL reference measurement channels for FDD	1017

A.3.2.1	General	1017

A.3.2.2	FRC for receiver requirements for QPSK	1018

A.3.2.3	FRC for maximum input level for 64QAM	1021

A.3.2.4	FRC for maximum input level for 256 QAM	1024

A.3.2.5	FRC for maximum input level for 1024 QAM	1027

A.3.3	DL reference measurement channels for TDD	1030

A.3.3.1	General	1030

A.3.3.2	FRC for receiver requirements for QPSK	1031

A.3.3.3	FRC for maximum input level for 64QAM	1034

A.3.3.4	FRC for maximum input level for 256 QAM	1037

A.3.3.5	FRC for maximum input level for 1024 QAM	1040

A.3M	LP-WUR reference channel	1043

A.3M.1	General	1043

A.4	CSI reference measurement channels	1043

A.5	OFDMA Channel Noise Generator (OCNG)	1044

A.5.1	OCNG Patterns for FDD	1044

A.5.1.1	OCNG FDD pattern 1: Generic OCNG FDD Pattern for all unused REs	1044

A.5.2	OCNG Patterns for TDD	1044

A.5.2.1	OCNG TDD pattern 1: Generic OCNG TDD Pattern for all unused REs	1044

A.6	Void	1044

A.7	V2X reference measurement channels	1045

A.7.1	General	1045

A.7.2	FRC for V2X receiver requirements for QPSK	1045

A.7.3	FRC for maximum input level for 64QAM	1047

A.7.4	FRC for maximum input level for 256QAM	1048

Annex B (informative): Void	1051

Annex C (informative): Downlink physical channels	1052

C.1	General	1052

C.2	Setup	1052

C.3	Connection	1052

C.3.1	Measurement of Receiver Characteristics	1052

Annex D (normative): Characteristics of the interfering signal	1053

D.1	General	1053

D.2	Void	1053

Annex E (normative):  Environmental conditions	1054

E.1	General	1054

E.2	Environmental	1054

E.2.1	Temperature	1054

E.2.2	Voltage	1054

E.2.3	Vibration	1055

Annex F (normative):  Transmit modulation	1056

F.0	General	1056

F.1	Measurement Point	1056

F.2	Basic Error Vector Magnitude measurement	1056

F.3	Basic in-band emissions measurement	1057

F.4	Modified signal under test	1058

F.5	Window length	1060

F.5.1	Timing offset	1060

F.5.2	Window length	1060

F.5.3	Window length for normal CP	1060

F.5.4	Window length for Extended CP	1061

F.5.5	Window length for PRACH	1062

F.6	Averaged EVM	1063

F.7	Spectrum Flatness	1064

F.8	EVM measurement for multiple Tx	1064

F.9	Phase offset measurement for DMRS bundling	1064

F.9.1	Measurement point	1064

F.9.2	Symbols used	1064

F.9.3	Modified test signal	1064

F.9.4	Phase offset measurement	1065

F.10	EVM for UL MIMO	1065

F10.1	General	1065

F10.2	MIMO Equalization	1066

F10.3	Layer processing	1067

Annex G (normative):  Difference of relative phase and power errors	1068

G.0	General	1068

G.1	Measurement Point	1068

G.2	Relative Phase Error Measurement	1068

G.2.1	Symbols and subcarriers used	1068

G.2.2	CFO (carrier frequency offset) correction	1069

G.2.3	Steps of the measurement method	1069

Annex H (informative): Void	1070

Annex I (informative): Void	1071

Annex J (informative): Void	1072

Annex K (informative): Void	1073

Annex L (normative): ModifiedMPR-Behavior	1074

L.1	Indication of modified MPR behavior	1074

Annex M (normative):  Declared Supported Post Antenna Gain for UE	1078

M.1	FRMCS operating bands	1078

Annex N (informative): Change history	1079