| 3GPP TS 38.133 V17.20.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	119

1	Scope	121

2	References	121

3	Definitions, symbols and abbreviations	122

3.1	Definitions	122

3.2	Symbols	124

3.3	Abbreviations	125

3.4	Test tolerances	127

3.5	Frequency bands grouping	127

3.5.1	Introduction	127

3.5.2	NR operating bands in FR1	128

3.5.2A	NR operating bands for satellite access in FR1	128

3.5.3	NR operating bands in FR2	129

3.6	Applicability of requirements in this specification version	129

3.6.1	RRC connected state requirements in DRX	130

3.6.2	Number of serving carriers	130

3.6.2.1	Number of serving carriers for SA	130

3.6.2.2	Number of serving carriers for EN-DC	130

3.6.2.3	Number of serving carriers for NE-DC	130

3.6.2.4	Number of serving carriers for NR-DC	131

3.6.3	Applicability for intra-band FR2	131

3.6.4	Applicability for FR2 UE power classes	131

3.6.5	Applicability for SDL bands	131

3.6.6	Applicability of requirements for NGEN-DC operation	131

3.6.7	Applicability of QCL	131

3.6.9	Applicability of requirements for scheduling availability	132

3.6.10	Applicability of requirements for measurement restrictions	132

3.6.11	Applicability of requirements for Redcap UEs	132

3.6.11.1	RRC connected state requirements in DRX	132

3.6.11.2	Applicability for FR2 Redcap UE power classes	132

3.6.11.3	Applicability of QCL	132

3.6.12	Applicability of requirements for Satellite Access	133

3.6.13	Applicability of requirements for FR2	133

3.6.14	Applicability of requirements for FR2 Power Class 6	133

3.6.15	Applicability of requirements for per-FR gap	133

4	SA: RRC_IDLE state mobility	133

4.1	Cell Selection	133

4.2	Cell Re-selection	134

4.2.1	Introduction	134

4.2.2	Requirements	134

4.2.2.1	UE measurement capability	134

4.2.2.2	Measurement and evaluation of serving cell	134

4.2.2.3	Measurements of intra-frequency NR cells	137

4.2.2.4	Measurements of inter-frequency NR cells	140

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	145

4.2.2.6	Maximum interruption in paging reception	148

4.2.2.7	General requirements	149

4.2.2.8	Minimum requirement at transitions	149

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	149

4.2.2.9.1	Introduction	149

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	150

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	152

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	154

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	155

4.2.2.10.1	Introduction	155

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	155

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	158

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	161

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	161

4.2.2.11.1	Introduction	161

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	162

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	164

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	165

4.2A	Cell Re-selection when subject to CCA	166

4.2A.1	Introduction	166

4.2A.2	Requirements	166

4.2A.2.1	UE measurement capability	166

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	167

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	167

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	169

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	171

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	171

4.2A.2.7	General requirements	171

4.2B	Cell Re-selection for RedCap	172

4.2B.1	Introduction	172

4.2B.2	Requirements	172

4.2B.2.1	UE measurement capability for RedCap	172

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	172

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	172

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	172

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	174

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	177

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	180

4.2B.2.6	Maximum interruption in paging reception for RedCap	182

4.2B.2.7	General requirements for RedCap	182

4.2B.2.8	Minimum requirement at transitions	182

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	183

4.2B.2.9.1	Introduction	183

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	184

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	187

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	187

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	187

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	188

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	188

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	188

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	188

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	189

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	192

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	194

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	194

4.2B.2.10.1	Introduction	194

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	195

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	197

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge-criterion	198

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	198

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	199

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	199

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	199

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criteria	199

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	200

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	202

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	205

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	205

4.2B.2.11.1	Introduction	205

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	206

4.2B.2.11.3	Measurements for a UE not-at-cell edge while stationary criterion	207

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	207

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	208

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not-at-cell-edge while stationary criteria	208

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	208

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	209

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criteria	209

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	209

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	211

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	212

4.2C	Cell Re-selection for NR UE for Satellite Access	212

4.2C.1	Introduction	212

4.2C.2	Requirements	213

4.2C.2.1	UE measurement capability	213

4.2C.2.2	Measurement and evaluation of serving cell	213

4.2C.2.3	Measurements of intra-frequency NR cells	214

4.2C.2.4	Measurements of inter-frequency NR cells	216

4.2C.2.5	Maximum interruption in paging reception	219

4.2C.2.6	Minimum requirement at transitions	220

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	220

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	220

4.2C.2.9	General requirements	220

4.3	Minimization of Drive Tests (MDT)	220

4.3.1	Introduction	220

4.3.2	Measurement Requirements	221

4.3.3	Requirements for Relative Time Stamp Accuracy	221

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	221

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	222

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	222

4.3C.1	Introduction	222

4.3C.2	Measurement Requirements	222

4.3C.3	Requirements for Relative Time Stamp Accuracy	223

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	223

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	223

4.4	Idle Mode CA/DC Measurements	224

4.4.1	Introduction	224

4.4.2	Measurement Requirements	224

4.4.2.1	Detected cell requirement during state transition and Idle mode	224

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	224

4.4.2.3	Measurements on serving cell	226

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	226

5	SA: RRC_INACTIVE state mobility	227

5.1	Cell Re-selection	227

5.1.1	Introduction	227

5.1.2	Requirements	227

5.1.2.1	UE measurement capability	227

5.1.2.2	Measurement and evaluation of serving cell	227

5.1.2.3	Measurements of intra-frequency NR cells	228

5.1.2.4	Measurements of inter-frequency NR cells	229

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	230

5.1.2.6	Maximum interruption in paging reception	231

5.1.2.7	General requirements	231

5.1.2.8	Minimum requirement at transitions	231

5.1.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	231

5.1.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	232

5.1.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	233

5.1A	Cell Re-selection with CCA	234

5.1A.1	Introduction	234

5.1A.2	Requirements	234

5.1A.2.1	UE measurement capability	234

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	234

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	234

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	234

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	234

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	234

5.1A.2.7	General requirements	234

5.1B	Cell Re-selection for RedCap	234

5.1B.1	Introduction	234

5.1B.2	Requirements	235

5.1B.2.1	UE measurement capability	235

5.1B.2.2	Measurement and evaluation of serving cell	235

5.1B.2.3	Measurements of intra-frequency NR cells	236

5.1B.2.4	Measurements of inter-frequency NR cells	237

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	237

5.1B.2.6	Maximum interruption in paging reception	238

5.1B.2.7	General requirements	238

5.1B.2.8	Minimum requirement at transitions	238

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	238

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	239

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	240

5.1C	Cell Re-selection	241

5.1C.1	Introduction	241

5.1C.2	Requirements	241

5.1C.2.1	UE measurement capability	241

5.1C.2.2	Measurement and evaluation of serving cell	241

5.1C.2.3	Measurements of intra-frequency NR cells	241

5.1C.2.4	Measurements of inter-frequency NR cells	242

5.1C.2.5	Maximum interruption in paging reception	242

5.1C.2.6	General requirements	242

5.2	Void	242

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	242

5.2B.1	Introduction	242

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	242

5.2B.2.1	Void	242

5.2B.3	TA validation requirements for RedCap	242

5.2B.3.1	Void	243

5.2B.3.2	Void	243

5.2B.4	Scheduling restriction	243

5.2B.5	Applicability conditions for CG-SDT for RedCap	244

5.3	Minimization of Drive Tests (MDT)	244

5.3.1	Introduction	244

5.3.2	Measurement Requirements	244

5.3.3	Requirements for Relative Time Stamp Accuracy	244

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	244

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	244

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	245

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	245

5.3C.1	Introduction	245

5.3C.2	Measurement Requirements	245

5.3C.3	Requirements for Relative Time Stamp Accuracy	245

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	245

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	245

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	246

5.4	Inactive Mode CA/DC Measurements	246

5.4.1	Introduction	246

5.4.2	Measurement Requirements	246

5.4.2.1	Detected cell requirement during state transition and inactive mode	246

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	246

5.4.2.3	Measurements on serving cell	246

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	246

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	246

5.5.1	Introduction	246

5.5.2	Requirements on UE synchronization for small data transmissions	246

5.5.3	TA validation requirements	247

5.5.4	Scheduling restriction	248

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	248

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	248

5.5.4.3	Scheduling availability of UE performing measurements on FR2	248

5.5.5	Applicability conditions for SDT	249

5.6	NR measurements for positioning	249

5.6.1	Introduction	249

5.6.2	RSTD measurements	250

5.6.2.1	Introduction	250

5.6.2.2	Requirements Applicability	250

5.6.2.3	Measurement Capability	250

5.6.2.5	Measurements Period Requirements	251

5.6.3	PRS-RSRP measurements	253

5.6.3.1	Introduction	253

5.6.3.2	Requirements applicability	253

5.6.3.3	Measurement Capability	254

5.6.3.4	Measurement Reporting Requirements	254

5.6.3.5	Measurement Period Requirements	254

5.6.4	UE Rx-Tx time difference measurements	256

5.6.4.1	Introduction	256

5.6.4.2	Requirements Applicability	256

5.6.4.3	Measurement Capability	257

5.6.4.4	Measurement Reporting Requirements	257

5.6.4.5	Measurement Period Requirements	257

5.6.5	PRS-RSRPP measurements	260

5.6.5.1	Introduction	260

5.6.5.2	Requirements Applicability	260

5.6.5.3	Measurement Capability	260

5.6.5.4	Measurement Reporting Requirements	260

5.6.5.5	Measurement Period Requirements	261

5.7	Random access based Small Data Transmissions (RA-SDT)	262

5.7.1	Introduction	262

5.7.2	Requirements for small data transmissions based on 2-step RA	262

5.7.3	Requirements for small data transmissions based on 4-step RA	262

5.7.4	Applicability conditions for SDT	262

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	262

5.7B.1	Introduction	262

5.7B.2	Requirements for small data transmissions based on 2-step RA	262

5.7B.3	Requirements for small data transmissions based on 4-step RA	263

5.7B.4	Applicability conditions for RA-SDT for RedCap	263

6	RRC_CONNECTED state mobility	263

6.1	Handover	263

6.1.1	NR Handover	263

6.1.1.1	Introduction	263

6.1.1.2	NR FR1 - NR FR1 Handover	263

6.1.1.2.1	Handover delay	263

6.1.1.2.2	Interruption time	263

6.1.1.3	NR FR2- NR FR1 Handover	264

6.1.1.3.1	Handover delay	264

6.1.1.3.2	Interruption time	264

6.1.1.4	NR FR2- NR FR2 Handover	265

6.1.1.4.1	Handover delay	265

6.1.1.4.2	Interruption time	265

6.1.1.5	NR FR1- NR FR2 Handover	266

6.1.1.5.1	Handover delay	266

6.1.1.5.2	Interruption time	266

6.1.2	NR Handover to other RATs	267

6.1.2.1	NR – E-UTRAN Handover	267

6.1.2.1.1	Introduction	267

6.1.2.1.2	Handover delay	267

6.1.2.1.3	Interruption time	267

6.1.2.2	NR – UTRAN Handover	268

6.1.2.2.1	Introduction	268

6.1.2.2.2	Handover delay	268

6.1.2.2.3	Interruption time	268

6.1.3	NR DAPS Handover	269

6.1.3.1	Introduction	269

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	269

6.1.3.2.1	DAPS handover delay	270

6.1.3.2.2	Interruption time	270

6.1.3.3	NR FR2- NR FR1 DAPS Handover	272

6.1.3.3.1	DAPS handover delay	272

6.1.3.3.2	Interruption time	273

6.1.3.4	NR FR1- NR FR2 DAPS Handover	273

6.1.3.4.1	DAPS handover delay	273

6.1.3.4.2	Interruption time	274

6.1.4	NR Conditional Handover	274

6.1.4.1	Introduction	274

6.1.4.2	NR FR1 – NR FR1 conditional handover	274

6.1.4.3	NR FR2 – NR FR1 conditional handover	276

6.1.4.4	NR FR2 – NR FR2 conditional handover	276

6.1.4.4.1	Handover delay	276

6.1.4.4.2	Measurement time	276

6.1.4.4.3	Preparation time	277

6.1.4.4.4	Interruption time	277

6.1.4.5	NR FR1 – NR FR2 conditional handover	277

6.1.5	NR Handover with PSCell	277

6.1.5.1	Introduction	277

6.1.5.2	Handover with PSCell from NR SA to EN-DC	278

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	278

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	278

6.1.5.3	HO with PSCell from NE-DC to NE-DC	279

6.1.5.3.1	Handover delay	279

6.1.5.3.2	HO with PSCell - PCell Interruption time	279

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	279

6.1.5.4	HO with PSCell from NR-DC to NR-DC	280

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	281

6.1.5.5.1	Introduction	281

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	281

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	282

6.1A	Void	283

6.1A.1	Void	283

6.1A.1.1	Void	283

6.1A.1.2	Void	283

6.1A.1.2.1	Void	283

6.1A.1.2.2	Void	283

6.1B	Handover to target cell using CCA	283

6.1B.1	NR Handover	283

6.1B.1.1	Introduction	283

6.1B.1.2	NR FR1 - NR FR1 Handover	283

6.1B.1.2.1	Handover delay	283

6.1B.1.2.2	Interruption time	283

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	284

6.1B.1.3.1	Handover delay	284

6.1B.1.3.2	Interruption time	284

6.1B.1.4	NR FR1- NR FR2-2 Handover	285

6.1B.1.4.1	Handover delay	285

6.1B.1.4.2	Interruption time	286

6.1C	Handover for SAN	287

6.1C.1	NR SAN Handover	287

6.1C.1.1	Introduction	287

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	287

6.1C.1.2.1	Handover delay	287

6.1C.1.2.2	Interruption time	287

6.1C.2	NR SAN Conditional Handover	288

6.1C.2.1	Introduction	288

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	288

6.1C.2.2.1	Handover delay	288

6.1C.2.2.2	Measurement time	288

6.1C.2.2.3	Preparation time	290

6.1C.2.2.4	Interruption time	290

6.1D	Handover for RedCap	290

6.1D.1	NR Handover	290

6.1D.1.1	Introduction	290

6.1D.1.2	NR FR1 - NR FR1 Handover	291

6.1D.1.2.1	Handover delay	291

6.1D.1.2.2	Interruption time	291

6.1D.1.3	NR FR2- NR FR2 Handover	292

6.1D.1.3.1	Handover delay	292

6.1D.1.3.2	Interruption time	292

6.1D.2	NR Handover to other RATs	294

6.1D.2.1	NR – E-UTRAN Handover	294

6.2	RRC Connection Mobility Control	294

6.2.1	SA: RRC Re-establishment	294

6.2.1.1	Introduction	294

6.2.1.2	Requirements	294

6.2.1.2.1	UE Re-establishment delay requirement	294

6.2.1A	RRC Re-establishment with CCA	296

6.2.1A.1	Introduction	296

6.2.1A.2	Requirements	296

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	297

6.2.1B	SA: RRC Re-establishment for RedCap	298

6.2.1B.1	Introduction	298

6.2.1B.2	Requirements	299

6.2.2	Random access	299

6.2.2.1	Introduction	299

6.2.2.2	Requirements for 4-step RA type	299

6.2.2.2.1	Contention based random access	300

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	300

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	300

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	300

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	300

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	300

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	300

6.2.2.2.2	Non-Contention based random access	300

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	300

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	301

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	301

6.2.2.2.3	UE behaviour when configured with supplementary UL	301

6.2.2.3	Requirements for 2-step RA type	301

6.2.2.3.1	Contention based random access	302

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	302

6.2.2.3.1.2	Correct behaviour when receiving MsgB	302

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	302

6.2.2.3.2	Non-Contention based random access	303

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	303

6.2.2.3.2.2	Correct behaviour when receiving MsgB	303

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	303

6.2.2.3.3	UE behaviour when configured with supplementary UL	303

6.2.2A	Random access when CCA is used on target frequency	303

6.2.2A.1	Introduction	303

6.2.2A.2	Requirements for 4-step RA type	304

6.2.2A.2.1	Contention based random access	304

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	304

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	304

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	304

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	305

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	305

6.2.2A.2.2	Non-Contention based random access	305

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	305

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	305

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	306

6.2.2A.3	Requirements for 2-step RA type	306

6.2.2A.3.1	Contention based random access	306

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	306

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	307

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	307

6.2.2A.3.2	Non-Contention based random access	307

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	307

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	308

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	308

6.2.2B	Random access for RedCap	308

6.2.2B.1	Introduction	308

6.2.2B.2	Requirements	309

6.2.3	SA: RRC Connection Release with Redirection	309

6.2.3.1	Introduction	309

6.2.3.2	Requirements	309

6.2.3.2.1	RRC connection release with redirection to NR	309

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	310

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	311

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	312

6.2.3A.1	Introduction	312

6.2.3A.2	Requirements	312

6.2.3A.2.1	RRC connection release with redirection to NR	312

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	312

6.2C	RRC Connection Mobility Control for Satellite Access	312

6.2C.1	SA: RRC Re-establishment for Satellite Access	312

6.2C.1.1	Introduction	312

6.2C.1.2	Requirements	313

6.2C.1.2.1	UE Re-establishment delay requirement	313

6.2C.2	Random access for satellite access	314

6.2C.2.1	Introduction	314

6.2C.2.2	Requirements for 4-step RA type	314

6.2C.2.2.1	Contention based random access	315

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	315

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	315

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	315

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	315

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	315

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	315

6.2C.2.2.2	Non-Contention based random access	315

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	315

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	316

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	316

6.2C.2.3	Requirements for 2-step RA type	316

6.2C.2.3.1	Contention based random access	317

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	317

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	317

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	317

6.2C.2.3.2	Non-Contention based random access	317

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	317

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	318

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	318

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	318

6.2C.3.1	Introduction	318

6.2C.3.2	Requirements	318

6.2C.3.2.1	RRC connection release with redirection to NR	318

7	Timing	319

7.1	UE transmit timing	319

7.1.1	Introduction	319

7.1.2	Requirements	319

7.1.2.1	Gradual timing adjustment	321

7.1.2.2	Void	322

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	322

7.1A	UE transmit timing for RedCap	322

7.1A.1	Introduction	322

7.1A.2	Requirements	323

7.1A.2.1	Gradual timing adjustment	324

7.1C	UE transmit timing for Satellite Access	324

7.1C.1	Introduction	324

7.1C.2	Requirements	324

7.1C.2.1	Gradual timing adjustment	325

7.2	UE timer accuracy	326

7.2.1	Introduction	326

7.2.2	Requirements	326

7.2A	UE timer accuracy for RedCap	326

7.2A.1	Introduction	326

7.2A.2	Requirements	326

7.2C	UE timer accuracy for satellite access	326

7.2C.1	Introduction	326

7.2C.2	Requirements	327

7.3	Timing advance	327

7.3.1	Introduction	327

7.3.2	Requirements	327

7.3.2.1	Timing Advance adjustment delay	327

7.3.2.2	Timing Advance adjustment accuracy	327

7.3A	Timing Advance for RedCap	327

7.3A.1	Introduction	327

7.3A.2	Requirements	328

7.3A.2.1	Timing Advance adjustment delay	328

7.3A.2.2	Timing Advance adjustment accuracy	328

7.3C	Timing advance for satellite access	328

7.3C.1	Introduction	328

7.3C.2	Requirements	328

7.3C.2.1	Timing Advance adjustment delay	328

7.3C.2.2	Timing Advance adjustment accuracy	328

7.4	Cell phase synchronization accuracy	329

7.4.1	Definition	329

7.4.2	Minimum requirements	329

7.5	Maximum Transmission Timing Difference	329

7.5.1	Introduction	329

7.5.2	Minimum Requirements for inter-band EN-DC	329

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	330

7.5.3	Minimum Requirements for intra-band EN-DC	330

7.5.4	Minimum Requirements for NR Carrier Aggregation	331

7.5.5	Minimum Requirements for inter-band NE-DC	331

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	332

7.5.6	Minimum Requirements for inter-band NR DC	332

7.6	Maximum Receive Timing Difference	333

7.6.1	Introduction	333

7.6.2	Minimum Requirements for inter-band EN-DC	333

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	334

7.6.3	Minimum Requirements for intra-band EN-DC	334

7.6.4	Minimum Requirements for NR Carrier Aggregation	335

7.6.5	Minimum Requirements for inter-band NE-DC	336

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	336

7.6.6	Minimum Requirements for inter-band NR DC	336

7.7	deriveSSB-IndexFromCell tolerance	337

7.7.1	Minimum requirements	337

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	338

7.7A.1	Minimum requirements	338

7.8	Void	338

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	338

7.9.1	Minimum requirements	338

8	Signalling characteristics	339

8.1	Radio Link Monitoring	339

8.1.1	Introduction	339

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	340

8.1.2	Requirements for SSB based radio link monitoring	340

8.1.2.1	Introduction	341

8.1.2.2	Minimum requirement	341

8.1.2.3	Measurement restrictions for SSB based RLM	345

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	345

8.1.3	Requirements for CSI-RS based radio link monitoring	346

8.1.3.1	Introduction	346

8.1.3.2	Minimum requirement	347

8.1.3.3	Measurement restrictions for CSI-RS based RLM	350

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	351

8.1.4	Minimum requirement at transitions	352

8.1.5	Minimum requirement for UE turning off the transmitter	352

8.1.6	Minimum requirement for L1 indication	352

8.1.7	Scheduling availability of UE during radio link monitoring	353

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	353

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	353

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	353

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	354

8.1A	Radio Link Monitoring with CCA on Target Frequency	354

8.1A.1	Introduction	354

8.1A.2	Requirements for SSB Based Radio Link Monitoring	355

8.1A.2.1	Introduction	355

8.1A.2.2	Minimum Requirement	356

8.1A.3	Minimum requirement at transitions	359

8.1A.4	Minimum requirement for UE turning off the transmitter	360

8.1A.5	Minimum requirement for L1 indication	360

8.1A.6	Scheduling availability of UE during radio link monitoring	360

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	360

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	361

8.1B	Radio Link Monitoring for RedCap	361

8.1B.1	Introduction	361

8.1B.2	Requirements for SSB based radio link monitoring	362

8.1B.2.1	Introduction	362

8.1B.2.2	Minimum requirement	363

8.1B.2.3	Measurement restrictions for SSB based RLM	365

8.1B.3	Requirements for CSI-RS based radio link monitoring	366

8.1B.3.1	Introduction	366

8.1B.3.2	Minimum requirement	366

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	369

8.1B.4	Minimum requirement at transitions	370

8.1B.5	Minimum requirement for UE turning off the transmitter	370

8.1B.6	Minimum requirement for L1 indication	370

8.1B.7	Scheduling availability of UE during radio link monitoring	370

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	371

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	371

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	371

8.1C	Radio Link Monitoring for Satellite Access	371

8.1C.1	Introduction	371

8.1C.2	Requirements for SSB based radio link monitoring	372

8.1C.2.1	Introduction	372

8.1C.2.2	Minimum requirement	373

8.1C.2.3	Measurement restrictions for SSB based RLM	374

8.1C.3	Requirements for CSI-RS based radio link monitoring	374

8.1C.3.1	Introduction	374

8.1C.3.2	Minimum requirement	375

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	376

8.1C.4	Minimum requirement at transitions	377

8.1C.5	Minimum requirement for UE turning off the transmitter	377

8.1C.6	Minimum requirement for L1 indication	377

8.1C.7	Scheduling availability of UE during radio link monitoring	377

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	378

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	378

8.2	Interruption	378

8.2.1	EN-DC Interruption	378

8.2.1.1	Introduction	378

8.2.1.2	Requirements	379

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	379

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	379

8.2.1.2.3	Interruptions at SCell addition/release	379

8.2.1.2.4	Interruptions at SCell activation/deactivation	380

8.2.1.2.5	Interruptions during measurements on SCC	382

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	382

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	382

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	382

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	383

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	383

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	384

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	385

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	385

8.2.1.2.8.2	Interruptions during direct SCell activation	385

8.2.1.2.9	Interruptions at SCell hibernation	385

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	386

8.2.1.2.11	Interruptions due to UE-specific CBW change	386

8.2.1.2.12	Interruptions at NR SRS carrier based switching	386

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	388

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	389

8.2.1.2.15	Interruptions due to SCell dormancy	389

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	389

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	389

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	390

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	390

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	390

8.2.1.2.18	Interruptions at NR SRS antenna port switching	391

8.2.1.2.19	Interruptions at fast SCell activation	392

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	393

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	393

8.2.2.1	Introduction	393

8.2.2.2	Requirements	394

8.2.2.2.1	Interruptions at SCell addition/release	394

8.2.2.2.2	Interruptions at SCell activation/deactivation	395

8.2.2.2.3	Interruptions during measurements on deactivated SCC	396

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	397

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	397

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	398

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	400

8.2.2.2.8	Interruptions due to UE-specific CBW change	400

8.2.2.2.9	Interruptions at NR SRS carrier based switching	400

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	402

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	402

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	403

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	403

8.2.2.2.11	Interruptions at direct SCell activation	403

8.2.2.2.12	Interruptions due to SCell dormancy	404

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	404

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	404

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	404

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	404

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	404

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	405

8.2.2.2.16	Interruptions at NR SRS antenna port switching	405

8.2.2.2.17	Interruptions at fast SCell activation	406

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	407

8.2.3	NE-DC Interruptions	407

8.2.3.1	Introduction	407

8.2.3.2	Requirements	408

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	408

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	408

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	408

8.2.3.2.4	Interruptions at SCell activation/deactivation	409

8.2.3.2.5	Interruptions during measurements on SCC	411

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	411

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	411

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	411

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	411

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	412

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	412

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	412

8.2.3.2.9	Interruptions at SCell hibernation	413

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	413

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	413

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	415

8.2.3.2.13	Interruptions due to SCell dormancy	415

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	416

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	416

8.2.3.2.17	Interruptions at fast SCell activation	418

8.2.3.2.18	Interruptions due to UE-specific CBW change	419

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	419

8.2.4	NR-DC: Interruptions	419

8.2.4.1	Introduction	419

8.2.4.2	Requirements	420

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	420

8.2.4.2.2	Interruptions at SCell activation/deactivation	421

8.2.4.2.3	Interruptions during measurements on SCC	422

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	422

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	423

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	423

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	423

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	424

8.2.4.2.9	Interruptions at NR SRS carrier based switching	424

8.2.4.2.10	Interruptions at direct SCell activation	425

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	426

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	426

8.2.4.2.13	 Interruptions due to SCell dormancy	427

8.2.4.2.14	Interruptions at NR SRS antenna port switching	427

8.2.4.2.15	Interruptions at fast SCell activation	428

8.2.4.2.16	Interruptions at SCG activation/deactivation	429

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	429

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PSCell	430

8.2.4.2.19	Interruptions due to UE-specific CBW change	430

8.2.4.2A	Void	430

8.2.4.2A.1	Void	430

8.2.4.2A.2	Void	430

8.2.4.2A.3	Void	430

8.3	SCell Activation and Deactivation Delay	430

8.3.1	Introduction	430

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	430

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	435

8.3.4	Direct SCell Activation at SCell addition	435

8.3.5	Direct SCell Activation at Handover	437

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	438

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	442

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	443

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	444

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	445

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	448

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	450

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	450

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	451

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	453

8.3A.1	Introduction	453

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	454

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	458

8.4	UE UL carrier RRC reconfiguration delay	459

8.4.1	Introduction	459

8.4.2	UE UL carrier configuration delay requirement	459

8.4.3	UE UL carrier deconfiguration delay requirement	459

8.5	Link Recovery Procedures	459

8.5.1	Introduction	459

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	460

8.5.2	Requirements for SSB based beam failure detection	461

8.5.2.1	Introduction	461

8.5.2.2	Minimum requirement	461

8.5.2.3	Measurement restriction for SSB based beam failure detection	464

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	465

8.5.3	Requirements for CSI-RS based beam failure detection	466

8.5.3.1	Introduction	466

8.5.3.2	Minimum requirement	466

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	470

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	471

8.5.4	Minimum requirement for L1 indication	471

8.5.5	Requirements for SSB based candidate beam detection	472

8.5.5.1	Introduction	472

8.5.5.2	Minimum requirement	472

8.5.5.3	Measurement restriction for SSB based candidate beam detection	475

8.5.6	Requirements for CSI-RS based candidate beam detection	475

8.5.6.1	Introduction	475

8.5.6.2	Minimum requirement	476

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	479

8.5.7	Scheduling availability of UE during beam failure detection	479

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	479

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	480

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	480

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	481

8.5.8	Scheduling availability of UE during candidate beam detection	481

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	481

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	481

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	481

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	482

8.5.9	Requirements for Beam Failure Recovery in SCell	482

8.5.9.1	Introduction	482

8.5.9.2	Requirement	482

8.5.10	Minimum requirement at transitions for beam failure detection	482

8.5A	Link Recovery Procedures when CCA is used on target frequency	483

8.5A.1	Introduction	483

8.5A.2	Requirements for SSB based beam failure detection	484

8.5A.2.1	Introduction	484

8.5A.2.2	Minimum requirement	484

8.5A.2.3	Measurement restriction for SSB based beam failure detection	486

8.5A.3	Void	486

8.5A.4	Minimum requirement for L1 indication	486

8.5A.5	Requirements for SSB based candidate beam detection	487

8.5A.5.1	Introduction	487

8.5A.5.2	Minimum requirement	487

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	489

8.5A.6	Void	490

8.5A.7	Scheduling availability of UE during beam failure detection	490

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	490

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	490

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	490

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	490

8.5A.8	Scheduling availability of UE during candidate beam detection	490

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	490

8.5A.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	491

8.5B	Link Recovery Procedures for Redcap	491

8.5B.1	Introduction	491

8.5B.2	Requirements for SSB based beam failure detection for Redcap	491

8.5B.2.1	Introduction	491

8.5B.2.2	Minimum requirement	492

8.5B.2.3	Measurement restriction for SSB based beam failure detection	493

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	494

8.5B.3.1	Introduction	494

8.5B.3.2	Minimum requirement	494

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	496

8.5B.4	Minimum requirement for L1 indication for Redcap	497

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	498

8.5B.5.1	Introduction	498

8.5B.5.2	Minimum requirement	498

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	500

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	500

8.5B.6.1	Introduction	500

8.5B.6.2	Minimum requirement	500

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	502

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	503

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	503

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	503

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	503

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	504

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	504

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	504

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	504

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	504

8.5C	Link Recovery Procedures for Satellite Access	505

8.5C.1	Introduction	505

8.5C.2	Requirements for SSB based beam failure detection	505

8.5C.2.1	Introduction	505

8.5C.2.2	Minimum requirement	506

8.5C.2.3	Measurement restriction for SSB based beam failure detection	506

8.5C.3	Requirements for CSI-RS based beam failure detection	507

8.5C.3.1	Introduction	507

8.5C.3.2	Minimum requirement	507

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	508

8.5C.4	Minimum requirement for L1 indication	509

8.5C.5	Requirements for SSB based candidate beam detection	509

8.5C.5.1	Introduction	509

8.5C.5.2	Minimum requirement	509

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	510

8.5C.6	Requirements for CSI-RS based candidate beam detection	511

8.5C.6.1	Introduction	511

8.5C.6.2	Minimum requirement	511

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	512

8.5C.7	Scheduling availability of UE during beam failure detection	512

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	512

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	512

8.5C.8	Scheduling availability of UE during candidate beam detection	513

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	513

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	513

8.5C.9	Minimum requirement at transitions for beam failure detection	513

8.6	Active BWP switch delay	513

8.6.1	Introduction	513

8.6.2	DCI and timer based BWP switch delay on a single CC	514

8.6.2A	DCI based BWP switch delay on multiple CCs	515

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	515

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	517

8.6.2B	Timer based BWP switch delay on multiple CCs	517

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	517

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	517

8.6.3	RRC based BWP switch delay on a single CC	518

8.6.3A	RRC based BWP switch delay on multiple CCs	519

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	519

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	519

8.6.4	BWP switch delay on Consistent UL CCA recovery	520

8.6A	Active BWP switch delay for RedCap	520

8.6A.1	Introduction	520

8.6A.2	DCI and timer based BWP switch delay on a single CC	520

8.6A.3	RRC based BWP switch delay on a single CC	522

8.6C	Active BWP switch delay for satellite access	522

8.6C.1	Introduction	522

8.6C.2	DCI and timer based BWP switch delay on a single CC	522

8.6C.3	RRC based BWP switch delay on a single CC	524

8.7	Void	524

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	524

8.8.1	Introduction	524

8.8.2	E-UTRAN PSCell Addition Delay Requirement	524

8.8.3	E-UTRAN PSCell Release Delay Requirement	525

8.9	NR-DC: PSCell Addition and Release Delay	525

8.9.1	Introduction	525

8.9.2	PSCell Addition Delay Requirement	525

8.9.3	PSCell Release Delay Requirement	526

8.9A	Conditional PSCell Addition Delay	526

8.9A.1	Introduction	526

8.9A.2	Conditional PSCell Addition Delay Requirement	526

8.9A.2.1	Measurement time	527

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	527

8.9B.1	Introduction	527

8.9B.2	PSCell Addition Delay Requirement	527

8.9B.3	PSCell Release Delay Requirement	528

8.10	Active TCI state switching delay	529

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	530

8.10.4	DCI based TCI state switch delay	530

8.10.5	RRC based TCI state switch delay	531

8.10.6	Active TCI state list update delay	531

8.10A	Active TCI state switching delay with CCA	531

8.10A.1	Introduction	531

8.10A.2	Known conditions for TCI state	532

8.10A.3	MAC-CE based TCI state switch delay	532

8.10A.4	DCI based TCI state switch delay	533

8.10A.5	RRC based TCI state switch delay	533

8.10A.6	Active TCI state list update delay	534

8.10B	Active TCI state switching delay for RedCap	534

8.10B.1	Introduction	534

8.10B.2	Known conditions for TCI state	534

8.10B.3	MAC-CE based TCI state switch delay	535

8.10B.4	DCI based TCI state switch delay	536

8.10B.5	RRC based TCI state switch delay	536

8.10B.6	Active TCI state list update delay	536

8.10C	Active TCI state switching delay for satellite access	537

8.10C.1	Introduction	537

8.10C.2	MAC-CE based TCI state switch delay	537

8.10C.4	DCI based TCI state switch delay	537

8.10C.5	RRC based TCI state switch delay	537

8.10C.6	Active TCI state list update delay	537

8.11	PSCell Change	538

8.11A	PSCell Change in Carriers with CCA	538

8.11B	Conditional PSCell Change	538

8.11B.1	Introduction	538

8.11B.2	Conditoinal PSCell Change delay	539

8.11B.2.1	Measurement time	539

8.11D	Conditional PSCell Change in Carriers with CCA	540

8.11D.1	Introduction	540

8.11D.2	Conditional PSCell Change delay	540

8.11D.2.1	Measurement time	541

8.12	Uplink spatial relation switch delay	541

8.12.1	Introduction	541

8.12.2	Known conditions for spatial relation when associated with DL-RS	541

8.12.3	MAC-CE based spatial relation switch delay	542

8.12.4	DCI based spatial relation switch delay	542

8.12.5	RRC based spatial relation switch delay	543

8.12A	Uplink spatial relation switch delay for RedCap	543

8.12A.1	Introduction	543

8.12A.2	Known conditions for spatial relation when associated with DL-RS	543

8.12A.3	MAC-CE based spatial relation switch delay	544

8.12A.4	DCI based spatial relation switch delay	544

8.12A.5	RRC based spatial relation switch delay	545

8.12C	Uplink spatial relation switch delay for satellite access	545

8.12C.1	Void	546

8.12C.2	Void	546

8.12C.3	Void	546

8.12C.4	Void	546

8.12C.5	Void	546

8.13	UE-specific CBW change	546

8.13.1	Introduction	546

8.13.2	UE-specific CBW change delay	546

8.13A	UE-specific CBW change for RedCap	546

8.13A.1	Introduction	546

8.13A.2	UE-specific CBW change delay	546

8.13C	UE-specific CBW change for satellite access	547

8.13C.1	Introduction	547

8.13C.2	UE-specific CBW change delay	547

8.14	Pathloss reference signal switching delay	547

8.14.1	Introduction	547

8.14.2	Known conditions for pathloss reference signal	548

8.14.3	MAC-CE based pathloss reference signal switch delay	548

8.14C	Pathloss reference signal switching delay for satellite access	549

8.14C.1	Introduction	549

8.14C.2	Known conditions for pathloss reference signal	549

8.14C.3	MAC-CE based pathloss reference signal switch delay	549

8.15	Active downlink TCI state switching delay for unified TCI	550

8.15.1	Introduction	550

8.15.4	DCI based downlink TCI state switch delay	552

8.15.5	Active Downlink TCI state list update delay	552

8.16	Active uplink TCI state switching delay for unified TCI	553

8.16.1	Introduction	553

8.16.4	DCI based uplink TCI state switch delay	555

8.16.5	Active Uplink TCI state list update delay	556

8.17	SCG Activation and Deactivation Delay	557

8.17.1	Introduction	557

8.17.2	SCG Activation Delay Requirement	557

8.17.3	SCG Deactivation Delay Requirement	558

8.18	TRP specific Link Recovery Procedures	559

8.18.1	Introduction	559

8.18.2	Requirements for TRP specific SSB based beam failure detection	559

8.18.2.1	Introduction	559

8.18.2.2	Minimum requirement	560

8.18.2.3	Measurement restriction for SSB based beam failure detection	562

8.18.3	Requirements for CSI-RS based beam failure detection	562

8.18.3.1	Introduction	562

8.18.3.2	Minimum requirement	563

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	565

8.18.4	Minimum requirement for L1 indication	566

8.18.5	Requirements for SSB based candidate beam detection	566

8.18.5.1	Introduction	566

8.18.5.2	Minimum requirement	566

8.18.5.3	Measurement restriction for SSB based candidate beam detection	568

8.18.6	Requirements for CSI-RS based candidate beam detection	569

8.18.6.1	Introduction	569

8.18.6.2	Minimum requirement	569

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	571

8.18.7	Requirements for TRP specific Beam Failure Recovery	572

8.18.7.1	Introduction	572

8.18.7.2	Requirement	572

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	572

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	572

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	573

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	573

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	573

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	574

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	574

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	574

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	574

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	575

8.19	Pre-configured measurement gap activation/deactivation delay	575

8.19.1	Introduction	575

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	575

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	575

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	575

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	576

9	Measurement Procedure	577

9.1	General measurement requirement	577

9.1.1	Introduction	577

9.1.2	Measurement gap	577

9.1.2.1	EN-DC: Measurement Gap Sharing	589

9.1.2.1a	SA: Measurement Gap Sharing	590

9.1.2.1b	NE-DC: Measurement Gap Sharing	591

9.1.2.1c	NR-DC: Measurement Gap Sharing	592

9.1.3	UE Measurement capability	593

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	593

9.1.3.1a	SA: Monitoring of multiple layers using gaps	593

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	594

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	594

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	595

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	596

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	596

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	597

9.1A.3.2	Void	598

9.1.3A	UE Measurement capability under operation mode with CCA	598

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	598

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	598

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	598

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	599

9.1.3C	UE Measurement capability under operation mode with satellite access	599

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	599

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	599

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	600

9.1.4.1	Introduction	600

9.1.4.2	Requirements	600

9.1.5	Carrier-specific scaling factor	604

9.1.5.1	Monitoring of multiple layers outside gaps	605

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	607

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	608

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	609

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	609

9.1.5.2	Monitoring of multiple layers within gaps	610

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	612

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	614

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	616

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	618

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	619

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	620

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	620

9.1.5.3	Monitoring of multiple layers within NCSG	620

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	621

9.1.6	Minimum requirement at transitions	621

9.1.7	Pre-configured measurement gap	622

9.1.7.1	Introduction	622

9.1.7.2	Requirements applicability	622

9.1.7.3	Requirements	623

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	623

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	624

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	624

9.1.8	Concurrent measurement gaps	624

9.1.8.1	Introduction	624

9.1.8.2	Requirements	624

9.1.8.3	Collision between concurrent measurement gaps	626

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	626

9.1.9	Network controlled small gap	626

9.1.9.1	Introduction	626

9.1.9.2	Requirements applicability	627

9.1.10	MUSIM gaps	630

9.1.11	UL gap for Tx power management	631

9.1A	General measurement requirement for RedCap	632

9.1A.1	Introduction	632

9.1A.2	Measurement gap	632

9.1A.2.1	SA: Measurement Gap Sharing	636

9.1A.3	UE Measurement capability	637

9.1A.3.1	SA: Monitoring of multiple layers using gaps	637

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	637

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	637

9.1A.4.1	Introduction	637

9.1A.4.2	Requirements	638

9.1A.5	Carrier-specific scaling factor	638

9.1A.5.1	Monitoring of multiple layers outside gaps	638

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	639

9.1A.5.2	Monitoring of multiple layers within gaps	639

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	639

9.1A.6	Minimum requirement at transitions	640

9.1C	General measurement requirement for SAN	641

9.1C.1	Introduction	641

9.1C.2	Measurement gap	641

9.1C.8	Concurrent measurement gaps for SAN	643

9.1C.8.1	Introduction	643

9.1C.8.2	Requirements	643

9.1C.8.3	Collision between concurrent measurement gaps	643

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	644

9.1C.9	Collision between SMTC and measurement gap for SAN	644

9.1C.9.1	Introduction	644

9.1C.9.2	Collision between SMTCs and measurement gap	644

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	644

9.2	NR intra-frequency measurements	645

9.2.1	Introduction	645

9.2.2	Requirements applicability	647

9.2.3	Number of cells and number of SSB	647

9.2.3.1	Requirements for FR1	647

9.2.3.2	Requirements for FR2	647

9.2.4	Measurement Reporting Requirements	647

9.2.4.1	Periodic Reporting	647

9.2.4.2	Event-triggered Periodic Reporting	648

9.2.4.3	Event Triggered Reporting	648

9.2.5	Intrafrequency measurements without measurement gaps	648

9.2.5.1	Intrafrequency cell identification	648

9.2.5.2	Measurement period	653

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	656

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	656

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	656

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	657

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	658

9.2.5.4	SFTD Measurements between PCell and PSCell	659

9.2.5.4.1	Introduction	659

9.2.5.4.2	SFTD Measurement delay	659

9.2.5.4.3	SFTD Measurement Reporting Delay	659

9.2.6	Intra-frequency measurements with measurement gaps	660

9.2.6.1	Void	660

9.2.6.2	Intra-frequency cell identification	660

9.2.6.3	Intrafrequency Measurement Period	662

9.2.7	Intra-frequency measurements with NCSG	664

9.2.7.1	Intra-frequency cell identification	664

9.2.7.2	Measurement period	666

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	667

9.2A	NR intra-frequency measurements with CCA	667

9.2A.1	Introduction	667

9.2A.2	Requirements applicability	668

9.2A.3	Number of cells and number of SSB	668

9.2A.3.1	Requirements for FR1	668

9.2A.3.2	Requirements for FR2-2	669

9.2A.4	Measurement Reporting Requirements	669

9.2A.5	Intra-frequency measurements without measurement gaps	670

9.2A.5.2	Measurement period	674

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	676

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	677

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	677

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	677

9.2A.6	Intra-frequency measurements with measurement gaps	677

9.2A.6.1	Intra-frequency cell identification	677

9.2A.6.2	Intra-frequency Measurement Period	680

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	681

9.2A.7.1	Intra-frequency RSSI measurements	681

9.2A.7.2	Intra-frequency Channel occupancy measurements	683

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	684

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	684

9.2B	NR intra-frequency measurements for RedCap	685

9.2B.1	Introduction	685

9.2B.2	Requirements applicability	685

9.2B.3	Number of cells and number of SSB	686

9.2B.3.1	Requirements for FR1	686

9.2B.3.2	Requirements for FR2	686

9.2B.4	Measurement Reporting Requirements	686

9.2B.4.1	Periodic Reporting	686

9.2B.4.2	Event-triggered Periodic Reporting	686

9.2B.4.3	Event Triggered Reporting	686

9.2B.5	Intra-frequency measurements without measurement gaps	687

9.2B.5.1	Intra-frequency cell identification	687

9.2B.5.2	Measurement period	689

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	690

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	690

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	690

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	691

9.2B.5.3.4	Scheduling availability of HD-FDD UE performing measurements on FR1	691

9.2B.6	Intra-frequency measurements with measurement gaps	692

9.2B.6.1	Intra-frequency cell identification	692

9.2B.6.2	Intra-frequency Measurement Period	693

9.2C	NR intra-frequency measurements for SAN	694

9.2C.1	Introduction	694

9.2C.2	Requirements applicability	695

9.2C.3.1	Requirements for FR1-NTN	696

9.2C.3.1	Requirements for FR1	696

9.2C.4	Measurement Reporting Requirements	696

9.2C.4.1	Periodic Reporting	696

9.2C.4.2	Event-triggered Periodic Reporting	696

9.2C.4.3	Event Triggered Reporting	696

9.2C.5	Intra frequency measurements without measurement gaps	697

9.2C.5.1	Intra frequency cell identification	697

9.2C.5.2	Measurement period	699

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	699

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	699

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	700

9.2C.6	Intra-frequency measurements with measurement gaps	700

9.2C.6.1	Intra-frequency cell identification	700

9.2C.6.3	Intrafrequency Measurement Period	702

9.3	NR inter-frequency measurements	702

9.3.1	Introduction	702

9.3.2	Requirements applicability	704

9.3.2.1	Void	704

9.3.2.2	Void	704

9.3.3	Number of cells and number of SSB	704

9.3.3.1	Requirements for FR1	704

9.3.3.2	Requirements for FR2	704

9.3.4	Inter-frequency measurement with measurement gaps	704

9.3.4.1	Void	708

9.3.4.2	Void	708

9.3.5	Inter-frequency measurements	708

9.3.5.1	Void	709

9.3.5.2	Void	709

9.3.5.3	Void	709

9.3.6	Inter-frequency measurements reporting requirements	709

9.3.6.1	Periodic Reporting	709

9.3.6.2	Event-triggered Periodic Reporting	709

9.3.6.3	Event-triggered Reporting	709

9.3.7	Void	710

9.3.8	Inter-frequency SFTD measurement requirements	710

9.3.8.1	Introduction	710

9.3.8.2	SFTD Measurement delay	710

9.3.8.3	SFTD Measurement reporting delay	711

9.3.9	Inter frequency measurements without measurement gaps	711

9.3.9.1	Inter frequency Cell identification	711

9.3.9.2	Measurement period	714

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	716

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	716

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	716

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	717

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	717

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	717

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	718

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	718

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	719

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	720

9.3.10	Inter-frequency measurement with NCSG	721

9.3.10.1	Inter-frequency cell identification	721

9.3.10.2	Measurement period	722

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	722

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	723

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	723

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	724

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	725

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	726

9.3A.1	Introduction	726

9.3A.2	Requirements applicability	726

9.3A.3	Number of cells and number of SSB	727

9.3A.3.1	Requirements for FR1	727

9.3A.3.2	Requirements for FR2-2	727

9.3A.4	Inter-frequency cell identification	727

9.3A.5	Inter-frequency measurements	729

9.3A.6	NR Inter-frequency measurements reporting requirements	731

9.3A.6.1	Periodic Reporting	731

9.3A.6.2	Event-triggered Periodic Reporting	731

9.3A.6.3	Event-triggered Reporting	731

9.3A.8	Inter-frequency RSSI measurements	731

9.3A.9	Inter-frequency channel occupancy measurements	732

9.3B	NR inter-frequency measurements for RedCap	733

9.3B.1	Introduction	733

9.3B.2	Requirements applicability	733

9.3B.3	Number of cells and number of SSB	734

9.3B.3.1	Requirements for FR1	734

9.3B.3.2	Requirements for FR2	734

9.3B.4	Inter-frequency measurement with measurement gaps	734

9.3B.5	Inter-frequency measurements	735

9.3B.6	Inter-frequency measurements reporting requirements	736

9.3B.6.1	Periodic Reporting	736

9.3B.6.2	Event-triggered Periodic Reporting	736

9.3B.6.3	Event-triggered Reporting	736

9.3B.7	Inter frequency measurements without measurement gaps	737

9.3B.7.1	Inter frequency Cell identification	737

9.3B.7.2	Measurement period	739

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	740

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	740

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	741

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	741

9.3B.7.3.4	Scheduling availability of HD-FDD UE performing measurements on FR1	741

9.3C	NR inter-frequency measurements for SAN	742

9.3C.1	Introduction	742

9.3C.2	Requirements applicability	743

9.3C.3	Number of cells and number of SSB	743

9.3C.3.1	Requirements for FR1-NTN	743

9.3C.4	Inter-frequency measurement with measurement gaps	743

9.3C.5	Inter-frequency measurements	745

9.3C.6	Inter-frequency measurements reporting requirements	745

9.3C.6.1	Periodic Reporting	745

9.3C.6.2	Event-triggered Periodic Reporting	745

9.3C.6.3	Event-triggered Reporting	745

9.3C.7	Inter frequency measurements without measurement gaps	746

9.3C.7.1	Inter frequency Cell identification	746

9.3C.7.2	Measurement period	747

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	747

9.3C.7.3.1	Void	748

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	748

9.4	Inter-RAT measurements	748

9.4.1	Introduction	748

9.4.2	NR − E-UTRAN FDD measurements	750

9.4.2.1	Introduction	750

9.4.2.2	Requirements when no DRX is used	750

9.4.2.3	Requirements when DRX is used	751

9.4.2.4	Measurement reporting requirements	753

9.4.2.4.1	Periodic Reporting	753

9.4.2.4.2	Event-Triggered Periodic Reporting	753

9.4.2.4.3	Event-Triggered Reporting	754

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	754

9.4.3	NR − E-UTRAN TDD measurements	754

9.4.3.1	Introduction	754

9.4.3.2	Requirements when no DRX is used	754

9.4.3.3	Requirements when DRX is used	756

9.4.3.4	Measurement reporting requirements	757

9.4.3.4.1	Periodic Reporting	757

9.4.3.4.2	Event-Triggered Periodic Reporting	758

9.4.3.4.3	Event-Triggered Reporting	758

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	758

9.4.4	Inter-RAT RSTD measurements	758

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	758

9.4.4.1.1	Introduction	758

9.4.4.1.2	Requirements	759

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	762

9.4.4.2.1	Introduction	762

9.4.4.2.2	Requirements	763

9.4.5	Inter-RAT E-CID measurements	766

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	766

9.4.5.1.1	Introduction	766

9.4.5.1.2	Requirements	766

9.4.5.1.3	Measurement Reporting Delay	766

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	767

9.4.5.2.1	Introduction	767

9.4.5.2.2	Requirements	767

9.4.5.2.3	Measurement Reporting Delay	767

9.4.6	NR − UTRAN FDD measurements	767

9.4.6.1	Introduction	767

9.4.6.2	Requirements when no DRX is used	767

9.4.6.3	Requirements when DRX is used	768

9.4.7	NR – E-UTRAN measurements with autonomous gaps	770

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	770

9.4.7.2	CGI reporting delay	770

9.4A	Inter-RAT measurements for RedCap	771

9.4A.1	Introduction	771

9.4A.2	NR − E-UTRAN FDD measurements	772

9.4A.2.1	Introduction	772

9.4A.2.2	Requirements when no DRX is used	772

9.4A.2.3	Requirements when DRX is used	773

9.4A.2.4	Measurement reporting requirements	774

9.4A.2.4.1	Periodic Reporting	774

9.4A.2.4.2	Event-Triggered Periodic Reporting	774

9.4A.2.4.3	Event-Triggered Reporting	774

9.4A.3	NR − E-UTRAN TDD measurements	775

9.4A.3.1	Introduction	775

9.4A.3.2	Requirements when no DRX is used	775

9.4A.3.3	Requirements when DRX is used	777

9.4A.3.4	Measurement reporting requirements	778

9.4A.3.4.1	Periodic Reporting	778

9.4A.3.4.2	Event-Triggered Periodic Reporting	778

9.4A.3.4.3	Event-Triggered Reporting	778

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	779

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	779

9.4A.4.2	CGI reporting delay	779

9.4A.4.3	CGI reporting scheduling restriction	779

9.5	L1-RSRP measurements for Reporting	780

9.5.1	Introduction	780

9.5.2	Requirements applicability	780

9.5.3	Measurement Reporting Requirements	781

9.5.3.1	Periodic Reporting	781

9.5.3.2	Semi-Persistent Reporting	781

9.5.3.3	Aperiodic Reporting	781

9.5.4	L1-RSRP measurement requirements	782

9.5.4.1	SSB based L1-RSRP Reporting	782

9.5.4.2	CSI-RS based L1-RSRP Reporting	785

9.5.4A	Void	788

9.5.4A.1	Void	788

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	788

9.5.5.1	Measurement restriction for SSB based L1-RSRP	788

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	789

9.5.6	Scheduling availability of UE during L1-RSRP measurement	790

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	790

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	790

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	790

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	792

9.5A	L1-RSRP measurements for Reporting under CCA	792

9.5A.1	Introduction	792

9.5A.2	Requirements applicability	792

9.5A.3	Measurement Reporting Requirements	792

9.5A.3.1	Periodic Reporting	793

9.5A.3.2	Semi-Persistent Reporting	793

9.5A.3.3	Aperiodic Reporting	793

9.5A.4	L1-RSRP measurement requirements	793

9.5A.4.1	SSB based L1-RSRP Reporting	793

9.5A.5	Measurement restriction for L1-RSRP measurement	796

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	796

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	796

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	796

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	797

9.5A.6.3	(Void)	797

9.5A.6.3A	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	797

9.5A.6.3B	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	797

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	798

9.5B	L1-RSRP measurements for Reporting for RedCap	798

9.5B.1	Introduction	798

9.5B.2	Requirements applicability	798

9.5B.3	Measurement Reporting Requirements	799

9.5B.3.1	Periodic Reporting	799

9.5B.3.2	Semi-Persistent Reporting	799

9.5B.3.3	Aperiodic Reporting	799

9.5B.4	L1-RSRP measurement requirements	800

9.5B.4.1	SSB based L1-RSRP Reporting	800

9.5B.4.2	CSI-RS based L1-RSRP Reporting	801

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	804

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	804

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	805

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	805

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	805

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	805

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	806

9.5C	L1-RSRP measurements for Reporting for satellite access	806

9.5C.1	Introduction	806

9.5C.3	Measurement Reporting Requirements	807

9.5C.3.1	Periodic Reporting	807

9.5C.3.2	Semi-Persistent Reporting	807

9.5C.3.3	Aperiodic Reporting	808

9.5C.4	L1-RSRP measurement requirements	808

9.5C.4.1	SSB based L1-RSRP Reporting	808

9.5C.5	Measurement restriction for L1-RSRP measurement	810

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	810

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	810

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	811

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	811

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	811

9.6	NE-DC: Measurements	811

9.6.1	Introduction	811

9.6.2	SFTD Measurements	811

9.6.2.1	Introduction	811

9.6.2.2	SFTD Measurement requirements	811

9.7	Cross Link Interference measurements	812

9.7.1	Introduction	812

9.7.2	SRS-RSRP measurements	812

9.7.2.1	Introduction	812

9.7.2.2	Requirements applicability	813

9.7.2.3	Measurement Reporting Requirements	813

9.7.2.3.1	Periodic Reporting	813

9.7.2.3.2	Event-triggered Periodic Reporting	813

9.7.2.3.3	Event Triggered Reporting	813

9.7.2.4	Measurement capability	813

9.7.2.5	SRS-RSRP measurement period	814

9.7.3	CLI-RSSI measurements	814

9.7.3.1	Introduction	814

9.7.3.2	Requirements applicability	814

9.7.3.3	Measurement Reporting Requirements	814

9.7.3.3.1	Periodic Reporting	814

9.7.3.3.2	Event-triggered Periodic Reporting	814

9.7.3.3.3	Event Triggered Reporting	815

9.7.3.4	Measurement capability	815

9.7.3.5	CLI-RSSI measurement period	815

9.7.4	Scheduling availability of UE during CLI measurements	815

9.7.4.1	Scheduling availability of UE performing measurement on FR1	815

9.7.4.2	Scheduling availability of UE performing measurement on FR2	816

9.8	L1-SINR measurements for Reporting	816

9.8.1	Introduction	816

9.8.2	Requirements applicability	817

9.8.3	Measurement Reporting Requirements	817

9.8.3.1	Periodic Reporting	817

9.8.3.2	Semi-Persistent Reporting	818

9.8.4	L1-SINR measurement requirements	818

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	818

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	823

9.8.5	Measurement restriction for L1-SINR measurement	824

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	825

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	825

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	826

9.8.6	Scheduling availability of UE during L1-SINR measurement	826

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	826

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	826

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	827

9.9	 NR measurements for positioning	828

9.9.1	Introduction	828

9.9.1.1	General Aspects of Gap-based Measurement	828

9.9.1.2	General Aspects of Gapless Measurement	828

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	829

9.9.2	RSTD measurements	830

9.9.2.1	Introduction	830

9.9.2.2	Requirements Applicability	830

9.9.2.3	Measurement Capability	831

9.9.2.4	Measurement Reporting Requirements	831

9.9.2.4.1	Void	831

9.9.2.4.2	Void	831

9.9.2.4.3	Void	831

9.9.2.5	Measurements Period Requirements	831

9.9.2.6	Void	834

9.9.2.7	Measurements Period Requirements without Measurement Gaps	834

9.9.2.8	Void	837

9.9.2.9	Measurements Period Requirements with both MG and PPW	837

9.9.3	PRS-RSRP measurements	838

9.9.3.1	Introduction	838

9.9.3.2	Requirements applicability	838

9.9.3.3	Measurement Capability	838

9.9.3.4	Measurement Reporting Requirements	838

9.9.3.5	Measurement Period Requirements	838

9.9.3.6	Measurement Period Requirements without Measurement Gaps	841

9.9.3.7	Void	843

9.9.3.8	Measurements Period Requirements with both MG and PPW	843

9.9.4	UE Rx-Tx time difference measurements	844

9.9.4.1	Introduction	844

9.9.4.2	Requirements Applicability	844

9.9.4.3	Measurement Capability	844

9.9.4.4	Measurement Reporting Requirements	844

9.9.4.5	Measurement Period Requirements	844

9.9.4.6	Measurement Period Requirements without Measurement Gaps	848

9.9.4.7	Void	851

9.9.4.8	Measurements Period Requirements with both MG and PPW	851

9.9.5	E-CID measurements	852

9.9.5.1	Introduction	852

9.9.5.2	Measurement Requirements	852

9.9.5.2.1	Intra-frequency Measurement Requirements	852

9.9.5.2.2	Inter-frequency Measurement Requirements	852

9.9.5.2.3	Measurement Reporting Delay	853

9.9.6	PRS-RSRPP measurements	853

9.9.6.1	Introduction	853

9.9.6.2	Requirements applicability	853

9.9.6.3	Measurement capability	853

9.9.6.4	Measurement reporting requirements	853

9.9.6.5	Measurement period requirements	854

9.9.6.6	Measurement Period Requirements without Measurement Gaps	854

9.9.6.7	Void	854

9.9.6.8	Measurements Period Requirements with both MG and PPW	854

9.10	CSI-RS based L3 measurements	854

9.10.1	Introduction	854

9.10.2	CSI-RS based intra-frequency measurements	854

9.10.2.1	Introduction	854

9.10.2.2	Requirements applicability	855

9.10.2.3	Number of cells and number of CSI-RS	856

9.10.2.3.1	Requirements for FR1	856

9.10.2.3.2	Requirements for FR2	856

9.10.2.4	Measurement Reporting Requirements	856

9.10.2.4.1	Periodic Reporting	857

9.10.2.4.2	Event-triggered Periodic Reporting	857

9.10.2.4.3	Event Triggered Reporting	857

9.10.2.5	Intra-frequency measurements without measurement gaps	857

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	859

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	859

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	859

9.10.3	CSI-RS based Inter-frequency measurements	859

9.10.3.1	Introduction	859

9.10.3.2	Requirements applicability	860

9.10.3.3	Number of cells and number of CSI-RS resources	860

9.10.3.3.1	Requirements for FR1	860

9.10.3.3.2	Requirements for FR2	861

9.10.3.4	Measurements reporting requirements	861

9.10.3.4.1	Periodic Reporting	861

9.10.3.4.2	Event-triggered Periodic Reporting	861

9.10.3.4.3	Event-triggered Reporting	861

9.10.3.5	Inter frequency measurements with measurement gaps	861

9.11	NR measurements with autonomous gaps	863

9.11.1	Introduction	863

9.11.2	CGI identification of an NR cell with autonomous gaps	863

9.11.3	CGI reporting delay	864

9.11A	NR measurements with autonomous gaps for RedCap	864

9.11A.1	Introduction	864

9.11A.2	CGI identification of an NR cell with autonomous gaps	865

9.11A.3	CGI reporting delay	866

9.11A.4	CGI reporting scheduling restriction	866

9.12	Measurement for Propagation Delay Compensation	866

9.12.1	Introduction	866

9.12.2	Requirements Applicability	866

9.12.3	Measurement Capability	867

9.12.4	Measurement period requirements	867

9.12.4.1	PRS Measurement Period	867

9.12.4.2	TRS Measurement Period	868

9.12.5	Measurement Reporting Requirements	869

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	869

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	869

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	869

9.13.1	Introduction	869

9.13.2	Requirements Applicability	870

9.13.3	Measurement Reporting Requirements	870

9.13.3.1	Periodic Reporting	871

9.13.3.2	Semi-Persistent Reporting	871

9.13.3.3	Aperiodic Reporting	871

9.13.4	L1-RSRP measurement requirements	871

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	871

9.13.5	Measurement restriction for L1-RSRP measurement	873

9.13.5.1	Measurement restriction for SSB based L1-RSRP	874

9.13.6	Scheduling availability of UE during L1-RSRP measurement	874

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	875

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	875

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	875

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	875

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	876

10	Measurement Performance requirements	877

10.1	NR measurements	877

10.1.1	Introduction	877

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	877

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	877

10.1.2.1.1	Absolute SS-RSRP Accuracy	877

10.1.2.1.2	Relative SS-RSRP Accuracy	878

10.1.2.2	Void	879

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	879

10.1.2.3.1	Absolute CSI-RSRP Accuracy	879

10.1.2.3.2	Relative CSI-RSRP Accuracy	880

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	881

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	881

10.1.2B.1.1	Absolute SS-RSRP Accuracy	881

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	882

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	882

10.1.2C.1.1	Absolute SS-RSRP Accuracy	882

10.1.2C.1.2	Relative SS-RSRP Accuracy	883

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	883

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	883

10.1.3.1.1	Absolute SS-RSRP Accuracy	883

10.1.3.1.2	Relative SS-RSRP Accuracy	884

10.1.3.2	Void	885

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	885

10.1.3.3.1	Absolute CSI-RSRP Accuracy	885

10.1.3.3.2	Relative CSI-RSRP Accuracy	886

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	887

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	887

10.1.3B.1.1	Absolute SS-RSRP Accuracy	887

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	888

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	888

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	888

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	889

10.1.4.2	Void	890

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	890

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	890

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	891

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	892

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	892

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	892

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	893

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	893

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	893

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	894

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	895

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	895

10.1.5.1.1	Absolute SS-RSRP Accuracy	895

10.1.5.1.2	Relative SS-RSRP Accuracy	895

10.1.5.2	Void	896

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	896

10.1.5.3.1	Absolute CSI-RSRP Accuracy	896

10.1.5.3.2	Relative CSI-RSRP Accuracy	897

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	898

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	898

10.1.5B.1.1	Absolute SS-RSRP Accuracy	898

10.1.6	RSRP Measurement Report Mapping	899

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	901

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	901

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	901

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	902

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	902

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	903

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	903

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	903

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	904

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	904

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	904

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	905

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	905

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	905

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	906

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	906

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	906

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	906

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	907

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	907

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	907

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	907

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	908

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	909

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	909

10.1.9.2.2	Relative CSI-RSRQ Accuracy	910

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	911

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	911

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	911

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	912

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	912

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	912

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	913

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	914

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	914

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	914

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	914

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	915

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	915

10.1.10.2.2	Relative CSI-RSRQ Accuracy	916

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	917

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	917

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	917

10.1.11	RSRQ report mapping	918

10.1.12	Intra-frequency SINR accuracy requirements for FR1	918

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	918

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	918

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	919

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	919

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	920

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	920

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	920

10.1.13	Intra-frequency SINR accuracy requirements for FR2	921

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	921

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	921

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	922

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	922

10.1.14	Inter-frequency SINR accuracy requirements for FR1	922

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	922

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	922

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	923

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	924

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	924

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	925

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	926

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	926

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	926

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	927

10.1.15	Inter-frequency SINR accuracy requirements for FR2	928

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	928

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	928

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	928

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	929

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	929

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	930

10.1.16	SINR report mapping	931

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	931

10.1.17	Power Headroom	932

10.1.17.1	Power Headroom Report	932

10.1.17.1.1	Power Headroom Report Mapping	932

10.1.18	PCMAX,c,f	933

10.1.18.1	Report Mapping	933

10.1.19	L1-RSRP accuracy requirements for FR1	933

10.1.19.1	SSB based L1-RSRP accuracy requirements	933

10.1.19.1.1	Absolute Accuracy	933

10.1.19.1.2	Relative Accuracy	934

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	935

10.1.19.2.1	Absolute Accuracy	935

10.1.19.2.2	Relative Accuracy	936

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	937

10.1.19C.1	SSB based L1-RSRP accuracy requirements	937

10.1.19C.1.1	Absolute Accuracy	937

10.1.19C.1.2	Relative Accuracy	938

10.1.19C.2	CSI-RS based L1-RSRP accuracy requirements	938

10.1.19C.2.1	Absolute Accuracy	938

10.1.19C.2.2	Relative Accuracy	939

10.1.20	L1-RSRP accuracy requirements for FR2	940

10.1.20.1	SSB based L1-RSRP accuracy requirements	940

10.1.20.1.1	Absolute Accuracy	940

10.1.20.1.2	Relative Accuracy	940

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	941

10.1.20.2.1	Absolute Accuracy	941

10.1.20.2.2	Relative Accuracy	942

10.1.21	SFTD accuracy requirements	943

10.1.21.1	SFTD acuracy requirements for NE-DC	943

10.1.21.2	SFTD acuracy requirements for NR-DC	945

10.1.21.3	Inter frequency SFTD acuracy requirements	946

10.1.22	CLI measurement accuracy requirements	947

10.1.22.1	SRS-RSRP	947

10.1.22.1.1	SRS-RSRP Accuracy	947

10.1.22.1.2	SRS-RSRP report mapping	949

10.1.22.2	CLI-RSSI	949

10.1.22.2.1	CLI-RSSI Accuracy	949

10.1.22.2.2	CLI-RSSI report mapping	950

10.1.23	RSTD Measurements	951

10.1.23.1	Introduction	951

10.1.23.2	Measurement Accuracy Requirements	951

10.1.23.3	Report mapping	961

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	961

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	963

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	965

10.1.24	PRS-RSRP Measurements	967

10.1.24.1	Introduction	967

10.1.24.2	Measurement Accuracy Requirements	967

10.1.24.2.1	Absolute PRS RSRP accuracy	967

10.1.24.2.2	Relative PRS RSRP accuracy	971

10.1.24.3	Report mapping	975

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	975

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	976

10.1.25	UE Rx-Tx Time Difference Measurements	978

10.1.25.1	Introduction	978

10.1.25.2	Measurement Accuracy Requirements	978

10.1.25.3	Report mapping	991

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	991

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	993

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	994

10.1.26	FR2 P-MPR report	996

10.1.26.1	Report mapping	996

10.1.27	L1-SINR accuracy requirements for FR1	996

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	996

10.1.27.1.1	Absolute Accuracy	996

10.1.27.1.2	Relative Accuracy	997

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	998

10.1.27.2.1	Absolute Accuracy	998

10.1.27.2.2	Relative Accuracy	1000

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1002

10.1.27.3.1	Absolute Accuracy	1002

10.1.27.3.2	Relative Accuracy	1004

10.1.28	L1-SINR accuracy requirements for FR2	1006

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1013

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1013

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1013

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1013

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1013

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	1013

10.1.30.1.2	Relative Accuracy of SS-RSRQ	1014

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1015

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1015

10.1.31.1.1	Absolute SS-SINR Accuracy	1015

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1015

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1015

10.1.32.1.1	Aboslute Accuracy of SS-SINR	1015

10.1.32.1.2	Relative Accuracy of SS-SINR	1016

10.1.33	L1-RSRP accuracy requirements under CCA	1017

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1017

10.1.33.1.1	Absolute Accuracy	1017

10.1.33.1.2	Relative Accuracy	1017

10.1.34	RSSI measurements under CCA	1018

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1018

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1018

10.1.34.3	RSSI measurement report mapping	1018

10.1.35	Channel occupancy measurements under CCA	1019

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1019

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1019

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1019

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1019

10.1.36.1.1	Absolute SS-RSRP Accuracy	1019

10.1.36.1.2	Relative SS-RSRP Accuracy	1020

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1021

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1021

10.1.37.1.1	Absolute Accuracy of SS-RSRP	1021

10.1.37.1.2	Relative Accuracy of SS-RSRP	1021

10.1.38	PRS-RSRPP Measurements	1022

10.1.38.1	Introduction	1022

10.1.38.2	Measurement Accuracy Requirements	1022

10.1.38.2.1	Absolute PRS RSRPP accuracy	1022

10.1.38.3	Report mapping	1027

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1027

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1028

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1029

10.1.39.1	 Void	1029

10.1.39.2	 Measurement Accuracy Requirements for PRS	1029

10.1.39.3	 Measurement Accuracy Requirements for TRS	1034

10.1.40	Void	1038

10.1A	NR measurements for RedCap	1038

10.1A.1	Introduction	1038

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1038

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1038

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1038

10.1A.2.1.2	Relative SS-RSRP Accuracy	1039

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1040

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1040

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1040

10.1A.3.1.2	Relative SS-RSRP Accuracy	1040

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1040

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1040

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1040

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	1041

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1042

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1042

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1042

10.1A.5.1.2	Relative SS-RSRP Accuracy	1042

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1042

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1042

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1042

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1043

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1043

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1043

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1043

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1043

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	1043

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	1044

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1045

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1045

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	1045

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	1045

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1045

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1045

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1045

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1046

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1046

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1046

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1046

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1046

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	1046

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	1047

10.1A.13	Inter-frequency SINR accuracy requirements for FR2	1048

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1048

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	1048

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	1048

10.1A.14	L1-RSRP accuracy requirements for FR1	1048

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1048

10.1A.14.1.1	Absolute Accuracy	1048

10.1A.14.1.2	Relative Accuracy	1049

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1050

10.1A.14.2.1	Absolute Accuracy	1050

10.1A.14.2.2	Relative Accuracy	1051

10.1A.15	 L1-RSRP accuracy requirements for FR2	1052

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1052

10.1A.15.1.1	Absolute Accuracy	1052

10.1A.15.1.2	Relative Accuracy	1052

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1052

10.1A.15.2.1	Absolute Accuracy	1052

10.1A.15.2.2	Relative Accuracy	1052

10.2	E-UTRAN measurements	1053

10.2.1	Introduction	1053

10.2.2	E-UTRAN RSRP measurements	1053

10.2.3	E-UTRAN RSRQ measurements	1053

10.2.4	E-UTRAN RSTD measurements	1053

10.2.5	E-UTRAN RS-SINR measurements	1054

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1054

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1054

10.2A	E-UTRAN measurements for RedCap	1055

10.2A.1	Introduction	1055

10.2A.2	E-UTRAN RSRP measurements	1055

10.2A.3	E-UTRAN RSRQ measurements	1055

10.2A.4	E-UTRAN RS-SINR measurements	1056

10.3	UTRAN FDD Measurements	1056

10.3.1	UTRAN FDD CPICH RSCP	1056

10.3.2	UTRAN FDD CPICH Ec/No	1057

10.4	V2X measurements	1057

10.4.1	Introduction	1057

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1057

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1057

10.4.2.2	PSBCH-RSRP Relative Accuracy	1058

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1059

10.4.3.1	Absolute SL-RSSI Accuracy	1059

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1059

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1059

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1060

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1060

11	Void	1061

12	V2X Requirements	1062

12.1	Introduction	1062

12.2	UE Transmit Timing	1062

12.2.1	Introduction	1062

12.2.2	GNSS as synchronization reference source	1062

12.2.3	NR Cell as synchronization reference source	1063

12.2.4	E-URTAN Cell as synchronization reference source	1063

12.2.5	SyncRef UE as synchronization reference source	1064

12.3	Initiation/Cease of SLSS Transmissions	1064

12.3.1	Introduction	1064

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1064

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1065

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1066

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1066

12.4	Selection / Reselection of V2X Synchronization Reference Source	1067

12.5	L1 SL-RSRP measurements	1068

12.5.1	Introduction	1068

12.5.2	SL-RSRP measurements	1069

12.6	Congestion Control measurements	1069

12.7	Interruption	1069

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1069

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1070

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1071

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1072

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1072

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1073

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1073

12.7.8	Interruptions at NR sidelink discovery configuration	1073

12.8	Reliability of GNSS signal	1074

12.9	Scheduling availability	1074

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1074

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1074

12.10	Selection / Reselection of relay UE	1075

12.10.1	Introduction	1075

12.10.2	Selection / Reselection of relay UE	1075

13	Measurement Performance Requirements for NR gNB	1075

13.1	UL-RTOA	1075

13.1.1	Report mapping	1075

13.1.1A	Additional Path Report Mapping for UL-RTOA	1077

13.2	gNB Rx-Tx time difference	1079

13.2.1	Report mapping	1079

13.2.1A	 Additional Path Report Mapping for gNB Rx-Tx	1081

13.2.2	Measurement Accuracy Requirements	1083

13.2.2.1	Introduction	1083

13.3	UL SRS RSRP measurement	1084

13.3.1	Report mapping	1084

13.3.2	Measurement accuracy requirements	1085

13.3.2.1	Introduction	1085

13.3.2.2	Requirements	1085

13.4	AoA/ZoA	1086

13.4.1	Report mapping	1086

13.5	Timing advance (TADV)	1087

13.5.1	Report mapping	1087

13.6	UL SRS RSRPP measurement	1088

13.6.1	Report mapping	1088

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1090

13.7.1	Report mapping	1090

13.7.2	Measurement Accuracy Requirements	1090

13.7.2.1	Introduction	1090

13.7.2.2	Requirements	1090

Annex A (normative):	Test Cases	1092

A.1	Purpose of annex	1092

A.2	Requirement classification for statistical testing	1092

A.2.1	Types of requirements in TS 38.133	1092

A.2.1.1	Time and delay requirements on UE higher layer actions	1092

A.2.1.2	Measurements of power levels, relative powers and time	1092

A.2.1.3	Implementation requirements	1093

A.2.1.4	Physical layer timing requirements	1093

A.2.1.5	Requirements under CCA	1093

A.3	RRM test configurations	1094

A.3.1	Reference measurement channels	1094

A.3.1.1	PDSCH	1094

A.3.1.1.1	FDD	1094

A.3.1.1.2	TDD	1095

A.3.1.2	CORESET for RMSI scheduling	1098

A.3.1.2.1	FDD	1098

A.3.1.2.2	TDD	1099

A.3.1.3	CORESET for RMC scheduling	1102

A.3.1.3.1	FDD	1102

A.3.1.3.2	TDD	1104

A.3.1.4	TDD UL/DL configuration	1109

A.3.1A	Reference measurement channels under CCA	1111

A.3.1A.1	PDSCH	1111

A.3.1A.1.1	TDD	1111

A.3.1A.2	CORESET for RMSI scheduling	1112

A.3.1A.2.1	TDD	1112

A.3.1A.3	CORESET for RMC scheduling	1113

A.3.1A.3.1	TDD	1113

A.3.1A.4	TDD UL/DL configuration	1114

A.3.1A.5	RMC burst transmission model	1114

A.3.2	OFDMA channel noise generator (OCNG)	1114

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1114

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1115

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1115

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1116

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1116

A.3.2.2	Void	1117

A.3.3	Reference DRX configurations	1117

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1117

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1118

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1118

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1118

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1119

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1119

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1119

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1120

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1120

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1120

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1121

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1121

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1121

A.3.3.14	DRX Configuration 14: DRX cycle = 160 ms and TAT = Infinity	1122

A.3.4	Test Cases with Different Channel Bandwidths	1122

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1122

A.3.4.1.1	Introduction	1122

A.3.4.1.2	Principle of testing	1122

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1122

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1122

A.3.5.1.1	Introduction	1122

A.3.5.1.2	Principle of Testing	1122

A.3.6	Antenna configurations	1123

A.3.6.1	Antenna configurations for FR1	1123

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1123

A.3.6.1.1.1	Introduction	1123

A.3.6.1.1.2	Principle of testing	1123

A.3.6.2	Antenna configurations for FR2	1126

A.3.6A	Antenna configurations with unlicensed bands	1126

A.3.6A.1	Antenna configurations for FR1	1126

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1126

A.3.6A.1.1.1	Introduction	1126

A.3.6A.1.1.2	Principle of testing	1126

A.3.7	EN-DC test setup	1128

A.3.7.1	Introduction	1128

A.3.7.2	E-UTRAN Serving Cell Parameters	1128

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1128

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1129

A.3.7A	NR FR1-FR2 test setup	1130

A.3.7B	EN-DC test setup with unlicensed bands	1131

A.3.7B.1	Introduction	1131

A.3.7B.2	E-UTRAN Serving Cell Parameters	1131

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1131

A.3.7C	LTE-FR1/FR2 test setup	1132

A.3.7D	NE-DC test setup	1132

A.3.7D.1	Introduction	1132

A.3.7D.2	E-UTRAN Serving Cell Parameters	1132

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1132

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1132

A.3.8	PRACH configurations	1132

A.3.8.1	Introduction	1132

A.3.8.2	PRACH configurations in FR1	1133

A.3.8.2.1	FR1 PRACH configuration 1	1133

A.3.8.2.2	FR1 PRACH configuration 2	1133

A.3.8.2.3	FR1 PRACH configuration 3	1134

A.3.8.2.4	FR1 PRACH configuration 4	1135

A.3.8.3	PRACH configurations in FR2	1136

A.3.8.3.1	FR2 PRACH configuration 1	1136

A.3.8.3.2	FR2 PRACH configuration 2	1137

A.3.8.3.3	FR2 PRACH configuration 3	1138

A.3.8.3.4	FR2 PRACH configuration 4	1139

A.3.8A	PRACH configurations under CCA	1140

A.3.8A.1	Introduction	1140

A.3.8A.2	PRACH configurations in FR1	1140

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1140

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1141

A.3.9	BWP configurations	1142

A.3.9.1	Introduction	1142

A.3.9.2	Downlink BWP configurations	1143

A.3.9.2.1	Initial BWP	1143

A.3.9.2.2	Dedicated BWP	1143

A.3.9.3	Uplink BWP configurations	1144

A.3.9.3.1	Initial BWP	1144

A.3.9.3.2	Dedicated BWP	1144

A.3.9A	BWP configurations for RedCap	1144

A.3.9A.1	Introduction	1144

A.3.9A.2	Downlink BWP configurations	1145

A.3.9A.2.1	Dedicated BWP	1145

A.3.9A.3	Uplink BWP configurations	1145

A.3.9A.3.1	Dedicated BWP	1145

A.3.10	SSB Configurations	1146

A.3.10.1	SSB Configurations for FR1	1146

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1146

A.3.10.1.2	SSB pattern 2 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1146

A.3.10.1.3	SSB pattern 3 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1147

A.3.10.1.4	SSB pattern 4 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1147

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1148

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1148

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1149

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1149

A.3.10.2	SSB Configurations for FR2	1150

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1150

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1150

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1151

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1151

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1152

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1152

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1153

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1153

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1154

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1154

A.3.10.2.11	SSB pattern 11 in FR2: SSB allocation for SSB SCS=480 kHz in 400 MHz	1155

A.3.10.2.12	SSB pattern 12 in FR2: SSB allocation for SSB SCS=960 kHz in 400 MHz	1155

A.3.10.2.13	SSB pattern 13 in FR2: SSB allocation for SSB SCS=480 kHz in 400 MHz	1156

A.3.10.2.14	SSB pattern 14 in FR2: SSB allocation for SSB SCS=960 kHz in 400 MHz	1156

A.3.10.2.15	SSB pattern 15 in FR2: SSB allocation for SSB SCS=480 kHz in 400 MHz	1157

A.3.10.2.16	SSB pattern 16 in FR2: SSB allocation for SSB SCS=960 kHz in 400 MHz	1157

A.3.10.2.17	SSB pattern 17 in FR2: SSB allocation for SSB SCS=480 kHz in 400 MHz	1158

A.3.10.2.18	SSB pattern 18 in FR2: SSB allocation for SSB SCS=960 kHz in 400 MHz	1158

A.3.10A	SSB Configurations under CCA	1159

A.3.10A.1	SSB Configurations under CCA for FR1	1159

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1159

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1159

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1160

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1160

A.3.10B	SSB Configurations for RedCap	1161

A.3.10B.1	SSB Configurations for FR1	1161

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1161

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1161

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1162

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1162

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1163

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1163

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1164

A.3.10B.2	SSB Configurations for FR2	1164

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1164

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1165

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1165

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1166

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1166

A.3.11	SMTC Configurations	1167

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1167

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1167

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1167

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1167

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1168

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1168

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1168

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1168

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	1168

A.3.11A	SMTC Configurations for RedCap	1169

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1169

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1169

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1169

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1169

A.3.11.12	SMTC pattern 12: SMTC period = 20 ms with SMTC duration = 5 ms	1169

A.3.12	Test Cases with Different CC Configurations	1170

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1170

A.3.12.1.1	Introduction	1170

A.3.12.1.2	Principle of testing	1170

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1170

A.3.12.2.1	Introduction	1170

A.3.12.2.2	Principle of testing	1170

A.3.13	Test Cases in SA and EN-DC Operations	1170

A.3.13.1	Introduction	1170

A.3.13.2	Principle of Testing	1171

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1171

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1171

A.3.13B.1.1	Introduction	1171

A.3.13B.1.2	Principle of Testing	1172

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1172

A.3.13B.2.1	Introduction	1172

A.3.13B.2.2	Principle of Testing	1172

A.3.14	CSI-RS configurations	1172

A.3.14.1	FDD	1172

A.3.14.2	TDD	1174

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1180

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1180

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1180

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1180

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1181

A.3.15.3	Setup 3: 2 AoAs	1181

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1181

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1181

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1181

A.3.16	TCI State Configuration	1182

A.3.16.1	Introduction	1182

A.3.16.2	TCI states	1182

A.3.16A	Unified TCI State Configuration	1182

A.3.16A.1	Introduction	1182

A.3.16A.2	DLorJoint TCI states	1183

A.3.16A.3	UL TCI states	1183

A.3.17	Configurations of CSI-RS for tracking	1184

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1184

A.3.17.1.1	FDD	1184

A.3.17.1.2	TDD	1186

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1188

A.3.17.2.1	TDD	1188

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1189

A.3.18.1	Introduction	1189

A.3.18.2	PRACH Power Measurement	1189

A.3.19	Test applicability for DAPS handover	1189

A.3.19.1	Introduction	1189

A.3.19.2	Principle of testing	1189

A.3.20	MsgA configurations	1190

A.3.20.1	Introduction	1190

A.3.20.2	MsgA configurations in FR1	1190

A.3.20.2.1	FR1 MsgA configuration 1	1190

A.3.20.2.2	FR1 MsgA configuration 2	1191

A.3.20.3	MsgA configurations in FR2	1193

A.3.20.3.1	FR2 MsgA configuration 1	1193

A.3.20.3.2	FR2 MsgA configuration 2	1194

A.3.20A	MsgA configurations under CCA	1196

A.3.20A.1	Introduction	1196

A.3.20A.2	MsgA configurations in FR1	1196

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1196

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1197

A.3.21	V2X sidelink communication	1200

A.3.21.1	Introduction	1200

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1200

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1204

A.3.21.4	Reference SL-DRX configurations	1205

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1205

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1205

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1205

A.3.22	CSI-IM configurations	1205

A.3.22.1	FDD	1205

A.3.22.2	TDD	1206

A.3.23	Spatial Relation Configuration	1207

A.3.23.1	Introduction	1207

A.3.23.2	Spatial Relation	1208

A.3.24	SRS configuration	1209

A.3.25	Channel bandwidth (CBW) configurations	1211

A.3.25.1	DL UE specific CBW	1211

A.3.25.2	UL UE specific CBW	1212

A.3.26	CCA model	1212

A.3.26.1	Introduction	1212

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1212

A.3.26.2.1	DL CCA model	1212

A.3.26.2.2	UL CCA model	1214

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1214

A.3.26.3.1	DL CCA model	1214

A.3.26.3.2	UL CCA model	1215

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1215

A.3.27.1	Introduction	1215

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1215

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1216

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1216

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1216

A.3.28	Discovery Burst Transmission Window configuration under CCA	1216

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1216

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1216

A.3.29.1	Introduction	1216

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1216

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1217

A.3.30	CSI-RS configurations for RRM	1218

A.3.30.1	FDD	1218

A.3.30.2	TDD	1219

A.3.31	PRS Configurations	1221

A.3.31.1.	PRS Configurations for FR1	1221

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1221

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1222

A.3.31.2.	PRS Configurations for FR2	1222

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1222

A.3.32	NR sidelink discovery	1222

A.3.32.1	Introduction	1222

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1222

A.3.32.3	Principle of Testing	1223

A.3.33	PRS Processing Window (PPW) configurations	1223

A.3.34	Testing principles for test cases related to PRS measurements	1223

A.3.34.1	Introduction	1223

A.3.34.2	Test cases in RRC_INACTIVE state	1224

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1224

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1224

A.3.35	Testing principle for RedCap UE	1225

A.3.35.1	Introduction	1225

A.3.35.2	Principle of testing for FR1	1225

A.3.35.3	Principle of testing for FR2	1225

A.3.36	Testing related to Satellite access	1225

A.3.36.1	Introduction	1225

A.3.36.2	Principle of testing GSO and NGSO scenarios	1225

A.3.36.2	Principle of testing different RRM requirements	1226

A.3.36.3	Principle of testing different ephemeris formats	1226

A.3.36.4	General setup for SIB19	1228

A.3.36.5	Satellite specific parameters configuration	1229

A.3.36.5.1	Satellite specific configuration for serving cell	1229

A.3.36.5.2	Satellite specific configuration for neighbour cell	1230

A.4	EN-DC tests with all NR cells in FR1	1231

A.4.1	Void	1231

A.4.2	Void	1231

A.4.3	RRC_CONNECTED state mobility	1231

A.4.3.1	Void	1231

A.4.3.2	RRC Connection Mobility Control	1231

A.4.3.2.1	Void	1231

A.4.3.2.2	Random Access	1231

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1231

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1235

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1239

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1243

A.4.3.2.3	Void	1247

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1247

A.4.3.3.1	Test Purpose and Environment	1247

A.4.3.3.2	Test Requirements	1253

A.4.4	Timing	1254

A.4.4.1	UE transmit timing	1254

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1254

A.4.4.1.1.1	Test Purpose and environment	1254

A.4.4.1.1.2	Test requirements	1258

A.4.4.2	UE timer accuracy	1258

A.4.4.3	Timing advance	1258

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1258

A.4.4.3.1.1	Test Purpose and Environment	1258

A.4.4.3.1.2	Test Parameters	1258

A.4.4.3.1.3	Test Requirements	1262

A.4.5	Signaling characteristics	1262

A.4.5.1	Radio link Monitoring	1262

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1263

A.4.5.1.1.1	Test Purpose and Environment	1263

A.4.5.1.1.2	Test Requirements	1267

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1267

A.4.5.1.2.1	Test Purpose and Environment	1267

A.4.5.1.2.2	Test Requirements	1273

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1273

A.4.5.1.3.1	Test Purpose and Environment	1273

A.4.5.1.3.2	Test Requirements	1278

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1278

A.4.5.1.4.1	Test Purpose and Environment	1278

A.4.5.1.4.2	Test Requirements	1284

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1284

A.4.5.1.5.1	Test Purpose and Environment	1284

A.4.5.1.5.2	Test Requirements	1289

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1289

A.4.5.1.6.1	Test Purpose and Environment	1289

A.4.5.1.6.2	Test Requirements	1295

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1295

A.4.5.1.7.1	Test Purpose and Environment	1295

A.4.5.1.7.2	Test Requirements	1300

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1301

A.4.5.1.8.1	Test Purpose and Environment	1301

A.4.5.1.8.2	Test Requirements	1306

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1306

A.4.5.1.9.1	Test Purpose and Environment	1306

A.4.5.2	Interruption	1312

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1312

A.4.5.2.1.1	Test Purpose and Environment	1312

A.4.5.2.1.2	Test Requirements	1316

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1316

A.4.5.2.2.1	Test Purpose and Environment	1316

A.4.5.2.2.2	Test Requirements	1320

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1320

A.4.5.2.3.1	Test Purpose and Environment	1320

A.4.5.2.3.2	Test Requirements	1327

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1328

A.4.5.2.4.1	Test Purpose and Environment	1328

A.4.5.2.4.2	Test Requirements	1332

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1333

A.4.5.2.5.1	Test Purpose and Environment	1333

A.4.5.2.5.2	Test Requirements	1336

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1336

A.4.5.2.6.1	Test Purpose and Environment	1336

A.4.5.2.6.2	Test Requirements	1340

A.4.5.2.7	Void	1340

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1340

A.4.5.2.8.1	Test Purpose and Environment	1340

A.4.5.2.8.2	Test Requirements	1343

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1344

A.4.5.2.9.1	Test Purpose and Environment	1344

A.4.5.2.9.2	Test Requirements	1348

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1349

A.4.5.2.10.1	Test Purpose and Environment	1349

A.4.5.2.10.2	Test Requirements	1352

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1352

A.4.5.2.11.1	Test Purpose and Environment	1352

A.4.5.2.11.2	Test Requirements	1357

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1358

A.4.5.2.12.1	Test Purpose and Environment	1358

A.4.5.3	SCell Activation and Deactivation Delay	1366

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1366

A.4.5.3.1.1	Test Purpose and Environment	1366

A.4.5.3.1.2	Test Requirements	1372

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1373

A.4.5.3.2.1	Test Purpose and Environment	1373

A.4.5.3.2.2	Test Requirements	1373

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1373

A.4.5.3.3.1	Test Purpose and Environment	1373

A.4.5.3.3.2	Test Requirements	1374

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1375

A.4.5.3.4.1	Test Purpose and Environment	1375

A.4.5.3.4.2	Test Requirements	1377

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1378

A.4.5.3.5.1	Test Purpose and Environment	1378

A.4.5.3.5.2	Test Requirements	1385

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1386

A.4.5.3.6.1	Test Purpose and Environment	1386

A.4.5.3.6.2	Test Requirements	1390

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1391

A.4.5.3.7.1	Test Purpose and Environment	1391

A.4.5.3.7.2	Test Requirements	1391

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1392

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1392

A.4.5.4.1.1	Test Purpose and Environment	1392

A.4.5.4.1.2	Test Requirements	1400

A.4.5.5	Beam Failure Detection and Link recovery procedures	1400

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1400

A.4.5.5.1.1	Test Purpose and Environment	1400

A.4.5.5.1.2	Test Requirements	1406

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1407

A.4.5.5.2.1	Test Purpose and Environment	1407

A.4.5.5.2.2	Test Requirements	1412

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1413

A.4.5.5.3.1	Test Purpose and Environment	1413

A.4.5.5.3.2	Test Requirements	1418

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1419

A.4.5.5.4.1	Test Purpose and Environment	1419

A.4.5.5.4.2	Test Requirements	1424

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1425

A.4.5.5.5.1	Test Purpose and Environment	1425

A.4.5.5.5.2	Test Requirements	1430

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1431

A.4.5.5.6.1	Test Purpose and Environment	1431

A.4.5.5.6.2	Test Requirements	1437

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1438

A.4.5.5.7.1	Test Purpose and Environment	1438

A.4.5.5.7.2	Test Requirements	1443

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1444

A.4.5.5.8.1	Test Purpose and Environment	1444

A.4.5.5.8.2	Test Requirements	1450

A.4.5.6	Active BWP switch	1451

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1451

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1451

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1456

A.4.5.6.2	RRC-based Active BWP Switch	1462

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1466

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1466

A.4.5.6.3.1.1	Test Purpose and Environment	1466

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1472

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1472

A.4.5.6.4.1.1	Test Purpose and Environment	1472

A.4.5.6.4.1.2	Test Requirements	1479

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1479

A.4.5.6.4.2.1	Test Purpose and Environment	1479

A.4.5.6.4.2.2	Test Requirements	1489

A.4.5.6.5	SCell dormancy switch	1490

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1490

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1495

A.4.5.6.5.2.1	Test Purpose and Environment	1495

A.4.5.6.5.2.2	Test Requirements	1501

A.4.5.7	PSCell addition and release delay	1501

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1501

A.4.5.7.1.1	Test purpose and environment	1501

A.4.5.7.1.2	Test Requirements	1506

A.4.5.8	DL Interruptions at switching between two uplink carriers	1507

A.4.5.8.1	Test Purpose and Environment	1507

A.4.5.8.2	Test Requirements	1510

A.4.5.9	UE specific CBW change	1511

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1511

A.4.5.9.1.1	Test Purpose and Environment	1511

A.4.5.9.1.2	Test Requirements	1515

A.4.5.10	PSCell activation and deactivation delay	1515

A.4.5.10.1	PSCell activation and deactivation delay	1515

A.4.5.10.1.1	Test purpose and environment	1515

A.4.5.10.1.2	Test Requirements	1519

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1520

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1520

A.4.5.11.1.1	Test purpose and environment	1520

A.4.5.11.1.2	Test Parameters	1520

A.4.5.11.1.3	Test Requirements	1524

A.4.6	Measurement procedure	1524

A.4.6.1	Intra-frequency Measurements	1524

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1524

A.4.6.1.1.1	Test purpose and Environment	1524

A.4.6.1.1.2	Test parameters	1524

A.4.6.1.1.3	Test Requirements	1528

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1528

A.4.6.1.2.1	Test purpose and Environment	1528

A.4.6.1.2.2	Test parameters	1528

A.4.6.1.2.2	Test Requirements	1532

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1532

A.4.6.1.3.1	Test purpose and Environment	1532

A.4.6.1.3.2	Test parameters	1532

A.4.6.1.3.3	Test Requirements	1536

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1536

A.4.6.1.4.1	Test purpose and Environment	1536

A.4.6.1.4.2	Test parameters	1536

A.4.6.1.4.3	Test Requirements	1540

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1540

A.4.6.1.5.1	Test purpose and Environment	1540

A.4.6.1.5.2	Test parameters	1540

A.4.6.1.5.3	Test Requirements	1542

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1543

A.4.6.1.6.1	Test purpose and Environment	1543

A.4.6.1.6.2	Test parameters	1543

A.4.6.1.6.3	Test Requirements	1545

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1546

A.4.6.1.7.1	Test purpose and Environment	1546

A.4.6.1.7.2	Test parameters	1546

A.4.6.1.7.3	Test Requirements	1550

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1550

A.4.6.1.8.1	Test Purpose and Environment	1550

A.4.6.1.8.2	Test Requirements	1555

A.4.6.2	Inter-frequency Measurements	1556

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1556

A.4.6.2.1.1	Test Purpose and Environment	1556

A.4.6.2.1.2	Test Requirements	1560

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1560

A.4.6.2.2.1	Test Purpose and Environment	1560

A.4.6.2.2.2	Test Requirements	1564

A.4.6.2.3	Void	1564

A.4.6.2.4	Void	1564

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1564

A.4.6.2.5.1	Test Purpose and Environment	1564

A.4.6.2.5.2	Test Requirements	1569

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1569

A.4.6.2.6.1	Test Purpose and Environment	1569

A.4.6.2.6.2	Test Requirements	1574

A.4.6.2.7	Void	1574

A.4.6.2.8	Void	1574

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1575

A.4.6.2.9.1	Test Purpose and Environment	1575

A.4.6.2.9.2	Test Requirements	1578

A.4.6.3	Void	1578

A.4.6.4	L1-RSRP measurement for beam reporting	1578

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1578

A.4.6.4.1.1	Test Purpose and Environment	1578

A.4.6.4.1.2	Test parameters	1579

A.4.6.4.1.3	Test Requirements	1582

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1582

A.4.6.4.2.1	Test Purpose and Environment	1582

A.4.6.4.2.2	Test parameters	1583

A.4.6.4.2.3	Test Requirements	1586

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1586

A.4.6.4.3.1	Test Purpose and Environment	1586

A.4.6.4.3.2	Test parameters	1587

A.4.6.4.3.3	Test Requirements	1590

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1590

A.4.6.4.4.1	Test Purpose and Environment	1590

A.4.6.4.4.2	Test parameters	1591

A.4.6.4.4.3	Test Requirements	1593

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1593

A.4.6.4.5.1	Test Purpose and Environment	1593

A.4.6.4.5.2	Test parameters	1594

A.4.6.4.5.3	Test Requirements	1597

A.4.6.5	CLI measurements	1597

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1597

A.4.6.5.1.1	Test Purpose and Environment	1597

A.4.6.5.1.2	Test Parameters	1598

A.4.6.5.1.3	Test Requirements	1601

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1601

A.4.6.5.2.1	Test Purpose and Environment	1601

A.4.6.5.2.2	Test Parameters	1602

A.4.6.5.2.3	Test Requirements	1603

A.4.6.6.1.2	Test Requirements	1608

A.4.6.7	L1-SINR measurement for beam reporting	1609

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1611

A.4.6.7.2.1	Test Purpose and Environment	1611

A.4.6.7.2.2	Test parameters	1612

A.4.6.7.2.3	Test Requirements	1614

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1614

A.4.6.7.3.1	Test Purpose and Environment	1614

A.4.6.7.3.2	Test parameters	1614

A.4.6.7.3.3	Test Requirements	1616

A.4.6.8	CSI-RS based intra-frequency Measurement	1617

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1617

A.4.6.8.1.1	Test purpose and Environment	1617

A.4.6.8.1.2	Test Requirements	1621

A.4.6.9	CSI-RS based inter-frequency Measurement	1621

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1621

A.4.6.9.1.1	Test Purpose and Environment	1621

A.4.6.9.1.2	Test Requirements	1625

A.4.7	Measurement Performance requirements	1626

A.4.7.1	SS-RSRP	1626

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1626

A.4.7.1.1.1	Test Purpose and Environment	1626

A.4.7.1.1.2	Test parameters	1626

A.4.7.1.1.3	Test Requirements	1630

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1631

A.4.7.1.2.1	Test Purpose and Environment	1631

A.4.7.1.2.2	Test parameters	1631

A.4.7.1.2.3	Test Requirements	1635

A.4.7.1.3	Void	1636

A.4.7.2	SS-RSRQ	1636

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1636

A.4.7.2.1.1	Test Purpose and Environment	1636

A.4.7.2.1.2	Test Parameters	1636

A.4.7.2.1.3	Test Requirements	1640

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1640

A.4.7.2.2.1	Test Purpose and Environment	1640

A.4.7.2.2.2	Test Parameters	1640

A.4.7.2.2.3	Test Requirements	1646

A.4.7.3	SS-SINR	1646

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1646

A.4.7.3.1.1	Test Purpose and Environment	1646

A.4.7.3.1.2	Test Parameters	1646

A.4.7.3.1.3	Test Requirements	1650

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1650

A.4.7.3.2.1	Test Purpose and Environment	1650

A.4.7.3.2.2	Test Parameters	1650

A.4.7.3.2.3	Test Requirements	1655

A.4.7.4	L1-RSRP measurement for beam reporting	1655

A.4.7.4.1	SSB based L1-RSRP measurement	1655

A.4.7.4.1.1	Test Purpose and Environment	1655

A.4.7.4.1.2	Test parameters	1655

A.4.7.4.1.3	Test Requirements	1659

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1659

A.4.7.4.2.1	Test Purpose and Environment	1659

A.4.7.4.2.2	Test parameters	1659

A.4.7.4.2.3	Test Requirements	1663

A.4.7.5	SFTD accuracy	1663

A.4.7.5.1	SFTD accuracy	1663

A.4.7.5.1.1	Test Purpose and Environment	1663

A.4.7.5.1.2	Test Parameters	1663

A.4.7.5.1.3	Test Requirements	1667

A.4.7.5.2	Void	1668

A.4.7.5.3	Void	1668

A.4.7.6	CLI measurements	1668

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1668

A.4.7.6.1.1	Test Purpose and Environment	1668

A.4.7.6.1.2	Test parameters	1668

A.4.7.6.1.3	Test Requirements	1673

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1673

A.4.7.6.2.1	Test Purpose and Environment	1673

A.4.7.6.2.2	Test parameters	1674

A.4.7.6.2.3	Test Requirements	1677

A.4.7.7	L1-SINR measurement for beam reporting	1677

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1681

A.4.7.7.2.1	Test Purpose and Environment	1681

A.4.7.7.2.2	Test parameters	1682

A.4.7.7.2.3	Test Requirements	1685

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1685

A.4.7.7.3.1	Test Purpose and Environment	1685

A.4.7.7.3.2	Test parameters	1686

A.4.7.7.3.3	Test Requirements	1690

A.4.7.8	CSI-RSRP	1690

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1690

A.4.7.8.1.1	Test Purpose and Environment	1690

A.4.7.8.1.2	Test parameters	1690

A.4.7.8.1.3	Test Requirements	1695

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1695

A.4.7.8.2.1	Test Purpose and Environment	1695

A.4.7.8.2.2	Test parameters	1695

A.4.7.8.2.3	Test Requirements	1701

A.4.7.9	CSI-RSRQ	1701

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1701

A.4.7.9.1.1	Test Purpose and Environment	1701

A.4.7.9.1.2	Test Parameters	1701

A.4.7.9.1.3	Test Requirements	1706

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1706

A.4.7.9.2.1	Test Purpose and Environment	1706

A.4.7.9.2.2	Test Parameters	1706

A.4.7.9.2.3	Test Requirements	1712

A.4.7.10	CSI-SINR	1713

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1713

A.4.7.10.1.1	Test Purpose and Environment	1713

A.4.7.10.1.2	Test Parameters	1713

A.4.7.10.1.3	Test Requirements	1718

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1718

A.4.7.10.2.1	Test Purpose and Environment	1718

A.4.7.10.2.2	Test Parameters	1718

A.4.7.10.2.3	Test Requirements	1723

A.4.8	Void	1723

A.4A	NE-DC test with all NR cells in FR1	1723

A.4A.1	Signaling characteristics	1723

A.4A.1.1	E-UTRAN PSCell addition	1723

A.4A.1.1.1	Test purpose and environment	1723

A.4A.1.1.2	Test Requirements	1728

A.4A.1.2	Active BWP switch	1729

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1729

A.4A.1.2.1.1	Test Purpose and Environment	1729

A.4A.1.2.1.2	Test Requirements	1733

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1734

A.4A.1.3.1	Test purpose and environment	1734

A.4A.1.3.2	Test Requirements	1738

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1739

A.4A.1.4.1	Test Purpose and Environment	1739

A.4A.1.4.2	Test Parameters	1739

A.4A.1.4.3	Test Requirements	1744

A.4A.1.4.3.1	Test Requirements for NR HO	1744

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1744

A.4A.2	Measurement performance	1745

A.4A.2.1	SFTD accuracy	1745

A.4A.2.1.1	SFTD accuracy	1745

A.4A.2.1.1.1	Test Purpose	1745

A.4A.2.1.1.2	Test Environment	1745

A.4A.2.1.1.3	Test Requirements	1749

A.5	EN-DC tests with one or more NR cells in FR2	1750

A.5.1	Void	1750

A.5.2	Void	1750

A.5.3	RRC_CONNECTED state mobility	1750

A.5.3.1	Void	1750

A.5.3.2	RRC Connection Mobility Control	1750

A.5.3.2.1	Void	1750

A.5.3.2.2	Random Access	1750

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1750

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1753

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1759

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1762

A.5.3.2.3	Void	1766

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1766

A.5.3.3.1	Test purpose and environment	1766

A.5.3.3.2	Test Requirements	1771

A.5.5.3.3	void	1772

A.5.5.3.4	void	1772

A.5.5.3.5	void	1772

A.5.5.3.6	void	1772

A.5.4	Timing	1772

A.5.4.1	UE transmit timing	1772

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1772

A.5.4.1.1.1	Test Purpose and environment	1772

A.5.4.1.1.2	Test requirements	1775

A.5.4.2	UE timer accuracy	1776

A.5.4.3	Timing advance	1776

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1776

A.5.4.3.1.1 Test Purpose and Environment	1776

A.5.4.3.1.2 Test Parameters	1776

A.5.4.3.1.3	Test Requirements	1780

A.5.5	Signaling characteristics	1780

A.5.5.1	Radio link Monitoring	1780

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1780

A.5.5.1.1.1	Test Purpose and Environment	1780

A.5.5.1.1.2	Test Requirements	1784

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1785

A.5.5.1.2.1	Test Purpose and Environment	1785

A.5.5.1.2.2	Test Requirements	1788

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1789

A.5.5.1.3.1	Test Purpose and Environment	1789

A.5.5.1.3.2	Test Requirements	1793

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1793

A.5.5.1.4.1	Test Purpose and Environment	1793

A.5.5.1.4.2	Test Requirements	1797

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1797

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1801

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1805

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1810

A.5.5.1.8.2	Test Requirements	1814

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1815

A.5.5.1.9.1	Test Purpose and Environment	1815

A.5.5.1.9.2	Test Requirements	1817

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1817

A.5.5.1.10.1	Test Purpose and Environment	1817

A.5.5.1.10.2	Test Requirements	1820

A.5.5.2	Interruption	1820

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1820

A.5.5.2.1.1	Test Purpose and Environment	1820

A.5.5.2.1.2	Test Requirements	1823

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1823

A.5.5.2.2.1	Test Purpose and Environment	1823

A.5.5.2.2.2	Test Requirements	1826

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1826

A.5.5.2.3.1	Test Purpose and Environment	1826

A.5.5.2.3.2	Test Requirements	1830

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1830

A.5.5.2.4.1	Test Purpose and Environment	1830

A.5.5.2.4.2	Test Requirements	1833

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1834

A.5.5.2.5.1	Test Purpose and Environment	1834

A.5.5.2.5.2	Test Requirements	1837

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1837

A.5.5.2.6.1	Test Purpose and Environment	1837

A.5.5.2.6.2	Test Requirements	1840

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1840

A.5.5.2.7.1	Test Purpose and Environment	1840

A.5.5.2.7.2	Test Requirements	1844

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1844

A.5.5.2.8.1 Test Purpose and Environment	1844

A.5.5.2.8.3	Test Requirements	1846

A.5.5.3	SCell Activation and Deactivation Delay	1846

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1846

A.5.5.3.1.1	Test Purpose and Environment	1846

A.5.5.3.1.2	Test Requirements	1848

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1848

A.5.5.3.2.1	Test Purpose and Environment	1848

A.5.5.3.2.2	Test Requirements	1852

A.5.5.3.3	Void	1852

A.5.5.3.4	Void	1852

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1852

A.5.5.3.5.1	Test Purpose and Environment	1852

A.5.5.3.5.2	Test Requirements	1855

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1856

A.5.5.3.6.1	Test Purpose and Environment	1856

A.5.5.3.6.2	Test Requirements	1860

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1861

A.5.5.3.7.1	Test Purpose and Environment	1861

A.5.5.3.7.2	Test Requirements	1864

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1864

A.5.5.3.8.1	Test Purpose and Environment	1864

A.5.5.3.8.2	Test Requirements	1868

A.5.5.3.9	PUCCH SCell Activation and deactivation of known SCell in FR2	1869

A.5.5.3.9.1	Test Purpose and Environment	1869

A.5.5.3.9.2	Test Requirements	1872

A.5.5.3.10	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1873

A.5.5.3.10.1	Test Purpose and Environment	1873

A.5.5.3.10.2	Test Requirements	1876

A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1877

A.5.5.3.11.1	Test Purpose and Environment	1877

A.5.5.3.11.2	Test Requirements	1880

A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1881

A.5.5.3.12.1	Test Purpose and Environment	1881

A.5.5.3.12.2	Test Requirements	1884

A.5.5.4	Void	1885

A.5.5.5	Beam Failure Detection and Link recovery procedures	1885

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1885

A.5.5.5.1.1	Test Purpose and Environment	1885

A.5.5.5.1.2	Test Requirements	1889

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1890

A.5.5.5.2.1	Test Purpose and Environment	1890

A.5.5.5.2.2	Test Requirements	1894

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1895

A.5.5.5.3.1	Test Purpose and Environment	1895

A.5.5.5.3.2	Test Requirements	1899

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1900

A.5.5.5.4.1	Test Purpose and Environment	1900

A.5.5.5.4.2	Test Requirements	1904

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1905

A.5.5.5.5.1	Test Purpose and Environment	1905

A.5.5.5.5.2	Test Requirements	1909

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1910

A.5.5.5.6.1	Test Purpose and Environment	1910

A.5.5.5.6.2	Test Requirements	1914

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1915

A.5.5.5.7.1	Test Purpose and Environment	1915

A.5.5.5.7.2	Test Requirements	1919

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1920

A.5.5.5.8.1	Test Purpose and Environment	1920

A.5.5.5.8.2	Test Requirements	1925

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	1925

A.5.5.5.9.1	Test Purpose and Environment	1925

A.5.5.5.9.2	Test Requirements	1929

A.5.5.6	Active BWP switch	1929

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1929

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1929

A.5.5.6.1.1.1	Test Purpose and Environment	1929

A.5.5.6.1.1.2	Test Requirements	1933

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1933

A.5.5.6.2	RRC-based Active BWP Switch	1938

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1938

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1942

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1942

A.5.5.6.4	SCell dormancy switch	1946

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1946

A.5.5.6.4.1.1	Test Purpose and Environment	1946

A.5.5.6.4.1.2	Test Requirements	1950

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1950

A.5.5.6.4.2.1	Test Purpose and Environment	1950

A.5.5.6.4.2.2	Test Requirements	1957

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1957

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1957

A.5.5.7	PSCell addition and release delay	1960

A.5.5.7.1	Addition and Release Delay of NR PSCell	1960

A.5.5.7.1.1	Test purpose and environment	1960

A.5.5.7.1.2	Test Requirements	1963

A.5.5.8	Active TCI state switch delay	1964

A.5.5.8.1	MAC-CE based active TCI state switch	1964

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1964

A.5.5.8.1.1.1	Test Purpose and Environment	1964

A.5.5.8.1.1.2	Test Requirements	1968

A.5.5.8.2	RRC based active TCI state switch	1968

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1968

A.5.5.8.2.1.1	Test Purpose and Environment	1968

A.5.5.8.2.1.2	Test Requirements	1972

A.5.5.9	Uplink spatial relation switch delay	1972

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1972

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1972

A.5.5.9.1.1.1	Test Purpose and Environment	1972

A.5.5.9.1.1.2	Test Requirements	1975

A.5.5.9.2	RRC based spatial relation switch	1975

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1975

A.5.5.9.2.1.1	Test Purpose and Environment	1975

A.5.5.9.2.1.2	Test Requirements	1978

A.5.5.10	UE specific CBW change	1978

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1978

A.5.5.10.1.1	Test Purpose and Environment	1978

A.5.5.10.1.2	Test Requirements	1981

A.5.5.11	Unified TCI state switch delay	1982

A.5.5.11.1	MAC-CE based active joint TCI state switch	1982

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	1982

A.5.5.11.1.1.1	Test Purpose and Environment	1982

A.5.5.11.1.1.2	Test parameters	1982

A.5.5.11.1.1.3	Test Requirements	1985

A.5.5.11.2	MAC-CE based active uplink TCI state switch	1985

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	1985

A.5.5.11.2.1.1	Test Purpose and Environment	1985

A.5.5.11.2.1.2	Test parameters	1986

A.5.5.11.2.1.3	Test Requirements	1988

A.5.5.11.3	MAC-CE based active downlink TCI state switch	1988

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	1988

A.5.5.11.3.1.1	Test Purpose and Environment	1988

A.5.5.11.3.1.2	Test Parameters	1989

A.5.5.11.3.1.3	Test Requirements	1993

A.5.5.12	PSCell activation and deactivation delay	1993

A.5.5.12.1	PSCell activation and deactivation delay	1993

A.5.5.12.1.1	Test purpose and environment	1993

A.5.5.12.1.2	Test Requirements	1996

A.5.5.13	Conditional PSCell addition and release delay	1997

A.5.5.13.1	Addition and Release Delay of NR PSCell	1997

A.5.5.13.1.1	Test purpose and environment	1997

A.5.5.13.1.2	Test Requirements	2000

A.5.5.13.2	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell	2001

A.5.5.13.2.1	Test Purpose and Environment	2001

A.5.5.13.2.2	Test Requirements	2004

A.5.6	Measurement procedure	2004

A.5.6.1	Intra-frequency Measurements	2004

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	2004

A.5.6.1.1.1	Test purpose and Environment	2004

A.5.6.1.1.2	Test Requirements	2008

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	2008

A.5.6.1.2.1	Test purpose and Environment	2008

A.5.6.1.2.2	Test Requirements	2010

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	2011

A.5.6.1.3.1	Test purpose and Environment	2011

A.5.6.1.3.2	Test Requirements	2015

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	2015

A.5.6.1.4.1	Test purpose and Environment	2015

A.5.6.1.4.2	Test Requirements	2018

A.5.6.2	Inter-frequency Measurements	2019

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2019

A.5.6.2.1.1	Test Purpose and Environment	2019

A.5.6.2.1.2	Test Requirements	2022

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2022

A.5.6.2.2.1	Test Purpose and Environment	2022

A.5.6.2.2.2	Test Requirements	2026

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2026

A.5.6.2.3.1	Test Purpose and Environment	2026

A.5.6.2.3.2	Test Requirements	2030

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2030

A.5.6.2.4.1	Test Purpose and Environment	2030

A.5.6.2.4.2	Test Requirements	2034

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2034

A.5.6.2.5.1	Test Purpose and Environment	2034

A.5.6.2.5.2	Test Requirements	2039

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2039

A.5.6.2.6.1	Test Purpose and Environment	2039

A.5.6.2.6.2	Test Requirements	2043

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2043

A.5.6.2.7.1	Test Purpose and Environment	2043

A.5.6.2.7.2	Test Requirements	2048

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2048

A.5.6.2.8.1	Test Purpose and Environment	2048

A.5.6.2.8.2	Test Requirements	2053

A.5.6.3	L1-RSRP measurement for beam reporting	2054

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2054

A.5.6.3.1.1	Test Purpose and Environment	2054

A.5.6.3.1.2	Test parameters	2054

A.5.6.3.1.3	Test Requirements	2056

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2056

A.5.6.3.2.1	Test Purpose and Environment	2056

A.5.6.3.2.2	Test parameters	2057

A.5.6.3.2.3	Test Requirements	2059

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2059

A.5.6.3.3.1	Test Purpose and Environment	2059

A.5.6.3.3.2	Test parameters	2060

A.5.6.3.3.3	Test Requirements	2062

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2062

A.5.6.3.4.1	Test Purpose and Environment	2062

A.5.6.3.4.2	Test parameters	2063

A.5.6.3.4.3	Test Requirements	2065

A.5.6.4	CLI measurements	2066

A.5.6.4.1	SRS-RSRP measurement with DRX	2066

A.5.6.4.1.1	Test Purpose and Environment	2066

A.5.6.4.1.2	Test Parameters	2066

A.5.6.4.1.3	Test Requirements	2068

A.5.6.4.2	CLI-RSSI measurement with DRX	2068

A.5.6.4.2.1	Test Purpose and Environment	2068

A.5.6.4.2.2	Test Parameters	2069

A.5.6.4.2.3	Test Requirements	2071

A.5.6.5	Measurements with autonomous gaps	2071

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2071

A.5.6.5.1.1	Test Purpose and Environment	2071

A.5.6.5.1.2	Test Requirements	2074

A.5.6.6	L1-SINR measurement for beam reporting	2075

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2077

A.5.6.6.2.1	Test Purpose and Environment	2077

A.5.6.6.2.2	Test parameters	2078

A.5.6.6.2.3	Test Requirements	2081

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2081

A.5.6.6.3.1	Test Purpose and Environment	2081

A.5.6.6.3.2	Test parameters	2082

A.5.6.6.3.3	Test Requirements	2084

A.5.6.7	CSI-RS based Intra-frequency Measurements	2084

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2084

A.5.6.7.1.1	Test purpose and Environment	2084

A.5.6.7.1.2	Test Requirements	2087

A.5.6.8	CSI-RS based Inter-frequency Measurements	2088

A.5.6.8.1	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2088

A.5.6.8.1.1	Test Purpose and Environment	2088

A.5.6.8.1.2	Test Requirements	2092

A.5.7	Measurement Performance requirements	2092

A.5.7.1	SS-RSRP	2092

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2092

A.5.7.1.1.1	Test Purpose and Environment	2092

A.5.7.1.1.2	Test parameters	2093

A.5.7.1.1.3	Test Requirements	2095

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2096

A.5.7.1.2.1	Test Purpose and Environment	2096

A.5.7.1.2.2	Test parameters	2096

A.5.7.1.2.3	Test Requirements	2100

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2101

A.5.7.1.3.1	Test Purpose and Environment	2101

A.5.7.1.3.2	Test parameters	2101

A.5.7.1.3.3	Test Requirements	2104

A.5.7.2	SS-RSRQ	2105

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2105

A.5.7.2.1.1	Test Purpose and Environment	2105

A.5.7.2.1.2	Test Parameters	2105

A.5.7.2.1.3	Test Requirements	2108

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2108

A.5.7.2.2.1	Test Purpose and Environment	2108

A.5.7.2.2.2	Test Parameters	2108

A.5.7.2.2.3	Test Requirements	2111

A.5.7.3	SS-SINR	2111

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2111

A.5.7.3.1.1	Test Purpose and Environment	2111

A.5.7.3.1.2	Test Parameters	2111

A.5.7.3.1.3	Test Requirements	2114

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2114

A.5.7.3.2.1	Test Purpose and Environment	2114

A.5.7.3.2.2	Test Parameters	2114

A.5.7.3.2.3	Test Requirements	2116

A.5.7.4	L1-RSRP measurement for beam reporting	2116

A.5.7.4.1	SSB based L1-RSRP measurement	2116

A.5.7.4.1.1	Test Purpose and Environment	2116

A.5.7.4.1.2	Test parameters	2117

A.5.7.4.1.3	Test Requirements	2119

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2120

A.5.7.4.2.1	Test Purpose and Environment	2120

A.5.7.4.2.2	Test parameters	2120

A.5.7.4.2.3	Test Requirements	2122

A.5.7.5	CLI measurements	2123

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2123

A.5.7.5.1.1	Test Purpose and Environment	2123

A.5.7.5.1.2	Test parameters	2123

A.5.7.5.1.3	Test Requirements	2126

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2127

A.5.7.5.2.1	Test Purpose and Environment	2127

A.5.7.5.2.2	Test parameters	2127

A.5.7.5.2.3	Test Requirements	2129

A.5.7.6	L1-SINR measurement for beam reporting	2130

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2133

A.5.7.6.2.1	Test Purpose and Environment	2133

A.5.7.6.2.2	Test parameters	2133

A.5.7.6.2.3	Test Requirements	2135

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2136

A.5.7.6.3.1	Test Purpose and Environment	2136

A.5.7.6.3.2	Test parameters	2136

A.5.7.6.3.3	Test Requirements	2138

A.5.7.7	CSI-RSRP	2139

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2139

A.5.7.7.1.1	Test Purpose and Environment	2139

A.5.7.7.1.2	Test parameters	2139

A.5.7.7.1.3	Test Requirements	2143

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2144

A.5.7.7.2.1	Test Purpose and Environment	2144

A.5.7.7.2.2	Test parameters	2144

A.5.7.7.2.3	Test Requirements	2148

A.5.7.8	CSI-RSRQ	2149

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2149

A.5.7.8.1.1	Test Purpose and Environment	2149

A.5.7.8.1.2	Test Parameters	2149

A.5.7.8.1.3	Test Requirements	2151

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2151

A.5.7.8.2.1	Test Purpose and Environment	2151

A.5.7.8.2.2	Test Parameters	2151

A.5.7.8.2.3	Test Requirements	2153

A.5.7.9	CSI-SINR	2153

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2153

A.5.7.9.1.1	Test Purpose and Environment	2153

A.5.7.9.1.2	Test Parameters	2154

A.5.7.9.1.3	Test Requirements	2156

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2156

A.5.7.9.2.1	Test Purpose and Environment	2156

A.5.7.9.2.2	Test Parameters	2157

A.5.7.9.2.3	Test Requirements	2159

A.5.8	Void	2159

A.6	NR standalone tests with all NR cells in FR1	2161

A.6.1	SA: RRC_IDLE state mobility	2161

A.6.1.1	Cell re-selection to NR	2161

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2161

A.6.1.1.1.1	Test Purpose and Environment	2161

A.6.1.1.1.2	Test Parameters	2161

A.6.1.1.1.3	Test Requirements	2165

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2165

A.6.1.1.2.1	Test Purpose and Environment	2165

A.6.1.1.2.2	Test Parameters	2165

A.6.1.1.2.3	Test Requirements	2169

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2169

A.6.1.1.3.1	Test Purpose and Environment	2169

A.6.1.1.3.2	Test Parameters	2169

A.6.1.1.3.3	Test Requirements	2174

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2174

A.6.1.1.4.1	Test Purpose and Environment	2174

A.6.1.1.4.2	Test Parameters	2174

A.6.1.1.4.3	Test Requirements	2177

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2177

A.6.1.1.5.1	Test Purpose and Environment	2177

A.6.1.1.5.2	Test Parameters	2177

A.6.1.1.5.3	Test Requirements	2182

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2182

A.6.1.1.6.1	Test Purpose and Environment	2182

A.6.1.1.6.2	Test Parameters	2183

A.6.1.1.6.3	Test Requirements	2187

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2187

A.6.1.1.7.1	Test Purpose and Environment	2187

A.6.1.1.7.2	Test Parameters	2188

A.6.1.1.7.3	Test Requirements	2191

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2191

A.6.1.1.8.1	Test Purpose and Environment	2191

A.6.1.1.8.2	Test Parameters	2191

A.6.1.1.8.3	Test Requirements	2195

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2195

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2195

A.6.1.2.1.1	Test Purpose and Environment	2195

A.6.1.2.1.2	Test Parameters	2195

A.6.1.2.1.3	Test Requirements	2198

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2199

A.6.1.2.2.1	Test Purpose and Environment	2199

A.6.1.2.2.2	Test Parameters	2199

A.6.1.2.2.3	Test Requirements	2202

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2203

A.6.1.2.3.1	Test Purpose and Environment	2203

A.6.1.2.3.2	Test Parameters	2203

A.6.1.2.3.3	Test Requirements	2206

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2207

A.6.1.2.4.1	Test Purpose and Environment	2207

A.6.1.2.4.2	Test Parameters	2207

A.6.1.2.4.3	Test Requirements	2210

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2211

A.6.1.2.5.1	Test Purpose and Environment	2211

A.6.1.2.5.2	Test Parameters	2211

A.6.1.2.5.3	Test Requirements	2215

A.6.1.1.7	Void	2216

A.6.2	SA: RRC_INACTIVE state mobility	2216

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2216

A.6.2.1.1	Test purpose and Environment	2216

A.6.2.1.2	Test Parameters	2218

A.6.2.1.3	Test requirements	2220

A.6.3	RRC_CONNECTED state mobility	2220

A.6.3.1	Handover	2220

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2220

A.6.3.1.1.1	Test Purpose and Environment	2220

A.6.3.1.1.2	Test Parameters	2220

A.6.3.1.1.3 Test Requirements	2224

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2224

A.6.3.1.2.1	Test Purpose and Environment	2224

A.6.3.1.2.2	Test Parameters	2224

A.6.3.1.2.3	Test Requirements	2228

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2228

A.6.3.1.3.1	Test Purpose and Environment	2228

A.6.3.1.3.2	Test Parameters	2228

A.6.3.1.3.3	Test Requirements	2232

A.6.3.1.4	SA NR - E-UTRAN handover	2232

A.6.3.1.4.1	Test Purpose and Environment	2232

A.6.3.1.4.2	Test Requirements	2238

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2238

A.6.3.1.5.1	Test Purpose and Environment	2238

A.6.3.1.5.2	Test Requirements	2244

A.6.3.1.6	 SA NR - UTRAN FDD handover	2244

A.6.3.1.6.1	Test Purpose and Environment	2244

A.6.3.1.6.2	Test Requirements	2248

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2248

A.6.3.1.7.1	Test Purpose and Environment	2248

A.6.3.1.7.2	Test Parameters	2248

A.6.3.1.7.3	Test Requirements	2252

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2253

A.6.3.1.8.1	Test Purpose and Environment	2253

A.6.3.1.8.2	Test Parameters	2253

A.6.3.1.8.3	Test Requirements	2257

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2258

A.6.3.1.9.1	Test Purpose and Environment	2258

A.6.3.1.9.2	Test Parameters	2258

A.6.3.1.9.3	Test Requirements	2262

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2262

A.6.3.1.10.1	Test Purpose and Environment	2262

A.6.3.1.10.2	Test Parameters	2262

A.6.3.1.10.3	Test Requirements	2265

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2265

A.6.3.1.11.1	Test Purpose and Environment	2265

A.6.3.1.11.2	Test Parameters	2265

A.6.3.1.11.3 Test Requirements	2272

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2272

A.6.3.1.12.1	Test Purpose and Environment	2272

A.6.3.1.12.2	Test Parameters	2273

A.6.3.1.12.3 Test Requirements	2280

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2280

A.6.3.1.13.1	Test Purpose and Environment	2280

A.6.3.1.13.2	Test Requirements	2289

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2289

A.6.3.1.14.1	Test Purpose and Environment	2289

A.6.3.1.14.2	Test Requirements	2298

A.6.3.2	RRC Connection Mobility Control	2299

A.6.3.2.1	SA: RRC Re-establishment	2299

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2299

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2303

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2307

A.6.3.2.2	Random Access	2310

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2310

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2314

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2319

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2324

A.6.3.2.3	SA: RRC Connection Release with Redirection	2328

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2328

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2332

A.6.3.3 Conditional handover	2339

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2339

A.6.3.3.1.1	Test Purpose and Environment	2339

A.6.3.3.1.2	Test Parameters	2339

A.6.3.3.1.3 Test Requirements	2343

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2343

A.6.3.3.2.1	Test Purpose and Environment	2343

A.6.3.3.2.2	Test Parameters	2343

A.6.3.3.2.3 Test Requirements	2347

A.6.4	Timing	2347

A.6.4.1	UE transmit timing	2347

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2347

A.6.4.1.1.1	Test Purpose and environment	2347

A.6.4.1.1.2	Test requirements	2351

A.6.4.2	UE timer accuracy	2351

A.6.4.3	Timing advance	2351

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2351

A.6.4.3.1.1	Test Purpose and Environment	2351

A.6.4.3.1.2	Test Parameters	2351

A.6.4.3.1.3	Test Requirements	2355

A.6.5	Signalling characteristics	2355

A.6.5.1	Radio link Monitoring	2355

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2356

A.6.5.1.1.1	Test Purpose and Environment	2356

A.6.5.1.1.2	Test Requirements	2361

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2361

A.6.5.1.2.1	Test Purpose and Environment	2361

A.6.5.1.2.2	Test Requirements	2367

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2367

A.6.5.1.3.1	Test Purpose and Environment	2367

A.6.5.1.3.2	Test Requirements	2373

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2373

A.6.5.1.4.1	Test Purpose and Environment	2373

A.6.5.1.4.2	Test Requirements	2379

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2379

A.6.5.1.5.1	Test Purpose and Environment	2379

A.6.5.1.5.2	Test Requirements	2385

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2385

A.6.5.1.6.1	Test Purpose and Environment	2385

A.6.5.1.6.2	Test Requirements	2390

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2390

A.6.5.1.7.1	Test Purpose and Environment	2390

A.6.5.1.7.2	Test Requirements	2394

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2394

A.6.5.1.8.1	Test Purpose and Environment	2394

A.6.5.1.8.2	Test Requirements	2400

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2400

A.6.5.1.9.1	Test Purpose and Environment	2400

A.6.5.1.9.2	Test Requirements	2406

A.6.5.2	Interruption	2406

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2406

A.6.5.2.1.2	Test Requirements	2410

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2411

A.6.5.2.2.1	Test Purpose and Environment	2411

A.6.5.2.2.2	Test Parameters	2411

A.6.5.2.2.3	Test Requirements	2413

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2414

A.6.5.2.3.1	Test Purpose and Environment	2414

A.6.5.2.3.2	Test Parameters	2414

A.6.5.2.3.3	Test Requirements	2416

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2417

A.6.5.2.4.1	Test Purpose and Environment	2417

A.6.5.2.4.2	Test Parameters	2417

A.6.5.2.4.3	Test Requirements	2419

A.6.5.3	SCell Activation and Deactivation Delay	2420

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2420

A.6.5.3.1.1	Test Purpose and Environment	2420

A.6.5.3.1.2	Test Requirements	2425

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2426

A.6.5.3.2.1	Test Purpose and Environment	2426

A.6.5.3.2.2	Test Requirements	2426

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2426

A.6.5.3.3.1	Test Purpose and Environment	2426

A.6.5.3.3.2	Test Requirements	2427

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2427

A.6.5.3.4.1	Test Purpose and Environment	2427

A.6.5.3.4.2	Test Requirements	2434

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2434

A.6.5.3.5.1	Test Purpose and Environment	2434

A.6.5.3.5.2	Test Requirements	2439

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2439

A.6.5.3.6.1	Test Purpose and Environment	2439

A.6.5.3.6.2	Test Requirements	2443

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2444

A.6.5.3.7.1	Test Purpose and Environment	2444

A.6.5.3.7.2	Test Requirements	2448

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2449

A.6.5.3.8.1	Test Purpose and Environment	2449

A.6.5.3.8.2	Test Requirements	2453

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2454

A.6.5.3.9.1	Test Purpose and Environment	2454

A.6.5.3.9.2	Test Requirements	2458

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2459

A.6.5.3.10.1	Test Purpose and Environment	2459

A.6.5.3.10.2	Test Requirements	2462

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2462

A.6.5.3.11.1	Test Purpose and Environment	2462

A.6.5.3.11.2	Test Requirements	2463

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2463

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2463

A.6.5.4.1.1	Test Purpose and Environment	2463

A.6.5.4.1.2	Test Requirements	2468

A.6.5.4.2	Void	2468

A.6.5.5	Beam Failure Detection and Link recovery procedures	2468

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2468

A.6.5.5.1.1	Test Purpose and Environment	2468

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2474

A.6.5.5.2.1	Test Purpose and Environment	2474

A.6.5.5.2.2	Test Requirements	2480

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2481

A.6.5.5.3.1	Test Purpose and Environment	2481

A.6.5.5.3.2	Test Requirements	2486

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2487

A.6.5.5.4.1	Test Purpose and Environment	2487

A.6.5.5.4.2	Test Requirements	2492

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2493

A.6.5.5.5.1	Test Purpose and Environment	2493

A.6.5.5.5.2	Test Requirements	2497

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2497

A.6.5.5.6.1	Test Purpose and Environment	2497

A.6.5.5.6.2	Test Requirements	2502

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2502

A.6.5.5.7.1	Test Purpose and Environment	2502

A.6.5.5.7.2	Test Requirements	2508

A.6.5.6	Active BWP switch	2509

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2509

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2509

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2515

A.6.5.6.2	RRC-based Active BWP Switch	2520

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2520

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2524

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2524

A.6.5.6.4	SCell dormancy switch	2531

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2531

A.6.5.6.4.1.1	Test Purpose and Environment	2531

A.6.5.6.4.1.2	Test Requirements	2537

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2537

A.6.5.6.4.2.1	 Test Purpose and Environment	2537

A.6.5.6.4.2.2	Test Requirements	2544

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2544

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2544

A.6.5.7	DL interruptions at switching between two uplink carriers	2549

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2549

A.6.5.7.1.1	Test Purpose and Environment	2549

A.6.5.7.1.2	Test Requirements	2553

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2553

A.6.5.7.2.1	Test Purpose and Environment	2553

A.6.5.7.2.2	Test Requirements	2557

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2557

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2557

A.6.5.7A.1.1	Test Purpose and Environment	2557

A.6.5.7A.1.2	Test Requirements	2561

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2561

A.6.5.7A.2.1	Test Purpose and Environment	2561

A.6.5.7A.2.2	Test Requirements	2565

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2565

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2565

A.6.5.7B.1.1	Test Purpose and Environment	2565

A.6.5.7B.1.2	Test Requirements	2569

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2569

A.6.5.7B.2.1	Test Purpose and Environment	2569

A.6.5.7B.2.2	Test Requirements	2573

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2573

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2573

A.6.5.7C.1.1	Test Purpose and Environment	2573

A.6.5.7C.1.2	Test Requirements	2577

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2578

A.6.5.7C.2.1	Test Purpose and Environment	2578

A.6.5.7C.2.2	Test Requirements	2582

A.6.5.8	UE specific CBW change	2583

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2583

A.6.5.8.1.1	Test Purpose and Environment	2583

A.6.5.8.1.2	Test Requirements	2587

A.6.5.9	Pathloss reference signal switching delay	2587

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2587

A.6.5.9.1.1	Test Purpose and Environment	2587

A.6.5.9.1.2	Test Requirements	2590

A.6.6	Measurement procedure	2591

A.6.6.1	Intra-frequency Measurements	2591

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2591

A.6.6.1.1.1	Test purpose and Environment	2591

A.6.6.1.1.2	Test parameters	2591

A.6.6.1.1.3	Test Requirements	2595

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2595

A.6.6.1.2.1	Test purpose and Environment	2595

A.6.6.1.2.2	Test parameters	2595

A.6.6.1.2.3	Test Requirements	2599

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2599

A.6.6.1.3.1	Test purpose and Environment	2599

A.6.6.1.3.2	Test parameters	2599

A.6.6.1.3.3	Test Requirements	2603

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2603

A.6.6.1.4.1	Test purpose and Environment	2603

A.6.6.1.4.2	Test parameters	2603

A.6.6.1.4.3	Test Requirements	2607

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2607

A.6.6.1.5.1	Test purpose and Environment	2607

A.6.6.1.5.2	Test parameters	2607

A.6.6.1.5.3	Test Requirements	2609

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2610

A.6.6.1.6.1	Test purpose and Environment	2610

A.6.6.1.6.2	Test parameters	2610

A.6.6.1.6.3	Test Requirements	2611

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2612

A.6.6.1.7.1	Test purpose and Environment	2612

A.6.6.1.7.2	Test parameters	2612

A.6.6.1.7.3	Test Requirements	2616

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2616

A.6.6.1.8.1	Test purpose and Environment	2616

A.6.6.1.8.2	Test parameters	2616

A.6.6.1.8.3	Test Requirements	2621

A.6.6.2	Inter-frequency Measurements	2621

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2621

A.6.6.2.1.1	Test Purpose and Environment	2621

A.6.6.2.1.2	Test Requirements	2625

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2625

A.6.6.2.2.1	Test Purpose and Environment	2625

A.6.6.2.2.2	Test Requirements	2629

A.6.6.2.3	Void	2630

A.6.6.2.4	Void	2630

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2630

A.6.6.2.5.1	Test Purpose and Environment	2630

A.6.6.2.5.2	Test Requirements	2634

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2634

A.6.6.2.6.1	Test Purpose and Environment	2634

A.6.6.2.6.2	Test Requirements	2638

A.6.6.2.7	Void	2638

A.6.6.2.8	Void	2638

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2638

A.6.6.2.9.1	Test Purpose and Environment	2638

A.6.6.2.9.2	Test Requirements	2642

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2642

A.6.6.2.10.1	Test Purpose and Environment	2642

A.6.6.2.10.2	Test Requirements	2646

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2650

A.6.6.2.12.1	Test Purpose and Environment	2650

A.6.6.2.12.2	Test Requirements	2654

A.6.6.3	Inter-RAT Measurements	2654

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2654

A.6.6.3.1.1	Test Purpose and Environment	2654

A.6.6.3.1.2	Test Requirements	2660

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2660

A.6.6.3.2.1	Test Purpose and Environment	2660

A.6.6.3.2.2	Test Requirements	2667

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2667

A.6.6.3.3.1	Test Purpose and Environment	2667

A.6.6.3.3.2	Test Requirements	2674

A.6.6.4	L1-RSRP measurement for beam reporting	2674

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2674

A.6.6.4.1.1	Test Purpose and Environment	2674

A.6.6.4.1.2	Test parameters	2674

A.6.6.4.1.3	Test Requirements	2677

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2677

A.6.6.4.2.1	Test Purpose and Environment	2677

A.6.6.4.2.2	Test parameters	2678

A.6.6.4.2.3	Test Requirements	2681

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2681

A.6.6.4.3.1	Test Purpose and Environment	2681

A.6.6.4.3.2	Test parameters	2682

A.6.6.4.3.3	Test Requirements	2685

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2685

A.6.6.4.4.1	Test Purpose and Environment	2685

A.6.6.4.4.2	Test parameters	2686

A.6.6.4.4.3	Test Requirements	2689

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2689

A.6.6.4.5.1	Test Purpose and Environment	2689

A.6.6.4.5.2	Test parameters	2690

A.6.6.4.5.3	Test Requirements	2693

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2693

A.6.6.4.6.1	Test Purpose and Environment	2693

A.6.6.4.6.2	Test parameters	2694

A.6.6.4.6.3	Test Requirements	2697

A.6.6.5	Inter-RAT UTRAN FDD measurements	2698

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2698

A.6.6.5.1.1	Test Purpose and Environment	2698

A.6.6.5.1.2	Test Requirements	2701

A.6.6.6	CLI measurements	2701

A.6.6.6.1	SRS-RSRP measurement with DRX	2701

A.6.6.6.1.1	Test Purpose and Environment	2701

A.6.6.6.1.2	Test Parameters	2702

A.6.6.6.1.3	Test Requirements	2705

A.6.6.6.2	CLI-RSSI measurement with DRX	2705

A.6.6.6.2.1	Test Purpose and Environment	2705

A.6.6.6.2.2	Test Parameters	2706

A.6.6.6.2.3	Test Requirements	2708

A.6.6.7	NR measurements with autonomous gaps	2708

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2708

A.6.6.7.1.1	Test Purpose and Environment	2708

A.6.6.7.1.2	Test Parameters	2708

A.6.6.7.1.3	Test Requirements	2712

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2712

A.6.6.7.2.1	Test Purpose and Environment	2712

A.6.6.7.2.2	Test Requirements	2715

A.6.6.8	L1-SINR measurement for beam reporting	2716

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2718

A.6.6.8.2.1	Test Purpose and Environment	2718

A.6.6.8.2.2	Test parameters	2719

A.6.6.8.2.3	Test Requirements	2723

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2723

A.6.6.8.3.1	Test Purpose and Environment	2723

A.6.6.8.3.2	Test parameters	2724

A.6.6.8.3.3	Test Requirements	2726

A.6.6.9	Idle Mode CA/DC Measurements	2726

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2726

A.6.6.9.1.1	Test Purpose and Environment	2726

A.6.6.9.1.2	Test Requirements	2733

A.6.6.10	CSI-RS based intra-frequency Measurements	2733

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2733

A.6.6.10.1.1	Test purpose and Environment	2733

A.6.6.10.1.2	Test Requirements	2737

A.6.6.11	CSI-RS based inter-frequency Measurements	2737

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2737

A.6.6.11.1.1	Test Purpose and Environment	2737

A.6.6.11.1.2	Test Requirements	2741

A.6.6.12	RSTD measurements	2741

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2741

A.6.6.12.1.1	Test Purpose and Environment	2741

A.6.6.12.1.2	Test Requirements	2748

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2749

A.6.6.12.2.1	Test Purpose and Environment	2749

A.6.6.12.2.2	Test Requirements	2755

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2755

A.6.6.12.3.1	Test Purpose and Environment	2755

A.6.6.12.3.2	Test Requirements	2761

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2762

A.6.6.12.4.1	Test Purpose and Environment	2762

A.6.6.12.4.2	Test Requirements	2765

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2766

A.6.6.12.5.1	Test Purpose and Environment	2766

A.6.6.12.5.2	Test Requirements	2770

A.6.6.13 PRS-RSRP measurements	2770

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2770

A.6.6.13.1.1	Test purpose and Environment	2770

A.6.6.13.1.2	Test Requirements	2774

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2774

A.6.6.13.2.1	Test purpose and Environment	2774

A.6.6.13.2.2	Test Requirements	2778

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2778

A.6.6.13.3.1	Test purpose and Environment	2778

A.6.6.13.3.2	Test Requirements	2781

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2781

A.6.6.13.4.1	Test purpose and Environment	2781

A.6.6.14	UE Rx-Tx time difference measurements	2786

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2786

A.6.6.14.1.1	Test purpose and environment	2786

A.6.6.14.1.2	Test requirements	2790

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2790

A.6.6.14.2.1	Test purpose and environment	2790

A.6.6.14.2.2	Test requirements	2795

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2795

A.6.6.14.3.1	Test purpose and environment	2795

A.6.6.14.3.2	Test requirements	2798

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2799

A.6.6.14.4.1	Test purpose and environment	2799

A.6.6.14.4.2	Test requirements	2801

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2802

A.6.6.14.4.1	Test purpose and environment	2802

A.6.6.14.4.2	Test requirements	2806

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2806

A.6.6.15.1	Test Purpose and Environment	2806

A.6.6.15.2	Test Requirements	2813

A.6.6.16	PRS-RSRPP measurements	2814

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2814

A.6.6.16.1.1	Test purpose and Environment	2814

A.6.6.16.1.2	Test Requirements	2816

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2816

A.6.6.16.2.1	Test purpose and Environment	2816

A.6.6.16.2.2	Test Requirements	2819

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2819

A.6.6.16.3.1	Test purpose and Environment	2819

A.6.6.16.3.2	Test Requirements	2822

A.6.6.17	SA event triggered reporting tests with Pre-MG	2822

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2822

A.6.6.17.1.1	Test purpose and Environment	2822

A.6.6.17.1.2	Test parameters	2822

A.6.6.17.1.3	Test Requirements	2826

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2826

A.6.6.17.2.1	Test purpose and Environment	2826

A.6.6.17.2.2	Test parameters	2826

A.6.6.17.2.3	Test Requirements	2831

A.6.6.17.3	Void	2831

A.6.6.17.3.1	Void	2831

A.6.6.17.3.2	Void	2831

A.6.6.17.3.3	Void	2831

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2831

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2831

A.6.6.18.1.1	Test Purpose and Environment	2831

A.6.6.18.1.2	Test Requirements	2835

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2835

A.6.6.18.2.1	Test Purpose and Environment	2835

A.6.6.18.2.2	Test Requirements	2839

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2839

A.6.6.18.3.1	Test Purpose and Environment	2839

A.6.6.18.3.2	Test Requirements	2846

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2846

A.6.6.18.4.1	Test Purpose and Environment	2846

A.6.6.18.4.2	Test Requirements	2850

A.6.6.19	SA event triggered reporting tests with NCSG	2851

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2851

A.6.6.19.1.1	Test purpose and Environment	2851

A.6.6.19.1.2	Test parameters	2851

A.6.6.19.1.3	Test Requirements	2855

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2855

A.6.6.19.2.1	Test Purpose and Environment	2855

A.6.6.19.2.2	Test parameters	2855

A.6.6.19.2.3	Test Requirements	2859

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2859

A.6.6.19.3.1	Test Purpose and Environment	2859

A.6.6.19.3.2	Test parameters	2860

A.6.6.19.3.3	Test Requirements	2865

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2865

A.6.6.19.4.1	Test purpose and Environment	2865

A.6.6.19.4.2	Test parameters	2865

A.6.6.19.4.3	Test Requirements	2868

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2868

A.6.6.20.1	Test purpose and environment	2868

A.6.6.20.2	Test requirements	2872

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2872

A.6.6.21.1	Test purpose and environment	2872

A.6.6.21.2	Test requirements	2874

A.6.7	Measurement Performance requirements	2875

A.6.7.1	SS-RSRP	2875

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2875

A.6.7.1.1.1	Test Purpose and Environment	2875

A.6.7.1.1.2	Test parameters	2875

A.6.7.1.1.3	Test Requirements	2880

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2880

A.6.7.1.2.1	Test Purpose and Environment	2880

A.6.7.1.2.2	Test parameters	2880

A.6.7.1.2.3	Test Requirements	2884

A.6.7.1.3	Void	2885

A.6.7.2	SS-RSRQ	2885

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2885

A.6.7.2.1.1	Test Purpose and Environment	2885

A.6.7.2.1.2	Test Parameters	2885

A.6.7.2.1.3	Test Requirements	2889

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2889

A.6.7.2.2.1	Test Purpose and Environment	2889

A.6.7.2.2.2	Test Parameters	2889

A.6.7.2.2.3	Test Requirements	2896

A.6.7.3	SS-SINR	2896

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2896

A.6.7.3.1.1	Test Purpose and Environment	2896

A.6.7.3.1.2	Test Parameters	2896

A.6.7.3.1.3	Test Requirements	2901

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2901

A.6.7.3.2.1	Test Purpose and Environment	2901

A.6.7.3.2.2	Test Parameters	2901

A.6.7.3.2.3	Test Requirements	2906

A.6.7.4	L1-RSRP measurement for beam reporting	2906

A.6.7.4.1	SSB based L1-RSRP measurement	2906

A.6.7.4.1.1	Test Purpose and Environment	2906

A.6.7.4.1.2	Test parameters	2907

A.6.7.4.1.3	Test Requirements	2912

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2912

A.6.7.4.2.1	Test Purpose and Environment	2912

A.6.7.4.2.2	Test parameters	2912

A.6.7.4.2.3	Test Requirements	2917

A.6.7.5	E-UTRAN RSRP	2917

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2917

A.6.7.5.1.1	Test Purpose and Environment	2917

A.6.7.5.1.2	Test parameters	2917

A.6.7.5.1.3	Test Requirements	2923

A.6.7.6	E-UTRAN RSRQ	2924

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2924

A.6.7.6.1.1	Test Purpose and Environment	2924

A.6.7.6.1.2	Test parameters	2924

A.6.7.6.1.3	Test Requirements	2929

A.6.7.7	E-UTRAN RS-SINR	2930

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2930

A.6.7.7.1.1	Test Purpose and Environment	2930

A.6.7.7.1.2	Test parameters	2930

A.6.7.7.1.3	Test Requirements	2936

A.6.7.8	CLI measurements	2937

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2937

A.6.7.8.1.1	Test Purpose and Environment	2937

A.6.7.8.1.2	Test parameters	2937

A.6.7.8.1.3	Test Requirements	2943

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2943

A.6.7.8.2.1	Test Purpose and Environment	2943

A.6.7.8.2.2	Test parameters	2944

A.6.7.8.2.3	Test Requirements	2947

A.6.7.9	L1-SINR measurement for beam reporting	2948

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2953

A.6.7.9.2.1	Test Purpose and Environment	2953

A.6.7.9.2.2	Test parameters	2953

A.6.7.9.2.3	Test Requirements	2958

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2958

A.6.7.9.3.1	Test Purpose and Environment	2958

A.6.7.9.3.2	Test parameters	2959

A.6.7.9.3.3	Test Requirements	2964

A.6.7.10	CSI-RSRP	2964

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2964

A.6.7.9.10.1	Test Purpose and Environment	2964

A.6.7.9.10.2	Test parameters	2964

A.6.7.10.1.3	Test Requirements	2969

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2969

A.6.7.9.10.1	Test Purpose and Environment	2969

A.6.7.10.2.2	Test parameters	2970

A.6.7.10.2.3	Test Requirements	2976

A.6.7.11	CSI-RSRQ	2976

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2976

A.6.7.11.1.1	Test Purpose and Environment	2976

A.6.7.11.1.2	Test Parameters	2976

A.6.7.11.1.3	Test Requirements	2982

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2982

A.6.7.11.2.1	Test Purpose and Environment	2982

A.6.7.11.2.2	Test Parameters	2982

A.6.7.11.2.3	Test Requirements	2988

A.6.7.12	CSI-SINR	2988

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2988

A.6.7.12.1.1	Test Purpose and Environment	2988

A.6.7.12.1.2	Test Parameters	2988

A.6.7.12.1.3	Test Requirements	2994

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2994

A.6.7.12.2.1	Test Purpose and Environment	2994

A.6.7.12.2.2	Test Parameters	2994

A.6.7.12.2.3	Test Requirements	3001

A.6.7.13	RSTD measurements	3001

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	3001

A.6.7.13.1.1	Test purpose and Environment	3001

A.6.7.13.1.2	Test Requirements	3004

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	3004

A.6.7.13.2.1	Test purpose and Environment	3004

A.6.7.13.2.2	Test Requirements	3008

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3008

A.6.7.13.3.1	Test purpose and Environment	3008

A.6.7.13.3.2	Test Requirements	3010

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	3011

A.6.7.14	PRS-RSRP measurements	3013

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	3013

A.6.7.14.1.1	Test Purpose and Environment	3013

A.6.7.14.1.2	Test parameters	3013

A.6.7.14.1.3	Test Requirements	3017

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	3017

A.6.7.14.2.1	Test Purpose and Environment	3017

A.6.7.14.2.2	Test parameters	3017

A.6.7.14.2.3	Test Requirements	3020

A.6.7.14.3	Void	3020

A.6.7.14.3.1	Void	3020

A.6.7.14.3.2	Void	3020

A.6.7.14.3.3	Void	3020

A.6.7.15	UE Rx-Tx time difference measurements	3020

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	3020

A.6.7.15.1.1	Test purpose and environment	3020

A.6.7.15.1.2	Test parameters	3021

A.6.7.15.1.3	Test requirements	3024

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	3025

A.6.7.15.2.1	Test purpose and environment	3025

A.6.7.15.2.2	Test parameters	3025

A.6.7.15.2.3	Test requirements	3027

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3027

A.6.7.15.3.1	Test purpose and environment	3027

A.6.7.15.3.2	Test parameters	3028

A.6.7.15.3.3	Test requirements	3031

A.6.7.16	PRS-RSRPP measurements	3031

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3031

A.6.7.16.1.1	Test Purpose and Environment	3031

A.6.7.16.1.2	Test parameters	3032

A.6.7.16.1.3	Test Requirements	3036

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3036

A.6.7.16.2.1	Test Purpose and Environment	3036

A.6.7.16.2.2	Test parameters	3036

A.6.8	Measurement procedure in RRC_INACTIVE	3040

A.6.8.1	RSTD measurements	3040

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3040

A.6.8.1.1.1	Test Purpose and Environment	3040

A.6.8.1.1.2	Test Requirements	3043

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3043

A.6.8.1.2.1	Test Purpose and Environment	3043

A.6.8.1.2.2	Test Requirements	3048

A.6.8.2	PRS-RSRP measurements	3048

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3048

A.6.8.2.1.1	Test purpose and Environment	3048

A.6.8.2.1.2	Test Requirements	3050

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3051

A.6.8.2.2.1	Test purpose and Environment	3051

A.6.8.2.2.2	Test Requirements	3055

A.6.8.3	UE Rx-Tx time difference measurements	3055

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3055

A.6.8.3.1.1	Test purpose and environment	3055

A.6.8.3.1.2	Test requirements	3059

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3059

A.6.8.3.2.1	Test purpose and environment	3059

A.6.8.3.2.2	Test requirements	3062

A.6.8.4	PRS-RSRPP measurements	3062

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3062

A.6.8.4.1.1	Test purpose and Environment	3062

A.6.8.4.1.2	Test Requirements	3065

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3065

A.6.8.4.2.1	Test purpose and Environment	3065

A.6.8.4.2.2	Test Requirements	3067

A.6.9	Measurement performance requirements in RRC_INACTIVE	3068

A.6.9.1	RSTD measurements	3068

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3068

A.6.9.1.1.1	Test purpose and Environment	3068

A.6.9.1.1.2	Test Requirements	3070

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3070

A.6.9.1.2.1	Test purpose and Environment	3070

A.6.9.1.2.2	Test Requirements	3072

A.6.9.2	PRS-RSRP measurements	3072

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3072

A.6.9.2.1.1	Test Purpose and Environment	3072

A.6.9.2.1.2	Test parameters	3072

A.6.9.2.1.3	Test Requirements	3076

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3076

A.6.9.2.2.1	Test Purpose and Environment	3076

A.6.9.2.2.2	Test parameters	3076

A.6.9.2.2.3	Test Requirements	3078

A.6.9.3	UE Rx-Tx time difference measurements	3079

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3079

A.6.9.3.1.1.1	Test purpose and environment	3079

A.6.9.3.1.1.2 Test parameters	3079

A.6.9.3.1.1.3	Test requirements	3081

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3081

A.6.9.3.2.1	Test purpose and environment	3081

A.6.9.3.2.2	Test parameters	3081

A.6.9.3.2.3	Test requirements	3084

A.6.9.4	PRS-RSRPP measurements	3084

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3084

A.6.9.4.1.1	Test Purpose and Environment	3084

A.6.9.4.1.2	Test parameters	3084

A.6.9.4.1.3	Test Requirements	3088

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3088

A.6.9.4.2.1	Test Purpose and Environment	3088

A.6.9.4.2.2	Test parameters	3088

A.6.9.4.2.3	Test Requirements	3090

A.7	NR standalone tests with one or more NR cells in FR2	3092

A.7.1	SA: RRC_IDLE state mobility	3092

A.7.1.1	Cell re-selection to NR	3092

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3092

A.7.1.1.1.1	Test Purpose and Environment	3092

A.7.1.1.1.2	Test Parameters	3092

A.7.1.1.1.3	Test Requirements	3095

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3096

A.7.1.1.2.1	Test Purpose and Environment	3096

A.7.1.1.2.2	Test Parameters	3096

A.7.1.1.2.3	Test Requirements	3098

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3099

A.7.1.1.3.1	Test Purpose and Environment	3099

A.7.1.1.3.2	Test Parameters	3099

A.7.1.1.3.3	Test Requirements	3102

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3102

A.7.1.1.4.1	Test Purpose and Environment	3102

A.7.1.1.4.2	Test Parameters	3102

A.7.1.1.4.3	Test Requirements	3105

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3105

A.7.1.1.5.1	Test Purpose and Environment	3105

A.7.1.1.5.2	Test Parameters	3105

A.7.1.1.5.3	Test Requirements	3108

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3109

A.7.1.1.6.1	Test Purpose and Environment	3109

A.7.1.1.6.2	Test Parameters	3109

A.7.1.1.6.3	Test Requirements	3112

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3113

A.7.1.1.7.1	Test Purpose and Environment	3113

A.7.1.1.7.2	Test Parameters	3113

A.7.1.1.7.3	Test Requirements	3117

A.7.2	SA: RRC_INACTIVE state mobility	3117

A.7.2.1	Small Data Transmission	3117

A.7.2.1.1	TA validation for CG-SDT in FR2	3117

A.7.2.1.1.1	Test Purpose and Environment	3117

A.7.2.1.1.2	Test Requirements	3121

A.7.3	RRC_CONNECTED state mobility	3121

A.7.3.1	Handover	3121

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3121

A.7.3.1.1.1	Test Purpose and Environment	3121

A.7.3.1.1.2	Test Parameters	3121

A.7.3.1.1.3	Test Requirements	3125

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3125

A.7.3.1.2.1	Test Purpose and Environment	3125

A.7.3.1.2.2	Test Parameters	3125

A.7.3.1.2.3	Test Requirements	3128

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3128

A.7.3.1.3.1	Test Purpose and Environment	3128

A.7.3.1.3.2	Test Parameters	3128

A.7.3.1.3.3	Test Requirements	3129

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3130

A.7.3.1.4.1	Test Purpose and Environment	3130

A.7.3.1.4.2	Test Parameters	3130

A.7.3.1.4.3 Test Requirements	3137

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3137

A.7.3.1.5.1	Test Purpose and Environment	3137

A.7.3.1.5.2	Test Parameters	3137

A.7.3.1.5.3 Test Requirements	3144

A.7.3.1.6	Handover with PSCell from SA to EN-DC with; unknown FR2 target PScell	3144

A.7.3.1.6.1	Test Purpose and Environment	3144

A.7.3.1.6.2	Test Parameters	3144

A.7.3.1.6.3	Test Requirements	3153

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3154

A.7.3.1.7.1	Test purpose and environment	3154

A.7.3.1.7.2	Test Requirements	3159

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3160

A.7.3.1.8.1	Test Purpose and Environment	3160

A.7.3.1.8.2	Test Requirements	3165

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3165

A.7.3.1.9.1	Test Purpose and Environment	3165

A.7.3.1.9.2	Test Parameters	3165

A.7.3.1.9.3	Test Requirements	3168

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3168

A.7.3.1.10.1	Test Purpose and Environment	3168

A.7.3.1.10.2	Test Parameters	3168

A.7.3.1.10.3	Test Requirements	3172

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3172

A.7.3.1.11.1	Test Purpose and Environment	3172

A.7.3.1.11.2	Test Parameters	3172

A.7.3.1.11.3	Test Requirements	3176

A.7.3.2	RRC Connection Mobility Control	3176

A.7.3.2.1	SA: RRC Re-establishment	3176

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3176

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3179

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3182

A.7.3.2.1.3.1	Test Purpose and Environment	3182

A.7.3.2.1.3.2	Test Requirements	3184

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3185

A.7.3.2.1.4.1	Test Purpose and Environment	3185

A.7.3.2.1.4.2	Test Requirements	3187

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3188

A.7.3.2.1.5.1	Test Purpose and Environment	3188

A.7.3.2.1.5.2	Test Requirements	3191

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3191

A.7.3.2.1.6.1	Test Purpose and Environment	3191

A.7.3.2.1.6.2	Test Requirements	3193

A.7.3.2.2	Random Access	3194

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3194

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3198

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3202

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3205

A.7.3.2.3	SA: RRC Connection Release with Redirection	3208

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3208

A.7.3.3 Conditional Handover	3212

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3212

A.7.3.3.1.1	Test Purpose and Environment	3212

A.7.3.3.1.2	Test Parameters	3212

A.7.3.3.1.2.3	Test Requirements	3215

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3215

A.7.3.3.2.1	Test Purpose and Environment	3215

A.7.3.3.2.2	Test Parameters	3215

A.7.3.3.2.3 Test Requirements	3217

A.7.4	Timing	3217

A.7.4.1	UE transmit timing	3217

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3217

A.7.4.1.1.1	Test Purpose and environment	3217

A.7.4.1.1.2	Test requirements	3220

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3220

A.7.4.1.2.1	Test Purpose and environment	3220

A.7.4.1.2.2	Test requirements	3225

A.7.4.2	UE timer accuracy	3226

A.7.4.3	Timing advance	3226

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3226

A.7.4.3.1.1	Test Purpose and Environment	3226

A.7.4.3.1.2	Test Parameters	3226

A.7.4.3.1.3 Test Requirements	3229

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3230

A.7.4.3.2.1	Test Purpose and Environment	3230

A.7.4.3.2.2	Test Parameters	3230

A.7.4.3.2.3	Test Requirements	3234

A.7.5	Signaling characteristics	3234

A.7.5.1	Radio link Monitoring	3234

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3234

A.7.5.1.1.1	Test Purpose and Environment	3234

A.7.5.1.1.2	Test Requirements	3237

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3238

A.7.5.1.2.1	Test Purpose and Environment	3238

A.7.5.1.2.2	Test Requirements	3242

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3243

A.7.5.1.3.1	Test Purpose and Environment	3243

A.7.5.1.3.2	Test Requirements	3247

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3247

A.7.5.1.4.1	Test Purpose and Environment	3247

A.7.5.1.4.2	Test Requirements	3252

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3252

A.7.5.1.5.1	Test Purpose and Environment	3252

A.7.5.1.5.2	Test Requirements	3257

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3257

A.7.5.1.6.1	Test Purpose and Environment	3257

A.7.5.1.6.2	Test Requirements	3261

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3261

A.7.5.1.7.1	Test Purpose and Environment	3261

A.7.5.1.7.2	Test Requirements	3265

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3265

A.7.5.1.8.1	Test Purpose and Environment	3265

A.7.5.1.8.2	Test Requirements	3270

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3270

A.7.5.1.9.1	Test Purpose and Environment	3270

A.7.5.1.9.2	Test Requirements	3273

A.7.5.2	Interruption	3273

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3273

A.7.5.2.1.1	Test Purpose and Environment	3273

A.7.5.2.1.2	Test Requirements	3276

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3277

A.7.5.2.2.1	Test Purpose and Environment	3277

A.7.5.2.2.2	Test Parameters	3277

A.7.5.2.2.3	Test Requirements	3279

A.7.5.3	SCell Activation and Deactivation Delay	3279

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3279

A.7.5.3.1.1	Test Purpose and Environment	3279

A.7.5.3.1.2	Test Requirements	3281

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3281

A.7.5.3.2.1	Test Purpose and Environment	3281

A.7.5.3.2.2	Test Requirements	3285

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3286

A.7.5.3.3.1	Test Purpose and Environment	3286

A.7.5.3.3.2	Test Requirements	3289

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3290

A.7.5.3.4.1	Test Purpose and Environment	3290

A.7.5.3.4.2	Test Requirements	3293

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3294

A.7.5.3.5.1	Test Purpose and Environment	3294

A.7.5.3.5.2	Test Requirements	3297

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3298

A.7.5.3.6.1	Test Purpose and Environment	3298

A.7.5.3.6.2	Test Requirements	3302

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3303

A.7.5.3.7.1	Test Purpose and Environment	3303

A.7.5.3.7.2	Test Requirements	3307

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3308

A.7.5.3.8.1	Test Purpose and Environment	3308

A.7.5.3.8.2	Test Requirements	3312

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3313

A.7.5.3.9.1	Test Purpose and Environment	3313

A.7.5.3.9.2	Test Requirements	3316

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3317

A.7.5.3.10.1	Test Purpose and Environment	3317

A.7.5.3.10.2	Test Requirements	3321

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3322

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3322

A.7.5.3.11.1.1	Test Purpose and Environment	3322

A.7.5.3.11.1.2	Test Requirements	3326

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3327

A.7.5.3.11.2.1	Test Purpose and Environment	3327

A.7.5.3.11.2.2	Test Requirements	3331

A.7.5.3.12	Void	3332

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3332

A.7.5.3.13.1	Test Purpose and Environment	3332

A.7.5.3.13.2	Test Requirements	3334

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3335

A.7.5.3.14.1	Test Purpose and Environment	3335

A.7.5.3.14.2	Test Requirements	3338

A.7.5.15	Void	3339

A.7.5.4	Void	3339

A.7.5.5	Beam Failure Detection and Link recovery procedures	3339

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3339

A.7.5.5.1.1	Test Purpose and Environment	3339

A.7.5.5.1.2	Test Requirements	3343

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3344

A.7.5.5.2.1	Test Purpose and Environment	3344

A.7.5.5.2.2	Test Requirements	3347

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3348

A.7.5.5.3.1	Test Purpose and Environment	3348

A.7.5.5.3.2	Test Requirements	3352

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3353

A.7.5.5.4.1	Test Purpose and Environment	3353

A.7.5.5.4.2	Test Requirements	3357

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3358

A.7.5.5.5.1	Test Purpose and Environment	3358

A.7.5.5.5.2	Test Requirements	3361

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3362

A.7.5.5.6.1	Test Purpose and Environment	3362

A.7.5.5.6.2	Test Requirements	3366

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3367

A.7.5.5.7.1	Test Purpose and Environment	3367

A.7.5.5.7.2	Test Requirements	3371

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3372

A.7.5.5.8.1	Test Purpose and Environment	3372

A.7.5.5.8.2	Test Requirements	3376

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3376

A.7.5.5.9.1	Test Purpose and Environment	3376

A.7.5.5.9.2	Test Requirements	3382

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3382

A.7.5.5.10.1	Test Purpose and Environment	3382

A.7.5.5.10.2	Test Requirements	3388

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3388

A.7.5.5.11.1	Test Purpose and Environment	3388

A.7.5.5.11.2	Test Requirements	3393

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3393

A.7.5.5.12.1	Test Purpose and Environment	3393

A.7.5.5.12.2	Test Requirements	3398

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3398

A.7.5.5.13.1	Test Purpose and Environment	3398

A.7.5.5.13.2	Test Requirements	3402

A.7.5.6	Active BWP switch	3402

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3402

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3402

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3407

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3412

A.7.5.6.1.3.1	Test Purpose and Environment	3412

A.7.5.6.1.3.2	Test Requirements	3415

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3415

A.7.5.6.1.4.1	Test Purpose and Environment	3415

A.7.5.6.1.4.2	Test Requirements	3420

A.7.5.6.2	RRC-based Active BWP Switch	3421

A.7.5.6.2.1.1	Test Purpose and Environment	3421

A.7.5.6.2.1.2	Test Requirements	3424

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3425

A.7.5.6.2.2.1	Test Purpose and Environment	3425

A.7.5.6.2.2.2	Test Requirements	3429

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3430

A.7.5.6.3.1.1	Test Purpose and Environment	3430

A.7.5.6.3.1.2	Test Requirements	3433

A.7.5.6.4	SCell dormancy switch	3433

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3433

A.7.5.6.4.1.1	Test Purpose and Environment	3433

A.7.5.6.4.1.2	Test Requirements	3438

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3438

A.7.5.6.4.2.1	 Test Purpose and Environment	3438

A.7.5.6.4.2.2	 Test Requirements	3443

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3443

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3443

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3446

A.7.5.6.5.2.1	Test Purpose and Environment	3446

A.7.5.6.5.2.2	Test Requirements	3450

A.7.5.7	PSCell addition and release delay	3451

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3451

A.7.5.7.1.1	Test Purpose and Environment	3451

A.7.5.7.1.2	Test Requirements	3454

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3454

A.7.5.7.2.1	Test Purpose and Environment	3454

A.7.5.7.2.2	Test Requirements	3457

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3457

A.7.5.7.3.1	Test Purpose and Environment	3457

A.7.5.7.3.2	Test Requirements	3460

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3460

A.7.5.7.4.1	Test Purpose and Environment	3460

A.7.5.7.4.2	Test Requirements	3463

A.7.5.8	Active TCI state switch delay	3463

A.7.5.8.1	MAC-CE based active TCI state switch	3463

A.7.5.8.2	RRC based active TCI state switch	3467

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3471

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3471

A.7.5.8.3.1.1	Test Purpose and Environment	3471

A.7.5.8.3.1.2	Test Requirements	3475

A.7.5.9	Uplink spatial relation switch delay	3476

A.7.5.9.1.1.1	Test Purpose and Environment	3476

A.7.5.9.1.1.2	Test Requirements	3479

A.7.5.9.2	RRC based spatial relation switch	3479

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3479

A.7.5.9.2.1.1	Test Purpose and Environment	3479

A.7.5.9.2.1.2	Test Requirements	3482

A.7.5.10	UE specific CBW change	3482

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3482

A.7.5.10.1.1	Test Purpose and Environment	3482

A.7.5.10.1.2	Test Requirements	3485

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3486

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3486

A.7.5.11.1.1	Test Purpose and Environment	3486

A.7.5.11.1.2	Test Requirements	3489

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3489

A.7.5.12.1	Addition and Release Delay of PSCell	3489

A.7.5.12.1.1	Test purpose and environment	3489

A.7.5.12.1.2	Test Parameters	3489

A.7.5.12.1.3	Test Requirements	3492

A.7.5.13	 Unified TCI state switching delay	3492

A.7.5.13.1	MAC-CE based active joint TCI state switching	3492

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3492

A.7.5.13.1.1.1	Test Purpose and Environment	3492

A.7.5.13.1.1.2	Test parameters	3493

A.7.5.13.1.1.3	Test Requirements	3495

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3495

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3495

A.7.5.13.2.1.1	Test Purpose and Environment	3495

A.7.5.13.2.1.2	Test parameters	3496

A.7.5.13.2.1.3	Test Requirements	3498

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3498

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3498

A.7.5.13.3.1.1	Test Purpose and Environment	3498

A.7.5.13.3.1.2	Test Parameters	3499

A.7.5.13.3.1.3	Test Requirements	3502

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3502

A.7.5.14.1	Test Purpose and Environment	3502

A.7.5.14.2	Test Requirements	3505

A.7.6	Measurement procedure	3506

A.7.6.1	Intra-frequency Measurements	3506

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3506

A.7.6.1.1.1	Test purpose and Environment	3506

A.7.6.1.1.2	Test Requirements	3509

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3509

A.7.6.1.2.1	Test purpose and Environment	3509

A.7.6.1.2.2	Test Requirements	3512

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3513

A.7.6.1.3.1	Test purpose and Environment	3513

A.7.6.1.3.2	Test Requirements	3516

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3516

A.7.6.1.4.1	Test purpose and Environment	3516

A.7.6.1.4.2	Test Requirements	3519

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3520

A.7.6.1.5.1	Test purpose and Environment	3520

A.7.6.1.5.2	Test Requirements	3523

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3523

A.7.6.1.6.1	Test purpose and Environment	3523

A.7.6.1.6.2	Test Requirements	3526

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3527

A.7.6.1.7.1	Test purpose and Environment	3527

A.7.6.1.7.2	Test Requirements	3529

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3530

A.7.6.1.8.1	Test purpose and Environment	3530

A.7.6.1.8.2	Test Requirements	3533

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3534

A.7.6.1.9.1	Test purpose and Environment	3534

A.7.6.1.9.2	Test Requirements	3536

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3537

A.7.6.1.10.1	Test purpose and Environment	3537

A.7.6.1.10.2	Test Requirements	3541

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3541

A.7.6.1.11.1	Test purpose and Environment	3541

A.7.6.1.11.2	Test Requirements	3544

A.7.6.2	Inter-frequency Measurements	3544

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3544

A.7.6.2.1.1	Test Purpose and Environment	3544

A.7.6.2.1.2	Test Requirements	3548

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3548

A.7.6.2.2.1	Test Purpose and Environment	3548

A.7.6.2.2.2	Test Requirements	3552

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3552

A.7.6.2.3.1	Test Purpose and Environment	3552

A.7.6.2.3.2	Test Requirements	3556

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3556

A.7.6.2.4.1	Test Purpose and Environment	3556

A.7.6.2.4.2	Test Requirements	3560

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3560

A.7.6.2.5.1	Test Purpose and Environment	3560

A.7.6.2.5.2	Test Requirements	3564

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3565

A.7.6.2.6.1	Test Purpose and Environment	3565

A.7.6.2.6.2	Test Requirements	3569

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3570

A.7.6.2.7.1	Test Purpose and Environment	3570

A.7.6.2.7.2	Test Requirements	3574

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3575

A.7.6.2.8.1	Test Purpose and Environment	3575

A.7.6.2.8.2	Test Requirements	3579

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3580

A.7.6.2.9.1	Test Purpose and Environment	3580

A.7.6.2.9.2	Test Requirements	3584

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3584

A.7.6.2.10.1	Test Purpose and Environment	3584

A.7.6.2.10.2	Test Requirements	3586

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3587

A.7.6.2.11.1	Test Purpose and Environment	3587

A.7.6.2.11.2	Test Requirements	3589

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3590

A.7.6.2.12.1	Test Purpose and Environment	3590

A.7.6.2.12.2	Test Requirements	3593

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3594

A.7.6.2.13.1	Test Purpose and Environment	3594

A.7.6.2.13.2	Test Requirements	3598

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3599

A.7.6.2.14.1	Test Purpose and Environment	3599

A.7.6.2.14.2	Test Requirements	3603

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3604

A.7.6.2.15.1	Test Purpose and Environment	3604

A.7.6.2.15.2	Test Requirements	3608

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3609

A.7.6.2.16.1	Test Purpose and Environment	3609

A.7.6.2.16.2	Test Requirements	3615

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3615

A.7.6.2.17.1	Test Purpose and Environment	3615

A.7.6.2.17.2	Test Requirements	3622

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3623

A.7.6.2.18.1	Test Purpose and Environment	3623

A.7.6.2.18.2	Test Requirements	3629

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3629

A.7.6.2.19.1	Test Purpose and Environment	3629

A.7.6.2.19.2	Test Requirements	3637

A.7.6.3	L1-RSRP measurement for beam reporting	3638

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3638

A.7.6.3.1.1	Test Purpose and Environment	3638

A.7.6.3.1.2	Test parameters	3638

A.7.6.3.1.3	Test Requirements	3640

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3640

A.7.6.3.2.1	Test Purpose and Environment	3640

A.7.6.3.2.2	Test parameters	3641

A.7.6.3.2.3	Test Requirements	3643

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3643

A.7.6.3.3.1	Test Purpose and Environment	3643

A.7.6.3.3.2	Test parameters	3644

A.7.6.3.3.3	Test Requirements	3646

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3647

A.7.6.3.4.1	Test Purpose and Environment	3647

A.7.6.3.4.2	Test parameters	3647

A.7.6.3.3.3	Test Requirements	3649

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3650

A.7.6.3.5.1	Test Purpose and Environment	3650

A.7.6.3.5.2	Test parameters	3650

A.7.6.3.5.3	Test Requirements	3652

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3652

A.7.6.3.6.1	Test Purpose and Environment	3652

A.7.6.3.6.2	Test parameters	3653

A.7.6.3.6.3	Test Requirements	3656

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3656

A.7.6.3.7.1	Test Purpose and Environment	3656

A.7.6.3.7.2	Test parameters	3657

A.7.6.3.7.3	Test Requirements	3660

A.7.6.4	CLI measurements	3661

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3661

A.7.6.4.1.1	Test Purpose and Environment	3661

A.7.6.4.1.2	Test Parameters	3661

A.7.6.4.1.3	Test Requirements	3663

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3663

A.7.6.4.2.1	Test Purpose and Environment	3663

A.7.6.4.2.2	Test Parameters	3664

A.7.6.4.2.3	Test Requirements	3665

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3666

A.7.6.5.1.1	Test Purpose and Environment	3666

A.7.6.5.1.2	Test Requirements	3669

A.7.6.6	L1-SINR measurement for beam reporting	3669

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3671

A.7.6.6.2.1	Test Purpose and Environment	3671

A.7.6.6.2.2	Test parameters	3672

A.7.6.6.2.3	Test Requirements	3674

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3674

A.7.6.6.3.1	Test Purpose and Environment	3674

A.7.6.6.3.2	Test parameters	3675

A.7.6.6.3.3	Test Requirements	3677

A.7.6.7	CSI-RS based intra-frequency Measurements	3677

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3677

A.7.6.7.1.1	Test purpose and Environment	3677

A.7.6.7.1.2	Test Requirements	3680

A.7.6.8	CSI-RS based inter-frequency Measurements	3681

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3681

A.7.6.8.1.1	Test Purpose and Environment	3681

A.7.6.8.1.2	Test Requirements	3684

A.7.6.9	RSTD measurements	3684

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3684

A.7.6.9.1.1	Test Purpose and Environment	3684

A.7.6.9.1.2	Test Requirements	3692

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3692

A.7.6.9.2.1	Test Purpose and Environment	3692

A.7.6.9.2.2	Test Requirements	3700

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3700

A.7.6.9.3.1	Test Purpose and Environment	3700

A.7.6.9.3.2	Test Requirements	3706

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3707

A.7.6.9.4.1	Test Purpose and Environment	3707

A.7.6.9.4.2	Test Requirements	3710

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3711

A.7.6.9.5.1	Test Purpose and Environment	3711

A.7.6.9.5.2	Test Requirements	3714

A.7.6.10 PRS-RSRP measurements	3715

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3715

A.7.6.10.1.1	Test Purpose and Environment	3715

A.7.6.10.1.2	Test Requirements	3719

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3719

A.7.6.10.2.1	Test Purpose and Environment	3719

A.7.6.10.2.2	Test Requirements	3723

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3723

A.7.6.10.3.1	Test Purpose and Environment	3723

A.7.6.10.3.2	Test Requirements	3728

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3728

A.7.6.10.4.1	Test Purpose and Environment	3728

A.7.6.10.4.2	Test Requirements	3733

A.7.6.11	UE Rx-Tx time difference measurements	3733

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3733

A.7.6.11.1.1	Test purpose and environment	3733

A.7.6.11.1.2	Test requirements	3737

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3737

A.7.6.11.2.1	Test purpose and environment	3737

A.7.6.11.2.2	Test requirements	3741

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3741

A.7.6.11.3.1	Test purpose and environment	3741

A.7.6.11.3.2	Test requirements	3743

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3744

A.7.6.11.4.1	Test purpose and environment	3744

A.7.6.11.4.2	Test requirements	3746

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3747

A.7.6.11.5.1	Test purpose and environment	3747

A.7.6.11.5.2	Test requirements	3749

A.7.6.12	PRS-RSRPP measurements	3750

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3750

A.7.6.12.1.1	Test Purpose and Environment	3750

A.7.6.12.1.2	Test Requirements	3753

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3753

A.7.6.12.2.1	Test Purpose and Environment	3753

A.7.6.12.2.2	Test Requirements	3756

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3756

A.7.6.12.3.1	Test Purpose and Environment	3756

A.7.6.12.3.2	Test Requirements	3759

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3760

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3760

A.7.6.13.1.1	Test purpose and environment	3760

A.7.6.13.1.2	Test requirements	3761

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3762

A.7.6.13.2.1	Test purpose and environment	3762

A.7.6.13.2.2	Test requirements	3763

A.7.6.14	SA event triggered reporting tests with Pre-MG	3764

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3764

A.7.6.14.1.1	Test purpose and Environment	3764

A.7.6.14.1.2	Test parameters	3764

A.7.6.14.1.3	Test Requirements	3767

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3767

A.7.6.14.2.1	Test purpose and Environment	3767

A.7.6.14.2.2	Test parameters	3767

A.7.6.14.2.3	Test Requirements	3770

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3770

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3770

A.7.6.15.1.1	Test Purpose and Environment	3770

A.7.6.15.1.2	Test Requirements	3773

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3773

A.7.6.15.2.1	Test Purpose and Environment	3773

A.7.6.15.2.2	Test Requirements	3776

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	3776

A.7.6.15.3.1	Test Purpose and Environment	3776

A.7.6.15.3.2	Test Requirements	3780

A.7.6.16	SA event triggered reporting tests with NCSG	3780

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3780

A.7.6.16.1.1	Test purpose and Environment	3780

A.7.6.16.1.2	Test Requirements	3784

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3784

A.7.6.16.2.1	Test Purpose and Environment	3784

A.7.6.16.2.2	Test Requirements	3787

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3787

A.7.6.16.3.1	Test Purpose and Environment	3787

A.7.6.16.3.2	Test Requirements	3790

A.7.7	Measurement Performance requirements	3790

A.7.7.1	SS-RSRP	3791

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3791

A.7.7.1.1.1	Test Purpose and Environment	3791

A.7.7.1.1.2	Test parameters	3791

A.7.7.1.1.3	Test Requirements	3795

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3796

A.7.7.1.2.1	Test Purpose and Environment	3796

A.7.7.1.2.2	Test parameters	3796

A.7.7.1.2.3	Test Requirements	3800

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3801

A.7.7.1.3.1	Test Purpose and Environment	3801

A.7.7.1.3.2	Test parameters	3801

A.7.7.1.3.3	Test Requirements	3805

A.7.7.2	SS-RSRQ	3805

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3805

A.7.7.2.1.1	Test Purpose and Environment	3805

A.7.7.2.1.2	Test Parameters	3805

A.7.7.2.1.3	Test Requirements	3807

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3807

A.7.7.2.2.1	Test Purpose and Environment	3807

A.7.7.2.2.2	Test Parameters	3807

A.7.7.2.2.3	Test Requirements	3809

A.7.7.3	SS-SINR	3809

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3809

A.7.7.3.1.1	Test Purpose and Environment	3809

A.7.7.3.1.2	Test Parameters	3809

A.7.7.3.1.3	Test Requirements	3811

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3811

A.7.7.3.2.1	Test Purpose and Environment	3811

A.7.7.3.2.2	Test Parameters	3811

A.7.7.3.2.3	Test Requirements	3813

A.7.7.4	L1-RSRP measurement for beam reporting	3813

A.7.7.4.1	SSB based L1-RSRP measurement	3813

A.7.7.4.1.1	Test Purpose and Environment	3813

A.7.7.4.1.2	Test parameters	3814

A.7.7.4.1.3	Test Requirements	3816

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3817

A.7.7.4.2.1	Test Purpose and Environment	3817

A.7.7.4.2.2	Test parameters	3817

A.7.7.4.2.3	Test Requirements	3819

A.7.7.5	CLI measurements	3820

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3820

A.7.7.5.1.1	Test Purpose and Environment	3820

A.7.7.5.1.2	Test parameters	3820

A.7.7.5.1.3	Test Requirements	3823

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3824

A.7.7.5.2.1	Test Purpose and Environment	3824

A.7.7.5.2.2	Test parameters	3824

A.7.7.5.2.3	Test Requirements	3826

A.7.7.6	L1-SINR measurement for beam reporting	3827

A.7.7.6.1.1	Test Purpose and Environment	3827

A.7.7.6.1.2	Test parameters	3827

A.7.7.6.1.3	Test Requirements	3829

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3830

A.7.7.6.2.1	Test Purpose and Environment	3830

A.7.7.6.2.2	Test parameters	3830

A.7.7.6.2.3	Test Requirements	3832

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3833

A.7.7.6.3.1	Test Purpose and Environment	3833

A.7.7.6.3.2	Test parameters	3833

A.7.7.6.3.3	Test Requirements	3835

A.7.7.7	CSI-RSRP	3836

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3836

A.7.7.7.1.1	Test Purpose and Environment	3836

A.7.7.7.1.2	Test parameters	3836

A.7.7.7.1.3	Test Requirements	3841

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3841

A.7.7.7.2.1	Test Purpose and Environment	3841

A.7.7.7.2.2	Test parameters	3842

A.7.7.7.2.3	Test Requirements	3846

A.7.7.8	CSI-RSRQ	3847

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3847

A.7.7.8.1.1	Test Purpose and Environment	3847

A.7.7.8.1.2	Test Parameters	3847

A.7.7.8.1.3	Test Requirements	3849

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3849

A.7.7.8.2.1	Test Purpose and Environment	3849

A.7.7.8.2.2	Test Parameters	3850

A.7.7.8.2.3	Test Requirements	3851

A.7.7.9	CSI-SINR	3851

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3851

A.7.7.9.1.1	Test Purpose and Environment	3851

A.7.7.9.1.2	Test Parameters	3852

A.7.7.9.1.3	Test Requirements	3854

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3855

A.7.7.9.2.1	Test Purpose and Environment	3855

A.7.7.9.2.2	Test Parameters	3855

A.7.7.9.2.3	Test Requirements	3857

A.7.7.10	RSTD measurements	3858

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3858

A.7.7.10.1.1	Test purpose and Environment	3858

A.7.7.10.1.2	Test Requirements	3860

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3860

A.7.7.10.2.1	Test purpose and Environment	3860

A.7.7.10.2.2	Test Requirements	3864

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3865

A.7.7.10.3.1	Test purpose and Environment	3865

A.7.7.10.3.2	Test Requirements	3866

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3867

A.7.7.10.4.1	Test purpose and Environment	3867

A.7.7.10.4.2	Test Requirements	3869

A.7.7.11	PRS-RSRP measurements	3869

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3869

A.7.7.11.1.1	Test Purpose and Environment	3869

A.7.7.11.1.2	Test parameters	3869

A.7.7.11.1.3	Test Requirements	3873

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3874

A.7.7.11.2.1	Test Purpose and Environment	3874

A.7.7.11.2.2	Test parameters	3874

A.7.7.11.2.3	Test Requirements	3876

A.7.7.12	UE Rx-Tx time difference measurements	3877

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3877

A.7.7.12.1.1	Test purpose and environment	3877

A.7.7.12.1.2	Test parameters	3877

A.7.7.12.1.3	Test requirements	3880

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3881

A.7.7.12.2.1	Test purpose and environment	3881

A.7.7.12.2.2	Test parameters	3881

A.7.7.12.2.3	Test requirements	3884

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3884

A.7.7.12.3.1	Test purpose and environment	3884

A.7.7.12.3.2	Test parameters	3884

A.7.7.12.3.3	Test requirements	3887

A.7.7.13	PRS-RSRPP measurements	3887

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3887

A.7.7.13.1.1	Test Purpose and Environment	3887

A.7.7.13.1.2	Test parameters	3888

A.7.7.13.1.3	Test Requirements	3890

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3891

A.7.7.13.2.1	Test Purpose and Environment	3891

A.7.7.13.2.2	Test parameters	3891

A.7.7.13.2.3	Test Requirements	3893

A.7.8	Measurement procedure in RRC_INACTIVE	3894

A.7.8.1	RSTD measurements	3894

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3894

A.7.8.1.1.1	Test Purpose and Environment	3894

A.7.8.1.1.2	Test Requirements	3897

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3897

A.7.8.1.2.1	Test Purpose and Environment	3897

A.7.8.1.2.2	Test Requirements	3903

A.7.8.2	PRS-RSRP measurements	3904

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3904

A.7.8.2.1.1	Test Purpose and Environment	3904

A.7.8.2.1.2	Test Requirements	3908

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3908

A.7.8.2.2.1	Test purpose and Environment	3908

A.7.8.2.2.2	Test Requirements	3912

A.7.8.3	UE Rx-Tx time difference measurements	3912

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3912

A.7.8.3.1.1	Test purpose and environment	3912

A.7.8.3.1.2	Test requirements	3916

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3916

A.7.8.3.2.1	Test purpose and environment	3916

A.7.8.3.2.2	Test requirements	3918

A.7.8.4	PRS-RSRPP measurements	3919

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3919

A.7.8.4.1.1	Test Purpose and Environment	3919

A.7.8.4.1.2	Test Requirements	3921

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3922

A.7.8.4.2.1	Test Purpose and Environment	3922

A.7.8.4.2.2	Test Requirements	3924

A.7.9	Measurement performance requirements in RRC_INACTIVE	3925

A.7.9.1	RSTD measurements	3925

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3925

A.7.9.1.1.1	Test purpose and Environment	3925

A.7.9.1.1.2	Test Requirements	3927

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3927

A.7.9.1.2.1	Test purpose and Environment	3927

A.7.9.1.2.2	Test Requirements	3929

A.7.9.2	PRS-RSRP measurements	3929

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	3929

A.7.9.2.1.1	Test Purpose and Environment	3929

A.7.9.2.1.2	Test parameters	3929

A.7.9.2.1.3	Test Requirements	3931

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	3932

A.7.9.2.2.1	Test Purpose and Environment	3932

A.7.9.2.2.2	Test parameters	3932

A.7.9.2.2.3	Test Requirements	3934

A.7.9.3	UE Rx-Tx time difference measurements	3935

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	3935

A.7.9.3.1.1	Test purpose and environment	3935

A.7.9.3.1.2	Test parameters	3935

A.7.9.3.1.3	Test requirements	3938

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3938

A.7.9.3.2.1	Test purpose and environment	3938

A.7.9.3.2.2	Test parameters	3938

A.7.9.3.2.3	Test requirements	3941

A.7.9.4	PRS-RSRPP measurements	3941

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	3941

A.7.9.4.1.1	Test Purpose and Environment	3941

A.7.9.4.1.2	Test parameters	3941

A.7.9.4.1.3	Test Requirements	3943

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	3944

A.7.9.4.2.1	Test Purpose and Environment	3944

A.7.9.4.2.2	Test parameters	3944

A.7.9.4.2.3	Test Requirements	3946

A.8	E-UTRA standalone tests for NR RRM	3947

A.8.1	Void	3947

A.8.2	RRC_IDLE state mobility	3947

A.8.2.1	Inter-RAT NR Cell re-selection	3947

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	3947

A.8.2.1.1.1	Test Purpose and Environment	3947

A.8.2.1.1.2	Test Requirements	3952

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3953

A.8.2.1.2.1	Test Purpose and Environment	3953

A.8.2.1.2.2	Test Requirements	3957

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	3958

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	3958

A.8.2.2.1.1	Test Purpose and Environment	3958

A.8.2.2.1.2	Test Requirements	3962

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	3962

A.8.2.2.2.1	Test Purpose and Environment	3962

A.8.2.2.2.2	Test Requirements	3965

A.8.3	RRC_CONNECTED state mobility	3966

A.8.3.1	Handover	3966

A.8.3.1.1	E-UTRAN - NR handover in FR1	3966

A.8.3.1.1.1	Test Purpose and Environment	3966

A.8.3.1.1.2	Test Requirements	3971

A.8.4	Measurement procedure	3971

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	3971

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	3971

A.8.4.1.1.1	Test Purpose and Environment	3971

A.8.4.1.1.2	Test Requirements	3973

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	3974

A.8.4.1.2.1	Test Purpose and Environment	3974

A.8.4.1.2.2	Test Requirements	3975

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	3975

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3975

A.8.4.2.1.1	Test Purpose and Environment	3975

A.8.4.2.1.2	Test Requirements	3980

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3980

A.8.4.2.2.1	Test Purpose and Environment	3980

A.8.4.2.2.2	Test Requirements	3984

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3984

A.8.4.2.3.1	Test Purpose and Environment	3984

A.8.4.2.3.2	Test Requirements	3988

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3988

A.8.4.2.4.1	Test Purpose and Environment	3988

A.8.4.2.4.2	Test Requirements	3992

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3992

A.8.4.2.5.1	Test Purpose and Environment	3992

A.8.4.2.5.2	Test Requirements	3994

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3995

A.8.4.2.6.1	Test Purpose and Environment	3995

A.8.4.2.6.2	Test Requirements	3997

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3998

A.8.4.2.7.1	Test Purpose and Environment	3998

A.8.4.2.7.2	Test Requirements	4000

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4001

A.8.4.2.8.1	Test Purpose and Environment	4001

A.8.4.2.8.2	Test Requirements	4003

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	4004

A.8.4.2.9.1	Test Purpose and Environment	4004

A.8.4.2.9.2	Test Requirements	4009

A.8.5	Measurement performance	4009

A.8.5.1	SFTD accuracy	4009

A.8.5.1.1	SFTD accuracy	4009

A.8.5.1.1.1	Test Purpose	4009

A.8.5.1.1.2	Test Environment	4009

A.8.5.1.1.3	Test Requirements	4015

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	4016

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4016

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4020

A.8.5.2.1.2.1	Test Purpose and Environment	4020

A.8.5.2.1.2.2	Test Parameters	4020

A.8.5.2.1.2.3	Test Requirements	4022

A.8.5.2.2	SS-RSRQ	4022

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4022

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4027

A.8.5.2.2.2.1	Test Purpose and Environment	4027

A.8.5.2.2.2.2	Test Parameters	4028

A.8.5.2.2.2.3	Test Requirements	4029

A.8.5.2.3	SS-SINR	4030

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4030

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4033

A.8.5.2.3.2.1	Test Purpose and Environment	4033

A.8.5.2.3.2.2	Test Parameters	4033

A.8.5.2.3.2.3	Test Requirements	4035

A.9	V2X Tests	4036

A.9.1	V2X Tests in FR1	4036

A.9.1.1	Test for V2X UE Transmit Timing	4036

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	4036

A.9.1.1.1.1	Test Purpose and Environment	4036

A.9.1.1.1.2	Test requirements	4036

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	4036

A.9.1.1.2.1	Test Purpose and Environment	4036

A.9.1.1.2.2	Test requirements	4037

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4037

A.9.1.1.3.1	Test Purpose and Environment	4037

A.9.1.1.3.2	Test requirements	4040

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4040

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4040

A.9.1.2.1.1	Test Purpose and Environment	4040

A.9.1.2.1.2	Test Requirements	4043

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4043

A.9.1.2.2.1	Test Purpose and Environment	4043

A.9.1.2.2.2	Test Requirements	4045

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4045

A.9.1.2.3.1	Test Purpose and Environment	4045

A.9.1.2.3.2	Test Requirements	4047

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4047

A.9.1.3.1	 Test for GNSS configured as the highest priority	4047

A.9.1.3.1.1	Test Purpose and Environment	4047

A.9.1.3.1.2	Test Requirements	4049

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4050

A.9.1.3.2.1	Test Purpose and Environment	4050

A.9.1.3.2.2	Test Requirements	4052

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4053

A.9.1.3.3.1	Test Purpose and Environment	4053

A.9.1.3.3.2	Test Requirements	4055

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4056

A.9.1.3.4.1	Test Purpose and Environment	4056

A.9.1.3.4.2	Test Requirements	4058

A.9.1.4	Test for L1 SL-RSRP Measurement	4059

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4059

A.9.1.4.1.1	Test Purpose and Environment	4059

A.9.1.4.1.2	Test Requirements	4062

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4062

A.9.1.4.2.1	Test Purpose and Environment	4062

A.9.1.4.2.2	Test Requirements	4066

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4066

A.9.1.4.3.1	Test Purpose and Environment	4066

A.9.1.4.3.2	Test Requirements	4073

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4073

A.9.1.4.4.1	Test Purpose and Environment	4073

A.9.1.4.4.2	Test Requirements	4077

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4078

A.9.1.4.5.1	Test Purpose and Environment	4078

A.9.1.4.5.2	Test Requirements	4080

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4080

A.9.1.4.6.1	Test Purpose and Environment	4080

A.9.1.4.6.2	Test Requirements	4084

A.9.1.5	Test for Congestion Control Measurement	4084

A.9.1.5.1	Test Purpose and Environment	4084

A.9.1.5.2	Test Requirements	4090

A.9.1.6	Test for Interruption	4090

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4090

A.9.1.6.1.1	Test Purpose and Environment	4090

A.9.1.6.1.2	Test Requirements	4094

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4094

A.9.1.6.2.1	Test Purpose and Environment	4094

A.9.1.6.2.2	Test Requirements	4098

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4098

A.9.1.6.3.1	Test Purpose and Environment	4098

A.9.1.6.3.2	Test Requirements	4103

A.9.1.7	Selection / Reselection of relay UE	4103

A.9.1.7.1	Test Purpose and Environment	4103

A.9.1.7.2	Test Requirements	4109

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4110

A.10.1	RRC_CONNECTED state mobility	4110

A.10.1.1	RRC connection mobility control	4110

A.10.1.1.1	Random Access	4110

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4110

A.10.1.1.1.1.1	Test Purpose and Environment	4110

A.10.1.1.1.1.2	Test Requirements	4113

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4113

A.10.1.1.1.1.2.2	Random Access Response Reception	4113

A.10.1.1.1.1.2.3	No Random Access Response Reception	4114

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4114

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4114

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4115

A.10.1.1.1.2.1	Test Purpose and Environment	4115

A.10.1.1.1.2.2	Test Requirements	4118

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4118

A.10.1.1.1.2.2.2	Random Access Response Reception	4119

A.10.1.1.1.2.2.3	No Random Access Response Reception	4119

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4119

A.10.1.1.1.3.1	Test Purpose and Environment	4119

A.10.1.1.1.3.2	Test Requirements	4123

A.10.1.1.1.3.2.1	MsgA Transmission	4123

A.10.1.1.1.3.2.2	MsgB Reception	4123

A.10.1.1.1.3.2.3	No MsgB Reception	4124

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4124

A.10.1.1.1.4.1	Test Purpose and Environment	4124

A.10.1.1.1.4.2	Test Requirements	4128

A.10.1.1.1.4.2.1	MsgA Transmission	4128

A.10.1.1.1.4.2.2	MsgB Reception	4129

A.10.1.1.1.4.2.3	No MsgB Reception	4129

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4129

A.10.1.2.1	Test Purpose and Environment	4129

A.10.1.2.2	Test Requirements	4136

A.10.2	Timing	4137

A.10.2.1	UE transmit timing	4137

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4137

A.10.2.1.1.1	Test Purpose and environment	4137

A.10.2.1.1.2	Test requirements	4140

A.10.2.2	UE timing advance	4141

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4141

A.10.2.2.1.1	Test Purpose and Environment	4141

A.10.2.2.1.2	Test Parameters	4141

A.10.2.2.1.3	Test Requirements	4145

A.10.3	Signalling characteristics	4145

A.10.3.1	Radio link monitoring	4145

A.10.3.1.1	Introduction	4145

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4146

A.10.3.1.2.1	Test purpose and environment	4146

A.10.3.1.2.2	Test requirements	4150

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4151

A.10.3.1.3.1	Test purpose and environment	4151

A.10.3.1.3.2	Test requirements	4156

A.10.3.1.4	Void	4156

A.10.3.1.4.1	Void	4156

A.10.3.1.4.2	Void	4156

A.10.3.1.5	Void	4156

A.10.3.1.5.1	Void	4156

A.10.3.1.5.2	Void	4156

A.10.3.2	Void	4156

A.10.3.3	SCell activation and deactivation delay	4156

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4156

A.10.3.3.1.1	Test Purpose and Environment	4156

A.10.3.3.1.2	Test Requirements	4161

A.10.3.3.2	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4161

A.10.3.3.2.1	Test Purpose and Environment	4161

A.10.3.3.2.2	Test Requirements	4162

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4162

A.10.3.3.3.1	Test Purpose and Environment	4162

A.10.3.3.3.2	Test Requirements	4163

A.10.3.4	Beam failure detection and link recovery procedures	4163

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4163

A.10.3.4.1.1	Test Purpose and Environment	4163

A.10.3.4.1.2	Test Requirements	4168

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4168

A.10.3.4.2.1	Test Purpose and Environment	4168

A.10.3.4.2.2	Test Requirements	4174

A.10.3.5	Active BWP switching	4174

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4174

A.10.3.5.1.2	Test Requirements	4179

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4180

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4180

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4183

A.10.3.5.3	RRC-based Active BWP Switch	4187

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4187

A.10.3.6	PSCell addition and release delay	4191

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4191

A.10.3.6.1.1	Test purpose and environment	4191

A.10.3.6.1.2	Test Requirements	4196

A.10.3.7	Void	4197

A.10.4	Measurement procedure	4197

A.10.4.1	Intra-frequency measurements	4197

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4197

A.10.4.1.1.1	Test purpose and environment	4197

A.10.4.1.1.2	Test parameters	4197

A.10.4.1.1.3	Test Requirements	4200

A.10.4.1.2	Void	4200

A.10.4.1.3	Void	4200

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4200

A.10.4.1.4.1	Test purpose and environment	4200

A.10.4.1.4.2	Test parameters	4201

A.10.4.1.4.3	Test Requirements	4205

A.10.4.1.5	Void	4206

A.10.4.1.6	Void	4206

A.10.4.1.7	Void	4206

A.10.4.1.8	Void	4206

A.10.4.1.9	Void	4206

A.10.4.1.10	Void	4206

A.10.4.1.11	Void	4206

A.10.4.1.12	Void	4206

A.10.4.2	Inter-frequency measurements	4206

A.10.4.2.1	Void	4206

A.10.4.2.2	Void	4206

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4206

A.10.4.2.3.1	Test Purpose and Environment	4206

A.10.4.2.3.2	Test Requirements	4210

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4211

A.10.4.2.4.1	Test Purpose and Environment	4211

A.10.4.2.4.2	Test Requirements	4216

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4216

A.10.4.2.5.1	Test Purpose and Environment	4216

A.10.4.2.5.2	Test Requirements	4220

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4221

A.10.4.2.6.1	Test Purpose and Environment	4221

A.10.4.2.6.2	Test Requirements	4226

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4226

A.10.4.2.7.1	Test Purpose and Environment	4226

A.10.4.2.7.2	Test Requirements	4232

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4232

A.10.4.2.8.1	Test Purpose and Environment	4232

A.10.4.2.8.2	Test Requirements	4238

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4238

A.10.4.2.9.1	Test Purpose and Environment	4238

A.10.4.2.9.2	Test Requirements	4244

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4244

A.10.4.2.10.1	Test Purpose and Environment	4244

A.10.4.2.10.2	Test Requirements	4250

A.10.4.3	L1-RSRP measurements for beam reporting	4251

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4251

A.10.4.3.1.1	Test Purpose and Environment	4251

A.10.4.3.1.2	Test parameters	4251

A.10.4.3.1.3	Test Requirements	4253

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4254

A.10.4.3.2.1	Test Purpose and Environment	4254

A.10.4.3.2.2	Test parameters	4254

A.10.4.3.2.3	Test Requirements	4256

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4257

A.10.4.3.3.1	Test Purpose and Environment	4257

A.10.4.3.3.2	Test parameters	4257

A.10.4.3.3.3	Test Requirements	4260

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4261

A.10.4.3.4.1	Test Purpose and Environment	4261

A.10.4.3.4.2	Test parameters	4261

A.10.4.3.4.3	Test Requirements	4264

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4265

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4265

A.10.4.4.1.1	Test Purpose and Environment	4265

A.10.4.4.1.2	Test Requirements	4272

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4272

A.10.4.4.2.1	Test Purpose and Environment	4272

A.10.4.4.2.2	Test Requirements	4279

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4279

A.10.4.4.3.1	Test Purpose and Environment	4279

A.10.4.4.3.2	Test Requirements	4287

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4287

A.10.4.4.4.1	Test Purpose and Environment	4287

A.10.4.4.4.2	Test Requirements	4295

A.10.5	Measurement performance	4295

A.10.5.1	SS-RSRP	4295

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4295

A.10.5.1.1.1	Test Purpose and Environment	4295

A.10.5.1.1.2	Test parameters	4295

A.10.5.1.1.3	Test Requirements	4298

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4298

A.10.5.1.2.1	Test Purpose and Environment	4298

A.10.5.1.2.2	Test parameters	4298

A.10.5.1.2.3	Test Requirements	4301

A.10.5.2	SS-RSRQ	4301

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4301

A.10.5.2.1.1	Test Purpose and Environment	4301

A.10.5.2.1.2	Test Parameters	4301

A.10.5.2.1.3	Test Requirements	4304

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4304

A.10.5.2.2.1	Test Purpose and Environment	4304

A.10.5.2.2.2	Test Parameters	4304

A.10.5.2.2.3	Test Requirements	4307

A.10.5.3	SS-SINR	4307

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4307

A.10.5.3.1.1	Test Purpose and Environment	4307

A.10.5.3.1.2	Test Parameters	4307

A.10.5.3.1.3	Test Requirements	4310

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4310

A.10.5.3.2.1	Test Purpose and Environment	4310

A.10.5.3.2.2	Test Parameters	4310

A.10.5.3.2.3	Test Requirements	4313

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4313

A.10.5.3.3.1	Test Purpose and Environment	4313

A.10.5.3.3.2	Test Parameters	4313

A.10.5.3.3.3	Test Requirements	4316

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4316

A.10.5.4.1	SSB based L1-RSRP measurement	4316

A.10.5.4.1.1	Test Purpose and Environment	4316

A.10.5.4.1.2	Test parameters	4317

A.10.5.4.1.3	Test Requirements	4319

A.10.5.5	RSSI	4319

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4319

A.10.5.5.1.1	Test Purpose and Environment	4319

A.10.5.5.1.2	Test parameters	4319

A.10.5.5.1.3	Test Requirements	4322

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4322

A.10.5.5.2.1	Test Purpose and Environment	4322

A.10.5.5.2.2	Test parameters	4322

A.10.5.5.2.3	Test Requirements	4325

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4325

A.10.5.5.3.1	Test Purpose and Environment	4325

A.10.5.5.3.2	Test parameters	4325

A.10.5.5.3.3	Test Requirements	4329

A.10.5.6	Channel occupancy	4329

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4329

A.10.5.6.1.1	Test Purpose and Environment	4329

A.10.5.6.1.2	Test parameters	4329

A.10.5.6.1.3	Test Requirements	4333

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4333

A.10.5.6.2.1	Test Purpose and Environment	4333

A.10.5.6.2.2	Test parameters	4333

A.10.5.6.2.3	Test Requirements	4336

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4336

A.10.5.6.3.1	Test Purpose and Environment	4336

A.10.5.6.3.2	Test parameters	4336

A.10.5.6.3.3	Test Requirements	4340

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4344

A.11.1	RRC_IDLE state mobility	4344

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4344

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4344

A.11.1.1.1.1	Test Purpose and Environment	4344

A.11.1.1.1.2	Test Parameters	4344

A.11.1.1.1.3	Test Requirements	4347

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4347

A.11.1.1.2.1	Test Purpose and Environment	4347

A.11.1.1.2.2	Test Parameters	4347

A.11.1.1.2.3	Test Requirements	4351

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4351

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4351

A.11.1.2.1.1	Test Purpose and Environment	4351

A.11.1.2.1.2	Test Parameters	4352

A.11.1.2.1.3	Test Requirements	4357

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4358

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4358

A.11.1.3.1.1	Test Purpose and Environment	4358

A.11.1.3.1.2	Test Parameters	4358

A.11.1.3.1.3	Test Requirements	4364

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4365

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4365

A.11.1.4.1.1	Test Purpose and Environment	4365

A.11.1.4.1.2	Test Parameters	4365

A.11.1.4.1.3	Test Requirements	4368

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4369

A.11.1.4.2.1	Test Purpose and Environment	4369

A.11.1.4.2.2	Test Requirements	4372

A.11.2	RRC_CONNECTED state mobility	4373

A.11.2.1	Handover	4373

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4373

A.11.2.1.1.1	Test Purpose and Environment	4373

A.11.2.1.1.2	Test Parameters	4373

A.11.2.1.1.3 Test Requirements	4376

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4376

A.11.2.1.2.1	Test Purpose and Environment	4376

A.11.2.1.2.2	Test Parameters	4376

A.11.2.1.2.3	Test Requirements	4379

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4379

A.11.2.1.3.1	Test Purpose and Environment	4379

A.11.2.1.3.2	Test Parameters	4379

A.11.2.1.3.3 Test Requirements	4382

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4382

A.11.2.1.4.1	Test Purpose and Environment	4382

A.11.2.1.4.2	Test Parameters	4382

A.11.2.1.4.3 Test Requirements	4386

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4387

A.11.2.1.5.1	Test Purpose and Environment	4387

A.11.2.1.5.2	Test Parameters	4387

A.11.2.1.5.3 Test Requirements	4390

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4391

A.11.2.1.6.1	Test Purpose and Environment	4391

A.11.2.1.6.2	Test Parameters	4391

A.11.2.1.6.3	Test Requirements	4395

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4396

A.11.2.1.7.1	Test Purpose and Environment	4396

A.11.2.1.7.2	Test Requirements	4402

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4402

A.11.2.1.8.1	Test Purpose and Environment	4402

A.11.2.1.8.2	Test Requirements	4407

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4407

A.11.2.1.9.1	Test Purpose and Environment	4407

A.11.2.1.9.2	Test Requirements	4416

A.11.2.2	RRC connection mobility control	4417

A.11.2.2.1	RRC re-establishment	4417

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4417

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4420

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4427

A.11.2.2.2	Random Access	4433

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4433

A.11.2.2.2.1.1	Test Purpose and Environment	4433

A.11.2.2.2.1.2	Test Requirements	4436

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4436

A.11.2.2.2.1.2.2	Random Access Response Reception	4436

A.11.2.2.2.1.2.3	No Random Access Response Reception	4437

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4437

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4437

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4438

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4438

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4438

A.11.2.2.2.2.1	Test Purpose and Environment	4438

A.11.2.2.2.2.2	Test Requirements	4441

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4441

A.11.2.2.2.2.2.2	Random Access Response Reception	4442

A.11.2.2.2.2.2.3	No Random Access Response Reception	4442

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4442

A.11.2.2.2.3.1	Test Purpose and Environment	4442

A.11.2.2.2.3.2	Test Requirements	4446

A.11.2.2.2.3.2.1	MsgA Transmission	4446

A.11.2.2.2.3.2.2	MsgB Reception	4446

A.11.2.2.2.3.2.3	No MsgB Reception	4447

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4447

A.11.2.2.2.4.1	Test Purpose and Environment	4447

A.11.2.2.2.4.2	Test Requirements	4451

A.11.2.2.2.4.2.1	MsgA Transmission	4451

A.11.2.2.2.4.2.2	MsgB Reception	4452

A.11.2.2.2.4.2.3	No MsgB Reception	4452

A.11.2.2.3	RRC connection release with redirection	4453

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4453

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4457

A.11.3	Timing	4462

A.11.3.1	UE transmit timing	4462

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4462

A.11.3.1.1.1	Test Purpose and environment	4462

A.11.3.1.1.2	Test requirements	4465

A.11.3.2	UE timing advance	4466

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4466

A.11.3.2.1.1	Test Purpose and Environment	4466

A.11.3.2.1.2	Test Parameters	4466

A.11.3.2.1.3	Test Requirements	4470

A.11.4	Signalling characteristics	4470

A.11.4.1	Radio link monitoring	4470

A.11.4.1.1	Introduction	4470

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4471

A.11.4.1.2.1	Test purpose and environment	4471

A.11.4.1.2.2	Test requirements	4475

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4475

A.11.4.1.3.1	Test purpose and environment	4475

A.11.4.1.3.2	Test requirements	4481

A.11.4.1.4	Void	4482

A.11.4.1.4.1	Void	4482

A.11.4.1.4.2	Void	4482

A.11.4.1.5	Void	4482

A.11.4.1.5.1	Void	4482

A.11.4.1.5.2	Void	4482

A.11.4.2	Void	4482

A.11.4.3	SCell activation and deactivation delay	4482

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4482

A.11.4.3.1.1	Test Purpose and Environment	4482

A.11.4.3.1.2	Test Requirements	4486

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4486

A.11.4.3.2.1	Test Purpose and Environment	4486

A.11.4.3.2.2	Test Requirements	4487

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4487

A.11.4.3.3.1	Test Purpose and Environment	4487

A.11.4.3.3.2	Test Requirements	4488

A.11.4.4	Beam failure detection and link recovery procedures	4488

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4488

A.11.4.4.1.1	Test Purpose and Environment	4488

A.11.4.4.1.2	Test Requirements	4493

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4493

A.11.4.4.2.1	Test Purpose and Environment	4493

A.11.4.4.2.2	Test Requirements	4499

A.11.4.5	Active BWP switching	4499

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4499

A.11.4.5.1.1	Test Purpose and Environment	4499

A.11.4.5.1.2	Test Requirements	4503

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4504

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4504

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4507

A.11.4.5.3	RRC-based Active BWP Switch	4510

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4510

A.11.4.6	Void	4513

A.11.5	Measurement procedure	4513

A.11.5.1	Intra-frequency measurements	4513

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4513

A.11.5.1.1.1	Test purpose and environment	4513

A.11.5.1.1.2	Test parameters	4513

A.11.5.1.1.3	Test Requirements	4517

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4517

A.11.5.1.2.1	Test purpose and environment	4517

A.11.5.1.2.2	Test parameters	4517

A.11.5.1.2.3	Test Requirements	4520

A.11.5.1.3	Void	4521

A.11.5.1.4	Void	4521

A.11.5.1.5	Void	4521

A.11.5.1.6	Void	4521

A.11.5.1.7	Void	4521

A.11.5.1.8	Void	4521

A.11.5.1.9	Void	4521

A.11.5.1.10	Void	4521

A.11.5.1.11	Void	4521

A.11.5.1.12	Void	4521

A.11.5.2	Inter-frequency measurements	4521

A.11.5.2.1	Void	4521

A.11.5.2.2	Void	4521

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4521

A.11.5.2.3.1	Test Purpose and Environment	4521

A.11.5.2.3.2	Test Requirements	4525

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4526

A.11.5.2.4.1	Test Purpose and Environment	4526

A.11.5.2.4.2	Test Requirements	4531

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4531

A.11.5.2.5.1	Test Purpose and Environment	4531

A.11.5.2.5.2	Test Requirements	4535

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4536

A.11.5.2.6.1	Test Purpose and Environment	4536

A.11.5.2.6.2	Test Requirements	4541

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4541

A.11.5.2.7.1	Test Purpose and Environment	4541

A.11.5.2.7.2	Test Requirements	4545

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4546

A.11.5.2.8.1	Test Purpose and Environment	4546

A.11.5.2.8.2	Test Requirements	4551

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4551

A.11.5.2.9.1	Test Purpose and Environment	4551

A.11.5.2.9.2	Test Requirements	4555

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4556

A.11.5.2.10.1	Test Purpose and Environment	4556

A.11.5.2.10.2	Test Requirements	4561

A.11.5.3	Inter-RAT E-UTRAN measurements	4561

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4561

A.11.5.3.1.1	Test Purpose and Environment	4561

A.11.5.3.1.2	Test Requirements	4567

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4567

A.11.5.3.2.1	Test Purpose and Environment	4567

A.11.5.3.2.2	Test Requirements	4573

A.11.5.4	L1-RSRP measurements for beam reporting	4573

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4573

A.11.5.4.1.1	Test Purpose and Environment	4573

A.11.5.4.1.2	Test parameters	4573

A.11.5.4.1.3	Test Requirements	4575

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4575

A.11.5.4.2.1	Test Purpose and Environment	4575

A.11.5.4.2.2	Test parameters	4576

A.11.5.4.2.3	Test Requirements	4578

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4578

A.11.5.4.3.1	Test Purpose and Environment	4578

A.11.5.4.3.2	Test parameters	4579

A.11.5.4.3.3	Test Requirements	4582

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4583

A.11.5.4.4.1	Test Purpose and Environment	4583

A.11.5.4.4.2	Test parameters	4583

A.11.5.4.4.3	Test Requirements	4586

A.11.6	Measurement performance	4587

A.11.6.1	SS-RSRP	4587

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4587

A.11.6.1.1.1	Test Purpose and Environment	4587

A.11.6.1.1.2	Test parameters	4587

A.11.6.1.1.3	Test Requirements	4589

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4589

A.11.6.1.2.1	Test Purpose and Environment	4589

A.11.6.1.2.2	Test parameters	4589

A.11.6.1.2.3	Test Requirements	4592

A.11.6.2	SS-RSRQ	4592

A.11.6.2.1	Intra-frequency measurement accuracy	4592

A.11.6.2.1.1	Test Purpose and Environment	4592

A.11.6.2.1.2	Test Parameters	4592

A.11.6.2.1.3	Test Requirements	4595

A.11.6.2.2	Inter-frequency measurement accuracy	4595

A.11.6.2.2.1	Test Purpose and Environment	4595

A.11.6.2.2.2	Test Parameters	4595

A.11.6.2.2.3	Test Requirements	4598

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4598

A.11.6.2.3.1	Test Purpose and Environment	4598

A.11.6.2.3.2	Test Parameters	4598

A.11.6.2.3.3	Test Requirements	4601

A.11.6.2.4	Inter-frequency measurement accuracy	4601

A.11.6.2.4.1	Test Purpose and Environment	4601

A.11.6.2.4.2	Test Parameters	4601

A.11.6.2.4.3	Test Requirements	4607

A.11.6.3	SS-SINR	4607

A.11.6.3.1	Intra-frequency measurement accuracy	4607

A.11.6.3.1.1	Test Purpose and Environment	4607

A.11.6.3.1.2	Test Parameters	4607

A.11.6.3.1.3	Test Requirements	4610

A.11.6.3.2	Inter-frequency measurement accuracy	4610

A.11.6.3.2.1	Test Purpose and Environment	4610

A.11.6.3.2.2	Test Parameters	4610

A.11.6.3.2.3	Test Requirements	4613

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4613

A.11.6.3.3.1	Test Purpose and Environment	4613

A.11.6.3.3.2	Test Parameters	4613

A.11.6.3.3.3	Test Requirements	4616

A.11.6.3.4	Inter-frequency measurement accuracy	4616

A.11.6.3.4.1	Test Purpose and Environment	4616

A.11.6.3.4.2	Test Parameters	4616

A.11.6.3.4.3	Test Requirements	4624

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4624

A.11.6.4.1	SSB based L1-RSRP measurement	4624

A.11.6.4.1.1	Test Purpose and Environment	4624

A.11.6.4.1.2	Test parameters	4624

A.11.6.4.1.3	Test Requirements	4627

A.11.6.5	RSSI	4627

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4627

A.11.6.5.1.1	Test Purpose and Environment	4627

A.11.6.5.1.2	Test parameters	4627

A.11.6.5.1.3	Test Requirements	4630

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4630

A.11.6.5.2.1	Test Purpose and Environment	4630

A.11.6.5.2.2	Test parameters	4630

A.11.6.5.2.3	Test Requirements	4633

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4633

A.11.6.5.3.1	Test Purpose and Environment	4633

A.11.6.5.3.2	Test parameters	4633

A.11.6.5.3.3	Test Requirements	4637

A.11.6.6	Channel occupancy	4637

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4637

A.11.6.6.1.1	Test Purpose and Environment	4637

A.11.6.6.1.2	Test parameters	4637

A.11.6.6.1.3	Test Requirements	4641

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4641

A.11.6.6.2.1	Test Purpose and Environment	4641

A.11.6.6.2.2	Test parameters	4641

A.11.6.6.2.3	Test Requirements	4644

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4644

A.11.6.6.3.1	Test Purpose and Environment	4644

A.11.6.6.3.2	Test parameters	4644

A.11.6.6.3.3	Test Requirements	4648

A.11.6.7	E-UTRAN RSRP	4649

A.11.6.8	E-UTRAN RSRQ	4649

A.11.6.9	E-UTRAN SINR	4649

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4649

A.12.1	RRC_IDLE state mobility	4649

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4649

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4649

A.12.1.1.1.1	Test Purpose and Environment	4649

A.12.1.1.1.2	Test Requirements	4654

A.12.2	RRC_CONNECTED state mobility	4655

A.12.2.1	Handover	4655

A.12.2.1.1	E-UTRAN - NR with CCA handover	4655

A.12.2.1.1.1	Test Purpose and Environment	4655

A.12.2.1.1.2	Test Requirements	4660

A.12.3	Void	4661

A.12.4	Measurement procedure	4661

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4661

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4661

A.12.4.1.1.1	Test Purpose and Environment	4661

A.12.4.1.1.2	Test Requirements	4665

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4665

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4665

A.12.4.2.1.1	Test Purpose and Environment	4665

A.12.4.2.1.2	Test Requirements	4671

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4671

A.12.4.2.2.1	Test Purpose and Environment	4671

A.12.4.2.2.2	Test Requirements	4677

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4677

A.12.4.2.3.1	Test Purpose and Environment	4677

A.12.4.2.3.2	Test Requirements	4682

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4682

A.12.4.2.4.1	Test Purpose and Environment	4682

A.12.4.2.4.2	Test Requirements	4687

A.12.4.2.5	Void	4687

A.12.4.2.6	Void	4687

A.12.5	Measurement performance	4687

A.12.5.1	E-UTRANNR SFTD	4687

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4687

A.12.5.1.1.1	Test Purpose	4687

A.12.5.1.1.2	Test Environment	4687

A.12.5.1.1.3	Test Requirements	4692

A.12.5.2	Void	4692

A.12.5.3	Void	4692

A.12.5.4	Void	4692

A.12.5.5	Void	4692

A.12.5.6	Void	4692

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4697

A.13.1	Void	4697

A.13.2	Signalling characteristics	4697

A.13.2.1	Void	4697

A.13.2.2	SCell activation and deactivation delay	4697

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4697

A.13.2.2.1.1	Test Purpose and Environment	4697

A.13.2.2.1.2	Test Requirements	4701

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4701

A.13.2.2.2.1	Test Purpose and Environment	4701

A.13.2.2.2.2	Test Requirements	4702

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4702

A.13.2.2.3.1	Test Purpose and Environment	4702

A.13.2.2.3.2	Test Requirements	4703

A.13.2.3	Void	4703

A.13.3	Measurement procedure	4703

A.13.3.1	Intra-frequency measurements	4703

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4703

A.13.3.1.1.1	Test purpose and environment	4703

A.13.3.1.1.2	Test parameters	4703

A.13.3.1.1.3	Test Requirements	4708

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4708

A.13.3.1.2.1	Test purpose and environment	4708

A.13.3.1.2.2	Test parameters	4708

A.13.3.1.2.3	Test Requirements	4713

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4713

A.13.3.1.3.1	Test purpose and environment	4713

A.13.3.1.3.2	Test parameters	4713

A.13.3.1.3.3	Test Requirements	4718

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4718

A.13.3.1.4.1	Test purpose and environment	4718

A.13.3.1.4.2	Test parameters	4718

A.13.3.1.4.3	Test Requirements	4724

A.13.3.1.5	Void	4724

A.13.3.1.6	Void	4724

A.13.3.2	Inter-frequency measurements	4724

A.13.3.2.1	Void	4724

A.13.3.2.2	Void	4724

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4724

A.13.3.2.3.1	Test Purpose and Environment	4724

A.13.3.2.3.2	Test Requirements	4729

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4729

A.13.3.2.4.1	Test Purpose and Environment	4729

A.13.3.2.4.2	Test Requirements	4734

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4735

A.13.3.2.5.1	Test Purpose and Environment	4735

A.13.3.2.5.2	Test Requirements	4740

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4740

A.13.3.2.6.1	Test Purpose and Environment	4740

A.13.3.2.6.2	Test Requirements	4745

A.13.3.3	L1-RSRP measurements for beam reporting	4746

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4746

A.13.3.3.1.1	Test Purpose and Environment	4746

A.13.3.3.1.2	Test parameters	4746

A.13.3.3.1.3	Test Requirements	4750

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4750

A.13.3.3.2.1	Test Purpose and Environment	4750

A.13.3.3.2.2	Test parameters	4750

A.13.3.3.2.3	Test Requirements	4754

A.13.4	Measurement performance	4754

A.13.4.1	SS-RSRP	4754

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4754

A.13.4.1.1.1	Test Purpose and Environment	4754

A.13.4.1.1.2	Test parameters	4754

A.13.4.1.1.3	Test Requirements	4757

A.13.4.2	SS-RSRQ	4757

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4757

A.13.4.2.1.1	Test Purpose and Environment	4757

A.13.4.2.1.2	Test Parameters	4757

A.13.4.2.1.3	Test Requirements	4763

A.13.4.3	SS-SINR	4763

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4763

A.13.4.3.1.1	Test Purpose and Environment	4763

A.13.4.3.1.2	Test Parameters	4763

A.13.4.3.1.3	Test Requirements	4770

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4770

A.13.4.4.1	SSB based L1-RSRP measurement	4770

A.13.4.4.1.1	Test Purpose and Environment	4770

A.13.4.4.1.2	Test parameters	4770

A.13.4.4.1.3	Test Requirements	4775

A.13.4.5	RSSI	4775

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4775

A.13.4.5.1.1	Test Purpose and Environment	4775

A.13.4.5.1.2	Test parameters	4775

A.13.4.5.1.3	Test Requirements	4778

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4778

A.13.4.5.2.1	Test Purpose and Environment	4778

A.13.4.5.2.2	Test parameters	4778

A.13.4.5.2.3	Test Requirements	4782

A.13.4.6	Channel occupancy	4782

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4782

A.13.4.6.1.1	Test Purpose and Environment	4782

A.13.4.6.1.2	Test parameters	4782

A.13.4.6.1.3	Test Requirements	4786

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4786

A.13.4.6.2.1	Test Purpose and Environment	4786

A.13.4.6.2.2	Test parameters	4786

A.13.4.6.2.3	Test Requirements	4790

A.14	NR standalone tests for Satellite access	4790

A.14.1	RRC_IDLE state mobility	4790

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4790

A.14.1.1.1	Test Purpose and Environment	4790

A.14.1.1.2	Test Parameters	4790

A.14.1.1.3	Test Requirements	4792

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4793

A.14.1.2.1	Test Purpose and Environment	4793

A.14.1.2.2	Test Parameters	4793

A.14.1.2.3	Test Requirements	4795

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4796

A.14.1.3.1	Test Purpose and Environment	4796

A.14.1.3.2	Test Parameters	4796

A.14.1.3.3	Test Requirements	4798

A.14.1.4	Location-based cell measurement initiation to FR1 intra-frequency NR cell reselection	4799

A.14.1.4.1	Test Purpose and Environment	4799

A.14.1.4.2	Test Parameters	4799

A.14.1.4.3	Test Requirements	4801

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4802

A.14.1.5.1	Test Purpose and Environment	4802

A.14.1.5.2	Test Parameters	4802

A.14.1.5.3	Test Requirements	4804

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements	4805

A.14.1.6.1	Test Purpose and Environment	4805

A.14.1.6.2	Test Parameters	4805

A.14.1.6.3	Test Requirements	4807

A.14.1.7	Time-based Cell measurement initiation to FR1 inter-frequency cell reselection	4808

A.14.1.7.1	Test Purpose and Environment	4808

A.14.1.7.2	Test Parameters	4808

A.14.1.7.3	Test Requirements	4810

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4811

A.14.1.8.1	Test Purpose and Environment	4811

A.14.1.8.2	Test Parameters	4811

A.14.1.8.3	Test Requirements	4813

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4814

A.14.1.9.1	Test Purpose and Environment	4814

A.14.1.9.2	Test Parameters	4814

A.14.1.9.3	Test Requirements	4817

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4817

A.14.1.10.1	Test Purpose and Environment	4817

A.14.1.10.2	Test Parameters	4817

A.14.1.10.3	Test Requirements	4819

A.14.2	RRC_CONNECTED state mobility	4820

A.14.2.1	Handover	4820

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4820

A.14.2.1.1.1	Test Purpose and Environment	4820

A.14.2.1.1.2	Test Parameters	4820

A.14.2.1.1.3	Test Requirements	4822

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4823

A.14.2.1.2.1	Test Purpose and Environment	4823

A.14.2.1.2.2	Test Parameters	4823

A.14.2.1.2.3	Test Requirements	4825

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4825

A.14.2.1.3.1	Test Purpose and Environment	4825

A.14.2.1.3.2	Test Parameters	4825

A.14.2.1.3.3	Test Requirements	4829

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4829

A.14.2.1.4.1	Test Purpose and Environment	4829

A.14.2.1.4.2	Test Parameters	4829

A.14.2.1.4.3	Test Requirements	4831

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4832

A.14.2.1.5.1	Test Purpose and Environment	4832

A.14.2.1.5.2	Test Parameters	4832

A.14.2.1.5.3	Test Requirements	4835

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4835

A.14.2.1.6.1	Test Purpose and Environment	4835

A.14.2.1.6.2	Test Parameters	4835

A.14.2.1.6.3	Test Requirements	4839

A.14.2.2	RRC Connection Mobility Control	4839

A.14.2.2.1	SA: RRC Re-establishment for SAN	4839

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4839

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4842

A.14.2.2.2	Random Access	4845

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4845

A.14.2.2.2.1.1	Test Purpose and Environment	4845

A.14.2.2.2.1.2	Test Requirements	4848

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4849

A.14.2.2.2.2.1	Test Purpose and Environment	4849

A.14.2.2.2.2.2	Test Requirements	4853

A.14.2.2.3	RRC Connection Release with Redirection	4854

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4854

A.14.2.2.3.1.1	Test Purpose and Environment	4854

A.14.2.2.3.1.2	Test Parameters	4854

A.14.2.2.3.1.3	Test Requirements	4857

A.14.3	Timing for Satellite Access	4857

A.14.3.1	UE transmit timing for Satellite Access	4857

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4857

A.14.3.1.1.1	Test Purpose and environment	4857

A.14.3.1.1.2	Test requirements	4859

A.14.3.2	Timing advance for satellite access	4860

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4860

A.14.3.2.1.1	Test Purpose and Environment	4860

A.14.3.2.1.2	Test Parameters	4860

A.14.3.2.1.3	Test Requirements	4863

A.14.4	Signalling characteristics	4863

A.14.4.1	Radio link Monitoring	4863

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4863

A.14.4.1.1.1	Test Purpose and Environment	4863

A.14.4.1.1.2	Test Requirements	4868

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4868

A.14.4.1.2.1	Test Purpose and Environment	4868

A.14.4.1.2.2	Test Requirements	4874

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4874

A.14.4.1.3.1	Test Purpose and Environment	4874

A.14.4.1.3.2	Test Requirements	4878

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4878

A.14.4.1.4.1	Test Purpose and Environment	4878

A.14.4.1.4.2	Test Requirements	4883

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4883

A.14.4.1.5.1	Test Purpose and Environment	4883

A.14.4.1.5.2	Test Requirements	4888

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4888

A.14.4.1.6.1	Test Purpose and Environment	4888

A.14.4.1.6.2	Test Requirements	4892

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4892

A.14.4.1.7.1	Test Purpose and Environment	4892

A.14.4.1.7.2	Test Requirements	4897

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4897

A.14.4.1.8.1	Test Purpose and Environment	4897

A.14.4.1.8.2	Test Requirements	4902

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4902

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4902

A.14.4.2.1.1	Test Purpose and Environment	4902

A.14.4.2.1.2	Test Requirements	4908

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4908

A.14.4.2.2.1	Test Purpose and Environment	4908

A.14.4.2.2.2	Test Requirements	4913

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4913

A.14.4.2.3.1	Test Purpose and Environment	4913

A.14.4.2.3.2	Test Requirements	4918

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4919

A.14.4.2.4.1	Test Purpose and Environment	4919

A.14.4.2.4.2	Test Requirements	4924

A.14.4.2.5	Void	4924

A.14.4.2.6	Void	4924

A.14.4.3	Active BWP switch for satellite access	4924

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4924

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4924

A.14.4.3.1.1.1	Test Purpose and Environment	4924

A.14.4.3.1.1.2	Test Requirements	4929

A.14.4.3.2	RRC-based Active BWP Switch	4929

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4929

A.14.4.3.2.1.1	Test Purpose and Environment	4929

A.14.4.3.2.1.2	Test Requirements	4933

A.14.4.4	UE specific CBW change for satellite access	4933

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4933

A.14.4.4.1.1	Test Purpose and Environment	4933

A.14.4.4.1.2	Test Requirements	4937

A.14.4.5	Pathloss reference signal switching delay	4937

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	4937

A.14.4.5.1.1	Test Purpose and Environment	4937

A.14.4.5.1.2	Test Requirements	4940

A.14.5	Measurement procedure	4941

A.14.5.1	Intra-frequency Measurements	4941

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4941

A.14.5.1.1.1	Test purpose and Environment	4941

A.14.5.1.1.2	Test parameters	4941

A.14.5.1.1.3	Test Requirements	4943

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4944

A.14.5.1.2.1	Test purpose and Environment	4944

A.14.5.1.2.2	Test parameters	4944

A.14.5.1.2.3	Test Requirements	4945

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4946

A.14.5.1.3.1	Test purpose and Environment	4946

A.14.5.1.3.2	Test parameters	4946

A.14.5.1.3.3	Test Requirements	4948

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	4949

A.14.5.1.4.1	Test purpose and Environment	4949

A.14.5.1.4.2	Test parameters	4949

A.14.5.1.4.3	Test Requirements	4951

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	4952

A.14.5.1.5.1	Test purpose and Environment	4952

A.14.5.1.5.2	Test parameters	4952

A.15.5.1.5.3	Test Requirements	4955

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	4955

A.14.5.1.6.1	Test purpose and Environment	4955

A.14.5.1.6.2	Test parameters	4955

A.14.5.1.6.3	Test Requirements	4957

A.14.5.2	Inter-frequency Measurements	4958

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4958

A.14.5.2.1.1	Test Purpose and Environment	4958

A.14.5.2.1.2	Test Requirements	4962

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4962

A.14.5.2.2.1	Test Purpose and Environment	4962

A.14.5.2.2.2	Test Requirements	4966

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	4966

A.14.5.2.3.1	Test Purpose and Environment	4966

A.14.5.2.3.2	Test Requirements	4970

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	4970

A.14.5.2.4.1	Test Purpose and Environment	4970

A.14.5.2.4.2	Test Requirements	4974

A.14.5.2.5	void	4974

A.14.5.2.5.1	void	4974

A.14.5.2.5.2	void	4974

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	4974

A.14.5.2.6.1	Test Purpose and Environment	4974

A.14.5.2.6.2	Test Requirements	4978

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	4978

A.14.5.2.7.1	Test purpose and Environment	4978

A.14.5.2.7.2	Test parameters	4978

A.14.5.2.7.3	Test Requirements	4980

A.14.5.2.8	Event triggered reporting tests without gap under DRX	4981

A.14.5.2.8.1	Test purpose and Environment	4981

A.14.5.2.8.2	Test parameters	4981

A.14.5.2.8.3	Test Requirements	4983

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	4984

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	4984

A.14.5.3.1.1	Test Purpose and Environment	4984

A.14.5.3.1.2	Test parameters	4984

A.14.5.3.1.3	Test Requirements	4987

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	4987

A.14.5.3.2.1	Test Purpose and Environment	4987

A.14.5.3.2.2	Test parameters	4988

A.14.5.3.2.3	Test Requirements	4991

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	4991

A.14.5.3.3.1	Test Purpose and Environment	4991

A.14.5.3.3.2	Test parameters	4991

A.14.5.3.3.3	Test Requirements	4993

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	4993

A.14.5.3.4.1	Test Purpose and Environment	4993

A.14.5.3.4.2	Test parameters	4993

A.14.5.3.4.3	Test Requirements	4997

A.14.6	Measurement Performance requirements	4997

A.14.6.1	 SS-RSRP for SAN	4997

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4997

A.14.6.1.1.1	Test Purpose and Environment	4997

A.14.6.1.1.2	Test parameters	4998

A.14.6.1.1.3	Test Requirements	5001

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5001

A.14.6.1.2.1	Test Purpose and Environment	5001

A.14.6.1.2.2	Test parameters	5001

A.14.6.1.2.3	Test Requirements	5004

A.14.6.2	SS-RSRQ	5004

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5004

A.14.6.2.1.1	Test Purpose and Environment	5004

A.14.6.2.1.2	Test Parameters	5004

A.14.6.2.1.3	Test Requirements	5007

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5007

A.14.6.2.2.1	Test Purpose and Environment	5007

A.14.6.2.2.2	Test Parameters	5007

A.14.6.2.2.3	Test Requirements	5010

A.14.6.3	SS-SINR	5010

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5010

A.14.6.3.1.1	Test Purpose and Environment	5010

A.14.6.3.1.2	Test Parameters	5010

A.14.6.3.1.3	Test Requirements	5013

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5013

A.14.6.3.2.1	Test Purpose and Environment	5013

A.14.6.3.2.2	Test Parameters	5013

A.14.6.3.2.3	Test Requirements	5016

A.14.6.4	L1-RSRP measurement for beam reporting	5016

A.14.6.4.1	SSB based L1-RSRP measurement	5016

A.14.6.4.1.1	Test Purpose and Environment	5016

A.14.6.4.1.2	Test parameters	5016

A.14.6.4.1.3	Test Requirements	5019

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5019

A.14.6.4.2.1	Test Purpose and Environment	5019

A.14.6.4.2.2	Test parameters	5019

A.14.6.4.2.3	Test Requirements	5022

A.15	NR standalone tests with one or more NR cells in FR2-2	5024

A.15.1	SA: RRC_IDLE state mobility	5024

A.15.1.1	Cell re-selection to NR	5024

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	5024

A.15.1.1.1.1	Test Purpose and Environment	5024

A.15.1.1.1.2	Test Parameters	5024

A.15.1.1.1.3	Test Requirements	5028

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	5028

A.15.1.2.1	Test Purpose and Environment	5028

A.15.1.2.2	Test Parameters	5028

A.15.1.2.3	Test Requirements	5032

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5032

A.15.1.3.1	Test Purpose and Environment	5032

A.15.1.3.2	Test Parameters	5032

A.15.1.3.3	Test Requirements	5035

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5035

A.15.1.4.1	Test Purpose and Environment	5035

A.15.1.4.2	Test Parameters	5035

A.15.1.4.3	Test Requirements	5038

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5038

A.15.1.5.1	Test Purpose and Environment	5038

A.15.1.5.2	Test Parameters	5038

A.15.1.5.3	Test Requirements	5042

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5042

A.15.1.6.1	Test Purpose and Environment	5042

A.15.1.6.2	Test Parameters	5042

A.15.1.6.3	Test Requirements	5045

A.15.2	Signaling characteristics	5045

A.15.2.1	SCell Activation and Deactivation Delay	5045

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5045

A.15.2.1.1.1	Test Purpose and Environment	5045

A.15.2.1.1.2	Test Requirements	5048

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5048

A.15.2.1.2.1	Test Purpose and Environment	5048

A.15.2.1.2.2	Test Requirements	5053

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5054

A.15.2.1.3.1	Test Purpose and Environment	5054

A.15.2.1.3.2	Test Requirements	5057

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5058

A.15.2.1.4.1	Test Purpose and Environment	5058

A.15.2.1.4.2	Test Requirements	5061

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5062

A.15.2.1.5.1	Test Purpose and Environment	5062

A.15.2.1.5.2	Test Requirements	5066

A.15.3	RRC_CONNECTED state mobility	5067

A.15.3.1	Handover	5067

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5067

A.15.3.1.1.1	Test Purpose and Environment	5067

A.15.3.1.1.2	Test Parameters	5067

A.15.3.1.1.3	Test Requirements	5071

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5071

A.15.3.1.2.1	Test Purpose and Environment	5071

A.15.3.1.2.2	Test Parameters	5071

A.15.3.1.2.3	Test Requirements	5075

A.15.4	Measurement procedure	5076

A.15.4.1	Intra-frequency Measurements	5076

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5076

A.15.4.1.1.1	Test purpose and Environment	5076

A.15.4.1.1.2	Test Requirements	5078

A.15.4.2	Inter-frequency Measurements	5079

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5079

A.15.4.2.1.1	Test Purpose and Environment	5079

A.15.4.2.1.2	Test Requirements	5082

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5083

A.16.1	SA: RRC_IDLE state mobility for RedCap	5083

A.16.1.1	Cell re-selection to NR	5083

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	5083

A.16.1.1.1.1	Test Purpose and Environment	5083

A.16.1.1.1.2	Test Parameters	5083

A.16.1.1.1.3	Test Requirements	5087

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	5087

A.16.1.1.2.1	Test Purpose and Environment	5087

A.16.1.1.2.2	Test Parameters	5087

A.16.1.1.2.3	Test Requirements	5091

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	5091

A.16.1.1.3.1	Test Purpose and Environment	5091

A.16.1.1.3.2	Test Parameters	5091

A.16.1.1.3.3	Test Requirements	5096

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	5096

A.16.1.1.4.1	Test Purpose and Environment	5096

A.16.1.1.4.2	Test Parameters	5096

A.16.1.1.4.3	Test Requirements	5101

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5101

A.16.1.1.5.1	Test Purpose and Environment	5101

A.16.1.1.5.2	Test Parameters	5101

A.16.1.1.5.3	Test Requirements	5106

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5106

A.16.1.1.6.1	Test Purpose and Environment	5106

A.16.1.1.6.2	Test Parameters	5106

A.16.1.1.6.3	Test Requirements	5111

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5111

A.16.1.1.7.1	Test Purpose and Environment	5111

A.16.1.1.7.2	Test Parameters	5111

A.16.1.1.7.3	Test Requirements	5116

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5117

A.16.1.1.8.1	Test Purpose and Environment	5117

A.16.1.1.8.2	Test Parameters	5117

A.16.1.1.8.3	Test Requirements	5121

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5122

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	5122

A.16.1.2.1.1	Test Purpose and Environment	5122

A.16.1.2.1.2	Test Parameters	5122

A.16.1.2.1.3	Test Requirements	5125

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	5126

A.16.1.2.2.1	Test Purpose and Environment	5126

A.16.1.2.2.2	Test Parameters	5126

A.16.1.2.2.3	Test Requirements	5129

A.16.1.2.3.1	Test Purpose and Environment	5130

A. 16.1.2.3.2	Test Parameters	5130

A.16.1.2.3.3	Test Requirements	5133

A.16.1.2.4.1	Test Purpose and Environment	5134

A.16.1.2.4.2	Test Parameters	5134

A.16.1.3.1.3	Test Requirements	5137

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5138

A.16.1.2.5.1	Test Purpose and Environment	5138

A.16.1.2.5.2	Test Parameters	5138

A.16.1.2.5.3	Test Requirements	5141

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5142

A.16.1.2.6.1	Test Purpose and Environment	5142

A.16.1.2.6.2	Test Parameters	5142

A.16.1.2.6.3	Test Requirements	5145

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5146

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5146

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	5146

A.16.2.1.1.1	Test purpose and Environment	5146

A.16.2.1.1.2	Test Parameters	5148

A.16.2.1.1.3	Test requirements	5151

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	5151

A.16.2.1.2.1	Test purpose and Environment	5151

A.16.2.1.2.2	Test Parameters	5153

A.16.2.1.2.3	Test requirements	5156

A.16.3	RRC_CONNECTED state mobility for RedCap	5156

A.16.3.1	Handover	5156

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5156

A.16.3.1.1.1	Test Purpose and Environment	5156

A.16.3.1.1.2	Test Parameters	5157

A.16.3.1.1.3	Test Requirements	5160

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5160

A.16.3.1.2.1	Test Purpose and Environment	5160

A.16.3.1.2.2	Test Parameters	5160

A.16.3.1.2.3	Test Requirements	5164

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5164

A.16.3.1.3.1	Test Purpose and Environment	5164

A.16.3.1.3.2	Test Parameters	5164

A.16.3.1.3.3	Test Requirements	5168

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5168

A.16.3.1.4.1	Test Purpose and Environment	5168

A.16.3.1.4.2	Test Parameters	5168

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5172

A.16.3.1.5.1	Test Purpose and Environment	5172

A.16.3.1.5.2	Test Parameters	5172

A.16.3.1.5.3	Test Requirements	5176

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5176

A.16.3.1.6.1	Test Purpose and Environment	5176

A.16.3.1.6.2	Test Parameters	5176

A.16.3.1.6.3	Test Requirements	5180

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	5180

A.16.3.1.7.1	Test Purpose and Environment	5180

A.16.3.1.7.2	Test Requirements	5186

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	5186

A.16.3.1.8.1	Test Purpose and Environment	5186

A.16.3.1.8.2	Test Requirements	5192

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5192

A.16.3.1.9.1	Test Purpose and Environment	5192

A.16.3.1.9.2	Test Requirements	5199

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5199

A.16.3.1.10.1	Test Purpose and Environment	5199

A.16.3.1.10.2	Test Requirements	5205

A.16.3.2	RRC Connection Mobility Control	5205

A.16.3.2.1	SA: RRC Re-establishment	5205

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5205

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5209

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5213

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5218

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5223

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5227

A.16.3.2.2	Random Access	5231

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5231

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5237

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5242

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5252

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5256

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5260

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5264

A.16.3.2.3	SA: RRC Connection Release with Redirection	5268

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5268

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5272

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5276

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5283

A.16.4	Timing for RedCap	5290

A.16.4.1	UE transmit timing	5290

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5290

A.16.4.1.1.1	Test Purpose and environment	5290

A.16.4.1.1.2	Test requirements	5295

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5295

A.16.4.1.2.1	Test Purpose and environment	5295

A.16.4.1.2.2	Test requirements	5298

A.16.4.2	UE timer accuracy	5298

A.16.4.3	Timing advance	5298

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5298

A.16.4.3.1.1	Test Purpose and Environment	5298

A.16.4.3.1.2	Test Parameters	5298

A.16.4.3.1.3	Test Requirements	5302

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5302

A.16.4.3.2.1	Test Purpose and Environment	5302

A.16.4.3.2.2	Test Parameters	5303

A.16.4.3.2.3	Test Requirements	5306

A.16.5	Signalling characteristics for RedCap	5307

A.16.5.1	Radio link Monitoring	5307

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5307

A.16.5.1.1.1	Test Purpose and Environment	5307

A.16.5.1.1.2	Test Requirements	5312

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5312

A.16.5.1.2.1	Test Purpose and Environment	5312

A.16.5.1.2.2	Test Requirements	5318

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5318

A.16.5.1.3.1	Test Purpose and Environment	5318

A.16.5.1.3.2	Test Requirements	5324

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5324

A.16.5.1.4.1	Test Purpose and Environment	5324

A.16.5.1.4.2	Test Requirements	5330

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5330

A.16.5.1.5.1	Test Purpose and Environment	5330

A.16.5.1.5.2	Test Requirements	5336

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5336

A.16.5.1.6.1	Test Purpose and Environment	5336

A.16.5.1.6.2	Test Requirements	5341

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5341

A.16.5.1.7.1	Test Purpose and Environment	5341

A.16.5.1.7.2	Test Requirements	5346

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5346

A.16.5.1.8.1	Test Purpose and Environment	5346

A.16.5.1.8.2	Test Requirements	5352

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5352

A.16.5.1.9.1	Test Purpose and Environment	5352

A.16.5.1.9.2	Test Requirements	5358

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5358

A.16.5.1.10.1	Test Purpose and Environment	5358

A.16.5.1.10.2	Test Requirements	5364

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5364

A.16.5.1.11.1	Test Purpose and Environment	5364

A.16.5.1.11.2	Test Requirements	5370

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5370

A.16.5.1.12.1	Test Purpose and Environment	5370

A.16.5.1.12.2	Test Requirements	5376

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5376

A.16.5.1.13.1	Test Purpose and Environment	5376

A.16.5.1.13.2	Test Requirements	5381

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5381

A.16.5.1.14.1	Test Purpose and Environment	5381

A.16.5.1.14.2	Test Requirements	5387

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5387

A.16.5.1.15.1	Test Purpose and Environment	5387

A.16.5.1.15.2	Test Requirements	5393

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5393

A.16.5.1.16.1	Test Purpose and Environment	5393

A.16.5.1.16.2	Test Requirements	5399

A.16.5.2	Beam Failure Detection and Link recovery procedures	5399

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5399

A.16.5.2.1.1	Test Purpose and Environment	5399

A.16.5.2.1.2	Test Requirements	5405

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5405

A.16.5.2.2.1	Test Purpose and Environment	5405

A.16.5.2.2.2	Test Requirements	5411

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5411

A.16.5.2.3.1	Test Purpose and Environment	5411

A.16.5.2.3.2	Test Requirements	5418

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5418

A.16.5.2.4.1	Test Purpose and Environment	5418

A.16.5.2.4.2	Test Requirements	5424

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5424

A.16.5.2.5.1	Test Purpose and Environment	5424

A.16.5.2.5.2	Test Requirements	5430

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5430

A.16.5.2.6.1	Test Purpose and Environment	5430

A.16.5.2.6.2	Test Requirements	5436

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5436

A.16.5.2.7.1	Test Purpose and Environment	5436

A.16.5.2.7.2	Test Requirements	5442

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5442

A.16.5.2.8.1	Test Purpose and Environment	5442

A.16.5.2.8.2	Test Requirements	5448

A.16.5.3	Active BWP switch	5448

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5448

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5448

A.16.5.3.1.1.1	Test Purpose and Environment	5448

A.16.5.3.1.2.1	Test Requirements	5453

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5453

A.16.5.3.1.2.1	Test Purpose and Environment	5453

A.16.5.3.1.2.2	Test Requirements	5458

A.16.5.3.2	RRC-based Active BWP Switch	5458

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5458

A.16.5.3.2.1.1	Test Purpose and Environment	5458

A.16.5.3.2.1.2	Test Requirements	5462

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5462

A.16.5.3.2.2.1	Test Purpose and Environment	5462

A.16.5.3.2.2.2	Test Requirements	5466

A.16.5.4	UE specific CBW change	5466

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5466

A.16.5.4.1.1	Test Purpose and Environment	5466

A.16.5.4.1.2	Test Requirements	5470

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5470

A.16.5.4.2.1	Test Purpose and Environment	5470

A.16.5.4.2.2	Test Requirements	5474

A.16.6	Measurement procedure for RedCap	5474

A.16.6.1	Intra-frequency Measurements	5474

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5474

A.16.6.1.1.1	Test purpose and Environment	5474

A.16.6.1.1.2	Test parameters	5474

A.16.6.1.1.3	Test Requirements	5478

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5478

A.16.6.1.2.1	Test purpose and Environment	5478

A.16.6.1.2.2	Test parameters	5478

A.16.6.1.2.3	Test Requirements	5482

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5482

A.16.6.1.3.1	Test purpose and Environment	5482

A.16.6.1.3.2	Test parameters	5482

A.16.6.1.3.3	Test Requirements	5486

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5486

A.16.6.1.4.1	Test purpose and Environment	5486

A.16.6.1.4.2	Test parameters	5486

A.16.6.1.4.3	Test Requirements	5490

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5490

A.16.6.1.5.1	Test purpose and Environment	5490

A.16.6.1.5.2	Test parameters	5490

A.16.6.1.5.3	Test Requirements	5494

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5494

A.16.6.1.6.1	Test purpose and Environment	5494

A.16.6.1.6.2	Test parameters	5494

A.16.6.1.6.3	Test Requirements	5498

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5498

A.16.6.1.7.1	Test purpose and Environment	5498

A.16.6.1.7.2	Test parameters	5498

A.16.6.1.7.3	Test Requirements	5502

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5502

A.16.6.1.8.1	Test purpose and Environment	5502

A.16.6.1.8.2	Test parameters	5502

A.16.6.1.8.3	Test Requirements	5506

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5506

A.16.6.1.9.1	Test purpose and Environment	5506

A.16.6.1.9.2	Test parameters	5506

A.16.6.1.9.3	Test Requirements	5508

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5509

A.16.6.1.10.1	Test purpose and Environment	5509

A.16.6.1.10.2	Test parameters	5509

A.16.6.1.10.3	Test Requirements	5510

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5511

A.16.6.1.11.1	Test purpose and Environment	5511

A.16.6.1.11.2	Test parameters	5511

A.16.6.1.11.3	Test Requirements	5513

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5514

A.16.6.1.12.1	Test purpose and Environment	5514

A.16.6.1.12.2	Test parameters	5514

A.16.6.1.12.3	Test Requirements	5516

A.16.6.2	Inter-frequency Measurements	5517

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5517

A.16.6.2.1.1	Test Purpose and Environment	5517

A.16.6.2.1.2	Test Requirements	5522

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5522

A.16.6.2.2.1	Test Purpose and Environment	5522

A.16.6.2.2.2	Test Requirements	5527

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5527

A.16.6.2.3.1	Test Purpose and Environment	5527

A.16.6.2.3.2	Test Requirements	5530

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5531

A.16.6.2.4.1	Test Purpose and Environment	5531

A.16.6.2.4.2	Test Requirements	5534

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5535

A.16.6.2.5.1	Test Purpose and Environment	5535

A.16.6.2.5.2	Test Requirements	5538

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5539

A.16.6.2.6.1	Test Purpose and Environment	5539

A.16.6.2.6.2	Test Requirements	5542

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5543

A.16.6.2.7.1	Test Purpose and Environment	5543

A.16.6.2.7.2	Test Requirements	5547

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5548

A.16.6.2.8.1	Test Purpose and Environment	5548

A.16.6.2.8.2	Test Requirements	5551

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5552

A.16.6.2.9.1	Test Purpose and Environment	5552

A.16.6.2.9.2	Test Requirements	5555

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5555

A.16.6.2.10.1	Test Purpose and Environment	5555

A.16.6.2.10.2	Test Requirements	5558

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5558

A.16.6.2.11.1	Test Purpose and Environment	5558

A.16.6.2.11.2	Test Requirements	5561

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5561

A.16.6.2.12.1	Test Purpose and Environment	5561

A.16.6.2.12.2	Test Requirements	5565

A.16.6.3	Inter-RAT Measurements	5566

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5566

A.16.6.3.1.1	Test purpose and Environment	5566

A.16.6.3.1.2	Test Requirements	5571

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5571

A.16.6.3.2.1	Test purpose and Environment	5571

A.16.6.3.2.2	Test Requirements	5576

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5576

A.16.6.3.3.1	Test purpose and Environment	5576

A.16.6.3.3.2	Test Requirements	5581

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5581

A.16.6.3.4.1	Test purpose and Environment	5581

A.16.6.3.4.2	Test Requirements	5586

A.16.6.4	L1-RSRP measurement for beam reporting	5586

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5586

A.16.6.4.1.1	Test Purpose and Environment	5586

A.16.6.4.1.2	Test parameters	5586

A.16.6.4.1.3	Test Requirements	5590

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5590

A.16.6.4.2.1	Test Purpose and Environment	5590

A.16.6.4.2.2	Test parameters	5591

A.16.6.4.2.3	Test Requirements	5594

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5594

A.16.6.4.3.1	Test Purpose and Environment	5594

A.16.6.4.3.2	Test parameters	5595

A.16.6.4.3.3	Test Requirements	5598

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5598

A.16.6.4.4.1	Test Purpose and Environment	5598

A.16.6.4.4.2	Test parameters	5599

A.16.6.4.4.3	Test Requirements	5602

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5602

A.16.6.4.5.1	Test Purpose and Environment	5602

A.16.6.4.5.2	Test parameters	5603

A.16.6.4.5.3	Test Requirements	5606

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5606

A.16.6.4.6.1	Test Purpose and Environment	5606

A.16.6.4.6.2	Test parameters	5607

A.16.6.4.6.3	Test Requirements	5610

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5610

A.16.6.4.7.1	Test Purpose and Environment	5610

A.16.6.4.7.2	Test parameters	5611

A.16.6.4.7.3	Test Requirements	5614

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5614

A.16.6.4.8.1	Test Purpose and Environment	5614

A.16.6.4.8.2	Test parameters	5615

A.16.6.4.8.3	Test Requirements	5618

A.16.6.5	NR measurements with autonomous gaps	5618

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5618

A.16.6.5.1.1	Test Purpose and Environment	5618

A.16.6.5.1.2	Test Parameters	5618

A.16.6.5.1.3	Test Requirements	5621

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5621

A.16.6.5.2.1	Test Purpose and Environment	5621

A.16.6.5.2.2	Test Parameters	5621

A.16.6.5.2.3	Test Requirements	5624

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5624

A.16.6.5.3.1	Test Purpose and Environment	5624

A.16.6.5.3.2	Test Requirements	5629

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5630

A.16.6.5.4.1	Test Purpose and Environment	5630

A.16.6.5.4.2	Test Requirements	5635

A.16.7	Measurement Performance requirements for RedCap	5636

A.16.7.1	SS-RSRP	5636

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5636

A.16.7.1.1.1	Test Purpose and Environment	5636

A.16.7.1.1.2	Test parameters	5636

A.16.7.1.1.3	Test Requirements	5641

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5641

A.16.7.1.2.1	Test Purpose and Environment	5641

A.16.7.1.2.2	Test parameters	5641

A.16.7.1.2.3	Test Requirements	5646

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5646

A.16.7.1.3.1	Test Purpose and Environment	5646

A.16.7.1.3.2	Test parameters	5646

A.16.7.1.3.3	Test Requirements	5650

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5650

A.16.7.1.4.1	Test Purpose and Environment	5650

A.16.7.1.4.2	Test parameters	5651

A.16.7.1.4.3	Test Requirements	5655

A.16.7.2	SS-RSRQ	5655

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5655

A.16.7.2.1.1	Test Purpose and Environment	5655

A.16.7.2.1.2	Test Parameters	5656

A.16.7.2.1.3	Test Requirements	5660

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5660

A.16.7.2.2.1	Test Purpose and Environment	5660

A.16.7.2.2.2	Test Parameters	5660

A.16.7.2.2.3	Test Requirements	5664

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5664

A.16.7.2.3.1	Test Purpose and Environment	5664

A.16.7.2.3.2	Test parameters	5664

A.16.7.2.3.3	Test Requirements	5669

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5669

A.16.7.2.4.1	Test Purpose and Environment	5669

A.16.7.2.4.2	Test parameters	5669

A.16.7.2.4.3	Test Requirements	5675

A.16.7.3	SS-SINR	5675

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5675

A.16.7.3.1.1	Test Purpose and Environment	5675

A.16.7.3.1.2	Test parameters	5676

A.16.7.3.1.3	Test Requirements	5680

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5680

A.16.7.3.2.1	Test Purpose and Environment	5680

A.16.7.3.2.2	Test parameters	5681

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5685

A.16.7.3.3.1	Test Purpose and Environment	5685

A.16.7.3.3.2	Test parameters	5685

A.16.7.3.3.3	Test Requirements	5690

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5691

A.16.7.3.4.1	Test Purpose and Environment	5691

A.16.7.3.4.2	Test parameters	5691

A.16.7.3.4.3	Test Requirements	5695

A.16.7.4	L1-RSRP measurement for beam reporting	5696

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5696

A.16.7.4.1.1	Test Purpose and Environment	5696

A.16.7.4.1.2	Test parameters	5696

A.16.7.4.1.3	Test Requirements	5700

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5700

A.16.7.4.2.1	Test Purpose and Environment	5700

A.16.7.4.2.2	Test parameters	5700

A.16.7.4.2.3	Test Requirements	5701

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5701

A.16.7.4.3.1	Test Purpose and Environment	5701

A.16.7.4.3.2	Test parameters	5701

A.16.7.4.3.3	Test Requirements	5705

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5705

A.16.7.4.4.1	Test Purpose and Environment	5705

A.16.7.4.4.2	Test parameters	5706

A.16.7.4.4.3	Test Requirements	5706

A.16.7.5	E-UTRAN RSRP	5706

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5706

A.16.7.5.1.1	Test Purpose and Environment	5706

A.16.7.5.1.2	Test parameters	5706

A.16.7.5.1.3	Test Requirements	5712

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5713

A.16.7.5.2.1	Test Purpose and Environment	5713

A.16.7.5.2.2	Test parameters	5713

A.16.7.5.2.3	Test Requirements	5719

A.16.7.6	E-UTRAN RSRQ	5720

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5720

A.16.7.6.1.1	Test Purpose and Environment	5720

A.16.7.6.1.2	Test parameters	5720

A.16.7.6.1.3	Test Requirements	5726

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5727

A.16.7.6.2.1	Test Purpose and Environment	5727

A.16.7.6.2.2	Test parameters	5727

A.16.7.6.2.3	Test Requirements	5733

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5734

A.17.1	SA: RRC_IDLE state mobility for RedCap	5734

A.17.1.1	Cell re-selection to NR	5734

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5734

A.17.1.1.1.1	Test Purpose and Environment	5734

A.17.1.1.1.2	Test Parameters	5734

A.17.1.1.1.3	Test Requirements	5738

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5738

A.17.1.1.2.1	Test Purpose and Environment	5738

A.17.1.1.2.2	Test Parameters	5738

A.17.1.1.2.3	Test Requirements	5742

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5742

A.17.1.1.3.1	Test Purpose and Environment	5742

A.17.1.1.3.2	Test Parameters	5742

A.17.1.1.3.3	Test Requirements	5745

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5745

A.17.1.1.4.1	Test Purpose and Environment	5745

A.17.1.1.4.2	Test Parameters	5745

A.17.1.1.4.3	Test Requirements	5749

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5749

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5749

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5749

A.17.2.1.1.1	Test Purpose and Environment	5749

A.17.2.1.1.2	Test Requirements	5753

A.17.3	RRC_CONNECTED state mobility for RedCap	5754

A.17.3.1	Handover for RedCap	5754

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5754

A.17.3.1.1.1	Test Purpose and Environment	5754

A.17.3.1.1.2	Test Parameters	5754

A.17.3.1.1.3	Test Requirements	5756

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5756

A.17.3.1.2.1	Test Purpose and Environment	5756

A.17.3.1.2.2	Test Parameters	5756

A.17.3.1.2.3 Test Requirements	5759

A.17.3.2	RRC Connection Mobility Control for RedCap	5759

A.17.3.2.1	SA: RRC Re-establishment	5759

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5759

A.17.3.2.1.1.1	Test Purpose and Environment	5759

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5762

A.17.3.2.1.2.1	Test Purpose and Environment	5762

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5765

A.17.3.2.1.3.1	Test Purpose and Environment	5765

A.17.3.2.1.3.2	Test Requirements	5767

A.17.3.2.2	Random Access	5768

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5768

A.17.3.2.2.1.1	Test Purpose and Environment	5768

A.17.3.2.2.1.2	Test Requirements	5770

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5772

A.17.3.2.2.2.1	Test Purpose and Environment	5772

A.17.3.2.2.2.2	Test Requirements	5774

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5776

A.17.3.2.2.3.1	Test Purpose and Environment	5776

A.17.3.2.2.3.2	Test Requirements	5778

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5779

A.17.3.2.2.4.1	Test Purpose and Environment	5779

A.17.3.2.2.4.2	Test Requirements	5781

A.17.3.2.3	SA: RRC Connection Release with Redirection	5782

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5782

A.17.3.2.3.1.1	Test Purpose and Environment	5782

A.17.3.2.3.1.2	Test Parameters	5782

A.17.3.2.3.1.3	Test Requirements	5786

A.17.4	Timing	5786

A.17.4.1	UE transmit timing	5786

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5786

A.17.4.1.1.1	Test Purpose and environment	5786

A.17.4.1.1.2	Test requirements	5789

A.17.4.2	UE timer accuracy	5790

A.17.4.3	Timing advance	5790

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5790

A.17.4.3.1.1	Test Purpose and Environment	5790

A.17.4.3.1.2	Test Parameters	5790

A.17.4.3.1.3 Test Requirements	5793

A.17.5	Signaling characteristics for RedCap	5793

A.17.5.1	Radio link Monitoring for RedCap	5793

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5793

A.17.5.1.1.1	Test Purpose and Environment	5793

A.17.5.1.1.2	Test Requirements	5796

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5797

A.17.5.1.2.1	Test Purpose and Environment	5797

A.17.5.1.2.2	Test Requirements	5801

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5802

A.17.5.1.3.1	Test Purpose and Environment	5802

A.17.5.1.3.2	Test Requirements	5806

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5806

A.17.5.1.4.1	Test Purpose and Environment	5806

A.17.5.1.4.2	Test Requirements	5811

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5811

A.17.5.1.5.1	Test Purpose and Environment	5811

A.17.5.1.5.2	Test Requirements	5816

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5816

A.17.5.1.6.1	Test Purpose and Environment	5816

A.17.5.1.6.2	Test Requirements	5820

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5820

A.17.5.1.7.1	Test Purpose and Environment	5820

A.17.5.1.7.2	Test Requirements	5824

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5824

A.17.5.1.8.1	Test Purpose and Environment	5824

A.17.5.1.8.2	Test Requirements	5829

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5829

A.17.5.1.9.1	Test Purpose and Environment	5829

A.17.5.1.9.2	Test Requirements	5832

A.17.5.2	Beam Failure Detection and Link recovery procedures	5832

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5832

A.17.5.2.1.1	Test Purpose and Environment	5832

A.17.5.2.1.2	Test Requirements	5836

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5836

A.17.5.2.2.1	Test Purpose and Environment	5836

A.17.5.2.2.2	Test Requirements	5840

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5840

A.17.5.2.3.1	Test Purpose and Environment	5840

A.17.5.2.3.2	Test Requirements	5844

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5844

A.17.5.2.4.1	Test Purpose and Environment	5844

A.17.5.2.4.2	Test Requirements	5848

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5848

A.17.5.2.5.1	Test Purpose and Environment	5848

A.17.5.2.5.2	Test Requirements	5852

A.17.5.3	Active BWP switch for RedCap	5852

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5852

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5852

A.17.5.3.1.1.1	Test Purpose and Environment	5852

A.17.5.3.1.1.2	Test Requirements	5855

A.17.5.3.2	RRC-based Active BWP Switch	5855

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5855

A.17.5.3.2.1.1	Test Purpose and Environment	5855

A.17.5.3.2.1.2	Test Requirements	5858

A.17.5.4	Active TCI state switch delay	5859

A.17.5.4.1	MAC-CE based active TCI state switch	5859

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5859

A.17.5.4.1.1.1	Test Purpose and Environment	5859

A.17.5.4.1.1.2	Test Requirements	5862

A.17.5.4.2	RRC based active TCI state switch	5862

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5862

A.17.5.4.2.1.1	Test Purpose and Environment	5862

A.17.5.4.2.1.2	Test Requirements	5866

A.17.5.5	Uplink spatial relation switch delay	5866

A.17.5.5.1	MAC-CE based Spatial Relation switch	5866

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5866

A.17.5.5.1.1.1	Test Purpose and Environment	5866

A.17.5.5.1.1.2	Test Requirements	5869

A.17.5.5.2	RRC based spatial relation switch	5869

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5869

A.17.5.5.2.1.2	Test Requirements	5872

A.17.5.6	UE specific CBW change	5872

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5872

A.17.5.6.1.1	Test Purpose and Environment	5872

A.17.5.6.1.2	Test Requirements	5875

A.17.6	Measurement procedure for RedCap	5876

A.17.6.1	Intra-frequency Measurements	5876

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5876

A.17.6.1.1.1	Test purpose and Environment	5876

A.17.6.1.1.2	Test Requirements	5879

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5879

A.17.6.1.2.1	Test purpose and Environment	5879

A.7.6.1.2.2	Test Requirements	5879

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5880

A.17.6.1.3.1	Test purpose and Environment	5880

A.17.6.1.3.2	Test Requirements	5884

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5884

A.17.6.1.4.1	Test purpose and Environment	5884

A.17.6.1.4.2	Test Requirements	5887

A.17.6.2	Inter-frequency Measurements	5888

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5888

A.17.6.2.1.1	Test Purpose and Environment	5888

A.17.6.2.1.2	Test Requirements	5891

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5891

A.17.6.2.2.1	Test Purpose and Environment	5891

A.17.6.2.2.2	Test Requirements	5895

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5895

A.17.6.2.3.1	Test Purpose and Environment	5895

A.17.6.2.3.2	Test Requirements	5899

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5899

A.17.6.2.4.1	Test Purpose and Environment	5899

A.17.6.2.4.2	Test Requirements	5903

A.17.6.3	L1-RSRP measurement for beam reporting	5904

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5904

A.17.6.3.1.1	Test Purpose and Environment	5904

A.17.6.3.1.2	Test parameters	5904

A.17.6.3.1.3	Test Requirements	5904

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5904

A.17.6.3.2.1	Test Purpose and Environment	5904

A.17.6.3.2.2	Test parameters	5904

A.17.6.3.2.3	Test Requirements	5906

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5906

A.17.6.3.3.1	Test Purpose and Environment	5906

A.17.6.3.3.2	Test parameters	5907

A.17.6.3.3.3	Test Requirements	5909

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5910

A.17.6.3.4.1	Test Purpose and Environment	5910

A.17.6.3.4.2	Test parameters	5910

A.7.6.3.3.3	Test Requirements	5912

A.17.6.4	NR Measurements with autonomous gaps	5913

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5913

A.17.6.4.1.1	Test Purpose and Environment	5913

A.17.6.4.1.2	Test Requirements	5916

A.17.7	Measurement Performance requirements	5916

A.17.7.1	SS-RSRP	5916

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5916

A.17.7.1.1.1	Test Purpose and Environment	5916

A.17.7.1.1.2	Test parameters	5916

A.17.7.1.1.3	Test Requirements	5920

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5921

A.17.7.1.2.1	Test Purpose and Environment	5921

A.17.7.1.2.2	Test parameters	5921

A.17.7.1.2.3	Test Requirements	5925

A.17.7.2	SS-RSRQ	5926

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5926

A.17.7.2.1.1	Test Purpose and Environment	5926

A.17.7.2.1.2	Test Parameters	5926

A.17.7.2.1.3	Test Requirements	5928

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5928

A.17.7.2.2.1	Test Purpose and Environment	5928

A.17.7.2.2.2	Test parameters	5928

A.17.7.2.2.3	Test Requirements	5930

A.17.7.3	L1-RSRP measurement for beam reporting	5930

A.17.7.3.1	SSB based L1-RSRP measurement	5930

A.17.7.3.1.1	Test Purpose and Environment	5930

A.17.7.3.1.2	Test parameters	5931

A.17.7.3.1.3	Test Requirements	5931

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5931

A.17.7.3.2.1	Test Purpose and Environment	5931

A.17.7.3.2.2	Test parameters	5931

A.17.7.3.2.3	Test Requirements	5931

A.17.7.4	SS-SINR	5932

A.17.7.4.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5932

A.17.7.4.1.1	Test Purpose and Environment	5932

A.17.7.4.1.2	Test parameters	5932

A.17.7.4.1.3	Test Requirements	5934

A.17.7.4.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5934

A.17.7.4.2.1	Test Purpose and Environment	5934

A.17.7.4.2.2	Test Parameters	5934

A.17.7.4.2.3	Test Requirements	5936

A.18	E-UTRA standalone tests for NR RRM for RedCap	5936

A.18.1	RRC_IDLE state mobility	5936

A.18.1.1	Inter-RAT NR Cell re-selection	5936

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5936

A.18.1.1.1.1	Test Purpose and Environment	5936

A.18.1.1.1.2	Test Requirements	5941

A.18.2	RRC_CONNECTED state mobility	5942

A.18.2.1	Handover	5942

A.18.2.1.1	E-UTRAN - NR handover in FR1	5942

A.18.2.1.1.1	Test Purpose and Environment	5942

A.18.2.1.1.2	Test Requirements	5947

A.18.2.2	RRC connection release with redirection	5947

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5947

A.18.2.2.1.1	Test Purpose and Environment	5947

A.18.2.2.1.2	Test Parameters	5947

A.18.2.2.1.3	Test Requirements	5954

A.18.3	Measurement procedure	5954

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5954

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5954

A.18.3.1.1.1	Test Purpose and Environment	5954

A.18.3.1.1.2	Test Requirements	5959

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5959

A.18.3.1.2.1	Test Purpose and Environment	5959

A.18.3.1.2.2	Test Requirements	5965

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5965

A.18.3.1.3.1	Test Purpose and Environment	5965

A.18.3.1.3.2	Test Requirements	5971

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5971

A.18.3.1.4.1	Test Purpose and Environment	5971

A.18.3.1.4.2	Test Requirements	5977

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5977

A.18.3.1.5.1	Test Purpose and Environment	5977

A.18.3.1.5.2	Test Requirements	5979

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5980

A.18.3.1.6.1	Test Purpose and Environment	5980

A.18.3.1.6.2	Test Requirements	5982

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5983

A.18.3.1.7.1	Test Purpose and Environment	5983

A.18.3.1.7.2	Test Requirements	5985

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5986

A.18.3.1.8.1	Test Purpose and Environment	5986

A.18.3.1.8.2	Test Requirements	5988

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	5990

B.1	Conditions for NR RRC_IDLE state mobility	5990

B.1.1	Introduction	5990

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5990

B.1.2A	Conditions for measurements on NR intra-frequency cells under CCA for cell re-selection	5991

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5992

B.1.3A	Conditions for measurements on NR inter-frequency cells under CCA for cell re-selection	5992

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5992

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5993

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5994

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5994

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5994

B.2.1	Introduction	5994

B.2.1.1	General	5994

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5994

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5995

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5995

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5995

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	5996

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	5996

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	5996

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	5997

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	5997

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	5998

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	5998

B.2.2	Conditions for NR intra-frequency measurements	5998

B.2.3	Conditions for NR inter-frequency measurements	6000

B.2.4	Conditions for NR L1-RSRP reporting	6001

B.2.4.1	Conditions for SSB based L1-RSRP reporting	6001

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	6002

B.2.5	Conditions for RRC connection release with redirection to NR	6004

B.2.6	Void	6005

B.2.6.1	Void	6005

B.2.6.2	Void	6005

B.2.7	Conditions for SRS-RSRP measurements	6005

B.2.8	Conditions for NR L1-SINR reporting	6006

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	6006

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	6007

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	6007

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	6007

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	6009

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	6009

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	6010

B.2.9	Conditions for NR intra-frequency measurements under CCA	6011

B.2.10	Conditions for NR inter-frequency measurements under CCA	6011

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6011

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6011

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	6012

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	6013

B.2.14	Conditions for NR PRS-based measurements	6015

B.2.15	Conditions for NR intra-frequency measurements for RedCap	6017

B.2.16	Conditions for NR inter-frequency measurements for RedCap	6018

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6020

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6021

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6021

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6021

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6021

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6022

B.3	RRM Requirements Exceptions	6022

B.3.1	Introduction	6022

B.3.2	Receiver sensitivity relaxation for CA	6022

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6022

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6022

B.3.2.2.1	Inter-band carrier aggregation	6022

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6022

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6023

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6023

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6023

B.3.2.4.1	Intra-band contiguous carrier aggregation	6023

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6023

B.3.3	Receiver sensitivity relaxation for DC	6023

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6023

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6023

B.3.4	Receiver sensitivity relaxation for SUL	6023

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6023

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6024

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6024

B.4	Conditions for V2X	6024

B.4.1	Test parameters for GNSS signals	6024

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6024

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6025

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6025

B.5	High level test procedure for SAN RRM tests	6025

Annex C (informative):	Change history	6027