| 3GPP TS 38.133 V19.3.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	170

1	Scope	172

2	References	172

3	Definitions, symbols and abbreviations	174

3.1	Definitions	174

3.2	Symbols	175

3.3	Abbreviations	176

3.4	Test tolerances	180

3.5	Frequency bands grouping	180

3.5.1	Introduction	180

3.5.2	NR operating bands in FR1	180

3.5.2A	NR operating bands for satellite access in FR1	148

3.5.3	NR operating bands in FR2	148

3.6	Applicability of requirements in this specification version	149

3.6.1	RRC connected state requirements in DRX	149

3.6.2	Number of serving carriers	150

3.6.2.1	Number of serving carriers for SA	150

3.6.2.2	Number of serving carriers for EN-DC	150

3.6.2.3	Number of serving carriers for NE-DC	150

3.6.2.4	Number of serving carriers for NR-DC	150

3.6.3	Applicability for intra-band FR2	150

3.6.4	Applicability for FR2 UE power classes	150

3.6.5	Applicability for SDL bands	151

3.6.6	Applicability of requirements for NGEN-DC operation	151

3.6.7	Applicability of QCL	151

3.6.9	Applicability of requirements for scheduling availability	152

3.6.10	Applicability of requirements for measurement restrictions	152

3.6.11	Applicability of requirements for Redcap UEs	152

3.6.11.1	RRC connected state requirements in DRX	152

3.6.11.2	Applicability for FR2 Redcap UE power classes	152

3.6.11.3	Applicability of QCL	152

3.6.12	Applicability of requirements for Satellite Access	152

3.6.13	Applicability of requirements for FR2	152

3.6.14	Applicability of requirements for FR2 Power Class 6	153

3.6.15	Applicability of requirements for per-FR gap	153

3.6.16	Applicability of requirements for ATG	153

3.6.17	Applicability of requirements for MUSIM gaps	153

3.6.18	Applicability of requirements for a UE operating on a cell with less than 5 MHz BW	153

3.6.19	Applicability of requirements for multi-Rx operation in FR2-1	153

3.6.20 Applicability of requirements for RedCap UE with satellite access	153

3.6.21	Applicability of requirements for UE supporting L3 fast beam sweeping operation in FR2-1	154

3.6.22	Applicability of requirements for UE with LP-WUR	154

3.6.23	Applicability of requirements for SBFD	154

4	SA: RRC_IDLE state mobility	154

4.1	Cell Selection	154

4.2	Cell Re-selection	155

4.2.1	Introduction	155

4.2.2	Requirements	155

4.2.2.1	UE measurement capability	155

4.2.2.2	Measurement and evaluation of serving cell	155

4.2.2.3	Measurements of intra-frequency NR cells	157

4.2.2.4	Measurements of inter-frequency NR cells	161

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	166

4.2.2.6	Maximum interruption in paging reception	168

4.2.2.7	General requirements	169

4.2.2.8	Minimum requirement at transitions	169

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	170

4.2.2.9.1	Introduction	170

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	170

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	172

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	174

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	175

4.2.2.10.1	Introduction	175

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	175

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	177

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	180

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	180

4.2.2.11.1	Introduction	180

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	181

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	182

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	184

4.2.2.12	 Measurements of inter-frequency NR cells with NTN carrier	184

4.2A	Cell Re-selection when subject to CCA	187

4.2A.1	Introduction	187

4.2A.2	Requirements	187

4.2A.2.1	UE measurement capability	187

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	188

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	189

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	190

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	192

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	192

4.2A.2.7	General requirements	192

4.2B	Cell Re-selection for RedCap	193

4.2B.1	Introduction	193

4.2B.2	Requirements	193

4.2B.2.1	UE measurement capability for RedCap	193

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	193

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	193

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	193

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	195

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	197

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	200

4.2B.2.6	Maximum interruption in paging reception for RedCap	202

4.2B.2.7	General requirements for RedCap	202

4.2B.2.8	Minimum requirement at transitions	202

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	203

4.2B.2.9.1	Introduction	203

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	204

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	206

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	207

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	207

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	207

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	208

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	208

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	208

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	208

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	211

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	213

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	214

4.2B.2.10.1	Introduction	214

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	214

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary  criterion	216

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	217

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	217

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	217

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	218

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	218

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	218

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	218

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	221

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	223

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	223

4.2B.2.11.1	Introduction	223

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	224

4.2B.2.11.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	225

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	225

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	226

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary  criteria	226

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	226

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	226

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	227

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	227

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	228

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	229

4.2C	Cell Re-selection for NR UE for Satellite Access	230

4.2C.1	Introduction	230

4.2C.2	Requirements	230

4.2C.2.1	UE measurement capability	230

4.2C.2.2	Measurement and evaluation of serving cell	230

4.2C.2.3	Measurements of intra-frequency NR cells	231

4.2C.2.4	Measurements of inter-frequency NR cells	234

4.2C.2.5	Maximum interruption in paging reception	237

4.2C.2.6	Minimum requirement at transitions	238

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	238

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	238

4.2C.2.9	General requirements	238

4.2C.2.10	Measurements of inter-frequency NR cells with TN carrier	238

4.2C.2.11	Measurements of inter-RAT E-UTRAN cells with TN carrier	241

4.2C.3	Void	242

4.2C.4	Void	242

4.2D	Cell Re-selection for ATG	242

4.2D.1	Introduction	242

4.2D.2	Requirements	243

4.2D.2.1	UE measurement capability	243

4.2D.2.2	Measurement and evaluation of serving cell	243

4.2D.2.3	Measurements of intra-frequency NR cells	243

4.2D.2.4	Measurements of inter-frequency NR cells	244

4.2D.2.5	Maximum interruption in paging reception	246

4.2D.2.6	General requirements	246

4.2E	Cell Re-selection for NR RedCap UE with Satellite Access	247

4.2E.1	Introduction	247

4.2E.2	Requirements for RedCap UE with Satellite Access	247

4.2E.2.1	UE measurement capability for RedCap with Satellite Access	247

4.2E.2.1.1	UE measurement capability for 1Rx RedCap UEs	247

4.2E.2.1.2	UE measurement capability for 2Rx RedCap UEs	247

4.2E.2.2	Measurement and evaluation of serving cell for RedCap UEs	247

4.2E.2.3	Measurements of intra-frequency NR cells for RedCap UE	249

4.2E.2.4	Measurements of inter-frequency NR cells for RedCap UE	251

4.2E.2.5	Maximum interruption in paging reception	255

4.2E.2.6	Minimum requirement at transitions for RedCap UE	255

4.2E.2.7	Measurements of intra-frequency NR cells for RedCap UE configured with relaxed measurement criterion	256

4.2E.2.7.1	Introduction	256

4.2E.2.7.2	Measurements for UE fulfilling low mobility criterion	256

4.2E.2.7.3	Measurements for UE fulfilling not-at-cell edge criterion	256

4.2E.2.7.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	256

4.2E.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	257

4.2E.2.8.1	Introduction	257

4.2E.2.8.2	Measurements for UE fulfilling low mobility criterion	257

4.2E.2.8.3	Measurements for UE fulfilling not-at-cell edge criterion	257

4.2E.2.8.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	258

4.2E.2.9	General requirements	258

4.2E.2.10	Measurements of inter-frequency NR cells with TN carrier	258

4.2E.2.11	Measurements of inter-RAT E-UTRAN cells with TN carrier	261

4.3	Minimization of Drive Tests (MDT)	263

4.3.1	Introduction	263

4.3.2	Measurement Requirements	263

4.3.3	Requirements for Relative Time Stamp Accuracy	264

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	264

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	264

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	264

4.3C.1	Introduction	264

4.3C.2	Measurement Requirements	265

4.3C.3	Requirements for Relative Time Stamp Accuracy	265

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	265

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	266

4.3D	Minimization of Drive Tests (MDT) for NR RedCap UE with Satellite Access	266

4.3D.1	Introduction	266

4.3D.2	Measurement Requirements	266

4.3D.3	Requirements for Relative Time Stamp Accuracy	266

4.3D.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	266

4.3D.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	266

4.4	Idle Mode CA/DC Measurements	267

4.4.1	Introduction	267

4.4.2	Measurement Requirements	267

4.4.2.1	Detected cell requirement during state transition and Idle mode	267

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	267

4.4.2.3	Measurements on serving cell	268

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	269

4.5	NR measurements for positioning	269

4.5.1	Introduction	269

4.5.2	RSTD measurements	270

4.5.2.1	Introduction	270

4.5.2.2	Requirements Applicability	270

4.5.2.3	Measurement Capability	270

4.5.2.4	Measurement Reporting Requirements	270

4.5.2.5	Measurements Period Requirements	270

4.5.2.6	Measurements Period Requirements with Bandwidth Aggregation	273

4.5.3	PRS-RSRP measurements	277

4.5.3.1	Introduction	277

4.5.3.2	Requirements applicability	277

4.5.3.3	Measurement Capability	277

4.5.3.4	Measurement Reporting Requirements	277

4.5.3.5	Measurement Period Requirements	278

4.5.4	PRS-RSRPP measurements	280

4.5.4.1	Introduction	280

4.5.4.2	Requirements Applicability	280

4.5.4.3	Measurement Capability	280

4.5.4.4	Measurement Reporting Requirements	280

4.5.4.5	Measurement Period Requirements	281

4.5.5	Measurement requirements for DL RSCPD reported with RSTD	281

4.5.5.1	Introduction	281

4.5.5.2	Requirements Applicability	281

4.5.5.3	Measurement Capability	281

4.5.5.4	Measurement Reporting Requirements	281

4.5.5.5	Measurements Period Requirements	282

4.6	NR measurements for positioning for RedCap	286

4.6.1	Introduction	286

4.6.2	RSTD measurements for RedCap	286

4.6.2.1	Introduction	286

4.6.2.2	Requirements Applicability	286

4.6.2.3	Measurement Capability	287

4.6.2.4	Measurement Reporting Requirements	287

4.6.2.5	Measurements Period Requirements without RX FH	287

4.6.2.6	Measurement Period Requirements with RX FH	288

4.6.3	PRS-RSRP measurements for RedCap	290

4.6.3.1	Introduction	290

4.6.3.2	Requirements applicability	290

4.6.3.3	Measurement Capability	290

4.6.3.4	Measurement Reporting Requirements	290

4.6.3.5	Measurement Period Requirements without RX FH	290

4.6.3.6	Measurement Period Requirements with RX FH	293

4.6.4	PRS-RSRPP measurements for RedCap	295

4.6.4.1	Introduction	295

4.6.4.2	Requirements Applicability	295

4.6.4.3	Measurement Capability	295

4.6.4.4	Measurement Reporting Requirements	295

4.6.4.5	Measurement Period Requirements without RX FH	296

4.6.4.6	Measurement Period Requirements with RX FH	296

4.7	Measurement report for fast CA/DC setup	296

4.7.1	Introduction	296

4.7.2	Void	296

4.7.3	Measurement Report Requirements	296

4.8	IDLE mode measurement for LP-WUS operation	297

4.8.1	Introduction	297

4.8.2 Requirements	297

4.8.2.1 UE Measurement Capability	297

4.8.2.1.1 LP-WUR measurement capability	297

4.8.2.1.2 MR measurement capability with LP-WUR	297

4.8.2.2 LP-WUR Serving cell measurement and evaluation requirement	297

4.8.2.2.1 General description	297

4.8.2.2.2 LP-WUR measurement and evaluation requirements for PSS/SSS	298

4.8.2.2.3 LP-WUR measurement and evaluation requirements for LP-SS	298

4.8.2.3	Measurement and evaluation of serving cell by MR	299

4.8.2.3.1	Requirements for evaluation of cell selection criterion	299

4.8.2.3.2	Requirements for evaluation of LP-WUS related conditions	299

4.8.2.3A	Measurement and evaluation of serving cell by RedCap UE	300

4.8.2.3A.1	Requirements for evaluation of cell selection criterion for RedCap UE	300

4.8.2.3A.2	Requirements for evaluation of LP-WUS related conditions for RedCap UE	300

4.8.2.4 Measurements of intra-frequency NR cells for UE with LP-WUR	300

4.8.2.4A	Measurements of intra-frequency NR cells for RedCap UE with LP-WUR	301

4.8.2.5	Measurements of inter-frequency NR cells for UE with LP-WUR	301

4.8.2.5.1	Introduction	301

4.8.2.5.2	Measurements for UE with LP-WUR fulfilling relaxed measurement criterion	301

4.8.2.5.3	Measurements for UE with LP-WUR fulfilling serving cell measurement offloading criterion	302

4.8.2.5A	Measurements of inter-frequency NR cells for Redcap with LP-WUR	303

4.8.2.5A.1	Introduction	303

4.8.2.5A.2	Measurements for UE with LP-WUR fulfilling relaxed measurement criterion	303

4.8.2.5A.3	Measurements for UE with LP-WUR fulfilling serving cell measurement offloading criterion	303

4.8.2.6     Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR	303

4.8.2.6.1	Introduction	303

4.8.2.6.2	Measurements for UE fulfilling relaxed measurement criteria	303

4.8.2.6.3	Measurements for UE fulfilling serving cell measurement offloading entry criteria	304

4.8.2.6A   Measurements of inter-RAT E-UTRAN cells for RedCap with LP-WUR	305

4.8.2.6A.1	Introduction	305

4.8.2.6A.2	Measurements for UE fulfilling relaxed measurement criteria	305

4.8.2.6A.3	Measurements for UE fulfilling serving cell measurement offloading entry criteria	305

5	SA: RRC_INACTIVE state mobility	305

5.1	Cell Re-selection	305

5.1.1	Introduction	305

5.1.2	Requirements	306

5.1.2.1	UE measurement capability	306

5.1.2.2	Measurement and evaluation of serving cell	306

5.1.2.3	Measurements of intra-frequency NR cells	308

5.1.2.4	Measurements of inter-frequency NR cells	310

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	312

5.1.2.6	Maximum interruption in paging reception	313

5.1.2.7	General requirements	313

5.1.2.8	Measurement of inter-frequency NR cells with NTN carrier	314

5.1.2.9	Minimum requirement at transitions	314

5.1.2.10	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	314

5.1.2.11	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	315

5.1.2.12	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	316

5.1A	Cell Re-selection with CCA	316

5.1A.1	Introduction	316

5.1A.2	Requirements	317

5.1A.2.1	UE measurement capability	317

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	317

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	317

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	317

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	317

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	317

5.1A.2.7	General requirements	317

5.1B	Cell Re-selection for RedCap	317

5.1B.1	Introduction	317

5.1B.2	Requirements	317

5.1B.2.1	UE measurement capability	317

5.1B.2.2	Measurement and evaluation of serving cell	317

5.1B.2.3	Measurements of intra-frequency NR cells	320

5.1B.2.4	Measurements of inter-frequency NR cells	322

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	324

5.1B.2.6	Maximum interruption in paging reception	325

5.1B.2.7	General requirements	325

5.1B.2.8	Minimum requirement at transitions	325

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	325

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	327

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	330

5.1C	Cell Re-selection for Satellite Access	331

5.1C.1	Introduction	331

5.1C.2	Requirements	331

5.1C.2.1	UE measurement capability	331

5.1C.2.2	Measurement and evaluation of serving cell	331

5.1C.2.3	Measurements of intra-frequency NR cells	331

5.1C.2.4	Measurements of inter-frequency NR cells	332

5.1C.2.5	Maximum interruption in paging reception	332

5.1C.2.6	General requirements	332

5.1C.2.7	Measurements of inter-frequency NR cells with TN carrier	332

5.1C.2.8	Measurements of inter-RAT E-UTRAN cells with TN carrier	332

5.1C.3	Void	332

5.1C.4	Void	332

5.1D	Cell Re-selection for ATG	332

5.1D.1	Introduction	332

5.1D.2	Requirements	332

5.1D.2.1	UE measurement capability	332

5.1D.2.2	Measurement and evaluation of serving cell	332

5.1D.2.3	Measurements of intra-frequency NR cells	332

5.1D.2.4	Measurements of inter-frequency NR cells	332

5.1D.2.5	Maximum interruption in paging reception	333

5.1D.2.6	General requirements	333

5.1E	Cell Re-selection for RedCap UE with Satellite Access	333

5.1E.1	Introduction	333

5.1E.2	Requirements	333

5.1E.2.1	UE measurement capability	333

5.1E.2.2	Measurement and evaluation of serving cell	333

5.1E.2.3	Measurements of intra-frequency NR cells	334

5.1E.2.4	Measurements of inter-frequency NR cells	335

5.1E.2.5	Maximum interruption in paging reception	335

5.1E.2.6	General requirements	335

5.1E.2.7	Minimum requirement at transitions	335

5.1E.2.8	Measurements of inter-frequency NR cells with TN carrier	335

5.1E.2.9	Measurements of inter-RAT E-UTRAN cells with TN carrier	336

5.2	Void	336

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	336

5.2B.1	Introduction	336

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	336

5.2B.2.1	Void	336

5.2B.3	TA validation requirements for RedCap	336

5.2B.3.1	Void	337

5.2B.3.2	Void	337

5.2B.4	Scheduling restriction	337

5.2B.5	Applicability conditions for CG-SDT for RedCap	337

5.3	Minimization of Drive Tests (MDT)	338

5.3.1	Introduction	338

5.3.2	Measurement Requirements	338

5.3.3	Requirements for Relative Time Stamp Accuracy	338

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	338

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	338

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	338

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	339

5.3C.1	Introduction	339

5.3C.2	Measurement Requirements	339

5.3C.3	Requirements for Relative Time Stamp Accuracy	339

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	339

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	339

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	339

5.3D	Minimization of Drive Tests (MDT) for NR RedCap UE with Satellite Access	340

5.3D.1	Introduction	340

5.3D.2	Measurement Requirements	340

5.3D.3	Requirements for Relative Time Stamp Accuracy	340

5.3D.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	340

5.3D.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	340

5.3D.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	340

5.4	Inactive Mode CA/DC Measurements	341

5.4.1	Introduction	341

5.4.2	Measurement Requirements	341

5.4.2.1	Detected cell requirement during state transition and inactive mode	341

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	341

5.4.2.3	Measurements on serving cell	341

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	341

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	341

5.5.1	Introduction	341

5.5.2	Requirements on UE synchronization for small data transmissions	341

5.5.3	TA validation requirements	341

5.5.4	Scheduling restriction	343

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	343

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	343

5.5.4.3	Scheduling availability of UE performing measurements on FR2	343

5.5.5	Applicability conditions for SDT	344

The UE is allowed to delay the reception of PRS resources on the positioning frequency layer until the SDT session is completed if the measurement using PRS resource overlaps with the SDT resources.	344

5.5D	Configured Grant based Small Data Transmissions (CG-SDT) for ATG	344

5.5E	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UEs with NTN	344

5.5E.1	Introduction	344

5.5E.2	Requirements on UE synchronization for small data transmissions	344

5.5E.3	TA validation requirements	344

5.5E.4	Scheduling restriction	345

5.5E.5	Applicability conditions for SDT	345

5.6	NR measurements for positioning	345

5.6.1	Introduction	345

5.6.1A	Cell re-selection for positioning	346

5.6.1A.1	Measurement and evaluation of serving cell	346

5.6.1A.2	Measurements of intra-frequency NR cells	347

5.6.2	RSTD measurements	348

5.6.2.1	Introduction	348

5.6.2.2	Requirements Applicability	348

5.6.2.3	Measurement Capability	348

5.6.2.5	Measurements Period Requirements	349

5.6.2.6	Measurements Period Requirements with Bandwidth Aggregation	352

5.6.3	PRS-RSRP measurements	355

5.6.3.1	Introduction	355

5.6.3.2	Requirements applicability	355

5.6.3.3	Measurement Capability	355

5.6.3.4	Measurement Reporting Requirements	355

5.6.3.5	Measurement Period Requirements	356

5.6.4	UE Rx-Tx time difference measurements	359

5.6.4.1	Introduction	359

5.6.4.2	Requirements Applicability	359

5.6.4.3	Measurement Capability	359

5.6.4.4	Measurement Reporting Requirements	359

5.6.4.5	Measurement Period Requirements	359

5.6.4.6	Measurement Period Requirements with Bandwidth Aggregation	363

5.6.5	PRS-RSRPP measurements	366

5.6.5.1	Introduction	366

5.6.5.2	Requirements Applicability	366

5.6.5.3	Measurement Capability	366

5.6.5.4	Measurement Reporting Requirements	366

5.6.5.5	Measurement Period Requirements	367

5.6.6	TA validation requirements for positioning	367

5.6.6.1	Introduction	367

5.6.6.2	TA validation requirements	367

5.6.6.3	TA validation requirements when configured with validity area	368

5.6.7	Measurement requirements for DL RSCPD reported with RSTD	369

5.6.7.1	Introduction	369

5.6.7.2	Requirements Applicability	369

5.6.7.3	Measurement Capability	369

5.6.7.4	Measurement Reporting Requirements	369

5.6.7.5	Measurements Period Requirements	370

5.6.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	372

5.6.8.1	Introduction	372

5.6.8.2	Requirements Applicability	372

5.6.8.3	Measurement Capability	372

5.6.8.4	Measurement Reporting Requirements	372

5.6.8.5	Measurement Period Requirements	373

5.6A	NR measurements for positioning for RedCap	375

5.6A.1	Introduction	375

5.6A.2	Cell re-selection for positioning	376

5.6A.2.1	Measurement and evaluation of serving cell	376

5.6A.2.2	Measurements of intra-frequency NR cells	377

5.6A.3	TA validation requirements for positioning SRS	378

5.6A.3.1	Introduction	378

5.6A.3.2	TA validation requirements	378

5.6A.3.3	TA validation requirements when configured with validity area	378

5.6A.4	RSTD measurements for RedCap	378

5.6A.4.1	 Introduction	378

5.6A.4.2	Requirements applicability	378

5.6A.4.3	Measurement Capability	379

5.6A.4.4	Measurement Reporting Requirements	379

5.6A.4.5	Measurement Period Requirement without RX FH	379

5.6A.4.6	Measurement Period Requirement with RX FH	383

5.6A.5	PRS-RSRP measurements for RedCap	384

5.6A.5.1	Introduction	384

5.6A.5.2	Requirements applicability	385

5.6A.5.3	Measurement Capability	385

5.6A.5.4	Measurement Reporting Requirements	385

5.6A.5.5	Measurement Period Requirements without RX FH	386

5.6A.5.6	Measurement Period Requirement with RX FH	388

5.6A.6	UE Rx-Tx time difference measurements for RedCap	390

5.6A.6.1	Introduction	390

5.6A.6.2	Requirements Applicability	390

5.6A.6.3	Measurement Capability	391

5.6A.6.4	Measurement Reporting Requirements	391

5.6A.6.5	Measurement Period Requirements without RX FH	391

5.6A.6.6	Measurement Period Requirements with RX FH	392

5.6A.7	PRS-RSRPP measurements for RedCap	394

5.6A.7.1	 Introduction	394

5.6A.7.2	Requirements applicability	394

5.6A.7.3	Measurement Capability	394

5.6A.7.4	Measurement Reporting Requirements	394

5.6A.7.5	Measurement Period Requirements without FH	395

5.6A.7.6	Measurement period requirement with FH	395

5.7	Random access based Small Data Transmissions (RA-SDT)	395

5.7.1	Introduction	395

5.7.2	Requirements for small data transmissions based on 2-step RA	395

5.7.3	Requirements for small data transmissions based on 4-step RA	395

5.7.4	Applicability conditions for SDT	395

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	396

5.7B.1	Introduction	396

5.7B.2	Requirements for small data transmissions based on 2-step RA	396

5.7B.3	Requirements for small data transmissions based on 4-step RA	396

5.7B.4	Applicability conditions for RA-SDT for RedCap	396

5.7D	Random access based Small Data Transmissions (RA-SDT) for ATG	396

5.7E	Random access based Small Data Transmissions (RA-SDT) for RedCap UEs with NTN	396

5.7E.1	Introduction	396

5.7E.2	Requirements for small data transmissions based on 2-step RA	396

5.7E.3	Requirements for small data transmissions based on 4-step RA	397

5.7E.4	Applicability conditions for RA-SDT	397

5.8	Measurement report for fast CA/DC setup	397

5.8.1	Introduction	397

5.8.2	Void	397

5.8.3	Measurement Report Requirements	397

5.9	INACTIVE mode measurement for LP-WUS operation	397

5.9.1	Introduction	397

5.9.2 Requirements	398

5.9.2.1 UE measurement capability	398

5.9.2.1.1 LP-WUR measurement capability	398

5.9.2.1.2 MR measurement capability with LP-WUR	398

5.9.2.2	LP-WUR serving cell measurement and evaluation requirements	398

5.9.2.3	Measurement and evaluation of serving cell by MR	398

5.9.2.3A	 Measurement and evaluation of serving cell by Redcap	398

The requirements in clause 4.8.2.3A shall apply.5.9.2.4	Measurements of intra-frequency NR cells for UE with LP-WUR	398

5.9.2.4A	Measurements of intra-frequency NR cells for RedCap UE with LP-WUR	398

5.9.2.5	Measurements of inter-frequency NR cells for UE with LP-WUR	398

5.9.2.5A	Measurements of inter-frequency NR cells for Redcap with LP-WUR	398

5.9.2.6	Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR	398

5.9.2.6A	Measurements of inter-RAT E-UTRAN cells for Redcap with LP-WUR	398

6	RRC_CONNECTED state mobility	399

6.1	Handover	399

6.1.1	NR Handover	399

6.1.1.1	Introduction	399

6.1.1.2	NR FR1 - NR FR1 Handover	399

6.1.1.2.1	Handover delay	399

6.1.1.2.2	Interruption time	399

6.1.1.3	NR FR2- NR FR1 Handover	400

6.1.1.3.1	Handover delay	400

6.1.1.3.2	Interruption time	401

6.1.1.4	NR FR2- NR FR2 Handover	402

6.1.1.4.1	Handover delay	402

6.1.1.4.2	Interruption time	402

6.1.1.5	NR FR1- NR FR2 Handover	403

6.1.1.5.1	Handover delay	403

6.1.1.5.2	Interruption time	403

6.1.2	NR Handover to other RATs	404

6.1.2.1	NR – E-UTRAN Handover	404

6.1.2.1.1	Introduction	404

6.1.2.1.2	Handover delay	405

6.1.2.1.3	Interruption time	405

6.1.2.2	NR – UTRAN Handover	405

6.1.2.2.1	Introduction	405

6.1.2.2.2	Handover delay	405

6.1.2.2.3	Interruption time	406

6.1.3	NR DAPS Handover	406

6.1.3.1	Introduction	406

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	406

6.1.3.2.1	DAPS handover delay	407

6.1.3.2.2	Interruption time	407

6.1.3.3	NR FR2- NR FR1 DAPS Handover	409

6.1.3.3.1	DAPS handover delay	409

6.1.3.3.2	Interruption time	410

6.1.3.4	NR FR1- NR FR2 DAPS Handover	410

6.1.3.4.1	DAPS handover delay	410

6.1.3.4.2	Interruption time	411

6.1.4	NR Conditional Handover	411

6.1.4.1	Introduction	411

6.1.4.2	NR FR1 – NR FR1 conditional handover	411

6.1.4.2.2	Measurement time	412

6.1.4.3	NR FR2 – NR FR1 conditional handover	414

6.1.4.4	NR FR2 – NR FR2 conditional handover	414

6.1.4.4.1	Handover delay	414

6.1.4.4.2	Measurement time	414

6.1.4.4.3	Preparation time	415

6.1.4.4.4	Interruption time	416

6.1.4.5	NR FR1 – NR FR2 conditional handover	416

6.1.5	NR Handover with PSCell	416

6.1.5.1	Introduction	416

6.1.5.2	Handover with PSCell from NR SA to EN-DC	416

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	417

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	417

6.1.5.3	HO with PSCell from NE-DC to NE-DC	418

6.1.5.3.1	Handover delay	418

6.1.5.3.2	HO with PSCell - PCell Interruption time	418

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	418

6.1.5.4	HO with PSCell from NR-DC to NR-DC	418

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	420

6.1.5.5.1	Introduction	420

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	420

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	421

6.1.6	NR Conditional Handover including target MCG and target SCG	422

6.1.6.1	Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC	422

6.1.6.1.1	CHO with PSCell – PCell Interruption time	422

6.1.6.1.2	CHO with PSCell – PSCell change delay	423

6.1.6.2	Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC	423

6.1.6.2.2	CHO with PSCell – PSCell change delay	424

6.1.7	NR Conditional Handover including target MCG and candidate SCG	425

6.1.7.1	Conditional handover including target MCG and candidate SCG for CPC in FR1 NR-DC	425

6.1.7.1.1	PCell conditional handover delay	426

6.1.7.1.2	PSCell conditional change delay	427

6.1.7.2	Conditional handover including target MCG in FR1 and Candidate SCG for CPC in FR2 in NR-DC	428

6.1.7.2.1	PCell handover delay	428

6.1.7.2.2	PSCell conditional change delay	429

6.1A	Void	431

6.1A.1	Void	431

6.1A.1.1	Void	431

6.1A.1.2	Void	431

6.1A.1.2.1	Void	431

6.1A.1.2.2	Void	431

6.1B	Handover to target cell using CCA	431

6.1B.1	NR Handover	431

6.1B.1.1	Introduction	431

6.1B.1.2	NR FR1 - NR FR1 Handover	431

6.1B.1.2.1	Handover delay	431

6.1B.1.2.2	Interruption time	431

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	432

6.1B.1.3.1	Handover delay	432

6.1B.1.3.2	Interruption time	432

6.1B.1.4	NR FR1- NR FR2-2 Handover	433

6.1B.1.4.1	Handover delay	433

6.1B.1.4.2	Interruption time	434

6.1C	Handover for SAN	435

6.1C.1	NR SAN Handover	435

6.1C.1.1	Introduction	435

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	435

6.1C.1.2.1	Handover delay	435

6.1C.1.2.2	Interruption time	435

6.1C.1.3	NR SAN FR2-NTN – NR SAN FR2-NTN Handover	436

6.1C.1.3.1	Handover delay	436

6.1C.1.3.2	Interruption time	437

6.1C.2	NR SAN Conditional Handover	437

6.1C.2.1	Introduction	437

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	437

6.1C.2.2.1	Handover delay	438

6.1C.2.2.2	Measurement time	438

6.1C.2.2.3	Preparation time	439

6.1C.2.2.4	Interruption time	440

6.1C.2.3	NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurement criteria	440

6.1C.2.3.1	Handover delay	440

6.1C.2.3.2	Preparation time	441

6.1C.2.3.3	Interruption time	441

6.1C.2.4	NR SAN FR2-NTN – NR SAN FR2-NTN conditional handover	442

6.1C.3	NR SAN Satellite switching with re-synchronization	442

6.1C.3.1	Introduction	442

6.1C.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization	442

6.1C.3.2.1	Satellite switching delay	442

6.1C.3.2.2	Interruption time for hard satellite switch with re-sync	442

6.1C.3.3	NR SAN FR2 – NR SAN FR2 Satellite switching with re-synchronization	444

6.1C.3.3.1	Satellite switching delay	444

6.1C.3.3.2	Interruption time for hard satellite switch with re-sync	444

6.1C.3.3.3	Satellite switch delay for soft satellite switch with re-sync	444

6.1D	Handover for RedCap	445

6.1D.1	NR Handover	445

6.1D.1.1	Introduction	445

6.1D.1.2	NR FR1 - NR FR1 Handover	445

6.1D.1.2.1	Handover delay	445

6.1D.1.2.2	Interruption time	446

6.1D.1.3	NR FR2- NR FR2 Handover	447

6.1D.1.3.1	Handover delay	447

6.1D.1.3.2	Interruption time	447

6.1D.2	NR Handover to other RATs	448

6.1D.2.1	NR – E-UTRAN Handover	448

6.1E	Handover for ATG	449

6.1E.1	NR Handover	449

6.1E.1.1	Introduction	449

6.1E.1.2	NR FR1 - NR FR1 Handover	449

6.1E.1.2.1	Handover delay	449

6.1E.1.2.2	Interruption time	449

6.1E.2	NR Conditional Handover	450

6.1E.2.1	Introduction	450

6.1E.2.2	NR FR1 – NR FR1 conditional handover	450

6.1E.2.2.1	Handover delay	450

6.1E.2.2.2	Measurement time	450

6.1E.2.2.3	Preparation time	451

6.1E.2.2.4	Interruption time	451

6.1F	Handover for RedCap UE with satellite access	451

6.1F.1	NR SAN Handover	451

6.1F.1.1	Introduction	451

6.1F.1.2	NR SAN FR1 – NR SAN FR1 Handover	452

6.1F.1.2.1	Handover delay	452

6.1F.1.2.2	Interruption time	452

6.1F.2	NR SAN Conditional Handover	452

6.1F.2.1	Introduction	452

6.1F.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	452

6.1F.2.2.1	Handover delay	452

6.1F.2.2.2	Measurement time	453

6.1F.2.2.3	Preparation time	453

6.1F.2.2.4	Interruption time	453

6.1F.2.3	NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurement criteria	453

6.1F.2.3.1	Handover delay	453

6.1F.2.3.2	Preparation time	453

6.1F.2.3.3	Interruption time	453

6.1F.3	NR SAN Satellite switching with re-synchronization	453

6.1F.3.1	Introduction	453

6.1F.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization	453

6.1F.3.2.1	Satellite switching delay	454

6.1F.3.2.2	Interruption time for hard satellite switch with re-sync	454

6.1F.3.2.3	Satellite switch delay for soft satellite switch with re-sync	454

6.2	RRC Connection Mobility Control	454

6.2.1	SA: RRC Re-establishment	454

6.2.1.1	Introduction	454

6.2.1.2	Requirements	455

6.2.1.2.1	UE Re-establishment delay requirement	455

6.2.1A	RRC Re-establishment with CCA	457

6.2.1A.1	Introduction	457

6.2.1A.2	Requirements	457

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	457

6.2.1B	SA: RRC Re-establishment for RedCap	459

6.2.1B.1	Introduction	459

6.2.1B.2	Requirements	459

6.2.2	Random access	460

6.2.2.1	Introduction	460

6.2.2.2	Requirements for 4-step RA type	460

6.2.2.2.1	Contention based random access	460

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	460

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	460

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	460

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	461

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	461

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	461

6.2.2.2.2	Non-Contention based random access	461

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	461

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	461

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	462

6.2.2.2.3	UE behaviour when configured with supplementary UL	462

6.2.2.3	Requirements for 2-step RA type	462

6.2.2.3.1	Contention based random access	462

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	462

6.2.2.3.1.2	Correct behaviour when receiving MsgB	463

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	463

6.2.2.3.2	Non-Contention based random access	463

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	463

6.2.2.3.2.2	Correct behaviour when receiving MsgB	463

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	464

6.2.2.3.3	UE behaviour when configured with supplementary UL	464

6.2.2A	Random access when CCA is used on target frequency	464

6.2.2A.1	Introduction	464

6.2.2A.2	Requirements for 4-step RA type	464

6.2.2A.2.1	Contention based random access	464

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	464

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	465

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	465

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	465

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	465

6.2.2A.2.2	Non-Contention based random access	466

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	466

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	466

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	466

6.2.2A.3	Requirements for 2-step RA type	467

6.2.2A.3.1	Contention based random access	467

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	467

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	467

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	468

6.2.2A.3.2	Non-Contention based random access	468

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	468

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	468

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	469

6.2.2B	Random access for RedCap	469

6.2.2B.1	Introduction	469

6.2.2B.2	Requirements	469

6.2.2C	PDCCH ordered Random Access for LTM	469

6.2.2C.1	Introduction	469

6.2.2C.2	PDCCH ordered Random Access delay	470

6.2.3	SA: RRC Connection Release with Redirection	470

6.2.3.1	Introduction	470

6.2.3.2	Requirements	471

6.2.3.2.1	RRC connection release with redirection to NR	471

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	472

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	472

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	473

6.2.3A.1	Introduction	473

6.2.3A.2	Requirements	474

6.2.3A.2.1	RRC connection release with redirection to NR	474

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	474

6.2C	RRC Connection Mobility Control for Satellite Access	474

6.2C.1	SA: RRC Re-establishment for Satellite Access	474

6.2C.1.1	Introduction	474

6.2C.1.2	Requirements	474

6.2C.1.2.1	UE Re-establishment delay requirement	475

6.2C.1.2.2	UE Re-establishment delay requirement for VSAT	477

6.2C.2	Random access for satellite access	477

6.2C.2.1	Introduction	477

6.2C.2.2	Requirements for 4-step RA type	477

6.2C.2.2.1	Contention based random access	477

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	477

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	477

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	478

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	478

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	478

6.2C.2.2.1.6	Correct behaviour when Contention Resolution Timer expires	478

6.2C.2.2.2	Non-Contention based random access	478

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	478

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	479

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	479

6.2C.2.3	Requirements for 2-step RA type	479

6.2C.2.3.1	Contention based random access	479

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	479

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	480

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	480

6.2C.2.3.2	Non-Contention based random access	480

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	480

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	480

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	481

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	481

6.2C.3.1	Introduction	481

6.2C.3.2	Requirements	481

6.2C.3.2.1	RRC connection release with redirection to NR	481

6.2D	RRC Connection Mobility Control for ATG	482

6.2D.1	SA: RRC Re-establishment	482

6.2D.1.1	Introduction	482

6.2D.1.2	Requirements	482

6.2D.1.2.1	UE Re-establishment delay requirement	482

6.2D.2	Random access	483

6.2D.2.1	Introduction	483

6.2D.2.2	Requirements for 4-step RA type	484

6.2D.2.3	Requirements for 2-step RA type	484

6.2D.3	SA: RRC Connection Release with Redirection	484

6.2D.3.1	Introduction	484

6.2D.3.2	Requirements	484

6.2D.3.2.1	RRC connection release with redirection to NR	484

6.2E	RRC Connection Mobility Control for RedCap UE with Satellite Access	485

6.2E.1	SA: RRC Re-establishment for RedCap UE with Satellite Access	485

6.2E.1.1	Introduction	485

6.2E.1.2	Requirements	485

6.2E.2	Random access for RedCap UE with satellite access	486

6.2E.2.1	Introduction	486

6.2E.3	SA: RRC Connection Release with Redirection for RedCap UE with Satellite Access	486

6.2E.3.1	Introduction	486

6.2E.3.2	Requirements	486

6.2E.3.2.1	RRC connection release with redirection to NR	486

6.3	L1/L2-Triggered Mobility	487

6.3.1	LTM PCell Cell Switch	487

6.3.1.1	Introduction	487

6.3.1.2	LTM Cell Switch delay	488

6.3.1.3	Interruption time	489

6.3.2	Conditional L1/L2-Triggered Mobility	490

6.3.2.1	Introduction	490

6.3.2.2	CLTM Cell Switch delay	491

6.3.2.2.1	Measurement time	491

6.3.2.2.2	Conditional LTM RRC processing time	492

6.3.2.2.3	Interruption time	492

6.3.2.3	Subsequent CLTM Cell Switch delay	494

7	Timing	494

7.1	UE transmit timing	494

7.1.1	Introduction	494

7.1.2	Requirements	495

7.1.2.1	Gradual timing adjustment	497

7.1.2.2	Void	498

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	498

7.1.2.4	UE transmit timing for positioning measurements	499

7.1A	UE transmit timing for RedCap	499

7.1A.1	Introduction	499

7.1A.2	Requirements	499

7.1A.2.1	Gradual timing adjustment	500

7.1A.2.2	UE transmit timing for positioning measurements	501

7.1C	UE transmit timing for Satellite Access	501

7.1C.1	Introduction	501

7.1C.2	Requirements	501

7.1C.2.1	Gradual timing adjustment	503

7.1D	UE transmit timing for ATG	503

7.1D.1	Introduction	503

7.1D.2	Requirements	503

7.1D.2.1	Gradual timing adjustment	504

7.1E	UE transmit timing for RedCap with Satellite Access	504

7.1E.1	Introduction	504

7.1E.2	Requirements	505

7.1E.2.1	Gradual timing adjustment	505

The requirements in clause 7.1C.2.1 for FR1-NTN shall apply.7.2	UE timer accuracy	505

7.2.1	Introduction	505

7.2.2	Requirements	505

7.2A	UE timer accuracy for RedCap	505

7.2A.1	Introduction	505

7.2A.2	Requirements	505

7.2C	UE timer accuracy for satellite access	506

7.2C.1	Introduction	506

7.2C.2	Requirements	506

7.2D	UE timer accuracy for ATG	506

7.2D.1	Introduction	506

7.2D.2	Requirements	506

7.2E	UE timer accuracy for RedCap with Satellite Access	507

7.2E.1	Introduction	507

7.2E.2	Requirements	507

The requirements in clause 7.2C.2 for FR1-NTN shall apply.7.3	Timing advance	507

7.3.1	Introduction	507

7.3.2	Requirements	507

7.3.2.1	Timing Advance adjustment delay	507

7.3.2.2	Timing Advance adjustment accuracy	507

7.3A	Timing Advance for RedCap	507

7.3A.1	Introduction	507

7.3A.2	Requirements	508

7.3A.2.1	Timing Advance adjustment delay	508

7.3A.2.2	Timing Advance adjustment accuracy	508

7.3C	Timing advance for satellite access	508

7.3C.1	Introduction	508

7.3C.2	Requirements	508

7.3C.2.1	Timing Advance adjustment delay	508

7.3C.2.2	Timing Advance adjustment accuracy	508

7.3D	Timing advance for ATG	509

7.3D.1	Introduction	509

7.3D.2	Requirements	509

7.3D.2.1	Timing Advance adjustment delay	509

7.3D.2.2	Timing Advance adjustment accuracy	509

7.3E	Timing advance for RedCap with Satellite Access	509

7.3E.1	Introduction	509

7.3E.2	Requirements	509

7.3E.2.1	Timing Advance adjustment delay	509

7.3E.2.2	Timing Advance adjustment accuracy	509

7.4	Cell phase synchronization accuracy	509

7.4.1	Definition	509

7.4.2	Minimum requirements	510

7.5	Maximum Transmission Timing Difference	510

7.5.1	Introduction	510

7.5.2	Minimum requirements for inter-band EN-DC	510

7.5.2.1	Minimum requirements for inter-band synchronous EN-DC	510

7.5.3	Minimum requirements for intra-band EN-DC	511

7.5.4	Minimum requirements for NR Carrier Aggregation	512

7.5.5	Minimum requirements for inter-band NE-DC	513

7.5.5.1	Minimum requirements for inter-band synchronous NE-DC	513

7.5.6	Minimum requirements for inter-band NR-DC	513

7.5.7	Minimum requirements for multi-TRP	514

7.6	Maximum Receive Timing Difference	514

7.6.1	Introduction	514

7.6.2	Minimum requirements for inter-band EN-DC	515

7.6.2.1	Minimum requirements for inter-band synchronous EN-DC	515

7.6.3	Minimum requirements for intra-band EN-DC	517

7.6.4	Minimum requirements for NR Carrier Aggregation	517

7.6.5	Minimum requirements for inter-band NE-DC	518

7.6.5.1	Minimum requirements for inter-band synchronous NE-DC	519

7.6.6	Minimum requirements for inter-band NR-DC	519

7.6.7	Minimum requirements for PC6 UE in FR2	520

7.6.8	Minimum requirements for Multi-TRPs	520

7.6D	Maximum Receive Timing Difference for ATG UE	521

7.6D.1	Introduction	521

7.6D.2	Minimum requirements for NR Carrier Aggregation	521

7.7	deriveSSB-IndexFromCell tolerance	521

7.7.1	Minimum requirements	521

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	522

7.7A.1	Minimum requirements	522

7.7D	DeriveSSB-IndexFromCell tolerance for ATG	522

7.7D.1	Minimum requirements	522

7.8	Void	522

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	522

7.9.1	Minimum requirements	522

7.9D	DeriveSSB-IndexFromCellInter-r17 tolerance for ATG	523

7.9D.1	Minimum requirements	523

8	Signalling characteristics	559

8.1	Radio Link Monitoring	559

8.1.1	Introduction	559

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	560

8.1.2	Requirements for SSB based radio link monitoring	561

8.1.2.1	Introduction	561

8.1.2.2	Minimum requirement	562

8.1.2.3	Measurement restrictions for SSB based RLM	566

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	567

8.1.3	Requirements for CSI-RS based radio link monitoring	568

8.1.3.1	Introduction	568

8.1.3.2	Minimum requirement	568

8.1.3.3	Measurement restrictions for CSI-RS based RLM	573

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	575

8.1.4	Minimum requirement at transitions	575

8.1.5	Minimum requirement for UE turning off the transmitter	576

8.1.6	Minimum requirement for L1 indication	576

8.1.7	Scheduling availability of UE during radio link monitoring	576

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	576

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	577

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	577

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	578

8.1.8	Minimum requirement under IDC Interference	578

8.1A	Radio Link Monitoring with CCA on Target Frequency	578

8.1A.1	Introduction	578

8.1A.2	Requirements for SSB Based Radio Link Monitoring	579

8.1A.2.1	Introduction	579

8.1A.2.2	Minimum Requirement	580

8.1A.2.3	Measurement Restrictions for SSB based RLM	583

8.1A.3	Minimum requirement at transitions	583

8.1A.4	Minimum requirement for UE turning off the transmitter	584

8.1A.5	Minimum requirement for L1 indication	584

8.1A.6	Scheduling availability of UE during radio link monitoring	584

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	584

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	585

8.1B	Radio Link Monitoring for RedCap	585

8.1B.1	Introduction	585

8.1B.2	Requirements for SSB based radio link monitoring	586

8.1B.2.1	Introduction	586

8.1B.2.2	Minimum requirement	587

8.1B.2.3	Measurement restrictions for SSB based RLM	589

8.1B.3	Requirements for CSI-RS based radio link monitoring	589

8.1B.3.1	Introduction	589

8.1B.3.2	Minimum requirement	590

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	592

8.1B.4	Minimum requirement at transitions	593

8.1B.5	Minimum requirement for UE turning off the transmitter	593

8.1B.6	Minimum requirement for L1 indication	593

8.1B.7	Scheduling availability of UE during radio link monitoring	594

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	594

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	594

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	594

8.1C	Radio Link Monitoring for Satellite Access	595

8.1C.1	Introduction	595

8.1C.2	Requirements for SSB based radio link monitoring	596

8.1C.2.1	Introduction	596

8.1C.2.2	Minimum requirement	597

8.1C.2.3	Measurement restrictions for SSB based RLM	598

8.1C.3	Requirements for CSI-RS based radio link monitoring	599

8.1C.3.1	Introduction	599

8.1C.3.2	Minimum requirement	599

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	601

8.1C.4	Minimum requirement at transitions	601

8.1C.5	Minimum requirement for UE turning off the transmitter	601

8.1C.6	Minimum requirement for L1 indication	602

8.1C.7	Scheduling availability of UE during radio link monitoring	602

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN and FR2-NTN	602

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN and FR2-NTN	602

8.1D	Radio Link Monitoring for ATG	602

8.1D.1	Introduction	602

8.1D.2	Requirements for SSB based radio link monitoring	603

8.1D.2.1	Introduction	603

8.1D.2.2	Minimum requirement	604

8.1D.2.3	Measurement restrictions for SSB based RLM	605

8.1D.3	Requirements for CSI-RS based radio link monitoring	605

8.1D.3.1	Introduction	605

8.1D.3.2	Minimum requirement	605

8.1D.3.3	Measurement restrictions for CSI-RS based RLM	607

8.1D.4	Minimum requirement at transitions	607

8.1D.5	Minimum requirement for UE turning off the transmitter	607

8.1D.6	Minimum requirement for L1 indication	607

8.1D.7	Scheduling availability of UE during radio link monitoring	607

8.1D.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	607

8.1D.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	607

8.1E	Radio Link Monitoring for RedCap UE with Satellite Access	608

8.1E.1	Introduction	608

8.1E.2	Requirements for SSB based radio link monitoring	608

8.1E.2.1	Introduction	608

8.1E.2.2	Minimum requirement	609

8.1E.2.3	Measurement restrictions for SSB based RLM	609

8.1E.3	Requirements for CSI-RS based radio link monitoring	609

8.1E.3.1	Introduction	609

8.1E.3.2	Minimum requirement	610

8.1E.3.3	Measurement restrictions for CSI-RS based RLM	611

8.1E.4	Minimum requirement at transitions	611

8.1E.5	Minimum requirement for UE turning off the transmitter	611

8.1E.6	Minimum requirement for L1 indication	611

8.1E.7	Scheduling availability of UE during radio link monitoring	611

8.1E.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH	611

8.1E.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	611

8.2	Interruption	611

8.2.1	EN-DC Interruption	611

8.2.1.1	Introduction	611

8.2.1.2	Requirements	612

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	612

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	613

8.2.1.2.3	Interruptions at SCell addition/release	613

8.2.1.2.4	Interruptions at SCell activation/deactivation	615

8.2.1.2.5	Interruptions during measurements on SCC	618

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	620

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	620

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	621

8.2.1.2.9	Interruptions at SCell hibernation	622

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	622

8.2.1.2.11	Interruptions due to UE-specific CBW change	622

8.2.1.2.12	Interruptions at NR SRS carrier based switching	622

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	624

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	625

8.2.1.2.15	Interruptions due to SCell dormancy	625

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	626

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	626

8.2.1.2.18	Interruptions at NR SRS antenna port switching	627

8.2.1.2.19	Interruptions at fast SCell activation	628

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	629

8.2.1.2.21	Interruptions at OD-SSB activation/deactivation	629

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	630

8.2.2.1	Introduction	630

8.2.2.2	Requirements	631

8.2.2.2.1	Interruptions at SCell addition/release	631

8.2.2.2.2	Interruptions at SCell activation/deactivation	632

8.2.2.2.3	Interruptions during measurements on deactivated SCC	634

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	636

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	636

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	637

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	638

8.2.2.2.8	Interruptions due to UE-specific CBW change	638

8.2.2.2.9	Interruptions at NR SRS carrier based switching	639

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	640

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	641

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	641

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	642

8.2.2.2.10D	DL Interruptions at UE switching across three or four uplink bands	642

8.2.2.2.10E	DL Interruptions at UE switching between two uplink bands with three transmit antenna connectors and maximum two transmit antenna connectors for each band	643

8.2.2.2.11	Interruptions at direct SCell activation	643

8.2.2.2.12	Interruptions due to SCell dormancy	644

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	644

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	644

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	644

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	644

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	644

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	645

8.2.2.2.16	Interruptions at NR SRS antenna port switching	645

8.2.2.2.17	Interruptions at fast SCell activation	646

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	647

8.2.2.2.19	Interruptions due to measurements without gap carried out by UE supporting NeedForInterruptionInfoNR	647

8.2.2.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	648

8.2.2.2.21	Interruptions at NR SRS bandwidth aggregation for positioning	648

8.2.2.2.22	Interruptions at OD-SSB activation/deactivation	651

8.2.3	NE-DC Interruptions	652

8.2.3.1	Introduction	652

8.2.3.2	Requirements	652

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	652

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	653

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	653

8.2.3.2.4	Interruptions at SCell activation/deactivation	654

8.2.3.2.5	Interruptions during measurements on SCC	655

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	655

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	655

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	656

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	656

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	656

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	657

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	657

8.2.3.2.9	Interruptions at SCell hibernation	657

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	658

8.2.3.2.11	Interruptions at NR SRS carrier based switching	658

8.2.3.2.12	Interruptions at E-UTRA SRS carrier based switching	659

8.2.3.2.13	Interruptions due to SCell dormancy	660

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	660

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	661

8.2.3.2.17	Interruptions at fast SCell activation	663

8.2.3.2.18	Interruptions due to UE-specific CBW change	663

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	663

8.2.4	NR-DC: Interruptions	664

8.2.4.1	Introduction	664

8.2.4.2	Requirements	664

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	664

8.2.4.2.2	Interruptions at SCell activation/deactivation	665

8.2.4.2.3	Interruptions during measurements on SCC	666

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	666

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	667

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	667

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	667

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	668

8.2.4.2.9	Interruptions at NR SRS carrier based switching	668

8.2.4.2.10	Interruptions at direct SCell activation	669

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	670

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	670

8.2.4.2.13	 Interruptions due to SCell dormancy	671

8.2.4.2.14	Interruptions at NR SRS antenna port switching	671

8.2.4.2.15	Interruptions at fast SCell activation	672

8.2.4.2.16	Interruptions at SCG activation/deactivation	673

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	673

8.2.4.2.18	Interruptions during RLM/BFD measurements on deactivated PSCell	673

8.2.4.2.19	Interruptions due to UE-specific CBW change	673

8.2.4.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	674

8.2.4.2.21	Interruptions at PSCell Cell switch	674

8.2.4.2.22	Interruptions at OD-SSB activation/deactivation	674

8.2.4.2A	Void	675

8.2.4.2A.1	Void	675

8.2.4.2A.2	Void	675

8.2.4.2A.3	Void	675

8.2D	Interruption for ATG UE	675

8.2D.1	Interruptions with Standalone NR Carrier Aggregation	675

8.2D.1.1	Introduction	675

8.2D.1.2	Requirements	676

8.2D.1.2.1	Interruptions at SCell addition/release	676

8.2D.1.2.2	Interruptions at SCell activation/deactivation	676

8.2D.1.2.3	Interruptions during measurements on deactivated SCC	677

8.2D.1.2.4	Interruptions at direct SCell activation	677

8.2D.1.2.5	Interruptions due to SCell dormancy	678

8.2D.1.2.6	Interruptions at fast SCell activation	678

8.2D.1.2.8	Interruptions due to UE-specific CBW change	679

8.2D.1.2.9	Interruptions when identifying CGI of an NR cell with autonomous gaps	680

8.2D.1.2.10	Interruptions at NR SRS antenna port switching	680

8.3	SCell Activation and Deactivation Delay	681

8.3.1	Introduction	681

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	681

8.3.2A	SCell Activation Delay Requirement for Deactivated SCell based on measurement in IDLE/INACTIVE mode	689

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	691

8.3.4	Direct SCell Activation at SCell addition	691

8.3.5	Direct SCell Activation at Handover	693

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	695

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	699

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	699

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	700

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	702

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	706

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	708

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	708

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	709

8.3.17	SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation	711

8.3.18	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting	715

8.3.19	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell	718

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.8.3.20	OD-SSB based SCell Deactivation Delay Requirement for Activated SCell	722

8.3.21	OD-SSB based Direct SCell Activation at SCell addition	722

8.3.22	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	724

8.3.23	OD-SSB based SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	728

8.3.25	OD-SSB based SCell Deactivation Delay Requirement for Activated PUCCH SCell	729

8.3.26	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation	729

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	729

8.3A.1	Introduction	729

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	730

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	734

8.3D	SCell Activation and Deactivation Delay for ATG	734

8.3D.1	Introduction	734

8.3D.2	SCell Activation Delay Requirement for Deactivated SCell	734

8.3D.3	SCell Deactivation Delay Requirement for Activated SCell	739

8.3D.4	Direct SCell Activation at SCell addition	740

8.3D.5	Direct SCell Activation at Handover	741

8.3D.6	Direct SCell Activation at RRC Resume	743

8.3D.7	Fast SCell Activation Delay Requirement for Deactivated SCell	743

8.3D.8	SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation	744

8.4	UE UL carrier RRC reconfiguration delay	747

8.4.1	Introduction	747

8.4.2	UE UL carrier configuration delay requirement	747

8.4.3	UE UL carrier deconfiguration delay requirement	747

8.5	Link Recovery Procedures	748

8.5.1	Introduction	748

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	749

8.5.2	Requirements for SSB based beam failure detection	750

8.5.2.1	Introduction	750

8.5.2.2	Minimum requirement	751

8.5.2.3	Measurement restriction for SSB based beam failure detection	755

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	756

8.5.3	Requirements for CSI-RS based beam failure detection	756

8.5.3.1	Introduction	756

8.5.3.2	Minimum requirement	757

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	762

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	763

8.5.4	Minimum requirement for L1 indication	764

8.5.5	Requirements for SSB based candidate beam detection	765

8.5.5.1	Introduction	765

8.5.5.2	Minimum requirement	765

8.5.5.3	Measurement restriction for SSB based candidate beam detection	769

8.5.6	Requirements for CSI-RS based candidate beam detection	770

8.5.6.1	Introduction	770

8.5.6.2	Minimum requirement	771

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	775

8.5.7	Scheduling availability of UE during beam failure detection	776

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	776

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	776

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	777

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	778

8.5.8	Scheduling availability of UE during candidate beam detection	778

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	778

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	778

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	779

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	780

8.5.9	Requirements for Beam Failure Recovery in SCell	780

8.5.9.1	Introduction	780

8.5.9.2	Requirement	780

8.5.10	Minimum requirement at transitions for beam failure detection	781

8.5.11	Minimum requirement under IDC Interference	781

8.5.12	Minimum requirement at transitions for candidate beam detection	781

8.5A	Link Recovery Procedures when CCA is used on target frequency	781

8.5A.1	Introduction	781

8.5A.2	Requirements for SSB based beam failure detection	782

8.5A.2.1	Introduction	782

8.5A.2.2	Minimum requirement	782

8.5A.2.3	Measurement restriction for SSB based beam failure detection	785

8.5A.3	Void	785

8.5A.4	Minimum requirement for L1 indication	785

8.5A.5	Requirements for SSB based candidate beam detection	785

8.5A.5.1	Introduction	785

8.5A.5.2	Minimum requirement	785

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	788

8.5A.6	Void	788

8.5A.7	Scheduling availability of UE during beam failure detection	788

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	788

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	788

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	789

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	789

8.5A.8	Scheduling availability of UE during candidate beam detection	789

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	789

8.5.8A.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	789

8.5B	Link Recovery Procedures for Redcap	789

8.5B.1	Introduction	789

8.5B.2	Requirements for SSB based beam failure detection for Redcap	790

8.5B.2.1	Introduction	790

8.5B.2.2	Minimum requirement	790

8.5B.2.3	Measurement restriction for SSB based beam failure detection	792

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	792

8.5B.3.1	Introduction	792

8.5B.3.2	Minimum requirement	792

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	794

8.5B.4	Minimum requirement for L1 indication for Redcap	795

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	796

8.5B.5.1	Introduction	796

8.5B.5.2	Minimum requirement	796

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	797

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	798

8.5B.6.1	Introduction	798

8.5B.6.2	Minimum requirement	798

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	800

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	800

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	801

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	801

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	801

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	801

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	801

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	802

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	802

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	802

8.5C	Link Recovery Procedures for Satellite Access	802

8.5C.1	Introduction	802

8.5C.2	Requirements for SSB based beam failure detection	803

8.5C.2.1	Introduction	803

8.5C.2.2	Minimum requirement	804

8.5C.2.3	Measurement restriction for SSB based beam failure detection	804

8.5C.3	Requirements for CSI-RS based beam failure detection	805

8.5C.3.1	Introduction	805

8.5C.3.2	Minimum requirement	805

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	806

8.5C.4	Minimum requirement for L1 indication	807

8.5C.5	Requirements for SSB based candidate beam detection	807

8.5C.5.1	Introduction	807

8.5C.5.2	Minimum requirement	807

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	808

8.5C.6	Requirements for CSI-RS based candidate beam detection	808

8.5C.6.1	Introduction	808

8.5C.6.2	Minimum requirement	808

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	809

8.5C.7	Scheduling availability of UE during beam failure detection	810

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	810

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	810

8.5C.8	Scheduling availability of UE during candidate beam detection	810

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	810

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	810

8.5C.9	Minimum requirement at transitions for beam failure detection	811

8.5D	Link Recovery Procedures for ATG	811

8.5D.1	Introduction	811

8.5D.2	Requirements for SSB based beam failure detection	812

8.5D.2.1	Introduction	812

8.5D.2.2	Minimum requirement	812

8.5D.2.3	Measurement restriction for SSB based beam failure detection	813

8.5D.3	Requirements for CSI-RS based beam failure detection	814

8.5D.3.1	Introduction	814

8.5D.3.2	Minimum requirement	814

8.5D.3.3	Measurement restrictions for CSI-RS beam failure detection	815

8.5D.4	Minimum requirement for L1 indication	816

8.5D.5	Requirements for SSB based candidate beam detection	816

8.5D.5.1	Introduction	816

8.5D.5.2	Minimum requirement	816

8.5D.5.3	Measurement restriction for SSB based candidate beam detection	817

8.5D.6	Requirements for CSI-RS based candidate beam detection	818

8.5D.6.1	Introduction	818

8.5D.6.2	Minimum requirement	818

8.5D.6.3	Measurement restriction for CSI-RS based candidate beam detection	819

8.5D.7	Scheduling availability of UE during beam failure detection	820

8.5D.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	820

8.5D.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	820

8.5D.8	Scheduling availability of UE during candidate beam detection	820

8.5D.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	820

8.5D.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	820

8.5D.9	Minimum requirement at transitions for beam failure detection	821

8.5D.10	Requirements for Beam Failure Recovery in SCell	821

8.5E	Link Recovery Procedures for RedCap UE with Satellite Access	821

8.5E.1	Introduction	821

8.5E.2	Requirements for SSB based beam failure detection for RedCap UE with satellite access	821

8.5E.2.1	Introduction	821

8.5E.2.2	Minimum requirement	822

8.5E.2.3	Measurement restrictions for SSB beam failure detection	822

8.5E.3	Requirements for CSI-RS based beam failure detection for RedCap UE with satellite access	822

8.5E.3.1	Introduction	822

8.5E.3.2	Minimum requirement	823

8.5E.3.3	Measurement restrictions for CSI-RS beam failure detection	823

8.5E.4	Minimum requirement for L1 indication for RedCap UE with satellite access	823

8.5E.5	Requirements for SSB based candidate beam detection for RedCap UE with satellite access	824

8.5E.6	Requirements for CSI-RS based candidate beam detection for RedCap UE with satellite access	824

8.5E.7	Scheduling availability of UE during beam failure detection for RedCap UE with satellite access	824

8.5E.8	Scheduling availability of UE during candidate beam detection for RedCap UE with satellite access	824

8.5E.9	Minimum requirement at transitions for beam failure detection for RedCap UE with satellite access	824

8.6	Active BWP switch delay	824

8.6.1	Introduction	824

8.6.2	DCI and timer based BWP switch delay on a single CC	824

8.6.2A	DCI based BWP switch delay on multiple CCs	826

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	826

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	827

8.6.2B	Timer based BWP switch delay on multiple CCs	828

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	828

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	828

8.6.3	RRC based BWP switch delay on a single CC	829

8.6.3A	RRC based BWP switch delay on multiple CCs	830

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	830

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	830

8.6.4	BWP switch delay on Consistent UL CCA recovery	831

8.6A	Active BWP switch delay for RedCap	831

8.6A.1	Introduction	831

8.6A.2	DCI and timer based BWP switch delay on a single CC	831

8.6A.3	RRC based BWP switch delay on a single CC	833

8.6C	Active BWP switch delay for satellite access	833

8.6C.1	Introduction	833

8.6C.2	DCI and timer based BWP switch delay on a single CC	833

8.6C.3	RRC based BWP switch delay on a single CC	835

8.6D	Active BWP switch delay for ATG	835

8.6D.1	Introduction	835

8.6D.2	DCI and timer based BWP switch delay on a single CC	835

8.6D.2A	DCI based BWP switch delay on multiple CCs	837

8.6D.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	837

8.6D.2B	Timer based BWP switch delay on multiple CCs	838

8.6D.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	838

8.6D.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	839

8.6D.3	RRC based BWP switch delay on a single CC	839

8.6D.3A	RRC based BWP switch delay on multiple CCs	840

8.6D.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	840

8.6E	Active BWP switch delay for RedCap UE with satellite access	841

8.6E.1	Introduction	841

8.6E.2	DCI and timer based BWP switch delay on a single CC	841

8.6E.3	RRC based BWP switch delay on a single CC	841

8.7	Void	841

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	841

8.8.1	Introduction	841

8.8.2	E-UTRAN PSCell Addition Delay Requirement	841

8.8.3	E-UTRAN PSCell Release Delay Requirement	842

8.9	NR-DC: PSCell Addition and Release Delay	842

8.9.1	Introduction	842

8.9.2	PSCell Addition Delay Requirement	842

8.9.3	PSCell Release Delay Requirement	843

8.9A	Conditional PSCell Addition Delay	843

8.9A.1	Introduction	843

8.9A.2	Conditional PSCell Addition Delay Requirement	843

8.9A.2.1	Measurement time	844

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	844

8.9B.1	Introduction	844

8.9B.2	PSCell Addition Delay Requirement	844

8.9B.3	PSCell Release Delay Requirement	845

8.9C	Subsequent Conditional PSCell Addition Delay	845

8.9C.1	Introduction	845

8.9C.2	Subsequent Conditional PSCell Addition Delay Requirement	845

8.9C.2.1	Measurement time	846

8.10	Active TCI state switching delay	846

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	848

8.10.4	DCI based TCI state switch delay	848

8.10.5	RRC based TCI state switch delay	849

8.10.6	Active TCI state list update delay	849

8.10A	Active TCI state switching delay with CCA	850

8.10A.1	Introduction	850

8.10A.2	Known conditions for TCI state	850

8.10A.3	MAC-CE based TCI state switch delay	850

8.10A.4	DCI based TCI state switch delay	851

8.10A.5	RRC based TCI state switch delay	851

8.10A.6	Active TCI state list update delay	852

8.10B	Active TCI state switching delay for RedCap	852

8.10B.1	Introduction	852

8.10B.2	Known conditions for TCI state	852

8.10B.3	MAC-CE based TCI state switch delay	853

8.10B.4	DCI based TCI state switch delay	854

8.10B.5	RRC based TCI state switch delay	854

8.10B.6	Active TCI state list update delay	855

8.10C	Active TCI state switching delay for satellite access	855

8.10C.1	Introduction	855

8.10C.2	MAC-CE based TCI state switch delay	855

8.10C.4	DCI based TCI state switch delay	855

8.10C.5	RRC based TCI state switch delay	855

8.10C.6	Active TCI state list update delay	856

8.10D	Active TCI state switching delay for ATG	856

8.10D.2	Void	856

8.10D.6	Active TCI state list update delay	857

8.10E	Active TCI state switching delay for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	857

8.10E.1	Introduction	857

8.10E.2	Known conditions for TCI state	857

8.10E.3	MAC-CE based dual DL TCI state switch delay	857

8.10E.3.1	MAC-CE based dual DL TCI state switching delay for sDCI	857

8.10E.3.2	MAC-CE based dual DL TCI state switching delay for mDCI	858

8.10E.4	DCI based dual DL TCI state switch delay for sDCI and mDCI	858

8.10E.4.1	DCI based dual DL TCI state switching delay for sDCI	858

8.10E.4.2	DCI based dual DL TCI state switching delay for mDCI	858

8.10E.5	RRC based dual DL TCI state switch delay	859

8.10E.6	Active DL TCI state list update delay	859

8.10E.6.1	Active DL TCI state list update delay for sDCI	859

8.10E.6.2	Active DL TCI state list update delay for mDCI	859

8.10F	Active TCI state switching delay for RedCap UE with satellite access	859

8.10F.1	Introduction	859

8.10F.2	MAC-CE based TCI state switch delay	859

8.10F.4	DCI based TCI state switch delay	859

8.10F.5	RRC based TCI state switch delay	859

8.10F.6	Active TCI state list update delay	860

8.11	PSCell Change	860

8.11A	PSCell Change in Carriers with CCA	860

8.11B	Conditional PSCell Change	861

8.11B.1	Introduction	861

8.11B.2	Conditional PSCell Change delay	861

8.11B.2.1	Measurement time	861

8.11D	Conditional PSCell Change in Carriers with CCA	862

8.11D.1	Introduction	862

8.11D.2	Conditional PSCell Change delay	862

8.11D.2.1	Measurement time	863

8.11E	Subsequent Conditional PSCell Change	863

8.11E.1	Introduction	863

8.11E.2	Subsequent Conditional PSCell Change delay	863

8.11E.2.1	Measurement time	864

8.12	Uplink spatial relation switch delay	864

8.12.1	Introduction	864

8.12.2	Known conditions for spatial relation when associated with DL-RS	864

8.12.3	MAC-CE based spatial relation switch delay	865

8.12.4	DCI based spatial relation switch delay	866

8.12.5	RRC based spatial relation switch delay	866

8.12A	Uplink spatial relation switch delay for RedCap	866

8.12A.1	Introduction	866

8.12A.2	Known conditions for spatial relation when associated with DL-RS	866

8.12A.3	MAC-CE based spatial relation switch delay	867

8.12A.4	DCI based spatial relation switch delay	868

8.12A.5	RRC based spatial relation switch delay	868

8.12C	Uplink spatial relation switch delay for satellite access	868

8.12C.1	Void	869

8.12C.2	Void	869

8.12C.3	Void	869

8.12C.4	Void	869

8.12C.5	Void	869

8.13	UE-specific CBW change	869

8.13.1	Introduction	869

8.13.2	UE-specific CBW change delay	869

8.13A	UE-specific CBW change for RedCap	869

8.13A.1	Introduction	869

8.13A.2	UE-specific CBW change delay	869

8.13C	UE-specific CBW change for satellite access	870

8.13C.1	Introduction	870

8.13C.2	UE-specific CBW change delay	870

8.13D	UE-specific CBW change for ATG	870

8.13D.1	Introduction	870

8.13D.2	UE-specific CBW change delay	870

8.13E	UE-specific CBW change for RedCap UE with satellite access	871

8.13E.1	Introduction	871

8.13E.2	UE-specific CBW change delay	871

8.14	Pathloss reference signal switching delay	871

8.14.1	Introduction	871

8.14.2	Known conditions for pathloss reference signal	871

8.14.3	MAC-CE based pathloss reference signal switch delay	871

8.14C	Pathloss reference signal switching delay for satellite access	872

8.14C.1	Introduction	872

8.14C.2	Known conditions for pathloss reference signal	873

8.14C.3	MAC-CE based pathloss reference signal switch delay	873

8.14D	Pathloss reference signal switching delay for ATG	873

8.14D.1	Introduction	873

8.14D.2	Known conditions for pathloss reference signal	873

8.14D.3	MAC-CE based pathloss reference signal switch delay	874

8.14E	Pathloss reference signal switching delay for RedCap UE with satellite access	874

8.14E.1	Introduction	874

8.14E.2	Known conditions for pathloss reference signal	874

8.14E.3	MAC-CE based pathloss reference signal switch delay	874

8.15	Active downlink TCI state switching delay for unified TCI	874

8.15.1	Introduction	874

8.15.4	DCI based downlink TCI state switch delay	876

8.15.5	Active Downlink TCI state list update delay	876

8.15D	Active downlink TCI state switching delay for unified TCI for ATG	877

8.15D.1	Introduction	877

8.15D.2	Void	877

8.15D.4	DCI based downlink TCI state switch delay	878

8.15D.5	Active Downlink TCI state list update delay	878

8.16	Active uplink TCI state switching delay for unified TCI	878

8.16.1	Introduction	878

8.16.4	DCI based uplink TCI state switch delay	881

8.16.5	Active Uplink TCI state list update delay	881

8.16D	Active uplink TCI state switching delay for unified TCI for ATG	883

8.16D.1	Introduction	883

8.16D.2	Void	883

8.16D.3	MAC-CE based uplink TCI state switch delay	883

8.16D.4	DCI based uplink TCI state switch delay	884

8.16D.5	Active Uplink TCI state list update delay	884

8.17	SCG Activation and Deactivation Delay	884

8.17.1	Introduction	884

8.17.2	SCG Activation Delay Requirement	885

8.17.3	SCG Deactivation Delay Requirement	886

8.18	TRP specific Link Recovery Procedures	886

8.18.1	Introduction	886

8.18.2	Requirements for TRP specific SSB based beam failure detection	887

8.18.2.1	Introduction	887

8.18.2.2	Minimum requirement	887

8.18.2.3	Measurement restriction for SSB based beam failure detection	889

8.18.3	Requirements for CSI-RS based beam failure detection	890

8.18.3.1	Introduction	890

8.18.3.2	Minimum requirement	890

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	894

8.18.4	Minimum requirement for L1 indication	895

8.18.5	Requirements for SSB based candidate beam detection	895

8.18.5.1	Introduction	895

8.18.5.2	Minimum requirement	895

8.18.5.3	Measurement restriction for SSB based candidate beam detection	898

8.18.6	Requirements for CSI-RS based candidate beam detection	899

8.18.6.1	Introduction	899

8.18.6.2	Minimum requirement	899

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	901

8.18.7	Requirements for TRP specific Beam Failure Recovery	902

8.18.7.1	Introduction	902

8.18.7.2	Requirement	903

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	903

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	903

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	903

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	903

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	904

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	904

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	904

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	905

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	905

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	905

8.19	Pre-configured measurement gap activation/deactivation delay	906

8.19.1	Introduction	906

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	906

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	906

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	906

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	906

8.19.5	Activation/deactivation delay requirements for concurrent measurement gaps with Pre-MG	906

8.19.5.1	Activation/deactivation delay requirements for non-overlapped activation/deactivation of concurrent measurement gaps with Pre- MG	907

8.19.5.2	Activation/deactivation delay requirements for fully overlapped activation/deactivation of concurrent measurement gaps with Pre- MG	907

8.19.5.3	Pre-MG activation/deactivation delay when colliding with a concurrent measurement gap	907

8.19D	Pre-configured measurement gap activation/deactivation delay for ATG	907

8.19D.1	Introduction	907

8.19D.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	907

8.19D.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	907

8.19D.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	908

8.19D.4	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	908

8.20	LTM PSCell Cell Switch	908

8.20.1	Introduction	908

8.20.2	LTM Cell Switch delay	909

8.20.3	Void	909

8.21	Active downlink TCI state switching delay for unified TCI for single-DCI mTRP	909

8.21.1	Introduction	909

8.21.2	Known conditions for downlink TCI state	910

8.21.3	MAC-CE based downlink TCI state switch delay	910

8.21.4	DCI based downlink TCI state switch delay	911

8.21.5	Active Downlink TCI state list update delay	911

8.22	Active downlink TCI state switching delay for unified TCI for multi-DCI mTRP	912

8.22.1	Introduction	912

8.22.2	Known conditions for downlink TCI state	913

8.22.3	MAC-CE based downlink TCI state switch delay	913

8.22.4	DCI based downlink TCI state switch delay	914

8.22.5	Active Downlink TCI state list update delay	914

8.23	Active uplink TCI state switching delay for unified TCI for single-DCI mTRP	915

8.23.1	Introduction	915

8.23.2	Known conditions for uplink TCI state	915

8.23.3	MAC-CE based uplink TCI state switch delay	916

8.23.4	DCI based uplink TCI state switch delay	917

8.23.5	Active uplink TCI state list update delay	917

8.24	Active uplink TCI state switching delay for unified TCI for multi-DCI mTRP	918

8.24.1	Introduction	918

8.24.2	Known conditions for uplink TCI state	919

8.24.3	MAC-CE based uplink TCI state switch delay	919

8.24.4	DCI based uplink TCI state switch delay	920

8.24.5	Active Uplink TCI state list update delay	920

8.25	TCI state activation for LTM candidate cell	921

8.25.1	Introduction	921

8.25.2	Known TCI state conditions	922

8.25.3	SSB based TCI state activation delay	922

9	Measurement Procedure	923

9.1	General measurement requirement	923

9.1.1	Introduction	923

9.1.2	Measurement gap	923

9.1.2.1	EN-DC: Measurement Gap Sharing	933

9.1.2.1a	SA: Measurement Gap Sharing	933

9.1.2.1b	NE-DC: Measurement Gap Sharing	934

9.1.2.1c	NR-DC: Measurement Gap Sharing	935

9.1.3	UE Measurement capability	936

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	936

9.1.3.1a	SA: Monitoring of multiple layers using gaps	937

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	937

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	938

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	938

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	939

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	940

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	940

9.1A.3.2	Void	941

9.1.3A	UE Measurement capability under operation mode with CCA	941

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	941

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	941

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	941

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	942

9.1.3C	UE Measurement capability under operation mode with satellite access	942

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	942

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	943

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	943

9.1.4.1	Introduction	943

9.1.4.2	Requirements	943

9.1.5	Carrier-specific scaling factor	946

9.1.5.1	Monitoring of multiple layers outside gaps	946

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	949

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	954

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	956

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	958

9.1.5.2	Monitoring of multiple layers within gaps	960

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	962

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	964

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	966

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	968

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	970

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	970

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	970

9.1.5.3	Monitoring of multiple layers within NCSG	971

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	972

9.1.5.4	L1-RSRP measurements within measurement gap	973

9.1.5.4.1	SA mode: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	974

9.1.5.4.2	NR-DC: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	975

9.1.6	Minimum requirement at transitions	977

9.1.7	Pre-configured measurement gap	978

9.1.7.1	Introduction	978

9.1.7.2	Requirements applicability	978

9.1.7.3	Requirements	978

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	978

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	979

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	980

9.1.8	Concurrent measurement gaps	980

9.1.8.1	Introduction	980

9.1.8.2	Requirements	980

9.1.8.3	Collision between concurrent measurement gaps	981

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	982

9.1.9	Network controlled small gap	982

9.1.9.1	Introduction	982

9.1.9.2	Requirements applicability	983

9.1.10	MUSIM gaps	985

9.1.10.1	Introduction	986

9.1.10.2	Priorities for MUSIM gaps	986

9.1.10.3	Keep solution for MUSIM gaps	987

9.1.10.4	Collisions between different MUSIM gaps	987

9.1.10.5	Collisions between MUSIM gaps and measurement gaps	987

9.1.10.6	MUSIM gap related requirements	988

9.1.11	UL gap for Tx power management	988

9.1.12	Concurrent measurement gaps with Pre-MG	988

9.1.12.1	Introduction	988

9.1.12.2	Requirements	988

9.1.12.3	Collisions involving Pre-MG(s)	989

9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap	990

9.1.12.5	Pre-MG related requirements	990

9.1.13	Concurrent measurement gaps with NCSG	990

9.1.13.1	Introduction	990

9.1.13.2	Requirements	990

9.1.13.3	Collision involving NCSGs	992

9.1.14	Measurement gap occasion cancelation	992

9.1.14.1	Introduction	992

9.1.14.2	Applicable measurement gap configurations	992

9.1.14.3	Applicability	992

9.1.14.4	Requirements for cancelling measurement gap occasions	993

9.1A	General measurement requirement for RedCap	993

9.1A.1	Introduction	993

9.1A.2	Measurement gap	993

9.1A.2.1	SA: Measurement Gap Sharing	997

9.1A.3	UE Measurement capability	998

9.1A.3.1	SA: Monitoring of multiple layers using gaps	998

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	998

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	998

9.1A.4.1	Introduction	998

9.1A.4.2	Requirements	999

9.1A.5	Carrier-specific scaling factor	999

9.1A.5.1	Monitoring of multiple layers outside gaps	999

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	1000

9.1A.5.2	Monitoring of multiple layers within gaps	1000

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	1000

9.1A.6	Minimum requirement at transitions	1002

9.1C	General measurement requirement for SAN	1002

9.1C.1	Introduction	1002

9.1C.2	Measurement gap	1003

9.1C.8	Concurrent measurement gaps for SAN	1005

9.1C.8.1	Introduction	1005

9.1C.8.2	Requirements	1005

9.1C.8.3	Collision between concurrent measurement gaps	1006

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	1006

9.1C.9	Collision between SMTC and measurement gap for SAN	1006

9.1C.9.1	Introduction	1006

9.1C.9.2	Collision between SMTCs and measurement gap	1006

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	1007

9.1D	General measurement requirement for ATG	1007

9.1D.1	Introduction	1007

9.1D.2	Measurement gap	1007

9.1D.2.1a	SA: Measurement Gap Sharing	1010

9.1D.3	UE Measurement capability	1010

9.1D.3.1	SA: Monitoring of multiple layers using gaps	1010

9.1D.3.2	SA: Maximum allowed layers for multiple monitoring	1010

9.1D.4	Void	1011

9.1D.5	Carrier-specific scaling factor	1011

9.1D.5.1	Monitoring of multiple layers outside gaps	1011

9.1D.5.1.1	Void	1011

9.1D.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements performed outside gaps	1011

9.1D.5.2	Monitoring of multiple layers within gaps	1012

9.1D.5.2.1	Void	1013

9.1D.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements performed within gaps	1013

9.1D.6	Void	1013

9.1D.7	Pre-configured measurement gap	1013

9.1D.7.1	Introduction	1013

9.1D.7.2	Requirements applicability	1014

9.1D.7.3	Requirements	1014

9.1D.7.3.1	Requirements for autonomous activation/deactivation mechanism	1014

9.1D.7.3.2	Requirements for network-controlled activation/deactivation mechanism	1015

9.1D.7.3.3	Requirements for reception/transmission during activation/deactivation	1015

9.1D.8	Capabilities for Support of Event Triggering and Reporting Criteria	1015

9.1D.8.1	Introduction	1015

9.1D.8.2	Requirements	1016

9.1D.9	Minimum requirement at transitions	1016

9.1E	General measurement requirement for RedCap with satellite access	1016

9.1E.1	Introduction	1016

9.1E.2	Measurement gap	1017

9.1E.8	Concurrent measurement gaps for RedCap with SAN	1017

9.1E.8.1	Introduction	1017

9.1E.8.2	Requirements	1017

9.1E.8.3	Collision between concurrent measurement gaps	1017

9.1E.8.4	Measurement gap related requirements of concurrent measurement gaps	1017

9.1E.9	Collision between SMTC and measurement gap for RedCap with satellite access	1017

9.1E.9.1	Introduction	1017

9.1E.9.2	Collision between SMTCs and measurement gap	1017

9.1E.9.3	Collision between multiple SMTCs on a SAN carrier	1018

9.2	NR intra-frequency measurements	1018

9.2.1	Introduction	1018

9.2.2	Requirements applicability	1021

9.2.3	Number of cells and number of SSB	1022

9.2.3.1	Requirements for FR1	1022

9.2.3.2	Requirements for FR2	1022

9.2.4	Measurement Reporting Requirements	1023

9.2.4.1	Periodic Reporting	1023

9.2.4.2	Event-triggered Periodic Reporting	1023

9.2.4.3	Event Triggered Reporting	1023

9.2.4.4	SCell activation Triggered Reporting	1023

9.2.5	Intrafrequency measurements without measurement gaps	1024

9.2.5.1	Intra-frequency cell identification	1024

9.2.5.2	Measurement period	1032

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	1035

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1036

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1037

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	1038

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1039

9.2.5.4	SFTD Measurements between PCell and PSCell	1039

9.2.5.4.1	Introduction	1039

9.2.5.4.2	SFTD Measurement delay	1039

9.2.5.4.3	SFTD Measurement Reporting Delay	1040

9.2.6	Intra-frequency measurements with measurement gaps	1040

9.2.6.1	Void	1040

9.2.6.2	Intra-frequency cell identification	1040

9.2.6.3	Intra-frequency Measurement Period	1044

9.2.7	Intra-frequency measurements with NCSG	1046

9.2.7.1	Intra-frequency cell identification	1046

9.2.7.2	Measurement period	1048

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	1049

9.2A	NR intra-frequency measurements with CCA	1050

9.2A.1	Introduction	1050

9.2A.2	Requirements applicability	1050

9.2A.3	Number of cells and number of SSB	1051

9.2A.3.1	Requirements for FR1	1051

9.2A.3.2	Requirements for FR2-2	1051

9.2A.4	Measurement Reporting Requirements	1051

9.2A.5	Intra-frequency measurements without measurement gaps	1052

9.2A.5.2	Measurement period	1056

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	1058

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1059

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1059

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	1059

9.2A.6	Intra-frequency measurements with measurement gaps	1060

9.2A.6.1	Intra-frequency cell identification	1060

9.2A.6.2	Intra-frequency Measurement Period	1062

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	1063

9.2A.7.1	Intra-frequency RSSI measurements	1063

9.2A.7.2	Intra-frequency Channel occupancy measurements	1065

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	1067

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	1067

9.2B	NR intra-frequency measurements for RedCap	1067

9.2B.1	Introduction	1067

9.2B.2	Requirements applicability	1068

9.2B.3	Number of cells and number of SSB	1068

9.2B.3.1	Requirements for FR1	1068

9.2B.3.2	Requirements for FR2	1068

9.2B.4	Measurement Reporting Requirements	1069

9.2B.4.1	Periodic Reporting	1069

9.2B.4.2	Event-triggered Periodic Reporting	1069

9.2B.4.3	Event Triggered Reporting	1069

9.2B.5	Intra-frequency measurements without measurement gaps	1070

9.2B.5.1	Intra-frequency cell identification	1070

9.2B.5.2	Measurement period	1072

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	1073

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1073

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1073

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	1074

9.2B.5.3.4	Scheduling availability of HD-FDD UE performing measurements on FR1	1074

9.2B.6	Intra-frequency measurements with measurement gaps	1075

9.2B.6.1	Intra-frequency cell identification	1075

9.2B.6.2	Intra-frequency Measurement Period	1076

9.2C	NR intra-frequency measurements for SAN	1077

9.2C.1	Introduction	1077

9.2C.2	Requirements applicability	1078

9.2C.3	Number of cells and number of SSB	1078

9.2C.3.1	Requirements for FR1-NTN	1078

9.2C.4	Measurement Reporting Requirements	1079

9.2C.4.1	Periodic Reporting	1079

9.2C.4.2	Event-triggered Periodic Reporting	1079

9.2C.4.3	Event Triggered Reporting	1079

9.2C.5	Intra-frequency measurements without measurement gaps	1079

9.2C.5.1	Intra-frequency cell identification	1079

9.2C.5.2	Measurement period	1082

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	1082

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	1082

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	1082

9.2C.6	Intra-frequency measurements with measurement gaps	1083

9.2C.6.1	Intra-frequency cell identification	1083

9.2C.6.3	Intrafrequency Measurement Period	1084

9.2C.7	Intra-frequency measurements without measurement gaps for NTN band above 10 GHz	1085

9.2C.7.1	Intra-frequency cell identification	1085

9.2C.7.2	Measurement period	1086

9.2C.7.3	Scheduling availability of UE during intra-frequency measurements	1087

9.2C.7.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10 GHz	1087

9.2C.8	Intra-frequency measurements with measurement gaps for NTN band above 10 GHz	1087

9.2C.8.1	Intra-frequency cell identification	1087

9.2C.8.3	Intra-frequency Measurement Period	1088

9.2D	NR intra-frequency measurements for ATG	1088

9.2D.1	Introduction	1088

9.2D.2	Requirements applicability	1089

9.2D.3	Number of cells and number of SSB	1089

9.2D.3.1	Requirements for FR1	1089

9.2D.4	Measurement Reporting Requirements	1089

9.2D.4.1	Periodic Reporting	1089

9.2D.4.2	Event-triggered Periodic Reporting	1089

9.2D.4.3	Event Triggered Reporting	1090

9.2D.4.4	SCell activation Triggered Reporting	1090

9.2D.5	Intra-frequency measurements without measurement gaps	1091

9.2D.5.1	Intra-frequency cell identification	1091

9.2D.5.2	Measurement period	1093

9.2D.5.3	Scheduling availability of UE during intra-frequency measurements	1094

9.2D.5.3.1	Scheduling availability of UE performing measurements on FR1	1094

9.2D.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1096

9.2D.6	Intra-frequency measurements with measurement gaps	1096

9.2D.6.1	Void	1096

9.2D.6.2	Intra-frequency cell identification	1096

9.2D.6.3	Intra-frequency Measurement Period	1097

9.2E	NR intra-frequency measurements for RedCap with SAN	1098

9.2E.1	Introduction	1098

9.2E.2	Requirements applicability	1098

9.2E.3	Number of cells and number of SSB	1099

9.2E.3.1	Requirements for FR1	1099

9.2E.4	Measurement Reporting Requirements	1099

9.2E.4.1	Periodic Reporting	1099

9.2E.4.2	Event-triggered Periodic Reporting	1099

9.2E.4.3	Event Triggered Reporting	1099

9.2E.5	Intra-frequency measurements without measurement gaps	1099

9.2E.5.1	Intra-frequency cell identification	1099

9.2E.5.2	Measurement period	1100

9.2E.5.3	Scheduling availability of UE during intra-frequency measurements	1100

9.2E.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1100

9.2E.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	1100

9.2E.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	1100

9.2E.6	Intra-frequency measurements with measurement gaps	1101

9.2E.6.1	Intra-frequency cell identification	1101

9.2E.6.2	Intra-frequency Measurement Period	1101

9.3	NR inter-frequency measurements	1101

9.3.1	Introduction	1101

9.3.2	Requirements applicability	1104

9.3.2.1	Void	1105

9.3.2.2	Void	1105

9.3.3	Number of cells and number of SSB	1105

9.3.3.1	Requirements for FR1	1105

9.3.3.2	Requirements for FR2	1105

9.3.4	Inter-frequency measurement with measurement gaps	1105

9.3.4.1	Void	1111

9.3.4.2	Void	1111

9.3.5	Inter-frequency measurements	1111

9.3.5.1	Void	1114

9.3.5.2	Void	1114

9.3.5.3	Void	1114

9.3.6	Inter-frequency measurements reporting requirements	1114

9.3.6.1	Periodic Reporting	1114

9.3.6.2	Event-triggered Periodic Reporting	1114

9.3.6.3	Event-triggered Reporting	1114

9.3.7	Void	1115

9.3.8	Inter-frequency SFTD measurement requirements	1115

9.3.8.1	Introduction	1115

9.3.8.2	SFTD Measurement delay	1115

9.3.8.3	SFTD Measurement reporting delay	1116

9.3.9	Inter-frequency measurements without measurement gaps	1116

9.3.9.1	Inter-frequency Cell identification	1116

9.3.9.2	Measurement period	1121

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	1123

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1124

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1125

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	1126

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1126

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	1126

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1127

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1127

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	1128

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1129

9.3.10	Inter-frequency measurement with NCSG	1130

9.3.10.1	Inter-frequency cell identification	1130

9.3.10.2	Measurement period	1132

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	1132

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1132

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1133

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	1133

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1135

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	1135

9.3A.1	Introduction	1135

9.3A.2	Requirements applicability	1136

9.3A.3	Number of cells and number of SSB	1136

9.3A.3.1	Requirements for FR1	1136

9.3A.3.2	Requirements for FR2-2	1136

9.3A.4	Inter-frequency cell identification	1137

9.3A.5	Inter-frequency measurements	1139

9.3A.6	Inter-frequency measurements reporting requirements	1140

9.3A.6.1	Periodic Reporting	1140

9.3A.6.2	Event-triggered Periodic Reporting	1140

9.3A.6.3	Event-triggered Reporting	1140

9.3A.8	Inter-frequency RSSI measurements	1141

9.3A.9	Inter-frequency channel occupancy measurements	1142

9.3B	NR inter-frequency measurements for RedCap	1142

9.3B.1	Introduction	1142

9.3B.2	Requirements applicability	1143

9.3B.3	Number of cells and number of SSB	1143

9.3B.3.1	Requirements for FR1	1143

9.3B.3.2	Requirements for FR2	1143

9.3B.4	Inter-frequency measurement with measurement gaps	1143

9.3B.5	Inter-frequency measurements	1145

9.3B.6	Inter-frequency measurements reporting requirements	1146

9.3B.6.1	Periodic Reporting	1146

9.3B.6.2	Event-triggered Periodic Reporting	1146

9.3B.6.3	Event-triggered Reporting	1146

9.3B.7	Inter-frequency measurements without measurement gaps	1146

9.3B.7.1	Inter-frequency Cell identification	1146

9.3B.7.2	Measurement period	1148

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	1149

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1149

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1150

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	1150

9.3B.7.3.4	Scheduling availability of HD-FDD UE performing measurements on FR1	1150

9.3C	NR inter-frequency measurements for SAN	1151

9.3C.1	Introduction	1151

9.3C.2	Requirements applicability	1152

9.3C.3	Number of cells and number of SSB	1152

9.3C.3.1	Requirements for FR1-NTN	1152

9.3C.4	Inter-frequency measurement with measurement gaps	1152

9.3C.5	Inter-frequency measurements	1154

9.3C.6	Inter-frequency measurements reporting requirements	1154

9.3C.6.1	Periodic Reporting	1154

9.3C.6.2	Event-triggered Periodic Reporting	1155

9.3C.6.3	Event-triggered Reporting	1155

9.3C.7	Inter-frequency measurements without measurement gaps	1155

9.3C.7.1	Inter-frequency Cell identification	1155

9.3C.7.2	Measurement period	1157

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	1157

9.3C.7.3.1	Void	1157

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	1157

9.3C.8	Inter-frequency measurement with measurement gaps for NTN band above 10 GHz	1158

9.3C.9	Inter-frequency measurements for NTN band above 10 GHz	1159

9.3C.10	Inter-frequency measurements without measurement gaps for NTN band above 10 GHz	1159

9.3C.10.1	Inter-frequency Cell identification	1159

9.3C.10.2	Measurement period	1160

9.3C.10.3	Scheduling availability of UE during inter-frequency measurements	1161

9.3C.10.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10 GHz	1161

9.3D	NR inter-frequency measurements for ATG	1161

9.3D.1	Introduction	1161

9.3D.2	Requirements applicability	1162

9.3D.3	Number of cells and number of SSB	1162

9.3D.3.1	Requirements for FR1	1162

9.3D.4	Inter-frequency measurement with measurement gaps	1162

9.3D.5	Inter-frequency measurements	1163

9.3D.6	Inter-frequency measurements reporting requirements	1164

9.3D.6.1	Periodic Reporting	1164

9.3D.6.2	Event-triggered Periodic Reporting	1164

9.3D.6.3	Event-triggered Reporting	1164

9.3D.7	Void	1164

9.3D.8	Void	1164

9.3D.9	Inter-frequency measurements without measurement gaps	1164

9.3D.9.1	Inter-frequency Cell identification	1164

9.3D.9.2	Measurement period	1166

9.3D.9.3	Scheduling availability of UE during inter-frequency measurements	1167

9.3D.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1167

9.3D.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1168

9.3E	NR inter-frequency measurements for Redcap UEs with satellite access	1169

9.3E.1	Introduction	1169

9.3E.2	Requirements applicability	1169

9.3E.3	Number of cells and number of SSB	1170

9.3E.3.1	Requirements for FR1	1170

9.3E.4	Inter-frequency measurement with measurement gaps	1170

9.3E.5	Inter-frequency measurements	1171

9.3E.6	Inter-frequency measurements reporting requirements	1171

9.3E.6.1	Periodic Reporting	1171

9.3E.6.2	Event-triggered Periodic Reporting	1171

9.3E.6.3	Event-triggered Reporting	1171

9.3E.7	Inter-frequency measurements without measurement gaps	1172

9.3E.7.1	Inter-frequency Cell identification	1172

9.3E.7.2	Measurement period	1173

9.3E.7.3	Scheduling availability of UE during inter-frequency measurements	1173

9.3E.7.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1173

9.3E.7.3.2	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	1173

9.4	Inter-RAT measurements	1173

9.4.1	Introduction	1173

9.4.2	NR − E-UTRAN FDD measurements	1176

9.4.2.1	Introduction	1176

9.4.2.2	Requirements when no DRX is used	1176

9.4.2.3	Requirements when DRX is used	1178

9.4.2.4	Measurement reporting requirements	1180

9.4.2.4.1	Periodic Reporting	1180

9.4.2.4.2	Event-Triggered Periodic Reporting	1180

9.4.2.4.3	Event-Triggered Reporting	1181

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	1181

9.4.3	NR − E-UTRAN TDD measurements	1181

9.4.3.1	Introduction	1181

9.4.3.2	Requirements when no DRX is used	1181

9.4.3.3	Requirements when DRX is used	1184

9.4.3.4	Measurement reporting requirements	1186

9.4.3.4.1	Periodic Reporting	1186

9.4.3.4.2	Event-Triggered Periodic Reporting	1186

9.4.3.4.3	Event-Triggered Reporting	1187

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	1187

9.4.4	Inter-RAT RSTD measurements	1187

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	1187

9.4.4.1.1	Introduction	1187

9.4.4.1.2	Requirements	1188

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	1191

9.4.4.2.1	Introduction	1191

9.4.4.2.2	Requirements	1191

9.4.5	Inter-RAT E-CID measurements	1194

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	1194

9.4.5.1.1	Introduction	1194

9.4.5.1.2	Requirements	1194

9.4.5.1.3	Measurement Reporting Delay	1194

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	1195

9.4.5.2.1	Introduction	1195

9.4.5.2.2	Requirements	1195

9.4.5.2.3	Measurement Reporting Delay	1195

9.4.6	NR − UTRAN FDD measurements	1195

9.4.6.1	Introduction	1195

9.4.6.2	Requirements when no DRX is used	1195

9.4.6.3	Requirements when DRX is used	1196

9.4.7	NR – E-UTRAN measurements with autonomous gaps	1198

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	1198

9.4.7.2	CGI reporting delay	1198

9.4.8	NR – E-UTRAN measurements without measurement gaps	1199

9.4.8.1	Introduction	1199

9.4.8.2	General requirements	1199

9.4.8.3	NR − E-UTRAN FDD measurements	1200

9.4.8.3.1	Introduction	1200

9.4.8.3.2	Requirements when no DRX is used	1200

9.4.8.3.3	Requirements when DRX is used	1201

9.4.8.3.4	Measurement reporting requirements	1201

9.4.8.3.5	Scheduling availability during NR − E-UTRAN FDD measurements	1202

9.4.8.4	NR − E-UTRAN TDD measurements	1202

9.4.8.4.1	Introduction	1202

9.4.8.4.2	Requirements when no DRX is used	1203

9.4.8.4.3	Requirements when DRX is used	1204

9.4.8.4.4	Measurement reporting requirements	1205

9.4.8.4.5	Scheduling availability during NR − E-UTRAN TDD measurements	1205

9.4A	Inter-RAT measurements for RedCap	1206

9.4A.1	Introduction	1206

9.4A.2	NR − E-UTRAN FDD measurements	1207

9.4A.2.1	Introduction	1207

9.4A.2.2	Requirements when no DRX is used	1207

9.4A.2.3	Requirements when DRX is used	1208

9.4A.2.4	Measurement reporting requirements	1209

9.4A.2.4.1	Periodic Reporting	1209

9.4A.2.4.2	Event-Triggered Periodic Reporting	1209

9.4A.2.4.3	Event-Triggered Reporting	1209

9.4A.3	NR − E-UTRAN TDD measurements	1210

9.4A.3.1	Introduction	1210

9.4A.3.2	Requirements when no DRX is used	1210

9.4A.3.3	Requirements when DRX is used	1212

9.4A.3.4	Measurement reporting requirements	1213

9.4A.3.4.1	Periodic Reporting	1213

9.4A.3.4.2	Event-Triggered Periodic Reporting	1213

9.4A.3.4.3	Event-Triggered Reporting	1213

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	1214

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	1214

9.4A.4.2	CGI reporting delay	1214

9.4A.4.3	CGI reporting scheduling restriction	1214

9.5	L1-RSRP measurements for Reporting	1215

9.5.1	Introduction	1215

9.5.2	Requirements applicability	1216

9.5.3	Measurement Reporting Requirements	1216

9.5.3.1	Periodic Reporting	1217

9.5.3.2	Semi-Persistent Reporting	1217

9.5.3.3	Aperiodic Reporting	1217

9.5.3.4	Event Triggered Reporting for the UE initiated beam management	1217

9.5.4	L1-RSRP measurement requirements	1219

9.5.4.1	SSB based L1-RSRP Reporting	1219

9.5.4.2	CSI-RS based L1-RSRP Reporting	1225

9.5.4A	Void	1230

9.5.4A.1	Void	1230

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1230

9.5.5.1	Measurement restriction for SSB based L1-RSRP	1230

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	1231

9.5.6	Scheduling availability of UE during L1-RSRP measurement	1232

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1233

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1233

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1234

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1235

9.5.7	Minimum requirement at transitions	1236

9.5A	L1-RSRP measurements for Reporting under CCA	1236

9.5A.1	Introduction	1236

9.5A.2	Requirements applicability	1236

9.5A.3	Measurement Reporting Requirements	1237

9.5A.3.1	Periodic Reporting	1237

9.5A.3.2	Semi-Persistent Reporting	1237

9.5A.3.3	Aperiodic Reporting	1237

9.5A.4	L1-RSRP measurement requirements	1238

9.5A.4.1	SSB based L1-RSRP Reporting	1238

9.5A.5	Measurement restriction for L1-RSRP measurement	1240

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	1240

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	1241

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1241

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1241

9.5A.6.3	Void	1241

9.5A.6.3A	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	1241

9.5A.6.3B	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	1241

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1242

9.5B	L1-RSRP measurements for Reporting for RedCap	1242

9.5B.1	Introduction	1242

9.5B.2	Requirements applicability	1242

9.5B.3	Measurement Reporting Requirements	1243

9.5B.3.1	Periodic Reporting	1243

9.5B.3.2	Semi-Persistent Reporting	1243

9.5B.3.3	Aperiodic Reporting	1244

9.5B.4	L1-RSRP measurement requirements	1244

9.5B.4.1	SSB based L1-RSRP Reporting	1244

9.5B.4.2	CSI-RS based L1-RSRP Reporting	1246

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1249

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	1249

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	1249

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	1249

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1250

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1250

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1250

9.5C	L1-RSRP measurements for Reporting for satellite access	1251

9.5C.1	Introduction	1251

9.5C.3	Measurement Reporting Requirements	1251

9.5C.3.1	Periodic Reporting	1252

9.5C.3.2	Semi-Persistent Reporting	1252

9.5C.3.3	Aperiodic Reporting	1252

9.5C.4	L1-RSRP measurement requirements	1252

9.5C.4.1	SSB based L1-RSRP Reporting	1252

9.5C.5	Measurement restriction for L1-RSRP measurement	1254

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	1254

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	1254

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	1255

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	1255

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	1255

9.5C.7	L1-RSRP measurement requirements for NTN band above 10 GHz	1255

9.5C.7.1	SSB based L1-RSRP Reporting	1255

9.5C.7.2	CSI-RS based L1-RSRP Reporting	1256

9.5C.8	Measurement restriction for L1-RSRP measurement for NTN band above 10 GHz	1257

9.5C.8.1	Measurement restriction for SSB based L1-RSRP	1257

9.5C.8.2	Measurement restriction for CSI-RS based L1-RSRP	1257

9.5C.9	Scheduling availability of UE during L1-RSRP measurement for NTN band above 10 GHz	1258

9.5C.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on NTN bands above 10 GHz	1258

9.5C.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10 GHz	1258

9.5D	L1-RSRP measurements for Reporting for ATG	1258

9.5D.1	Introduction	1258

9.5D.2	Requirements applicability	1258

9.5D.3	Measurement Reporting Requirements	1259

9.5D.3.1	Periodic Reporting	1259

9.5D.3.2	Semi-Persistent Reporting	1259

9.5D.3.3	Aperiodic Reporting	1259

9.5D.4	L1-RSRP measurement requirements	1259

9.5D.4.1	SSB based L1-RSRP Reporting	1259

9.5D.4.2	CSI-RS based L1-RSRP Reporting	1261

9.5D.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1263

9.5D.5.1	Measurement restriction for SSB based L1-RSRP	1263

9.5D.5.2	Measurement restriction for CSI-RS based L1-RSRP	1263

9.5D.6	Scheduling availability of UE during L1-RSRP measurement	1264

9.5D.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1264

9.5D.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1264

9.5E	L1-RSRP measurements for Reporting for RedCap UEs with satellite access	1264

9.5E.1	Introduction	1264

9.5E.2	Requirements applicability	1264

9.5E.3	Measurement Reporting Requirements	1265

9.5E.3.1	Periodic Reporting	1265

9.5E.3.2	Semi-Persistent Reporting	1265

9.5E.3.3	Aperiodic Reporting	1266

9.5E.4	L1-RSRP measurement requirements	1266

9.5E.4.1	SSB based L1-RSRP Reporting	1266

9.5E.4.2	CSI-RS based L1-RSRP Reporting	1266

9.5E.5	Measurement restriction for L1-RSRP measurement	1266

9.5E.6	Scheduling availability of UE during L1-RSRP measurement	1266

9.5E.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1266

9.5E.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1266

9.6	NE-DC: Measurements	1267

9.6.1	Introduction	1267

9.6.2	SFTD Measurements	1267

9.6.2.1	Introduction	1267

9.6.2.2	SFTD Measurement requirements	1267

9.7	Cross Link Interference measurements	1268

9.7.1	Introduction	1268

9.7.2	SRS-RSRP measurements	1268

9.7.2.1	Introduction	1268

9.7.2.2	Requirements applicability	1268

9.7.2.3	Measurement Reporting Requirements	1268

9.7.2.3.1	Periodic Reporting	1269

9.7.2.3.2	Event-triggered Periodic Reporting	1269

9.7.2.3.3	Event Triggered Reporting	1269

9.7.2.4	Measurement capability	1269

9.7.2.5	SRS-RSRP measurement period	1269

9.7.3	CLI-RSSI measurements	1269

9.7.3.1	Introduction	1269

9.7.3.2	Requirements applicability	1270

9.7.3.3	Measurement Reporting Requirements	1270

9.7.3.3.1	Periodic Reporting	1270

9.7.3.3.2	Event-triggered Periodic Reporting	1270

9.7.3.3.3	Event Triggered Reporting	1270

9.7.3.4	Measurement capability	1270

9.7.3.5	CLI-RSSI measurement period	1270

9.7.4	Scheduling availability of UE during CLI measurements	1271

9.7.4.1	Scheduling availability of UE performing measurement on FR1	1271

9.7.4.2	Scheduling availability of UE performing measurement on FR2	1272

9.8	L1-SINR measurements for Reporting	1272

9.8.1	Introduction	1272

9.8.2	Requirements applicability	1273

9.8.3	Measurement Reporting Requirements	1274

9.8.3.1	Periodic Reporting	1274

9.8.3.2	Semi-Persistent Reporting	1274

9.8.4	L1-SINR measurement requirements	1275

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1275

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1282

9.8.5	Measurement restriction for L1-SINR measurement	1284

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	1284

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	1285

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	1286

9.8.6	Scheduling availability of UE during L1-SINR measurement	1287

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1287

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1287

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1289

9.8.7	Minimum requirement at transitions	1289

9.8D	L1-SINR measurements for Reporting for ATG	1290

9.8D.1	Introduction	1290

9.8D.2	Requirements applicability	1290

9.8D.3	Measurement Reporting Requirements	1291

9.8D.3.1	Periodic Reporting	1291

9.8D.3.2	Semi-Persistent Reporting	1291

9.8D.3.3	Aperiodic Reporting	1291

9.8D.4	L1-SINR measurement requirements	1291

9.8D.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1291

9.8D.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1293

9.8D.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1294

9.8D.5	Measurement restriction for L1-SINR measurement	1295

9.8D.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	1295

9.8D.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	1295

9.8D.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	1296

9.8D.6	Scheduling availability of UE during L1-SINR measurement	1296

9.8D.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1296

9.8D.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1296

9.9	NR measurements for positioning	1297

9.9.1	Introduction	1297

9.9.1.1	General Aspects of Gap-based Measurement	1297

9.9.1.2	General Aspects of Gapless Measurement	1298

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1299

9.9.2	RSTD measurements	1300

9.9.2.1	Introduction	1300

9.9.2.2	Requirements Applicability	1300

9.9.2.3	Measurement Capability	1300

9.9.2.4	Measurement Reporting Requirements	1300

9.9.2.4.1	Void	1300

9.9.2.4.2	Void	1300

9.9.2.4.3	Void	1300

9.9.2.5	Measurements Period Requirements	1300

9.9.2.6	Void	1303

9.9.2.7	Measurements Period Requirements without Measurement Gaps	1303

9.9.2.8	Void	1306

9.9.2.9	Measurements Period Requirements with both MG and PPW	1306

9.9.2.10	Measurements Period Requirements with Bandwidth Aggregation	1307

9.9.3	PRS-RSRP measurements	1310

9.9.3.1	Introduction	1310

9.9.3.2	Requirements applicability	1311

9.9.3.3	Measurement Capability	1311

9.9.3.4	Measurement Reporting Requirements	1311

9.9.3.5	Measurement Period Requirements	1311

9.9.3.6	Measurement Period Requirements without Measurement Gaps	1314

9.9.3.7	Void	1316

9.9.3.8	Measurements Period Requirements with both MG and PPW	1316

9.9.4	UE Rx-Tx time difference measurements	1317

9.9.4.1	Introduction	1317

9.9.4.2	Requirements Applicability	1317

9.9.4.3	Measurement Capability	1317

9.9.4.4	Measurement Reporting Requirements	1317

9.9.4.5	Measurement Period Requirements	1317

9.9.4.6	Measurement Period Requirements without Measurement Gaps	1321

9.9.4.7	Void	1324

9.9.4.8	Measurements Period Requirements with both MG and PPW	1324

9.9.4.9	Measurements Period Requirements with Bandwidth Aggregation	1325

9.9.5	E-CID measurements	1329

9.9.5.1	Introduction	1329

9.9.5.2	Measurement Requirements	1329

9.9.5.2.1	Intra-frequency Measurement Requirements	1329

9.9.5.2.2	Inter-frequency Measurement Requirements	1329

9.9.5.2.3	Measurement Reporting Delay	1330

9.9.6	PRS-RSRPP measurements	1330

9.9.6.1	Introduction	1330

9.9.6.2	Requirements applicability	1330

9.9.6.3	Measurement capability	1330

9.9.6.4	Measurement reporting requirements	1330

9.9.6.5	Measurement period requirements	1330

9.9.6.6	Measurement Period Requirements without Measurement Gaps	1331

9.9.6.7	Void	1331

9.9.6.8	Measurements Period Requirements with both MG and PPW	1331

9.9.7	Measurement requirements for DL RSCPD reported with RSTD	1331

9.9.7.1	Introduction	1331

9.9.7.2	Requirements Applicability	1331

9.9.7.3	Measurement Capability	1331

9.9.7.4	Measurement Reporting Requirements	1331

9.9.7.5	Measurements Period Requirements for DL RSCPD reported with RSTD	1332

9.9.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	1334

9.9.8.1	Introduction	1334

9.9.8.2	Requirements Applicability	1335

9.9.8.3	Measurement Capability	1335

9.9.8.4	Measurement Reporting Requirements	1335

9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference	1335

9.9A	NR measurements for positioning for RedCap	1339

9.9A.1	Introduction	1339

9.9A.1.1	General Aspects of Gap-based Measurement	1339

9.9A.1.2	General Aspects of Gapless Measurement for RedCap positioning without FH	1340

9.9A.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps for RedCap positioning without FH	1341

9.9A.2	RSTD measurements for RedCap	1342

9.9A.2.1	Introduction	1342

9.9A.2.2	Requirements Applicability	1342

9.9A.2.3	Measurement Capability	1342

9.9A.2.4	Measurement Reporting Requirements	1342

9.9A.2.5	Measurements Period Requirements without FH	1342

9.9A.2.5.1	Measurements Period Requirements without FH with MG	1342

9.9A.2.5.2	Measurements Period Requirements without FH without MG	1346

9.9A.2.5.3	Measurements Period Requirements without FH with both MG and PPW	1348

9.9A.2.6	Measurements Period Requirements with FH	1349

9.9A.2.6.1	Measurements Period Requirements with FH with MG	1349

9.9A.3	PRS-RSRP measurements for RedCap	1351

9.9A.3.1	Introduction	1351

9.9A.3.2	Requirements applicability	1351

9.9A.3.3	Measurement Capability	1351

9.9A.3.4	Measurement Reporting Requirements	1351

9.9A.3.5	Measurements Period Requirements without FH	1351

9.9A.3.5.1	Measurement Period Requirements without FH with MG	1351

9.9A.3.5.2	Measurement Period Requirements without FH without MG	1354

9.9A.3.5.3	Measurements Period Requirements without FH with both MG and PPW	1356

9.9A.3.6	Measurements Period Requirements with FH	1357

9.9A.3.6.1	Measurements Period Requirements with FH with MG	1357

9.9A.4	UE Rx-Tx time difference measurements for RedCap	1359

9.9A.4.1	Introduction	1359

9.9A.4.2	Requirements Applicability	1359

9.9A.4.3	Measurement Capability	1359

9.9A.4.4	Measurement Reporting Requirements	1359

9.9A.4.5	Measurement Period Requirements without FH with MG	1359

9.9A.4.6	Measurement Period Requirements without FH without MG	1359

9.9A.4.7	Measurements Period Requirements without FH with both MG and PPW	1359

9.9A.4.8	Measurements Period Requirements with FH	1360

9.9A.5	PRS-RSRPP measurements for RedCap	1361

9.9A.5.1	Introduction	1361

9.9A.5.2	Requirements Applicability	1361

9.9A.5.3	Measurement Capability	1362

9.9A.5.4	Measurement Reporting Requirements	1362

9.9A.5.5	Measurement Period Requirements without FH with MG	1362

9.9A.5.6	Measurement Period Requirements without FH without MG	1362

9.9A.5.7	Measurements Period Requirements without FH with both MG and PPW	1362

9.9A.5.8	Measurements Period Requirements with FH	1362

9.9C	NR measurements for positioning in Satellite Access	1362

9.9C.1	Introduction	1362

9.9C.1.1	General Aspects of Gap-based Measurement	1362

9.9C.1.2	General Aspects of Gapless Measurement	1363

9.9C.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1364

9.9C.2	Void	1364

9.9C.3	Void	1364

9.9C.4	UE Rx-Tx time difference measurements	1364

9.9C.4.1 Introduction	1364

9.9C.4.2 Requirements Applicability	1364

9.9C.4.3	Measurement Capability	1364

9.9C.4.4	Measurement Reporting Requirements	1364

9.9C.4.5	Measurement Period Requirements	1365

9.9C.4.6	Measurement Period Requirements without Measurement Gaps	1367

9.9D	NR measurements for positioning for RedCap in Satellite Access	1369

9.9D.1	Introduction	1369

9.9D.1.1	General Aspects of Gap-based Measurement	1369

9.9D.1.2	General Aspects of Gapless Measurement	1369

9.9D.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1369

9.9D.2	Void	1370

9.9D.3	Void	1370

9.9D.4	UE Rx-Tx time difference measurements	1370

9.9D.4.1 Introduction	1370

9.9D.4.2 Requirements Applicability	1370

9.9D.4.3	Measurement Capability	1370

9.9D.4.4	Measurement Reporting Requirements	1370

9.9D.4.5	Measurement Period Requirements	1370

9.9D.4.6	Measurement Period Requirements without Measurement Gaps	1371

9.10	CSI-RS based L3 measurements	1371

9.10.1	Introduction	1371

9.10.2	CSI-RS based intra-frequency measurements	1371

9.10.2.1	Introduction	1371

9.10.2.2	Requirements applicability	1372

9.10.2.3	Number of cells and number of CSI-RS	1373

9.10.2.3.1	Requirements for FR1	1373

9.10.2.3.2	Requirements for FR2	1373

9.10.2.4	Measurement Reporting Requirements	1373

9.10.2.4.1	Periodic Reporting	1373

9.10.2.4.2	Event-triggered Periodic Reporting	1373

9.10.2.4.3	Event Triggered Reporting	1374

9.10.2.5	Intra-frequency measurements without measurement gaps	1374

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1376

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1376

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	1377

9.10.3	CSI-RS based Inter-frequency measurements	1377

9.10.3.1	Introduction	1377

9.10.3.2	Requirements applicability	1377

9.10.3.3	Number of cells and number of CSI-RS resources	1378

9.10.3.3.1	Requirements for FR1	1378

9.10.3.3.2	Requirements for FR2	1378

9.10.3.4	Measurements reporting requirements	1378

9.10.3.4.1	Periodic Reporting	1378

9.10.3.4.2	Event-triggered Periodic Reporting	1378

9.10.3.4.3	Event-triggered Reporting	1379

9.10.3.5	Inter-frequency measurements with measurement gaps	1379

9.10D	CSI-RS based L3 measurements for ATG	1381

9.10D.1	Introduction	1381

9.10D.2	CSI-RS based intra-frequency measurements	1381

9.10D.2.1	Introduction	1381

9.10D.2.2	Requirements applicability	1381

9.10D.2.3	Number of cells and number of CSI-RS	1382

9.10D.2.3.1	Requirements for FR1	1382

9.10D.2.4	Measurement Reporting Requirements	1382

9.10D.2.4.1	Periodic Reporting	1383

9.10D.2.4.2	Event-triggered Periodic Reporting	1383

9.10D.2.4.3	Event Triggered Reporting	1383

9.10D.2.5	Intra-frequency measurements without measurement gaps	1383

9.10D.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1385

9.10D.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1385

9.10D.3	CSI-RS based Inter-frequency measurements	1386

9.10D.3.1	Introduction	1386

9.10D.3.2	Requirements applicability	1386

9.10D.3.3	Number of cells and number of CSI-RS resources	1386

9.10D.3.3.1	Requirements for FR1	1386

9.10D.3.4	Measurements reporting requirements	1387

9.10D.3.4.1	Periodic Reporting	1387

9.10D.3.4.2	Event-triggered Periodic Reporting	1387

9.10D.3.4.3	Event-triggered Reporting	1387

9.10D.3.5	Inter-frequency measurements with measurement gaps	1387

9.11	NR measurements with autonomous gaps	1389

9.11.1	Introduction	1389

9.11.2	CGI identification of an NR cell with autonomous gaps	1389

9.11.3	CGI reporting delay	1390

9.11A	NR measurements with autonomous gaps for RedCap	1390

9.11A.1	Introduction	1390

9.11A.2	CGI identification of an NR cell with autonomous gaps	1390

9.11A.3	CGI reporting delay	1391

9.11A.4	CGI reporting scheduling restriction	1391

9.11D	NR measurements with autonomous gaps for ATG	1392

9.11D.1	Introduction	1392

9.11D.2	CGI identification of an NR cell with autonomous gaps	1392

9.11D.3	CGI reporting delay	1393

9.12	Measurement for Propagation Delay Compensation	1393

9.12.1	Introduction	1393

9.12.2	Requirements Applicability	1393

9.12.3	Measurement Capability	1394

9.12.4	Measurement period requirements	1394

9.12.4.1	PRS Measurement Period	1394

9.12.4.2	TRS Measurement Period	1395

9.12.5	Measurement Reporting Requirements	1395

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	1396

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	1396

9.12.8	Measurement requirement for Propagation Delay Compensation with MUSIM gaps	1396

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	1396

9.13.1	Introduction	1396

9.13.2	Requirements Applicability	1397

9.13.3	Measurement Reporting Requirements	1397

9.13.3.1	Periodic Reporting	1398

9.13.3.2	Semi-Persistent Reporting	1398

9.13.3.3	Aperiodic Reporting	1398

9.13.3.4	Event triggered reporting for UE initiated beam management	1398

9.13.4	L1-RSRP measurement requirements	1399

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	1399

9.13.5	Measurement restriction for L1-RSRP measurement	1402

9.13.5.1	Measurement restriction for SSB based L1-RSRP	1402

9.13.6	Scheduling availability of UE during L1-RSRP measurement	1403

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1403

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1403

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1404

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1405

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1405

9.14	NR intra-frequency L1-RSRP measurements for neighbor cell	1405

9.14.1	Introduction	1405

9.14.2	Requirements Applicability	1406

9.14.3	Measurement Reporting Requirements	1406

9.14.3.1	Periodic Reporting	1406

9.14.3.2	Semi-Persistent Reporting	1407

9.14.3.3	Aperiodic Reporting	1407

9.14.3.4	Event Triggered Reporting	1407

9.14.3.5	Event-triggered Periodic Reporting	1407

9.14.4	Number of SSB frequency layers, number of cells and number of SSBs	1408

9.14.5	L1-RSRP intra-frequency measurement requirements without measurement gaps	1408

9.14.5.1	SSB based L1-RSRP reporting	1408

9.14.6	Measurement restriction for L1-RSRP measurement	1411

9.14.6.1	Measurement restriction for SSB based L1-RSRP	1411

9.14.7	Scheduling availability of UE during L1-RSRP measurement	1412

9.14.7.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1412

9.14.7.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1412

9.14.7.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1412

9.14.7.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1413

9.14.7.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1413

9.14a	CSI-RS based Intra-frequency L1-RSRP measurements for neighbor cell	1413

9.14a.1	Introduction	1413

The UE shall report the measurement quantity and send periodic, semi-persistent or aperiodic reports, according to the higher layer parameter ltm-ReportConfigType of each reporting setting LTM-CSI-ReportConfig.9.14a.2	Requirements Applicability	1414

9.14a.3	Measurement Reporting Requirements	1414

9.14a.3.1	Periodic Reporting	1414

9.14a.3.2	Semi-Persistent Reporting	1415

9.14a.3.3	Aperiodic Reporting	1415

9.14a.3.4	Event-triggered Periodic Reporting	1415

9.14a.3.5	Event Triggered Reporting	1415

9.14a.4	Number of CSI-RS resources, number of cells	1415

9.14a.5	CSI-RS based L1-RSRP measurement requirements without measurement gaps	1416

9.14a.6	Measurement restriction for CSI-RS based L1-RSRP measurement	1417

9.14a.6.1	Measurement restriction for CSI-RS based L1-RSRP measurement	1417

9.14a.7	Scheduling availability of UE during CSI-RS based L1-RSRP measurement	1419

9.14a.7.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1419

9.14a.7.2	Scheduling availability of UE performing L1-RSRP measurement on FR2	1419

9.14a.7.3	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1419

9.14a.7.4	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1420

9.15	NR inter-frequency L1-RSRP measurements for neighbor cell	1420

9.15.1	Introduction	1420

9.15.2	Requirements Applicability	1420

9.15.3	Measurement Reporting Requirements	1421

9.15.3.1	Periodic Reporting	1421

9.15.3.2	Semi-Persistent Reporting	1421

9.15.3.3	Aperiodic Reporting	1421

9.15.3.4	Event Triggered Reporting	1421

9.15.3.5	Event Triggered Periodic Reporting	1422

9.15.4	Number of SSB frequency layers, number of cells and number of SSBs	1422

9.15.5	L1-RSRP inter-frequency measurement requirements with measurement gaps	1422

9.15.5.1	Inter-frequency SSB based L1-RSRP reporting	1422

9.15.6	L1-RSRP inter-frequency L1-RSRP measurement requirements without measurement gaps	1424

9.15.6.1	Inter-frequency L1-RSRP measurement requirements	1424

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement	1424

9.15.6.2	Measurement restriction for inter-frequency L1-RSRP measurement	1427

9.15.6.2.1	Measurement restriction for SSB based L1-RSRP	1427

9.15.6.3	Scheduling availability of UE during inter-frequency L1-RSRP measurements	1428

9.15.6.3.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1428

9.15.6.3.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1428

9.15.6.3.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1429

9.15.6.3.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1429

9.15.6.3.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1429

9.16	CJT calibration reporting for Delay offset and Frequency offset	1430

9.16.1	Introduction	1430

9.16.2	Requirements applicability	1430

9.16.3	Measurement Reporting Requirements	1430

9.16.3.1	Aperiodic Reporting	1430

After the UE receives CSI request in DCI with (reportQuantity set to ‘cjtc-Dd’, ‘cjtc-F’), the UE shall transmit the aperiodic CJTC reporting on PUSCH over the air interface at the time specified according to relevant clause in [26].9.16.4 Measurement Requirements	1430

9.16.4.1 CSI -RS based delay and frequency offset reporting	1430

9.16.5	Measurement restriction for UE during CJT calibration reporting	1432

9.16.5.1	Measurement restriction for CJT calibration reporting	1432

9.16.6	Scheduling availability of UE during CJT calibration reporting	1432

9.16.6.1	Scheduling availability of UE performing measurement for CJT calibration reporting on FR1	1432

9.17	OD-SSB based L3 measurement for an SCell	1433

9.17.1	Introduction	1433

9.17.2	Requirements Applicability	1433

9.17.3	Number of cells and number of SSB	1433

9.17.4	Measurement Reporting Requirements	1433

9.17.5	OD-SSB based Intra-frequency measurements without measurement gaps	1434

9.17.5.1	Intra-frequency cell identification for active SCell	1434

9.17.5.2	Measurement period for active SCell	1439

9.17.5.3	Intra-frequency cell identification for deactivated SCell	1441

9.17.5.4	Measurement period for deactivated SCell	1443

9.17.5.5	Scheduling availability of UE during intra-frequency measurements based on On-demand SSB	1444

9.17.5.5.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1444

9.17.5.5.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1445

9.17.5.5.3	Scheduling availability of UE performing measurements on FR2-1	1445

9.17.5.5.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1446

9.18	L1 Cross Link Interference measurements	1447

9.18.1	Introduction	1447

9.18.2	L1-SRS-RSRP measurements	1447

9.18.2.1	Introduction	1447

9.18.2.2	Requirements applicability	1447

9.18.2.3	Measurement Reporting Requirements	1448

9.18.2.3.1	Aperiodic Reporting	1448

9.18.2.4	Measurement capability	1448

9.18.2.5	L1-SRS-RSRP measurement period	1448

9.18.2.6	Scheduling availability of UE during L1-CLI measurements	1449

9.18.2.6.1	Scheduling availability of UE performing L1-SRS-RSRP measurement on FR1	1449

9.18.2.6.2	Scheduling availability of UE performing L1-SRS-RSRP measurement on FR2	1449

9.18.3	L1-CLI-RSSI measurements	1450

9.18.3.1	Introduction	1450

9.18.3.2	Requirements applicability	1450

9.18.3.3	Measurement Reporting Requirements	1450

9.18.3.3.1	Periodic Reporting	1451

9.18.3.3.2	Aperiodic Reporting	1451

9.18.3.4	Measurement capability	1451

9.18.3.5	L1-CLI-RSSI measurement period	1451

9.18.3.6	Scheduling availability of UE during L1-CLI-RSSI measurements	1452

9.18.3.6.1	Scheduling availability of UE performing L1-CLI-RSSI measurement on FR1	1452

9.18.3.6.2	Scheduling availability of UE performing L1-CLI-RSSI measurement on FR2	1452

10	Measurement Performance requirements	1454

10.1	NR measurements	1454

10.1.1	Introduction	1454

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	1454

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	1454

10.1.2.1.1	Absolute SS-RSRP Accuracy	1454

10.1.2.1.2	Relative SS-RSRP Accuracy	1455

10.1.2.2	Void	1456

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	1456

10.1.2.3.1	Absolute CSI-RSRP Accuracy	1456

10.1.2.3.2	Relative CSI-RSRP Accuracy	1457

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1458

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	1458

10.1.2B.1.1	Absolute SS-RSRP Accuracy	1458

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	1459

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	1459

10.1.2C.1.1	Absolute SS-RSRP Accuracy	1459

10.1.2C.1.2	Relative SS-RSRP Accuracy	1460

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	1461

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	1461

10.1.3.1.1	Absolute SS-RSRP Accuracy	1461

10.1.3.1.2	Relative SS-RSRP Accuracy	1461

10.1.3.2	Void	1462

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	1462

10.1.3.3.1	Absolute CSI-RSRP Accuracy	1462

10.1.3.3.2	Relative CSI-RSRP Accuracy	1463

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1464

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	1464

10.1.3B.1.1	Absolute SS-RSRP Accuracy	1464

10.1.3C	Intra-frequency RSRP accuracy requirements for FR2-NTN	1464

10.1.3C.1	Intra-frequency SS-RSRP accuracy requirements	1464

10.1.3C.1.1	Absolute SS-RSRP Accuracy	1464

10.1.3C.1.2	Relative SS-RSRP Accuracy	1465

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	1466

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	1466

10.1.4.1.1	Absolute SS-RSRP Accuracy in FR1	1466

10.1.4.1.2	Relative SS-RSRP Accuracy in FR1	1467

10.1.4.2	Void	1468

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	1468

10.1.4.3.1	Absolute CSI-RSRP Accuracy in FR1	1468

10.1.4.3.2	Relative CSI-RSRP Accuracy in FR1	1469

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1470

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	1470

10.1.4B.1.1	Absolute SS-RSRP Accuracy in FR1	1471

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	1472

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	1472

10.1.4C.1.1	Absolute SS-RSRP Accuracy in FR1	1472

10.1.4C.1.2	Relative SS-RSRP Accuracy in FR1	1472

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	1473

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	1473

10.1.5.1.1	Absolute SS-RSRP Accuracy	1473

10.1.5.1.2	Relative SS-RSRP Accuracy	1473

10.1.5.2	Void	1474

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	1474

10.1.5.3.1	Absolute CSI-RSRP Accuracy	1474

10.1.5.3.2	Relative CSI-RSRP Accuracy	1475

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1476

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	1476

10.1.5B.1.1	Absolute SS-RSRP Accuracy	1476

10.1.5C	Inter-frequency RSRP accuracy requirements for FR2-NTN	1477

10.1.5C.1	Inter-frequency SS-RSRP accuracy requirements	1477

10.1.5C.1.1	Absolute SS-RSRP Accuracy	1477

10.1.5C.1.2	Relative SS-RSRP Accuracy	1477

10.1.6	RSRP Measurement Report Mapping	1478

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	1480

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1480

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	1480

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	1481

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	1481

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1482

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1482

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	1482

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	1483

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1483

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	1483

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	1484

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1484

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	1484

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	1484

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	1484

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1485

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1485

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	1485

10.1.8C	Intra-frequency RSRQ accuracy requirements for FR2-NTN	1486

10.1.8C.1	Intra-frequency SS-RSRQ accuracy requirements in FR2-NTN	1486

10.1.8C.1.1	Absolute SS-RSRQ Accuracy in FR2-NTN	1486

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	1487

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1487

10.1.9.1.1	Absolute SS-RSRQ Accuracy in FR1	1487

10.1.9.1.2	Relative SS-RSRQ Accuracy in FR1	1487

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	1488

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	1488

10.1.9.2.2	Relative CSI-RSRQ Accuracy	1489

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1490

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1490

10.1.9B.1.1	Absolute SS-RSRQ Accuracy in FR1	1490

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	1491

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1491

10.1.9C.1.1	Absolute SS-RSRQ Accuracy in FR1	1491

10.1.9C.1.2	Relative SS-RSRQ Accuracy in FR1	1492

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	1492

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1492

10.1.10.1.1	Absolute SS-RSRQ Accuracy in FR2	1492

10.1.10.1.2	Relative SS-RSRQ Accuracy in FR2	1493

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	1494

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	1494

10.1.10.2.2	Relative CSI-RSRQ Accuracy	1495

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1496

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1496

10.1.10B.1.1	Absolute SS-RSRQ Accuracy in FR2	1496

10.1.10C	Inter-frequency RSRQ accuracy requirements for FR2-NTN	1497

10.1.10C.1	Inter-frequency SS-RSRQ accuracy requirements in FR2-NTN	1497

10.1.10C.1.1	Absolute SS-RSRQ Accuracy in FR2-NTN	1497

10.1.10C.1.2	Relative SS-RSRQ Accuracy in FR2-NTN	1497

10.1.11	RSRQ report mapping	1498

10.1.12	Intra-frequency SINR accuracy requirements for FR1	1499

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	1499

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	1499

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	1499

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	1499

10.1.12C	 Intra-frequency SINR accuracy requirements for FR1 SAN	1500

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	1500

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	1500

10.1.13	Intra-frequency SINR accuracy requirements for FR2	1501

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	1501

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	1501

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	1502

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	1502

10.1.13C	Intra-frequency SINR accuracy requirements for FR2-NTN	1502

10.1.13C.1	Intra-frequency SS-SINR accuracy requirements in FR2-NTN	1502

10.1.13C.1.1	Absolute SS-SINR Accuracy in FR2-NTN	1502

10.1.14	Inter-frequency SINR accuracy requirements for FR1	1503

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	1503

10.1.14.1.1	Absolute SS-SINR Accuracy in FR1	1503

10.1.14.1.2	Relative SS-SINR Accuracy in FR1	1504

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	1505

10.1.14.2.1	Absolute CSI-SINR Accuracy in FR1	1505

10.1.14.2.2	Relative CSI-SINR Accuracy in FR1	1506

10.1.14C	 Inter-frequency SINR accuracy requirements for FR1 SAN	1507

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	1507

10.1.14C.1.1	Absolute SS-SINR Accuracy in FR1	1507

10.1.14C.1.2	Relative SS-SINR Accuracy in FR1	1508

10.1.15	Inter-frequency SINR accuracy requirements for FR2	1509

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	1509

10.1.15.1.1	Absolute SS-SINR Accuracy in FR2	1509

10.1.15.1.2	Relative SS-SINR Accuracy in FR2	1509

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	1510

10.1.15.2.1	Absolute CSI-SINR Accuracy in FR2	1510

10.1.15.2.2	Relative CSI-SINR Accuracy in FR2	1511

10.1.15C	Inter-frequency SINR accuracy requirements for FR2-NTN	1512

10.1.15C.1	Inter-frequency SS-SINR accuracy requirements in FR2-NTN	1512

10.1.15C.1.1	Absolute SS-SINR Accuracy in FR2-NTN	1512

10.1.15C.1.2	Relative SS-SINR Accuracy in FR2-NTN	1512

10.1.16	SINR report mapping	1513

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	1513

10.1.17	Power Headroom	1514

10.1.17.1	Power Headroom Report	1514

10.1.17.1.1	Power Headroom Report Mapping	1514

10.1.18	PCMAX,c,f	1514

10.1.18.1	Report Mapping	1514

10.1.19	L1-RSRP accuracy requirements for FR1	1515

10.1.19.1	SSB based L1-RSRP accuracy requirements	1515

10.1.19.1.1	Absolute Accuracy	1515

10.1.19.1.2	Relative Accuracy	1516

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	1517

10.1.19.2.1	Absolute Accuracy	1517

10.1.19.2.2	Relative Accuracy	1518

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	1519

10.1.19C.1	SSB based L1-RSRP accuracy requirements	1519

10.1.19C.1.1	Absolute Accuracy	1519

10.1.19C.1.2	Relative Accuracy	1520

10.1.19C.2	CSI-RS based L1-RSRP accuracy requirements	1520

10.1.19C.2.1	Absolute Accuracy	1520

10.1.19C.2.2	Relative Accuracy	1521

10.1.19D	LTM Intra-frequency L1-RSRP accuracy requirements for FR1	1522

10.1.19D.1	SSB based intra-frequency L1-RSRP accuracy requirements	1522

10.1.19D.1.1	Absolute Accuracy	1522

10.1.19D.1.2	Relative Accuracy	1523

10.1.19E	  LTM Inter-frequency L1-RSRP accuracy requirements for FR1	1523

10.1.19E.1	SSB based Inter-frequency L1-RSRP accuracy requirements	1523

10.1.19E.1.1	Absolute Accuracy	1523

10.1.19E.1.2	Relative Accuracy	1524

10.1.20	L1-RSRP accuracy requirements for FR2	1525

10.1.20.1	SSB based L1-RSRP accuracy requirements	1525

10.1.20.1.1	Absolute Accuracy	1525

10.1.20.1.2	Relative Accuracy	1526

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	1527

10.1.20.2.1	Absolute Accuracy	1527

10.1.20.2.2	Relative Accuracy	1527

10.1.20A	  LTM Intra-frequency L1-RSRP accuracy requirements for FR2	1528

10.1.20A.1	SSB based intra-frequency L1-RSRP accuracy requirements	1528

10.1.20A.1.1	Absolute Accuracy	1528

10.1.20A.1.2	Relative Accuracy	1529

10.1.20B	  LTM Inter-frequency L1-RSRP accuracy requirements for FR2	1530

10.1.20B.1	SSB based inter-frequency L1-RSRP accuracy requirements	1530

10.1.20B.1.1	Absolute Accuracy	1530

10.1.20B.1.2	Relative Accuracy	1530

10.1.20C	L1-RSRP accuracy requirements for FR2-NTN	1531

10.1.20C.1	SSB based L1-RSRP accuracy requirements	1531

10.1.20C.1.1	Absolute Accuracy	1531

10.1.20C.1.2	Relative Accuracy	1531

10.1.20C.2	CSI-RS based L1-RSRP accuracy requirements	1532

10.1.20C.2.1	Absolute Accuracy	1532

10.1.20C.2.2	Relative Accuracy	1533

10.1.21	SFTD accuracy requirements	1534

10.1.21.1	SFTD acuracy requirements for NE-DC	1534

10.1.21.2	SFTD acuracy requirements for NR-DC	1535

10.1.21.3	Inter-frequency SFTD acuracy requirements	1536

10.1.22	CLI measurement accuracy requirements	1538

10.1.22.1	SRS-RSRP	1538

10.1.22.1.1	SRS-RSRP Accuracy	1538

10.1.22.1.2	SRS-RSRP report mapping	1539

10.1.22.2	CLI-RSSI	1540

10.1.22.2.1	CLI-RSSI Accuracy	1540

10.1.22.2.2	CLI-RSSI report mapping	1541

10.1.23	RSTD Measurements	1541

10.1.23.1	Introduction	1541

10.1.23.2	Measurement Accuracy Requirements	1541

10.1.23.3	Report mapping	1549

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	1549

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	1552

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	1555

10.1.23A	RSTD Measurements Based on PRS Aggregation	1559

10.1.23A.1	Introduction	1559

10.1.23A.3	Report Mapping	1566

10.1.23A.3.1	Absolute DL RSTD Measurement Reporting	1566

10.1.23A.3.2	Differential Reporting for DL RSTD Measurement	1566

10.1.23A.3.3	Additional Path Report Mapping for DL RSTD	1566

10.1.24	PRS-RSRP Measurements	1566

10.1.24.1	Introduction	1566

10.1.24.2	Measurement Accuracy Requirements	1567

10.1.24.2.1	Absolute PRS-RSRP accuracy	1567

10.1.24.2.2	Relative PRS RSRP accuracy	1571

10.1.24.3	Report mapping	1575

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	1575

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	1576

10.1.24A	PRS-RSRP Measurements Based on PRS Aggregation	1578

10.1.24A.1	Introduction	1578

10.1.24A.2	Measurement Accuracy Requirements	1579

10.1.24A.2.1	Absolute PRS RSRP Accuracy Requirement	1579

10.1.24A.2.2	Relative PRS RSRP Accuracy Requirement	1579

10.1.24A.3	Report Mapping	1579

10.1.24A.3.1	Absolute PRS-RSRP Measurement Report Mapping	1579

10.1.24A.3.2	Differential Report Mapping for PRS-RSRP Measurement	1579

10.1.25	UE Rx-Tx Time Difference Measurements	1579

10.1.25.1	Introduction	1579

10.1.25.2	Measurement Accuracy Requirements	1579

10.1.25.3	Report mapping	1591

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1591

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	1594

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1597

10.1.25A	UE Rx-Tx Time Difference Measurement Based on PRS Aggregation	1600

10.1.25A.1	Introduction	1600

10.1.25A.2	Measurement Accuracy Requirements	1601

10.1.25A.3	Report mapping	1617

10.1.25C	UE Rx-Tx Time Difference Measurements in Satellite Accesss	1617

10.1.25C.1	Introduction	1617

10.1.25C.2	Measurement Accuracy Requirements	1617

10.1.25C.3	Report mapping	1618

10.1.26	FR2 P-MPR report	1618

10.1.26.1	Report mapping	1618

10.1.27	L1-SINR accuracy requirements for FR1	1619

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1619

10.1.27.1.1	Absolute Accuracy	1619

10.1.27.1.2	Relative Accuracy	1620

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1621

10.1.27.2.1	Absolute Accuracy	1621

10.1.27.2.2	Relative Accuracy	1623

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1625

10.1.27.3.1	Absolute Accuracy	1625

10.1.27.3.2	Relative Accuracy	1627

10.1.28	L1-SINR accuracy requirements for FR2	1629

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1636

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1636

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1636

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1636

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1636

10.1.30.1.1	Absolute SS-RSRQ Accuracy	1636

10.1.30.1.2	Relative SS-RSRQ Accuracy	1637

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1638

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1638

10.1.31.1.1	Absolute SS-SINR Accuracy	1638

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1638

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1638

10.1.32.1.1	Absolute SS-SINR Accuracy	1638

10.1.32.1.2	Relative SS-SINR Accuracy	1639

10.1.33	L1-RSRP accuracy requirements under CCA	1640

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1640

10.1.33.1.1	Absolute Accuracy	1640

10.1.33.1.2	Relative Accuracy	1640

10.1.34	RSSI measurements under CCA	1641

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1641

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1641

10.1.34.3	RSSI measurement report mapping	1641

10.1.35	Channel occupancy measurements under CCA	1642

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1642

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1642

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1642

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1642

10.1.36.1.1	Absolute SS-RSRP Accuracy	1642

10.1.36.1.2	Relative SS-RSRP Accuracy	1643

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1643

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1643

10.1.37.1.1	Absolute SS-RSRP	1643

10.1.37.1.2	Relative SS-RSRP Accuracy	1644

10.1.38	PRS-RSRPP Measurements	1645

10.1.38.1	Introduction	1645

10.1.38.2	Measurement Accuracy Requirements	1645

10.1.38.2.1	Absolute PRS RSRPP accuracy	1645

10.1.38.3	Report mapping	1649

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1649

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1650

10.1.38A	PRS-RSRPP Measurements Based on PRS Aggregation	1651

10.1. 38A.1	Introduction	1651

10.1.38A.2	Measurement Accuracy Requirements	1652

10.1.38A.2.1	Absolute PRS RSRPP accuracy	1652

10.1.38A.3	Report mapping	1652

10.1.38A.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1652

10.1.38A.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1652

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1652

10.1.39.1	Void	1652

10.1.39.2	 Measurement Accuracy Requirements for PRS	1652

10.1.39.3	 Measurement Accuracy Requirements for TRS	1655

10.1.40	Void	1659

10.1.41	FR1 DPC report	1659

10.1.41.1	Report mapping	1659

10.1.42	TDCP Measurement Report Mapping	1659

10.1.43	DL-RSCPD Measurements	1661

10.1.43.1	Introduction	1661

10.1.43.2.1	Measurement Accuracy Requirements	1661

10.1.43.3	Report Mapping	1668

10.1.43.3.1	Absolute DL RSCPD Measurement Reporting	1668

10.1.44	DL-RSCP Measurements	1669

10.1.44.1	Introduction	1669

10.1.44.2	Measurement Accuracy Requirements	1669

10.1.44.3	Report Mapping	1677

10.1.44.3.1	Relative DL RSCP Measurement Reporting	1677

10.1A	NR measurements for RedCap	1678

10.1A.1	Introduction	1678

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1678

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1678

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1678

10.1A.2.1.2	Relative SS-RSRP Accuracy	1679

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1680

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1680

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1680

10.1A.3.1.2	Relative SS-RSRP Accuracy	1680

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1680

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1680

10.1A.4.1.1	Absolute SS-RSRP Accuracy in FR1	1680

10.1A.4.1.2	Relative SS-RSRP Accuracy in FR1	1681

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1682

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1682

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1682

10.1A.5.1.2	Relative SS-RSRP Accuracy	1682

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1682

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1682

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1682

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1683

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1683

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1683

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1683

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1683

10.1A.8.1.1	Absolute SS-RSRQ in FR1	1683

10.1A.8.1.2	Relative SS-RSRQ Accuracy in FR1	1684

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1685

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1685

10.1A.9.1.1	Absolute SS-RSRQ Accuracy in FR2	1685

10.1A.9.1.2	Relative SS-RSRQ Accuracy in FR2	1685

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1685

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1685

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1685

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1686

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1686

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1686

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1686

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1686

10.1A.12.1.1	Absolute SS-SINR Accuracy in FR1	1686

10.1A.12.1.2	Relative SS-SINR Accuracy in FR1	1687

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1688

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1688

10.1A.13.1.1	Absolute SS-SINR Accuracy in FR2	1688

10.1A.13.1.2	Relative SS-SINR Accuracy in FR2	1688

10.1A.14	L1-RSRP accuracy requirements for FR1	1688

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1688

10.1A.14.1.1	Absolute Accuracy	1688

10.1A.14.1.2	Relative Accuracy	1689

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1690

10.1A.14.2.1	Absolute Accuracy	1690

10.1A.14.2.2	Relative Accuracy	1691

10.1A.15	 L1-RSRP accuracy requirements for FR2	1692

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1692

10.1A.15.1.1	Absolute Accuracy	1692

10.1A.15.1.2	Relative Accuracy	1692

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1692

10.1A.15.2.1	Absolute Accuracy	1692

10.1A.15.2.2	Relative Accuracy	1692

10.1A.16	RSTD Measurements for RedCap Positioning	1693

10.1A.16.1	Introduction	1693

10.1A.16.2	Measurement Accuracy Requirements	1693

10.1A.16.2.1	Accuracy requirement for RSTD measurement without RX FH	1693

10.1A.16.2.2	Accuracy requirement for RSTD measurement with RX FH	1700

10.1A.16.3	Report Mapping	1715

10.1A.16.3.1	Absolute DL RSTD Measurement Reporting	1715

10.1A.16.3.2	Differential Reporting for DL RSTD Measurement	1715

10.1A.16.3.3	Additional Path Report Mapping for DL RSTD	1715

10.1A.17	PRS-RSRP Measurements for RedCap positioning	1715

10.1A.17.1	Introduction	1715

10.1A.17.2	Measurement Accuracy Requirements	1715

10.1A.17.2.1	Absolute PRS RSRP Accuracy Requirement	1715

10.1A.17.2.2	Relative PRS RSRP Accuracy Requirement	1718

10.1A.17.3	Report Mapping	1718

10.1A.17.3.1	Absolute PRS-RSRP Measurement Report Mapping	1718

10.1A.17.3.2	Differential Report Mapping for PRS-RSRP Measurement	1718

10.1A.18	  UE Rx-Tx Time Difference Measurements for RedCap Positioning	1719

10.1A.18.1	Introduction	1719

10.1A.18.2	Measurement Accuracy Requirements	1719

10.1A.18.2.1	UE Rx-Tx Accuracy Requirement for 2RX RedCap UE without FH	1719

10.1A.18.2.2	UE Rx-Tx Accuracy Requirement for 1RX RedCap UE without FH	1720

10.1A.18.2.3	UE Rx-Tx Accuracy Requirement for 2RX RedCap UE with FH	1726

10.1A.18.3	Report mapping	1736

10.1A.18.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1736

10.1A.18.3.2	Differential UE Rx-Tx Measurement Report Mapping	1736

10.1A.18.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1736

10.1A.19	PRS-RSRPP Measurements for RedCap Positioning	1736

10.1A.19.1	Introduction	1736

10.1A.19.2	Measurement Accuracy Requirements	1737

10.1A.19.2.1	Absolute PRS RSRPP accuracy	1737

10.1A.19.3	Report mapping	1739

10.1A.19.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1739

10.1A.19.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1740

10.2	E-UTRAN measurements	1740

10.2.1	Introduction	1740

10.2.2	E-UTRAN RSRP measurements	1740

10.2.3	E-UTRAN RSRQ measurements	1740

10.2.4	E-UTRAN RSTD measurements	1740

10.2.5	E-UTRAN RS-SINR measurements	1741

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1741

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1741

10.2A	E-UTRAN measurements for RedCap	1742

10.2A.1	Introduction	1742

10.2A.2	E-UTRAN RSRP measurements	1742

10.2A.3	E-UTRAN RSRQ measurements	1742

10.2A.4	E-UTRAN RS-SINR measurements	1743

10.3	UTRAN FDD Measurements	1743

10.3.1	UTRAN FDD CPICH RSCP	1743

10.3.2	UTRAN FDD CPICH Ec/No	1744

10.4	V2X measurements	1744

10.4.1	Introduction	1744

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1744

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1744

10.4.2.2	PSBCH-RSRP Relative Accuracy	1745

10.4.2A	Intra-frequency PSBCH-RSRP accuracy requirements for FR1 under CCA	1746

10.4.2A.1	PSBCH-RSRP Absolute Accuracy	1746

10.4.2A.2	PSBCH-RSRP Relative Accuracy	1746

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1747

10.4.3.1	Absolute SL-RSSI Accuracy	1747

10.4.3A	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1 under CCA	1747

10.4.3A.1	Absolute SL-RSSI Accuracy	1747

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1748

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1748

10.4.4A	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1 under CCA	1748

10.4.4A.1	Absolute L1 SL-RSRP Accuracy	1748

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1749

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1749

10.4A	NR Sidelink Measurements for Positioning	1750

10.4A.1	Introduction	1750

10.4A.2	SL RSTD measurements	1750

10.4A.2.1	Measurement Report Mapping	1750

10.4A.2.1.1	Absolute SL RSTD Measurement Reporting	1750

10.4A.2.2	Measurement Accuracy Requirements	1751

10.4A.3	SL PRS-RSRP measurements	1753

10.4A.3.1	Measurement Report Mapping	1753

10.4A.3.1.1	Absolute SL PRS-RSRP Measurement Report Mapping	1753

10.4A.3.2	Measurement Accuracy Requirements	1754

10.4A.3.2.1	Absolute SL PRS-RSRP accuracy	1754

10.4A.4	SL Rx-Tx measurements	1755

10.4A.4.1	Measurement Report Mapping	1755

10.4A.4.1.1	Absolute SL Rx-Tx Measurement Report Mapping	1755

10.4A.4.2	Measurement Accuracy	1757

10.4A.5	SL PRS-RSRPP measurements	1758

10.4A.5.1	Measurement Report Mapping	1758

10.4A.5.1.1	Absolute SL PRS-RSRPP Measurement Report Mapping	1758

10.4A.5.2	Measurement Accuracy	1759

10.4A.5.2.1	Introduction	1759

10.4A.5.2.2	Measurement Accuracy Requirements	1760

10.4A.5.2.2.2	Absolute SL PRS-RSRPP accuracy	1760

10.4A.6	SL AoA measurements	1761

10.4A.6.1	Measurement Report Mapping	1761

10.4A.6.1.1	Absolute SL AoA Measurement Report Mapping	1761

10.4A.7	SL RTOA measurements	1762

10.4A.7.1	Measurement Report Mapping	1762

10.4A.7.1.1	Absolute SL RTOA Measurement Report Mapping	1762

11	Void	1764

12	V2X Requirements	1765

12.1	Introduction	1765

12.2	UE Transmit Timing	1765

12.2.1	Introduction	1765

12.2.2	GNSS as synchronization reference source	1766

12.2.3	NR Cell as synchronization reference source	1766

12.2.4	E-URTAN Cell as synchronization reference source	1766

12.2.5	SyncRef UE as synchronization reference source	1767

12.3	Initiation/Cease of SLSS Transmissions	1767

12.3.1	Introduction	1767

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1767

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1768

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1769

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1769

12.3A	Initiation/Cease of SLSS Transmissions with CCA	1770

12.3A.1	Introduction	1770

12.3A.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1770

12.3A.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1770

12.3A.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1770

12.3A.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1771

12.4	Selection / Reselection of V2X Synchronization Reference Source	1771

12.4A	Selection / Reselection of Sidelink Synchronization Reference Source with CCA	1773

12.5	L1 SL-RSRP measurements	1775

12.5.1	Introduction	1775

12.5.2	SL-RSRP measurements	1775

12.6	Congestion Control measurements	1776

12.7	Interruption	1776

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1776

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1776

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1778

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1778

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1779

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1779

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1780

12.7.8	Interruptions at NR sidelink discovery configuration	1780

12.7.9	Interruptions to WAN due to sidelink carrier addition/release	1780

12.8	Reliability of GNSS signal	1781

12.9	Scheduling availability	1781

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1781

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1781

12.10	Selection / Reselection of relay UE	1782

12.10.1	Introduction	1782

12.10.2	Selection / Reselection of relay UE	1782

12.11	Component Carrier Addition and Release Delay for Sidelink Carrier Aggregation	1782

12.12	Selection / Reselection of Synchronization Reference Source for NR SL Carrier Aggregation	1783

12A	NR Sidelink Measurements for Positioning	1784

12A.1	Introduction	1784

12A.2	SL RSTD measurements	1785

12A.2.1	Introduction	1785

12A.2.3	Measurement Capability	1785

12A.2.4	Measurement Reporting Requirements	1785

12A.2.5	Measurements Period Requirements	1785

12A.3	SL PRS-RSRP measurements	1786

12A.3.1	Introduction	1786

12A.3.2	Requirements Applicability	1787

12A.3.4	Measurement Reporting Requirements	1787

12A.3.5	Measurements Period Requirements	1787

12A.4	SL Rx-Tx measurements	1788

12A.4.1	Introduction	1788

12A.4.2	Requirements Applicability	1788

12A.4.3	Measurement Capability	1788

12A.4.4	Measurement Reporting Requirements	1788

12A.4.5	Measurement Period Requirements	1789

12A.5	SL PRS-RSRPP measurements	1790

12A.5.1	Introduction	1790

12A.5.2	Requirements Applicability	1790

12A.5.3	Measurement Capability	1790

12A.5.4	Measurement Reporting Requirements	1790

12A.5.5	Measurement Period Requirements	1790

12A.6	SL AoA measurements	1791

12A.6.1	Introduction	1791

12A.6.2	Requirements Applicability	1791

12A.6.3	Measurement Capability	1791

12A.6.4	Measurement Reporting Requirements	1791

12A.6.5	Measurement Period Requirements	1792

12A.7	SL RTOA measurements	1792

12A.7.1	Introduction	1792

12A.7.2	Requirements Applicability	1793

12A.7.3	Measurement Capability	1793

12A.7.4	Measurement Reporting Requirements	1793

12A.7.5	Measurement Period Requirements	1793

13	Measurement Performance Requirements for NR gNB	1794

13.1	UL-RTOA	1794

13.1.1	Report mapping	1794

13.1.1A	Additional Path Report Mapping for UL-RTOA	1798

13.2	gNB Rx-Tx time difference	1801

13.2.1	Report mapping	1801

13.2.1A	Additional Path Report Mapping for gNB Rx-Tx	1805

13.2.2	Measurement Accuracy Requirements	1808

13.2.2.1	Introduction	1808

13.3	UL SRS RSRP measurement	1810

13.3.1	Report mapping	1810

13.3.2	Measurement accuracy requirements	1810

13.3.2.1	Introduction	1810

13.3.2.2	Requirements	1811

13.4	AoA/ZoA	1811

13.4.1	Report mapping	1811

13.5	Timing advance (TADV)	1812

13.5.1	Report mapping	1812

13.6	UL SRS RSRPP measurement	1813

13.6.1	Report mapping	1813

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1813

13.7.1	Report mapping	1813

13.7.2	Measurement Accuracy Requirements	1814

13.7.2.1	Introduction	1814

13.7.2.2	Requirements	1814

13.8	UL-RSCP measurement	1815

13.8.1	Report mapping	1815

Annex A (normative):	Test Cases	1816

A.1	Purpose of annex	1816

A.2	Requirement classification for statistical testing	1816

A.2.1	Types of requirements in TS 38.133	1816

A.2.1.1	Time and delay requirements on UE higher layer actions	1816

A.2.1.2	Measurements of power levels, relative powers and time	1817

A.2.1.3	Implementation requirements	1817

A.2.1.4	Physical layer timing requirements	1817

A.2.1.5	Requirements under CCA	1817

A.3	RRM test configurations	1818

A.3.1	Reference measurement channels	1818

A.3.1.1	PDSCH	1818

A.3.1.1.1	FDD	1818

A.3.1.1.2	TDD	1819

A.3.1.2	CORESET for RMSI scheduling	1822

A.3.1.2.1	FDD	1822

A.3.1.2.2	TDD	1823

A.3.1.3	CORESET for RMC scheduling	1825

A.3.1.3.1	FDD	1825

A.3.1.3.2	TDD	1827

A.3.1.4	TDD UL/DL configuration	1831

A.3.1A	Reference measurement channels under CCA	1833

A.3.1A.1	PDSCH	1833

A.3.1A.1.1	TDD	1833

A.3.1A.2	CORESET for RMSI scheduling	1834

A.3.1A.2.1	TDD	1834

A.3.1A.3	CORESET for RMC scheduling	1835

A.3.1A.3.1	TDD	1835

A.3.1A.4	TDD UL/DL configuration	1835

A.3.1A.5	RMC burst transmission model	1836

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1836

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1836

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1837

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1837

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1838

A.3.2.2	Void	1839

A.3.3	Reference DRX configurations	1839

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1839

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1839

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1839

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1840

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1840

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1840

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1840

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1841

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1841

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1841

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1841

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1842

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1842

A.3.3.14	DRX Configuration 14: DRX cycle = 160 ms and TAT = Infinity	1842

A.3.4	Test Cases with Different Channel Bandwidths	1842

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1842

A.3.4.1.1	Introduction	1842

A.3.4.1.2	Principle of testing	1843

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1843

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1843

A.3.5.1.1	Introduction	1843

A.3.5.1.2	Principle of Testing	1843

A.3.6	Antenna configurations	1843

A.3.6.1	Antenna configurations for FR1	1843

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1843

A.3.6.1.1.1	Introduction	1843

A.3.6.1.1.2	Principle of testing	1843

A.3.6.1.2	Antenna connection for 8 Rx capable UEs	1846

A.3.6.1.2.1	Introduction	1846

A.3.6.1.2.2	Principle of testing	1846

A.3.6.1.3	Antenna connection for 6 Rx capable UEs	1848

A.3.6.1.3.1	Introduction	1848

A.3.6.1.3.2	Principle of testing	1848

A.3.6.2	Antenna configurations for FR2	1849

A.3.6A	Antenna configurations with unlicensed bands	1849

A.3.6A.1	Antenna configurations for FR1	1849

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1849

A.3.6A.1.1.1	Introduction	1849

A.3.6A.1.1.2	Principle of testing	1849

A.3.7	EN-DC test setup	1851

A.3.7.1	Introduction	1851

A.3.7.2	E-UTRAN Serving Cell Parameters	1851

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1851

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1852

A.3.7A	NR FR1-FR2 test setup	1853

A.3.7B	EN-DC test setup with unlicensed bands	1853

A.3.7B.1	Introduction	1853

A.3.7B.2	E-UTRAN Serving Cell Parameters	1853

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1853

A.3.7C	LTE-FR1/FR2 test setup	1854

A.3.7D	NE-DC test setup	1855

A.3.7D.1	Introduction	1855

A.3.7D.2	E-UTRAN Serving Cell Parameters	1855

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1855

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1855

A.3.8	PRACH configurations	1855

A.3.8.1	Introduction	1855

A.3.8.2	PRACH configurations in FR1	1855

A.3.8.2.1	FR1 PRACH configuration 1	1855

A.3.8.2.2	FR1 PRACH configuration 2	1856

A.3.8.2.3	FR1 PRACH configuration 3	1856

A.3.8.2.4	FR1 PRACH configuration 4	1857

A.3.8.2.5	FR1 PRACH configuration 5	1857

A.3.8.2.6	FR1 PRACH configuration 6	1858

A.3.8.3	PRACH configurations in FR2	1858

A.3.8.3.1	FR2 PRACH configuration 1	1858

A.3.8.3.2	FR2 PRACH configuration 2	1859

A.3.8.3.3	FR2 PRACH configuration 3	1860

A.3.8.3.4	FR2 PRACH configuration 4	1860

A.3.8.3.5	FR2 PRACH configuration 5	1861

A.3.8.3.6	FR2 PRACH configuration 6	1861

A.3.8A	PRACH configurations under CCA	1862

A.3.8A.1	Introduction	1862

A.3.8A.2	PRACH configurations in FR1	1862

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1862

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1863

A.3.9	BWP configurations	1864

A.3.9.1	Introduction	1864

A.3.9.2	Downlink BWP configurations	1864

A.3.9.2.1	Initial BWP	1864

A.3.9.2.2	Dedicated BWP	1865

A.3.9.3	Uplink BWP configurations	1865

A.3.9.3.1	Initial BWP	1865

A.3.9.3.2	Dedicated BWP	1866

A.3.9A	BWP configurations for RedCap	1866

A.3.9A.1	Introduction	1866

A.3.9A.2	Downlink BWP configurations	1866

A.3.9A.2.1	Dedicated BWP	1866

A.3.9A.3	Uplink BWP configurations	1867

A.3.9A.3.1	Dedicated BWP	1867

A.3.10	SSB Configurations	1867

A.3.10.1	SSB Configurations for FR1	1867

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1867

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1869

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1869

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1869

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1870

A.3.10.1.9	SSB pattern 9 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1870

A.3.10.1.10	SSB pattern 10 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1870

A.3.10.1.11	SSB pattern 11 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1871

A.3.10.1.12	SSB pattern 12 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1871

A.3.10.1.13	SSB pattern 13 in FR1: SSB allocation for SSB SCS=15 kHz in 3 MHz	1871

A.3.10.2	SSB Configurations for FR2	1872

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1872

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1872

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1873

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1873

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1873

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1874

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1874

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1874

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1875

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1875

A.3.10.2.19	SSB pattern 19 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1879

A.3.10.2.20	SSB pattern 20 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1880

A.3.10.2.21	SSB pattern 21 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1880

A.3.10.2.22	SSB pattern 22 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1881

A.3.10.2.23	SSB pattern 23 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1881

A.3.10.2.24	SSB pattern 24 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1881

A.3.10.2.25	SSB pattern 25 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1882

A.3.10.2.26	SSB pattern 26 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1882

A.3.10.2.27	SSB pattern 27 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1883

A.3.10A	SSB Configurations under CCA	1883

A.3.10A.1	SSB Configurations under CCA for FR1	1883

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1883

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1884

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1884

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1885

A.3.10B	SSB Configurations for RedCap	1885

A.3.10B.1	SSB Configurations for FR1	1885

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1885

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1886

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1886

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1887

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1887

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1888

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1888

A.3.10B.2	SSB Configurations for FR2	1889

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1889

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1889

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1890

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1890

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1891

A.3.11	SMTC Configurations	1891

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1891

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1891

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1891

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1892

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1892

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1892

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1892

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1892

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	1892

A.3.11.10	SMTC pattern 10: SMTC period = 80 ms with SMTC duration = 1 ms	1893

A.3.11.11	SMTC pattern 11: SMTC period = 80 ms with SMTC duration = 5 ms	1893

A.3.11.12	SMTC pattern 12: SMTC period = 20 ms with SMTC duration = 5 ms	1893

A.3.11A	SMTC Configurations for RedCap	1893

A.3.11A.0	Introduction	1893

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1894

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1894

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1894

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1894

A.3.12	Test Cases with Different CC Configurations	1894

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1894

A.3.12.1.1	Introduction	1894

A.3.12.1.2	Principle of testing	1894

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1895

A.3.12.2.1	Introduction	1895

A.3.12.2.2	Principle of testing	1895

A.3.13	Test Cases in SA and EN-DC Operations	1895

A.3.13.1	Introduction	1895

A.3.13.2	Principle of Testing	1895

A.3.13B	 Test Cases for EN-DC and NE-DC Operations	1896

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1896

A.3.13B.1.1	Introduction	1896

A.3.13B.1.2	Principle of Testing	1896

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1896

A.3.13B.2.1	Introduction	1896

A.3.13B.2.2	Principle of Testing	1897

A.3.14	CSI-RS configurations	1897

A.3.14.1	FDD	1897

A.3.14.2	TDD	1899

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1904

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1904

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1904

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1904

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1905

A.3.15.3	Setup 3: 2 AoAs	1905

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1905

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1905

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1905

A.3.15.4.3	Setup 4c: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak for power class 6 UE supporting simultaneous reception from multiple directions	1905

A.3.15.5	Setup 5: 2 AoAs for simultaneous reception with QCL Type-D	1906

A.3.15.6	Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D	1906

A.3.15.7	Setup 7: 3 AoAs	1906

A.3.15C	Angle of Arrival (AoA) for FR2-NTN RRM test cases	1906

A.3.15C.1	Setup 1: Single AoA	1906

A.3.15C.2	Setup 2: 2 AoAs	1907

A.3.16	TCI State Configuration	1907

A.3.16.1	Introduction	1907

A.3.16.2	TCI states	1907

A.3.16A	Unified TCI State Configuration	1907

A.3.16A.1	Introduction	1907

A.3.16A.2	DLorJoint TCI states	1908

A.3.16A.3	UL TCI states	1909

A.3.16B	 LTM Candidate TCI State Configuration	1909

A.3.16B.1	Introduction	1909

A.3.16B.2	LTM candidate DLorJoint TCI states	1910

A.3.16B.3	LTM candidate UL TCI states	1910

A.3.17	Configurations of CSI-RS for tracking	1911

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1911

A.3.17.1.1	FDD	1911

A.3.17.1.2	TDD	1914

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1918

A.3.17.2.1	TDD	1918

A.3.17.2.2	FDD	1921

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1921

A.3.18.1	Introduction	1921

A.3.18.2	PRACH Power Measurement	1922

A.3.19	Test applicability for DAPS handover	1922

A.3.19.1	Introduction	1922

A.3.19.2	Principle of testing	1922

A.3.20	MsgA configurations	1922

A.3.20.1	Introduction	1922

A.3.20.2	MsgA configurations in FR1	1922

A.3.20.2.1	FR1 MsgA configuration 1	1922

A.3.20.2.2	FR1 MsgA configuration 2	1923

A.3.20.3	MsgA configurations in FR2	1924

A.3.20.3.1	FR2 MsgA configuration 1	1924

A.3.20.3.2	FR2 MsgA configuration 2	1925

A.3.20A	MsgA configurations under CCA	1926

A.3.20A.1	Introduction	1926

A.3.20A.2	MsgA configurations in FR1	1926

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1926

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1927

A.3.21	V2X sidelink communication	1928

A.3.21.1	Introduction	1928

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1929

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1932

A.3.21.4	Reference SL-DRX configurations	1933

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1933

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1933

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1933

A.3.21A	NR Sidelink Measurements for Positioning	1933

A.3.21A.1	Introduction	1933

A.3.21A.2	NR SL-PRS configurations	1934

A.3.21A.2.1	NR SL-PRS configurations for FR1	1934

A.3.22	CSI-IM configurations	1934

A.3.22.1	FDD	1934

A.3.22.2	TDD	1934

A.3.23	Spatial Relation Configuration	1935

A.3.23.1	Introduction	1935

A.3.23.2	Spatial Relation	1936

A.3.24	SRS configuration	1936

A.3.25	Channel bandwidth (CBW) configurations	1938

A.3.25.1	DL UE specific CBW	1938

A.3.25.2	UL UE specific CBW	1939

A.3.26	CCA model	1939

A.3.26.1	Introduction	1939

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1939

A.3.26.2.1	DL CCA model	1939

A.3.26.2.2	UL CCA model	1940

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1941

A.3.26.3.1	DL CCA model	1941

A.3.26.3.2	UL CCA model	1941

A.3.26.4	CCA model for operation on a sidelink carrier frequency with CCA	1942

A.3.26.4.1	CCA model for SyncRef UE	1942

A.3.27	Void	1943

A.3.27.1	Void	1943

A.3.27.2	Void	1943

A.3.27.3	Void	1943

A.3.27.4	Void	1943

A.3.27.5	Void	1943

A.3.28	Discovery Burst Transmission Window configuration under CCA	1943

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1943

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1943

A.3.29.1	Introduction	1943

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1943

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1944

A.3.30	CSI-RS configurations for RRM	1944

A.3.30.1	FDD	1944

A.3.30.2	TDD	1945

A.3.31	PRS Configurations	1946

A.3.31.1	PRS Configurations for FR1	1946

A.3.31.1.1	PRS pattern 1 in FR1: SCS=15 kHz	1946

A.3.31.1.2	PRS pattern 2 in FR1: SCS=30 kHz	1947

A.3.31.2	PRS Configurations for FR2	1948

A.3.31.2.1	PRS pattern 1 in FR2: SCS=120 kHz	1948

A.3.32	NR sidelink discovery	1948

A.3.32.1	Introduction	1948

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1948

A.3.32.3	Principle of Testing	1949

A.3.33	PRS Processing Window (PPW) configurations	1949

A.3.34	Testing principles for test cases related to PRS measurements	1949

A.3.34.1	Introduction	1949

A.3.34.2	Test cases in RRC_INACTIVE state	1949

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1950

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1950

A.3.34.5	Testing principles for positioning measurements by aggregating PRS resources from multiple PFLs	1950

A.3.34.6	Testing principles for carrier phase measurement for positioning	1951

A.3.34.7	Test cases in RRC_IDLE state	1951

A.3.35	Testing principle for RedCap UE	1951

A.3.35.1	Introduction	1951

A.3.35.2	Principle of testing for FR1	1951

A.3.35.3	Principle of testing for FR2	1951

A.3.35.4	Principle of testing for PRS measurement	1951

A.3.36	Testing related to Satellite access	1952

A.3.36.1	Introduction	1952

A.3.36.1	Introduction	1952

A.3.36.2	Principle of testing GSO and NGSO scenarios	1952

A.3.36.2	Principle of testing different RRM requirements	1952

A.3.36.3	Principle of testing different ephemeris formats	1953

A.3.36.4	General setup for SIB19	1955

A.3.36.5	Satellite specific parameters configuration	1956

A.3.36.5.1	Satellite specific configuration for serving cell	1956

A.3.36.5.2	Satellite specific configuration for neighbour cell	1956

A.3.37	Reference Cell DTX configurations	1957

A.3.37.1	Cell DTX Configuration 1: Cell DTX cycle = 160 ms and TAT = Infinity	1957

A.3.38	DL-PRS Measurement Time Window configurations	1957

A.4	EN-DC tests with all NR cells in FR1	1959

A.4.1	Void	1959

A.4.2	Void	1959

A.4.3	RRC_CONNECTED state mobility	1959

A.4.3.1	Void	1959

A.4.3.2	RRC Connection Mobility Control	1959

A.4.3.2.1	Void	1959

A.4.3.2.2	Random Access	1959

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1959

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1962

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1964

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	1967

A.4.3.2.3	Void	1969

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1969

A.4.3.3.1	Test Purpose and Environment	1969

A.4.3.3.2	Test Requirements	1972

A.4.4	Timing	1973

A.4.4.1	UE transmit timing	1973

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1973

A.4.4.1.1.1	Test Purpose and environment	1973

A.4.4.1.1.2	Test requirements	1975

A.4.4.1.2	NR UE Transmit Timing Test for two TRPs in FR1	1976

A.4.4.1.2.1	Test Purpose and environment	1976

A.4.4.1.2.2	Test requirements	1979

A.4.4.2	UE timer accuracy	1980

A.4.4.3	Timing advance	1980

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1980

A.4.4.3.1.1	Test Purpose and Environment	1980

A.4.4.3.1.2	Test Parameters	1980

A.4.4.3.1.3	Test Requirements	1983

A.4.5	Signaling characteristics	1983

A.4.5.1	Radio link Monitoring	1983

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1983

A.4.5.1.1.1	Test Purpose and Environment	1983

A.4.5.1.1.2	Test Requirements	1987

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1987

A.4.5.1.2.1	Test Purpose and Environment	1987

A.4.5.1.2.2	Test Requirements	1990

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1990

A.4.5.1.3.1	Test Purpose and Environment	1990

A.4.5.1.3.2	Test Requirements	1993

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1993

A.4.5.1.4.1	Test Purpose and Environment	1993

A.4.5.1.4.2	Test Requirements	1996

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1996

A.4.5.1.5.1	Test Purpose and Environment	1996

A.4.5.1.5.2	Test Requirements	1999

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	2000

A.4.5.1.6.1	Test Purpose and Environment	2000

A.4.5.1.6.2	Test Requirements	2002

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	2003

A.4.5.1.7.1	Test Purpose and Environment	2003

A.4.5.1.7.2	Test Requirements	2005

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	2006

A.4.5.1.8.1	Test Purpose and Environment	2006

A.4.5.1.8.2	Test Requirements	2009

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	2009

A.4.5.1.9.1	Test Purpose and Environment	2009

A.4.5.1.10	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	2012

A.4.5.1.10.1	Test Purpose and Environment	2012

A.4.5.1.11	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	2012

A.4.5.1.11.1	Test Purpose and Environment	2012

A.4.5.1.11.2	Test Requirements	2013

A.4.5.1.12	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	2013

A.4.5.1.12.1	Test Purpose and Environment	2013

A.4.5.1.12.2	Test Requirements	2016

A.4.5.2	Interruption	2016

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	2016

A.4.5.2.1.1	Test Purpose and Environment	2016

A.4.5.2.1.2	Test Requirements	2018

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	2018

A.4.5.2.2.1	Test Purpose and Environment	2018

A.4.5.2.2.2	Test Requirements	2020

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	2020

A.4.5.2.3.1	Test Purpose and Environment	2020

A.4.5.2.3.2	Test Requirements	2024

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	2025

A.4.5.2.4.1	Test Purpose and Environment	2025

A.4.5.2.4.2	Test Requirements	2029

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	2029

A.4.5.2.5.1	Test Purpose and Environment	2029

A.4.5.2.5.2	Test Requirements	2031

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	2032

A.4.5.2.6.1	Test Purpose and Environment	2032

A.4.5.2.6.2	Test Requirements	2034

A.4.5.2.7	Void	2034

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	2034

A.4.5.2.8.1	Test Purpose and Environment	2034

A.4.5.2.8.2	Test Requirements	2037

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	2037

A.4.5.2.9.1	Test Purpose and Environment	2037

A.4.5.2.9.2	Test Requirements	2040

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	2040

A.4.5.2.10.1	Test Purpose and Environment	2040

A.4.5.2.10.2	Test Requirements	2042

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	2042

A.4.5.2.11.1	Test Purpose and Environment	2042

A.4.5.2.11.2	Test Requirements	2045

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	2045

A.4.5.2.12.1	Test Purpose and Environment	2045

A.4.5.3	SCell Activation and Deactivation Delay	2049

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160 ms SCell measurement cycle	2049

A.4.5.3.1.1	Test Purpose and Environment	2049

A.4.5.3.1.2	Test Requirements	2055

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640 ms SCell measurement cycle	2055

A.4.5.3.2.1	Test Purpose and Environment	2055

A.4.5.3.2.2	Test Requirements	2055

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	2056

A.4.5.3.3.1	Test Purpose and Environment	2056

A.4.5.3.3.2	Test Requirements	2056

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	2056

A.4.5.3.4.1	Test Purpose and Environment	2057

A.4.5.3.4.2	Test Requirements	2059

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	2059

A.4.5.3.5.1	Test Purpose and Environment	2059

A.4.5.3.5.2	Test Requirements	2064

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160 ms SCell measurement cycle	2064

A.4.5.3.6.1	Test Purpose and Environment	2064

A.4.5.3.6.2	Test Requirements	2067

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	2068

A.4.5.3.7.1	Test Purpose and Environment	2068

A.4.5.3.7.2	Test Requirements	2068

A.4.5.3.8	SCell Activation and deactivation of unknown SCell in FR1 for UE capable of short measurement interval	2068

A.4.5.3.8.1	Test Purpose and Environment	2068

A.4.5.3.8.2	Test Requirements	2069

A.4.5.3.9	SCell Activation of unknown SCell with valid L3 measurement results in FR1 for 160 ms SCell measurement cycle	2070

A.4.5.3.9.1	Test Purpose and Environment	2070

A.4.5.3.9.2	Test Requirements	2075

A.4.5.3.10	SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation command in non-DRX	2076

A.4.5.3.10.1	Test Purpose and Environment	2076

A.4.5.3.10.2	Test Requirements	2078

A.4.5.3.11	TRS-based SCell Activation of SSB-less SCell in FR1 collocated inter-band	2079

A.4.5.3.11.1	Test Purpose and Environment	2079

A.4.5.3.11.2	Test Requirements	2082

A.4.5.3.12	Inter-band SSB-less Scell activation using A-TRS	2083

A.4.5.3.12.1	Test Purpose and Environment	2083

A.4.5.3.12.2	Test Requirements	2086

A.4.5.4	UE UL carrier RRC reconfiguration Delay	2086

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	2086

A.4.5.4.1.1 Test Purpose and Environment	2086

A.4.5.4.1.2	Test Requirements	2091

A.4.5.5	Beam Failure Detection and Link recovery procedures	2091

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2091

A.4.5.5.1.1	Test Purpose and Environment	2091

A.4.5.5.1.2	Test Requirements	2095

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2095

A.4.5.5.2.1	Test Purpose and Environment	2095

A.4.5.5.2.2	Test Requirements	2098

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	2099

A.4.5.5.3.1	Test Purpose and Environment	2099

A.4.5.5.3.2	Test Requirements	2102

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2103

A.4.5.5.4.1	Test Purpose and Environment	2103

A.4.5.5.4.2	Test Requirements	2106

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2107

A.4.5.5.5.1	Test Purpose and Environment	2107

A.4.5.5.5.2	Test Requirements	2110

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2111

A.4.5.5.6.1	Test Purpose and Environment	2111

A.4.5.5.6.2	Test Requirements	2114

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2115

A.4.5.5.7.1	Test Purpose and Environment	2115

A.4.5.5.7.2	Test Requirements	2118

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2119

A.4.5.5.8.1	Test Purpose and Environment	2119

A.4.5.5.8.2	Test Requirements	2123

A.4.5.6	Active BWP switch	2123

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	2123

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2123

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2127

A.4.5.6.2	RRC-based Active BWP Switch	2132

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2135

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	2135

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	2140

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	2140

A.4.5.6.4.1.1	Test Purpose and Environment	2140

A.4.5.6.4.1.2	Test Requirements	2144

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	2144

A.4.5.6.4.2.1	Test Purpose and Environment	2144

A.4.5.6.4.2.2	Test Requirements	2150

A.4.5.6.5	SCell dormancy switch	2150

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	2150

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	2155

A.4.5.6.5.2.1	Test Purpose and Environment	2155

A.4.5.6.5.2.2	Test Requirements	2159

A.4.5.7	PSCell addition and release delay	2159

A.4.5.7.1	Addition and Release Delay of known NR PSCell	2159

A.4.5.7.1.1	Test purpose and environment	2159

A.4.5.7.1.2	Test Requirements	2162

A.4.5.8	DL Interruptions at switching between two uplink carriers	2162

A.4.5.8.1	Test Purpose and Environment	2162

A.4.5.8.2	Test Requirements	2166

A.4.5.9	UE specific CBW change	2166

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	2166

A.4.5.9.1.1	Test Purpose and Environment	2166

A.4.5.9.1.2	Test Requirements	2169

A.4.5.10	PSCell activation and deactivation delay	2169

A.4.5.10.1	PSCell activation and deactivation delay	2169

A.4.5.10.1.1	Test purpose and environment	2169

A.4.5.10.1.2	Test Requirements	2171

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	2172

A.4.5.11.1	Conditional PSCell Addition and Release Delay	2172

A.4.5.11.1.1	Test purpose and environment	2172

A.4.5.11.1.2	Test Parameters	2172

A.4.5.11.1.3	Test Requirements	2174

A.4.6	Measurement procedure	2174

A.4.6.1	Intra-frequency Measurements	2174

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	2174

A.4.6.1.1.1	Test purpose and Environment	2174

A.4.6.1.1.2	Test parameters	2174

A.4.6.1.1.3	Test Requirements	2176

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	2176

A.4.6.1.2.1	Test purpose and Environment	2176

A.4.6.1.2.2	Test parameters	2177

A.4.6.1.2.2	Test Requirements	2179

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	2179

A.4.6.1.3.1	Test purpose and Environment	2179

A.4.6.1.3.2	Test parameters	2179

A.4.6.1.3.3	Test Requirements	2181

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	2181

A.4.6.1.4.1	Test purpose and Environment	2181

A.4.6.1.4.2	Test parameters	2181

A.4.6.1.4.3	Test Requirements	2183

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	2184

A.4.6.1.5.1	Test purpose and Environment	2184

A.4.6.1.5.2	Test parameters	2184

A.4.6.1.5.3	Test Requirements	2185

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	2185

A.4.6.1.6.1	Test purpose and Environment	2185

A.4.6.1.6.2	Test parameters	2185

A.4.6.1.6.3	Test Requirements	2187

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2187

A.4.6.1.7.1	Test purpose and Environment	2187

A.4.6.1.7.2	Test parameters	2187

A.4.6.1.7.3	Test Requirements	2189

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	2189

A.4.6.1.8.1	Test Purpose and Environment	2189

A.4.6.1.8.2	Test Requirements	2193

A.4.6.1.9	EN-DC event triggered reporting tests without gap under non-DRX with NCD-SSB	2193

A.4.6.1.9.1	Test purpose and Environment	2193

A.4.6.1.9.2	Test parameters	2193

A.4.6.1.9.3	Test Requirements	2195

A.4.6.1.10	EN-DC event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP	2195

A.4.6.110.1	Test purpose and Environment	2195

A.4.6.1.10.2	Test Requirements	2195

A.4.6.2	Inter-frequency Measurements	2195

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2196

A.4.6.2.1.1	Test Purpose and Environment	2196

A.4.6.2.1.2	Test Requirements	2198

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2198

A.4.6.2.2.1	Test Purpose and Environment	2198

A.4.6.2.2.2	Test Requirements	2201

A.4.6.2.3	Void	2201

A.4.6.2.4	Void	2201

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2201

A.4.6.2.5.1	Test Purpose and Environment	2201

A.4.6.2.5.2	Test Requirements	2204

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2204

A.4.6.2.6.1	Test Purpose and Environment	2204

A.4.6.2.6.2	Test Requirements	2206

A.4.6.2.7	Void	2207

A.4.6.2.8	Void	2207

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2207

A.4.6.2.9.1	Test Purpose and Environment	2207

A.4.6.2.9.2	Test Requirements	2210

A.4.6.3	Void	2210

A.4.6.4	L1-RSRP measurement for beam reporting	2210

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2210

A.4.6.4.1.1	Test Purpose and Environment	2210

A.4.6.4.1.2	Test parameters	2210

A.4.6.4.1.3	Test Requirements	2211

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	2212

A.4.6.4.2.1	Test Purpose and Environment	2212

A.4.6.4.2.2	Test parameters	2212

A.4.6.4.2.3	Test Requirements	2213

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2214

A.4.6.4.3.1	Test Purpose and Environment	2214

A.4.6.4.3.2	Test parameters	2214

A.4.6.4.3.3	Test Requirements	2215

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2216

A.4.6.4.4.1	Test Purpose and Environment	2216

A.4.6.4.4.2	Test parameters	2216

A.4.6.4.4.3	Test Requirements	2217

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2218

A.4.6.4.5.1	Test Purpose and Environment	2218

A.4.6.4.5.2	Test parameters	2218

A.4.6.4.5.3	Test Requirements	2219

A.4.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2219

A.4.6.4.6.1	Test Purpose and Environment	2220

A.4.6.4.7	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2220

A.4.6.4.7.1	Test Purpose and Environment	2220

A.4.6.4.7.2	Test Requirements	2220

A.4.6.4.8	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	2220

A.4.6.4.8.1	Test Purpose and Environment	2220

A.4.6.4.8.2	Test parameters	2220

A.4.6.4.8.3	Test Requirements	2222

A.4.6.5	CLI measurements	2222

A.4.6.5.1	SRS-RSRP measurement with non-DRX	2222

A.4.6.5.1.1	Test Purpose and Environment	2222

A.4.6.5.1.2	Test Parameters	2222

A.4.6.5.1.3	Test Requirements	2225

A.4.6.5.2	CLI-RSSI measurement with non-DRX	2225

A.4.6.5.2.1	Test Purpose and Environment	2225

A.4.6.5.2.2	Test Parameters	2225

A.4.6.5.2.3	Test Requirements	2226

A.4.6.6.1.2	Test Requirements	2230

A.4.6.7	L1-SINR measurement for beam reporting	2230

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	2232

A.4.6.7.2.1	Test Purpose and Environment	2232

A.4.6.7.2.2	Test parameters	2233

A.4.6.7.2.3	Test Requirements	2234

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	2234

A.4.6.7.3.1	Test Purpose and Environment	2235

A.4.6.7.3.2	Test parameters	2235

A.4.6.7.3.3	Test Requirements	2236

A.4.6.8	CSI-RS based intra-frequency Measurement	2237

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	2237

A.4.6.8.1.1	Test purpose and Environment	2237

A.4.6.8.1.2	Test Requirements	2239

A.4.6.9	CSI-RS based inter-frequency Measurement	2239

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	2239

A.4.6.9.1.1	Test Purpose and Environment	2239

A.4.6.9.1.2	Test Requirements	2241

A.4.7	Measurement Performance requirements	2243

A.4.7.1	SS-RSRP	2243

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2243

A.4.7.1.1.1	Test Purpose and Environment	2243

A.4.7.1.1.2	Test parameters	2243

A.4.7.1.1.3	Test Requirements	2248

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2248

A.4.7.1.2.1	Test Purpose and Environment	2248

A.4.7.1.2.2	Test parameters	2248

A.4.7.1.2.3	Test Requirements	2251

A.4.7.1.3	Void	2251

A.4.7.2	SS-RSRQ	2251

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2251

A.4.7.2.1.1	Test Purpose and Environment	2251

A.4.7.2.1.2	Test Parameters	2251

A.4.7.2.1.3	Test Requirements	2255

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2255

A.4.7.2.2.1	Test Purpose and Environment	2255

A.4.7.2.2.2	Test Parameters	2255

A.4.7.2.2.3	Test Requirements	2259

A.4.7.3	SS-SINR	2259

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2259

A.4.7.3.1.1	Test Purpose and Environment	2259

A.4.7.3.1.2	Test Parameters	2259

A.4.7.3.1.3	Test Requirements	2263

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2263

A.4.7.3.2.1	Test Purpose and Environment	2263

A.4.7.3.2.2	Test Parameters	2263

A.4.7.3.2.3	Test Requirements	2266

A.4.7.4	L1-RSRP measurement for beam reporting	2266

A.4.7.4.1	SSB based L1-RSRP measurement	2266

A.4.7.4.1.1	Test Purpose and Environment	2266

A.4.7.4.1.2	Test parameters	2266

A.4.7.4.1.3	Test Requirements	2269

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2269

A.4.7.4.2.1	Test Purpose and Environment	2269

A.4.7.4.2.2	Test parameters	2270

A.4.7.4.2.3	Test Requirements	2273

A.4.7.5	SFTD accuracy	2273

A.4.7.5.1	SFTD accuracy	2273

A.4.7.5.1.1	Test Purpose and Environment	2273

A.4.7.5.1.2	Test Parameters	2273

A.4.7.5.1.3	Test Requirements	2276

A.4.7.5.2	Void	2276

A.4.7.5.3	Void	2276

A.4.7.6	CLI measurements	2276

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	2276

A.4.7.6.1.1	Test Purpose and Environment	2276

A.4.7.6.1.2	Test parameters	2276

A.4.7.6.1.3	Test Requirements	2279

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	2279

A.4.7.6.2.1	Test Purpose and Environment	2279

A.4.7.6.2.2	Test parameters	2280

A.4.7.6.2.3	Test Requirements	2281

A.4.7.7	L1-SINR measurement for beam reporting	2281

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2285

A.4.7.7.2.1	Test Purpose and Environment	2285

A.4.7.7.2.2	Test parameters	2285

A.4.7.7.2.3	Test Requirements	2288

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2288

A.4.7.7.3.1	Test Purpose and Environment	2288

A.4.7.7.3.2	Test parameters	2289

A.4.7.7.3.3	Test Requirements	2292

A.4.7.8	CSI-RSRP	2292

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2292

A.4.7.8.1.1	Test Purpose and Environment	2292

A.4.7.8.1.2	Test parameters	2292

A.4.7.8.1.3	Test Requirements	2296

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2296

A.4.7.8.2.1	Test Purpose and Environment	2296

A.4.7.8.2.2	Test parameters	2297

A.4.7.8.2.3	Test Requirements	2300

A.4.7.9	CSI-RSRQ	2300

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2300

A.4.7.9.1.1	Test Purpose and Environment	2300

A.4.7.9.1.2	Test Parameters	2300

A.4.7.9.1.3	Test Requirements	2304

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2304

A.4.7.9.2.1	Test Purpose and Environment	2304

A.4.7.9.2.2	Test Parameters	2304

A.4.7.9.2.3	Test Requirements	2308

A.4.7.10	CSI-SINR	2308

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2308

A.4.7.10.1.1	Test Purpose and Environment	2308

A.4.7.10.1.2	Test Parameters	2308

A.4.7.10.1.3	Test Requirements	2311

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2312

A.4.7.10.2.1	Test Purpose and Environment	2312

A.4.7.10.2.2	Test Parameters	2312

A.4.7.10.2.3	Test Requirements	2315

A.4.7.11	TDCP amplitude measurement accuracy	2315

A.4.7.11.1	TDCP amplitude measurement accuracy in EN-DC	2315

A.4.7.11.1.1	Test Purpose and Environment	2315

A.4.7.11.1.2	Test parameters	2316

A.4.7.11.1.3	Test Requirements	2317

A.4.8	Void	2317

A.4A	NE-DC test with all NR cells in FR1	2317

A.4A.1	Signaling characteristics	2317

A.4A.1.1	E-UTRAN PSCell addition	2317

A.4A.1.1.1	Test purpose and environment	2317

A.4A.1.1.2	Test Requirements	2321

A.4A.1.2	Active BWP switch	2321

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	2321

A.4A.1.2.1.1	Test Purpose and Environment	2321

A.4A.1.2.1.2	Test Requirements	2324

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	2325

A.4A.1.3.1	Test purpose and environment	2325

A.4A.1.3.2	Test Requirements	2328

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	2329

A.4A.1.4.1	Test Purpose and Environment	2329

A.4A.1.4.2	Test Parameters	2329

A.4A.1.4.3	Test Requirements	2332

A.4A.1.4.3.1	Test Requirements for NR HO	2332

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	2333

A.4A.2	Measurement performance	2333

A.4A.2.1	SFTD accuracy	2333

A.4A.2.1.1	SFTD accuracy	2333

A.4A.2.1.1.1	Test Purpose	2333

A.4A.2.1.1.2	Test Environment	2333

A.4A.2.1.1.3	Test Requirements	2336

A.5	EN-DC tests with one or more NR cells in FR2	2336

A.5.1	Void	2336

A.5.2	Void	2337

A.5.3	RRC_CONNECTED state mobility	2337

A.5.3.1	Void	2337

A.5.3.2	RRC Connection Mobility Control	2337

A.5.3.2.1	Void	2337

A.5.3.2.2	Random Access	2337

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	2337

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	2340

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	2343

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	2346

A.5.3.2.3	Void	2348

A.5.3.3	Handover with PSCell with known FR2 target PSCell	2348

A.5.3.3.1	Test purpose and environment	2348

A.5.3.3.2	Test Requirements	2351

A.5.3.3.3	Void	2351

A.5.3.3.4	Void	2351

A.5.3.3.5	Void	2351

A.5.3.3.6	Void	2352

A.5.4	Timing	2352

A.5.4.1	UE transmit timing	2352

A.5.4.1.1	NR UE Transmit Timing Test for FR2	2352

A.5.4.1.1.1	Test Purpose and environment	2352

A.5.4.1.1.2	Test requirements	2354

A.5.4.1.2	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	2354

A.5.4.1.2.1	Test Purpose and environment	2354

A.5.4.1.2.2	Test requirements	2359

A.5.4.2	UE timer accuracy	2359

A.5.4.3	Timing advance	2359

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	2359

A.5.4.3.1.1 Test Purpose and Environment	2359

A.5.4.3.1.2 Test Parameters	2359

A.5.4.3.1.3	Test Requirements	2362

A.5.5	Signaling characteristics	2362

A.5.5.1	Radio link Monitoring	2362

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	2362

A.5.5.1.1.1	Test Purpose and Environment	2362

A.5.5.1.1.2	Test Requirements	2365

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	2365

A.5.5.1.2.1	Test Purpose and Environment	2365

A.5.5.1.2.2	Test Requirements	2368

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	2369

A.5.5.1.3.1	Test Purpose and Environment	2369

A.5.5.1.3.2	Test Requirements	2371

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	2371

A.5.5.1.4.1	Test Purpose and Environment	2371

A.5.5.1.4.2	Test Requirements	2374

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	2374

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	2377

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	2380

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	2383

A.5.5.1.8.2	Test Requirements	2387

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	2387

A.5.5.1.9.1	Test Purpose and Environment	2387

A.5.5.1.9.2	Test Requirements	2388

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	2389

A.5.5.1.10.1	Test Purpose and Environment	2389

A.5.5.1.10.2	Test Requirements	2391

A.5.5.1.11	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	2391

A.5.5.1.11.1	Test Purpose and Environment	2391

A.5.5.1.11.2	Test Requirements	2394

A.5.5.1.12	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	2395

A.5.5.1.12.1	Test Purpose and Environment	2395

A.5.5.1.13	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	2395

A.5.5.1.13.1	Test Purpose and Environment	2395

A.5.5.1.13.2	Test Requirements	2395

A.5.5.1.14	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	2395

A.5.5.1.14.1	Test Purpose and Environment	2395

A.5.5.1.14.2	Test Requirements	2398

A.5.5.2	Interruption	2399

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	2399

A.5.5.2.1.1	Test Purpose and Environment	2399

A.5.5.2.1.2	Test Requirements	2401

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	2401

A.5.5.2.2.1	Test Purpose and Environment	2401

A.5.5.2.2.2	Test Requirements	2403

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	2403

A.5.5.2.3.1	Test Purpose and Environment	2403

A.5.5.2.3.2	Test Requirements	2405

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	2406

A.5.5.2.4.1	Test Purpose and Environment	2406

A.5.5.2.4.2	Test Requirements	2408

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	2409

A.5.5.2.5.1	Test Purpose and Environment	2409

A.5.5.2.5.2	Test Requirements	2411

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	2411

A.5.5.2.6.1	Test Purpose and Environment	2411

A.5.5.2.6.2	Test Requirements	2413

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	2414

A.5.5.2.7.1	Test Purpose and Environment	2414

A.5.5.2.7.2	Test Requirements	2416

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	2416

A.5.5.2.8.1 Test Purpose and Environment	2416

A.5.5.2.8.3	Test Requirements	2418

A.5.5.2.9	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell	2418

A.5.5.2.9.1	Test Purpose and Environment	2418

A.5.5.2.9.2	Test Requirements	2421

A.5.5.3	SCell Activation and Deactivation Delay	2421

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	2421

A.5.5.3.1.1	Test Purpose and Environment	2421

A.5.5.3.1.2	Test Requirements	2422

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160 ms SCell measurement cycle	2423

A.5.5.3.2.1	Test Purpose and Environment	2423

A.5.5.3.2.2	Test Requirements	2425

A.5.5.3.3	Void	2425

A.5.5.3.4	Void	2425

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	2425

A.5.5.3.5.1	Test Purpose and Environment	2425

A.5.5.3.5.2	Test Requirements	2428

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	2428

A.5.5.3.6.1	Test Purpose and Environment	2428

A.5.5.3.6.2	Test Requirements	2431

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	2431

A.5.5.3.7.1	Test Purpose and Environment	2431

A.5.5.3.7.2	Test Requirements	2434

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	2434

A.5.5.3.8.1	Test Purpose and Environment	2434

A.5.5.3.8.2	Test Requirements	2437

A.5.5.3.9	PUCCH SCell Activation and deactivation of known SCell in FR2	2437

A.5.5.3.9.1	Test Purpose and Environment	2437

A.5.5.3.9.2	Test Requirements	2440

A.5.5.3.10	PUCCH SCell Activation and deactivation of unknown SCell in FR2	2440

A.5.5.3.10.1	Test Purpose and Environment	2440

A.5.5.3.10.2	Test Requirements	2443

A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	2443

A.5.5.3.11.1	Test Purpose and Environment	2443

A.5.5.3.11.2	Test Requirements	2446

A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	2447

A.5.5.3.12.1	Test Purpose and Environment	2447

A.5.5.3.12.2	Test Requirements	2449

A.5.5.3.13	SCell Activation and deactivation of unknown SCell in FR2 for UE in DRX, capable of small beam sweeping factors and/or short measurement interval	2450

A.5.5.3.13.1	Test Purpose and Environment	2450

A.5.5.3.13.2	Test Requirements	2453

A.5.5.3.14	PUCCH SCell activation and deactivation with FR1 PSCell based on L3 reporting after SCell activation command	2455

A.5.5.3.14.1	Test Purpose and Environment	2455

A.5.5.3.14.2	Test Requirements	2459

A.5.5.3.15	SCell Activation of unknown SCell in FR2 in non-DRX for 160 ms SCell measurement cycle with the L3 reporting during activation	2459

A.5.5.3.15.1	Test Purpose and Environment	2459

A.5.5.3.15.2	Test Requirements	2463

A.5.5.4	Void	2464

A.5.5.5	Beam Failure Detection and Link recovery procedures	2464

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	2464

A.5.5.5.1.1	Test Purpose and Environment	2464

A.5.5.5.1.2	Test Requirements	2467

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	2467

A.5.5.5.2.1	Test Purpose and Environment	2467

A.5.5.5.2.2	Test Requirements	2471

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	2471

A.5.5.5.3.1	Test Purpose and Environment	2471

A.5.5.5.3.2	Test Requirements	2474

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2475

A.5.5.5.4.1	Test Purpose and Environment	2475

A.5.5.5.4.2	Test Requirements	2478

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	2478

A.5.5.5.5.1	Test Purpose and Environment	2478

A.5.5.5.5.2	Test Requirements	2481

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	2481

A.5.5.5.6.1	Test Purpose and Environment	2481

A.5.5.5.6.2	Test Requirements	2485

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	2485

A.5.5.5.7.1	Test Purpose and Environment	2485

A.5.5.5.7.2	Test Requirements	2488

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2489

A.5.5.5.8.1	Test Purpose and Environment	2489

A.5.5.5.8.2	Test Requirements	2492

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	2492

A.5.5.5.9.1	Test Purpose and Environment	2492

A.5.5.5.9.2	Test Requirements	2495

A.5.5.6	Active BWP switch	2496

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	2496

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2496

A.5.5.6.1.1.1	Test Purpose and Environment	2496

A.5.5.6.1.1.2	Test Requirements	2498

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	2499

A.5.5.6.2	RRC-based Active BWP Switch	2502

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2502

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2505

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	2505

A.5.5.6.4	SCell dormancy switch	2508

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	2508

A.5.5.6.4.1.1	Test Purpose and Environment	2508

A.5.5.6.4.1.2	Test Requirements	2511

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	2512

A.5.5.6.4.2.1	Test Purpose and Environment	2512

A.5.5.6.4.2.2	Test Requirements	2516

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2516

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	2516

A.5.5.7	PSCell addition and release delay	2519

A.5.5.7.1	Addition and Release Delay of NR PSCell	2519

A.5.5.7.1.1	Test purpose and environment	2519

A.5.5.7.1.2	Test Requirements	2521

A.5.5.8	Active TCI state switch delay	2521

A.5.5.8.1	MAC-CE based active TCI state switch	2522

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2522

A.5.5.8.1.1.1	Test Purpose and Environment	2522

A.5.5.8.1.1.2	Test Requirements	2524

A.5.5.8.2	RRC based active TCI state switch	2525

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2525

A.5.5.8.2.1.1	Test Purpose and Environment	2525

A.5.5.8.2.1.2	Test Requirements	2528

A.5.5.9	Uplink spatial relation switch delay	2528

A.5.5.9.1	MAC-CE based uplink spatial relation switch	2528

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	2528

A.5.5.9.1.1.1	Test Purpose and Environment	2528

A.5.5.9.1.1.2	Test Requirements	2530

A.5.5.9.2	RRC based spatial relation switch	2530

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	2530

A.5.5.9.2.1.1	Test Purpose and Environment	2531

A.5.5.9.2.1.2	Test Requirements	2532

A.5.5.10	UE specific CBW change	2533

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	2533

A.5.5.10.1.1	Test Purpose and Environment	2533

A.5.5.10.1.2	Test Requirements	2535

A.5.5.11	Unified TCI state switch delay	2536

A.5.5.11.1	MAC-CE based active joint TCI state switch	2536

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	2536

A.5.5.11.1.1.1	Test Purpose and Environment	2536

A.5.5.11.1.1.2	Test parameters	2536

A.5.5.11.1.1.3	Test Requirements	2538

A.5.5.11.2	MAC-CE based active uplink TCI state switch	2538

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	2538

A.5.5.11.2.1.1	Test Purpose and Environment	2538

A.5.5.11.2.1.2	Test parameters	2539

A.5.5.11.2.1.3	Test Requirements	2540

A.5.5.11.3	MAC-CE based active downlink TCI state switch	2541

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	2541

A.5.5.11.3.1.1	Test Purpose and Environment	2541

A.5.5.11.3.1.2	Test Parameters	2541

A.5.5.11.3.1.3	Test Requirements	2544

A.5.5.12	PSCell activation and deactivation delay	2544

A.5.5.12.1	PSCell activation and deactivation delay	2544

A.5.5.12.1.1	Test purpose and environment	2544

A.5.5.12.1.2	Test Requirements	2546

A.5.5.13	Conditional PSCell addition and release delay	2547

A.5.5.13.1	Addition and Release Delay of NR PSCell	2547

A.5.5.13.1.1	Test purpose and environment	2547

A.5.5.13.1.2	Test Requirements	2549

A.5.6	Measurement procedure	2549

A.5.6.1	Intra-frequency Measurements	2549

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	2549

A.5.6.1.1.1	Test purpose and Environment	2549

A.5.6.1.1.2	Test Requirements	2552

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	2552

A.5.6.1.2.1	Test purpose and Environment	2552

A.5.6.1.2.2	Test Requirements	2554

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	2555

A.5.6.1.3.1	Test purpose and Environment	2555

A.5.6.1.3.2	Test Requirements	2557

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	2557

A.5.6.1.4.1	Test purpose and Environment	2557

A.5.6.1.4.2	Test Requirements	2560

A.5.6.1.5	EN-DC event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP	2560

A.5.6.1.5.1	Test purpose and Environment	2560

A.5.6.1.5.2	Test Requirements	2560

A.5.6.1.6	EN-DC event triggered reporting test without gap under non-DRX	2560

A.5.6.1.6.1	Test purpose and Environment	2560

A.5.6.1.6.2	Test Requirements	2563

A.5.6.2	Inter-frequency Measurements	2563

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2563

A.5.6.2.1.1	Test Purpose and Environment	2563

A.5.6.2.1.2	Test Requirements	2566

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2566

A.5.6.2.2.1	Test Purpose and Environment	2566

A.5.6.2.2.2	Test Requirements	2568

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2569

A.5.6.2.3.1	Test Purpose and Environment	2569

A.5.6.2.3.2	Test Requirements	2571

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2571

A.5.6.2.4.1	Test Purpose and Environment	2571

A.5.6.2.4.2	Test Requirements	2574

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2574

A.5.6.2.5.1	Test Purpose and Environment	2574

A.5.6.2.5.2	Test Requirements	2577

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2577

A.5.6.2.6.1	Test Purpose and Environment	2577

A.5.6.2.6.2	Test Requirements	2580

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2581

A.5.6.2.7.1	Test Purpose and Environment	2581

A.5.6.2.7.2	Test Requirements	2583

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2584

A.5.6.2.8.1	Test Purpose and Environment	2584

A.5.6.2.8.2	Test Requirements	2587

A.5.6.3	L1-RSRP measurement for beam reporting	2587

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2587

A.5.6.3.1.1	Test Purpose and Environment	2587

A.5.6.3.1.2	Test parameters	2588

A.5.6.3.1.3	Test Requirements	2589

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2589

A.5.6.3.2.1	Test Purpose and Environment	2589

A.5.6.3.2.2	Test parameters	2590

A.5.6.3.2.3	Test Requirements	2591

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2591

A.5.6.3.3.1	Test Purpose and Environment	2591

A.5.6.3.3.2	Test parameters	2591

A.5.6.3.3.3	Test Requirements	2593

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2593

A.5.6.3.4.1	Test Purpose and Environment	2593

A.5.6.3.4.2	Test parameters	2594

A.5.6.3.4.3	Test Requirements	2595

A.5.6.3.5	CSI-RS based L1-RSRP measurement when DRX is not used and when CD-SSB is outside active BWP	2595

A.5.6.3.5.1	Test Purpose and Environment	2595

A.5.6.3.6	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2596

A.5.6.3.6.1	Test Purpose and Environment	2596

A.5.6.3.6.2	Test Requirements	2596

A.5.6.3.7	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	2596

A.5.6.3.7.1	Test Purpose and Environment	2596

A.5.6.3.7.2	Test parameters	2596

A.5.6.3.7.3	Test Requirements	2598

A.5.6.4	CLI measurements	2598

A.5.6.4.1	SRS-RSRP measurement with DRX	2598

A.5.6.4.1.1	Test Purpose and Environment	2598

A.5.6.4.1.2	Test Parameters	2598

A.5.6.4.1.3	Test Requirements	2600

A.5.6.4.2	CLI-RSSI measurement with DRX	2601

A.5.6.4.2.1	Test Purpose and Environment	2601

A.5.6.4.2.2	Test Parameters	2601

A.5.6.4.2.3	Test Requirements	2602

A.5.6.5	Measurements with autonomous gaps	2602

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2602

A.5.6.5.1.1	Test Purpose and Environment	2602

A.5.6.5.1.2	Test Requirements	2605

A.5.6.6	L1-SINR measurement for beam reporting	2605

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2607

A.5.6.6.2.1	Test Purpose and Environment	2607

A.5.6.6.2.2	Test parameters	2607

A.5.6.6.2.3	Test Requirements	2609

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2609

A.5.6.6.3.1	Test Purpose and Environment	2609

A.5.6.6.3.2	Test parameters	2610

A.5.6.6.3.3	Test Requirements	2611

A.5.6.7	CSI-RS based Intra-frequency Measurements	2611

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2611

A.5.6.7.1.1	Test purpose and Environment	2611

A.5.6.7.1.2	Test Requirements	2613

A.5.6.8	CSI-RS based Inter-frequency Measurements	2613

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2613

A.5.6.8.1.1	Test Purpose and Environment	2613

A.5.6.8.1.2	Test Requirements	2615

A.5.7	Measurement Performance requirements	2616

A.5.7.1	SS-RSRP	2616

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2616

A.5.7.1.1.1	Test Purpose and Environment	2616

A.5.7.1.1.2	Test parameters	2616

A.5.7.1.1.3	Test Requirements	2618

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2618

A.5.7.1.2.1	Test Purpose and Environment	2618

A.5.7.1.2.2	Test parameters	2619

A.5.7.1.2.3	Test Requirements	2621

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2622

A.5.7.1.3.1	Test Purpose and Environment	2622

A.5.7.1.3.2	Test parameters	2622

A.5.7.1.3.3	Test Requirements	2624

A.5.7.2	SS-RSRQ	2624

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2624

A.5.7.2.1.1	Test Purpose and Environment	2624

A.5.7.2.1.2	Test Parameters	2625

A.5.7.2.1.3	Test Requirements	2626

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2626

A.5.7.2.2.1	Test Purpose and Environment	2626

A.5.7.2.2.2	Test Parameters	2626

A.5.7.2.2.3	Test Requirements	2628

A.5.7.3	SS-SINR	2628

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2628

A.5.7.3.1.1	Test Purpose and Environment	2628

A.5.7.3.1.2	Test Parameters	2628

A.5.7.3.1.3	Test Requirements	2630

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2630

A.5.7.3.2.1	Test Purpose and Environment	2630

A.5.7.3.2.2	Test Parameters	2630

A.5.7.3.2.3	Test Requirements	2631

A.5.7.4	L1-RSRP measurement for beam reporting	2631

A.5.7.4.1	SSB based L1-RSRP measurement	2632

A.5.7.4.1.1	Test Purpose and Environment	2632

A.5.7.4.1.2	Test parameters	2632

A.5.7.4.1.3	Test Requirements	2633

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2634

A.5.7.4.2.1	Test Purpose and Environment	2634

A.5.7.4.2.2	Test parameters	2634

A.5.7.4.2.3	Test Requirements	2635

A.5.7.5	CLI measurements	2636

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2636

A.5.7.5.1.1	Test Purpose and Environment	2636

A.5.7.5.1.2	Test parameters	2636

A.5.7.5.1.3	Test Requirements	2638

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2638

A.5.7.5.2.1	Test Purpose and Environment	2638

A.5.7.5.2.2	Test parameters	2638

A.5.7.5.2.3	Test Requirements	2640

A.5.7.6	L1-SINR measurement for beam reporting	2640

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2642

A.5.7.6.2.1	Test Purpose and Environment	2643

A.5.7.6.2.2	Test parameters	2643

A.5.7.6.2.3	Test Requirements	2644

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2645

A.5.7.6.3.1	Test Purpose and Environment	2645

A.5.7.6.3.2	Test parameters	2645

A.5.7.6.3.3	Test Requirements	2647

A.5.7.7	CSI-RSRP	2647

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2647

A.5.7.7.1.1	Test Purpose and Environment	2647

A.5.7.7.1.2	Test parameters	2647

A.5.7.7.1.3	Test Requirements	2649

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2650

A.5.7.7.2.1	Test Purpose and Environment	2650

A.5.7.7.2.2	Test parameters	2650

A.5.7.7.2.3	Test Requirements	2652

A.5.7.8	CSI-RSRQ	2653

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2653

A.5.7.8.1.1	Test Purpose and Environment	2653

A.5.7.8.1.2	Test Parameters	2653

A.5.7.8.1.3	Test Requirements	2655

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2655

A.5.7.8.2.1	Test Purpose and Environment	2655

A.5.7.8.2.2	Test Parameters	2655

A.5.7.8.2.3	Test Requirements	2657

A.5.7.9	CSI-SINR	2657

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2657

A.5.7.9.1.1	Test Purpose and Environment	2657

A.5.7.9.1.2	Test Parameters	2657

A.5.7.9.1.3	Test Requirements	2659

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2659

A.5.7.9.2.1	Test Purpose and Environment	2659

A.5.7.9.2.2	Test Parameters	2659

A.5.7.9.2.3	Test Requirements	2661

A.5.8	Void	2661

A.6	NR standalone tests with all NR cells in FR1	2663

A.6.1	SA: RRC_IDLE state mobility	2663

A.6.1.1	Cell re-selection to NR	2663

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2663

A.6.1.1.1.1	Test Purpose and Environment	2663

A.6.1.1.1.2	Test Parameters	2663

A.6.1.1.1.3	Test Requirements	2665

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2665

A.6.1.1.2.1	Test Purpose and Environment	2665

A.6.1.1.2.2	Test Parameters	2666

A.6.1.1.2.3	Test Requirements	2668

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2668

A.6.1.1.3.1	Test Purpose and Environment	2668

A.6.1.1.3.2	Test Parameters	2668

A.6.1.1.3.3	Test Requirements	2671

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2671

A.6.1.1.4.1	Test Purpose and Environment	2671

A.6.1.1.4.2	Test Parameters	2671

A.6.1.1.4.3	Test Requirements	2673

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2674

A.6.1.1.5.1	Test Purpose and Environment	2674

A.6.1.1.5.2	Test Parameters	2674

A.6.1.1.5.3	Test Requirements	2676

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2677

A.6.1.1.6.1	Test Purpose and Environment	2677

A.6.1.1.6.2	Test Parameters	2677

A.6.1.1.6.3	Test Requirements	2679

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2680

A.6.1.1.7.1	Test Purpose and Environment	2680

A.6.1.1.7.2	Test Parameters	2680

A.6.1.1.7.3	Test Requirements	2683

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2683

A.6.1.1.8.1	Test Purpose and Environment	2683

A.6.1.1.8.2	Test Parameters	2683

A.6.1.1.8.3	Test Requirements	2686

A.6.1.1.9	Cell reselection to FR1 intra-frequency NR case for UE operating on a cell with less than 5 MHz BW	2686

A.6.1.1.9.1	Test Purpose and Environment	2686

A.6.1.1.9.3	Test Requirements	2687

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2687

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2687

A.6.1.2.1.1	Test Purpose and Environment	2687

A.6.1.2.1.2	Test Parameters	2688

A.6.1.2.1.3	Test Requirements	2690

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2690

A.6.1.2.2.1	Test Purpose and Environment	2690

A.6.1.2.2.2	Test Parameters	2690

A.6.1.2.2.3	Test Requirements	2693

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2693

A.6.1.2.3.1	Test Purpose and Environment	2693

A.6.1.2.3.2	Test Parameters	2693

A.6.1.2.3.3	Test Requirements	2696

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2696

A.6.1.2.4.1	Test Purpose and Environment	2696

A.6.1.2.4.2	Test Parameters	2697

A.6.1.2.4.3	Test Requirements	2699

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2699

A.6.1.2.5.1	Test Purpose and Environment	2700

A.6.1.2.5.2	Test Parameters	2700

A.6.1.2.5.3	Test Requirements	2702

A.6.1.1.7	Void	2702

A.6.2	SA: RRC_INACTIVE state mobility	2703

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2703

A.6.2.1.1	Test purpose and Environment	2703

A.6.2.1.2	Test Parameters	2704

A.6.2.1.3	Test requirements	2705

A.6.2.2	Cell reselection for positioning	2706

A.6.2.2.1	Cell reselection to FR1 intra-frequency NR case with RRC_ INACTIVE eDRX and positioning SRS	2706

A.6.2.2.1.1	Test Purpose and Environment	2706

A.6.2.2.1.2	Test Parameters	2706

A.6.2.2.1.3	Test Requirements	2710

A.6.3	RRC_CONNECTED state mobility	2710

A.6.3.1	Handover	2710

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2710

A.6.3.1.1.1	Test Purpose and Environment	2710

A.6.3.1.1.2	Test Parameters	2710

A.6.3.1.1.3 Test Requirements	2712

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2712

A.6.3.1.2.1	Test Purpose and Environment	2712

A.6.3.1.2.2	Test Parameters	2712

A.6.3.1.2.3	Test Requirements	2714

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2714

A.6.3.1.3.1	Test Purpose and Environment	2714

A.6.3.1.3.2	Test Parameters	2714

A.6.3.1.3.3	Test Requirements	2716

A.6.3.1.4	SA NR - E-UTRAN handover	2716

A.6.3.1.4.1	Test Purpose and Environment	2716

A.6.3.1.4.2	Test Requirements	2719

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2720

A.6.3.1.5.1	Test Purpose and Environment	2720

A.6.3.1.5.2	Test Requirements	2723

A.6.3.1.6	 SA NR - UTRAN FDD handover	2723

A.6.3.1.6.1	Test Purpose and Environment	2723

A.6.3.1.6.2	Test Requirements	2725

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2725

A.6.3.1.7.1	Test Purpose and Environment	2725

A.6.3.1.7.2	Test Parameters	2725

A.6.3.1.7.3	Test Requirements	2728

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2728

A.6.3.1.8.1	Test Purpose and Environment	2728

A.6.3.1.8.2	Test Parameters	2729

A.6.3.1.8.3	Test Requirements	2731

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2731

A.6.3.1.9.1	Test Purpose and Environment	2731

A.6.3.1.9.2	Test Parameters	2732

A.6.3.1.9.3	Test Requirements	2734

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2734

A.6.3.1.10.1	Test Purpose and Environment	2734

A.6.3.1.10.2	Test Parameters	2734

A.6.3.1.10.3	Test Requirements	2736

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2736

A.6.3.1.11.1	Test Purpose and Environment	2736

A.6.3.1.11.2	Test Parameters	2736

A.6.3.1.11.3 Test Requirements	2740

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2741

A.6.3.1.12.1	Test Purpose and Environment	2741

A.6.3.1.12.2	Test Parameters	2741

A.6.3.1.12.3	Test Requirements	2745

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2745

A.6.3.1.13.1	Test Purpose and Environment	2745

A.6.3.1.13.2	Test Requirements	2750

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PSCell addition	2750

A.6.3.1.14.1	Test Purpose and Environment	2750

A.6.3.1.14.2	Test Requirements	2755

A.6.3.1.15	Intra-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB	2756

A.6.3.1.15.1	Test Purpose and Environment	2756

A.6.3.1.15.2	Test Parameters	2756

A.6.3.1.15.3	Test Requirements	2759

A.6.3.1.16	Inter-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB	2759

A.6.3.1.16.1	Test Purpose and Environment	2759

A.6.3.1.16.2	Test Parameters	2759

A.6.3.1.16.3 Test Requirements	2761

A.6.3.1.17	Handover with PSCell change delay from NR-DC (FR1-FR1) to NR-DC (FR1-FR1)	2761

A.6.3.1.17.1	Test Purpose and Environment	2761

A.6.3.1.17.2	Test Requirements	2765

A.6.3.1.18	Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth	2765

A.6.3.1.18.2	Test Parameters	2765

A.6.3.1.18.3	Test Requirements	2766

A.6.3.1.19	Handover with PSCell change delay where target PSCell is with 12PRB SSB bandwidth	2767

A.6.3.1.19.1	Test Purpose and Environment	2767

A.6.3.1.19.2	Test Parameters	2767

A.6.3.1.19.3	Test Requirements	2769

A.6.3.2	RRC Connection Mobility Control	2770

A.6.3.2.1	SA: RRC Re-establishment	2770

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2770

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2773

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2775

A.6.3.2.2	Random Access	2778

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2778

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2781

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2784

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2786

A.6.3.2.3	SA: RRC Connection Release with Redirection	2789

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2789

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2791

A.6.3.2.4	LTM PDCCH-order Random Access	2794

A.6.3.2.4.1	PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP	2794

A.6.3.2.4.2	PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM	2798

A.6.3.2.4.3	PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP	2802

A.6.3.3	Conditional handover	2805

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2805

A.6.3.3.1.1	Test Purpose and Environment	2805

A.6.3.3.1.2	Test Parameters	2805

A.6.3.3.1.3	Test Requirements	2807

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2807

A.6.3.3.2.1	Test Purpose and Environment	2807

A.6.3.3.2.2	Test Parameters	2807

A.6.3.3.2.3	Test Requirements	2809

A.6.3.3.3	NR conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2809

A.6.3.3.3.1	Test Purpose and Environment	2809

A.6.3.3.3.2	Test Requirements	2812

A.6.3.3.4	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2813

A.6.3.3.4.1	Test Purpose and Environment	2813

A.6.3.3.4.2	Test Parameters	2813

A.6.3.3.4.3	Test Requirements	2817

A.6.3.3.5	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC with complementary conditional handover configuration	2817

A.6.3.3.5.1	Test Purpose and Environment	2817

A.6.3.3.5.2	Test Parameters	2817

A.6.3.3.5.3	Test Requirements	2821

A.6.3.3.6	NES triggering intra-frequency conditional handover from FR1 to FR1	2821

A.6.3.3.6.1	Test Purpose and Environment	2821

A.6.3.3.6.2	Test Parameters	2821

A.6.3.3.6.3	Test Requirements	2823

A.6.3.3.7	NES-based Inter-frequency conditional handover from FR1 to FR1	2823

A.6.3.3.7.1	Test Purpose and Environment	2823

A.6.3.3.7.2	Test Parameters	2823

A.6.3.3.7.3	Test Requirements	2825

A.6.3.4	LTM PCell Switch	2825

A.6.3.4.1	RACH-based Intra-frequency PCell switch from FR1 to FR1	2825

A.6.3.4.1.1	Test Purpose and Environment	2825

A.6.3.4.1.2	Test Parameters	2825

A.6.3.4.1.3	Test Requirements	2828

A.6.3.4.2	RACH based Inter-frequency LTM PCell switch from FR1 to FR1	2829

A.6.3.4.2.1	Test Purpose and Environment	2829

A.6.3.4.2.2	Test Parameters	2829

A.6.3.4.2.3	Test Requirements	2832

A.6.3.4.3	RACH-less Intra-frequency PCell switch from FR1 to FR1	2833

A.6.3.4.3.1	Test Purpose and Environment	2833

A.6.3.4.3.2	Test Parameters	2833

A.6.3.4.3.3	Test Requirements	2837

A.6.3.4.4	RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement	2837

A.6.3.4.4.1	Test Purpose and Environment	2837

A.6.3.4.4.2	Test Parameters	2837

A.6.3.4.4.3	Test Requirements	2841

A.6.3.5	LTM PSCell Switch	2841

A.6.3.5.1 RACH-based intra-frequency LTM PSCell switch from FR1 to FR1	2841

A.6.3.5.1.1	Test Purpose and Environment	2841

A.6.3.5.1.2	Test Parameters	2841

A.6.3.5.1.3	Test Requirements	2846

A.6.4	Timing	2846

A.6.4.1	UE transmit timing	2846

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2846

A.6.4.1.1.1	Test Purpose and environment	2846

A.6.4.1.1.2	Test requirements	2848

A.6.4.1.2	NR UE Transmit Timing Test for two TRPs in FR1	2849

A.6.4.1.2.1	Test Purpose and environment	2849

A.6.4.1.2.2	Test requirements	2851

A.6.4.2	UE timer accuracy	2852

A.6.4.3	Timing advance	2852

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2852

A.6.4.3.1.1	Test Purpose and Environment	2852

A.6.4.3.1.2	Test Parameters	2852

A.6.4.3.1.3	Test Requirements	2855

A.6.5	Signalling characteristics	2855

A.6.5.1	Radio link Monitoring	2855

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2855

A.6.5.1.1.1	Test Purpose and Environment	2855

A.6.5.1.1.2	Test Requirements	2858

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2858

A.6.5.1.2.1	Test Purpose and Environment	2858

A.6.5.1.2.2	Test Requirements	2861

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2861

A.6.5.1.3.1	Test Purpose and Environment	2861

A.6.5.1.3.2	Test Requirements	2864

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2864

A.6.5.1.4.1	Test Purpose and Environment	2864

A.6.5.1.4.2	Test Requirements	2867

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2867

A.6.5.1.5.1	Test Purpose and Environment	2867

A.6.5.1.5.2	Test Requirements	2870

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2870

A.6.5.1.6.1	Test Purpose and Environment	2870

A.6.5.1.6.2	Test Requirements	2873

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2873

A.6.5.1.7.1	Test Purpose and Environment	2873

A.6.5.1.7.2	Test Requirements	2876

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2877

A.6.5.1.8.1	Test Purpose and Environment	2877

A.6.5.1.8.2	Test Requirements	2880

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2880

A.6.5.1.9.1	Test Purpose and Environment	2880

A.6.5.1.9.2	Test Requirements	2883

A.6.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	2883

A.6.5.1.10.1	Test Purpose and Environment	2883

A.6.5.1.11	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	2884

A.6.5.1.11.1	Test Purpose and Environment	2884

A.6.5.1.11.2	Test Requirements	2884

A.6.5.1.12	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	2884

A.6.5.1.12.1	Test Purpose and Environment	2884

A.6.5.1.12.2	Test Requirements	2887

A.6.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5 MHz BW	2887

A.6.5.1.13.1	Test Purpose and Environment	2887

A.6.5.1.13.2	Test Requirements	2888

A.6.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5 MHz BW	2888

A.6.5.1.14.1	Test Purpose and Environment	2888

A.6.5.1.14.2	Test Requirements	2889

A.6.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell with 3 MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode	2889

A.6.5.1.15.1	Test Purpose and Environment	2889

A.6.5.1.15.2	Test Requirements	2890

A.6.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode	2890

A.6.5.1.16.1	Test Purpose and Environment	2890

A.6.5.1.16.2	Test Requirements	2891

A.6.5.2	Interruption	2892

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2892

A.6.5.2.1.2	Test Requirements	2895

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2896

A.6.5.2.2.1	Test Purpose and Environment	2896

A.6.5.2.2.2	Test Parameters	2896

A.6.5.2.2.3	Test Requirements	2898

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2898

A.6.5.2.3.1	Test Purpose and Environment	2898

A.6.5.2.3.2	Test Parameters	2898

A.6.5.2.3.3	Test Requirements	2900

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2900

A.6.5.2.4.1	Test Purpose and Environment	2900

A.6.5.2.4.2	Test Parameters	2900

A.6.5.2.4.3	Test Requirements	2902

A.6.5.3	SCell Activation and Deactivation Delay	2902

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle	2902

A.6.5.3.1.1	Test Purpose and Environment	2902

A.6.5.3.1.2	Test Requirements	2907

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2907

A.6.5.3.2.1	Test Purpose and Environment	2907

A.6.5.3.2.2	Test Requirements	2908

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2908

A.6.5.3.3.1	Test Purpose and Environment	2908

A.6.5.3.3.2	Test Requirements	2909

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2909

A.6.5.3.4.1	Test Purpose and Environment	2909

A.6.5.3.4.2	Test Requirements	2912

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2913

A.6.5.3.5.1	Test Purpose and Environment	2913

A.6.5.3.5.2	Test Requirements	2917

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2918

A.6.5.3.6.1	Test Purpose and Environment	2918

A.6.5.3.6.2	Test Requirements	2921

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2921

A.6.5.3.7.1	Test Purpose and Environment	2921

A.6.5.3.7.2	Test Requirements	2924

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2925

A.6.5.3.8.1	Test Purpose and Environment	2925

A.6.5.3.8.2	Test Requirements	2928

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2929

A.6.5.3.9.1	Test Purpose and Environment	2929

A.6.5.3.9.2	Test Requirements	2932

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle	2932

A.6.5.3.10.1	Test Purpose and Environment	2932

A.6.5.3.10.2	Test Requirements	2935

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2936

A.6.5.3.11.1	Test Purpose and Environment	2936

A.6.5.3.11.2	Test Requirements	2936

A.6.5.3.12	SCell Activation and deactivation of unknown SCell in FR1 in DRX for UE capable of short measurement interval	2936

A.6.5.3.12.1	Test Purpose and Environment	2936

A.6.5.3.12.2	Test Requirements	2939

A.6.5.3.13	SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation commandin non-DRX	2939

A.6.5.3.13.1	Test Purpose and Environment	2939

A.6.5.3.13.2	Test Requirements	2944

A.6.5.3.14	SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle	2944

A.6.5.3.14.1	Test Purpose and Environment	2944

A.6.5.3.14.2	Test Requirements	2949

A.6.5.3.15	TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX	2950

A.6.5.3.15.1	Test Purpose and Environment	2950

A.6.5.3.15.2	Test Requirements	2954

A.6.5.3.16	Inter-band SSB-less SCell Activation based on A-TRS	2954

A.6.5.3.16.1	Test Purpose and Environment	2954

A.6.5.3.16.2	Test Requirements	2958

A.6.5.3.17.1	Test Purpose and Environment	2959

A.6.5.3.17.2	Test Requirements	2960

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2960

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2960

A.6.5.4.1.1	Test Purpose and Environment	2960

A.6.5.4.1.2	Test Requirements	2963

A.6.5.4.2	Void	2963

A.6.5.5	Beam Failure Detection and Link recovery procedures	2963

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2963

A.6.5.5.1.1	Test Purpose and Environment	2963

A.6.5.5.1.2	Test Requirements	2967

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2967

A.6.5.5.2.1	Test Purpose and Environment	2967

A.6.5.5.2.2	Test Requirements	2971

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2971

A.6.5.5.3.1	Test Purpose and Environment	2971

A.6.5.5.3.2	Test Requirements	2974

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2975

A.6.5.5.4.1	Test Purpose and Environment	2975

A.6.5.5.4.2	Test Requirements	2979

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2979

A.6.5.5.5.1	Test Purpose and Environment	2979

A.6.5.5.5.2	Test Requirements	2982

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2983

A.6.5.5.6.1	Test Purpose and Environment	2983

A.6.5.5.6.2	Test Requirements	2986

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2987

A.6.5.5.7.1	Test Purpose and Environment	2987

A.6.5.5.7.2	Test Requirements	2990

A.6.5.5.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW	2991

A.6.5.5.8.1	Test Purpose and Environment	2991

A.6.5.5.8.2	Test Requirements	2992

A.6.5.6	Active BWP switch	2992

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2992

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2992

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2997

A.6.5.6.2	RRC-based Active BWP Switch	3000

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	3000

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3002

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	3002

A.6.5.6.4	SCell dormancy switch	3008

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	3008

A.6.5.6.4.1.1	Test Purpose and Environment	3008

A.6.5.6.4.1.2	Test Requirements	3012

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	3013

A.6.5.6.4.2.1	 Test Purpose and Environment	3013

A.6.5.6.4.2.2	Test Requirements	3018

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3018

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	3018

A.6.5.7	DL interruptions at switching between two uplink carriers	3022

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	3022

A.6.5.7.1.1	Test Purpose and Environment	3022

A.6.5.7.1.2	Test Requirements	3026

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	3026

A.6.5.7.2.1	Test Purpose and Environment	3026

A.6.5.7.2.2	Test Requirements	3029

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	3029

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	3029

A.6.5.7A.1.1	Test Purpose and Environment	3029

A.6.5.7A.1.2	Test Requirements	3033

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	3033

A.6.5.7A.2.1	Test Purpose and Environment	3033

A.6.5.7A.2.2	Test Requirements	3035

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	3036

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	3036

A.6.5.7B.1.1	Test Purpose and Environment	3036

A.6.5.7B.1.2	Test Requirements	3040

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	3040

A.6.5.7B.2.1	Test Purpose and Environment	3040

A.6.5.7B.2.2	Test Requirements	3045

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	3045

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	3045

A.6.5.7C.1.1	Test Purpose and Environment	3045

A.6.5.7C.1.2	Test Requirements	3051

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	3051

A.6.5.7C.2.1	Test Purpose and Environment	3051

A.6.5.7C.2.2	Test Requirements	3056

A.6.5.7D	DL interruptions at UE switching across three or four uplink bands	3056

A.6.5.7D.1	DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG	3056

A.6.5.7D.1.1	Test Purpose and Environment	3056

A.6.5.7D.1.2	Test Requirements	3060

A.6.5.7D.2	DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG	3060

A.6.5.7D.2.1	Test Purpose and Environment	3060

A.6.5.7D.2.2	Test Requirements	3064

A.6.5.7D.3	DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs	3064

A.6.5.7D.3.1	Test Purpose and Environment	3064

A.6.5.7D.3.2	Test Requirements	3068

A.6.5.7D.4	DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs	3068

A.6.5.7D.4.1	Test Purpose and Environment	3068

A.6.5.7D.7.2	Test Requirements	3073

A.6.5.8	UE specific CBW change	3073

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	3073

A.6.5.8.1.1	Test Purpose and Environment	3073

A.6.5.8.1.2	Test Requirements	3075

A.6.5.9	Pathloss reference signal switching delay	3075

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	3075

A.6.5.9.1.1	Test Purpose and Environment	3075

A.6.5.9.1.2	Test Requirements	3078

A.6.5.10	Conditional PSCell addition and release delay (FR1 NR-DC)	3078

A.6.5.10.1	Conditional PSCell Addition and Release Delay	3078

A.6.5.10.1.1	Test purpose and environment	3078

A.6.5.10.1.2	Test Parameters	3078

A.6.5.10.1.3	Test Requirements	3081

A.6.5.11	PSCell addition and release delay	3081

A.6.5.11.1	Addition and Release Delay of unknown NR FR1 PSCell	3081

A.6.5.11.1.1	Test purpose and environment	3081

A.6.5.11.1.2	Test Requirements	3083

A.6.5.11.2	Addition and Release Delay of unknown NR FR1 PSCell with less than 5 MHz	3084

A.6.5.11.2.1	Test purpose and environment	3084

A.6.5.11.2.2	Test Requirements	3084

A.6.5.12	Subsequent conditional PSCell addition/change	3085

A.6.5.12.1	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC	3085

A.6.5.12.1.1	Test purpose and environment	3085

A.6.5.12.1.2	Test Parameters	3085

A.6.5.12.1.3	Test Requirements	3087

A.6.5.12.2	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC	3088

A.6.5.12.2.1	Test purpose and environment	3088

A.6.5.12.2.2	Test Parameters	3088

A.6.5.12.2.3	Test Requirements	3091

A.6.5.12.3	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth	3091

A.6.5.12.3.1	Test purpose and environment	3091

A.6.5.12.3.2	Test Parameters	3091

A.6.5.12.3.3	Test Requirements	3092

A.6.5.12.4	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth	3092

A.6.5.12.4.1	Test purpose and environment	3092

A.6.5.12.4.2	Test Parameters	3092

A.6.5.12.4.3	Test Requirements	3093

A.6.5.13	Active TCI state switch delay	3093

A.6.5.13.1	MAC-CE based joint TCI state switch for mDCI with two TA when RTD is larger than CP	3094

A.6.5.13.1.1	Test Purpose and Environment	3094

A.6.5.13.1.2	Test Requirements	3096

A.6.6	Measurement procedure	3097

A.6.6.1	Intra-frequency Measurements	3097

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	3097

A.6.6.1.1.1	Test purpose and Environment	3097

A.6.6.1.1.2	Test parameters	3097

A.6.6.1.1.3	Test Requirements	3099

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	3099

A.6.6.1.2.1	Test purpose and Environment	3099

A.6.6.1.2.2	Test parameters	3099

A.6.6.1.2.3	Test Requirements	3101

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	3101

A.6.6.1.3.1	Test purpose and Environment	3101

A.6.6.1.3.2	Test parameters	3101

A.6.6.1.3.3	Test Requirements	3103

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	3103

A.6.6.1.4.1	Test purpose and Environment	3103

A.6.6.1.4.2	Test parameters	3104

A.6.6.1.4.3	Test Requirements	3106

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	3106

A.6.6.1.5.1	Test purpose and Environment	3106

A.6.6.1.5.2	Test parameters	3106

A.6.6.1.5.3	Test Requirements	3107

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	3107

A.6.6.1.6.1	Test purpose and Environment	3107

A.6.6.1.6.2	Test parameters	3108

A.6.6.1.6.3	Test Requirements	3109

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	3109

A.6.6.1.7.1	Test purpose and Environment	3109

A.6.6.1.7.2	Test parameters	3109

A.6.6.1.7.3	Test Requirements	3111

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	3112

A.6.6.1.8.1	Test purpose and Environment	3112

A.6.6.1.8.2	Test parameters	3112

A.6.6.1.8.3	Test Requirements	3114

A.6.6.1.9	SA event triggered reporting tests with MUSIM gap configured	3114

A.6.6.1.9.1	Test purpose and Environment	3114

A.6.6.1.9.2	Test parameters	3114

A.6.6.1.9.3	Test requirements	3116

A.6.6.1.10	SA event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP	3116

A.6.6.1.10.1	Test purpose and Environment	3116

A.6.6.1.10.2	Test Requirements	3117

A.6.6.1.11	SA event triggered reporting tests without gap under non-DRX with NCD-SSB	3117

A.6.6.1.11.1	Test purpose and Environment	3117

A.6.6.1.11.2	Test parameters	3117

A.6.6.1.11.3	Test Requirements	3118

A.6.6.1.12	SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB	3119

A.6.6.1.12.1	Test purpose and Environment	3119

A.6.6.1.12.2	Test parameters	3119

A.6.6.1.12.3	Test Requirements	3119

A.6.6.1.13	SA event triggered reporting tests without gap under Cell DTX	3119

A.6.6.1.13.1	Test purpose and Environment	3120

A.6.6.1.13.2	Test parameters	3120

A.6.6.1.13.3	Test Requirements	3121

A.6.6.1.14	Deactivated PSCell measurement test with 12 PRB SSB bandwidth in FR1	3122

A.6.6.1.14.1	Test Purpose and Environment	3122

A.6.6.1.14.2	Test Parameters	3122

A.6.6.1.14.3	Test Requirements	3125

A.6.6.1.15	SA event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell	3125

A.6.6.1.15.1	Test purpose and Environment	3125

A.6.6.1.15.2	Test parameters	3125

A.6.6.1.15.3	Test requirements	3127

A.6.6.2	Inter-frequency Measurements	3128

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3128

A.6.6.2.1.1	Test Purpose and Environment	3128

A.6.6.2.1.2	Test Requirements	3130

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3130

A.6.6.2.2.1	Test Purpose and Environment	3130

A.6.6.2.2.2	Test Requirements	3133

A.6.6.2.3	Void	3133

A.6.6.2.4	Void	3133

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3133

A.6.6.2.5.1	Test Purpose and Environment	3133

A.6.6.2.5.2	Test Requirements	3135

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3136

A.6.6.2.6.1	Test Purpose and Environment	3136

A.6.6.2.6.2	Test Requirements	3138

A.6.6.2.7	Void	3138

A.6.6.2.8	Void	3138

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	3138

A.6.6.2.9.1	Test Purpose and Environment	3138

A.6.6.2.9.2	Test Requirements	3141

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	3141

A.6.6.2.10.1	Test Purpose and Environment	3141

A.6.6.2.10.2	Test Requirements	3143

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	3145

A.6.6.2.12.1	Test Purpose and Environment	3146

A.6.6.2.12.2	Test Requirements	3148

A.6.6.2.13	SA event triggered reporting tests for FR1 with measurement gap with priority and periodic MUSIM gap configured	3148

A.6.6.2.13.1	Test Purpose and Environment	3148

A.6.6.2.13.2	Test Requirements	3151

A.6.6.2.14	SA event triggered reporting tests for FR1 with measurement gap without priority and periodic MUSIM gap configured	3151

A.6.6.2.14.1	Test Purpose and Environment	3151

A.6.6.2.14.2	Test Requirements	3154

A.6.6.2.15	SA event triggered reporting tests for FR1 with 3 MHz Channel Bandwidth configured without SSB time index detection when DRX is used	3154

A.6.6.2.15.1	Test Purpose and Environment	3154

A.6.6.2.15.2	Test Requirements	3154

A.6.6.3	Inter-RAT Measurements	3155

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	3155

A.6.6.3.1.1	Test Purpose and Environment	3155

A.6.6.3.1.2	Test Requirements	3158

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	3158

A.6.6.3.2.1	Test Purpose and Environment	3158

A.6.6.3.2.2	Test Requirements	3161

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	3161

A.6.6.3.3.1	Test Purpose and Environment	3161

A.6.6.3.3.2	Test Requirements	3164

A.6.6.4	L1-RSRP measurement for beam reporting	3165

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	3165

A.6.6.4.1.1	Test Purpose and Environment	3165

A.6.6.4.1.2	Test parameters	3165

A.6.6.4.1.3	Test Requirements	3166

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	3166

A.6.6.4.2.1	Test Purpose and Environment	3166

A.6.6.4.2.2	Test parameters	3167

A.6.6.4.2.3	Test Requirements	3168

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	3168

A.6.6.4.3.1	Test Purpose and Environment	3168

A.6.6.4.3.2	Test parameters	3169

A.6.6.4.3.3	Test Requirements	3170

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	3170

A.6.6.4.4.1	Test Purpose and Environment	3170

A.6.6.4.4.2	Test parameters	3170

A.6.6.4.4.3	Test Requirements	3172

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	3172

A.6.6.4.5.1	Test Purpose and Environment	3172

A.6.6.4.5.2	Test parameters	3172

A.6.6.4.5.3	Test Requirements	3174

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	3174

A.6.6.4.6.1	Test Purpose and Environment	3174

A.6.6.4.6.2	Test parameters	3174

A.6.6.4.6.3	Test Requirements	3176

A.6.6.4.7	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	3176

A.6.6.4.7.1	Test Purpose and Environment	3176

A.6.6.4.7.2	Test Requirements	3177

A.6.6.4.8	CSI-RS based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	3177

A.6.6.4.8.1	Test Purpose and Environment	3177

A.6.6.4.9	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	3177

A.6.6.4.9.1	Test Purpose and Environment	3177

A.6.6.4.9.2	Test parameters	3177

A.6.6.4.9.3	Test Requirements	3179

A.6.6.5	Inter-RAT UTRAN FDD measurements	3179

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	3179

A.6.6.5.1.1	Test Purpose and Environment	3179

A.6.6.5.1.2	Test Requirements	3181

A.6.6.6	CLI measurements	3182

A.6.6.6.1	SRS-RSRP measurement with DRX	3182

A.6.6.6.1.1	Test Purpose and Environment	3182

A.6.6.6.1.2	Test Parameters	3182

A.6.6.6.1.3	Test Requirements	3184

A.6.6.6.2	CLI-RSSI measurement with DRX	3184

A.6.6.6.2.1	Test Purpose and Environment	3184

A.6.6.6.2.2	Test Parameters	3185

A.6.6.6.2.3	Test Requirements	3186

A.6.6.7	NR measurements with autonomous gaps	3186

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	3186

A.6.6.7.1.1	Test Purpose and Environment	3186

A.6.6.7.1.2	Test Parameters	3186

A.6.6.7.1.3	Test Requirements	3189

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	3189

A.6.6.7.2.1	Test Purpose and Environment	3189

A.6.6.7.2.2	Test Requirements	3191

A.6.6.8	L1-SINR measurement for beam reporting	3192

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	3192

A.6.6.8.1.1	Test Purpose and Environment	3192

A.6.6.8.1.2	Test parameters	3192

A.6.6.8.1.3	Test Requirements	3194

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	3194

A.6.6.8.2.1	Test Purpose and Environment	3194

A.6.6.8.2.2	Test parameters	3194

A.6.6.8.2.3	Test Requirements	3196

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	3196

A.6.6.8.3.1	Test Purpose and Environment	3196

A.6.6.8.3.2	Test parameters	3197

A.6.6.8.3.3	Test Requirements	3198

A.6.6.9	Idle Mode CA/DC Measurements	3198

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	3198

A.6.6.9.1.1	Test Purpose and Environment	3198

A.6.6.9.1.2	Test Requirements	3202

A.6.6.9.2	 Idle mode fast CA/DC eEMR measurement for FR1 without valid reporting	3202

A.6.6.9.2.1	Test Purpose and Environment	3202

A.6.6.9.2.2	Test Requirements	3205

A.6.6.9.3	Idle mode fast CA/DC cell reselection measurement for FR1 without valid reporting	3205

A.6.6.9.3.1	Test Purpose and Environment	3205

A.6.6.9.3.2	Test Requirements	3208

A.6.6.9.4	Idle mode fast CA/DC cell reselection measurement for FR1 with valid reporting	3208

A.6.6.9.4.1	Test Purpose and Environment	3208

A.6.6.9.4.2	Test Requirements	3211

A.6.6.9.5	SA Idle mode CA/DC measurement for FR1 with 12RB SSB	3211

A.6.6.9.5.1	Test Purpose and Environment	3211

A.6.6.9.5.2	Test Requirements	3212

A.6.6.10	CSI-RS based intra-frequency Measurements	3212

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	3212

A.6.6.10.1.1	Test purpose and Environment	3212

A.6.6.10.1.2	Test Requirements	3214

A.6.6.11	CSI-RS based inter-frequency Measurements	3214

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	3214

A.6.6.11.1.1	Test Purpose and Environment	3214

A.6.6.11.1.2	Test Requirements	3216

A.6.6.12	RSTD measurements	3217

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	3217

A.6.6.12.1.1	Test Purpose and Environment	3217

A.6.6.12.1.2	Test Requirements	3220

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	3220

A.6.6.12.2.1	Test Purpose and Environment	3221

A.6.6.12.2.2	Test Requirements	3224

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	3224

A.6.6.12.3.1	Test Purpose and Environment	3224

A.6.6.12.3.2	Test Requirements	3227

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	3227

A.6.6.12.4.1	Test Purpose and Environment	3227

A.6.6.12.4.2	Test Requirements	3231

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	3231

A.6.6.12.5.1	Test Purpose and Environment	3231

A.6.6.12.5.2	Test Requirements	3234

A.6.6.12.6	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_CONNECTED mode	3234

A.6.6.12.6.1	Test Purpose and Environment	3234

A.6.6.12.6.2	Test Requirements	3241

A.6.6.13 PRS-RSRP measurements	3241

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	3241

A.6.6.13.1.1	Test purpose and Environment	3241

A.6.6.13.1.2	Test Requirements	3243

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3243

A.6.6.13.2.1	Test purpose and Environment	3243

A.6.6.13.2.2	Test Requirements	3246

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	3246

A.6.6.13.3.1	Test purpose and Environment	3246

A.6.6.13.3.2	Test Requirements	3248

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3248

A.6.6.13.4.1	Test purpose and Environment	3248

A.6.6.14	UE Rx-Tx time difference measurements	3251

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3251

A.6.6.14.1.1	Test purpose and environment	3251

A.6.6.14.1.2	Test requirements	3253

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	3253

A.6.6.14.2.1	Test purpose and environment	3253

A.6.6.14.2.2	Test requirements	3255

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	3256

A.6.6.14.3.1	Test purpose and environment	3256

A.6.6.14.3.2	Test requirements	3258

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	3258

A.6.6.14.4.1	Test purpose and environment	3258

A.6.6.14.4.2	Test requirements	3260

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	3260

A.6.6.14.4.1	Test purpose and environment	3260

A.6.6.14.4.2	Test requirements	3262

A.6.6.14.6	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR1 SA	3263

A.6.6.14.6.1	Test purpose and environment	3263

A.6.6.14.6.2	Test requirements	3266

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	3266

A.6.6.15.1	Test Purpose and Environment	3266

A.6.6.15.2	Test Requirements	3270

A.6.6.16	PRS-RSRPP measurements	3271

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	3271

A.6.6.16.1.1	Test purpose and Environment	3271

A.6.6.16.1.2	Test Requirements	3273

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3273

A.6.6.16.2.1	Test purpose and Environment	3273

A.6.6.16.2.2	Test Requirements	3275

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	3275

A.6.6.16.3.1	Test purpose and Environment	3275

A.6.6.16.3.2	Test Requirements	3277

A.6.6.17	SA event triggered reporting tests with Pre-MG	3278

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	3278

A.6.6.17.1.1	Test purpose and Environment	3278

A.6.6.17.1.2	Test parameters	3278

A.6.6.17.1.3	Test Requirements	3280

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	3281

A.6.6.17.2.1	Test purpose and Environment	3281

A.6.6.17.2.2	Test parameters	3281

A.6.6.17.2.3	Test Requirements	3283

A.6.6.17.3	Void	3284

A.6.6.17.3.1	Void	3284

A.6.6.17.3.2	Void	3284

A.6.6.17.3.3	Void	3284

A.6.6.18	SA event triggered reporting tests with concurrent gaps	3284

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	3284

A.6.6.18.1.1	Test Purpose and Environment	3284

A.6.6.18.1.2	Test Requirements	3286

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	3286

A.6.6.18.2.1	Test Purpose and Environment	3286

A.6.6.18.2.2	Test Requirements	3289

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	3289

A.6.6.18.3.1	Test Purpose and Environment	3289

A.6.6.18.3.2	Test Requirements	3293

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	3294

A.6.6.18.4.1	Test Purpose and Environment	3294

A.6.6.18.4.2	Test Requirements	3296

A.6.6.19	SA event triggered reporting tests with NCSG	3297

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	3297

A.6.6.19.1.1	Test purpose and Environment	3297

A.6.6.19.1.2	Test parameters	3297

A.6.6.19.1.3	Test Requirements	3299

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	3299

A.6.6.19.2.1	Test Purpose and Environment	3299

A.6.6.19.2.2	Test parameters	3299

A.6.6.19.2.3	Test Requirements	3302

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	3302

A.6.6.19.3.1	Test Purpose and Environment	3302

A.6.6.19.3.2	Test parameters	3302

A.6.6.19.3.3	Test Requirements	3305

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	3306

A.6.6.19.4.1	Test purpose and Environment	3306

A.6.6.19.4.2	Test parameters	3306

A.6.6.19.4.3	Test Requirements	3308

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	3308

A.6.6.20.1	Test purpose and environment	3308

A.6.6.20.2	Test requirements	3310

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	3310

A.6.6.21.1	Test purpose and environment	3310

A.6.6.21.2	Test requirements	3312

A.6.6.22	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG	3312

A.6.6.22.1	SA event triggered reporting tests for FR1 concurrent gap with Pre-MG with partially partial overalpping scenario for SSB-based measurements in both intra-frequency and inter-frequency layers	3312

A.6.6.22.1.1	Test Purpose and Environment	3312

A.6.6.22.1.2	Test Requirements	3315

A.6.6.22.2	SA event triggered reporting tests for concurrent gap with pre-configured gaps and network-controlled activation/deactivation	3316

A.6.6.22.2.1	Test purpose and Environment	3316

A.6.6.22.2.2	Test parameters	3316

A.6.6.22.2.3	Test Requirements	3318

A.6.6.23	SA event triggered reporting tests for concurrent measurement gaps with NCSG	3319

A.6.6.23.1	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers [one MG + one NCSG]	3319

A.6.6.23.1.1	Test Purpose and Environment	3319

A.6.6.23.1.2	Test Requirements	3321

A.6.6.23.2	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers [two NCSG]	3322

A.6.6.23.2.1	Test Purpose and Environment	3322

A.6.6.23.2.2	Test Requirements	3324

A.6.6.23.3	Event triggered reporting on SCC with deactivated SCell test with per-UE Con-NCSG under non-DRX	3324

A.6.6.23.3.1	Test purpose and Environment	3324

A.6.6.23.3.2	Test parameters	3325

A.6.6.23.3.3	Test Requirements	3327

A.6.6.24	SA event triggered reporting tests with NeedForGap in FR1	3327

A.6.6.24.1	SA event triggered reporting tests without gaps, with interruptions, under non-DRX	3327

A.6.6.24.1.1	Test purpose and Environment	3327

A.6.6.24.1.2	Test parameters	3327

A.6.6.24.1.3	Test Requirements	3329

A.6.6.24.2	SA event triggered reporting tests for FR1 without gap with interruption for inter-frequency measurement with SSB time index detection when DRX is not used	3329

A.6.6.24.2.1	Test Purpose and Environment	3329

A.6.6.24.2.2	Test parameters	3329

A.6.6.24.2.3	Test Requirements	3332

A.6.6.24.3	SA event triggered reporting tests for FR1 with ‘no-gap-with-interruption’, without measurement gap or DRX	3332

A.6.6.24.3.1	Test Purpose and Environment	3332

A.6.6.24.3.2	Test Requirements	3334

A.6.6.24.4	SA event triggered reporting tests for FR1 NeedForGaps without gap without interruption when DRX is not used	3334

A.6.6.24.4.1	Test Purpose and Environment	3334

A.6.6.24.4.2	Test parameters	3335

A.6.6.24.4.3	Test Requirements	3337

A.6.6.24.5	SA event triggered reporting tests without gap under non-DRX for UE indicating no-gap-no-interruption	3337

A.6.6.24.5.1	Test purpose and Environment	3337

A.6.6.24.5.2	Test parameters	3337

A.6.6.24.5.3	Test Requirements	3339

A.6.6.25	SA NR - E-UTRAN event-triggered without measurement gaps	3339

A.6.6.25.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	3339

A.6.6.25.1.1	Test Purpose and Environment	3339

A.6.6.25.1.2	Test Requirements	3342

A.6.6.25.2	SA NR - E-UTRAN event-triggered reporting without gap under non-DRX in FR1	3343

A.6.6.25.2.1	Test Purpose and Environment	3343

A.6.6.25.2.2	Test parameters	3343

A.6.6.25.2.3	Test Requirements	3344

A.6.6.25.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for UE capable of inter-RAT EUTRAN measurement without gap when CRS is contained within UE’s active DL BWP	3344

A.6.6.25.3.1	Test Purpose and Environment	3344

A.6.6.25.3.2	Test Requirements	3347

A.6.6.26	LTM Intra-frequency L1-RSRP measurement	3348

A.6.6.26.1	Intra-frequency SSB based L1-RSRP measurement in FR1	3348

A.6.6.26.1.1	Test Purpose and Environment	3348

A.6.6.26.1.2	Test Parameters	3348

A.6.6.26.1.3	Test Requirements	3350

A.6.6.27	LTM Inter-frequency L1-RSRP measurement with measurement gap	3350

A.6.6.27.1	Inter-frequency SSB based L1-RSRP measurement with measurement gap	3350

A.6.6.27.1.1	Test Purpose and Environment	3350

A.6.6.27.1.2	Test parameters	3351

A.6.6.27.1.3	Test Requirements	3353

A.6.6.28	LTM Inter-frequency L1-RSRP measurement without measurement gap	3353

A.6.6.28.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap	3353

A.6.6.28.1.1	Test Purpose and Environment	3353

A.6.6.28.1.2	Test parameters	3353

A.6.6.28.1.3	Test Requirements	3355

A.6.6.29	RSCPD Measurements	3356

A.6.6.29.1	NR RSCPD with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state	3356

A.6.6.29.1.1	Test Purpose and Environment	3356

A.6.6.29.1.2	Test Requirements	3363

A.6.6.30	RSCP Measurements	3363

A.6.6.30.1	DL RSCP with UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3363

A.6.6.30.1.1	Test purpose and environment	3363

A.6.6.30.1.2	Test requirements	3367

A.6.7	Measurement Performance requirements	3367

A.6.7.1	SS-RSRP	3367

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3367

A.6.7.1.1.1	Test Purpose and Environment	3367

A.6.7.1.1.2	Test parameters	3367

A.6.7.1.1.3	Test Requirements	3371

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3371

A.6.7.1.2.1	Test Purpose and Environment	3371

A.6.7.1.2.2	Test parameters	3371

A.6.7.1.2.3	Test Requirements	3374

A.6.7.1.3	Void	3374

A.6.7.2	SS-RSRQ	3374

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3374

A.6.7.2.1.1	Test Purpose and Environment	3374

A.6.7.2.1.2	Test Parameters	3375

A.6.7.2.1.3	Test Requirements	3378

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3378

A.6.7.2.2.1	Test Purpose and Environment	3378

A.6.7.2.2.2	Test Parameters	3379

A.6.7.2.2.3	Test Requirements	3382

A.6.7.3	SS-SINR	3382

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3382

A.6.7.3.1.1	Test Purpose and Environment	3383

A.6.7.3.1.2	Test Parameters	3383

A.6.7.3.1.3	Test Requirements	3386

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3386

A.6.7.3.2.1	Test Purpose and Environment	3386

A.6.7.3.2.2	Test Parameters	3386

A.6.7.3.2.3	Test Requirements	3389

A.6.7.4	L1-RSRP measurement for beam reporting	3389

A.6.7.4.1	SSB based L1-RSRP measurement	3390

A.6.7.4.1.1	Test Purpose and Environment	3390

A.6.7.4.1.2	Test parameters	3390

A.6.7.4.1.3	Test Requirements	3393

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3393

A.6.7.4.2.1	Test Purpose and Environment	3393

A.6.7.4.2.2	Test parameters	3393

A.6.7.4.2.3	Test Requirements	3396

A.6.7.5	E-UTRAN RSRP	3396

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3396

A.6.7.5.1.1	Test Purpose and Environment	3396

A.6.7.5.1.2	Test parameters	3396

A.6.7.5.1.3	Test Requirements	3399

A.6.7.6	E-UTRAN RSRQ	3399

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3399

A.6.7.6.1.1	Test Purpose and Environment	3399

A.6.7.6.1.2	Test parameters	3399

A.6.7.6.1.3	Test Requirements	3403

A.6.7.7	E-UTRAN RS-SINR	3403

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3403

A.6.7.7.1.1	Test Purpose and Environment	3403

A.6.7.7.1.2	Test parameters	3403

A.6.7.7.1.3	Test Requirements	3406

A.6.7.8	CLI measurements	3406

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	3406

A.6.7.8.1.1	Test Purpose and Environment	3406

A.6.7.8.1.2	Test parameters	3406

A.6.7.8.1.3	Test Requirements	3409

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	3409

A.6.7.8.2.1	Test Purpose and Environment	3409

A.6.7.8.2.2	Test parameters	3410

A.6.7.8.2.3	Test Requirements	3411

A.6.7.9	L1-SINR measurement for beam reporting	3411

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3414

A.6.7.9.2.1	Test Purpose and Environment	3415

A.6.7.9.2.2	Test parameters	3415

A.6.7.9.2.3	Test Requirements	3418

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3418

A.6.7.9.3.1	Test Purpose and Environment	3418

A.6.7.9.3.2	Test parameters	3419

A.6.7.9.3.3	Test Requirements	3422

A.6.7.10	CSI-RSRP	3422

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3422

A.6.7.9.10.1	Test Purpose and Environment	3422

A.6.7.9.10.2	Test parameters	3422

A.6.7.10.1.3	Test Requirements	3425

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3425

A.6.7.9.10.1	Test Purpose and Environment	3426

A.6.7.10.2.2	Test parameters	3426

A.6.7.10.2.3	Test Requirements	3429

A.6.7.11	CSI-RSRQ	3429

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3429

A.6.7.11.1.1	Test Purpose and Environment	3429

A.6.7.11.1.2	Test Parameters	3429

A.6.7.11.1.3	Test Requirements	3433

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3433

A.6.7.11.2.1	Test Purpose and Environment	3433

A.6.7.11.2.2	Test Parameters	3433

A.6.7.11.2.3	Test Requirements	3437

A.6.7.12	CSI-SINR	3437

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3437

A.6.7.12.1.1	Test Purpose and Environment	3437

A.6.7.12.1.2	Test Parameters	3437

A.6.7.12.1.3	Test Requirements	3440

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3441

A.6.7.12.2.1	Test Purpose and Environment	3441

A.6.7.12.2.2	Test Parameters	3441

A.6.7.12.2.3	Test Requirements	3444

A.6.7.13	RSTD measurements	3444

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	3444

A.6.7.13.1.1	Test purpose and Environment	3444

A.6.7.13.1.2	Test Requirements	3446

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	3446

A.6.7.13.2.1	Test purpose and Environment	3446

A.6.7.13.2.2	Test Requirements	3448

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3448

A.6.7.13.3.1	Test purpose and Environment	3448

A.6.7.13.3.2	Test Requirements	3450

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	3450

A.6.7.13.5	NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA in RRC_CONNECTED mode	3452

A.6.7.13.5.1	Test purpose and Environment	3452

A.6.7.13.5.2	Test Requirements	3455

A.6.7.14	PRS-RSRP measurements	3455

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	3455

A.6.7.14.1.1	Test Purpose and Environment	3455

A.6.7.14.1.2	Test parameters	3455

A.6.7.14.1.3	Test Requirements	3457

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	3457

A.6.7.14.2.1	Test Purpose and Environment	3457

A.6.7.14.2.2	Test parameters	3457

A.6.7.14.2.3	Test Requirements	3459

A.6.7.14.3	Void	3459

A.6.7.14.3.1	Void	3459

A.6.7.14.3.2	Void	3459

A.6.7.14.3.3	Void	3459

A.6.7.15	UE Rx-Tx time difference measurements	3459

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	3459

A.6.7.15.1.1	Test purpose and environment	3459

A.6.7.15.1.2	Test parameters	3460

A.6.7.15.1.3	Test requirements	3461

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	3461

A.6.7.15.2.1	Test purpose and environment	3461

A.6.7.15.2.2	Test parameters	3462

A.6.7.15.2.3	Test requirements	3463

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3463

A.6.7.15.3.1	Test purpose and environment	3463

A.6.7.15.3.2	Test parameters	3464

A.6.7.15.3.3	Test requirements	3466

A.6.7.15.4	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR1 SA	3466

A.6.7.15.4.1	Test purpose and environment	3466

A.6.7.15.4.2	Test requirements	3469

A.6.7.16	PRS-RSRPP measurements	3469

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3469

A.6.7.16.1.1	Test Purpose and Environment	3469

A.6.7.16.1.2	Test parameters	3469

A.6.7.16.1.3	Test Requirements	3471

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3471

A.6.7.16.2.1	Test Purpose and Environment	3471

A.6.7.16.2.2	Test parameters	3471

A.6.7.17	LTM L1-RSRP measurement	3473

A.6.7.17.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1	3473

A.6.7.17.1.1	Test Purpose and Environment	3473

A.6.7.17.1.2	Test parameters	3474

A.6.7.17.1.3	Test Requirements	3477

A.6.7.18	TDCP amplitude measurement accuracy	3477

A.6.7.18.1	TDCP amplitude measurement accuracy in FR1	3477

A.6.7.18.1.1	Test Purpose and Environment	3477

A.6.7.18.1.2	Test parameters	3477

A.6.7.18.1.3	Test Requirements	3478

A.6.7.19	RSCPD Measurements	3479

A.6.7.19.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_CONNECTED	3479

A.6.7.19.1.1	Test purpose and environment	3479

A.6.7.19.1.2	Test parameters	3479

A.6.7.19.1.3	Test requirements	3482

A.6.7.20	RSCP Measurements	3482

A.6.7.20.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR1 SA	3482

A.6.7.20.1.1	Test purpose and environment	3482

A.6.7.20.1.2	Test parameters	3483

A.6.7.20.1.3	Test requirements	3486

A.6.8	Measurement procedure in RRC_INACTIVE	3486

A.6.8.1	RSTD measurements	3486

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3486

A.6.8.1.1.1	Test Purpose and Environment	3486

A.6.8.1.1.2	Test Requirements	3489

A.6.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3489

A.6.8.1.2.1	Test Purpose and Environment	3489

A.6.8.1.2.1	Test Purpose and Environment	3489

A.6.8.1.2.2	Test Requirements	3492

A.6.8.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_INACTIVE state	3493

A.6.8.1.3.1	Test purpose and environment	3493

A.6.8.1.3.2	Test requirements	3496

A.6.8.1.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state when eDRX cycle > 10.24s for non-RedCap UE	3497

A.6.8.1.4.1	Test Purpose and Environment	3497

A.6.8.1.4.2	Test Requirements	3500

A.6.8.2	PRS-RSRP measurements	3500

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3500

A.6.8.2.1.1	Test purpose and Environment	3500

A.6.8.2.1.2	Test Requirements	3502

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3502

A.6.8.2.2.1	Test purpose and Environment	3502

A.6.8.2.2.2	Test Requirements	3504

A.6.8.2.3	PRS-RSRP reporting delay test case in RRC_INACTIVE state in FR1 with eDRX cycle > 10.24s	3505

A.6.8.2.3.1	Test purpose and Environment	3505

A.6.8.2.3.2	Test Requirements	3507

A.6.8.3	UE Rx-Tx time difference measurements	3508

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3508

A.6.8.3.1.1	Test purpose and environment	3508

A.6.8.3.1.2	Test requirements	3510

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3510

A.6.8.3.2.1	Test purpose and environment	3510

A.6.8.3.2.2	Test requirements	3512

A.6.8.3.3	UE Rx-Tx time difference measurement for single positioning frequency layer with eDRX > 10.24s in FR1 SA	3513

A.6.8.3.3.1	Test purpose and environment	3513

A.6.8.3.3.2	Test requirements	3517

A.6.8.3.4	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR1 SA	3517

A.6.8.3.4.1	Test purpose and environment	3517

A.6.8.3.4.2	Test requirements	3520

A.6.8.4	PRS-RSRPP measurements	3520

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3520

A.6.8.4.1.1	Test purpose and Environment	3521

A.6.8.4.1.2	Test Requirements	3522

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3523

A.6.8.4.2.1	Test purpose and Environment	3523

A.6.8.4.2.2	Test Requirements	3525

A.6.8.4.3	PRS-RSRPP reporting delay in RRC_INACTIVE with eDRX	3525

A.6.8.4.3.1	Test purpose and Environment	3525

A.6.8.4.3.2	Test Requirements	3528

A.6.8.5	RSCPD Measurements	3528

A.6.8.5.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3528

A.6.8.5.1.1	Test Purpose and Environment	3528

A.6.8.5.1.2	Test Requirements	3528

A.6.8.6	RSCP Measurements	3529

A.6.8.6.1	DL RSCP with UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3529

A.6.8.6.1.1	Test purpose and environment	3529

A.6.8.6.1.2	Test requirements	3533

A.6.9	Measurement performance requirements in RRC_INACTIVE	3533

A.6.9.1	RSTD measurements	3533

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3533

A.6.9.1.1.1	Test purpose and Environment	3533

A.6.9.1.1.2	Test Requirements	3535

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3535

A.6.9.1.2.1	Test purpose and Environment	3535

A.6.9.1.2.2	Test Requirements	3537

A.6.9.1.3	RSTD measurement accuracy for PRS aggregation in FR1 in RRC_INACTIVE state	3537

A.6.9.1.3.1	Test purpose and Environment	3537

A.6.9.1.3.2	Test Requirements	3540

A.6.9.2	PRS-RSRP measurements	3540

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3540

A.6.9.2.1.1	Test Purpose and Environment	3540

A.6.9.2.1.2	Test parameters	3540

A.6.9.2.1.3	Test Requirements	3542

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3542

A.6.9.2.2.1	Test Purpose and Environment	3542

A.6.9.2.2.2	Test parameters	3542

A.6.9.2.2.3	Test Requirements	3544

A.6.9.3	UE Rx-Tx time difference measurements	3544

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3544

A.6.9.3.1.1.1	Test purpose and environment	3544

A.6.9.3.1.1.2 Test parameters	3545

A.6.9.3.1.1.3	Test requirements	3546

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3546

A.6.9.3.2.1	Test purpose and environment	3546

A.6.9.3.2.2	Test parameters	3546

A.6.9.3.2.3	Test requirements	3548

A.6.9.3.3	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR1 SA	3548

A.6.9.3.3.1	Test purpose and environment	3548

A.6.9.3.3.2	Test requirements	3551

A.6.9.4	PRS-RSRPP measurements	3551

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3551

A.6.9.4.1.1	Test Purpose and Environment	3551

A.6.9.4.1.2	Test parameters	3551

A.6.9.4.1.3	Test Requirements	3553

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3553

A.6.9.4.2.1	Test Purpose and Environment	3553

A.6.9.4.2.2	Test parameters	3554

A.6.9.4.2.3	Test Requirements	3555

A.6.9.5	RSCPD Measurements	3556

A.6.9.5.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_INACTIVE	3556

A.6.9.5.1.1	Test purpose and environment	3556

A.6.9.5.1.2	Test parameters	3556

A.6.9.5.1.3	Test requirements	3559

A.6.9.6	RSCP Measurements	3559

A.6.9.6.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR1 SA	3559

A.6.9.6.1.1	Test purpose and environment	3559

A.6.9.6.1.2	Test parameters	3560

A.6.9.6.1.3	Test requirements	3563

A.6.10	Measurement Procedure in RRC_IDLE	3563

A.6.10.1	RSTD Measurements	3563

A.6.10.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state for non-RedCap UE	3563

A.6.10.1.1.1	Test purpose and environment	3563

A.6.10.1.1.2	Test requirements	3566

A.6.10.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s for non-RedCap UE	3567

A.6.10.1.2.1	Test Purpose and Environment	3567

A.6.10.1.2.2	Test Requirements	3570

A.6.10.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_IDLE state	3570

A.6.10.1.3.1	Test purpose and environment	3570

A.6.10.1.3.2	Test requirements	3570

A.6.10.2	 PRS-RSRP Measurements	3571

A.6.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE state for non-RedCap UE in FR1	3571

A.6.10.2.1.1	Test purpose and Environment	3571

A.6.10.2.1.2	Test Requirements	3573

A.6.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR1 when eDRX cycle > 10.24s	3574

A.6.10.2.2.1	Test purpose and Environment	3574

A.6.10.2.2.2	Test Requirements	3574

A.6.10.3	RSCPD Measurements	3574

A.6.10.3.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state	3574

A.6.10.3.1.1	Test Purpose and Environment	3574

A.6.10.3.1.2	Test Requirements	3575

A.6.11	Measurement Performance Requirements in RRC_IDLE	3575

A.6.11.1	RSTD Measurements	3575

A.6.11.1.1	NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA in RRC_IDLE state for non-RedCap UE	3575

A.6.11.1.1.1	Test purpose and environment	3575

A.6.11.1.1.2	Test requirements	3577

A.6.11.1.2	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_IDLE state with eDRX>10.24s for non-RedCap UE	3577

A.6.11.1.2.1	Test purpose and Environment	3577

A.6.11.1.2.2	Test Requirements	3579

A.6.11.1.3	NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA in RRC_IDLE state	3579

A.6.11.1.3.1	Test purpose and environment	3579

A.6.11.1.3.2	Test requirements	3579

A.6.11.2	PRS-RSRP measurements	3579

A.6.11.2.1	PRS-RSRP measurement accuracy test case for non-RedCap UE in FR1 in RRC_IDLE state	3579

A.6.11.2.1.1	Test Purpose and Environment	3579

A.6.11.2.1.2	Test parameters	3580

A.6.11.2.1.3	Test Requirements	3582

A.6.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR1 when eDRX cycle > 10.24s	3582

A.6.11.2.2.1	Test purpose and Environment	3582

A.6.11.2.2.2	Test parameters	3582

A.6.11.2.2.3	Test Requirements	3583

A.6.11.3	RSCPD Measurements	3583

A.6.11.3.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_IDLE	3583

A.6.11.3.1.1	Test purpose and environment	3583

A.6.11.3.1.2	Test parameters	3583

A.6. 11.3.1.3	Test requirements	3586

A.7	NR standalone tests with one or more NR cells in FR2	3587

A.7.1	SA: RRC_IDLE state mobility	3587

A.7.1.1	Cell re-selection to NR	3587

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3587

A.7.1.1.1.1	Test Purpose and Environment	3587

A.7.1.1.1.2	Test Parameters	3587

A.7.1.1.1.3	Test Requirements	3589

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3589

A.7.1.1.2.1	Test Purpose and Environment	3590

A.7.1.1.2.2	Test Parameters	3590

A.7.1.1.2.3	Test Requirements	3591

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3592

A.7.1.1.3.1	Test Purpose and Environment	3592

A.7.1.1.3.2	Test Parameters	3592

A.7.1.1.3.3	Test Requirements	3594

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3594

A.7.1.1.4.1	Test Purpose and Environment	3594

A.7.1.1.4.2	Test Parameters	3595

A.7.1.1.4.3	Test Requirements	3596

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3597

A.7.1.1.5.1	Test Purpose and Environment	3597

A.7.1.1.5.2	Test Parameters	3597

A.7.1.1.5.3	Test Requirements	3599

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3599

A.7.1.1.6.1	Test Purpose and Environment	3599

A.7.1.1.6.2	Test Parameters	3600

A.7.1.1.6.3	Test Requirements	3601

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3602

A.7.1.1.7.1	Test Purpose and Environment	3602

A.7.1.1.7.2	Test Parameters	3602

A.7.1.1.7.3	Test Requirements	3604

A.7.1.1.8	Cell reselection to FR2 inter-frequency NR case for UE configured with highSpeedMeasFlagFR2-r17	3605

A.7.1.1.8.1	Test Purpose and Environment	3605

A.7.1.1.8.2	Test Parameters	3605

A.7.1.1.8.3	Test Requirements	3607

A.7.2	SA: RRC_INACTIVE state mobility	3607

A.7.2.1	Small Data Transmission	3607

A.7.2.1.1	TA validation for CG-SDT in FR2	3607

A.7.2.1.1.1	Test Purpose and Environment	3607

A.7.2.1.1.2	Test Requirements	3610

A.7.2.2	Cell reselection for positioning	3611

A.7.2.2.1	Cell reselection to FR2 intra-frequency NR case with RRC_ INACTIVE eDRX and positioning SRS	3611

A.7.2.2.1.1	Test Purpose and Environment	3611

A.7.2.2.1.2	Test Parameters	3611

A.7.2.2.1.3	Test Requirements	3613

A.7.3	RRC_CONNECTED state mobility	3613

A.7.3.1	Handover	3613

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3613

A.7.3.1.1.1	Test Purpose and Environment	3613

A.7.3.1.1.2	Test Parameters	3613

A.7.3.1.1.3	Test Requirements	3615

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3616

A.7.3.1.2.1	Test Purpose and Environment	3616

A.7.3.1.2.2	Test Parameters	3616

A.7.3.1.2.3	Test Requirements	3617

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3617

A.7.3.1.3.1	Test Purpose and Environment	3617

A.7.3.1.3.2	Test Parameters	3617

A.7.3.1.3.3	Test Requirements	3619

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3619

A.7.3.1.4.1	Test Purpose and Environment	3619

A.7.3.1.4.2	Test Parameters	3619

A.7.3.1.4.3 Test Requirements	3622

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3623

A.7.3.1.5.1	Test Purpose and Environment	3623

A.7.3.1.5.2	Test Parameters	3623

A.7.3.1.5.3 Test Requirements	3626

A.7.3.1.6	Handover with PSCell from SA to EN-DC with unknown FR2 target PScell	3627

A.7.3.1.6.1	Test Purpose and Environment	3627

A.7.3.1.6.2	Test Parameters	3627

A.7.3.1.6.3	Test Requirements	3632

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3632

A.7.3.1.7.1	Test purpose and environment	3632

A.7.3.1.7.2	Test Requirements	3636

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3637

A.7.3.1.8.1	Test Purpose and Environment	3637

A.7.3.1.8.2	Test Requirements	3640

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3640

A.7.3.1.9.1	Test Purpose and Environment	3640

A.7.3.1.9.2	Test Parameters	3640

A.7.3.1.9.3	Test Requirements	3642

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3643

A.7.3.1.10.1	Test Purpose and Environment	3643

A.7.3.1.10.2	Test Parameters	3643

A.7.3.1.10.3	Test Requirements	3645

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3645

A.7.3.1.11.1	Test Purpose and Environment	3645

A.7.3.1.11.2	Test Parameters	3645

A.7.3.1.11.3	Test Requirements	3647

A.7.3.1.12	Intra-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB	3648

A.7.3.1.12.1	Test Purpose and Environment	3648

A.7.3.1.12.2	Test Parameters	3648

A.7.3.1.12.3	Test Requirements	3649

A.7.3.1.13	Inter-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB	3650

A.7.3.1.13.1	Test Purpose and Environment	3650

A.7.3.1.13.2	Test Parameters	3650

A.7.3.1.13.3	Test Requirements	3652

A.7.3.1.14	Handover with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC with target PSCell in FR1	3652

A.7.3.1.14.1	Test Purpose and Environment	3652

A.7.3.1.14.2	Test Requirements	3656

A.7.3.1.15	HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC	3656

A.7.3.1.15.1	Test Purpose and Environment	3656

A.7.3.1.15.2	Test Requirements	3661

A.7.3.2	RRC Connection Mobility Control	3661

A.7.3.2.1	SA: RRC Re-establishment	3661

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3661

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3663

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3666

A.7.3.2.1.3.1	Test Purpose and Environment	3666

A.7.3.2.1.3.2	Test Requirements	3667

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3668

A.7.3.2.1.4.1	Test Purpose and Environment	3668

A.7.3.2.1.4.2	Test Requirements	3670

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3670

A.7.3.2.1.5.1	Test Purpose and Environment	3670

A.7.3.2.1.5.2	Test Requirements	3672

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3673

A.7.3.2.1.6.1	Test Purpose and Environment	3673

A.7.3.2.1.6.2	Test Requirements	3674

A.7.3.2.2	Random Access	3675

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3675

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3678

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3681

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3684

A.7.3.2.3	SA: RRC Connection Release with Redirection	3687

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3687

A.7.3.2.4	LTM PDCCH-order Random Access	3689

A.7.3.2.4.1	PDCCH-order RACH on neighbor cell in FR2 when RACH BW is within active BWP	3689

A.7.3.2.4.2	PDCCH-order RACH on inter-frequency neighbor cell in FR2	3692

A.7.3.3	Conditional Handover	3695

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3695

A.7.3.3.1.1	Test Purpose and Environment	3695

A.7.3.3.1.2	Test Parameters	3695

A.7.3.3.1.2.3	Test Requirements	3696

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3697

A.7.3.3.2.1	Test Purpose and Environment	3697

A.7.3.3.2.2	Test Parameters	3697

A.7.3.3.2.3Test Requirements	3698

A.7.3.3.3	NES triggering intra-frequency target CHO delay From FR2 to FR2	3698

A.7.3.3.3.1	Test Purpose and Environment	3698

A.7.3.3.3.2	Test Parameters	3699

A.7.3.3.3.2.3	Test Requirements	3700

A.7.3.3.4	NES triggering inter-frequency conditional handover from FR2 to FR1	3700

A.7.3.3.4.1	Test Purpose and Environment	3700

A.7.3.3.4.2	Test Parameters	3700

A.7.3.3.4.3	Test Requirements	3702

A.7.3.3.5	NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3703

A.7.3.3.5.1	Test Purpose and Environment	3703

A.7.3.3.5.2	Test Requirements	3706

A.7.3.3.5.2.1	Test Requirements for NR conditional handover	3706

A.7.3.3.5.2.2	Test Requirements for NR PSCell change	3706

A.7.3.3.6	NR conditional Handover including target MCG and candidate SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3706

A.7.3.3.6.1	Test Purpose and Environment	3706

A.7.3.3.6.2	Test Parameters	3706

A.7.3.3.6.3 Test Requirements	3710

A.7.3.4	LTM PCell Switch	3710

A.7.3.4.1	RACH based Intra-frequency PCell switch from FR2 to FR2	3710

A.7.3.4.1.1	Test Purpose and Environment	3710

A.7.3.4.1.2	Test Parameters	3710

A.7.3.4.1.3	Test Requirements	3713

A.7.3.4.2	RACH-less Intra-frequency PCell switch from FR2 to FR2	3714

A.7.3.4.2.1	Test Purpose and Environment	3714

A.7.3.4.2.2	Test Parameters	3714

A.7.3.4.2.3	Test Requirements	3718

A.7.3.4.3	RACH-based Inter-frequency LTM PCell switch from FR2 to FR2	3718

A.7.3.4.3.1	Test Purpose and Environment	3718

A.7.3.4.3.2	Test Parameters	3718

A.7.3.4.3.3	Test Requirements	3721

A.7.3.5	LTM PSCell Switch	3722

A.7.3.5.1	RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2	3722

A.7.3.5.1.1	Test Purpose and Environment	3722

A.7.3.5.1.2	Test Parameters	3722

A.7.3.5.1.3	Test Requirements	3726

A.7.4	Timing	3727

A.7.4.1	UE transmit timing	3727

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3727

A.7.4.1.1.1	Test Purpose and environment	3727

A.7.4.1.1.2	Test requirements	3729

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3729

A.7.4.1.2.1	Test Purpose and environment	3729

A.7.4.1.2.2	Test requirements	3732

A.7.4.1.3	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	3732

A.7.4.1.3.1	Test Purpose and environment	3732

A.7.4.1.3.2	Test requirements	3736

A.7.4.2	UE timer accuracy	3736

A.7.4.3	Timing advance	3736

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3736

A.7.4.3.1.1	Test Purpose and Environment	3736

A.7.4.3.1.2	Test Parameters	3737

A.7.4.3.1.3 Test Requirements	3739

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3739

A.7.4.3.2.1	Test Purpose and Environment	3739

A.7.4.3.2.2	Test Parameters	3739

A.7.4.3.2.3	Test Requirements	3741

A.7.5	Signaling characteristics	3744

A.7.5.1	Radio link Monitoring	3744

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3744

A.7.5.1.1.1	Test Purpose and Environment	3744

A.7.5.1.1.2	Test Requirements	3747

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3747

A.7.5.1.2.1	Test Purpose and Environment	3747

A.7.5.1.2.2	Test Requirements	3750

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3750

A.7.5.1.3.1	Test Purpose and Environment	3750

A.7.5.1.3.2	Test Requirements	3752

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3753

A.7.5.1.4.1	Test Purpose and Environment	3753

A.7.5.1.4.2	Test Requirements	3755

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3755

A.7.5.1.5.1	Test Purpose and Environment	3755

A.7.5.1.5.2	Test Requirements	3758

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3758

A.7.5.1.6.1	Test Purpose and Environment	3758

A.7.5.1.6.2	Test Requirements	3760

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3761

A.7.5.1.7.1	Test Purpose and Environment	3761

A.7.5.1.7.2	Test Requirements	3763

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3763

A.7.5.1.8.1	Test Purpose and Environment	3763

A.7.5.1.8.2	Test Requirements	3766

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3767

A.7.5.1.9.1	Test Purpose and Environment	3767

A.7.5.1.9.2	Test Requirements	3768

A.7.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	3768

A.7.5.1.10.1	Test Purpose and Environment	3768

A.7.5.1.10.2	Test Requirements	3771

A.7.5.1.11	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	3772

A.7.5.1.11.1	Test Purpose and Environment	3772

A.7.5.1.12	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	3772

A.7.5.1.12.1	Test Purpose and Environment	3772

A.7.5.1.12.2	Test Requirements	3772

A.7.5.1.13	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	3772

A.7.5.1.13.1	Test Purpose and Environment	3772

A.7.5.1.13.2	Test Requirements	3775

A.7.5.2	Interruption	3775

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3775

A.7.5.2.1.1	Test Purpose and Environment	3775

A.7.5.2.1.2	Test Requirements	3778

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3778

A.7.5.2.2.1	Test Purpose and Environment	3778

A.7.5.2.2.2	Test Parameters	3778

A.7.5.2.2.3	Test Requirements	3780

A.7.5.3	SCell Activation and Deactivation Delay	3780

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3780

A.7.5.3.1.1	Test Purpose and Environment	3780

A.7.5.3.1.2	Test Requirements	3782

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3782

A.7.5.3.2.1	Test Purpose and Environment	3782

A.7.5.3.2.2	Test Requirements	3784

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3785

A.7.5.3.3.1	Test Purpose and Environment	3785

A.7.5.3.3.2	Test Requirements	3787

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3788

A.7.5.3.4.1	Test Purpose and Environment	3788

A.7.5.3.4.2	Test Requirements	3790

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3791

A.7.5.3.5.1	Test Purpose and Environment	3791

A.7.5.3.5.2	Test Requirements	3793

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3794

A.7.5.3.6.1	Test Purpose and Environment	3794

A.7.5.3.6.2	Test Requirements	3796

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3797

A.7.5.3.7.1	Test Purpose and Environment	3797

A.7.5.3.7.2	Test Requirements	3800

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3801

A.7.5.3.8.1	Test Purpose and Environment	3801

A.7.5.3.8.2	Test Requirements	3804

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3805

A.7.5.3.9.1	Test Purpose and Environment	3805

A.7.5.3.9.2	Test Requirements	3807

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3808

A.7.5.3.10.1	Test Purpose and Environment	3808

A.7.5.3.10.2	Test Requirements	3811

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3812

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3812

A.7.5.3.11.1.1	Test Purpose and Environment	3812

A.7.5.3.11.1.2	Test Requirements	3815

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3816

A.7.5.3.11.2.1	Test Purpose and Environment	3816

A.7.5.3.11.2.2	Test Requirements	3819

A.7.5.3.12	Void	3820

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3820

A.7.5.3.13.1	Test Purpose and Environment	3820

A.7.5.3.13.2	Test Requirements	3822

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3822

A.7.5.3.14.1	Test Purpose and Environment	3822

A.7.5.3.14.2	Test Requirements	3824

A.7.5.3.15	PUCCH SCell activation and deactivation with FR1 PCell based on L3 reporting after SCell activation command	3825

A.7.5.3.15.1	Test Purpose and Environment	3825

A.7.5.3.15.2	Test Requirements	3829

A.7.5.3.16	PUCCH SCell activation and deactivation with FR2 PCell based on L3 reporting after SCell activation command	3829

A.7.5.3.16.1	Test Purpose and Environment	3829

A.7.5.3.16.2	Test Requirements	3832

A.7.5.3.17	SCell Activation and deactivation for SCell in FR2 inter-band in DRX for UE capable of small beam sweeping factors and/or short measurement interval	3833

A.7.5.3.17.1	Test Purpose and Environment	3833

A.7.5.3.17.2	Test Requirements	3835

A.7.5.3.18	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2, in DRX, for UE capable of small beam sweeping factors and/or short measurement interval	3837

A.7.5.3.18.1	Test Purpose and Environment	3837

A.7.5.3.18.2	Test Requirements	3840

A.7.5.3.19	SCell Activation and deactivation of FR2 unknown SCell with FR1 PCell in non-DRX with L3 reporting during activation	3842

A.7.5.3.19.1	Test Purpose and Environment	3842

A.7.5.3.19.2	Test Requirements	3845

A.7.5.3.20	SCell Activation and Deactivation of FR2 unkown SCell with FR2 PCell in non-DRX with L3 reporting during activation	3845

A.7.5.3.20.1	Test Purpose and Environment	3846

A.7.5.3.20.2	Test Requirements	3848

A.7.5.4	Void	3849

A.7.5.5	Beam Failure Detection and Link recovery procedures	3849

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3849

A.7.5.5.1.1	Test Purpose and Environment	3849

A.7.5.5.1.2	Test Requirements	3852

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3853

A.7.5.5.2.1	Test Purpose and Environment	3853

A.7.5.5.2.2	Test Requirements	3856

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3856

A.7.5.5.3.1	Test Purpose and Environment	3856

A.7.5.5.3.2	Test Requirements	3859

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3859

A.7.5.5.4.1	Test Purpose and Environment	3859

A.7.5.5.4.2	Test Requirements	3862

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3863

A.7.5.5.5.1	Test Purpose and Environment	3863

A.7.5.5.5.2	Test Requirements	3866

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3866

A.7.5.5.6.1	Test Purpose and Environment	3866

A.7.5.5.6.2	Test Requirements	3869

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3869

A.7.5.5.7.1	Test Purpose and Environment	3869

A.7.5.5.7.2	Test Requirements	3872

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3873

A.7.5.5.8.1	Test Purpose and Environment	3873

A.7.5.5.8.2	Test Requirements	3876

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3876

A.7.5.5.9.1	Test Purpose and Environment	3876

A.7.5.5.9.2	Test Requirements	3879

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3880

A.7.5.5.10.1	Test Purpose and Environment	3880

A.7.5.5.10.2	Test Requirements	3883

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3883

A.7.5.5.11.1	Test Purpose and Environment	3883

A.7.5.5.11.2	Test Requirements	3886

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3886

A.7.5.5.12.1	Test Purpose and Environment	3886

A.7.5.5.12.2	Test Requirements	3889

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3890

A.7.5.5.13.1	Test Purpose and Environment	3890

A.7.5.5.13.2	Test Requirements	3892

A.7.5.5.14	TRP specific Beam Failure Detection and Link Recovery for FR2 PCell configured with CSI-RS-based BFD and LR and multi-Rx operation in DRX mode	3892

A.7.5.5.14.1	Test Purpose and Environment	3892

A.7.5.5.14.2	Test Requirements	3896

A.7.5.6	Active BWP switch	3896

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3896

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3896

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3900

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3904

A.7.5.6.1.3.1	Test Purpose and Environment	3904

A.7.5.6.1.3.2	Test Requirements	3906

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3906

A.7.5.6.1.4.1	Test Purpose and Environment	3906

A.7.5.6.1.4.2	Test Requirements	3909

A.7.5.6.2	RRC-based Active BWP Switch	3910

A.7.5.6.2.1.1	Test Purpose and Environment	3910

A.7.5.6.2.1.2	Test Requirements	3913

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3913

A.7.5.6.2.2.1	Test Purpose and Environment	3913

A.7.5.6.2.2.2	Test Requirements	3915

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3916

A.7.5.6.3.1.1	Test Purpose and Environment	3916

A.7.5.6.3.1.2	Test Requirements	3918

A.7.5.6.4	SCell dormancy switch	3919

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3919

A.7.5.6.4.1.1	Test Purpose and Environment	3919

A.7.5.6.4.1.2	Test Requirements	3921

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3922

A.7.5.6.4.2.1	 Test Purpose and Environment	3922

A.7.5.6.4.2.2	 Test Requirements	3925

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3925

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3925

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3927

A.7.5.6.5.2.1	Test Purpose and Environment	3927

A.7.5.6.5.2.2	Test Requirements	3930

A.7.5.7	PSCell addition and release delay	3930

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3930

A.7.5.7.1.1	Test Purpose and Environment	3930

A.7.5.7.1.2	Test Requirements	3933

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3933

A.7.5.7.2.1	Test Purpose and Environment	3933

A.7.5.7.2.2	Test Requirements	3935

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3935

A.7.5.7.3.1	Test Purpose and Environment	3935

A.7.5.7.3.2	Test Requirements	3938

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3938

A.7.5.7.4.1	Test Purpose and Environment	3938

A.7.5.7.4.2	Test Requirements	3940

A.7.5.8	Active TCI state switch delay	3941

A.7.5.8.1	MAC-CE based active TCI state switch	3941

A.7.5.8.2	RRC based active TCI state switch	3944

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3947

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3947

A.7.5.8.3.1.1	Test Purpose and Environment	3947

A.7.5.8.3.1.2	Test Requirements	3950

A.7.5.8.3.2	NR PCell FR2 HST active TCI state switch for PC6 UE supporting tci-StateSwitchInd-r18 for a known TCI state	3951

A.7.5.8.3.2.1	Test Purpose and Environment	3951

A.7.5.8.3.2.2	Test Requirements	3954

A.7.5.8.4	DCI based active TCI state switch with m-DCI for simultaneous reception	3954

A.7.5.8.4.1	Test Purpose and Environment	3954

A.7.5.8.4.2	Test Requirements	3957

A.7.5.8.5	Single-DCI FR2 DCI based active TCI state switch with known target TCI states for simultaneous reception	3957

A.7.5.8.5.1	Test Purpose and Environment	3957

A.7.5.8.5.1.2	Test Requirements	3959

A.7.5.9	Uplink spatial relation switch delay	3960

A.7.5.9.1.1.1	Test Purpose and Environment	3960

A.7.5.9.1.1.2	Test Requirements	3962

A.7.5.9.2	RRC based spatial relation switch	3962

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3962

A.7.5.9.2.1.1	Test Purpose and Environment	3962

A.7.5.9.2.1.2	Test Requirements	3964

A.7.5.10	UE specific CBW change	3964

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3964

A.7.5.10.1.1	Test Purpose and Environment	3964

A.7.5.10.1.2	Test Requirements	3966

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3967

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3967

A.7.5.11.1.1	Test Purpose and Environment	3967

A.7.5.11.1.2	Test Requirements	3969

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3969

A.7.5.12.1	Addition and Release Delay of PSCell	3969

A.7.5.12.1.1	Test purpose and environment	3969

A.7.5.12.1.2	Test Parameters	3969

A.7.5.12.1.3	Test Requirements	3971

A.7.5.13	Unified TCI state switching delay	3971

A.7.5.13.1	MAC-CE based active joint TCI state switching	3971

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3971

A.7.5.13.1.1.1	Test Purpose and Environment	3971

A.7.5.13.1.1.2	Test parameters	3972

A.7.5.13.1.1.3	Test Requirements	3973

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3974

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3974

A.7.5.13.2.1.1	Test Purpose and Environment	3974

A.7.5.13.2.1.2	Test parameters	3974

A.7.5.13.2.1.3	Test Requirements	3976

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3976

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3976

A.7.5.13.3.1.1	Test Purpose and Environment	3976

A.7.5.13.3.1.2	Test Parameters	3976

A.7.5.13.3.1.3	Test Requirements	3979

A.7.5.13.4	sDCI MAC-CE based joint TCI state switching	3980

A.7.5.13.4.1	NR PCell FR2 dual downlink and uplink TCI state switch in sDCI for known case	3980

A.7.5.13.4.1.1	Test Purpose and Environment	3980

A.7.5.13.4.1.2	Test parameters	3980

A.7.5.13.4.1.3	Test Requirements	3982

A.7.5.13.5	MAC-CE based dual downlink TCI state switching delay for unified TCI for single-DCI mTRP	3982

A.7.5.13.5.1	NR PCell FR2 dual downlink TCI state switch in sDCI for known case	3982

A.7.5.13.5.1.1	Test Purpose and Environment	3982

A.7.5.13.5.1.2	Test Parameters	3983

A.7.5.13.5.1.3	Test Requirements	3985

A.7.5.13.6	 MAC-CE based active uplink TCI state switch for single-DCI mTRP	3985

A.7.5.13.6.1	 NR FR2 PCell uplink TCI state switch for two known TCI states	3985

A.7.5.13.6.1.1	Test Purpose and Environment	3985

A.7.5.13.6.1.2	Test parameters	3986

A.7.5.13.6.1.3	Test Requirements	3987

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3987

A.7.5.14.1	Test Purpose and Environment	3987

A.7.5.14.2	Test Requirements	3990

A.7.5.15	Void	3990

A.7.5.16	UE L1-RSRP Scheduling and Measurement Restrictions on FR2-1	3991

A.7.5.16.1	Test Purpose and Environment	3991

A.7.5.16.2	Test Requirements	3993

A.7.5.17	SCG Activation and deactivation for FR1+FR1 inter-band with target PSCell in FR1	3994

A.7.5.17.1	Test Purpose and Environment	3994

A.7.5.17.2	Test Requirements	3996

A.7.5.18	Subsequent conditional PSCell addition/change	3997

A.7.5.18.1	Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3997

A.7.5.18.1.1	Test purpose and environment	3997

A.7.5.18.1.2	Test Requirements	4000

A.7.5.18.2	Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC	4000

A.7.5.18.2.1	Test Purpose and Environment	4000

A.7.5.18.2.2	Test Requirements	4003

A.7.6	Measurement procedure	4005

A.7.6.1	Intra-frequency Measurements	4005

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	4005

A.7.6.1.1.1	Test purpose and Environment	4005

A.7.6.1.1.2	Test Requirements	4007

A.7.6.1.2	SA event triggered reporting test without gap under DRX	4007

A.7.6.1.2.1	Test purpose and Environment	4007

A.7.6.1.2.2	Test Requirements	4009

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	4009

A.7.6.1.3.1	Test purpose and Environment	4009

A.7.6.1.3.2	Test Requirements	4012

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	4012

A.7.6.1.4.1	Test purpose and Environment	4012

A.7.6.1.4.2	Test Requirements	4014

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	4015

A.7.6.1.5.1	Test purpose and Environment	4015

A.7.6.1.5.2	Test Requirements	4017

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	4017

A.7.6.1.6.1	Test purpose and Environment	4017

A.7.6.1.6.2	Test Requirements	4019

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	4020

A.7.6.1.7.1	Test purpose and Environment	4020

A.7.6.1.7.2	Test Requirements	4022

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	4023

A.7.6.1.8.1	Test purpose and Environment	4023

A.7.6.1.8.2	Test Requirements	4025

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	4026

A.7.6.1.9.1	Test purpose and Environment	4026

A.7.6.1.9.2	Test Requirements	4028

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	4029

A.7.6.1.10.1	Test purpose and Environment	4029

A.7.6.1.10.2	Test Requirements	4031

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	4031

A.7.6.1.11.1	Test purpose and Environment	4031

A.7.6.1.11.2	Test Requirements	4033

A.7.6.1.12	SA event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP	4034

A.7.6.1.12.1	Test purpose and Environment	4034

A.7.6.1.12.2	Test Requirements	4034

A.7.6.1.13	SA event triggered reporting test without gap under non-DRX with NCD-SSB	4034

A.7.6.1.13.1	Test purpose and Environment	4034

A.7.6.1.13.2	Test Requirements	4036

A.7.6.1.14	SA event triggered reporting test without gap under non-DRX for power class 6 UE supporting measEnhCAInterFreqFR2-r18	4037

A.7.6.1.14.1	Test Purpose and Environment	4037

A.7.6.1.14.2	Test Requirements	4038

A.7.6.2	Inter-frequency Measurements	4038

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	4038

A.7.6.2.1.1	Test Purpose and Environment	4038

A.7.6.2.1.2	Test Requirements	4040

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR2)	4041

A.7.6.2.2.1	Test Purpose and Environment	4041

A.7.6.2.2.2	Test Requirements	4043

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	4043

A.7.6.2.3.1	Test Purpose and Environment	4043

A.7.6.2.3.2	Test Requirements	4045

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2)	4046

A.7.6.2.4.1	Test Purpose and Environment	4046

A.7.6.2.4.2	Test Requirements	4048

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	4048

A.7.6.2.5.1	Test Purpose and Environment	4048

A.7.6.2.5.2	Test Requirements	4051

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	4051

A.7.6.2.6.1	Test Purpose and Environment	4051

A.7.6.2.6.2	Test Requirements	4054

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	4054

A.7.6.2.7.1	Test Purpose and Environment	4054

A.7.6.2.7.2	Test Requirements	4057

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	4057

A.7.6.2.8.1	Test Purpose and Environment	4057

A.7.6.2.8.2	Test Requirements	4060

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	4060

A.7.6.2.9.1	Test Purpose and Environment	4060

A.7.6.2.9.2	Test Requirements	4062

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	4063

A.7.6.2.10.1	Test Purpose and Environment	4063

A.7.6.2.10.2	Test Requirements	4064

A.7.6.2.11	SA event triggered reporting test without gap under DRX	4064

A.7.6.2.11.1	Test Purpose and Environment	4065

A.7.6.2.11.2	Test Requirements	4066

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	4067

A.7.6.2.12.1	Test Purpose and Environment	4067

A.7.6.2.12.2	Test Requirements	4069

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	4069

A.7.6.2.13.1	Test Purpose and Environment	4069

A.7.6.2.13.2	Test Requirements	4072

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	4073

A.7.6.2.14.1	Test Purpose and Environment	4073

A.7.6.2.14.2	Test Requirements	4075

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	4076

A.7.6.2.15.1	Test Purpose and Environment	4076

A.7.6.2.15.2	Test Requirements	4078

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	4079

A.7.6.2.16.1	Test Purpose and Environment	4079

A.7.6.2.16.2	Test Requirements	4083

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	4083

A.7.6.2.17.1	Test Purpose and Environment	4083

A.7.6.2.17.2	Test Requirements	4087

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	4088

A.7.6.2.18.1	Test Purpose and Environment	4088

A.7.6.2.18.2	Test Requirements	4091

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	4092

A.7.6.2.19.1	Test Purpose and Environment	4092

A.7.6.2.19.2	Test Requirements	4096

A.7.6.2.20	SA event triggered reporting tests for FR2 with measurement gap with priority and two periodic MUSIM gaps configured	4097

A.7.6.2.20.1	Test Purpose and Environment	4097

A.7.6.2.20.2	Test Requirements	4099

A.7.6.2.21	SA event triggered reporting tests for FR2 with measurement gap without priority and periodic MUSIM gap configured	4099

A.7.6.2.21.1	Test Purpose and Environment	4099

A.7.6.2.21.2	Test Requirements	4101

A.7.6.2.22	SA event triggered reporting tests with SSB time index detection when DRX is not used (PCell in FR2) for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	4102

A.7.6.2.22.1	Test Purpose and Environment	4102

A.7.6.2.22.2	Test Requirements	4104

A.7.6.2.23	SA event triggered reporting tests without SSB time index detection when DRX is not used (PCell in FR2) for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	4104

A.7.6.2.23.1	Test Purpose and Environment	4104

A.7.6.2.23.2	Test Requirements	4106

A.7.6.3	L1-RSRP measurement for beam reporting	4106

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	4106

A.7.6.3.1.1	Test Purpose and Environment	4106

A.7.6.3.1.2	Test parameters	4106

A.7.6.3.1.3	Test Requirements	4108

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	4108

A.7.6.3.2.1	Test Purpose and Environment	4108

A.7.6.3.2.2	Test parameters	4108

A.7.6.3.2.3	Test Requirements	4109

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	4110

A.7.6.3.3.1	Test Purpose and Environment	4110

A.7.6.3.3.2	Test parameters	4110

A.7.6.3.3.3	Test Requirements	4111

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	4111

A.7.6.3.4.1	Test Purpose and Environment	4111

A.7.6.3.4.2	Test parameters	4112

A.7.6.3.3.3	Test Requirements	4113

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	4113

A.7.6.3.5.1	Test Purpose and Environment	4113

A.7.6.3.5.2	Test parameters	4114

A.7.6.3.5.3	Test Requirements	4115

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	4115

A.7.6.3.6.1	Test Purpose and Environment	4115

A.7.6.3.6.2	Test parameters	4116

A.7.6.3.6.3	Test Requirements	4118

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	4118

A.7.6.3.7.1	Test Purpose and Environment	4118

A.7.6.3.7.2	Test parameters	4119

A.7.6.3.7.3	Test Requirements	4120

A.7.6.3.8	CSI-RS based L1-RSRP measurement when DRX is not used and when CD-SSB is outside active BWP	4120

A.7.6.3.8.1	Test Purpose and Environment	4120

A.7.6.3.9	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	4121

A.7.6.3.9.1	Test Purpose and Environment	4121

A.7.6.3.9.2	Test Requirements	4121

A.7.6.3.10	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	4121

A.7.6.3.10.1	Test Purpose and Environment	4121

A.7.6.3.10.2	Test parameters	4121

A.7.6.3.10.3	Test Requirements	4123

A.7.6.3.11	SSB based L1-RSRP measurement when DRX is used for power class 6 UE supporting simultaneousReceptionTwoQCL-r18	4123

A.7.6.3.11.1	Test Purpose and Environment	4123

A.7.6.3.11.2	Test parameters	4123

A.7.6.3.11.3	Test Requirements	4125

A.7.6.4	CLI measurements	4125

A.7.6.4.1	SRS-RSRP measurement with non-DRX	4125

A.7.6.4.1.1	Test Purpose and Environment	4125

A.7.6.4.1.2	Test Parameters	4125

A.7.6.4.1.3	Test Requirements	4127

A.7.6.4.2	CLI-RSSI measurement with non-DRX	4127

A.7.6.4.2.1	Test Purpose and Environment	4127

A.7.6.4.2.2	Test Parameters	4128

A.7.6.4.2.3	Test Requirements	4129

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	4129

A.7.6.5.1.1	Test Purpose and Environment	4129

A.7.6.5.1.2	Test Requirements	4132

A.7.6.6	L1-SINR measurement for beam reporting	4132

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	4134

A.7.6.6.2.1	Test Purpose and Environment	4134

A.7.6.6.2.2	Test parameters	4134

A.7.6.6.2.3	Test Requirements	4135

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	4136

A.7.6.6.3.1	Test Purpose and Environment	4136

A.7.6.6.3.2	Test parameters	4136

A.7.6.6.3.3	Test Requirements	4137

A.7.6.7	CSI-RS based intra-frequency Measurements	4138

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	4138

A.7.6.7.1.1	Test purpose and Environment	4138

A.7.6.7.1.2	Test Requirements	4139

A.7.6.8	CSI-RS based inter-frequency Measurements	4140

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	4140

A.7.6.8.1.1	Test Purpose and Environment	4140

A.7.6.8.1.2	Test Requirements	4142

A.7.6.9	RSTD measurements	4142

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	4142

A.7.6.9.1.1	Test Purpose and Environment	4142

A.7.6.9.1.2	Test Requirements	4146

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	4146

A.7.6.9.2.1	Test Purpose and Environment	4146

A.7.6.9.2.2	Test Requirements	4149

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	4150

A.7.6.9.3.1	Test Purpose and Environment	4150

A.7.6.9.3.2	Test Requirements	4152

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	4153

A.7.6.9.4.1	Test Purpose and Environment	4153

A.7.6.9.4.2	Test Requirements	4155

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	4156

A.7.6.9.5.1	Test Purpose and Environment	4156

A.7.6.9.5.2	Test Requirements	4159

A.7.6.9.6	NR RSTD measurement reporting delay test case for PRS aggregation in FR2 SA in RRC_CONNECTED mode	4159

A.7.6.9.6.1	Test Purpose and Environment	4159

A.7.6.9.6.2	Test Requirements	4167

A.7.6.10 PRS-RSRP measurements	4167

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	4167

A.7.6.10.1.1	Test Purpose and Environment	4167

A.7.6.10.1.2	Test Requirements	4169

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	4170

A.7.6.10.2.1	Test Purpose and Environment	4170

A.7.6.10.2.2	Test Requirements	4172

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	4172

A.7.6.10.3.1	Test Purpose and Environment	4172

A.7.6.10.3.2	Test Requirements	4174

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	4175

A.7.6.10.4.1	Test Purpose and Environment	4175

A.7.6.10.4.2	Test Requirements	4177

A.7.6.11	UE Rx-Tx time difference measurements	4177

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	4177

A.7.6.11.1.1	Test purpose and environment	4177

A.7.6.11.1.2	Test requirements	4179

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	4179

A.7.6.11.2.1	Test purpose and environment	4179

A.7.6.11.2.2	Test requirements	4181

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	4182

A.7.6.11.3.1	Test purpose and environment	4182

A.7.6.11.3.2	Test requirements	4183

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	4184

A.7.6.11.4.1	Test purpose and environment	4184

A.7.6.11.4.2	Test requirements	4185

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	4186

A.7.6.11.5.1	Test purpose and environment	4186

A.7.6.11.5.2	Test requirements	4187

A.7.6.11.6	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR2 SA	4188

A.7.6.11.6.1	Test purpose and environment	4188

A.7.6.11.6.2	Test requirements	4191

A.7.6.12	PRS-RSRPP measurements	4191

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	4191

A.7.6.12.1.1	Test Purpose and Environment	4191

A.7.6.12.1.2	Test Requirements	4194

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	4194

A.7.6.12.2.1	Test Purpose and Environment	4194

A.7.6.12.2.2	Test Requirements	4196

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	4196

A.7.6.12.3.1	Test Purpose and Environment	4196

A.7.6.12.3.2	Test Requirements	4199

A.7.6.13	UE Rx-Tx time difference measurements for PDC	4199

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	4199

A.7.6.13.1.1	Test purpose and environment	4199

A.7.6.13.1.2	Test requirements	4200

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	4201

A.7.6.13.2.1	Test purpose and environment	4201

A.7.6.13.2.2	Test requirements	4202

A.7.6.14	SA event triggered reporting tests with Pre-MG	4202

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	4202

A.7.6.14.1.1	Test purpose and Environment	4202

A.7.6.14.1.2	Test parameters	4203

A.7.6.14.1.3	Test Requirements	4204

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	4205

A.7.6.14.2.1	Test purpose and Environment	4205

A.7.6.14.2.2	Test parameters	4205

A.7.6.14.2.3	Test Requirements	4207

A.7.6.15	SA event triggered reporting tests with concurrent gaps	4207

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	4207

A.7.6.15.1.1	Test Purpose and Environment	4207

A.7.6.15.1.2	Test Requirements	4209

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	4210

A.7.6.15.2.1	Test Purpose and Environment	4210

A.7.6.15.2.2	Test Requirements	4212

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	4212

A.7.6.15.3.1	Test Purpose and Environment	4212

A.7.6.15.3.2	Test Requirements	4215

A.7.6.16	SA event triggered reporting tests with NCSG	4215

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	4215

A.7.6.16.1.1	Test purpose and Environment	4215

A.7.6.16.1.2	Test Requirements	4218

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	4218

A.7.6.16.2.1	Test Purpose and Environment	4218

A.7.6.16.2.2	Test Requirements	4220

A.7.6.16.3	Event triggered reporting test on deactivated SCell measurement via NCSG in FR2 in non-DRX	4221

A.7.6.16.3.1	Test Purpose and Environment	4221

A.7.6.16.3.2	Test Requirements	4223

A.7.6.17	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG in FR2	4223

A.7.6.17.1	SA event triggered reporting test for FR2 with one pre-configured gap and one measurement gap	4223

A.7.6.17.1.1	Test Purpose and Environment	4223

A.7.6.17.1.2	Test Requirements	4225

A.7.6.17.2	Inter-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MGs in FR2	4226

A.7.6.17.2.1	Test purpose and Environment	4226

A.7.6.17.2.2	Test parameters	4226

A.7.6.17.2.3	Test Requirements	4228

A.7.6.18	SA event triggered reporting tests with concurrent gaps and NCSG	4229

A.7.6.18.1	SA event triggered reporting tests For FR2 with concurrent measurement gaps and NCSG without SSB time index detection when DRX is not used (PCell in FR2)	4229

A.7.6.18.1.1	Test Purpose and Environment	4229

A.7.6.18.1.2	Test Requirements	4231

A.7.6.19	SA event triggered reporting tests with NeedForGap in FR2	4232

A.7.6.19.1	SA event triggered reporting test for UE indicating NeedforInterruptionInfoNR under non-DRX and no interruption outside configured measurement gaps	4232

A.7.6.19.1.1	Test purpose and Environment	4232

A.7.6.19.1.2	Test Requirements	4234

A.7.6.19.2	SA event triggered reporting test without gap under non-DRX	4234

A.7.6.19.2.1	Test purpose and Environment	4234

A.7.6.19.2.2	Test Requirements	4236

A.7.6.19.3	SA event triggered reporting test without gap without interruption under non-DRX	4237

A.7.6.19.3.1	Test Purpose and Environment	4237

A.7.6.19.3.2	Test Requirements	4239

A.7.6.20	LTM Intra-frequency L1-RSRP measurement	4239

A.7.6.20.1	Intra-frequency SSB based L1-RSRP measurement in FR2	4239

A.7.6.20.1.1	Test Purpose and Environment	4239

A.7.6.20.1.2	Test parameters	4240

A.7.6.20.1.3	Test Requirements	4242

A.7.6.21	LTM Inter-frequency L1-RSRP measurement with measurement gap	4242

A.7.6.21.1	Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM in FR2	4242

A.7.6.21.1.1	Test Purpose and Environment	4242

A.7.6.21.1.2	Test parameters	4242

A.7.6.21.1.3	Test Requirements	4244

A.7.6.22	LTM Inter-frequency L1-RSRP measurement without measurement gap	4244

A.7.6.22.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2	4244

A.7.6.22.1.1	Test Purpose and Environment	4244

A.7.6.22.1.2	Test parameters	4245

A.7.6.22.1.3	Test Requirements	4247

A.7.6.23	Idle Mode CA/DC Measurements	4247

A.7.6.23.1	Test case for Idle mode fast CA/DC eEMR measurement for FR2 without valid reporting	4247

A.7.6.23.1.1	Test Purpose and Environment	4247

A.7.6.23.1.2	Test Requirements	4250

A.7.6.23.2	Test case for Idle mode fast CA/DC cell reselection measurement for FR2 without valid reporting	4250

A.7.6.23.2.1	Test Purpose and Environment	4251

A.7.6.23.2.2	Test Requirements	4254

A.7.6.23.3 Test case for Idle mode fast CA/DC cell reselection measurement for FR2 with valid reporting	4254

A.7.6.23.3.1	Test Purpose and Environment	4254

A.7.6.23.3.2	Test Requirements	4257

A.7.6.24	RSCPD measurements	4258

A.7.6.24.1	NR RSCPD with RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state	4258

A.7.6.24.1.1	Test Purpose and Environment	4258

A.7.6.24.1.2	Test Requirements	4265

A.7.6.25	RSCP measurements	4265

A.7.6.25.1	DL RSCP with UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	4265

A.7.6.25.1.1	Test purpose and environment	4265

A.7.6.25.1.2	Test requirements	4269

A.7.7	Measurement Performance requirements	4269

A.7.7.1	SS-RSRP	4269

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4269

A.7.7.1.1.1	Test Purpose and Environment	4269

A.7.7.1.1.2	Test parameters	4269

A.7.7.1.1.3	Test Requirements	4271

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4272

A.7.7.1.2.1	Test Purpose and Environment	4272

A.7.7.1.2.2	Test parameters	4272

A.7.7.1.2.3	Test Requirements	4274

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	4275

A.7.7.1.3.1	Test Purpose and Environment	4275

A.7.7.1.3.2	Test parameters	4275

A.7.7.1.3.3	Test Requirements	4277

A.7.7.2	SS-RSRQ	4277

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4277

A.7.7.2.1.1	Test Purpose and Environment	4277

A.7.7.2.1.2	Test Parameters	4277

A.7.7.2.1.3	Test Requirements	4279

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4279

A.7.7.2.2.1	Test Purpose and Environment	4279

A.7.7.2.2.2	Test Parameters	4279

A.7.7.2.2.3	Test Requirements	4281

A.7.7.3	SS-SINR	4281

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4281

A.7.7.3.1.1	Test Purpose and Environment	4281

A.7.7.3.1.2	Test Parameters	4281

A.7.7.3.1.3	Test Requirements	4282

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4283

A.7.7.3.2.1	Test Purpose and Environment	4283

A.7.7.3.2.2	Test Parameters	4283

A.7.7.3.2.3	Test Requirements	4284

A.7.7.4	L1-RSRP measurement for beam reporting	4284

A.7.7.4.1	SSB based L1-RSRP measurement	4284

A.7.7.4.1.1	Test Purpose and Environment	4284

A.7.7.4.1.2	Test parameters	4285

A.7.7.4.1.3	Test Requirements	4286

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4286

A.7.7.4.2.1	Test Purpose and Environment	4286

A.7.7.4.2.2	Test parameters	4287

A.7.7.4.2.3	Test Requirements	4288

A.7.7.5	CLI measurements	4288

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	4288

A.7.7.5.1.1	Test Purpose and Environment	4288

A.7.7.5.1.2	Test parameters	4289

A.7.7.5.1.3	Test Requirements	4290

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	4291

A.7.7.5.2.1	Test Purpose and Environment	4291

A.7.7.5.2.2	Test parameters	4291

A.7.7.5.2.3	Test Requirements	4292

A.7.7.6	L1-SINR measurement for beam reporting	4293

A.7.7.6.1.1	Test Purpose and Environment	4293

A.7.7.6.1.2	Test parameters	4293

A.7.7.6.1.3	Test Requirements	4294

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	4295

A.7.7.6.2.1	Test Purpose and Environment	4295

A.7.7.6.2.2	Test parameters	4295

A.7.7.6.2.3	Test Requirements	4296

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	4297

A.7.7.6.3.1	Test Purpose and Environment	4297

A.7.7.6.3.2	Test parameters	4297

A.7.7.6.3.3	Test Requirements	4298

A.7.7.7	CSI-RSRP	4299

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4299

A.7.7.7.1.1	Test Purpose and Environment	4299

A.7.7.7.1.2	Test parameters	4299

A.7.7.7.1.3	Test Requirements	4301

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4302

A.7.7.7.2.1	Test Purpose and Environment	4302

A.7.7.7.2.2	Test parameters	4302

A.7.7.7.2.3	Test Requirements	4303

A.7.7.8	CSI-RSRQ	4304

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4304

A.7.7.8.1.1	Test Purpose and Environment	4304

A.7.7.8.1.2	Test Parameters	4304

A.7.7.8.1.3	Test Requirements	4306

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4306

A.7.7.8.2.1	Test Purpose and Environment	4306

A.7.7.8.2.2	Test Parameters	4306

A.7.7.8.2.3	Test Requirements	4308

A.7.7.9	CSI-SINR	4308

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4308

A.7.7.9.1.1	Test Purpose and Environment	4308

A.7.7.9.1.2	Test Parameters	4308

A.7.7.9.1.3	Test Requirements	4310

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4310

A.7.7.9.2.1	Test Purpose and Environment	4310

A.7.7.9.2.2	Test Parameters	4310

A.7.7.9.2.3	Test Requirements	4311

A.7.7.10	RSTD measurements	4312

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	4312

A.7.7.10.1.1	Test purpose and Environment	4312

A.7.7.10.1.2	Test Requirements	4313

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	4313

A.7.7.10.2.1	Test purpose and Environment	4313

A.7.7.10.2.2	Test Requirements	4315

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	4315

A.7.7.10.3.1	Test purpose and Environment	4315

A.7.7.10.3.2	Test Requirements	4317

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	4317

A.7.7.10.4.1	Test purpose and Environment	4317

A.7.7.10.4.2	Test Requirements	4318

A.7.7.10.5	NR RSTD measurement accuracy test case for PRS aggregation in FR2 SA in RRC_CONNECTED mode	4318

A.7.7.10.5.1	Test purpose and Environment	4318

A.7.7.10.5.2	Test Requirements	4321

A.7.7.11	PRS-RSRP measurements	4321

A.7.7.11.1	SA measurement accuracy with PRS in FR2	4321

A.7.7.11.1.1	Test Purpose and Environment	4321

A.7.7.11.1.2	Test parameters	4321

A.7.7.11.1.3	Test Requirements	4323

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	4323

A.7.7.11.2.1	Test Purpose and Environment	4323

A.7.7.11.2.2	Test parameters	4323

A.7.7.11.2.3	Test Requirements	4325

A.7.7.12	UE Rx-Tx time difference measurements	4325

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	4325

A.7.7.12.1.1	Test purpose and environment	4325

A.7.7.12.1.2	Test parameters	4326

A.7.7.12.1.3	Test requirements	4327

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	4327

A.7.7.12.2.1	Test purpose and environment	4328

A.7.7.12.2.2	Test parameters	4328

A.7.7.12.2.3	Test requirements	4329

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	4329

A.7.7.12.3.1	Test purpose and environment	4329

A.7.7.12.3.2	Test parameters	4330

A.7.7.12.3.3	Test requirements	4331

A.7.7.12.4	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR2 SA	4332

A.7.7.12.4.1	Test purpose and environment	4332

A.7.7.12.4.2	Test requirements	4336

A.7.7.13	PRS-RSRPP measurements	4336

A.7.7.13.1	SA measurement accuracy with PRS in FR2	4336

A.7.7.13.1.1	Test Purpose and Environment	4336

A.7.7.13.1.2	Test parameters	4336

A.7.7.13.1.3	Test Requirements	4338

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	4338

A.7.7.13.2.1	Test Purpose and Environment	4338

A.7.7.13.2.2	Test parameters	4338

A.7.7.13.2.3	Test Requirements	4340

A.7.7.14	L1-RSRP measurement for group-based beam reporting	4340

A.7.7.14.1	SSB based L1-RSRP measurement	4340

A.7.7.14.1.1	Test Purpose and Environment	4340

A.7.7.14.1.2	Test parameters	4340

A.7.7.14.1.3	Test Requirements	4342

A.7.7.14.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4342

A.7.7.14.2.1	Test Purpose and Environment	4342

A.7.7.14.2.2	Test parameters	4342

A.7.7.14.2.3	Test Requirements	4344

A.7.7.15	LTM L1-RSRP measurement	4344

A.7.7.15.1	SSB based inter-frequency L1-RSRP measurement	4344

A.7.7.15.1.1	Test Purpose and Environment	4344

A.7.7.15.1.2	Test parameters	4345

A.7.7.15.1.3	Test Requirements	4346

A.7.7.16	RSCPD Measurements	4347

A.7.7.16.1	RSCPD with RSTD measurement accuracy in FR2 SA in RRC_CONNECTED	4347

A.7.7.16.1.1	Test purpose and environment	4347

A.7.7.16.1.2	Test parameters	4347

A.7.7.16.1.3	Test requirements	4349

A.7.7.17	RSCP with UE Rx-Tx time difference measurements	4349

A.7.7.17.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR2 SA	4349

A.7.7.17.1.1	Test purpose and environment	4349

A.7.7.17.1.2	Test parameters	4350

A.7.7.17.1.3	Test requirements	4353

A.7.8	Measurement procedure in RRC_INACTIVE	4353

A.7.8.1	RSTD measurements	4353

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	4353

A.7.8.1.1.1	Test Purpose and Environment	4353

A.7.8.1.1.2	Test Requirements	4356

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	4356

A.7.8.1.2.1	Test Purpose and Environment	4356

A.7.8.1.2.2	Test Requirements	4359

A.7.8.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR2 SA in RRC_INACTIVE state	4359

A.7.8.1.3.1	Test purpose and environment	4359

A.7.8.1.3.2	Test requirements	4363

A.7.8.1.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state with eDRX > 10.24s	4363

A.7.8.1.4.1	Test purpose and environment	4363

A.7.8.1.4.2	Test requirements	4366

A.7.8.2	PRS-RSRP measurements	4366

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	4366

A.7.8.2.1.1	Test Purpose and Environment	4366

A.7.8.2.1.2	Test Requirements	4368

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	4369

A.7.8.2.2.1	Test purpose and Environment	4369

A.7.8.2.2.2	Test Requirements	4371

A.7.8.2.3	PRS-RSRP reporting delay in RRC_INACTIVE with eDRX	4371

A.7.8.2.3.1	Test Purpose and Environment	4371

A.7.8.2.3.2	Test Requirements	4375

A.7.8.3	UE Rx-Tx time difference measurements	4375

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	4375

A.7.8.3.1.1	Test purpose and environment	4375

A.7.8.3.1.2	Test requirements	4377

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	4377

A.7.8.3.2.1	Test purpose and environment	4377

A.7.8.3.2.2	Test requirements	4379

A.7.8.3.3	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR2 SA	4379

A.7.8.3.3.1	Test purpose and environment	4379

A.7.8.3.3.2	Test requirements	4382

A.7.8.3.4	UE Rx-Tx time difference measurements for single positioning frequency layer with eDRX > 10.24s in FR2 SA	4382

A.7.8.3.4.1	Test purpose and environment	4382

A.7.8.3.4.2	Test requirements	4385

A.7.8.4	PRS-RSRPP measurements	4385

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	4385

A.7.8.4.1.1	Test Purpose and Environment	4385

A.7.8.4.1.2	Test Requirements	4388

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	4388

A.7.8.4.2.1	Test Purpose and Environment	4388

A.7.8.4.2.2	Test Requirements	4390

A.7.8.4.3	PRS-RSPP reporting delay in RRC_INACTIVE state with eDRX > 10.24s in FR2	4391

A.7.8.4.3.1	Test purpose and environment	4391

A.7.8.4.3.2	Test requirements	4394

A.7.8.5	RSCPD Measurements	4394

A.7.8.5.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	4394

A.7.8.5.1.1	Test Purpose and Environment	4394

A.7.8.5.1.2	Test Requirements	4395

A.7.8.6	RSCP Measurements	4395

A.7.8.6.1	DL RSCP with UE Rx-Tx time difference measurements in RRC_INACTIVE for single positioning frequency layer in FR2 SA	4395

A.7.8.6.1.1	Test purpose and environment	4395

A.7.8.6.1.2	Test requirements	4399

A.7.9	Measurement performance requirements in RRC_INACTIVE	4399

A.7.9.1	RSTD measurements	4399

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	4399

A.7.9.1.1.1	Test purpose and Environment	4399

A.7.9.1.1.2	Test Requirements	4401

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	4401

A.7.9.1.2.1	Test purpose and Environment	4401

A.7.9.1.2.2	Test Requirements	4403

A.7.9.2	PRS-RSRP measurements	4405

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	4405

A.7.9.2.1.1	Test Purpose and Environment	4405

A.7.9.2.1.2	Test parameters	4405

A.7.9.2.1.3	Test Requirements	4406

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	4407

A.7.9.2.2.1	Test Purpose and Environment	4407

A.7.9.2.2.2	Test parameters	4407

A.7.9.2.2.3	Test Requirements	4408

A.7.9.3	UE Rx-Tx time difference measurements	4409

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	4409

A.7.9.3.1.1	Test purpose and environment	4409

A.7.9.3.1.2	Test parameters	4409

A.7.9.3.1.3	Test requirements	4410

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	4410

A.7.9.3.2.1	Test purpose and environment	4410

A.7.9.3.2.2	Test parameters	4411

A.7.9.3.2.3	Test requirements	4412

A.7.9.3.3	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR2 SA in RRC_INACTIVE state	4412

A.7.9.3.3.1	Test purpose and environment	4412

A.7.9.3.3.2	Test requirements	4416

A.7.9.4	PRS-RSRPP measurements	4416

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	4416

A.7.9.4.1.1	Test Purpose and Environment	4416

A.7.9.4.1.2	Test parameters	4416

A.7.9.4.1.3	Test Requirements	4418

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	4418

A.7.9.4.2.1	Test Purpose and Environment	4418

A.7.9.4.2.2	Test parameters	4418

A.7.9.4.2.3	Test Requirements	4420

A.7.9.5	RSCPD Measurements	4420

A.7.9.5.1	RSCPD with RSTD measurement accuracy in FR2 SA in RRC_INACTIVE	4420

A.7.9.5.1.1	Test purpose and environment	4420

A.7.9.5.1.2	Test parameters	4420

A.7.9.5.1.3	Test requirements	4422

A.7.9.6	RSCP Measurements	4422

A.7.9.6.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR2 SA	4422

A.7.9.6.1.1	Test purpose and environment	4422

A.7.9.6.1.2	Test parameters	4423

A.7.9.6.1.3	Test requirements	4424

A.7.10	Measurement Procedure in RRC_IDLE	4424

A.7.10.1	RSTD Measurements	4424

A.7.10.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state for non-RedCap UE	4424

A.7.10.1.1.1	Test purpose and environment	4424

A.7.10.1.1.2	Test requirements	4427

A.7.10.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	4427

A.7.10.1.2.1	Test purpose and environment	4427

A.7.10.1.2.2	Test requirements	4430

A.7.10.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR2 SA in RRC_IDLE state	4430

A.7.10.1.3.1	Test purpose and environment	4430

A.7.10.1.3.2	Test requirements	4431

A.7.10.2	PRS-RSRP Measurements	4431

A.7.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE state for non-RedCap UE in FR2	4431

A.7.10.2.1.1	Test Purpose and Environment	4431

A.7.10.2.1.2	Test Requirements	4435

A.7.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR2 when eDRX cycle > 10.24s	4435

A.7.10.2.2.1	Test Purpose and Environment	4435

A.7.10.2.2.2	Test Requirements	4435

A.7.10.3	RSCPD Measurements	4436

A.7.10.3.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state	4436

A.7.10.3.1.1	Test Purpose and Environment	4436

A.7.10.3.1.2	Test Requirements	4436

A.7.11	Measurement Performance Requirements in RRC_IDLE	4437

A.7.11.1	RSTD Measurements	4437

A.7.11.1.1	NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state for non-RedCap UE	4437

A.7.11.1.1.1	Test purpose and environment	4437

A.7.11.1.1.2	Test requirements	4438

A.7.11.1.2	RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	4439

A.7.11.1.2.1	Test purpose and environment	4439

A.7.11.1.2.2	Test requirements	4440

A.7.11.1.3	NR RSTD measurement accuracy test case for PRS aggregation in FR2 SA in RRC_IDLE state	4441

A.7.11.1.3.1	Test purpose and environment	4441

A.7.11.1.3.2	Test requirements	4441

A.7.11.2	PRS-RSRP measurements	4441

A.7.11.2.1	PRS-RSRP measurement accuracy test case for non-RedCap UE in FR2 in RRC_IDLE state	4441

A.7.11.2.1.1	Test Purpose and Environment	4441

A.7.11.2.1.2	Test parameters	4441

A.7.11.2.1.3	Test Requirements	4443

A.7.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR2 for case 2 when eDRX cycle > 10.24s	4443

A.7.11.2.2.1	Test purpose and Environment	4443

A.7.11.2.2.1	Test parameters	4444

A.7.11.2.2.2	Test Requirements	4444

A.7.11.3	RSCPD measurements	4444

A.7.11.3.1	RSCPD with RSTD measurement accuracy in FR2 SA in RRC_IDLE	4444

A.7.11.3.1.1	Test purpose and environment	4444

A.7.11.3.1.2	Test parameters	4444

A.7.11.3.1.3	Test requirements	4446

A.8	E-UTRA standalone tests for NR RRM	4448

A.8.1	Void	4448

A.8.2	RRC_IDLE state mobility	4448

A.8.2.1	Inter-RAT NR Cell re-selection	4448

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	4448

A.8.2.1.1.1	Test Purpose and Environment	4448

A.8.2.1.1.2	Test Requirements	4451

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	4451

A.8.2.1.2.1	Test Purpose and Environment	4451

A.8.2.1.2.2	Test Requirements	4454

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	4455

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	4455

A.8.2.2.1.1	Test Purpose and Environment	4455

A.8.2.2.1.2	Test Requirements	4457

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	4458

A.8.2.2.2.1	Test Purpose and Environment	4458

A.8.2.2.2.2	Test Requirements	4460

A.8.3	RRC_CONNECTED state mobility	4460

A.8.3.1	Handover	4460

A.8.3.1.1	E-UTRAN - NR handover in FR1	4460

A.8.3.1.1.1	Test Purpose and Environment	4460

A.8.3.1.1.2	Test Requirements	4464

A.8.4	Measurement procedure	4464

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	4464

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	4464

A.8.4.1.1.1	Test Purpose and Environment	4464

A.8.4.1.1.2	Test Requirements	4466

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	4466

A.8.4.1.2.1	Test Purpose and Environment	4466

A.8.4.1.2.2	Test Requirements	4467

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	4467

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4467

A.8.4.2.1.1	Test Purpose and Environment	4467

A.8.4.2.1.2	Test Requirements	4470

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4470

A.8.4.2.2.1	Test Purpose and Environment	4470

A.8.4.2.2.2	Test Requirements	4473

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4474

A.8.4.2.3.1	Test Purpose and Environment	4474

A.8.4.2.3.2	Test Requirements	4477

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4477

A.8.4.2.4.1	Test Purpose and Environment	4477

A.8.4.2.4.2	Test Requirements	4480

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	4480

A.8.4.2.5.1	Test Purpose and Environment	4480

A.8.4.2.5.2	Test Requirements	4482

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	4483

A.8.4.2.6.1	Test Purpose and Environment	4483

A.8.4.2.6.2	Test Requirements	4484

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	4485

A.8.4.2.7.1	Test Purpose and Environment	4485

A.8.4.2.7.2	Test Requirements	4487

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4487

A.8.4.2.8.1	Test Purpose and Environment	4487

A.8.4.2.8.2	Test Requirements	4489

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	4489

A.8.4.2.9.1	Test Purpose and Environment	4489

A.8.4.2.9.2	Test Requirements	4492

A.8.4.3	E-UTRAN - NR Inter-RAT event-triggered without measurement gaps	4493

A.8.4.3.1	NR Inter-RAT event triggered reporting tests for FR2 without MG nor DRX	4493

A.8.4.3.1.1	Test Purpose and Environment	4493

A.8.4.3.1.2	Test Requirements	4494

A.8.4.3.2	NR Inter-RAT event triggered reporting tests for FR1 without gaps when DRX is not used	4495

A.8.4.3.2.1	Test Purpose and Environment	4495

A.8.4.3.2.2	Test Requirements	4498

A.8.5	Measurement performance	4498

A.8.5.1	SFTD accuracy	4498

A.8.5.1.1	SFTD accuracy	4498

A.8.5.1.1.1	Test Purpose	4498

A.8.5.1.1.2	Test Environment	4498

A.8.5.1.1.3	Test Requirements	4502

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	4502

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4502

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4505

A.8.5.2.1.2.1	Test Purpose and Environment	4505

A.8.5.2.1.2.2	Test Parameters	4505

A.8.5.2.1.2.3	Test Requirements	4507

A.8.5.2.2	SS-RSRQ	4507

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4507

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4510

A.8.5.2.2.2.1	Test Purpose and Environment	4510

A.8.5.2.2.2.2	Test Parameters	4510

A.8.5.2.2.2.3	Test Requirements	4512

A.8.5.2.3	SS-SINR	4512

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4512

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4515

A.8.5.2.3.2.1	Test Purpose and Environment	4515

A.8.5.2.3.2.2	Test Parameters	4516

A.8.5.2.3.2.3	Test Requirements	4517

A.9	V2X Tests	4517

A.9.1	V2X Tests in FR1	4517

A.9.1.1	Test for V2X UE Transmit Timing	4517

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	4517

A.9.1.1.1.1	Test Purpose and Environment	4517

A.9.1.1.1.2	Test requirements	4518

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	4518

A.9.1.1.2.1	Test Purpose and Environment	4518

A.9.1.1.2.2	Test requirements	4518

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4518

A.9.1.1.3.1	Test Purpose and Environment	4519

A.9.1.1.3.2	Test requirements	4520

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4520

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4520

A.9.1.2.1.1	Test Purpose and Environment	4520

A.9.1.2.1.2	Test Requirements	4522

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4522

A.9.1.2.2.1	Test Purpose and Environment	4522

A.9.1.2.2.2	Test Requirements	4524

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4524

A.9.1.2.3.1	Test Purpose and Environment	4524

A.9.1.2.3.2	Test Requirements	4526

A.9.1.2.4	Test for SyncRef UE as synchronization reference source with CCA	4526

A.9.1.2.4.1	Test Purpose and Environment	4526

A.9.1.2.4.2	Test Requirements	4527

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4528

A.9.1.3.1	 Test for GNSS configured as the highest priority	4528

A.9.1.3.1.1	Test Purpose and Environment	4528

A.9.1.3.1.2	Test Requirements	4529

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4530

A.9.1.3.2.1	Test Purpose and Environment	4530

A.9.1.3.2.2	Test Requirements	4532

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4533

A.9.1.3.3.1	Test Purpose and Environment	4533

A.9.1.3.3.2	Test Requirements	4534

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4535

A.9.1.3.4.1	Test Purpose and Environment	4535

A.9.1.3.4.2	Test Requirements	4537

A.9.1.4	Test for L1 SL-RSRP Measurement	4537

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4537

A.9.1.4.1.1	Test Purpose and Environment	4537

A.9.1.4.1.2	Test Requirements	4539

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4540

A.9.1.4.2.1	Test Purpose and Environment	4540

A.9.1.4.2.2	Test Requirements	4541

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4542

A.9.1.4.3.1	Test Purpose and Environment	4542

A.9.1.4.3.2	Test Requirements	4545

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4545

A.9.1.4.4.1	Test Purpose and Environment	4545

A.9.1.4.4.2	Test Requirements	4547

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4547

A.9.1.4.5.1	Test Purpose and Environment	4547

A.9.1.4.5.2	Test Requirements	4549

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4550

A.9.1.4.6.1	Test Purpose and Environment	4550

A.9.1.4.6.2	Test Requirements	4552

A.9.1.5	Test for Congestion Control Measurement	4552

A.9.1.5.1	Test Purpose and Environment	4552

A.9.1.5.2	Test Requirements	4555

A.9.1.6	Test for Interruption	4555

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4555

A.9.1.6.1.1	Test Purpose and Environment	4555

A.9.1.6.1.2	Test Requirements	4558

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4558

A.9.1.6.2.1	Test Purpose and Environment	4558

A.9.1.6.2.2	Test Requirements	4560

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4560

A.9.1.6.3.1	Test Purpose and Environment	4560

A.9.1.6.3.2	Test Requirements	4563

A.9.1.7	Selection / Reselection of relay UE	4563

A.9.1.7.1	Test Purpose and Environment	4563

A.9.1.7.2	Test Requirements	4567

A.9A	Tests for NR Sidelink Measurements for Positioning	4568

A.9A.1	Tests for NR Sidelink Measurements for Positioning in FR1	4568

A.9A.1.1	Measurement delay tests	4568

A.9A.1.1.1	NR SL RSTD measurement reporting delay test case in FR1 SA	4568

A.9A.1.1.1.1	Test Purpose and Environment	4568

A.9A.1.1.1.2	Test Requirements	4573

A.9A.1.1.2	SL Rx-Tx measurement delay tests	4573

A.9A.1.1.2.1	Test Purpose and Environment	4573

A.9A.1.1.2.2	Test Requirements	4576

A.9A.1.1.3	NR SL AoA measurements reporting delay test in FR1 SA	4577

A.9A.1.1.3.1	Test Purpose and Environment	4577

A.9A.1.1.3.2	Test Requirements	4580

A.9A.1.1.4	NR SL RTOA measurements reporting delay test in FR1 SA	4581

A.9A.1.1.4.1	Test Purpose and Environment	4581

A.9A.1.1.4.2	Test Requirements	4584

A.9A.1.1.5	NR SL PRS-RSRP measurement reporting delay test case in FR1 SA	4585

A.9A.1.1.5.1	Test Purpose and Environment	4585

A.9A.1.1.5.2	Test Requirements	4585

A.9A.1.1.6	NR SL PRS-RSRPP measurement reporting delay test case in FR1 SA	4585

A.9A.1.1.6.1	Test Purpose and Environment	4585

A.9A.1.1.6.2	Test Requirements	4585

A.9A.1.2	Measurement Accuracy Tests	4586

A.9A.1.2.1	NR SL RSTD measurement accuracy test case in FR1 SA	4586

A.9A.1.2.1.1	Test Purpose and Environment	4586

A.9A.1.2.1.2	Test Requirements	4589

A.9A.1.2.2	SL Rx-Tx measurement accuracy test case in FR1	4590

A.9A.1.2.2.1	Test Purpose and Environment	4590

A.9A.1.2.2.2	Test Requirements	4593

A.9A.1.2.3	NR SL PRS-RSRP measurement accuracy test case in FR1 SA	4593

A.9A.1.2.3.1	Test Purpose and Environment	4593

A.9A.1.2.3.2	Test Requirements	4594

A.9A.1.2.4	NR SL PRS-RSRPP measurement accuracy test case in FR1 SA	4594

A.9A.1.2.4.1	Test Purpose and Environment	4594

A.9A.1.2.4.2	Test Requirements	4594

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4596

A.10.1	RRC_CONNECTED state mobility	4596

A.10.1.1	RRC connection mobility control	4596

A.10.1.1.1	Random Access	4596

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4596

A.10.1.1.1.1.1	Test Purpose and Environment	4596

A.10.1.1.1.1.2	Test Requirements	4597

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4598

A.10.1.1.1.1.2.2	Random Access Response Reception	4598

A.10.1.1.1.1.2.3	No Random Access Response Reception	4598

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4599

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4599

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4599

A.10.1.1.1.2.1	Test Purpose and Environment	4599

A.10.1.1.1.2.2	Test Requirements	4600

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4601

A.10.1.1.1.2.2.2	Random Access Response Reception	4601

A.10.1.1.1.2.2.3	No Random Access Response Reception	4601

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4602

A.10.1.1.1.3.1	Test Purpose and Environment	4602

A.10.1.1.1.3.2	Test Requirements	4603

A.10.1.1.1.3.2.1	MsgA Transmission	4604

A.10.1.1.1.3.2.2	MsgB Reception	4604

A.10.1.1.1.3.2.3	No MsgB Reception	4604

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4605

A.10.1.1.1.4.1	Test Purpose and Environment	4605

A.10.1.1.1.4.2	Test Requirements	4606

A.10.1.1.1.4.2.1	MsgA Transmission	4607

A.10.1.1.1.4.2.2	MsgB Reception	4607

A.10.1.1.1.4.2.3	No MsgB Reception	4607

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4608

A.10.1.2.1	Test Purpose and Environment	4608

A.10.1.2.2	Test Requirements	4612

A.10.2	Timing	4613

A.10.2.1	UE transmit timing	4613

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4613

A.10.2.1.1.1	Test Purpose and environment	4613

A.10.2.1.1.2	Test requirements	4615

A.10.2.2	UE timing advance	4616

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4616

A.10.2.2.1.1	Test Purpose and Environment	4616

A.10.2.2.1.2	Test Parameters	4616

A.10.2.2.1.3	Test Requirements	4618

A.10.3	Signalling characteristics	4618

A.10.3.1	Radio link monitoring	4618

A.10.3.1.1	Introduction	4618

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4619

A.10.3.1.2.1	Test purpose and environment	4619

A.10.3.1.2.2	Test requirements	4621

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4622

A.10.3.1.3.1	Test purpose and environment	4622

A.10.3.1.3.2	Test requirements	4624

A.10.3.1.4	Void	4624

A.10.3.1.4.1	Void	4624

A.10.3.1.4.2	Void	4624

A.10.3.1.5	Void	4625

A.10.3.1.5.1	Void	4625

A.10.3.1.5.2	Void	4625

A.10.3.2	Void	4625

A.10.3.3	SCell activation and deactivation delay	4625

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4625

A.10.3.3.1.1	Test Purpose and Environment	4625

A.10.3.3.1.2	Test Requirements	4628

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4628

A.10.3.3.2.1	Test Purpose and Environment	4628

A.10.3.3.2.2	Test Requirements	4629

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4629

A.10.3.3.3.1	Test Purpose and Environment	4629

A.10.3.3.3.2	Test Requirements	4629

A.10.3.4	Beam failure detection and link recovery procedures	4630

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4630

A.10.3.4.1.1	Test Purpose and Environment	4630

A.10.3.4.1.2	Test Requirements	4633

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4633

A.10.3.4.2.1	Test Purpose and Environment	4633

A.10.3.4.2.2	Test Requirements	4636

A.10.3.5	Active BWP switching	4637

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4637

A.10.3.5.1.2	Test Requirements	4639

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4640

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4640

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4643

A.10.3.5.3	RRC-based Active BWP Switch	4647

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4647

A.10.3.6	PSCell addition and release delay	4649

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4649

A.10.3.6.1.1	Test purpose and environment	4649

A.10.3.6.1.2	Test Requirements	4652

A.10.3.7	Void	4652

A.10.4	Measurement procedure	4652

A.10.4.1	Intra-frequency measurements	4653

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4653

A.10.4.1.1.1	Test purpose and environment	4653

A.10.4.1.1.2	Test parameters	4653

A.10.4.1.1.3	Test Requirements	4655

A.10.4.1.2	Void	4655

A.10.4.1.3	Void	4655

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4655

A.10.4.1.4.1	Test purpose and environment	4655

A.10.4.1.4.2	Test parameters	4655

A.10.4.1.4.3	Test Requirements	4658

A.10.4.1.5	Void	4658

A.10.4.1.6	Void	4658

A.10.4.1.7	Void	4658

A.10.4.1.8	Void	4658

A.10.4.1.9	Void	4658

A.10.4.1.10	Void	4658

A.10.4.1.11	Void	4658

A.10.4.1.12	Void	4658

A.10.4.2	Inter-frequency measurements	4658

A.10.4.2.1	Void	4658

A.10.4.2.2	Void	4658

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4659

A.10.4.2.3.1	Test Purpose and Environment	4659

A.10.4.2.3.2	Test Requirements	4661

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4661

A.10.4.2.4.1	Test Purpose and Environment	4661

A.10.4.2.4.2	Test Requirements	4664

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4665

A.10.4.2.5.1	Test Purpose and Environment	4665

A.10.4.2.5.2	Test Requirements	4667

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4667

A.10.4.2.6.1	Test Purpose and Environment	4667

A.10.4.2.6.2	Test Requirements	4670

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4671

A.10.4.2.7.1	Test Purpose and Environment	4671

A.10.4.2.7.2	Test Requirements	4674

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4674

A.10.4.2.8.1	Test Purpose and Environment	4674

A.10.4.2.8.2	Test Requirements	4678

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4679

A.10.4.2.9.1	Test Purpose and Environment	4679

A.10.4.2.9.2	Test Requirements	4682

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4682

A.10.4.2.10.1	Test Purpose and Environment	4682

A.10.4.2.10.2	Test Requirements	4686

A.10.4.3	L1-RSRP measurements for beam reporting	4687

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4687

A.10.4.3.1.1	Test Purpose and Environment	4687

A.10.4.3.1.2	Test parameters	4687

A.10.4.3.1.3	Test Requirements	4688

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4689

A.10.4.3.2.1	Test Purpose and Environment	4689

A.10.4.3.2.2	Test parameters	4689

A.10.4.3.2.3	Test Requirements	4690

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4691

A.10.4.3.3.1	Test Purpose and Environment	4691

A.10.4.3.3.2	Test parameters	4691

A.10.4.3.3.3	Test Requirements	4692

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4693

A.10.4.3.4.1	Test Purpose and Environment	4693

A.10.4.3.4.2	Test parameters	4693

A.10.4.3.4.3	Test Requirements	4694

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4695

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4695

A.10.4.4.1.1	Test Purpose and Environment	4695

A.10.4.4.1.2	Test Requirements	4698

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4699

A.10.4.4.2.1	Test Purpose and Environment	4699

A.10.4.4.2.2	Test Requirements	4702

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4703

A.10.4.4.3.1	Test Purpose and Environment	4703

A.10.4.4.3.2	Test Requirements	4706

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4706

A.10.4.4.4.1	Test Purpose and Environment	4706

A.10.4.4.4.2	Test Requirements	4710

A.10.5	Measurement performance	4710

A.10.5.1	SS-RSRP	4710

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4710

A.10.5.1.1.1	Test Purpose and Environment	4711

A.10.5.1.1.2	Test parameters	4711

A.10.5.1.1.3	Test Requirements	4712

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4712

A.10.5.1.2.1	Test Purpose and Environment	4712

A.10.5.1.2.2	Test parameters	4713

A.10.5.1.2.3	Test Requirements	4714

A.10.5.2	SS-RSRQ	4714

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4714

A.10.5.2.1.1	Test Purpose and Environment	4714

A.10.5.2.1.2	Test Parameters	4714

A.10.5.2.1.3	Test Requirements	4716

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4716

A.10.5.2.2.1	Test Purpose and Environment	4716

A.10.5.2.2.2	Test Parameters	4716

A.10.5.2.2.3	Test Requirements	4718

A.10.5.3	SS-SINR	4718

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4718

A.10.5.3.1.1	Test Purpose and Environment	4718

A.10.5.3.1.2	Test Parameters	4718

A.10.5.3.1.3	Test Requirements	4719

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4720

A.10.5.3.2.1	Test Purpose and Environment	4720

A.10.5.3.2.2	Test Parameters	4720

A.10.5.3.2.3	Test Requirements	4721

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4721

A.10.5.3.3.1	Test Purpose and Environment	4721

A.10.5.3.3.2	Test Parameters	4721

A.10.5.3.3.3	Test Requirements	4723

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4723

A.10.5.4.1	SSB based L1-RSRP measurement	4723

A.10.5.4.1.1	Test Purpose and Environment	4723

A.10.5.4.1.2	Test parameters	4724

A.10.5.4.1.3	Test Requirements	4725

A.10.5.5	RSSI	4725

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4725

A.10.5.5.1.1	Test Purpose and Environment	4725

A.10.5.5.1.2	Test parameters	4725

A.10.5.5.1.3	Test Requirements	4727

A.10.5.5.2	RSSI measurement accuracy on SCC with CCA	4727

A.10.5.5.2.1	Test Purpose and Environment	4727

A.10.5.5.2.2	Test parameters	4727

A.10.5.5.2.3	Test Requirements	4728

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4728

A.10.5.5.3.1	Test Purpose and Environment	4728

A.10.5.5.3.2	Test parameters	4729

A.10.5.5.3.3	Test Requirements	4730

A.10.5.6	Channel occupancy	4730

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4730

A.10.5.6.1.1	Test Purpose and Environment	4730

A.10.5.6.1.2	Test parameters	4730

A.10.5.6.1.3	Test Requirements	4732

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4732

A.10.5.6.2.1	Test Purpose and Environment	4732

A.10.5.6.2.2	Test parameters	4732

A.10.5.6.2.3	Test Requirements	4734

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4734

A.10.5.6.3.1	Test Purpose and Environment	4734

A.10.5.6.3.2	Test parameters	4734

A.10.5.6.3.3	Test Requirements	4736

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4738

A.11.1	RRC_IDLE state mobility	4738

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4738

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4738

A.11.1.1.1.1	Test Purpose and Environment	4738

A.11.1.1.1.2	Test Parameters	4738

A.11.1.1.1.3	Test Requirements	4740

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4741

A.11.1.1.2.1	Test Purpose and Environment	4741

A.11.1.1.2.2	Test Parameters	4741

A.11.1.1.2.3	Test Requirements	4743

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4743

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4743

A.11.1.2.1.1	Test Purpose and Environment	4743

A.11.1.2.1.2	Test Parameters	4744

A.11.1.2.1.3	Test Requirements	4746

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4747

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4747

A.11.1.3.1.1	Test Purpose and Environment	4747

A.11.1.3.1.2	Test Parameters	4747

A.11.1.3.1.3	Test Requirements	4750

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4751

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4751

A.11.1.4.1.1	Test Purpose and Environment	4751

A.11.1.4.1.2	Test Parameters	4751

A.11.1.4.1.3	Test Requirements	4754

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4754

A.11.1.4.2.1	Test Purpose and Environment	4754

A.11.1.4.2.2	Test Requirements	4756

A.11.2	RRC_CONNECTED state mobility	4757

A.11.2.1	Handover	4757

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4757

A.11.2.1.1.1	Test Purpose and Environment	4757

A.11.2.1.1.2	Test Parameters	4757

A.11.2.1.1.3 Test Requirements	4759

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4760

A.11.2.1.2.1	Test Purpose and Environment	4760

A.11.2.1.2.2	Test Parameters	4760

A.11.2.1.2.3	Test Requirements	4762

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4762

A.11.2.1.3.1	Test Purpose and Environment	4762

A.11.2.1.3.2	Test Parameters	4762

A.11.2.1.3.3	Test Requirements	4764

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4765

A.11.2.1.4.1	Test Purpose and Environment	4765

A.11.2.1.4.2	Test Parameters	4765

A.11.2.1.4.3	Test Requirements	4768

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4768

A.11.2.1.5.1	Test Purpose and Environment	4768

A.11.2.1.5.2	Test Parameters	4768

A.11.2.1.5.3 Test Requirements	4771

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4771

A.11.2.1.6.1	Test Purpose and Environment	4771

A.11.2.1.6.2	Test Parameters	4771

A.11.2.1.6.3	Test Requirements	4774

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4774

A.11.2.1.7.1	Test Purpose and Environment	4774

A.11.2.1.7.2	Test Requirements	4777

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4778

A.11.2.1.8.1	Test Purpose and Environment	4778

A.11.2.1.8.2	Test Requirements	4781

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4781

A.11.2.1.9.1	Test Purpose and Environment	4781

A.11.2.1.9.2	Test Requirements	4787

A.11.2.2	RRC connection mobility control	4788

A.11.2.2.1	RRC re-establishment	4788

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4788

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4791

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4797

A.11.2.2.2	Random Access	4800

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4800

A.11.2.2.2.1.1	Test Purpose and Environment	4800

A.11.2.2.2.1.2	Test Requirements	4801

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4801

A.11.2.2.2.1.2.2	Random Access Response Reception	4802

A.11.2.2.2.1.2.3	No Random Access Response Reception	4802

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4802

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4802

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4803

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4803

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4803

A.11.2.2.2.2.1	Test Purpose and Environment	4803

A.11.2.2.2.2.2	Test Requirements	4804

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4805

A.11.2.2.2.2.2.2	Random Access Response Reception	4805

A.11.2.2.2.2.2.3	No Random Access Response Reception	4805

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4806

A.11.2.2.2.3.1	Test Purpose and Environment	4806

A.11.2.2.2.3.2	Test Requirements	4807

A.11.2.2.2.3.2.1	MsgA Transmission	4807

A.11.2.2.2.3.2.2	MsgB Reception	4808

A.11.2.2.2.3.2.3	No MsgB Reception	4808

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4809

A.11.2.2.2.4.1	Test Purpose and Environment	4809

A.11.2.2.2.4.2	Test Requirements	4810

A.11.2.2.2.4.2.1	MsgA Transmission	4810

A.11.2.2.2.4.2.2	MsgB Reception	4811

A.11.2.2.2.4.2.3	No MsgB Reception	4811

A.11.2.2.3	RRC connection release with redirection	4812

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4812

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4814

A.11.3	Timing	4817

A.11.3.1	UE transmit timing	4817

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4817

A.11.3.1.1.1	Test Purpose and environment	4817

A.11.3.1.1.2	Test requirements	4819

A.11.3.2	UE timing advance	4820

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4820

A.11.3.2.1.1	Test Purpose and Environment	4820

A.11.3.2.1.2	Test Parameters	4820

A.11.3.2.1.3	Test Requirements	4822

A.11.4	Signalling characteristics	4822

A.11.4.1	Radio link monitoring	4822

A.11.4.1.1	Introduction	4822

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4823

A.11.4.1.2.1	Test purpose and environment	4823

A.11.4.1.2.2	Test requirements	4825

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4826

A.11.4.1.3.1	Test purpose and environment	4826

A.11.4.1.3.2	Test requirements	4829

A.11.4.1.4	Void	4829

A.11.4.1.4.1	Void	4829

A.11.4.1.4.2	Void	4829

A.11.4.1.5	Void	4829

A.11.4.1.5.1	Void	4829

A.11.4.1.5.2	Void	4829

A.11.4.2	Void	4829

A.11.4.3	SCell activation and deactivation delay	4829

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4829

A.11.4.3.1.1	Test Purpose and Environment	4829

A.11.4.3.1.2	Test Requirements	4832

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4832

A.11.4.3.2.1	Test Purpose and Environment	4832

A.11.4.3.2.2	Test Requirements	4833

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4833

A.11.4.3.3.1	Test Purpose and Environment	4833

A.11.4.3.3.2	Test Requirements	4833

A.11.4.4	Beam failure detection and link recovery procedures	4834

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4834

A.11.4.4.1.1	Test Purpose and Environment	4834

A.11.4.4.1.2	Test Requirements	4837

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4838

A.11.4.4.2.1	Test Purpose and Environment	4838

A.11.4.4.2.2	Test Requirements	4841

A.11.4.5	Active BWP switching	4841

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4841

A.11.4.5.1.1	Test Purpose and Environment	4841

A.11.4.5.1.2	Test Requirements	4844

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4844

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4844

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4847

A.11.4.5.3	RRC-based Active BWP Switch	4850

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4850

A.11.4.6	Void	4852

A.11.5	Measurement procedure	4852

A.11.5.1	Intra-frequency measurements	4852

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4852

A.11.5.1.1.1	Test purpose and environment	4852

A.11.5.1.1.2	Test parameters	4852

A.11.5.1.1.3	Test Requirements	4854

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4854

A.11.5.1.2.1	Test purpose and environment	4854

A.11.5.1.2.2	Test parameters	4854

A.11.5.1.2.3	Test Requirements	4856

A.11.5.1.3	Void	4857

A.11.5.1.4	Void	4857

A.11.5.1.5	Void	4857

A.11.5.1.6	Void	4857

A.11.5.1.7	Void	4857

A.11.5.1.8	Void	4857

A.11.5.1.9	Void	4857

A.11.5.1.10	Void	4857

A.11.5.1.11	Void	4857

A.11.5.1.12	Void	4857

A.11.5.2	Inter-frequency measurements	4857

A.11.5.2.1	Void	4857

A.11.5.2.2	Void	4857

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4857

A.11.5.2.3.1	Test Purpose and Environment	4857

A.11.5.2.3.2	Test Requirements	4860

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4860

A.11.5.2.4.1	Test Purpose and Environment	4860

A.11.5.2.4.2	Test Requirements	4863

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4863

A.11.5.2.5.1	Test Purpose and Environment	4863

A.11.5.2.5.2	Test Requirements	4866

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4866

A.11.5.2.6.1	Test Purpose and Environment	4866

A.11.5.2.6.2	Test Requirements	4869

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4870

A.11.5.2.7.1	Test Purpose and Environment	4870

A.11.5.2.7.2	Test Requirements	4872

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4873

A.11.5.2.8.1	Test Purpose and Environment	4873

A.11.5.2.8.2	Test Requirements	4876

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4876

A.11.5.2.9.1	Test Purpose and Environment	4876

A.11.5.2.9.2	Test Requirements	4879

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4879

A.11.5.2.10.1	Test Purpose and Environment	4879

A.11.5.2.10.2	Test Requirements	4883

A.11.5.3	Inter-RAT E-UTRAN measurements	4883

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4883

A.11.5.3.1.1	Test Purpose and Environment	4883

A.11.5.3.1.2	Test Requirements	4886

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4886

A.11.5.3.2.1	Test Purpose and Environment	4886

A.11.5.3.2.2	Test Requirements	4890

A.11.5.4	L1-RSRP measurements for beam reporting	4890

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4890

A.11.5.4.1.1	Test Purpose and Environment	4890

A.11.5.4.1.2	Test parameters	4890

A.11.5.4.1.3	Test Requirements	4892

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4892

A.11.5.4.2.1	Test Purpose and Environment	4892

A.11.5.4.2.2	Test parameters	4892

A.11.5.4.2.3	Test Requirements	4894

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4894

A.11.5.4.3.1	Test Purpose and Environment	4894

A.11.5.4.3.2	Test parameters	4895

A.11.5.4.3.3	Test Requirements	4896

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4896

A.11.5.4.4.1	Test Purpose and Environment	4896

A.11.5.4.4.2	Test parameters	4897

A.11.5.4.4.3	Test Requirements	4898

A.11.6	Measurement performance	4898

A.11.6.1	SS-RSRP	4898

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4898

A.11.6.1.1.1	Test Purpose and Environment	4898

A.11.6.1.1.2	Test parameters	4899

A.11.6.1.1.3	Test Requirements	4900

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4900

A.11.6.1.2.1	Test Purpose and Environment	4900

A.11.6.1.2.2	Test parameters	4900

A.11.6.1.2.3	Test Requirements	4902

A.11.6.2	SS-RSRQ	4902

A.11.6.2.1	Intra-frequency measurement accuracy	4902

A.11.6.2.1.1	Test Purpose and Environment	4902

A.11.6.2.1.2	Test Parameters	4902

A.11.6.2.1.3	Test Requirements	4904

A.11.6.2.2	Inter-frequency measurement accuracy	4904

A.11.6.2.2.1	Test Purpose and Environment	4904

A.11.6.2.2.2	Test Parameters	4904

A.11.6.2.2.3	Test Requirements	4907

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4907

A.11.6.2.3.1	Test Purpose and Environment	4907

A.11.6.2.3.2	Test Parameters	4907

A.11.6.2.3.3	Test Requirements	4909

A.11.6.2.4	Inter-frequency measurement accuracy	4909

A.11.6.2.4.1	Test Purpose and Environment	4909

A.11.6.2.4.2	Test Parameters	4909

A.11.6.2.4.3	Test Requirements	4916

A.11.6.3	SS-SINR	4916

A.11.6.3.1	Intra-frequency measurement accuracy	4916

A.11.6.3.1.1	Test Purpose and Environment	4916

A.11.6.3.1.2	Test Parameters	4916

A.11.6.3.1.3	Test Requirements	4918

A.11.6.3.2	Inter-frequency measurement accuracy	4918

A.11.6.3.2.1	Test Purpose and Environment	4918

A.11.6.3.2.2	Test Parameters	4918

A.11.6.3.2.3	Test Requirements	4919

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4919

A.11.6.3.3.1	Test Purpose and Environment	4919

A.11.6.3.3.2	Test Parameters	4920

A.11.6.3.3.3	Test Requirements	4921

A.11.6.3.4	Inter-frequency measurement accuracy	4921

A.11.6.3.4.1	Test Purpose and Environment	4921

A.11.6.3.4.2	Test Parameters	4921

A.11.6.3.4.3	Test Requirements	4925

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4926

A.11.6.4.1	SSB based L1-RSRP measurement	4926

A.11.6.4.1.1	Test Purpose and Environment	4926

A.11.6.4.1.2	Test parameters	4926

A.11.6.4.1.3	Test Requirements	4927

A.11.6.5	RSSI	4927

A.11.6.5.1	Intra-frequency RSSI measurement accuracy on PCC with CCA	4927

A.11.6.5.1.1	Test Purpose and Environment	4927

A.11.6.5.1.2	Test parameters	4927

A.11.6.5.1.3	Test Requirements	4929

A.11.6.5.2	Intra-frequency RSSI measurement accuracy on SCC with CCA	4929

A.11.6.5.2.1	Test Purpose and Environment	4929

A.11.6.5.2.2	Test parameters	4929

A.11.6.5.2.3	Test Requirements	4931

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4931

A.11.6.5.3.1	Test Purpose and Environment	4931

A.11.6.5.3.2	Test parameters	4931

A.11.6.5.3.3	Test Requirements	4932

A.11.6.6	Channel occupancy	4933

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4933

A.11.6.6.1.1	Test Purpose and Environment	4933

A.11.6.6.1.2	Test parameters	4933

A.11.6.6.1.3	Test Requirements	4934

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4934

A.11.6.6.2.1	Test Purpose and Environment	4934

A.11.6.6.2.2	Test parameters	4934

A.11.6.6.2.3	Test Requirements	4936

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4936

A.11.6.6.3.1	Test Purpose and Environment	4936

A.11.6.6.3.2	Test parameters	4936

A.11.6.6.3.3	Test Requirements	4938

A.11.6.7	E-UTRAN RSRP	4938

A.11.6.8	E-UTRAN RSRQ	4938

A.11.6.9	E-UTRAN SINR	4938

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4938

A.12.1	RRC_IDLE state mobility	4938

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4938

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4938

A.12.1.1.1.1	Test Purpose and Environment	4938

A.12.1.1.1.2	Test Requirements	4941

A.12.2	RRC_CONNECTED state mobility	4942

A.12.2.1	Handover	4942

A.12.2.1.1	E-UTRAN - NR with CCA handover	4942

A.12.2.1.1.1	Test Purpose and Environment	4942

A.12.2.1.1.2	Test Requirements	4945

A.12.3	Void	4946

A.12.4	Measurement procedure	4946

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4946

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4946

A.12.4.1.1.1	Test Purpose and Environment	4946

A.12.4.1.1.2	Test Requirements	4948

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4948

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4948

A.12.4.2.1.1	Test Purpose and Environment	4948

A.12.4.2.1.2	Test Requirements	4952

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4952

A.12.4.2.2.1	Test Purpose and Environment	4952

A.12.4.2.2.2	Test Requirements	4955

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4955

A.12.4.2.3.1	Test Purpose and Environment	4955

A.12.4.2.3.2	Test Requirements	4959

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4959

A.12.4.2.4.1	Test Purpose and Environment	4959

A.12.4.2.4.2	Test Requirements	4962

A.12.4.2.5	Void	4963

A.12.4.2.6	Void	4963

A.12.5	Measurement performance	4963

A.12.5.1	E-UTRANNR SFTD	4963

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4963

A.12.5.1.1.1	Test Purpose	4963

A.12.5.1.1.2	Test Environment	4963

A.12.5.1.1.3	Test Requirements	4965

A.12.5.2	Void	4965

A.12.5.3	Void	4965

A.12.5.4	Void	4965

A.12.5.5	Void	4966

A.12.5.6	Void	4966

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4969

A.13.1	Void	4969

A.13.1.1	Void	4969

A.13.1.2	Void	4969

A.13.2	Signalling characteristics	4969

A.13.2.1	Void	4969

A.13.2.2	SCell activation and deactivation delay	4969

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4969

A.13.2.2.1.1	Test Purpose and Environment	4969

A.13.2.2.1.2	Test Requirements	4972

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4972

A.13.2.2.2.1	Test Purpose and Environment	4972

A.13.2.2.2.2	Test Requirements	4973

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4973

A.13.2.2.3.1	Test Purpose and Environment	4973

A.13.2.2.3.2	Test Requirements	4973

A.13.2.3	Void	4974

A.13.3	Measurement procedure	4974

A.13.3.1	Intra-frequency measurements	4974

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4974

A.13.3.1.1.1	Test purpose and environment	4974

A.13.3.1.1.2	Test parameters	4974

A.13.3.1.1.3	Test Requirements	4977

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4977

A.13.3.1.2.1	Test purpose and environment	4977

A.13.3.1.2.2	Test parameters	4977

A.13.3.1.2.3	Test Requirements	4980

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4980

A.13.3.1.3.1	Test purpose and environment	4980

A.13.3.1.3.2	Test parameters	4980

A.13.3.1.3.3	Test Requirements	4983

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4984

A.13.3.1.4.1	Test purpose and environment	4984

A.13.3.1.4.2	Test parameters	4984

A.13.3.1.4.3	Test Requirements	4987

A.13.3.1.5	Void	4987

A.13.3.1.6	Void	4987

A.13.3.2	Inter-frequency measurements	4987

A.13.3.2.1	Void	4987

A.13.3.2.2	Void	4987

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4987

A.13.3.2.3.1	Test Purpose and Environment	4987

A.13.3.2.3.2	Test Requirements	4990

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4991

A.13.3.2.4.1	Test Purpose and Environment	4991

A.13.3.2.4.2	Test Requirements	4994

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4994

A.13.3.2.5.1	Test Purpose and Environment	4994

A.13.3.2.5.2	Test Requirements	4997

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4997

A.13.3.2.6.1	Test Purpose and Environment	4997

A.13.3.2.6.2	Test Requirements	5001

A.13.3.3	L1-RSRP measurements for beam reporting	5001

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	5001

A.13.3.3.1.1	Test Purpose and Environment	5001

A.13.3.3.1.2	Test parameters	5001

A.13.3.3.1.3	Test Requirements	5003

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	5004

A.13.3.3.2.1	Test Purpose and Environment	5004

A.13.3.3.2.2	Test parameters	5004

A.13.3.3.2.3	Test Requirements	5006

A.13.4	Measurement performance	5006

A.13.4.1	SS-RSRP	5006

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	5006

A.13.4.1.1.1	Test Purpose and Environment	5006

A.13.4.1.1.2	Test parameters	5006

A.13.4.1.1.3	Test Requirements	5008

A.13.4.2	SS-RSRQ	5008

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	5008

A.13.4.2.1.1	Test Purpose and Environment	5008

A.13.4.2.1.2	Test Parameters	5008

A.13.4.2.1.3	Test Requirements	5013

A.13.4.3	SS-SINR	5013

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	5013

A.13.4.3.1.1	Test Purpose and Environment	5013

A.13.4.3.1.2	Test Parameters	5013

A.13.4.3.1.3	Test Requirements	5017

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	5017

A.13.4.4.1	SSB based L1-RSRP measurement	5017

A.13.4.4.1.1	Test Purpose and Environment	5017

A.13.4.4.1.2	Test parameters	5018

A.13.4.4.1.3	Test Requirements	5020

A.13.4.5	RSSI	5020

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	5020

A.13.4.5.1.1	Test Purpose and Environment	5020

A.13.4.5.1.2	Test parameters	5020

A.13.4.5.1.3	Test Requirements	5022

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	5022

A.13.4.5.2.1	Test Purpose and Environment	5022

A.13.4.5.2.2	Test parameters	5022

A.13.4.5.2.3	Test Requirements	5024

A.13.4.6	Channel occupancy	5024

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	5024

A.13.4.6.1.1	Test Purpose and Environment	5024

A.13.4.6.1.2	Test parameters	5024

A.13.4.6.1.3	Test Requirements	5026

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	5026

A.13.4.6.2.1	Test Purpose and Environment	5026

A.13.4.6.2.2	Test parameters	5026

A.13.4.6.2.3	Test Requirements	5028

A.14	NR standalone tests for Satellite access	5028

A.14.1	RRC_IDLE state mobility	5028

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	5028

A.14.1.1.1	Test Purpose and Environment	5028

A.14.1.1.2	Test Parameters	5028

A.14.1.1.3	Test Requirements	5030

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	5030

A.14.1.2.1	Test Purpose and Environment	5030

A.14.1.2.2	Test Parameters	5031

A.14.1.2.3	Test Requirements	5032

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	5033

A.14.1.3.1	Test Purpose and Environment	5033

A.14.1.3.2	Test Parameters	5033

A.14.1.3.3	Test Requirements	5034

A.14.1.4	Location-based measurement initiation to FR1 intra-frequency NR cell reselection	5034

A.14.1.4.1	Test Purpose and Environment	5035

A.14.1.4.2	Test Parameters	5035

A.14.1.4.3	Test Requirements	5036

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	5036

A.14.1.5.1	Test Purpose and Environment	5036

A.14.1.5.2	Test Parameters	5037

A.14.1.5.3	Test Requirements	5038

A.14.1.6	Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements	5039

A.14.1.6.1	Test Purpose and Environment	5039

A.14.1.6.2	Test Parameters	5039

A.14.1.6.3	Test Requirements	5040

A.14.1.7	Time-based measurement initiation to FR1 inter-frequency cell reselection	5041

A.14.1.7.1	Test Purpose and Environment	5041

A.14.1.7.2	Test Parameters	5041

A.14.1.7.3	Test Requirements	5042

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	5043

A.14.1.8.1	Test Purpose and Environment	5043

A.14.1.8.2	Test Parameters	5043

A.14.1.8.3	Test Requirements	5044

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5044

A.14.1.9.1	Test Purpose and Environment	5044

A.14.1.9.2	Test Parameters	5045

A.14.1.9.3	Test Requirements	5046

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5047

A.14.1.10.1	Test Purpose and Environment	5047

A.14.1.10.2	Test Parameters	5047

A.14.1.10.3	Test Requirements	5049

A.14.1.11	Cell reselection to FR1 inter-RAT for NR NTN carrier	5049

A.14.1.11.1	Test purpose and Environment	5049

A.14.1.11.2	Test parameters	5049

A.14.1.11.3	Test requirements	5051

A.14.1.12	Cell re-selection to FR1 inter-frequency NR case with TN carrier	5052

A.14.1.12.1	Test purpose and Environment	5052

A.14.1.12.2	Test parameters	5052

A.14.1.12.3	Test requirements	5054

A.14.2	RRC_CONNECTED state mobility	5054

A.14.2.1	Handover	5054

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	5054

A.14.2.1.1.1	Test Purpose and Environment	5054

A.14.2.1.1.2	Test Parameters	5054

A.14.2.1.1.3	Test Requirements	5056

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	5056

A.14.2.1.2.1	Test Purpose and Environment	5056

A.14.2.1.2.2	Test Parameters	5056

A.14.2.1.2.3	Test Requirements	5058

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	5058

A.14.2.1.3.1	Test Purpose and Environment	5058

A.14.2.1.3.2	Test Parameters	5059

A.14.2.1.3.3	Test Requirements	5060

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	5061

A.14.2.1.4.1	Test Purpose and Environment	5061

A.14.2.1.4.2	Test Parameters	5061

A.14.2.1.4.3	Test Requirements	5062

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	5063

A.14.2.1.5.1	Test Purpose and Environment	5063

A.14.2.1.5.2	Test Parameters	5063

A.14.2.1.5.3	Test Requirements	5065

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	5065

A.14.2.1.6.1	Test Purpose and Environment	5065

A.14.2.1.6.2	Test Parameters	5065

A.14.2.1.6.3	Test Requirements	5067

A.14.2.1.7	Intra-frequency intra-satellite Handover from FR2-NTN to FR2-NTN	5068

A.14.2.1.7.1	Test Purpose and Environment	5068

A.14.2.1.7.2	Test Parameters	5068

A.14.2.1.7.3	Test Requirements	5070

A.14.2.1.8	Intra-frequency SAN Handover from FR1 to FR1	5070

A.14.2.1.8.1	Test Purpose and Environment	5070

A.14.2.1.8.2	Test Parameters	5070

A.14.2.1.8.3	Test Requirements	5072

A.14.2.1.9	Intra-frequency inter-satellite handover from FR2-NTN to FR2-NTN	5072

A.14.2.1.9.1	Test Purpose and Environment	5072

A.14.2.1.9.2	Test Parameters	5072

A.14.2.1.9.3	Test Requirements	5074

A.14.2.2	RRC Connection Mobility Control	5074

A.14.2.2.1	SA: RRC Re-establishment for SAN	5074

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	5074

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	5076

A.14.2.2.2	Random Access	5079

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	5079

A.14.2.2.2.1.1	Test Purpose and Environment	5079

A.14.2.2.2.1.2	Test Requirements	5080

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	5081

A.14.2.2.2.2.1	Test Purpose and Environment	5082

A.14.2.2.2.2.2	Test Requirements	5083

A.14.2.2.3	RRC Connection Release with Redirection	5084

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	5084

A.14.2.2.3.1.1	Test Purpose and Environment	5084

A.14.2.2.3.1.2	Test Parameters	5084

A.14.2.2.3.1.3	Test Requirements	5086

A.14.2.2.4	RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1	5087

A.14.2.2.4.1	Test Purpose and Environment	5087

A.14.2.2.4.2	Test Parameters	5087

A.14.2.2.4.3	Test Requirements	5089

A.14.2.2.5	RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1	5089

A.14.2.2.5.1	Test Purpose and Environment	5089

A.14.2.2.5.2	Test Parameters	5089

A.14.2.2.5.3	Test Requirements	5091

A.14.2.3	Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1	5091

A.14.2.3.1	Test Purpose and Environment	5091

A.14.2.3.2	Test Parameters	5091

A.14.2.3.3	Test Requirements	5093

A.14.2.4	Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1	5093

A.14.2.4.1	Test Purpose and Environment	5093

A.14.2.4.2	Test Parameters	5094

A.14.2.4.3	Test Requirements	5095

A.14.3	Timing for Satellite Access	5095

A.14.3.1	UE transmit timing for Satellite Access	5095

A.14.3.1.1	NR UE Transmit Timing Test for FR1	5095

A.14.3.1.1.1	Test Purpose and environment	5096

A.14.3.1.1.2	Test requirements	5097

A.14.3.1.2	NR UE Transmit Timing Test for FR2-NTN	5098

A.14.3.1.2.1	Test Purpose and environment	5098

A.14.3.1.2.2	Test requirements	5100

A.14.3.2	Timing advance for satellite access	5101

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	5101

A.14.3.2.1.1	Test Purpose and Environment	5101

A.14.3.2.1.2	Test Parameters	5101

A.14.3.2.1.3	Test Requirements	5103

A.14.3.2.3	SA FR2-NTN timing advance adjustment accuracy	5103

A.14.3.2.3.1	Test Purpose and Environment	5103

A.14.3.2.3.2	Test Parameters	5103

A.14.3.2.1.3	Test Requirements	5106

A.14.4	Signalling characteristics	5106

A.14.4.1	Radio link Monitoring	5106

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	5106

A.14.4.1.1.1	Test Purpose and Environment	5106

A.14.4.1.1.2	Test Requirements	5109

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	5109

A.14.4.1.2.1	Test Purpose and Environment	5109

A.14.4.1.2.2	Test Requirements	5111

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	5111

A.14.4.1.3.1	Test Purpose and Environment	5111

A.14.4.1.3.2	Test Requirements	5114

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	5114

A.14.4.1.4.1	Test Purpose and Environment	5114

A.14.4.1.4.2	Test Requirements	5116

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	5116

A.14.4.1.5.1	Test Purpose and Environment	5116

A.14.4.1.5.2	Test Requirements	5119

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	5119

A.14.4.1.6.1	Test Purpose and Environment	5119

A.14.4.1.6.2	Test Requirements	5122

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	5122

A.14.4.1.7.1	Test Purpose and Environment	5122

A.14.4.1.7.2	Test Requirements	5125

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	5125

A.14.4.1.8.1	Test Purpose and Environment	5125

A.14.4.1.8.2	Test Requirements	5128

A.14.4.1.9	Radio Link Monitoring Out-of-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode	5128

A.14.4.1.9.1	Test Purpose and Environment	5128

A.14.4.1.9.2	Test Requirements	5130

A.14.4.1.10	Radio Link Monitoring In-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode	5131

A.14.4.1.10.1	Test Purpose and Environment	5131

A.14.4.1.10.2	Test Requirements	5133

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	5134

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	5134

A.14.4.2.1.1	Test Purpose and Environment	5134

A.14.4.2.1.2	Test Requirements	5136

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	5137

A.14.4.2.2.1	Test Purpose and Environment	5137

A.14.4.2.2.2	Test Requirements	5139

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	5140

A.14.4.2.3.1	Test Purpose and Environment	5140

A.14.4.2.3.2	Test Requirements	5142

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	5143

A.14.4.2.4.1	Test Purpose and Environment	5143

A.14.4.2.4.2	Test Requirements	5145

A.14.4.2.5	Void	5146

A.14.4.2.6	Void	5146

A.14.4.3	Active BWP switch for satellite access	5146

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	5146

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	5146

A.14.4.3.1.1.1	Test Purpose and Environment	5146

A.14.4.3.1.1.2	Test Requirements	5148

A.14.4.3.2	RRC-based Active BWP Switch	5148

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	5148

A.14.4.3.2.1.1	Test Purpose and Environment	5148

A.14.4.3.2.1.2	Test Requirements	5150

A.14.4.4	UE specific CBW change for satellite access	5150

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	5150

A.14.4.4.1.1	Test Purpose and Environment	5150

A.14.4.4.1.2	Test Requirements	5152

A.14.4.5	Pathloss reference signal switching delay	5153

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	5153

A.14.4.5.1.1	Test Purpose and Environment	5153

A.14.4.5.1.2	Test Requirements	5155

A.14.5	Measurement procedure	5155

A.14.5.1	Intra-frequency Measurements	5155

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	5155

A.14.5.1.1.1	Test purpose and Environment	5155

A.14.5.1.1.2	Test parameters	5155

A.14.5.1.1.3	Test Requirements	5157

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	5157

A.14.5.1.2.1	Test purpose and Environment	5157

A.14.5.1.2.2	Test parameters	5157

A.14.5.1.2.3	Test Requirements	5159

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	5159

A.14.5.1.3.1	Test purpose and Environment	5159

A.14.5.1.3.2	Test parameters	5159

A.14.5.1.3.3	Test Requirements	5161

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	5161

A.14.5.1.4.1	Test purpose and Environment	5161

A.14.5.1.4.2	Test parameters	5161

A.14.5.1.4.3	Test Requirements	5163

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	5163

A.14.5.1.5.1	Test purpose and Environment	5163

A.14.5.1.5.2	Test parameters	5163

A.15.5.1.5.3	Test Requirements	5165

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	5165

A.14.5.1.6.1	Test purpose and Environment	5165

A.14.5.1.6.2	Test parameters	5165

A.14.5.1.6.3	Test Requirements	5167

A.14.5.1.7	SA event triggered reporting test with SSB time index reading without gap under non-DRX for FR2-NTN	5167

A.14.5.1.7.1	Test purpose and Environment	5167

A.14.5.1.7.2	Test parameters	5167

A.14.5.1.7.3	Test Requirements	5169

A.14.5.2	Inter-frequency Measurements	5169

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	5169

A.14.5.2.1.1	Test Purpose and Environment	5169

A.14.5.2.1.2	Test Requirements	5171

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	5172

A.14.5.2.2.1	Test Purpose and Environment	5172

A.14.5.2.2.2	Test Requirements	5174

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	5174

A.14.5.2.3.1	Test Purpose and Environment	5174

A.14.5.2.3.2	Test Requirements	5176

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	5177

A.14.5.2.4.1	Test Purpose and Environment	5177

A.14.5.2.4.2	Test Requirements	5179

A.14.5.2.5	void	5179

A.14.5.2.5.1	void	5179

A.14.5.2.5.2	void	5179

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	5179

A.14.5.2.6.1	Test Purpose and Environment	5179

A.14.5.2.6.2	Test Requirements	5181

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	5181

A.14.5.2.7.1	Test purpose and Environment	5181

A.14.5.2.7.2	Test parameters	5182

A.14.5.2.7.3	Test Requirements	5183

A.14.5.2.8	Event triggered reporting tests without gap under DRX	5183

A.14.5.2.8.1	Test purpose and Environment	5183

A.14.5.2.8.2	Test parameters	5183

A.14.5.2.8.3	Test Requirements	5185

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	5185

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	5185

A.14.5.3.1.1	Test Purpose and Environment	5185

A.14.5.3.1.2	Test parameters	5185

A.14.5.3.1.3	Test Requirements	5187

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	5187

A.14.5.3.2.1	Test Purpose and Environment	5187

A.14.5.3.2.2	Test parameters	5187

A.14.5.3.2.3	Test Requirements	5189

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	5189

A.14.5.3.3.1	Test Purpose and Environment	5189

A.14.5.3.3.2	Test parameters	5189

A.14.5.3.3.3	Test Requirements	5191

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	5191

A.14.5.3.4.1	Test Purpose and Environment	5191

A.14.5.3.4.2	Test parameters	5191

A.14.5.3.4.3	Test Requirements	5193

A.14.5.3.5	SSB based L1-RSRP measurement when DRX is not used in FR2-NTN	5193

A.14.5.3.5.1	Test Purpose and Environment	5193

A.14.5.3.5.2	Test parameters	5193

A.14.5.3.5.3	Test Requirements	5194

A.14.6	Measurement Performance requirements	5195

A.14.6.1	SS-RSRP for SAN	5195

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5195

A.14.6.1.1.1	Test Purpose and Environment	5195

A.14.6.1.1.2	Test parameters	5195

A.14.6.1.1.3	Test Requirements	5196

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5196

A.14.6.1.2.1	Test Purpose and Environment	5196

A.14.6.1.2.2	Test parameters	5197

A.14.6.1.2.3	Test Requirements	5198

A.14.6.1.3	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5198

A.14.6.1.3.1	Test Purpose and Environment	5198

A.14.6.1.3.2	Test parameters	5198

A.14.6.1.3.3	Test Requirements	5200

A.14.6.2	SS-RSRQ	5200

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5200

A.14.6.2.1.1	Test Purpose and Environment	5200

A.14.6.2.1.2	Test Parameters	5200

A.14.6.2.1.3	Test Requirements	5202

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5202

A.14.6.2.2.1	Test Purpose and Environment	5202

A.14.6.2.2.2	Test Parameters	5202

A.14.6.2.2.3	Test Requirements	5203

A.14.6.3	SS-SINR	5203

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5203

A.14.6.3.1.1	Test Purpose and Environment	5203

A.14.6.3.1.2	Test Parameters	5203

A.14.6.3.1.3	Test Requirements	5205

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5205

A.14.6.3.2.1	Test Purpose and Environment	5205

A.14.6.3.2.2	Test Parameters	5205

A.14.6.3.2.3	Test Requirements	5206

A.14.6.4	L1-RSRP measurement for beam reporting	5206

A.14.6.4.1	SSB based L1-RSRP measurement	5206

A.14.6.4.1.1	Test Purpose and Environment	5206

A.14.6.4.1.2	Test parameters	5206

A.14.6.4.1.3	Test Requirements	5208

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5208

A.14.6.4.2.1	Test Purpose and Environment	5208

A.14.6.4.2.2	Test parameters	5208

A.14.6.4.2.3	Test Requirements	5209

A.14.6.4.3	SSB based L1-RSRP measurement for VSAT UE in FR2-NTN when DRX is not used	5209

A.14.6.4.3.1	Test Purpose and Environment	5209

A.14.6.4.3.2	Test parameters	5209

A.14.6.4.3.3	Test Requirements	5211

A.15	NR standalone tests with one or more NR cells in FR2-2	5213

A.15.1	SA: RRC_IDLE state mobility	5213

A.15.1.1	Cell re-selection to NR	5213

A.15.1.1.1	Cell re-selection to FR2-2 intra-frequency NR case	5213

A.15.1.1.1.1	Test Purpose and Environment	5213

A.15.1.1.1.2	Test Parameters	5213

A.15.1.1.1.3	Test Requirements	5215

A.15.1.2	Cell re-selection to FR2-2 inter-frequency NR case	5215

A.15.1.2.1	Test Purpose and Environment	5215

A.15.1.2.2	Test Parameters	5216

A.15.1.2.3	Test Requirements	5218

A.15.1.3	Cell re-selection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5218

A.15.1.3.1	Test Purpose and Environment	5218

A.15.1.3.2	Test Parameters	5218

A.15.1.3.3	Test Requirements	5220

A.15.1.4	Cell re-selection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5221

A.15.1.4.1	Test Purpose and Environment	5221

A.15.1.4.2	Test Parameters	5221

A.15.1.4.3	Test Requirements	5223

A.15.1.5	Cell re-selection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5223

A.15.1.5.1	Test Purpose and Environment	5223

A.15.1.5.2	Test Parameters	5223

A.15.1.5.3	Test Requirements	5225

A.15.1.6	Cell re-selection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5226

A.15.1.6.1	Test Purpose and Environment	5226

A.15.1.6.2	Test Parameters	5226

A.15.1.6.3	Test Requirements	5228

A.15.2	Signaling characteristics	5228

A.15.2.1	SCell Activation and Deactivation Delay	5228

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5228

A.15.2.1.1.1	Test Purpose and Environment	5228

A.15.2.1.1.2	Test Requirements	5230

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5230

A.15.2.1.2.1	Test Purpose and Environment	5230

A.15.2.1.2.2	Test Requirements	5233

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5233

A.15.2.1.3.1	Test Purpose and Environment	5234

A.15.2.1.3.2	Test Requirements	5236

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5237

A.15.2.1.4.1	Test Purpose and Environment	5237

A.15.2.1.4.2	Test Requirements	5239

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5240

A.15.2.1.5.1	Test Purpose and Environment	5240

A.15.2.1.5.2	Test Requirements	5242

A.15.3	RRC_CONNECTED state mobility	5243

A.15.3.1	Handover	5243

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5243

A.15.3.1.1.1	Test Purpose and Environment	5243

A.15.3.1.1.2	Test Parameters	5243

A.15.3.1.1.3	Test Requirements	5245

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5246

A.15.3.1.2.1	Test Purpose and Environment	5246

A.15.3.1.2.2	Test Parameters	5246

A.15.3.1.2.3	Test Requirements	5248

A.15.4	Measurement procedure	5248

A.15.4.1	Intra-frequency Measurements	5248

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5248

A.15.4.1.1.1	Test purpose and Environment	5248

A.15.4.1.1.2	Test Requirements	5251

A.15.4.2	Inter-frequency Measurements	5252

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5252

A.15.4.2.1.1	Test Purpose and Environment	5252

A.15.4.2.1.2	Test Requirements	5254

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5255

A.16.1	SA: RRC_IDLE state mobility for RedCap	5255

A.16.1.1	Cell re-selection to NR	5255

A.16.1.1.1	Cell re-selection to FR1 intra-frequency NR case for 1 Rx UE	5255

A.16.1.1.1.1	Test Purpose and Environment	5255

A.16.1.1.1.2	Test Parameters	5255

A.16.1.1.1.3	Test Requirements	5257

A.16.1.1.2	Cell re-selection to FR1 intra-frequency NR case for 2 Rx UE	5258

A.16.1.1.2.1	Test Purpose and Environment	5258

A.16.1.1.2.2	Test Parameters	5258

A.16.1.1.2.3	Test Requirements	5260

A.16.1.1.3	Cell re-selection to FR1 inter-frequency NR case for 1 Rx UE	5260

A.16.1.1.3.1	Test Purpose and Environment	5260

A.16.1.1.3.2	Test Parameters	5260

A.16.1.1.3.3	Test Requirements	5263

A.16.1.1.4	Cell re-selection to FR1 inter-frequency NR case for 2 Rx UE	5263

A.16.1.1.4.1	Test Purpose and Environment	5263

A.16.1.1.4.2	Test Parameters	5263

A.16.1.1.4.3	Test Requirements	5266

A.16.1.1.5	Cell re-selection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5266

A.16.1.1.5.1	Test Purpose and Environment	5266

A.16.1.1.5.2	Test Parameters	5266

A.16.1.1.5.3	Test Requirements	5269

A.16.1.1.6	Cell re-selection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5269

A.16.1.1.6.1	Test Purpose and Environment	5269

A.16.1.1.6.2	Test Parameters	5269

A.16.1.1.6.3	Test Requirements	5271

A.16.1.1.7	Cell re-selection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5272

A.16.1.1.7.1	Test Purpose and Environment	5272

A.16.1.1.7.2	Test Parameters	5272

A.16.1.1.7.3	Test Requirements	5274

A.16.1.1.8	Cell re-selection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5275

A.16.1.1.8.1	Test Purpose and Environment	5275

A.16.1.1.8.2	Test Parameters	5275

A.16.1.1.8.3	Test Requirements	5277

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5278

A.16.1.2.1	Cell re-selection to higher priority E-UTRAN for 1 RX	5278

A.16.1.2.1.1	Test Purpose and Environment	5278

A.16.1.2.1.2	Test Parameters	5278

A.16.1.2.1.3	Test Requirements	5281

A.16.1.2.2	Cell re-selection to higher priority E-UTRAN for 2 RX	5281

A.16.1.2.2.1	Test Purpose and Environment	5281

A.16.1.2.2.2	Test Parameters	5281

A.16.1.2.2.3	Test Requirements	5284

A.16.1.2.3.1	Test Purpose and Environment	5284

A. 16.1.2.3.2	Test Parameters	5284

A.16.1.2.3.3	Test Requirements	5287

A.16.1.2.4.1	Test Purpose and Environment	5287

A.16.1.2.4.2	Test Parameters	5287

A.16.1.3.1.3	Test Requirements	5290

A.16.1.2.5	Cell re-selection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5290

A.16.1.2.5.1	Test Purpose and Environment	5290

A.16.1.2.5.2	Test Parameters	5290

A.16.1.2.5.3	Test Requirements	5293

A.16.1.2.6	Cell re-selection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5293

A.16.1.2.6.1	Test Purpose and Environment	5293

A.16.1.2.6.2	Test Parameters	5294

A.16.1.2.6.3	Test Requirements	5296

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5297

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5297

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1 Rx RedCap UE	5297

A.16.2.1.1.1	Test purpose and Environment	5297

A.16.2.1.1.2	Test Parameters	5298

A.16.2.1.1.3	Test requirements	5300

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2 Rx RedCap UE	5300

A.16.2.1.2.1	Test purpose and Environment	5300

A.16.2.1.2.2	Test Parameters	5302

A.16.2.1.2.3	Test requirements	5303

A.16.2.2	Cell Reselection for Positioning	5304

A.16.2.2.1	Cell re-selection to FR1 intra-frequency NR case with RRC_INACTIVE eDRX and positioning SRS	5304

A.16.2.2.1.1	Test Purpose and Environment	5304

A.16.2.2.1.2	Test Parameters	5304

A.16.2.2.1.3	Test Requirements	5307

A.16.3	RRC_CONNECTED state mobility for RedCap	5307

A.16.3.1	Handover	5307

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5307

A.16.3.1.1.1	Test Purpose and Environment	5307

A.16.3.1.1.2	Test Parameters	5307

A.16.3.1.1.3	Test Requirements	5309

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5309

A.16.3.1.2.1	Test Purpose and Environment	5309

A.16.3.1.2.2	Test Parameters	5309

A.16.3.1.2.3	Test Requirements	5311

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5311

A.16.3.1.3.1	Test Purpose and Environment	5311

A.16.3.1.3.2	Test Parameters	5312

A.16.3.1.3.3	Test Requirements	5314

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5314

A.16.3.1.4.1	Test Purpose and Environment	5314

A.16.3.1.4.2	Test Parameters	5314

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5316

A.16.3.1.5.1	Test Purpose and Environment	5316

A.16.3.1.5.2	Test Parameters	5316

A.16.3.1.5.3	Test Requirements	5318

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5319

A.16.3.1.6.1	Test Purpose and Environment	5319

A.16.3.1.6.2	Test Parameters	5319

A.16.3.1.6.3	Test Requirements	5321

A.16.3.1.7	SA NR - E-UTRAN handover for 1 Rx UE	5321

A.16.3.1.7.1	Test Purpose and Environment	5321

A.16.3.1.7.2	Test Requirements	5325

A.16.3.1.8	SA NR - E-UTRAN handover for 2 Rx UE	5325

A.16.3.1.8.1	Test Purpose and Environment	5325

A.16.3.1.8.2	Test Requirements	5328

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5328

A.16.3.1.9.1	Test Purpose and Environment	5328

A.16.3.1.9.2	Test Requirements	5331

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5331

A.16.3.1.10.1	Test Purpose and Environment	5331

A.16.3.1.10.2	Test Requirements	5335

A.16.3.2	RRC Connection Mobility Control	5335

A.16.3.2.1	SA: RRC Re-establishment	5335

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5335

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5338

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5340

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5343

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5346

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5349

A.16.3.2.2	Random Access	5352

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5352

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5355

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5358

A.16.3.2.2.4	4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	5361

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5365

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5368

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5370

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5373

A.16.3.2.3	SA: RRC Connection Release with Redirection	5376

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5376

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5378

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5381

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5384

A.16.4	Timing for RedCap	5388

A.16.4.1	UE transmit timing	5388

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1 Rx RedCap UE	5388

A.16.4.1.1.1	Test Purpose and environment	5388

A.16.4.1.1.2	Test requirements	5390

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2 Rx RedCap UE	5390

A.16.4.1.2.1	Test Purpose and environment	5390

A.16.4.1.2.2	Test requirements	5392

A.16.4.2	Void	5392

A.16.4.3	Timing advance	5392

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5392

A.16.4.3.1.1	Test Purpose and Environment	5392

A.16.4.3.1.2	Test Parameters	5392

A.16.4.3.1.3	Test Requirements	5394

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5395

A.16.4.3.2.1	Test Purpose and Environment	5395

A.16.4.3.2.2	Test Parameters	5395

A.16.4.3.2.3	Test Requirements	5397

A.16.5	Signalling characteristics for RedCap	5397

A.16.5.1	Radio link Monitoring	5397

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5397

A.16.5.1.1.1	Test Purpose and Environment	5397

A.16.5.1.1.2	Test Requirements	5400

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5400

A.16.5.1.2.1	Test Purpose and Environment	5400

A.16.5.1.2.2	Test Requirements	5403

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5403

A.16.5.1.3.1	Test Purpose and Environment	5403

A.16.5.1.3.2	Test Requirements	5406

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5406

A.16.5.1.4.1	Test Purpose and Environment	5406

A.16.5.1.4.2	Test Requirements	5409

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5409

A.16.5.1.5.1	Test Purpose and Environment	5409

A.16.5.1.5.2	Test Requirements	5412

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5412

A.16.5.1.6.1	Test Purpose and Environment	5412

A.16.5.1.6.2	Test Requirements	5415

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5415

A.16.5.1.7.1	Test Purpose and Environment	5415

A.16.5.1.7.2	Test Requirements	5418

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5418

A.16.5.1.8.1	Test Purpose and Environment	5418

A.16.5.1.8.2	Test Requirements	5421

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5421

A.16.5.1.9.1	Test Purpose and Environment	5421

A.16.5.1.9.2	Test Requirements	5424

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5424

A.16.5.1.10.1	Test Purpose and Environment	5424

A.16.5.1.10.2	Test Requirements	5427

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5427

A.16.5.1.11.1	Test Purpose and Environment	5427

A.16.5.1.11.2	Test Requirements	5430

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5430

A.16.5.1.12.1	Test Purpose and Environment	5430

A.16.5.1.12.2	Test Requirements	5433

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5434

A.16.5.1.13.1	Test Purpose and Environment	5434

A.16.5.1.13.2	Test Requirements	5436

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5437

A.16.5.1.14.1	Test Purpose and Environment	5437

A.16.5.1.14.2	Test Requirements	5439

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5440

A.16.5.1.15.1	Test Purpose and Environment	5440

A.16.5.1.15.2	Test Requirements	5443

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5443

A.16.5.1.16.1	Test Purpose and Environment	5443

A.16.5.1.16.2	Test Requirements	5446

A.16.5.2	Beam Failure Detection and Link recovery procedures	5446

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5446

A.16.5.2.1.1	Test Purpose and Environment	5446

A.16.5.2.1.2	Test Requirements	5449

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5450

A.16.5.2.2.1	Test Purpose and Environment	5450

A.16.5.2.2.2	Test Requirements	5453

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5453

A.16.5.2.3.1	Test Purpose and Environment	5453

A.16.5.2.3.2	Test Requirements	5456

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5457

A.16.5.2.4.1	Test Purpose and Environment	5457

A.16.5.2.4.2	Test Requirements	5460

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5460

A.16.5.2.5.1	Test Purpose and Environment	5460

A.16.5.2.5.2	Test Requirements	5463

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5464

A.16.5.2.6.1	Test Purpose and Environment	5464

A.16.5.2.6.2	Test Requirements	5467

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5467

A.16.5.2.7.1	Test Purpose and Environment	5467

A.16.5.2.7.2	Test Requirements	5470

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5471

A.16.5.2.8.1	Test Purpose and Environment	5471

A.16.5.2.8.2	Test Requirements	5474

A.16.5.3	Active BWP switch	5474

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5474

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5474

A.16.5.3.1.1.1	Test Purpose and Environment	5474

A.16.5.3.1.1.2	Test Requirements	5477

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5477

A.16.5.3.1.2.1	Test Purpose and Environment	5477

A.16.5.3.1.2.2	Test Requirements	5479

A.16.5.3.2	RRC-based Active BWP Switch	5480

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5480

A.16.5.3.2.1.1	Test Purpose and Environment	5480

A.16.5.3.2.1.2	Test Requirements	5482

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5482

A.16.5.3.2.2.1	Test Purpose and Environment	5482

A.16.5.3.2.2.2	Test Requirements	5484

A.16.5.4	UE specific CBW change	5485

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5485

A.16.5.4.1.1	Test Purpose and Environment	5485

A.16.5.4.1.2	Test Requirements	5487

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5487

A.16.5.4.2.1	Test Purpose and Environment	5487

A.16.5.4.2.2	Test Requirements	5490

A.16.6	Measurement procedure for RedCap	5490

A.16.6.1	Intra-frequency Measurements	5490

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5490

A.16.6.1.1.1	Test purpose and Environment	5490

A.16.6.1.1.2	Test parameters	5490

A.16.6.1.1.3	Test Requirements	5492

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5492

A.16.6.1.2.1	Test purpose and Environment	5492

A.16.6.1.2.2	Test parameters	5492

A.16.6.1.2.3	Test Requirements	5494

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5494

A.16.6.1.3.1	Test purpose and Environment	5494

A.16.6.1.3.2	Test parameters	5494

A.16.6.1.3.3	Test Requirements	5496

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5497

A.16.6.1.4.1	Test purpose and Environment	5497

A.16.6.1.4.2	Test parameters	5497

A.16.6.1.4.3	Test Requirements	5499

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5499

A.16.6.1.5.1	Test purpose and Environment	5499

A.16.6.1.5.2	Test parameters	5499

A.16.6.1.5.3	Test Requirements	5501

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5501

A.16.6.1.6.1	Test purpose and Environment	5501

A.16.6.1.6.2	Test parameters	5501

A.16.6.1.6.3	Test Requirements	5503

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5504

A.16.6.1.7.1	Test purpose and Environment	5504

A.16.6.1.7.2	Test parameters	5504

A.16.6.1.7.3	Test Requirements	5506

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5506

A.16.6.1.8.1	Test purpose and Environment	5506

A.16.6.1.8.2	Test parameters	5506

A.16.6.1.8.3	Test Requirements	5508

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5508

A.16.6.1.9.1	Test purpose and Environment	5508

A.16.6.1.9.2	Test parameters	5509

A.16.6.1.9.3	Test Requirements	5510

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5510

A.16.6.1.10.1	Test purpose and Environment	5510

A.16.6.1.10.2	Test parameters	5510

A.16.6.1.10.3	Test Requirements	5512

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5512

A.16.6.1.11.1	Test purpose and Environment	5512

A.16.6.1.11.2	Test parameters	5512

A.16.6.1.11.3	Test Requirements	5514

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5514

A.16.6.1.12.1	Test purpose and Environment	5514

A.16.6.1.12.2	Test parameters	5514

A.16.6.1.12.3	Test Requirements	5515

A.16.6.2	Inter-frequency Measurements	5516

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5516

A.16.6.2.1.1	Test Purpose and Environment	5516

A.16.6.2.1.2	Test Requirements	5518

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5519

A.16.6.2.2.1	Test Purpose and Environment	5519

A.16.6.2.2.2	Test Requirements	5521

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5521

A.16.6.2.3.1	Test Purpose and Environment	5522

A.16.6.2.3.2	Test Requirements	5524

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5524

A.16.6.2.4.1	Test Purpose and Environment	5524

A.16.6.2.4.2	Test Requirements	5526

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5526

A.16.6.2.5.1	Test Purpose and Environment	5526

A.16.6.2.5.2	Test Requirements	5529

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5529

A.16.6.2.6.1	Test Purpose and Environment	5529

A.16.6.2.6.2	Test Requirements	5531

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5531

A.16.6.2.7.1	Test Purpose and Environment	5531

A.16.6.2.7.2	Test Requirements	5534

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5534

A.16.6.2.8.1	Test Purpose and Environment	5534

A.16.6.2.8.2	Test Requirements	5536

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5537

A.16.6.2.9.1	Test Purpose and Environment	5537

A.16.6.2.9.2	Test Requirements	5539

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5539

A.16.6.2.10.1	Test Purpose and Environment	5539

A.16.6.2.10.2	Test Requirements	5541

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5541

A.16.6.2.11.1	Test Purpose and Environment	5542

A.16.6.2.11.2	Test Requirements	5544

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5544

A.16.6.2.12.1	Test Purpose and Environment	5544

A.16.6.2.12.2	Test Requirements	5547

A.16.6.3	Inter-RAT Measurements	5547

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5547

A.16.6.3.1.1	Test purpose and Environment	5547

A.16.6.3.1.2	Test Requirements	5550

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5551

A.16.6.3.2.1	Test purpose and Environment	5551

A.16.6.3.2.2	Test Requirements	5554

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5554

A.16.6.3.3.1	Test purpose and Environment	5554

A.16.6.3.3.2	Test Requirements	5557

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5558

A.16.6.3.4.1	Test purpose and Environment	5558

A.16.6.3.4.2	Test Requirements	5561

A.16.6.4	L1-RSRP measurement for beam reporting	5561

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5561

A.16.6.4.1.1	Test Purpose and Environment	5561

A.16.6.4.1.2	Test parameters	5562

A.16.6.4.1.3	Test Requirements	5563

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5563

A.16.6.4.2.1	Test Purpose and Environment	5563

A.16.6.4.2.2	Test parameters	5564

A.16.6.4.2.3	Test Requirements	5565

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5565

A.16.6.4.3.1	Test Purpose and Environment	5565

A.16.6.4.3.2	Test parameters	5565

A.16.6.4.3.3	Test Requirements	5567

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5567

A.16.6.4.4.1	Test Purpose and Environment	5567

A.16.6.4.4.2	Test parameters	5567

A.16.6.4.4.3	Test Requirements	5569

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5569

A.16.6.4.5.1	Test Purpose and Environment	5569

A.16.6.4.5.2	Test parameters	5569

A.16.6.4.5.3	Test Requirements	5571

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5571

A.16.6.4.6.1	Test Purpose and Environment	5571

A.16.6.4.6.2	Test parameters	5571

A.16.6.4.6.3	Test Requirements	5573

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5573

A.16.6.4.7.1	Test Purpose and Environment	5573

A.16.6.4.7.2	Test parameters	5573

A.16.6.4.7.3	Test Requirements	5575

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5575

A.16.6.4.8.1	Test Purpose and Environment	5575

A.16.6.4.8.2	Test parameters	5575

A.16.6.4.8.3	Test Requirements	5577

A.16.6.5	NR measurements with autonomous gaps	5577

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5577

A.16.6.5.1.1	Test Purpose and Environment	5577

A.16.6.5.1.2	Test Parameters	5577

A.16.6.5.1.3	Test Requirements	5579

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5579

A.16.6.5.2.1	Test Purpose and Environment	5579

A.16.6.5.2.2	Test Parameters	5579

A.16.6.5.2.3	Test Requirements	5581

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5581

A.16.6.5.3.1	Test Purpose and Environment	5581

A.16.6.5.3.2	Test Requirements	5584

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5584

A.16.6.5.4.1	Test Purpose and Environment	5584

A.16.6.5.4.2	Test Requirements	5587

A.16.6.6	RSTD Measurements	5588

A.16.6.6.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR1 SA	5588

A.16.6.6.1.1	Test Purpose and Environment	5588

A.16.6.6.1.2	Test Requirements	5593

A.16.6.6.2	NR RSTD measurement reporting delay test case with PRS frequency hopping	5593

A.16.6.6.2.1	Test Purpose and Environment	5593

A.16.6.6.2.2	Test Requirements	5597

A.16.6.7	UE Rx-Tx Measurements	5598

A.16.6.7.1	UE Rx-Tx measurement reporting delay test case for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_CONNECTED mode	5598

A.16.6.7.1.1	Test purpose and environment	5598

A.16.6.7.1.2	Test requirements	5602

A.16.6.7.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR1 SA in RRC_CONNECTED state	5602

A.16.6.7.2.1	Test purpose and environment	5602

A.16.6.7.2.2	Test requirements	5606

A.16.6.8	PRS-RSRP measurements	5606

A.16.6.8.1	PRS-RSRP measurement delay test case for single positioning frequency layer	5606

A.16.6.8.1.1	Test purpose and Environment	5606

A.16.6.8.1.2	Test Requirements	5610

A.16.6.8.2	PRS-RSRP measurement delay with FH in RRC_CONNECTED state in FR1	5610

A.16.6.8.2.1	Test purpose and Environment	5610

A.16.6.8.2.2	Test Requirements	5614

A.16.6.9	PRS-RSRPP Measurements	5614

A.16.6.9.1	PRS-RSRPP measurement delay without FH in RRC_CONNECTED state in FR1	5614

A.16.6.9.1.1	Test purpose and Environment	5614

A.16.6.9.1.2	Test Requirements	5616

A.16.6.9.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state	5617

A.16.6.9.2.1	Test purpose and Environment	5617

A.16.6.9.2.2	Test Requirements	5619

A.16.7	Measurement Performance requirements for RedCap	5619

A.16.7.1	SS-RSRP	5619

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5619

A.16.7.1.1.1	Test Purpose and Environment	5619

A.16.7.1.1.2	Test parameters	5619

A.16.7.1.1.3	Test Requirements	5623

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 RX UE	5623

A.16.7.1.2.1	Test Purpose and Environment	5623

A.16.7.1.2.2	Test parameters	5623

A.16.7.1.2.3	Test Requirements	5627

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5627

A.16.7.1.3.1	Test Purpose and Environment	5627

A.16.7.1.3.2	Test parameters	5627

A.16.7.1.3.3	Test Requirements	5630

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5630

A.16.7.1.4.1	Test Purpose and Environment	5630

A.16.7.1.4.2	Test parameters	5630

A.16.7.1.4.3	Test Requirements	5633

A.16.7.2	SS-RSRQ	5633

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5633

A.16.7.2.1.1	Test Purpose and Environment	5633

A.16.7.2.1.2	Test Parameters	5633

A.16.7.2.1.3	Test Requirements	5637

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5637

A.16.7.2.2.1	Test Purpose and Environment	5637

A.16.7.2.2.2	Test Parameters	5637

A.16.7.2.2.3	Test Requirements	5640

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5640

A.16.7.2.3.1	Test Purpose and Environment	5640

A.16.7.2.3.2	Test parameters	5641

A.16.7.2.3.3	Test Requirements	5644

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5644

A.16.7.2.4.1	Test Purpose and Environment	5644

A.16.7.2.4.2	Test parameters	5644

A.16.7.2.4.3	Test Requirements	5648

A.16.7.3	SS-SINR	5648

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5648

A.16.7.3.1.1	Test Purpose and Environment	5648

A.16.7.3.1.2	Test parameters	5648

A.16.7.3.1.3	Test Requirements	5651

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5651

A.16.7.3.2.1	Test Purpose and Environment	5651

A.16.7.3.2.2	Test parameters	5651

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5654

A.16.7.3.3.1	Test Purpose and Environment	5654

A.16.7.3.3.2	Test parameters	5654

A.16.7.3.3.3	Test Requirements	5657

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5657

A.16.7.3.4.1	Test Purpose and Environment	5657

A.16.7.3.4.2	Test parameters	5657

A.16.7.3.4.3	Test Requirements	5660

A.16.7.4	L1-RSRP measurement for beam reporting	5661

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5661

A.16.7.4.1.1	Test Purpose and Environment	5661

A.16.7.4.1.2	Test parameters	5661

A.16.7.4.1.3	Test Requirements	5663

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5663

A.16.7.4.2.1	Test Purpose and Environment	5663

A.16.7.4.2.2	Test parameters	5664

A.16.7.4.2.3	Test Requirements	5664

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5664

A.16.7.4.3.1	Test Purpose and Environment	5664

A.16.7.4.3.2	Test parameters	5664

A.16.7.4.3.3	Test Requirements	5667

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5667

A.16.7.4.4.1	Test Purpose and Environment	5667

A.16.7.4.4.2	Test parameters	5667

A.16.7.4.4.3	Test Requirements	5667

A.16.7.5	E-UTRAN RSRP	5667

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5667

A.16.7.5.1.1	Test Purpose and Environment	5667

A.16.7.5.1.2	Test parameters	5667

A.16.7.5.1.3	Test Requirements	5671

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5671

A.16.7.5.2.1	Test Purpose and Environment	5671

A.16.7.5.2.2	Test parameters	5671

A.16.7.5.2.3	Test Requirements	5674

A.16.7.6	E-UTRAN RSRQ	5674

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5674

A.16.7.6.1.1	Test Purpose and Environment	5674

A.16.7.6.1.2	Test parameters	5674

A.16.7.6.1.3	Test Requirements	5678

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5678

A.16.7.6.2.1	Test Purpose and Environment	5678

A.16.7.6.2.2	Test parameters	5678

A.16.7.6.2.3	Test Requirements	5681

A.16.7.7	RSTD measurements	5681

A.16.7.7.1	RSTD measurement accuracy test case for RedCap UE without FH	5681

A.16.7.7.1.1	Test purpose and Environment	5681

A.16.7.7.1.2	Test Requirements	5683

A.16.7.8	UE Rx-Tx measurements	5687

A.16.7.8.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_CONNECTED mode	5687

A.16.7.8.1.1	Test purpose and environment	5687

A.16.7.8.1.2	Test parameters	5688

A.16.7.8.1.3	Test requirements	5691

A.16.7.8.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_CONNECTED state in FR1	5691

A.16.7.8.2.1 	Test purpose and Environment	5691

A.16.7.8.2.2	Test parameters	5692

A.16.7.8.2.3	Test requirements	5695

A.16.7.9	PRS-RSRP Measurements	5695

A.16.7.9.1	PRS-RSRP measurement accuracy without FH in RRC_CONNECTED state in FR1	5695

A.16.7.9.1.1	Test Purpose and Environment	5695

A.16.7.9.1.2	Test parameters	5695

A.16.7.9.1.3	Test Requirements	5699

A.16.7.9.2	PRS-RSRP measurement accuracy with FH in RRC_CONNECTED state in FR1	5700

A.16.7.9.2.1	Test Purpose and Environment	5700

A.16.7.9.2.2	Test parameters	5700

A.16.7.9.2.3	Test Requirements	5703

A.16.7.10	PRS-RSRPP measurements	5704

A.16.7.10.1	PRS-RSRPP measurement accuracy without FH in RRC_CONNECTED state in FR1	5704

A.16.7.10.1.1	Test Purpose and Environment	5704

A.16.7.10.1.2	Test parameters	5704

A.16.7.10.1.3	Test Requirements	5707

A.16.7.10.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_CONNECTED state in FR1	5707

A.16.7.10.2.1	Test purpose and Environment	5707

A.16.7.10.2.2	Test parameters	5707

A.16.7.10.2.3	Test requirements	5711

A.16.8	Measurement Procedure for RedCap in RRC_INACTIVE	5711

A.16.8.1	RSTD Measurements	5711

A.16.8.1.1	NR RSTD measurement reporting delay test case for for RedCap UE without FH in FR1 SA in RRC_INACTIVE state	5711

A.16.8.1.1.1	Test Purpose and Environment	5711

A.16.8.1.1.2	Test Requirements	5715

A.16.8.1.2	NR RSTD measurement reporting delay test case with PRS frequency hopping	5715

A.16.8.1.2.1	Test Purpose and Environment	5715

A.16.8.1.2.2	Test Requirements	5719

A.16.8.1.3	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state when eDRX cycle > 10.24s for RedCap UE	5719

A.16.8.1.3.1	Test Purpose and Environment	5719

A.16.8.1.3.2	Test Requirements	5723

A.16.8.2	UE Rx-Tx Measurements	5723

A.16.8.2.1	UE Rx-Tx measurement reporting delay test case for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_INACTIVE mode	5723

A.16.8.2.1.1	Test purpose and environment	5723

A.16.8.2.1.2	Test requirements	5727

A.16.8.2.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	5727

A.16.8.2.2.1	Test purpose and environment	5727

A.16.8.2.2.2	Test requirements	5731

A.16.8.2.3.	UE Rx-Tx time difference measurement for single positioning frequency layer with eDRX > 10.24s in FR1 SA	5731

A.16.8.2.3.1	Test purpose and environment	5731

A.16.8.2.3.2	Test requirements	5735

A.16.8.3	PRS-RSRP Measurements	5735

A.16.8.3.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	5735

A.16.8.3.1.1	Test purpose and Environment	5735

A.16.8.3.1.2	Test Requirements	5737

A.16.8.3.3	PRS-RSRP reporting delay test case in RRC_INACTIVE state in FR1 when eDRX cycle > 10.24s	5738

A.16.8.3.3.1	Test purpose and Environment	5738

A.16.8.3.3.2	Test Requirements	5740

A.16.8.4	PRS-RSRPP Measurements	5741

A.16.8.4.1	PRS-RSRPP measurement delay without FH in RRC_INACTIVE state in FR1	5741

A.16.8.4.1.1	Test purpose and Environment	5741

A.16.8.4.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	5744

A.16.8.4.2.1	Test purpose and Environment	5744

A.16.8.4.2.2	Test Requirements	5746

A.16.9	Measurement Performance Requirements for RedCap in RRC_INACTIVE	5749

A.16.9.1	 RSTD Measurements	5749

A.16.9.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR1 in RRC_INACTIVE state	5749

A.16.9.1.1.1	Test purpose and Environment	5749

A.16.9.1.1.2	Test Requirements	5751

A.16.9.1.2	RSTD measurement accuracy test case for RedCap UE with FH in FR1 in RRC_INACTIVE state	5751

A.16.9.1.2.1	Test purpose and Environment	5751

A.16.9.1.2.2	Test Requirements	5753

A.16.9.2	UE Rx-Tx measurements	5753

A.16.9.2.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_INACTIVE mode	5753

A.16.9.2.1.1	Test purpose and environment	5753

A.16.9.2.1.2	Test parameters	5754

A.16.9.2.1.3	Test requirements	5755

A.16.9.2.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_INACTIVE state in FR1	5756

A.16.9.2.2.1	Test purpose and Environment	5756

A.16.9.2.2.2	Test parameters	5756

A.16.9.2.2.3	Test requirements	5759

A.16.9.3	PRS-RSRP Measurements	5759

A.16.9.3.1	PRS-RSRP measurement accuracy without FH in RRC_INACTIVE state in FR1	5759

A.16.9.3.1.1	Test Purpose and Environment	5759

A.16.9.3.1.2	Test parameters	5759

A.16.9.3.1.3	Test Requirements	5762

A.16.9.3.2	PRS-RSRP measurement accuracy with FH in RRC_INACTIVE state in FR1	5762

A.16.9.3.2.1	Test Purpose and Environment	5762

A.16.9.3.2.2	Test parameters	5762

A.16.9.3.2.3	Test Requirements	5765

A.16.9.4	PRS-RSRPP measurements	5765

A.16.9.4.1	PRS-RSRPP measurement accuracy without Rx FH in RRC_INACTIVE state in FR1	5765

A.16.9.4.1.1	Test purpose and Environment	5765

A.16.9.4.1.2	Test parameters	5765

A.16.9.4.1.3	Test requirements	5769

A.16.9.4.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_INACTIVE state in FR1	5769

A.16.9.4.2.1	Test purpose and Environment	5769

A.16.9.4.2.2	Test parameters	5769

A.16.9.4.2.3	Test requirements	5772

A.16.10	 Measurement procedure for RedCap in RRC_IDLE	5772

A.16.10.1	RSTD measurements	5772

A.16.10.1.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR1 SA in RRC_IDLE state without eDRX	5772

A.16.10.1.1.1	Test Purpose and Environment	5772

A.16.10.1.1.2	Test Requirements	5776

A.16.10.1.2	NR RSTD measurement reporting delay test case for RedCap UE without RX FH in FR1 SA in RRC_IDLE state when eDRX > 10.24s	5776

A.16.10.1.2.1	Test Purpose and Environment	5776

A.16.10.1.2.2	Test Requirements	5780

A.16.10.2	PRS-RSRP Measurements	5780

A.16.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE	5780

A.16.10.2.1.1	Test purpose and Environment	5780

A.16.10.2.1.2	Test Requirements	5782

A.16.10.2.2	PRS-RSRP measurement without Rx FH reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s	5783

A.16.10.2.2.1	Test purpose and Environment	5783

A.16.10.2.2.2	Test Requirements	5785

A.16.11	 Measurement Performance Requirements for RedCap in RRC_IDLE	5785

A.16.11.1	RSTD Measurements	5785

A.16.11.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR1 in RRC_IDLE state without eDRX	5785

A.16.11.1.1.1	Test purpose and Environment	5785

A.16.11.1.1.2	Test Requirements	5787

A.16.11.1.2	RSTD measurement accuracy test case for RedCap UE without FH in FR1 in RRC_IDLE state with eDRX > 10.24s	5788

A.16.11.1.2.1	Test purpose and Environment	5788

A.16.11.1.2.2	Test Requirements	5790

A.16.11.2	PRS-RSRP Measurements	5790

A.16.11.2.1	PRS-RSRP measurement accuracy test case for RedCap UE in FR1 in RRC_IDLE state	5790

A.16.11.2.1.1	Test Purpose and Environment	5790

A.16.11.2.1.2	Test parameters	5790

A.16.11.2.1.3	Test Requirements	5792

A.16.11.2.2	PRS-RSRP measurement without Rx FH accuracy test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s	5792

A.16.11.2.2.1	Test purpose and Environment	5792

A.16.11.2.2.2	Test Requirements	5794

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5796

A.17.1	SA: RRC_IDLE state mobility for RedCap	5796

A.17.1.1	Cell re-selection to NR	5796

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5796

A.17.1.1.1.1	Test Purpose and Environment	5796

A.17.1.1.1.2	Test Parameters	5796

A.17.1.1.1.3	Test Requirements	5798

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5798

A.17.1.1.2.1	Test Purpose and Environment	5798

A.17.1.1.2.2	Test Parameters	5798

A.17.1.1.2.3	Test Requirements	5800

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5801

A.17.1.1.3.1	Test Purpose and Environment	5801

A.17.1.1.3.2	Test Parameters	5801

A.17.1.1.3.3	Test Requirements	5803

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5803

A.17.1.1.4.1	Test Purpose and Environment	5803

A.17.1.1.4.2	Test Parameters	5803

A.17.1.1.4.3	Test Requirements	5805

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5806

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5806

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5806

A.17.2.1.1.1	Test Purpose and Environment	5806

A.17.2.1.1.2	Test Requirements	5809

A.17.2.2	Cell Reselection for Positioning	5809

A.17.2.2.1	Cell reselection to FR2 intra-frequency NR case with RRC_INACTIVE eDRX and positioning SRS	5809

A.17.2.2.1.1	Test Purpose and Environment	5809

A.17.2.2.1.2	Test Parameters	5809

A.17.2.2.1.3	Test Requirements	5809

A.17.3	RRC_CONNECTED state mobility for RedCap	5809

A.17.3.1	Handover for RedCap	5809

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5809

A.17.3.1.1.1	Test Purpose and Environment	5809

A.17.3.1.1.2	Test Parameters	5809

A.17.3.1.1.3	Test Requirements	5811

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5811

A.17.3.1.2.1	Test Purpose and Environment	5811

A.17.3.1.2.2	Test Parameters	5811

A.17.3.1.2.3 Test Requirements	5813

A.17.3.2	RRC Connection Mobility Control for RedCap	5813

A.17.3.2.1	SA: RRC Re-establishment	5813

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5813

A.17.3.2.1.1.1	Test Purpose and Environment	5813

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5815

A.17.3.2.1.2.1	Test Purpose and Environment	5815

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5817

A.17.3.2.1.3.1	Test Purpose and Environment	5817

A.17.3.2.1.3.2	Test Requirements	5819

A.17.3.2.2	Random Access	5819

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5819

A.17.3.2.2.1.1	Test Purpose and Environment	5819

A.17.3.2.2.1.2	Test Requirements	5821

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5823

A.17.3.2.2.2.1	Test Purpose and Environment	5823

A.17.3.2.2.2.2	Test Requirements	5824

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5826

A.17.3.2.2.3.1	Test Purpose and Environment	5826

A.17.3.2.2.3.2	Test Requirements	5827

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5828

A.17.3.2.2.4.1	Test Purpose and Environment	5828

A.17.3.2.2.4.2	Test Requirements	5830

A.17.3.2.3	SA: RRC Connection Release with Redirection	5831

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5831

A.17.3.2.3.1.1	Test Purpose and Environment	5831

A.17.3.2.3.1.2	Test Parameters	5831

A.17.3.2.3.1.3	Test Requirements	5833

A.17.4	Timing	5833

A.17.4.1	UE transmit timing	5833

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5833

A.17.4.1.1.1	Test Purpose and environment	5833

A.17.4.1.1.2	Test requirements	5835

A.17.4.2	UE timer accuracy	5836

A.17.4.3	Timing advance	5836

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5836

A.17.4.3.1.1	Test Purpose and Environment	5836

A.17.4.3.1.2	Test Parameters	5836

A.17.4.3.1.3 Test Requirements	5839

A.17.5	Signaling characteristics for RedCap	5839

A.17.5.1	Radio link Monitoring for RedCap	5839

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5839

A.17.5.1.1.1	Test Purpose and Environment	5839

A.17.5.1.1.2	Test Requirements	5842

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5842

A.17.5.1.2.1	Test Purpose and Environment	5842

A.17.5.1.2.2	Test Requirements	5845

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5845

A.17.5.1.3.1	Test Purpose and Environment	5845

A.17.5.1.3.2	Test Requirements	5848

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5848

A.17.5.1.4.1	Test Purpose and Environment	5848

A.17.5.1.4.2	Test Requirements	5850

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5851

A.17.5.1.5.1	Test Purpose and Environment	5851

A.17.5.1.5.2	Test Requirements	5853

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5854

A.17.5.1.6.1	Test Purpose and Environment	5854

A.17.5.1.6.2	Test Requirements	5856

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5857

A.17.5.1.7.1	Test Purpose and Environment	5857

A.17.5.1.7.2	Test Requirements	5859

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5859

A.17.5.1.8.1	Test Purpose and Environment	5859

A.17.5.1.8.2	Test Requirements	5862

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5863

A.17.5.1.9.1	Test Purpose and Environment	5863

A.17.5.1.9.2	Test Requirements	5864

A.17.5.2	Beam Failure Detection and Link recovery procedures	5865

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5865

A.17.5.2.1.1	Test Purpose and Environment	5865

A.17.5.2.1.2	Test Requirements	5867

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5868

A.17.5.2.2.1	Test Purpose and Environment	5868

A.17.5.2.2.2	Test Requirements	5871

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5871

A.17.5.2.3.1	Test Purpose and Environment	5871

A.17.5.2.3.2	Test Requirements	5874

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5874

A.17.5.2.4.1	Test Purpose and Environment	5874

A.17.5.2.4.2	Test Requirements	5877

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5877

A.17.5.2.5.1	Test Purpose and Environment	5877

A.17.5.2.5.2	Test Requirements	5880

A.17.5.3	Active BWP switch for RedCap	5881

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5881

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5881

A.17.5.3.1.1.1	Test Purpose and Environment	5881

A.17.5.3.1.1.2	Test Requirements	5883

A.17.5.3.2	RRC-based Active BWP Switch	5883

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5883

A.17.5.3.2.1.1	Test Purpose and Environment	5883

A.17.5.3.2.1.2	Test Requirements	5886

A.17.5.4	Active TCI state switch delay	5886

A.17.5.4.1	MAC-CE based active TCI state switch	5886

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5886

A.17.5.4.1.1.1	Test Purpose and Environment	5886

A.17.5.4.1.1.2	Test Requirements	5889

A.17.5.4.2	RRC based active TCI state switch	5889

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5889

A.17.5.4.2.1.1	Test Purpose and Environment	5889

A.17.5.4.2.1.2	Test Requirements	5892

A.17.5.5	Uplink spatial relation switch delay	5892

A.17.5.5.1	MAC-CE based Spatial Relation switch	5892

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5892

A.17.5.5.1.1.1	Test Purpose and Environment	5892

A.17.5.5.1.1.2	Test Requirements	5894

A.17.5.5.2	RRC based spatial relation switch	5895

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5895

A.17.5.5.2.1.2	Test Requirements	5897

A.17.5.6	UE specific CBW change	5897

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5897

A.17.5.6.1.1	Test Purpose and Environment	5897

A.17.5.6.1.2	Test Requirements	5899

A.17.6	Measurement procedure for RedCap	5900

A.17.6.1	Intra-frequency Measurements	5900

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5900

A.17.6.1.1.1	Test purpose and Environment	5900

A.17.6.1.1.2	Test Requirements	5902

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5902

A.17.6.1.2.1	Test purpose and Environment	5902

A.7.6.1.2.2	Test Requirements	5903

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5903

A.17.6.1.3.1	Test purpose and Environment	5903

A.17.6.1.3.2	Test Requirements	5906

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5906

A.17.6.1.4.1	Test purpose and Environment	5906

A.17.6.1.4.2	Test Requirements	5908

A.17.6.2	Inter-frequency Measurements	5909

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5909

A.17.6.2.1.1	Test Purpose and Environment	5909

A.17.6.2.1.2	Test Requirements	5911

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5911

A.17.6.2.2.1	Test Purpose and Environment	5911

A.17.6.2.2.2	Test Requirements	5913

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5914

A.17.6.2.3.1	Test Purpose and Environment	5914

A.17.6.2.3.2	Test Requirements	5916

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5916

A.17.6.2.4.1	Test Purpose and Environment	5916

A.17.6.2.4.2	Test Requirements	5918

A.17.6.3	L1-RSRP measurement for beam reporting	5919

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5919

A.17.6.3.1.1	Test Purpose and Environment	5919

A.17.6.3.1.2	Test parameters	5919

A.17.6.3.1.3	Test Requirements	5919

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5919

A.17.6.3.2.1	Test Purpose and Environment	5919

A.17.6.3.2.2	Test parameters	5920

A.17.6.3.2.3	Test Requirements	5921

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5921

A.17.6.3.3.1	Test Purpose and Environment	5921

A.17.6.3.3.2	Test parameters	5921

A.17.6.3.3.3	Test Requirements	5923

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5923

A.17.6.3.4.1	Test Purpose and Environment	5923

A.17.6.3.4.2	Test parameters	5924

A.7.6.3.3.3	Test Requirements	5925

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5925

A.17.6.4.1.1	Test Purpose and Environment	5925

A.17.6.4.1.2	Test Requirements	5928

A.17.6.5	RSTD measurements	5928

A.17.6.5.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR2 SA	5928

A.17.6.5.1.1	Test Purpose and Environment	5928

A.17.6.5.1.2	Test Requirements	5935

A.17.6.5.2	NR RSTD measurement reporting delay test case with PRS frequency hopping	5935

A.17.6.5.2.1	Test Purpose and Environment	5935

A.17.6.5.2.2	Test Requirements	5940

A.17.6.6	UE Rx-Tx Measurements	5941

A.17.6.6.1	UE Rx-Tx measurement reporting delay for single positioning frequency layer in FR2 SA without RX FH in RRC_CONNECTED mode	5941

A.17.6.6.1.1	Test purpose and environment	5941

A.17.6.6.1.2	Test requirements	5945

A.17.6.6.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR2 SA in RRC_CONNECTED state	5945

A.17.6.6.2.1	Test purpose and environment	5945

A.17.6.6.2.2	Test requirements	5949

A.17.6.7	PRS-RSRP measurements	5949

A.17.6.7.1	PRS-RSRP measurement delay test case for RedCap positioning without Rx FH in RRC_CONNECTED state in FR2	5949

A.17.6.7.1.1	PRS-RSRP measurement delay test case for single positioning frequency layer	5949

A.17.6.7.1.1.1	Test Purpose and Environment	5949

A.17.6.7.1.1.2	Test Requirements	5953

A.17.6.7.1.2	PRS-RSRP measurement delay test case for dual positioning frequency layer	5953

A.17.6.7.1.2.1	Test Purpose and Environment	5953

A.17.6.7.1.2.2	Test Requirements	5957

A.17.6.7.2	PRS-RSRP measurement delay with FH in RRC_CONNECTED state in FR2	5957

A.17.6.7.2.1	Test Purpose and Environment	5957

A.17.6.7.2.2	Test Requirements	5961

A.17.6.8	PRS-RSRPP Measurements	5961

A.17.6.8.1	PRS-RSRPP measurement delay without FH in RRC_CONNECTED state in FR2	5961

A.17.6.8.1.1	Test Purpose and Environment	5961

A.17.6.8.1.2	Test Requirements	5964

A.17.6.8.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state	5964

A.17.6.8.2.1	Test Purpose and Environment	5964

A.17.6.8.2.2	Test Requirements	5966

A.17.7	Measurement Performance requirements	5967

A.17.7.1	SS-RSRP	5967

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5967

A.17.7.1.1.1	Test Purpose and Environment	5967

A.17.7.1.1.2	Test parameters	5967

A.17.7.1.1.3	Test Requirements	5969

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5969

A.17.7.1.2.1	Test Purpose and Environment	5969

A.17.7.1.2.2	Test parameters	5969

A.17.7.1.2.3	Test Requirements	5971

A.17.7.2	SS-RSRQ	5972

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5972

A.17.7.2.1.1	Test Purpose and Environment	5972

A.17.7.2.1.2	Test Parameters	5972

A.17.7.2.1.3	Test Requirements	5974

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5974

A.17.7.2.2.1	Test Purpose and Environment	5974

A.17.7.2.2.2	Test parameters	5974

A.17.7.2.2.3	Test Requirements	5976

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5976

A.17.7.3	L1-RSRP measurement for beam reporting	5976

A.17.7.3.1	SSB based L1-RSRP measurement	5976

A.17.7.3.1.1	Test Purpose and Environment	5976

A.17.7.3.1.2	Test parameters	5976

A.17.7.3.1.3	Test Requirements	5977

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5977

A.17.7.3.2.1	Test Purpose and Environment	5977

A.17.7.3.2.2	Test parameters	5977

A.17.7.3.2.3	Test Requirements	5977

A.17.7.4	SS-SINR	5978

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5978

A.17.7.4.1.1	Test Purpose and Environment	5978

A.17.7.4.1.2	Test parameters	5978

A.17.7.4.1.3	Test Requirements	5980

A.17.7.5	RSTD measurements	5980

A.17.7.5.1	RSTD measurement accuracy test case for RedCap UE without FH	5980

A.17.7.5.1.1	Test purpose and Environment	5980

A.17.7.5.1.2	Test Requirements	5982

A.17.7.6	UE Rx-Tx Measurements	5984

A.17.7.6.1	UE Rx-Tx measurement accuracy for single positioning frequency layer in FR2 SA without RX FH in RRC_CONNECTED mode	5984

A.17.7.6.1.1	Test purpose and environment	5984

A.17.7.6.1.2	Test parameters	5985

A.17.7.6.1.3	Test requirements	5988

A.17.7.6.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_CONNECTED state in FR2	5988

A.17.7.6.2.1	Test purpose and Environment	5988

A.17.7.6.2.2	Test parameters	5989

A.17.7.6.2.3	Test requirements	5992

A.17.7.7	PRS-RSRP Measurements	5992

A.17.7.7.1	PRS-RSRP measurement accuracy without FH in RRC_CONNECTED state in FR2	5992

A.17.7.7.1.1	Test Purpose and Environment	5992

A.17.7.7.1.2	Test parameters	5992

A.17.7.7.1.3	Test Requirements	5995

A.17.7.7.2	PRS-RSRP measurement accuracy with FH in RRC_CONNECTED state in FR2	5995

A.17.7.7.2.1	Test Purpose and Environment	5995

A.17.7.7.2.2	Test parameters	5996

A.17.7.7.2.3	Test Requirements	5998

A.17.7.8	PRS-RSRPP Measurements	5998

A.17.7.8.1	PRS-RSRPP measurement accuracy without FH in RRC_CONNECTED state in FR2	5998

A.17.7.8.1.1	Test Purpose and Environment	5998

A.17.7.8.1.2	Test parameters	5999

A.17.7.8.1.3	Test Requirements	6001

A.17.7.8.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_CONNECTED state in FR2	6001

A.17.7.8.2.1	Test purpose and Environment	6001

A.17.7.8.2.2	Test parameters	6002

A.17.7.8.2.3	Test requirements	6004

A.17.8	Measurement Procedure for RedCap in RRC_INACTIVE	6005

A.17.8.1	RSTD Measurements	6005

A.17.8.1.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR2 SA in RRC_INACTIVE state	6005

A.17.8.1.1.1	Test Purpose and Environment	6005

A.17.8.1.1.2	Test Requirements	6008

A.17.8.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	6008

A.17.8.1.2.1	Test Purpose and Environment	6008

A.17.8.1.2.2	Test Requirements	6011

A.17.8.1.3	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state with eDRX > 10.24s	6011

A.17.8.1.3.1	Test purpose and environment	6011

A.17.8.1.3.2	Test requirements	6011

A.17.8.2	UE Rx-Tx Measurements	6012

A.17.8.2.1	UE Rx-Tx measurement reporting delay for single positioning frequency layer in FR2 SA without RX FH in RRC_INACTIVE mode	6012

A.17.8.2.1.1	Test purpose and environment	6012

A.17.8.2.1.2	Test requirements	6015

A.17.8.2.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	6015

A.17.8.2.2.1	Test purpose and environment	6015

A.17.8.2.2.2	Test requirements	6019

A.17.8.2.3	UE Rx-Tx time difference measurements for single positioning frequency layer with eDRX > 10.24s in FR2 SA	6019

A.17.8.2.3.1	Test purpose and environment	6019

A.17.8.2.3.2	Test requirements	6019

A.17.8.3	PRS-RSRP Measurements	6020

A.17.8.3.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	6020

A.17.8.3.1.1	Test Purpose and Environment	6020

A.17.8.3.1.2	Test Requirements	6024

A.17.8.3.2.2	Test Requirements	6028

A.17.8.3.3	PRS-RSRP reporting delay in RRC_INACTIVE with eDRX	6028

A.17.8.3.3.1	Test Purpose and Environment	6028

A.17.8.3.3.2	Test Requirements	6032

A.17.8.4	PRS-RSRPP Measurements	6032

A.17.8.4.1	PRS-RSRPP measurement delay without FH in RRC_INACTIVE state in FR2	6032

A.17.8.4.1.1	Test Purpose and Environment	6032

A.17.8.4.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	6035

A.17.8.4.2.1	Test Purpose and Environment	6035

A.17.8.4.2.2	Test Requirements	6037

A.17.8.4.3	PRS-RSPP reporting delay in RRC_INACTIVE state with eDRX > 10.24s	6037

A.17.8.4.3.1	Test purpose and environment	6037

A.17.8.4.3.2	Test requirements	6037

A.17.9	Measurement Performance Requirements for RedCap in RRC_INACTIVE	6038

A.17.9.1	RSTD Measurements	6038

A.17.9.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR2 in RRC_INACTIVE state	6038

A.17.9.1.1.1	Test purpose and Environment	6038

A.17.9.1.1.2	Test Requirements	6040

A.17.9.1.2	RSTD measurement accuracy test case for RedCap UE with FH in FR2 in RRC_INACTIVE state	6040

A.17.9.1.2.1	Test purpose and Environment	6040

A.17.9.1.2.2	Test Requirements	6042

A.17.9.2	UE Rx-Tx Measurements	6042

A.17.9.2.1	UE Rx-Tx measurement accuracy for single positioning frequency layer in FR2 SA without RX FH in RRC_INACTIVE mode	6042

A.17.9.2.1.1	Test purpose and environment	6042

A.17.9.2.1.2	Test parameters	6043

A.17.9.2.1.3	Test requirements	6046

A.17.9.2.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_INACTIVE state in FR2	6046

A.17.9.2.2.1	Test purpose and Environment	6046

A.17.9.2.2.2	Test parameters	6046

A.17.9.2.2.3	Test requirements	6049

A.17.9.3	PRS-RSRP Measurements	6049

A.17.9.3.2	PRS-RSRP measurement accuracy with FH in RRC_INACTIVE state in FR2	6051

A.17.9.3.2.1	Test Purpose and Environment	6051

A.17.9.3.2.2	Test parameters	6052

A.17.9.3.2.3	Test Requirements	6053

A.17.9.4	PRS-RSRPP Measurements	6053

A.17.9.4.1	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_INACTIVE state in FR2	6053

A.17.9.4.1.1	Test Purpose and Environment	6053

A.17.9.4.1.2	Test parameters	6054

A.17.9.4.1.3	Test Requirements	6056

A.17.9.4.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_INACTIVE state in FR2	6056

A.17.9.4.2.1	Test Purpose and Environment	6056

A.17.9.4.2.2	Test parameters	6057

A.17.9.4.2.3	Test Requirements	6059

A.17.10	Measurement Procedure for RedCap in RRC_IDLE	6060

A.17.10.1	RSTD Measurements	6060

A.17.10.1.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR2 SA in RRC_IDLE state without eDRX	6060

A.17.10.1.1.1	Test Purpose and Environment	6060

A.17.10.1.1.2	Test Requirements	6063

A.17.10.1.2	NR RSTD without FH measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	6063

A.17.10.1.2.1	Test purpose and environment	6063

A.17.10.1.2.2	Test requirements	6063

A.17.10.2	PRS-RSRP Measurements	6064

A.17.10.2.1	PRS-RSRP measurement delay test case for single positioning frequency layer in RRC_IDLE	6064

A.17.10.2.1.1	Test Purpose and Environment	6064

A.17.10.2.1.2	Test Requirements	6068

A.17.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR2 when eDRX cycle > 10.24s	6068

A.17.10.2.2.1	Test Purpose and Environment	6068

A.17.10.2.2.2	Test Requirements	6068

A.17.11	 Measurement Performance Requirements for RedCap in RRC_IDLE	6069

A.17.11.1	RSTD Measurements	6069

A.17.11.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR2 in RRC_IDLE state without eDRX	6069

A.17.11.1.1.1	Test purpose and Environment	6069

11.1.1.2	Test Requirements	6071

A.17.11.1.2	RSTD without FH measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	6071

A.17.11.1.2.1	Test purpose and environment	6071

A.17.11.1.2.2	Test requirements	6073

A.17.11.2	PRS-RSRP Measurements	6073

A.17.11.2.1	PRS-RSRP measurement accuracy test case for RedCap UE in FR2 in RRC_IDLE state	6073

A.17.11.2.1.1	Test Purpose and Environment	6073

A.17.11.2.1.2	Test parameters	6073

A.17.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR2 when eDRX cycle > 10.24s	6075

A.17.11.2.2.1	Test purpose and Environment	6075

A.17.11.2.2.1	Test parameters	6076

A.17.11.2.2.2	Test Requirements	6076

A.18	E-UTRA standalone tests for NR RRM for RedCap	6076

A.18.1	RRC_IDLE state mobility	6076

A.18.1.1	Inter-RAT NR Cell re-selection	6076

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	6076

A.18.1.1.1.1	Test Purpose and Environment	6076

A.18.1.1.1.2	Test Requirements	6079

A.18.2	RRC_CONNECTED state mobility	6079

A.18.2.1	Handover	6079

A.18.2.1.1	E-UTRAN - NR handover in FR1	6079

A.18.2.1.1.1	Test Purpose and Environment	6079

A.18.2.1.1.2	Test Requirements	6083

A.18.2.2	RRC connection release with redirection	6083

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	6083

A.18.2.2.1.1	Test Purpose and Environment	6083

A.18.2.2.1.2	Test Parameters	6083

A.18.2.2.1.3	Test Requirements	6086

A.18.3	Measurement procedure	6087

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	6087

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	6087

A.18.3.1.1.1	Test Purpose and Environment	6087

A.18.3.1.1.2	Test Requirements	6090

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	6090

A.18.3.1.2.1	Test Purpose and Environment	6090

A.18.3.1.2.2	Test Requirements	6094

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	6094

A.18.3.1.3.1	Test Purpose and Environment	6094

A.18.3.1.3.2	Test Requirements	6098

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	6098

A.18.3.1.4.1	Test Purpose and Environment	6098

A.18.3.1.4.2	Test Requirements	6102

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	6102

A.18.3.1.5.1	Test Purpose and Environment	6102

A.18.3.1.5.2	Test Requirements	6104

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	6104

A.18.3.1.6.1	Test Purpose and Environment	6104

A.18.3.1.6.2	Test Requirements	6106

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	6107

A.18.3.1.7.1	Test Purpose and Environment	6107

A.18.3.1.7.2	Test Requirements	6108

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	6109

A.18.3.1.8.1	Test Purpose and Environment	6109

A.18.3.1.8.2	Test Requirements	6111

A.19	NR standalone tests for ATG	6112

A.19.1	RRC_IDLE state mobility	6112

A.19.1.1	Cell reselection to FR1 intra-frequency NR case	6112

A.19.1.1.1	Test Purpose and Environment	6112

A.19.1.1.2	Test Parameters	6112

A.19.1.1.3	Test Requirements	6113

A.19.1.2	Cell reselection to FR1 inter-frequency NR case	6113

A.19.1.2.1	Test Purpose and Environment	6113

A.19.1.2.2	Test Parameters	6113

A.19.1.2.3	Test Requirements	6115

A.19.1.3	Cell reselection to FR1 inter-frequency NR case for UE configured with hs-ATG-cellReselectionSet-r18	6116

A.19.1.3.1	Test Purpose and Environment	6116

A.19.1.3.2	Test Parameters	6116

A.19.1.3.3	Test Requirements	6118

A.19.2	RRC_CONNECTED state mobility	6119

A.19.2.1	Handover	6119

A.19.2.1.1	Intra-frequency handover from FR1 to FR1; known target cell	6119

A19.2.1.1.1	Test Purpose and Environment	6119

A.19.2.1.1.2	Test Parameters	6119

A.19.2.1.2.3	Test Requirements	6119

A.19.2.1.2	Inter-frequency handover from FR1 to FR1; unknown target cell	6120

A.19.2.1.2.1	Test Purpose and Environment	6120

A.19.2.1.2.2	Test Parameters	6120

A.19.2.1.2.3	Test Requirements	6121

A.19.2.2	Conditional Handover	6121

A.19.2.2.1	Intra-frequency distance-based conditional Handover from FR1 to FR1	6121

A.19.2.2.1.1	Test Purpose and Environment	6121

A.19.2.2.1.2	Test Parameters	6121

A.19.2.2.1.3	Test Requirements	6123

A.19.2.2.2	Inter-frequency distance-based conditional Handover from FR1 to FR1	6124

A.19.2.2.2.1	Test Purpose and Environment	6124

A.19.2.2.2.2	Test Parameters	6124

A.19.2.2.2.3	Test Requirements	6126

A.19.2.3	RRC Connection Mobility Control	6126

A.19.2.3.1	SA: RRC Re-establishment	6126

A.19.2.3.1.1	Intra-frequency RRC Re-establishment in FR1 for ATG	6126

A.19.2.3.1.1.1	Test Purpose and Environment	6127

A.19.2.3.1.1.2 Test Requirements	6127

A.19.2.3.1.2	Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG	6127

A.19.2.3.1.2.1	Test Purpose and Environment	6127

A.19.2.3.1.2.2	Test Requirements	6129

A.19.2.3.2	Random Access for ATG UE	6130

A.19.2.3.2.1.1	Test Purpose and Environment	6130

A.19.2.3.2.1.2	Test Requirements	6131

A.19.2.3.2.2.1	Test Purpose and Environment	6131

A.19.2.3.2.2.2	Test Requirements	6131

A.19.2.3.2.3	2-step RA type contention based random access test in FR1 for NR standalone	6131

A.19.2.3.2.3.1	Test Purpose and Environment	6131

A.19.2.3.2.3.2	Test Requirements	6132

A.19.2.3.2.4	2-step RA type non-contention based test in FR1 for NR standalone	6132

A.19.2.3.2.4.1	Test Purpose and Environment	6132

A.19.2.3.2.4.2	Test Requirements	6133

A.19.2.3.3.1.1	Test Purpose and Environment	6133

A.19.2.3.3.1.2	Test Parameters	6133

A.19.2.3.3.1.3	Test Requirements	6133

A.19.3	Timing	6134

A.19.3.1	UE transmit timing	6134

A.19.3.1.1	ATG UE Transmit Timing Test for FR1	6134

A.19.3.1.1.1	Test Purpose and environment	6134

A.19.3.1.1.2	Test requirements	6135

A.19.3.2	UE timer accuracy	6136

A.19.3.3	Timing advance	6136

A.19.3.3.1	SA FR1 timing advance adjustment accuracy	6136

A.19.3.3.1.1	Test Purpose and Environment	6136

A.19.3.3.1.2	Test Parameters	6136

A.19.3.3.1.3	Test Requirements	6137

A.19.4	Signalling characteristics	6137

A.19.4.1	Radio link Monitoring	6137

A.19.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6137

A.19.4.1.1.1	Test Purpose and Environment	6137

A.19.4.1.1.2	Test Requirements	6140

A.19.4.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6140

A.19.4.1.2.1	Test Purpose and Environment	6140

A.19.4.1.2.2	Test Requirements	6143

A.19.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	6143

A.19.4.1.3.1	Test Purpose and Environment	6143

A.19.4.1.3.2	Test Requirements	6146

A.19.4.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	6147

A.19.4.1.4.1	Test Purpose and Environment	6147

A.19.4.1.4.2	Test Requirements	6150

A.19.4.2	Beam Failure Detection and Link recovery procedures	6150

A.19.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	6150

A.19.4.2.1.1	Test Purpose and Environment	6150

A.19.4.2.1.2	Test Requirements	6154

A.19.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	6154

A.19.4.2.2.1	Test Purpose and Environment	6154

A.19.4.2.2.2	Test Requirements	6158

A.19.4.3	Active BWP switch	6158

A.19.4.3.1	DCI-based and Timer-based Active BWP Switch	6158

A.19.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	6158

A.19.4.3.2	RRC-based Active BWP Switch	6161

A.19.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	6161

A.19.4.4	UE specific CBW change	6163

A19.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	6163

A19.4.4.1.1	Test Purpose and Environment	6163

A.19.4.4.1.2	Test Requirements	6166

A.19.4.5	Pathloss reference signal switching delay	6166

A.19.4.5.1	MAC-CE based pathloss reference signal switch delay	6166

A.19.4.5.1.1	Test Purpose and Environment	6166

A.19.4.5.1.2	Test Requirements	6168

A.19.5	Measurement procedure	6169

A.19.5.1	Intra-frequency Measurements	6169

A.19.5.1.1	SA event triggered reporting tests without gap without SSB index reading under non-DRX	6169

A.19.5.1.1.1	Test purpose and Environment	6169

A.19.5.1.1.2	Test parameters	6169

A.19.5.1.1.3	Test Requirements	6169

A.19.5.1.2	SA event triggered reporting tests with per-UE gaps under non-DRX	6170

A.19.5.1.2.1	Test purpose and Environment	6170

A.19.5.1.2.2	Test parameters	6170

A.19.5.1.2.3	Test Requirements	6170

A.19.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	6170

A.19.5.1.3.1	Test purpose and Environment	6170

A.19.5.1.3.2	Test parameters	6171

A.19.5.1.3.3	Test Requirements	6171

A.19.5.1.4	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	6171

A.19.5.1.4.1	Test purpose and Environment	6171

A.19.5.1.4.2	Test parameters	6171

A.19.5.1.4.3	Test Requirements	6172

A.19.5.2	Inter-frequency Measurements	6172

A.19.5.2.1.2	Test parameters	6172

A.19.5.2.1.3	Test Requirements	6173

A.19.5.2.2.2	Test parameters	6174

A.19.5.2.3.2	Test parameters	6174

A.19.5.2.3.3	Test Requirements	6175

A.19.5.3	L1-RSRP measurement for beam reporting for ATG	6175

A.19.5.3.1	SSB based L1-RSRP measurement when DRX is not used	6175

A.19.5.3.1.1	Test Purpose and Environment	6175

A.19.5.3.1.2	Test parameters	6175

A.19.5.3.1.3	Test Requirements	6176

A.19.5.3.2	CSI-RS based L1-RSRP measurement when DRX is not used	6176

A.19.5.3.2.1	Test Purpose and Environment	6176

A.19.5.3.2.2	Test parameters	6176

A.19.5.3.2.3	Test Requirements	6176

A.19.5.4	L1-SINR measurement for beam reporting for ATG	6176

A.19.5.4.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	6176

A.19.5.4.1.3	Test Requirements	6177

A.19.5.4.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	6177

A.19.5.4.2.1	Test Purpose and Environment	6177

A.19.5.4.2.2	Test parameters	6177

A.19.5.4.2.3	Test Requirements	6178

A.19.5.4.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	6178

A.19.5.4.3.1	Test Purpose and Environment	6178

A.19.5.4.3.2	Test parameters	6178

A.19.5.4.3.3	Test Requirements	6178

A.19.5.5	NR measurements with autonomous gaps for ATG	6179

A.19.5.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	6179

A.19.5.5.1.1	Test Purpose and Environment	6179

A.19.5.5.1.2	Test Parameters	6179

A.19.5.5.1.3	Test Requirements	6179

A.19.6	Measurement Performance requirements	6179

A.19.6.1	SS-RSRP for ATG UE	6180

A.19.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6180

A.19.6.1.1.1	Test Purpose and Environment	6180

A.19.6.1.1.2	Test parameters	6180

A.19.6.1.1.3	Test Requirements	6180

A.19.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6180

A.19.6.1.2.1	Test Purpose and Environment	6180

A.19.6.1.2.2	Test parameters	6181

A.19.6.1.2.3	Test Requirements	6181

A.19.6.2	SS-RSRQ for ATG UE	6181

A.19.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6181

A.19.6.2.1.1	Test Purpose and Environment	6181

A.19.6.2.1.2	Test Parameters	6181

A.19.6.2.1.3	Test Requirements	6182

A.19.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6182

A.19.6.2.2.1	Test Purpose and Environment	6182

A.19.6.2.2.2	Test Parameters	6182

A.19.6.2.2.3	Test Requirements	6182

A.19.6.3	SS-SINR for ATG UE	6183

A.19.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6183

A.19.6.3.1.1	Test Purpose and Environment	6183

A.19.6.3.1.2	Test Parameters	6183

A.19.6.3.1.3	Test Requirements	6183

A.19.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6183

A.19.6.3.2.1	Test Purpose and Environment	6183

A.19.6.3.2.2	Test Parameters	6183

A.19.6.3.2.3	Test Requirements	6184

A.19.6.4	L1-RSRP measurement for beam reporting for ATG UE	6184

A.19.6.4.1	SSB based L1-RSRP measurement	6184

A.19.6.4.1.1	Test Purpose and Environment	6184

A.19.6.4.1.2	Test parameters	6184

A.19.6.4.1.3	Test Requirements	6185

A.19.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	6185

A.19.6.4.2.1	Test Purpose and Environment	6185

A.19.6.4.2.2	Test parameters	6185

A.19.6.4.2.3	Test Requirements	6185

A.19.6.5	L1-SINR measurement for beam reporting based CMR for ATG UE	6186

A.19.6.5.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	6186

A.19.6.5.1.1	Test Purpose and Environment	6186

A.19.6.5.1.2	Test parameters	6186

A.19.6.5.1.3	Test Requirements	6186

A.19.6.5.2	L1-SINR measurement with SSB based CMR and dedicated IMR	6186

A.19.6.5.2.1	Test Purpose and Environment	6186

A.19.6.5.2.2	Test parameters	6187

A.19.6.5.2.3	Test Requirements	6187

A.19.6.5.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	6187

A.19.6.5.3.1	Test Purpose and Environment	6187

A.19.6.5.3.2	Test parameters	6187

A.19.6.5.3.3	Test Requirements	6188

A.19.6.6	CSI-RSRP for ATG UE	6188

A.19.6.6.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6188

A.19.6.6.1.1	Test Purpose and Environment	6188

A.19.6.6.1.2	Test parameters	6188

A.19.6.6.1.3	Test Requirements	6188

A.19.6.6.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6189

A.19.6.6.2.1	Test Purpose and Environment	6189

A.19.6.6.2.2	Test parameters	6189

A.19.6.6.2.3	Test Requirements	6189

A.19.6.7	CSI-RSRQ for ATG UE	6189

A.19.6.7.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6189

A.19.6.7.1.1	Test Purpose and Environment	6189

A.19.6.7.1.2	Test Parameters	6189

A.19.6.7.1.3	Test Requirements	6190

A.19.6.7.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6190

A.19.6.7.2.1	Test Purpose and Environment	6190

A.19.6.7.2.2	Test Parameters	6190

A.19.6.7.2.3	Test Requirements	6191

A.19.6.8	CSI-SINR for ATG UE	6191

A.19.6.8.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6191

A.19.6.8.1.1	Test Purpose and Environment	6191

A.19.6.8.1.2	Test Parameters	6191

A.19.6.8.1.3	Test Requirements	6191

A.19.6.8.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6192

A.19.6.8.2.1	Test Purpose and Environment	6192

A.19.6.8.2.2	Test Parameters	6192

A.19.6.8.2.3	Test Requirements	6192

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	6193

B.1	Conditions for NR RRC_IDLE state mobility	6193

B.1.1	Introduction	6193

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	6193

B.1.2A	Conditions for measurements on NR intra-frequency cells under CCA for cell re-selection	6195

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	6196

B.1.3A	Conditions for measurements on NR inter-frequency cells under CCA for cell re-selection	6196

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	6196

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	6199

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	6199

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	6199

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	6199

B.2.1	Introduction	6199

B.2.1.1	General	6199

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	6200

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	6200

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	6200

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	6201

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	6201

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	6201

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	6201

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	6202

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	6202

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	6203

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	6203

B.2.1.7	Derivation of Minimum SSB_RP values for FR2-NTN for satellite access	6203

B.2.1.7.1	Minimum SSB_RP values for Rx Beam	6203

B.2.1.8	Gain to SS-RSRP for FR2-NTN for satellite access	6204

B.2.2	Conditions for NR intra-frequency measurements	6205

B.2.3	Conditions for NR inter-frequency measurements	6207

B.2.4	Conditions for NR L1-RSRP reporting	6209

B.2.4.1	Conditions for SSB based L1-RSRP reporting	6209

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	6211

B.2.5	Conditions for RRC connection release with redirection to NR	6213

B.2.6	Void	6215

B.2.6.1	Void	6215

B.2.6.2	Void	6215

B.2.7	Conditions for SRS-RSRP measurements	6215

B.2.8	Conditions for NR L1-SINR reporting	6216

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	6216

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	6218

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	6218

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	6220

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	6222

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	6222

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	6224

B.2.9	Conditions for NR intra-frequency measurements under CCA	6226

B.2.10	Conditions for NR inter-frequency measurements under CCA	6226

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6226

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6226

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	6227

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	6228

B.2.14	Conditions for NR PRS-based measurements	6229

B.2.15	Conditions for NR intra-frequency measurements for RedCap	6231

B.2.16	Conditions for NR inter-frequency measurements for RedCap	6232

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6234

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6234

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6234

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6235

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6235

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6235

B.3	RRM Requirements Exceptions	6235

B.3.1	Introduction	6235

B.3.2	Receiver sensitivity relaxation for CA	6236

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6236

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6236

B.3.2.2.1	Inter-band carrier aggregation	6236

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6236

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6236

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6237

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6237

B.3.2.4.1	Intra-band contiguous carrier aggregation	6237

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6237

B.3.3	Receiver sensitivity relaxation for DC	6237

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6237

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6237

B.3.4	Receiver sensitivity relaxation for SUL	6237

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6237

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6237

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6237

B.4	Conditions for V2X	6238

B.4.1	Test parameters for GNSS signals	6238

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6238

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6238

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6239

B.4.5	Conditions for PSBCH-RSRP Accuracy Requirements under CCA	6239

B.4.6	Conditions for Selection/Reselection to Intra-frequency SyncRef UE under CCA	6239

B.4.7	Conditions for L1 SL-RSRP Accuracy Requirements under CCA	6240

B.4A	Conditions for NR Sidelink Positioning Measurement Procedures and Performance Requirements	6240

B.4A.1	Conditions for NR SL-PRS based measurements	6240

B.5	High level test procedure for SAN RRM tests	6240

Annex C (informative):	Change history	6242