3GPP TS 38.133 V15.11.0 (2020-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Requirements for support of radio resource management

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	31

1	Scope	32

2	References	32

3	Definitions, symbols and abbreviations	33

3.1	Definitions	33

3.2	Symbols	34

3.3	Abbreviations	34

3.4	Test tolerances	36

3.5	Frequency bands grouping	36

3.5.1	Introduction	36

3.5.2	NR operating bands in FR1	36

3.5.3	NR operating bands in FR2	36

3.6	Applicability of requirements in this specification version	37

3.6.1	RRC connected state requirements in DRX	37

3.6.2	Number of serving carriers	38

3.6.2.1	Number of serving carriers for SA	38

3.6.2.2	Number of serving carriers for EN-DC	38

3.6.2.3	Number of serving carriers for NE-DC	38

3.6.2.4	Number of serving carriers for NR-DC	38

3.6.3	Applicability for intra-band FR2	38

3.6.4	Applicability for FR2 UE power classes	39

3.6.5	Applicability for SDL bands	39

3.6.6	Applicability of requirements for NGEN-DC operation	39

3.6.7	Applicability of QCL	39

4	SA: RRC_IDLE state mobility	39

4.1	Cell Selection	39

4.2	Cell Re-selection	39

4.2.1	Introduction	39

4.2.2	Requirements	40

4.2.2.1	UE measurement capability	40

4.2.2.2	Measurement and evaluation of serving cell	40

4.2.2.3 	Measurements of intra-frequency NR cells	40

4.2.2.4	Measurements of inter-frequency NR cells	41

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	43

4.2.2.6	Maximum interruption in paging reception	44

4.2.2.7	General requirements	44

5	SA: RRC_INACTIVE state mobility	45

5.1	Cell Re-selection	45

5.1.1	Introduction	45

5.1.2	Requirements	45

5.1.2.1	UE measurement capability	45

5.1.2.2	Measurement and evaluation of serving cell	45

5.1.2.3	Measurements of intra-frequency NR cells	45

5.1.2.4	Measurements of inter-frequency NR cells	45

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	45

5.1.2.6	Maximum interruption in paging reception	45

5.1.2.7	General requirements	45

5.2	Void	46

6	RRC_CONNECTED state mobility	46

6.1	Handover	46

6.1.1	NR Handover	46

6.1.1.1	Introduction	46

6.1.1.2	NR FR1 - NR FR1 Handover	46

6.1.1.2.1	Handover delay	46

6.1.1.2.2	Interruption time	46

6.1.1.3	NR FR2- NR FR1 Handover	47

6.1.1.3.1	Handover delay	47

6.1.1.3.2	Interruption time	47

6.1.1.4	NR FR2- NR FR2 Handover	48

6.1.1.4.1	Handover delay	48

6.1.1.4.2	Interruption time	48

6.1.1.5	NR FR1- NR FR2 Handover	49

6.1.1.5.1	Handover delay	49

6.1.1.5.2	Interruption time	49

6.1.2	NR Handover to other RATs	50

6.1.2.1	NR – E-UTRAN Handover	50

6.1.2.1.1	Introduction	50

6.1.2.1.2	Handover delay	50

6.1.2.1.3	Interruption time	50

6.2	RRC Connection Mobility Control	50

6.2.1	SA: RRC Re-establishment	50

6.2.1.1	Introduction	50

6.2.1.2	Requirements	51

6.2.1.2.1	UE Re-establishment delay requirement	51

6.2.2	Random access	52

6.2.2.1	Introduction	52

6.2.2.2	Requirements	52

6.2.2.2.1	Contention based random access	52

6.2.2.2.2	Non-Contention based random access	53

6.2.2.2.3	UE behaviour when configured with supplementary UL	54

6.2.3	SA: RRC Connection Release with Redirection	54

6.2.3.1	Introduction	54

6.2.3.2	Requirements	55

6.2.3.2.1	RRC connection release with redirection to NR	55

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	55

7	Timing	56

7.1	UE transmit timing	56

7.1.1	Introduction	56

7.1.2	Requirements	56

7.1.2.1	Gradual timing adjustment	57

7.1.2.2	Void	58

7.2	UE timer accuracy	58

7.2.1	Introduction	58

7.2.2	Requirements	58

7.3	Timing advance	58

7.3.1	Introduction	58

7.3.2	Requirements	58

7.3.2.1	Timing Advance adjustment delay	58

7.3.2.2	Timing Advance adjustment accuracy	59

7.4	Cell phase synchronization accuracy	59

7.4.1	Definition	59

7.4.2	Minimum requirements	59

7.5	Maximum Transmission Timing Difference	59

7.5.1	Introduction	59

7.5.2	Minimum Requirements for inter-band EN-DC	59

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	60

7.5.3	Minimum Requirements for intra-band EN-DC	60

7.5.4	Minimum Requirements for NR Carrier Aggregation	60

7.5.5	Minimum Requirements for inter-band NE-DC	61

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	61

7.5.6	Minimum Requirements for inter-band NR DC	61

7.6	Maximum Receive Timing Difference	62

7.6.1	Introduction	62

7.6.2	Minimum Requirements for inter-band EN-DC	62

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	62

7.6.3	Minimum Requirements for intra-band EN-DC	63

7.6.4	Minimum Requirements for NR Carrier Aggregation	63

7.6.5	Minimum Requirements for inter-band NE-DC	64

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	64

7.6.6	Minimum Requirements for inter-band NR DC	64

7.7	deriveSSB-IndexFromCell tolerance	65

7.7.1	Minimum requirements	65

7.8	Void	65

8	Signalling characteristics	65

8.1	Radio Link Monitoring	65

8.1.1	Introduction	65

8.1.2	Requirements for SSB based radio link monitoring	66

8.1.2.1	Introduction	66

8.1.2.2	Minimum requirement	67

8.1.2.3	Measurement restrictions for SSB based RLM	68

8.1.3	Requirements for CSI-RS based radio link monitoring	69

8.1.3.1	Introduction	69

8.1.3.2	Minimum requirement	70

8.1.3.3	Measurement restrictions for CSI-RS based RLM	72

8.1.4	Minimum requirement at transitions	72

8.1.5	Minimum requirement for UE turning off the transmitter	73

8.1.6	Minimum requirement for L1 indication	73

8.1.7	Scheduling availability of UE during radio link monitoring	73

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	73

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	73

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	74

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	74

8.2	Interruption	74

8.2.1	EN-DC Interruption	74

8.2.1.1	Introduction	74

8.2.1.2	Requirements	75

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	75

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	75

8.2.1.2.3	Interruptions at SCell addition/release	75

8.2.1.2.4	Interruptions at SCell activation/deactivation	76

8.2.1.2.5	Interruptions during measurements on SCC	77

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	78

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	78

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	79

8.2.2.1	Introduction	79

8.2.2.2	Requirements	80

8.2.2.2.1	Interruptions at SCell addition/release	80

8.2.2.2.2	Interruptions at SCell activation/deactivation	80

8.2.2.2.3	Interruptions during measurements on deactivated SCC	81

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	81

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	82

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	83

8.2.3	NE-DC Interruptions	84

8.2.3.1	Introduction	84

8.2.3.2	Requirements	84

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	84

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	84

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	85

8.2.3.2.4	Interruptions at SCell activation/deactivation	86

8.2.3.2.5	Interruptions during measurements on SCC	87

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	87

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	88

8.2.4	NR-DC: Interruptions	88

8.2.4.1	Introduction	88

8.2.4.2	Requirements	88

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	88

8.2.4.2.2	Interruptions at SCell activation/deactivation	89

8.2.4.2.3	Interruptions during measurements on SCC	90

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	90

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	90

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	91

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	91

8.3	SCell Activation and Deactivation Delay	91

8.3.1	Introduction	91

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	91

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	94

8.4	UE UL carrier RRC reconfiguration delay	95

8.4.1	Introduction	95

8.4.2	UE UL carrier configuration delay requirement	95

8.4.3	UE UL carrier deconfiguration delay requirement	95

8.5	Link Recovery Procedures	95

8.5.1	Introduction	95

8.5.2	Requirements for SSB based beam failure detection	96

8.5.2.1	Introduction	96

8.5.2.2	Minimum requirement	96

8.5.2.3	Measurement restriction for SSB based beam failure detection	98

8.5.3	Requirements for CSI-RS based beam failure detection	98

8.5.3.1	Introduction	98

8.5.3.2	Minimum requirement	99

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	101

8.5.4	Minimum requirement for L1 indication	101

8.5.5	Requirements for SSB based candidate beam detection	102

8.5.5.1	Introduction	102

8.5.5.2	Minimum requirement	102

8.5.5.3	Measurement restriction for SSB based candidate beam detection	104

8.5.6	Requirements for CSI-RS based candidate beam detection	104

8.5.6.1	Introduction	104

8.5.6.2	Minimum requirement	104

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	106

8.5.7	Scheduling availability of UE during beam failure detection	106

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	107

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	107

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	107

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	107

8.5.8	Scheduling availability of UE during candidate beam detection	108

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	108

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	108

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	108

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	109

8.6	Active BWP switch delay	109

8.6.1	Introduction	109

8.6.2	DCI and timer based BWP switch delay	109

8.6.3	RRC based BWP switch delay	110

8.7	Void	110

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	110

8.8.1	Introduction	110

8.8.2	E-UTRAN PSCell Addition Delay Requirement	110

8.8.3	E-UTRAN PSCell Release Delay Requirement	111

8.9	NR-DC: PSCell Addition and Release Delay	111

8.9.1	Introduction	111

8.9.2	PSCell Addition Delay Requirement	111

8.9.3	PSCell Release Delay Requirement	112

8.10	Active TCI state switching delay	112

8.10.6	Active TCI state list update delay	114

8.11	PSCell Change	114

9	Measurement Procedure	115

9.1	General measurement requirement	115

9.1.1	Introduction	115

9.1.2	Measurement gap	115

9.1.2.1	EN-DC: Measurement Gap Sharing	122

9.1.2.1a	SA: Measurement Gap Sharing	123

9.1.2.1b	NE-DC: Measurement Gap Sharing	123

9.1.2.1c	NR-DC: Measurement Gap Sharing	124

9.1.3	UE Measurement capability	125

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	125

9.1.3.1a	SA: Monitoring of multiple layers using gaps	125

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	126

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	126

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	127

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	127

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	128

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	128

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	129

9.1.4.1	Introduction	129

9.1.4.2	Requirements	129

9.1.5	Carrier-specific scaling factor	130

9.1.5.1	Monitoring of multiple layers outside gaps	130

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	131

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	131

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	132

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	132

9.1.5.2	Monitoring of multiple layers within gaps	133

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed within gaps	133

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed within gaps	135

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps	136

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps	137

9.1.6	Minimum requirement at transitions	138

9.2	NR intra-frequency measurements	139

9.2.1	Introduction	139

9.2.2	Requirements applicability	139

9.2.3	Number of cells and number of SSB	139

9.2.3.1	Requirements for FR1	139

9.2.3.2	Requirements for FR2	140

9.2.4	Measurement Reporting Requirements	140

9.2.4.1	Periodic Reporting	140

9.2.4.2	Event-triggered Periodic Reporting	140

9.2.4.3	Event Triggered Reporting	140

9.2.5	Intrafrequency measurements without measurement gaps	141

9.2.5.1	Intrafrequency cell identification	141

9.2.5.2	Measurement period	143

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	144

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	144

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	144

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	145

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	145

9.2.5.4	SFTD Measurements between PCell and PSCell	145

9.2.5.4.1	Introduction	145

9.2.5.4.2	SFTD Measurement delay	146

9.2.5.4.3	SFTD Measurement Reporting Delay	146

9.2.6	Intra-frequency measurements with measurement gaps	147

9.2.6.1	Void	147

9.2.6.2	Intra-frequency cell identification	147

9.2.6.3	Intra-frequency Measurement Period	148

9.3	NR inter-frequency measurements	148

9.3.1	Introduction	148

9.3.2	Requirements applicability	149

9.3.2.1	Void	149

9.3.2.2	Void	149

9.3.3	Number of cells and number of SSB	149

9.3.3.1	Requirements for FR1	149

9.3.3.2	Requirements for FR2	149

9.3.4	Inter-frequency cell identification	150

9.3.4.1	Void	151

9.3.4.2	Void	151

9.3.5	Inter-frequency measurements	151

9.3.5.1	Void	152

9.3.5.2	Void	152

9.3.5.3	Void	152

9.3.6	Inter-frequency measurements reporting requirements	152

9.3.6.1	Periodic Reporting	152

9.3.6.2	Event-triggered Periodic Reporting	152

9.3.6.3	Event-triggered Reporting	152

9.3.7	Void	152

9.3.8	Inter-frequency SFTD measurement requirements	152

9.3.8.1	Introduction	152

9.3.8.2	SFTD Measurement delay	153

9.3.8.3	SFTD Measurement reporting delay	154

9.4	Inter-RAT measurements	154

9.4.1	Introduction	154

9.4.2	NR − E-UTRAN FDD measurements	155

9.4.2.1	Introduction	155

9.4.2.2	Requirements when no DRX is used	156

9.4.2.3	Requirements when DRX is used	156

9.4.2.4	Measurement reporting requirements	157

9.4.2.4.1	Periodic Reporting	157

9.4.2.4.2	Event-Triggered Periodic Reporting	157

9.4.2.4.3	Event-Triggered Reporting	157

9.4.3	NR − E-UTRAN TDD measurements	158

9.4.3.1	Introduction	158

9.4.3.2	Requirements when no DRX is used	158

9.4.3.3	Requirements when DRX is used	159

9.4.3.4	Measurement reporting requirements	159

9.4.3.4.1	Periodic Reporting	159

9.4.3.4.2	Event-Triggered Periodic Reporting	160

9.4.3.4.3	Event-Triggered Reporting	160

9.4.4	Inter-RAT RSTD measurements	160

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	160

9.4.4.1.1	Introduction	160

9.4.4.1.2	Requirements	161

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	164

9.4.4.2.1	Introduction	164

9.4.4.2.2	Requirements	165

9.4.5	Inter-RAT E-CID measurements	168

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	168

9.4.5.1.1	Introduction	168

9.4.5.1.2	Requirements	168

9.4.5.1.3	Measurement Reporting Delay	168

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	169

9.4.5.2.1	Introduction	169

9.4.5.2.2	Requirements	169

9.4.5.2.3	Measurement Reporting Delay	169

9.5	L1-RSRP measurements for Reporting	169

9.5.1	Introduction	169

9.5.2	Requirements applicability	169

9.5.3	Measurement Reporting Requirements	170

9.5.3.1	Periodic Reporting	170

9.5.3.2	Semi-Persistent Reporting	170

9.5.3.3	Aperiodic Reporting	170

9.5.4	L1-RSRP measurement requirements	170

9.5.4.1	SSB based L1-RSRP Reporting	170

9.5.4.2	CSI-RS based L1-RSRP Reporting	172

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	174

9.5.5.1	Measurement restriction for SSB based L1-RSRP	175

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	175

9.5.6	Scheduling availability of UE during L1-RSRP measurement	176

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	176

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	176

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	176

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	177

9.6	NE-DC: Measurements	177

9.6.1	Introduction	177

9.6.2	SFTD Measurements	177

9.6.2.1	Introduction	177

9.6.2.2	SFTD Measurement requirements	177

10	Measurement Performance requirements	178

10.1	NR measurements	178

10.1.1	Introduction	178

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	179

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	179

10.1.2.1.1	Absolute SS-RSRP Accuracy	179

10.1.2.1.2	Relative SS-RSRP Accuracy	179

10.1.2.2	Void	180

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	180

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	180

10.1.3.1.1	Absolute SS-RSRP Accuracy	180

10.1.3.1.2	Relative SS-RSRP Accuracy	181

10.1.3.2	Void	182

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	182

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	182

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	182

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	182

10.1.4.2	Void	183

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	183

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	183

10.1.5.1.1	Absolute SS-RSRP Accuracy	183

10.1.5.1.2	Relative SS-RSRP Accuracy	184

10.1.5.2	Void	185

10.1.6 RSRP Measurement Report Mapping	185

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	187

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	187

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	187

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	188

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	188

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	188

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	189

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	189

10.1.9.1.1	Aboslute Accuracy of SS-RSRQ in FR1	189

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	190

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	190

10.1.11	RSRQ report mapping	192

10.1.12	Intra-frequency SINR accuracy requirements for FR1	192

10.1.13	Intra-frequency SINR accuracy requirements for FR2	193

10.1.14	Inter-frequency SINR accuracy requirements for FR1	194

10.1.15	Inter-frequency SINR accuracy requirements for FR2	195

10.1.16	SINR report mapping	197

10.1.17	Power Headroom	197

10.1.18	PCMAX,c,f	198

10.1.19	L1-RSRP accuracy requirements for FR1	198

10.1.20	L1-RSRP accuracy requirements for FR2	202

10.1.21	SFTD accuracy requirements	205

10.2	E-UTRAN measurements	209

10.2.1	Introduction	209

10.2.2	E-UTRAN RSRP measurements	209

10.2.3	E-UTRAN RSRQ measurements	210

10.2.4	E-UTRAN RSTD measurements	210

10.2.5	E-UTRAN RS-SINR measurements	210

11	Void	210

Annex A (normative):	Test Cases	209

A.1	Purpose of annex	209

A.2	Requirement classification for statistical testing	209

A.2.1	Types of requirements in TS 38.133	209

A.2.1.1	Time and delay requirements on UE higher layer actions	209

A.2.1.2	Measurements of power levels, relative powers and time	210

A.2.1.3	Implementation requirements	210

A.2.1.4	Physical layer timing requirements	210

A.3	RRM test configurations	211

A.3.1	Reference measurement channels	211

A.3.1.1	PDSCH	211

A.3.1.1.1	FDD	211

A.3.1.1.2	TDD	212

A.3.1.2	CORESET for RMSI scheduling	215

A.3.1.2.1	FDD	215

A.3.1.2.2	TDD	216

A.3.1.3	CORESET for RMC scheduling	219

A.3.1.3.1	FDD	219

A.3.1.3.2	TDD	220

A.3.1.4	TDD UL/DL configuration	223

A.3.2	OFDMA channel noise generator (OCNG)	224

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	224

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	224

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	225

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as PDSCH RMC	225

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	226

A.3.2.2	Void	226

A.3.3	Reference DRX configurations	226

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	226

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	227

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	227

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	227

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	228

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	228

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	228

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	229

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	229

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms	229

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	230

A.3.4	Test Cases with Different Channel Bandwidths	230

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	230

A.3.4.1.1	Introduction	230

A.3.4.1.2	Principle of testing	230

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	230

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	230

A.3.5.1.1	Introduction	230

A.3.5.1.2	Principle of Testing	230

A.3.6	Antenna configurations	231

A.3.6.1	Antenna configurations for FR1	231

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	231

A.3.6.1.1.1	Introduction	231

A.3.6.1.1.2	Principle of testing	231

A.3.6.2	Antenna configurations for FR2	233

A.3.7	EN-DC test setup	234

A.3.7.1	Introduction	234

A.3.7.2	E-UTRAN Serving Cell Parameters	234

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	234

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	235

A.3.7A	NR FR1-FR2 test setup	236

A.3.8	PRACH configurations	236

A.3.8.1	Introduction	236

A.3.8.2	PRACH configurations in FR1	236

A.3.8.2.1	FR1 PRACH configuration 1	236

A.3.8.2.2	FR1 PRACH configuration 2	237

A.3.8.2.3	FR1 PRACH configuration 3	238

A.3.8.2.4	FR1 PRACH configuration 4	239

A.3.8.3	PRACH configurations in FR2	240

A.3.8.3.1	FR2 PRACH configuration 1	240

A.3.8.3.2	FR2 PRACH configuration 2	241

A.3.8.3.3	FR2 PRACH configuration 3	242

A.3.8.3.4	FR2 PRACH configuration 4	243

A.3.9	BWP configurations	244

A.3.9.1	Introduction	244

A.3.9.2	Downlink BWP configurations	245

A.3.9.2.1	Initial BWP	245

A.3.9.2.2	Dedicated BWP	245

A.3.9.3	Uplink BWP configurations	245

A.3.9.3.1	Initial BWP	245

A.3.9.3.2	Dedicated BWP	246

A.3.10	SSB Configurations	246

A.3.10.1	SSB Configurations for FR1	246

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	246

A.3.10.1.2	SSB pattern 2 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	247

A.3.10.1.3	SSB pattern 3 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	247

A.3.10.1.4	SSB pattern 4 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	248

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	249

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	249

A.3.10.2	SSB Configurations for FR2	250

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	250

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	250

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	251

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	251

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	252

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	252

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	253

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	253

A.3.11	SMTC Configurations	253

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	253

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	254

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	254

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	254

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	254

A.3.12	Test Cases with Different CC Configurations	254

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	254

A.3.12.1.1	Introduction	254

A.3.12.1.2	Principle of testing	255

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	255

A.3.12.2.1	Introduction	255

A.3.12.2.2	Principle of testing	255

A.3.13	Test Cases in SA and EN-DC Operations	255

A.3.13.1	Introduction	255

A.3.13.2	Principle of Testing	255

A.3.14	CSI-RS configurations	256

A.3.14.1	FDD	256

A.3.14.2	TDD	257

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	260

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	260

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	260

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	260

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	260

A.3.15.3	Setup 3: 2 AoAs	260

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	261

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	261

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	261

A.3.16	TCI State Configuration	261

A.3.16.1	Introduction	261

A.3.16.2	TCI states	262

A.3.17	Configurations of CSI-RS for tracking	262

A.3.17.1	Configuration of CSI-RS for tracking for FR1	262

A.3.17.1.1	FDD	262

A.3.17.1.2	TDD	263

A.3.17.2	Configuration of CSI-RS for tracking for FR2	264

A.3.17.2.1	TDD	264

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	265

A.3.18.1	Introduction	265

A.3.18.2	PRACH Power Measurement	265

A.4	EN-DC tests with all NR cells in FR1	266

A.4.1	Void	266

A.4.2	Void	266

A.4.3	RRC_CONNECTED state mobility	266

A.4.3.1	Void	266

A.4.3.2	RRC Connection Mobility Control	266

A.4.3.2.1	Void	266

A.4.3.2.2	Random Access	266

A.4.3.2.2.1	Contention based random access test in FR1 for PSCell in EN-DC	266

A.4.3.2.2.1.1	Test Purpose and Environment	266

A.4.3.2.2.2	Non-contention based random access test in FR1 for PSCell in EN-DC	269

A.4.3.2.3	Void	272

A.4.4	Timing	272

A.4.4.1	UE transmit timing	272

A.4.4.1.1	NR UE Transmit Timing Test for FR1	272

A.4.4.1.1.1	Test Purpose and environment	272

A.4.4.1.1.2	Test requirements	275

A.4.4.2	UE timer accuracy	276

A.4.4.3	Timing advance	276

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	276

A.4.4.3.1.1	Test Purpose and Environment	276

A.4.4.3.1.2	Test Parameters	276

A.4.4.3.1.3	Test Requirements	279

A.4.5	Signaling characteristics	279

A.4.5.1	Radio link Monitoring	279

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	280

A.4.5.1.1.1	Test Purpose and Environment	280

A.4.5.1.1.2	Test Requirements	283

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	283

A.4.5.1.2.2	Test Requirements	287

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	287

A.4.5.1.3.1	Test Purpose and Environment	287

A.4.5.1.3.2	Test Requirements	290

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	291

A.4.5.1.4.1	Test Purpose and Environment	291

A.4.5.1.4.2	Test Requirements	294

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	294

A.4.5.1.5.1	Test Purpose and Environment	294

A.4.5.1.5.2	Test Requirements	297

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	298

A.4.5.1.6.1	Test Purpose and Environment	298

A.4.5.1.6.2	Test Requirements	301

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	301

A.4.5.1.7.1	Test Purpose and Environment	301

A.4.5.1.7.2	Test Requirements	305

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	305

A.4.5.1.8.1	Test Purpose and Environment	305

A.4.5.1.8.2	Test Requirements	309

A.4.5.2	Interruption	309

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	309

A.4.5.2.1.1	Test Purpose and Environment	309

A.4.5.2.1.2	Test Requirements	311

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	312

A.4.5.2.2.1	Test Purpose and Environment	312

A.4.5.2.2.2	Test Requirements	314

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	315

A.4.5.2.3.1	Test Purpose and Environment	315

A.4.5.2.3.2	Test Requirements	317

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	318

A.4.5.2.4.1	Test Purpose and Environment	318

A.4.5.2.4.2	Test Requirements	322

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	323

A.4.5.2.5.1	Test Purpose and Environment	323

A.4.5.2.5.2	Test Requirements	325

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	326

A.4.5.2.6.1	Test Purpose and Environment	326

A.4.5.2.6.2	Test Requirements	328

A.4.5.2.7	Void	329

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	329

A.4.5.3.1.1	Test Purpose and Environment	329

A.4.5.3.1.2	Test Requirements	332

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	333

A.4.5.3.2.1	Test Purpose and Environment	333

A.4.5.3.2.2	Test Requirements	333

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	333

A.4.5.3.3.1	Test Purpose and Environment	333

A.4.5.3.3.2	Test Requirements	334

A.4.5.4	UE UL carrier RRC reconfiguration Delay	335

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	335

A.4.5.4.1.1 Test Purpose and Environment	335

A.4.5.4.1.2	Test Requirements	340

A.4.5.5	Beam Failure Detection and Link recovery procedures	340

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	340

A.4.5.5.1.1	Test Purpose and Environment	340

A.4.5.5.1.2	Test Requirements	344

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	345

A.4.5.5.2.1	Test Purpose and Environment	345

A.4.5.5.2.2	Test Requirements	349

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	349

A.4.5.5.3.1	Test Purpose and Environment	349

A.4.5.5.3.2	Test Requirements	353

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	353

A.4.5.5.4.1	Test Purpose and Environment	353

A.4.5.5.4.2	Test Requirements	357

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	357

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	357

A.4.5.6.1.1.1	Test Purpose and Environment	357

A.4.5.6.1.1.2	Test Requirements	360

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	361

A.4.5.6.1.2.1	Test Purpose and Environment	361

A.4.5.6.1.2.2	Test Requirements	364

A.4.5.6.2	RRC-based Active BWP Switch	365

A.4.5.6.2.1.1	Test Purpose and Environment	365

A.4.5.6.2.1.2	Test Requirements	369

A.4.5.7	PSCell addition and release delay	369

A.4.5.7.1	Addition and Release Delay of known NR PSCell	369

A.4.5.7.1.1	Test purpose and environment	369

A.4.5.7.1.2	Test Requirements	373

A.4.6	Measurement procedure	373

A.4.6.1	Intra-frequency Measurements	373

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	373

A.4.6.1.1.1	Test purpose and Environment	373

A.4.6.1.1.2	Test parameters	373

A.4.6.1.1.3	Test Requirements	375

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	376

A.4.6.1.2.1	Test purpose and Environment	376

A.4.6.1.2.2	Test parameters	376

A.4.6.1.2.2	Test Requirements	378

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	378

A.4.6.1.3.1	Test purpose and Environment	378

A.4.6.1.3.2	Test parameters	378

A.4.6.1.3.3	Test Requirements	381

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	381

A.4.6.1.4.1	Test purpose and Environment	381

A.4.6.1.4.2	Test parameters	381

A.4.6.1.4.3	Test Requirements	383

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	384

A.4.6.1.5.1	Test purpose and Environment	384

A.4.6.1.5.2	Test parameters	384

A.4.6.1.5.3	Test Requirements	385

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	386

A.4.6.1.6.1	Test purpose and Environment	386

A.4.6.1.6.2	Test parameters	386

A.4.6.1.6.3	Test Requirements	388

A.4.6.2	Inter-frequency Measurements	389

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	389

A.4.6.2.1.1	Test Purpose and Environment	389

A.4.6.2.1.2	Test Requirements	392

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	392

A.4.6.2.2.1	Test Purpose and Environment	392

A.4.6.2.2.2	Test Requirements	395

A.4.6.2.3	Void	396

A.4.6.2.4	Void	396

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	396

A.4.6.2.5.1	Test Purpose and Environment	396

A.4.6.2.5.2	Test Requirements	399

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	399

A.4.6.2.6.1	Test Purpose and Environment	399

A.4.6.2.6.2	Test Requirements	403

A.4.6.2.7	Void	403

A.4.6.2.8	Void	403

A.4.6.3	Void	403

A.4.6.4	L1-RSRP measurement for beam reporting	403

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	403

A.4.6.4.1.1	Test Purpose and Environment	403

A.4.6.4.1.2	Test parameters	404

A.4.6.4.1.3	Test Requirements	406

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	406

A.4.6.4.2.1	Test Purpose and Environment	406

A.4.6.4.2.2	Test parameters	406

A.4.6.4.2.3	Test Requirements	408

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	408

A.4.6.4.3.1	Test Purpose and Environment	408

A.4.6.4.3.2	Test parameters	409

A.4.6.4.3.3	Test Requirements	411

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	411

A.4.6.4.4.1	Test Purpose and Environment	411

A.4.6.4.4.2	Test parameters	411

A.4.6.4.4.3	Test Requirements	414

A.4.7	Measurement Performance requirements	414

A.4.7.1	SS-RSRP	414

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	414

A.4.7.1.1.1	Test Purpose and Environment	414

A.4.7.1.1.2	Test parameters	415

A.4.7.1.1.3	Test Requirements	417

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	418

A.4.7.1.2.1	Test Purpose and Environment	418

A.4.7.1.2.2	Test parameters	418

A.4.7.1.2.3	Test Requirements	421

A.4.7.1.3	Void	421

A.4.7.2	SS-RSRQ	421

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	421

A.4.7.2.1.1	Test Purpose and Environment	421

A.4.7.2.1.2	Test Parameters	421

A.4.7.2.1.3	Test Requirements	425

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	425

A.4.7.2.2.1	Test Purpose and Environment	425

A.4.7.2.2.2	Test Parameters	425

A.4.7.2.2.3	Test Requirements	428

A.4.7.3	SS-SINR	429

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	429

A.4.7.3.1.1	Test Purpose and Environment	429

A.4.7.3.1.2	Test Parameters	429

A.4.7.3.1.3	Test Requirements	432

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	432

A.4.7.3.2.1	Test Purpose and Environment	432

A.4.7.3.2.2	Test Parameters	432

A.4.7.3.2.3	Test Requirements	436

A.4.7.4	L1-RSRP measurement for beam reporting	436

A.4.7.4.1	SSB based L1-RSRP measurement	436

A.4.7.4.1.1	Test Purpose and Environment	436

A.4.7.4.1.2	Test parameters	437

A.4.7.4.1.3	Test Requirements	440

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	440

A.4.7.4.2.1	Test Purpose and Environment	440

A.4.7.4.2.2	Test parameters	440

A.4.7.4.2.3	Test Requirements	443

A.4.7.5	SFTD accuracy	443

A.4.7.5.1	SFTD accuracy	443

A.4.7.5.1.1	Test Purpose and Environment	443

A.4.7.5.1.2	Test Parameters	443

A.4.7.5.1.3	Test Requirements	446

A.4.7.5.2	Void	446

A.4.7.5.3	Void	446

A.4.8	Void	446

A.5	EN-DC tests with one or more NR cells in FR2	447

A.5.1	Void	447

A.5.2	Void	447

A.5.3	RRC_CONNECTED state mobility	447

A.5.3.1	Void	447

A.5.3.2	RRC Connection Mobility Control	447

A.5.3.2.1	Void	447

A.5.3.2.2	Random Access	447

A.5.3.2.2.1	Contention based random access test in FR2 for PSCell/SCell in EN-DC	447

A.5.3.2.2.2	Non-contention based random access test in FR2 for PSCell/SCell in EN-DC	451

A.5.3.2.3	Void	455

A.5.4	Timing	455

A.5.4.1	UE transmit timing	455

A.5.4.1.1	NR UE Transmit Timing Test for FR2	455

A.5.4.1.1.1	Test Purpose and environment	455

A.5.4.1.1.2	Test requirements	457

A.5.4.2	UE timer accuracy	458

A.5.4.3	Timing advance	458

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	458

A.5.4.3.1.1 Test Purpose and Environment	458

A.5.4.3.1.2 Test Parameters	458

A.5.4.3.1.3	Test Requirements	462

A.5.5	Signaling characteristics	462

A.5.5.1	Radio link Monitoring	462

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	462

A.5.5.1.1.1	Test Purpose and Environment	462

A.5.5.1.1.2	Test Requirements	465

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	466

A.5.5.1.2.1	Test Purpose and Environment	466

A.5.5.1.2.2	Test Requirements	469

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	469

A.5.5.1.3.1	Test Purpose and Environment	469

A.5.5.1.3.2	Test Requirements	473

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	473

A.5.5.1.4.1	Test Purpose and Environment	473

A.5.5.1.4.2	Test Requirements	476

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	476

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	480

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	484

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	488

A.5.5.1.8.2	Test Requirements	492

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	492

A.5.5.1.9.1	Test Purpose and Environment	492

A.5.5.1.9.2	Test Requirements	494

A.5.5.2	Interruption	494

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	494

A.5.5.2.1.1	Test Purpose and Environment	494

A.5.5.2.1.2	Test Requirements	497

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	497

A.5.5.2.2.1	Test Purpose and Environment	497

A.5.5.2.2.2	Test Requirements	500

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	500

A.5.5.2.3.1	Test Purpose and Environment	500

A.5.5.2.3.2	Test Requirements	503

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	504

A.5.5.2.4.1	Test Purpose and Environment	504

A.5.5.2.4.2	Test Requirements	507

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	508

A.5.5.2.5.1	Test Purpose and Environment	508

A.5.5.2.5.2	Test Requirements	511

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	512

A.5.5.2.6.1	Test Purpose and Environment	512

A.5.5.2.6.2	Test Requirements	514

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	515

A.5.5.3.1.1	Test Purpose and Environment	515

A.5.5.3.1.2	Test Requirements	517

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	517

A.5.5.3.2.1	Test Purpose and Environment	517

A.5.5.3.2.2	Test Requirements	520

A.5.5.3.3	Void	520

A.5.5.3.4	Void	520

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	520

A.5.5.3.5.1	Test Purpose and Environment	520

A.5.5.3.5.2	Test Requirements	522

A.5.5.4	Void	523

A.5.5.5	Beam Failure Detection and Link recovery procedures	523

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	523

A.5.5.5.1.1	Test Purpose and Environment	523

A.5.5.5.1.2	Test Requirements	526

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	527

A.5.5.5.2.1	Test Purpose and Environment	527

A.5.5.5.2.2	Test Requirements	530

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	530

A.5.5.5.3.1	Test Purpose and Environment	530

A.5.5.5.3.2	Test Requirements	533

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	534

A.5.5.5.4.1	Test Purpose and Environment	534

A.5.5.5.4.2	Test Requirements	537

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	537

A.5.5.5.5.1	Test Purpose and Environment	537

A.5.5.5.5.2	Test Requirements	541

A.5.5.6	Active BWP switch	542

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	542

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	542

A.5.5.6.1.1.1	Test Purpose and Environment	542

A.5.5.6.1.1.2	Test Requirements	545

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 DL active BWP switch with FR2 SCell in non-DRX in synchronous EN-DC	546

A.5.5.6.1.2.1	Test Purpose and Environment	546

A.5.5.6.1.2.2	Test Requirements	549

A.5.5.6.2	RRC-based Active BWP Switch	550

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	550

A.5.5.6.2.1.1	Test Purpose and Environment	550

A.5.5.6.2.1.2	Test Requirements	553

A.5.5.7	PSCell addition and release delay	553

A.5.5.7.1	Addition and Release Delay of NR PSCell	553

A.5.5.7.1.1	Test purpose and environment	553

A.5.5.7.1.2	Test Requirements	557

A.5.5.8	Active TCI state switch delay	558

A.5.5.8.1	MAC-CE based active TCI state switch	558

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	558

A.5.5.8.1.1.1	Test Purpose and Environment	558

A.5.5.8.1.1.2	Test Requirements	560

A.5.5.8.2	RRC based active TCI state switch	560

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	560

A.5.5.8.2.1.1	Test Purpose and Environment	560

A.5.5.8.2.1.2	Test Requirements	563

A.5.6	Measurement procedure	563

A.5.6.1	Intra-frequency Measurements	563

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	563

A.5.6.1.1.1	Test purpose and Environment	563

A.5.6.1.1.2	Test Requirements	565

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	566

A.5.6.1.2.1	Test purpose and Environment	566

A.5.6.1.2.2	Test Requirements	568

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	569

A.5.6.1.3.1	Test purpose and Environment	569

A.5.6.1.3.2	Test Requirements	571

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	572

A.5.6.1.4.1	Test purpose and Environment	572

A.5.6.1.4.2	Test Requirements	574

A.5.6.2	Inter-frequency Measurements	575

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	575

A.5.6.2.1.1	Test Purpose and Environment	575

A.5.6.2.1.2	Test Requirements	578

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	578

A.5.6.2.2.1	Test Purpose and Environment	578

A.5.6.2.2.2	Test Requirements	581

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	581

A.5.6.2.3.1	Test Purpose and Environment	581

A.5.6.2.3.2	Test Requirements	584

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	584

A.5.6.2.4.1	Test Purpose and Environment	584

A.5.6.2.2.4	Test Requirements	588

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	588

A.5.6.2.5.1	Test Purpose and Environment	588

A.5.6.2.5.2	Test Requirements	592

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	593

A.5.6.2.6.1	Test Purpose and Environment	593

A.5.6.2.6.2	Test Requirements	596

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	597

A.5.6.2.7.1	Test Purpose and Environment	597

A.5.6.2.7.2	Test Requirements	600

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	601

A.5.6.2.8.1	Test Purpose and Environment	601

A.5.6.2.8.2	Test Requirements	604

A.5.6.3	L1-RSRP measurement for beam reporting	605

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	605

A.5.6.3.1.1	Test Purpose and Environment	605

A.5.6.3.1.2	Test parameters	605

A.5.6.3.1.3	Test Requirements	607

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	607

A.5.6.3.2.1	Test Purpose and Environment	607

A.5.6.3.2.2	Test parameters	608

A.5.6.3.2.3	Test Requirements	610

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	610

A.5.6.3.3.1	Test Purpose and Environment	610

A.5.6.3.3.2	Test parameters	611

A.5.6.3.3.3	Test Requirements	613

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	614

A.5.6.3.4.1	Test Purpose and Environment	614

A.5.6.3.4.2	Test parameters	614

A.5.6.3.4.3	Test Requirements	616

A.5.7	Measurement Performance requirements	617

A.5.7.1	SS-RSRP	617

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	617

A.5.7.1.1.1	Test Purpose and Environment	617

A.5.7.1.1.2	Test parameters	617

A.5.7.1.1.3	Test Requirements	619

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	620

A.5.7.1.2.1	Test Purpose and Environment	620

A.5.7.1.2.2	Test parameters	620

A.5.7.1.2.3	Test Requirements	622

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	623

A.5.7.1.3.1	Test Purpose and Environment	623

A.5.7.1.3.2	Test parameters	624

A.5.7.1.3.3	Test Requirements	626

A.5.7.2	SS-RSRQ	626

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	626

A.5.7.2.1.1	Test Purpose and Environment	626

A.5.7.2.1.2	Test Parameters	626

A.5.7.2.1.3	Test Requirements	631

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	631

A.5.7.2.2.1	Test Purpose and Environment	631

A.5.7.2.2.2	Test Parameters	631

A.5.7.2.2.3	Test Requirements	634

A.5.7.3	SS-SINR	634

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	634

A.5.7.3.1.1	Test Purpose and Environment	634

A.5.7.3.1.2	Test Parameters	634

A.5.7.3.1.3	Test Requirements	637

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	637

A.5.7.3.2.1	Test Purpose and Environment	637

A.5.7.3.2.2	Test Parameters	637

A.5.7.3.2.3	Test Requirements	640

A.5.7.4	L1-RSRP measurement for beam reporting	640

A.5.7.4.1	SSB based L1-RSRP measurement	640

A.5.7.4.1.1	Test Purpose and Environment	640

A.5.7.4.1.2	Test parameters	640

A.5.7.4.1.3	Test Requirements	642

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	643

A.5.7.4.2.1	Test Purpose and Environment	643

A.5.7.4.2.2	Test parameters	643

A.5.7.4.2.3	Test Requirements	645

A.5.8	Void	646

A.6	NR standalone tests with all NR cells in FR1	639

A.6.1	SA: RRC_IDLE state mobility	639

A.6.1.1	Cell re-selection to NR	639

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	639

A.6.1.1.1.1	Test Purpose and Environment	639

A.6.1.1.1.2	Test Parameters	639

A.6.1.1.1.3	Test Requirements	642

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	642

A.6.1.1.2.1	Test Purpose and Environment	642

A.6.1.1.2.2	Test Parameters	642

A.6.1.1.2.3	Test Requirements	644

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	645

A.6.1.2.1.1	Test Purpose and Environment	645

A.6.1.2.1.2	Test Parameters	645

A.6.1.2.1.3	Test Requirements	648

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	648

A.6.1.2.2.1	Test Purpose and Environment	648

A.6.1.2.2.2	Test Parameters	648

A.6.1.2.2.3	Test Requirements	651

A.6.2	SA: RRC_INACTIVE state mobility	652

A.6.3	RRC_CONNECTED state mobility	652

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	652

A.6.3.1.1.1	Test Purpose and Environment	652

A.6.3.1.1.2	Test Parameters	652

A.6.3.1.1.3 Test Requirements	654

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	654

A.6.3.1.2.1	Test Purpose and Environment	654

A.6.3.1.2.2	Test Parameters	654

A.6.3.1.2.3	Test Requirements	656

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	656

A.6.3.1.3.1	Test Purpose and Environment	656

A.6.3.1.3.2	Test Parameters	656

A.6.3.1.3.3 Test Requirements	658

A.6.3.1.4	 SA NR - E-UTRAN handover	658

A.6.3.1.4.1	Test Purpose and Environment	658

A.6.3.1.4.2	Test Requirements	662

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	662

A.6.3.1.5.1	Test Purpose and Environment	662

A.6.3.1.5.2	Test Requirements	665

A.6.3.2.1	SA: RRC Re-establishment	666

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	666

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	669

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	672

A.6.3.2.2	Random Access	675

A.6.3.2.2.1	Contention based random access test in FR1 for NR standalone	675

A.6.3.2.2.2	Non-Contention based random access test in FR1 for NR standalone	678

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	681

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	684

A.6.4	Timing	687

A.6.4.1.1	NR UE Transmit Timing Test for FR1	687

A.6.4.1.1.1	Test Purpose and environment	687

A.6.4.1.1.2	Test requirements	690

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	691

A.6.4.3.1.1	Test Purpose and Environment	691

A.6.4.3.1.2	Test Parameters	691

A.6.4.3.1.3	Test Requirements	694

A.6.5	Signalling characteristics	694

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	695

A.6.5.1.1.1	Test Purpose and Environment	695

A.6.5.1.1.2	Test Requirements	698

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	698

A.6.5.1.2.1	Test Purpose and Environment	698

A.6.5.1.2.2	Test Requirements	702

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	702

A.6.5.1.3.1	Test Purpose and Environment	702

A.6.5.1.3.2	Test Requirements	706

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	706

A.6.5.1.4.1	Test Purpose and Environment	706

A.6.5.1.4.2	Test Requirements	710

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	710

A.6.5.1.5.1	Test Purpose and Environment	710

A.6.5.1.5.2	Test Requirements	715

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	715

A.6.5.1.6.1	Test Purpose and Environment	715

A.6.5.1.6.2	Test Requirements	719

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	719

A.6.5.1.7.1	Test Purpose and Environment	719

A.6.5.1.7.2	Test Requirements	723

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	723

A.6.5.1.8.1	Test Purpose and Environment	723

A.6.5.1.8.2	Test Requirements	727

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	727

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	730

A.6.5.3.1.1	Test Purpose and Environment	730

A.6.5.3.1.2	Test Requirements	733

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	733

A.6.5.3.2.1	Test Purpose and Environment	733

A.6.5.3.2.2	Test Requirements	734

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	734

A.6.5.3.3.1	Test Purpose and Environment	734

A.6.5.3.3.2	Test Requirements	735

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	735

A.6.5.4.1.1	Test Purpose and Environment	735

A.6.5.4.1.2	Test Requirements	741

A.6.5.4.2	Void	741

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	741

A.6.5.5.1.1	Test Purpose and Environment	741

A.6.5.5.1.2	Test Requirements	745

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	745

A.6.5.5.2.1	Test Purpose and Environment	745

A.6.5.5.2.2	Test Requirements	749

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	750

A.6.5.5.3.1	Test Purpose and Environment	750

A.6.5.5.3.2	Test Requirements	753

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	754

A.6.5.5.4.1	Test Purpose and Environment	754

A.6.5.5.4.2	Test Requirements	757

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	757

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	757

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	761

A.6.5.6.2	RRC-based Active BWP Switch	764

A.6.5.6.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	764

A.6.6	Measurement procedure	767

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	767

A.6.6.1.1.1	Test purpose and Environment	767

A.6.6.1.1.2	Test parameters	767

A.6.6.1.1.3	Test Requirements	769

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	769

A.6.6.1.2.1	Test purpose and Environment	769

A.6.6.1.2.2	Test parameters	769

A.6.6.1.2.3	Test Requirements	771

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	771

A.6.6.1.3.1	Test purpose and Environment	771

A.6.6.1.3.2	Test parameters	771

A.6.6.1.3.3	Test Requirements	773

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	773

A.6.6.1.4.1	Test purpose and Environment	773

A.6.6.1.4.2	Test parameters	774

A.6.6.1.4.3	Test Requirements	776

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	776

A.6.6.1.5.1	Test purpose and Environment	776

A.6.6.1.5.2	Test parameters	776

A.6.6.1.5.3	Test Requirements	778

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	778

A.6.6.1.6.1	Test purpose and Environment	778

A.6.6.1.6.2	Test parameters	778

A.6.6.1.6.3	Test Requirements	780

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	780

A.6.6.2.1.1	Test Purpose and Environment	780

A.6.6.2.1.2	Test Requirements	783

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	783

A.6.6.2.2.1	Test Purpose and Environment	783

A.6.6.2.2.2	Test Requirements	786

A.6.6.2.3	Void	787

A.6.6.2.4	Void	787

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	787

A.6.6.2.5.1	Test Purpose and Environment	787

A.6.6.2.5.2	Test Requirements	791

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	791

A.6.6.2.6.1	Test Purpose and Environment	791

A.6.6.2.6.2	Test Requirements	794

A.6.6.2.7	Void	795

A.6.6.2.8	Void	795

A.6.6.3	Inter-RAT Measurements	795

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	795

A.6.6.3.1.1	Test Purpose and Environment	795

A.6.6.3.1.2	Test Requirements	798

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	798

A.6.6.3.2.1	Test Purpose and Environment	798

A.6.6.3.2.2	Test Requirements	802

A.6.6.4	L1-RSRP measurement for beam reporting	803

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	803

A.6.6.4.1.1	Test Purpose and Environment	803

A.6.6.4.1.2	Test parameters	803

A.6.6.4.1.3	Test Requirements	805

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	805

A.6.6.4.2.1	Test Purpose and Environment	805

A.6.6.4.2.2	Test parameters	805

A.6.6.4.2.3	Test Requirements	807

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	807

A.6.6.4A.6.6.4.3.1	Test Purpose and Environment	807

A.6.6.4.3.2	Test parameters	808

A.6.6.4.3.3	Test Requirements	810

A.6.6.4.4.1	Test Purpose and Environment	810

A.6.6.4.4.2	Test parameters	810

A.6.6.4.4.3	Test Requirements	812

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	813

A.6.7	Measurement Performance requirements	813

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	813

A.6.7.1.1.1	Test Purpose and Environment	813

A.6.7.1.1.2	Test parameters	813

A.6.7.1.1.3	Test Requirements	818

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	818

A.6.7.1.2.1	Test Purpose and Environment	818

A.6.7.1.2.2	Test parameters	818

A.6.7.1.2.3	Test Requirements	821

A.6.7.1.3	Void	821

A.6.7.2	SS-RSRQ	821

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	821

A.6.7.2.1.1	Test Purpose and Environment	821

A.6.7.2.1.2	Test Parameters	821

A.6.7.2.1.3	Test Requirements	825

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	825

A.6.7.2.2.1	Test Purpose and Environment	825

A.6.7.2.2.2	Test Parameters	825

A.6.7.2.2.3	Test Requirements	829

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	829

A.6.7.3.1.1	Test Purpose and Environment	829

A.6.7.3.1.2	Test Parameters	829

A.6.7.3.1.3	Test Requirements	832

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	832

A.6.7.3.2.1	Test Purpose and Environment	832

A.6.7.3.2.2	Test Parameters	832

A.6.7.3.2.3	Test Requirements	835

A.6.7.4.1	SSB based L1-RSRP measurement	836

A.6.7.4.1.1	Test Purpose and Environment	836

A.6.7.4.1.2	Test parameters	836

A.6.7.4.1.3	Test Requirements	839

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	839

A.6.7.4.2.1	Test Purpose and Environment	839

A.6.7.4.2.2	Test parameters	839

A.6.7.4.2.3	Test Requirements	842

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	842

A.6.7.5.1.1	Test Purpose and Environment	842

A.6.7.5.1.2	Test parameters	842

A.6.7.5.1.3	Test Requirements	846

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	846

A.6.7.6.1.1	Test Purpose and Environment	846

A.6.7.6.1.2	Test parameters	847

A.6.7.6.1.3	Test Requirements	850

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	851

A.6.7.7.1.1	Test Purpose and Environment	851

A.6.7.7.1.2	Test parameters	851

A.6.7.7.1.3	Test Requirements	855

A.7	NR standalone tests with one or more NR cells in FR2	855

A.7.1	SA: RRC_IDLE state mobility	855

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	855

A.7.1.1.1.1	Test Purpose and Environment	855

A.7.1.1.1.2	Test Parameters	855

A.7.1.1.1.3	Test Requirements	858

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	858

A.7.1.1.2.1	Test Purpose and Environment	858

A.7.1.1.2.2	Test Parameters	858

A.7.1.1.2.3	Test Requirements	861

A.7.2	SA: RRC_INACTIVE state mobility	861

A.7.3	RRC_CONNECTED state mobility	861

A.7.3.1	Handover	861

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	861

A.7.3.1.1.1	Test Purpose and Environment	861

A.7.3.1.1.2	Test Parameters	861

A.7.3.1.1.3	Test Requirements	864

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	864

A.7.3.1.2.1	Test Purpose and Environment	864

A.7.3.1.2.2	Test Parameters	864

A.7.3.1.2.3	Test Requirements	866

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	866

A.7.3.1.3.1	Test Purpose and Environment	866

A.7.3.1.3.2	Test Parameters	866

A.7.3.1.3.3 Test Requirements	868

A.7.3.2.1	SA: RRC Re-establishment	868

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	868

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	871

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	874

A.7.3.2.1.3.1	Test Purpose and Environment	874

A.7.3.2.1.3.2	Test Requirements	876

A.7.3.2.2	Random Access	877

A.7.3.2.2.1	Contention based random access test in FR2 for NR Standalone	877

A.7.3.2.2.2	Non-contention based random access test in FR2 for NR Standalone	880

A.7.3.2.3	SA: RRC Connection Release with Redirection	884

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	884

A.7.4	Timing	886

A.7.4.1.1	NR UE Transmit Timing Test for FR2	886

A.7.4.1.1.1	Test Purpose and environment	886

A.7.4.1.1.2	Test requirements	889

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	890

A.7.4.3.1.1	Test Purpose and Environment	890

A.7.4.3.1.2	Test Parameters	890

A.7.4.3.1.3 Test Requirements	894

A.7.5	Signaling characteristics	894

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	894

A.7.5.1.1.1	Test Purpose and Environment	894

A.7.5.1.1.2	Test Requirements	897

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	897

A.7.5.1.2.1	Test Purpose and Environment	897

A.7.5.1.2.2	Test Requirements	902

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	903

A.7.5.1.3.1	Test Purpose and Environment	903

A.7.5.1.3.2	Test Requirements	906

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	906

A.7.5.1.4.1	Test Purpose and Environment	906

A.7.5.1.4.2	Test Requirements	910

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	910

A.7.5.1.5.1	Test Purpose and Environment	910

A.7.5.1.5.2	Test Requirements	914

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	914

A.7.5.1.6.1	Test Purpose and Environment	914

A.7.5.1.6.2	Test Requirements	918

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	918

A.7.5.1.7.1	Test Purpose and Environment	918

A.7.5.1.7.2	Test Requirements	921

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	921

A.7.5.1.8.1	Test Purpose and Environment	921

A.7.5.1.8.2	Test Requirements	925

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	925

A.7.5.1.9.1	Test Purpose and Environment	925

A.7.5.1.9.2	Test Requirements	926

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	927

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	930

A.7.5.3.1.1	Test Purpose and Environment	930

A.7.5.3.1.2	Test Requirements	932

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	932

A.7.5.3.2.1	Test Purpose and Environment	932

A.7.5.3.2.2	Test Requirements	935

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	936

A.7.5.5.1.1	Test Purpose and Environment	936

A.7.5.5.1.2	Test Requirements	939

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	940

A.7.5.5.2.1	Test Purpose and Environment	940

A.7.5.5.2.2	Test Requirements	943

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	943

A.7.5.5.3.1	Test Purpose and Environment	943

A.7.5.5.3.2	Test Requirements	948

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	948

A.7.5.5.4.1	Test Purpose and Environment	948

A.7.5.5.4.2	Test Requirements	953

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	954

A.7.5.5.5.1	Test Purpose and Environment	954

A.7.5.5.5.2	Test Requirements	958

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	958

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of PCell with non-DRX in SA	958

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA	963

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	967

A.7.5.6.1.3.1	Test Purpose and Environment	967

A.7.5.6.1.3.2	Test Requirements	970

A.7.5.6.2	RRC-based Active BWP Switch	970

A.7.5.7.1	Addition and Release Delay of known NR PSCell	973

A.7.5.7.1.1	Test Purpose and Environment	973

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	977

A.7.5.7.2.1	Test Purpose and Environment	977

A.7.5.8.1	MAC-CE based active TCI state switch	981

A.7.5.8.2	RRC based active TCI state switch	984

A.7.6	Measurement procedure	987

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	987

A.7.6.1.1.1	Test purpose and Environment	987

A.7.6.1.1.2	Test Requirements	989

A.7.6.1.2	SA event triggered reporting test without gap under DRX	989

A.7.6.1.2.1	Test purpose and Environment	989

A.7.6.1.2.2	Test Requirements	992

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	992

A.7.6.1.3.1	Test purpose and Environment	992

A.7.6.1.3.2	Test Requirements	994

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	995

A.7.6.1.4.1	Test purpose and Environment	995

A.7.6.1.4.2	Test Requirements	997

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	998

A.7.6.2.1.1	Test Purpose and Environment	998

A.7.6.2.1.2	Test Requirements	1002

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1002

A.7.6.2.2.1	Test Purpose and Environment	1002

A.7.6.2.2.2	Test Requirements	1005

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1005

A.7.6.2.3.1	Test Purpose and Environment	1005

A.7.6.2.3.2	Test Requirements	1009

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1009

A.7.6.2.4.1	Test Purpose and Environment	1009

A.7.6.2.4.2	Test Requirements	1012

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1012

A.7.6.2.5.1	Test Purpose and Environment	1012

A.7.6.2.5.2	Test Requirements	1015

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1015

A.7.6.2.6.1	Test Purpose and Environment	1015

A.7.6.2.6.2	Test Requirements	1019

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1020

A.7.6.2.7.1	Test Purpose and Environment	1020

A.7.6.2.7.2	Test Requirements	1023

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1023

A.7.6.2.8.1	Test Purpose and Environment	1023

A.7.6.2.8.2	Test Requirements	1027

A.7.6.3	L1-RSRP measurement for beam reporting	1027

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1027

A.7.6.3.1.1	Test Purpose and Environment	1027

A.7.6.3.1.2	Test parameters	1028

A.7.6.3.1.3	Test Requirements	1030

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1030

A.7.6.3.2.1	Test Purpose and Environment	1030

A.7.6.3.2.2	Test parameters	1031

A.7.6.3.2.3	Test Requirements	1033

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1033

A.7.6.3.3.1	Test Purpose and Environment	1033

A.7.6.3.3.2	Test parameters	1034

A.7.6.3.3.3	Test Requirements	1036

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1037

A.7.6.3.4.1	Test Purpose and Environment	1037

A.7.6.3.4.2	Test parameters	1037

A.7.6.3.4.3	Test Requirements	1039

A.7.7	Measurement Performance requirements	1040

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1040

A.7.7.1.1.1	Test Purpose and Environment	1040

A.7.7.1.1.2	Test parameters	1040

A.7.7.1.1.3	Test Requirements	1043

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1043

A.7.7.1.2.1	Test Purpose and Environment	1043

A.7.7.1.2.2	Test parameters	1044

A.7.7.1.2.3	Test Requirements	1047

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1047

A.7.7.1.3.1	Test Purpose and Environment	1047

A.7.7.1.3.2	Test parameters	1048

A.7.7.1.3.3	Test Requirements	1050

A.7.7.2	SS-RSRQ	1050

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1050

A.7.7.2.1.1	Test Purpose and Environment	1050

A.7.7.2.1.2	Test Parameters	1050

A.7.7.2.1.3	Test Requirements	1054

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1054

A.7.7.2.2.1	Test Purpose and Environment	1054

A.7.7.2.2.2	Test Parameters	1054

A.7.7.2.2.3	Test Requirements	1057

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1057

A.7.7.3.1.1	Test Purpose and Environment	1057

A.7.7.3.1.2	Test Parameters	1057

A.7.7.3.1.3	Test Requirements	1059

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1060

A.7.7.3.2.1	Test Purpose and Environment	1060

A.7.7.3.2.2	Test Parameters	1060

A.7.7.3.2.3	Test Requirements	1063

A.7.7.4.1	SSB based L1-RSRP measurement	1063

A.7.7.4.1.1	Test Purpose and Environment	1063

A.7.7.4.1.2	Test parameters	1063

A.7.7.4.1.3	Test Requirements	1065

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1066

A.7.7.4.2.1	Test Purpose and Environment	1066

A.7.7.4.2.2	Test parameters	1066

A.7.7.4.2.3	Test Requirements	1068

A.8	E-UTRA standalone tests for NR RRM	1069

A.8.1	Void	1069

A.8.2	RRC_IDLE state mobility	1069

A.8.2.1	Inter-RAT NR Cell re-selection	1069

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	1069

A.8.2.1.1.1	Test Purpose and Environment	1069

A.8.2.1.1.2	Test Requirements	1072

A.8.3	RRC_CONNECTED state mobility	1073

A.8.3.1	Handover	1073

A.8.3.1.1	E-UTRAN - NR handover in FR1	1073

A.8.3.1.1.1	Test Purpose and Environment	1073

A.8.3.1.1.2	Test Requirements	1076

A.8.4	Measurement procedure	1077

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	1077

A.8.4.1.1.1	Test Purpose and Environment	1077

A.8.4.1.1.2	Test Requirements	1080

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	1080

A.8.4.1.2.1	Test Purpose and Environment	1080

A.8.4.1.2.2	Test Requirements	1081

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	1081

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1081

A.8.4.2.1.1	Test Purpose and Environment	1081

A.8.4.2.1.2	Test Requirements	1085

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1086

A.8.4.2.2.1	Test Purpose and Environment	1086

A.8.4.2.2.2	Test Requirements	1089

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1090

A.8.4.2.3.1	Test Purpose and Environment	1090

A.8.4.2.3.2	Test Requirements	1093

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1094

A.8.4.2.4.1	Test Purpose and Environment	1094

A.8.4.2.4.2	Test Requirements	1097

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	1098

A.8.4.2.5.1	Test Purpose and Environment	1098

A.8.4.2.5.2	Test Requirements	1100

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	1101

A.8.4.2.6.1	Test Purpose and Environment	1101

A.8.4.2.6.2	Test Requirements	1103

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	1104

A.8.4.2.7.1	Test Purpose and Environment	1104

A.8.4.2.7.2	Test Requirements	1106

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	1107

A.8.4.2.8.1	Test Purpose and Environment	1107

A.8.4.2.8.2	Test Requirements	1109

A.8.5	Measurement performance	1110

A.8.5.1.1	SFTD accuracy	1110

A.8.5.1.1.1	Test Purpose	1110

A.8.5.1.1.2	Test Environment	1110

A.8.5.1.1.3	Test Requirements	1114

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	1114

A.8.5.2.1	SS-RSRP	1114

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1114

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1118

A.8.5.2.1.2.1	Test Purpose and Environment	1118

A.8.5.2.1.2.2	Test Parameters	1118

A.8.5.2.1.2.3	Test Requirements	1120

A.8.5.2.2	SS-RSRQ	1120

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1120

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1124

A.8.5.2.2.2.1	Test Purpose and Environment	1124

A.8.5.2.2.2.2	Test Parameters	1124

A.8.5.2.2.2.3	Test Requirements	1126

A.8.5.2.3	SS-SINR	1127

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1127

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1130

A.8.5.2.3.2.1	Test Purpose and Environment	1130

A.8.5.2.3.2.2	Test Parameters	1130

A.8.5.2.3.2.3	Test Requirements	1132

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	1120

B.1	Conditions for NR RRC_IDLE state mobility	1120

B.1.1	Introduction	1120

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1120

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1121

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1122

B.2.1	Introduction	1122

B.2.1.1	General	1122

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1122

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1122

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1122

B.2.1.4	Gain to SS-RSRP measurement point for FR1	1123

B.2.1.5	Gain to SS-RSRP measurement point for FR2	1124

B.2.1.5.1	Gain to SS-RSRP measurement point for Rx Beam Peak angle of arrival	1124

B.2.2	Conditions for NR intra-frequency measurements	1124

B.2.3	Conditions for NR inter-frequency measurements	1125

B.2.4	Conditions for NR L1-RSRP reporting	1127

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1127

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1128

B.2.5	Conditions for RRC connection release with redirection to NR	1129

B.2.6	Conditions for UE transmit timing	1130

B.2.6.1	Conditions for SSB based UE transmit timing	1130

B.2.6.2	Void	1131

B.3	RRM Requirements Exceptions	1131

B.3.1	Introduction	1131

B.3.2	Receiver sensitivity relaxation for CA	1131

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1131

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1131

B.3.2.2.1	Inter-band carrier aggregation	1131

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1131

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1132

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1132

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1132

B.3.2.4.1	Intra-band contiguous carrier aggregation	1132

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1132

B.3.3	Receiver sensitivity relaxation for DC	1132

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1132

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1132

B.3.4	Receiver sensitivity relaxation for SUL	1132

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1132

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1133

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1133

Annex C (informative):	Change history	1134