3GPP TSG-RAN WG4 Meeting # 100-e	R4-2111921

Electronic Meeting, August 16-27, 2021

Title: 	Discussion on NR repeater radiated emission requirements

Source: 	CATT

Agenda item:	9.5.3.2

Document for:	Approval

- Introduction

This contribution provides our further view on the decision of NR repeater radiated emission requirements.

- Discussion

For the FR2 emission requirements, the open issues are ACLR, OBUE and UL spurious emission.

  - ACLR

FR2 ACLR requirement have the same problem with FR1 that it should be confirmed if it can be measurable when BS requirement is considered.

The followings are two existing mmWave repeater performance specifications.

Example (1)			Example (2)

Figure 1: Two examples of mmWave repeater specification

The noise floor compared with BS ACLR requirement can be calculated as following Table 1.

Table 1: Noise floor VS. BS ACLR requirement

| P(total TRP, dBm/BW) | 25 | 25 | 25 | 25 | 20 | 20 | 20 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW (MHz) | 50 | 100 | 200 | 400 | 50 | 100 | 200 | 400 |
| NF | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Gain | 75 | 75 | 75 | 75 | 95 | 95 | 95 | 95 |
| Noise floor after PA (dBm/MHz) | -27 | -27 | -27 | -27 | -7 | -7 | -7 | -7 |
| BS ACLR absolute basic limit (dBm/MHz) | -20 | -20 | -20 | -20 | -20 | -20 | -20 | -20 |
| Ajacent channel Power for 26 dBc requirement (dBm/MHz) | -15 | -18 | -21 | -24 | -20 | -23 | -23 | -23 |

From the above analysis, if the gain is smaller than ~70 dB, the ACLR can be measured when BS ACLR requirement is reused. However, our understanding is that the gain for many mmWave repeaters may be larger than 70 dB because of the high path loss and blocking problem for mmWave. High gain and high power may be critical for mmWave repeaters. There’re some product information shown that some mmWave repeaters allows maximum path loss up to 115 ~ 130 dB. If the gain is 10 dB less than the path loss, the gain will be in the level of  > 100 dB. Therefore, from our understanding, the ACLR requirement is difficult to be defined to adapt to all of the products. But more information from repeater vendors are needed to confirm our analysis.

Observation 1: FR2 repeater ACLR is not measurable when the gain is larger than 70 dB.

  - OBUE

For the OBUE requirement, it’s straight forward to consider BS requirements. The BS OBUE category B requirements are copied as below.

Table 9.7.4.3.3-1: OBUE limits applicable in the frequency range 24.25 – 33.4 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

Table 9.7.4.3.3-2: OBUE limits applicable in the frequency range 37 – 52.6 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 41 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

Comparing the requirement with the analysis in Table 1, when repeater gain is large up to the level of 95 dB, the noise floor is -7 dBm/MHz/path (3dBm/10MHz/path) , it seems more consideration is needed. Taking Min(-5 dBm, Max(Prated,t,TRP – 31 dB, -10 dBm)) /10 MHz requirement as example, only when the Prated,t,TRP/path is larger than 38 dBm (when 95 dB gain is assumed) can meet the requirement. More analysis from the repeater vendors is needed before BS OBUE requirement is directly reused.

Observation 2: When FR2 repeater gain is large, the noise floor may be higher than the FR2 BS OBUE requirements.

  - UL spurious emission

For spurious emission requirement, the current BS category A requirement requires -13 dBm/MHz as the total TRP for all of the path for category A. Category B is more stringent. According to the analysis in 2.1, the noise floor can be -7 dBm/MHz per path. So the requirement may need more analysis. Similar to the OBUE requirement, more inputs from mmWave repeater vendors are needed for the implementation capability.

Observation 3: When FR2 repeater gain is large, the noise floor may be higher than the FR2 BS spurious emission requirements.  DL spurious emission agreement may also need to be revisited.

- Summary

This contribution provides our proposal for the remaining issues of NR repeater conducted requirements.

Observation 1: FR2 repeater ACLR is not measurable when the gain is larger than 70 dB.

Observation 2: When FR2 repeater gain is large, the noise floor may be higher than the FR2 BS OBUE requirements.

Observation 3: When FR2 repeater gain is large, the noise floor may be higher than the FR2 BS spurious emission requirements.  DL spurious emission agreement may also need to be revisited.

Reference

[1] R4-2108090, WF on emission related radiated requirements,