3GPP TSG-RAN4 Meeting #100-e	R4-2111994

# Electronic meeting, Aug. 16 – 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test case for PRS-RSRP measurement requirements for FR2 in SA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The gap configuration is in bracket. - The measurement requirements are not related to UE power class, so the PC differentiation in the time of T2 is not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the bracket in gap configuration. - Remove the PC differentiation in T2. - Some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case for PRS-RSRP measurement requirements in SA is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.7.9.3.3, A.7, A.6.6.13, A.7.6.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.7.6.10 PRS-RSRP measurements

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer

A.7.6.10.1.1	Test Purpose and Environment

The purpose of the test is to verify the PRS RSRP measurement requirements specified in Clause 9.9.3.5 for single positioning frequency layer under AWGN propagation conditions in standalone scenario. Supported test configurations are shown in table A.7.6.10.1.1-1

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The test parameters are as given in table A.7.6.10.1.1-2, and table A.7.6.10.1.1-3.

Table A. 7.6.6.1.1-1: supported test configurations for PRS RSRP measurement for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.10.1.1-2: General test parameters for PRS RSRP measurement reporting delay

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | Config 1 | NR cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| Gap Pattern Id |  | Config 1 | GP#13 or GP#24Note1 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC parameters |  | Config 1 | SMTC.1 | As specified in clause A.3.11 |
| SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| Expected RSTD | s | Config 1 | 3 |  |
| Expected RSTD uncertainty | s | Config 1 | 5 |  |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. |  |  |  |  |

Table A.7.6.10.1.1-3: Cell-specific test parameters for PRS RSRP measurement reporting delay

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.3.1 TDD |  | - |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | - |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| PRS configuration |  |  | Config 1 | PRS.1.1 FR2 |  | PRS.1.2 FR2 |  |
| PRS muting configuration |  |  | Config 1 | ‘10’ |  | ‘01’ |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -102 |  | -102 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -93 |  | -93 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -Infinity | -96 | -Infinity | -103 |
| PRS |  | dB | Config 1 | -Infinity | -3 | -Infinity | -10 |
| PRS |  | dB | Config 1 | -Infinity | -3 | -Infinity | -10 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.56 |  | -55.38 |  |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.6.10.1.2	Test Requirements

The PRS RSRP measurement time fulfils the requirements specified in Clause 9.9.3.5.The UE shall perform and report the PRS RSRP measurements for Cell 2 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

A.7.6.10.2 PRS-RSRP reporting delay test case for dual positioning frequency layer

A.7.6.10.2.1	Test Purpose and Environment

The purpose of the test is to verify the PRS RSRP measurement requirements specified in Clause 9.9.3.5 for dual positioning frequency layers under AWGN propagation conditions in standalone scenario. Supported test configurations are shown in table A.7.6.10.2.1-1

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the different frequency from the PCell.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The test parameters are as given in table A.7.6.10.2.1-2, and table A.7.6.10.2.1-3.

Table A.7.6.10.2.1-1: supported test configurations for PRS RSRP measurement for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.10.2.1-2: General test parameters for PRS RSRP measurement reporting delay

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | Config 1 | NR cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| Gap Pattern Id |  | Config 1 | GP#13 or GP#24Note1 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC parameters |  | Config 1 | SMTC.1 | As specified in clause A.3.11 |
| SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| Expected RSTD | s | Config 1 | 3 |  |
| Expected RSTD uncertainty | s | Config 1 | 5 |  |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. |  |  |  |  |

Table A.7.6.10.2.1-3: Cell-specific test parameters for PRS RSRP measurement reporting delay

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.3.1 TDD |  | - |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | - |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| PRS configuration |  |  | Config 1 | PRS.1.1 FR2 |  | PRS.1.2 FR2 |  |
| PRS muting configuration |  |  | Config 1 | ‘10’ |  | ‘01’ |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -92.7 | -92.7 | -Infinity | -85.7 |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -Infinity | -92.7 | -Infinity | -85.7 |
| PRS |  | dB | Config 1 | -Infinity | -3 | -Infinity | -10 |
| PRS |  | dB | Config 1 | -Infinity | -3 | -Infinity | -10 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.6.10.2.2	Test Requirements

The PRS RSRP measurement time fulfils the requirements specified in Clause 9.9.3.5.The UE shall perform and report the PRS RSRP measurements for Cell 2 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

<End of Change 1>