3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113048

# Electronic Meeting, August 16-27, 2021

Source:	Huawei, HiSilicon, CMCC

Title:	BS EVM requirements for support of 1024 QAM

Agenda item:	9.6.2.2

Document for:	Approval

1 Introduction

In RAN#89 e-meeting, a new work item, i.e. introduction of DL 1024-QAM for NR FR1 was approved [1]. The objective is to specify downlink 1024-QAM for NR PDSCH operation in FR1, together with related procedures, signaling and necessary RF requirements. The RAN4 objectives are listed below.

- Specify corresponding UE and BS RF core requirements [RAN4]

  - UE and BS RF core requirements are specified for stationary wireless scenarios with up to 2 layer DL MIMO

  - The cell size(s) and type of stationary wireless scenarios for which UE and BS RF core requirements are defined will be studied and decided by RAN4.

On wide area BS class applicability, the following 2 options will be further discussed in August RAN4 meeting.

- Option 1: 1024QAM applicable for all BS classes

- Option 2: 1024QAM applicable to MR BS and LA BS, but not applicable for WA BS class

Scenarios for support of 1024 QAM was discussed in [3] and more system-level simulation results are provided in [5]. Furthermore, we provide consideration on EVM requirements for 1024 QAM.

2 Discussion

2.1 Summary from simulations

From the system level simulation in [5],

Observation 1: for Macro scenarios, 1024-QAM can not provide notable performance gain

Observation 2: for Macro scenarios, few UEs can use 1024-QAM MCS

From link level simulation in [6],

Observation 3: from link level simulation, 3% TX EVM can provide observed gain for 1024 QAM compared to 256 QAM.

2.2 EVM requirements

The TX EVM is related to the whole transmitter chain. Per WF approved in last meeting [4], the following parameters were identified as needing more consideration for EVM:

  - Wider bandwidths

  - 30kHz SCS (as well as 15kHz SCS)

  - Spectral Utilization

  - CFR (Crest Factor Reduction)

  - TX linearity (in particular PA non-linearity)

  - Effects in the digital domain

  - I/Q compression

Phase noise comes from the imperfection of analog components like frequency synthesizer and VCO in the PLL. To achieve better EVM, better components and better design of PLL can be adopted. In the FR1 high band frequency, the achieved EVM by LO generator is about ~1%.

CFR module is always used in base station as an efficient way to reduce the signal PAPR, thereby improving the power efficiency of the transmitter front-end. CFR algorithm has a great impact on EVM performance. Lowering clipping threshold would bring down PAPR while increasing EVM. The simulation result of PAPR after CFR module is shown in figure 2-1, with respect to EVM impairment. It can be observed that significant power back off will be needed to achieve better EVM which will have negative impact to the overall network performance. A compromise is that to set the CFR at ~2% EVM point.

In digital domain, except CFR, the wider channel bandwidth, digital filter due to higher spectral utilization and I/Q compression all will contribute EVM degradation. We estimate it as 1.5%.

TX channel in particular PA non-linearity will also contribute about 1.5% EVM.

Base on the evaluation above, the overall achievable EVM is ~ 3%. And from link level simulation in clause 2.1, 3% TX EVM can provide performance gain over 256 QAM.

Based on the investigation above, we propose the following 3 options for further discussion.

Option 1: 1024QAM applicable to MR BS and LA BS, but not applicable for WA BS class.

Option 2: 1024QAM is applicable for all BS classes but the EVM requirements is not defined for Macro BS.

Option 3: 1024QAM is applicable for all BS classes and 3 % TX EVM is proposed for DL 1024 QAM

3 Conclusion

In this contribution, we provide consideration on the support of 1024-QAM. We propose to further discuss the following 3 options.

Option 1: 1024QAM applicable to MR BS and LA BS, but not applicable for WA BS class.

Option 2: 1024QAM is applicable for all BS classes but the EVM requirements is not defined for Macro BS.

Option 3: 1024QAM is applicable for all BS classes and 3 % TX EVM is proposed for DL 1024 QAM

Reference

- “RAN1 Chairman’s Notes”, RAN1#103-e, October 26th – November 13th, 2020.

- RP-202866, “Revised WID on Introduction of DL 1024-QAM for NR FR1”, Nokia, RAN#90e, December 7-11, 2020.

- R4-2106487, “Scenarios for support of 1024QAM”, Huawei, HiSilicon, CMCC, China Unicom

- R4-2106121, “WF on 1024QAM BS RF requirements”, Moderator (Ericsson)

- R4-2110664, “BS EVM requirements for support of 1024 QAM”, Huawei, HiSilicon, CMCC, China Unicom

- R4-2113048, “Link simulation for support of 1024QAM”, Huawei, Hisilicon

Appendix A

Table A.1. System-level evaluation assumptions

| Scenario | Urban Macro |
| --- | --- |
| Layout | 7 sites, 3 sector per site |
| Inter-BS distance | 500m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 100 MHz |
| Numerology | 30KHz |
| Duplex configuration | TDD: DDDSU |
| Channel model | 3D UMa |
| BS total Tx power | 49dBm |
| BS antenna configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.7)λ |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 6.4 dBi |
| UE Tx power | Maximum 23dBm |
| UE antenna configuration | 2Tx4Rx |
| UE antenna gain | 0dBi |
| UE antenna element gain pattern | Omnidirectional |
| UE receiver noise figure | 9dB |
| UE distribution | 10 UEs per sector; 80% indoor+20% outdoor; 100% outdoor |
| UE receiver | MMSE-IRC |
| Channel estimation | Real |
| Traffic model | 1) Full buffer; 2) FTP model with packet size 2.5Mbytes |
| MIMO scheme | SU-MIMO with rank adaptation and MU-MIMO |
| CSI scheme | Reciprocity based; 2T SRS |