# 3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2113189

# Electronic Meeting, August 16-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.175 | CR |  | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.175 on IAB EMC performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of performance requirements in IAB EMC specification is required to complete the EMC IAB standard. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draft CR introduces performance requirements in IAB EMC specification TS 38.175. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of tests configurations and performance requirements to proceed with IAB EMC testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3;9.3.3;9.4.3;9.5.2;9.5.3;9.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Start of new section ------------------------------

### 9.2.3	Performance criteria

IAB node:

The performance criteria of clause  6.1shall apply.

Ancillary equipment:

The performance criteria of clause  6.3shall apply.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

### 9.3.3	Performance criteria

IAB node:

The performance criteria of clause 6.2 shall apply.

Ancillary equipment:

The performance criteria of clause 6.4 shall apply.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

### 9.4.3	Performance criteria

IAB node:

The performance criteria of clause 6.2 shall apply.

Ancillary equipment:

The performance criteria of clause 6.4 shall apply.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

### 9.5.2	Test method and level

The test method shall be in accordance with IEC 61000-4-6 [16]:

-	The test signal shall be amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 50 kHz in the frequency range 150 kHz to 5 MHz and 1% frequency increment of the momentary frequency in the frequency range 5 MHz to 80 MHz;

-	The test level shall be severity level 2 as given in IEC 61000-4-6 [16] corresponding to 3 V rms, at a transfer impedance of 150 Ω;

-	The test shall be performed over the frequency range 150 kHz - 80 MHz;

-	The injection method to be used shall be selected according to the basic standard IEC 61000-4-6 [16];

-	Responses of stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see clause 4.3;

-	The frequencies of the immunity test signal selected and used during the test shall be recorded in the test report.

### 9.5.3	Performance criteria

IAB node:

The performance criteria of clause 6.1 shall apply.

Ancillary equipment:

The performance criteria of clause 6.3 shall apply.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

### 9.7.3	Performance criteria

IAB node:

The performance criteria of clause 6.2 shall apply.

Ancillary equipment:

The performance criteria of clause 6.4 shall apply.

----------------------------- End of proposed text ------------------------------