3GPP TSG-RAN WG4 Meeting #100-e	R4-2113194

Electronic Meeting, August 16-27, 2021

Source:	ZTE Corporation

Title:          Discussion on NR FR2 HST scenario-B

Agenda Item:	9.9.2.2

Document for:	Approval

- Introduction

The transmission scheme and deployment for scenario-B were discussed in RAN 4# 99-e meeting. While the deployment type for scenario-B was not reached and the number of beam for scenario-B needs more discussion.

In this contribution, we give our views about deployment and transmission scheme for HST_FR2 scenario-B.

- Discussion

In RAN4# 99-e meeting, the following issues of deployment for scenario-Bwere captured[1]:

- Comparison btw. uni- and bi-directional RRH deployments for Scenario-B: - From signal strength and beam coverage perspective: - FFS Bi-directional deployment’s advantage over uni-directional deployment based on deployment scenario analysis. - FFS only need to consider uni-directional deployment for Scenario-B

For comparison between uni-directional and bi-directional deployment for scenario-B, we assume 2 beams per RRH panel and 1 beam per CPE panel. Table A-1 in appendix summarizes our evaluation assumptions for these two deployments of scenario-A. Figure 2 and figure 3 show uni-directional deployment and bi-directional deployment respectively. The Rx power comparison between uni-directional deployment and bi-directional deployment is shown in figure 1.

Figure 1, Rx power comparison between uni-directional and bi-directional deployment for scenario-B when a CPE travels from Ds to 2*Ds

Figure 2, Depiction of bi-directional deployment for scenario-B

Figure 3, Depiction of uni-directional deployment for scenario-B

From figure 1 we can find that bi-directional deployment provides slightly better coverage performance than that of uni-directional deployment at the cost of more beam switching. From figure A-1 and A-2 in appendix we can find that the number of beam switching for uni-directional deployment is 3 and the number of beam switching for bi-directional deployment is 6 when only Rx power is considered.

On the other hand, the issue of propagation delay jump for uni-directional deployment was identified in RAN4 # 99-e meeting, whether the method to handle the propagation delay hopping is still under discussion in RRM session.

Observation 1: Bi-directional deployment provides slightly better coverage than that of uni-directional deployment at the cost of more beam switching.

Proposal 1: To wait for the conclusion on handling of propagation delay hopping in RRM session to determine the deployment for scenario-B.

For the number of Rx beam, the following is still FFS in RAN4# 99-e meeting[1]:

- Number of Beam(s) for uni-directional (if confirmed to be used), Scenario-B: - RRH parameter: - 2 beams per RRH panel - Other options not precluded - FFS the benefits of implementing more beams per RRH panel - UE parameter: - 1 beam per UE panel - Other options not precluded - FFS the benefits of implementing more beams per UE panel

- Number of Beam(s) for bi-directional (if confirmed to be used), Scenario-B: - RRH parameter: - 2 beams per RRH panel - Other options not precluded - FFS the benefits of implementing more beams per RRH panel - UE parameter: - 1 beam per UE panel - Other options not precluded - FFS the benefits of implementing more beams per UE panel

In general, more Rx beams mean better performance from coverage point of view at the cost of more beam switching. From RAN 4 requirement point of view, 1 beam per UE panel and 2 beams per RRH panel are preferred.

Proposal 2: 1 beam per UE panel and 2 beams per RRH panel are preferred for scenario-B.

- Conclusion

In this contribution, we provide the observation and proposals for HST_FR2 scenario-B:

Observation 1: Bi-directional deployment provides slightly better coverage than that of uni-directional deployment at the cost of more beam switching.

Proposal 1: To wait for the conclusion on handling of propagation delay hopping in RRM session to determine the deployment for scenario-B.

Proposal 2: 1 beam per UE panel and 2 beams per RRH panel are preferred for scenario-B.

- Reference

R4-2108660WF on FR2 HST Deployment Scenario Analysis

- Appendix

Table A-1: Evaluation assumptions for scenario-A bi-directional deployment

| Parameters | Deployments for scenario-B |  |
| --- | --- | --- |
|  | Uni-directional | Bi-directional |
| RRH antenna configuration | [4 8 2 1 1] dg,H=0.5λ, dg,V=0.5λ |  |
| CPE antenna configuration | [4 2 2 1 1] dg,H=0.5λ, dg,V=0.5λ |  |
| RRH antenna panel placement | -18 degree azimuth for right panel -162 degree azimuth for left panel 0 degree down tilt for right panel 0 degree down tilt for left panel |  |
| CPE antenna panel placement | 17.1887 degree azimuth for right panel 160.4282 degree azimuth for left panel 0 degree down tilt for right panel 0 degree down tilt for left panel |  |
| RRH Tx power | 30 dBm |  |
| RRH antenna height | 15 m |  |
| CPE antenna height | 5 m |  |
| Antenna loss | 3 dB |  |
| Paht loss model | RMaLOS PL1 TS38901 |  |
| Tx Beam* | Beam 1 Azimuth: -23.1986 degree Elevation: 91.5043 degree | Beam 1 Azimuth: -23.1986 degree Elevation: 91.5043 degree |
|  | Beam 2 Azimuth: -12.0948 degree Elevation: 90.8003 degree | Beam 2 Azimuth: -12.0948 degree Elevation: 90.8003 degree |
|  | - | Beam 3 Azimuth: -167.9052 degree Elevation: 90.8003 degree |
|  | - | Beam 4 Azimuth: -156.8014 degree Elevation: 91.5043 degree |
| Rx Beam* | Left panel Azimuth: 160.4282 degree Elevation: 88.8085 degree |  |
|  | Right panel Azimuth: 17.1887 degree Elevation: 88.8085 degree |  |

- * The track is parallel to x-axis. The CPE moves in the positive direction of the x-axis. The y-axis coordinate of the RRH is Dmin. The 0 degree of elevation is parallel to z-axis.

Figure A-1, Rx power of different beam and max Rx power for scenario-B bi-directional deployment

Figure A-2, Rx power of different beam and max Rx power for scenario-B uni-directional deployment