3GPP TSG-RAN WG4 Meeting #100-e	R4-2113198

Electronic Meeting, August 16-27, 2021

Source:	ZTE Corporation

Title:          Simultaneous operation of IAB child and parent links

Agenda Item:	9.17.2.1

Document for:	Approval

- Introduction

In this contribution, the analysis on the issue of Rx power imbalance for simultaneous reception between child link and parent link is provided.

- Discussion

In RAN4 #99-e meeting, the issue of power imbalance was identified[1] as following:

Simultaneous operation of IAB-node’s child and parent links by FDM

Way forward: Discuss further scenario of IAB-MT and IAB-DU share the same antenna array to support IAB simultaneous operation by FDM way includes but not limits to below aspects: - For case different beams applied for MT and DU FFS on feasible isolation between beams and associated RF impact. - For case one beam shared between MT and DU FFS on - Tx power imbalance between MT and DU for simultaneous MT TX and DU TX - RX power imbalance between MT and DU for simultaneous MT RX and DU RX - Timing difference due to IAB-MT TA if any impact - Whether simultaneous MT TX/DU RX and/or MT RX/DU TX can be removed for this scenario - Others

We collect the Rx power in system level simulation platform. The Rx power difference between DL Rx and UL Rx of the co-located IAB-DU and IAB-MT is shown in figure 1 for 4 GHz carrier frequency. The simulation assumption is summarized in the appendix.

Figure 1, Rx power difference between co-located IAB-MT and IAB-DU @4GHz

From figure 1 we can find that the difference of Rx power between child link and parent link is very large. So when one IAB performs simultaneous Rx from child node and parent node, the Rx power difference should be considered.

Observation 1: Based on the simulation result, the Rx power differences between parent link and child link is very large which can be seen that the CDF of 0.5 corresponds to 40dB.

To determine an appropriate level for Rx power imbalance between MT and DU for simultaneous MT RX and DU RX, the BS ICS requirement could be a starting point:

FR2: e.g. around 14dBc power imbalance between interfering signal and wanted signal.

FR1: e.g. around 25dBc power imbalance between interfering signal and wanted signal.

Proposal 1: To use BS ICS requirement as a starting point for determining the range of Rx power imbalance between MT and DU when simultaneous RX is performed by a shared Rx chain.

- Conclusion

Based on the analysis, the following observation and proposal are given:

Observation 1: Based on the simulation result, the Rx power differences between parent link and child link is very large which can be seen that the CDF of 0.5 corresponds to 40dB.

Proposal 1: To use BS ICS requirement as a starting point for determining the range of Rx power imbalance between MT and DU when simultaneous RX is performed by a shared Rx chain.

- Reference

2108640 WF on simultaneous operating and timing case#6/7 for enhanced IAB

- Appendix

Table1 System simulation assumption

| Deployment scenario | Heterogeneous scenario (dense urban), 7 cells, 3 sectors/cell Two layer Macro layer: Hex. Grid (all macro BSs are IAB-donors) 7 sites Micro layer: Random drop (All micro BSs are all outdoor and are IAB-nodes) - 3 micro BSs per macro BS See Figures A.2.1-3 of TR 38.802 IAB-node is assumed to have 3 panels with 120 degree shift relative to each other. orientation options: Option 2: Random orientation (independent of topology) Panel orientation is assumed fixed for a simulation run. |
| --- | --- |
| Frequency | 4GHz, 30GHz |
| Antenna configurations [M,N,P,Mg,Ng] | gNB [8,4,2,1,1] IAB [8,4,2,1,1] UE [2,2,2,1,2] |
| BS antenna element gain + connector loss | See Table A.2.1-4 of TR38.802 |
| UE antenna gain | Follow the modeling of TR36.873 |
| BS receiver noise figure | Below 6GHz: 5dB Above 6GHz: 7dB |
| UE receiver noise figure | Below 6GHz: 9dB Above 6GHz: 13dB |
| BS antenna height | 25m |
| UE antenna height | 1.5m |
| ISD | 500m |
| BS and UE Power | Macro：44dbm for 4G Micro：33dbm UE ：23dbm |
| IAB node topology formation | Based on RSRP |
| Number of UE per macro sector | 30 |
| Min Distance | For ISD=500m Minimum distance between IAB Nodes: 40m Minimum distance between gNB and UE:35m Minimum distance between IAB Node and UE:10m Minimum distance between IAB Node and gNB: 40m |