3GPP TSG-RAN WG4 Meeting # 100-e                	R4-2113243

Electronic Meeting, 16th - 27th August 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on beam failure detection and link recovery requirements with CCA

Agenda item: 	6.1.1.6.3.13

Document for: 	Discussion

Introduction

This paper presents Nokia’s view on remaining test cases configuration of beam failure detection and link recovery.

Discussion

As agreed in RAN4 in previous meetings, the test cases should not include behavior when exceeding Lmax in test cases. In the case of beam failure detection this is specified in Clause 8.5A.5.2 of 38.133 [1]:

Table 8.5A.5.2-1: Evaluation period TEvaluate_CBD_SSB_CCA Configuration; TEvaluate_CBD_SSB_CCA (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil((3 + LCBD) ´ P) ´ TSSB). DRX cycle > 320ms; Ceil((3 + LCBD) ´ P) ´ TDRX. Note 1: TSSB is the periodicity of SSB in the set q1. TDRX is the DRX cycle length. Note 2: LCBD is the number of CBD-RS SSB occasions not available at the UE during TEvaluate_CBD_SSB_CCA where LCBD ≤ LCBD,max. [The UE is not required to determine the availability of SSB occasions more frequent than once per DRX cycle length, when configured with DRX.] Note 3: LCBD,max=7 for Max(TDRX, TSSB) ≤ 40 assuming TDRX=0 for non-DRX, LCBD,max=5 for 40 < Max(TDRX, TSSB) ≤ 320, LCBD,max=3 for TDRX > 320. Note 4 If LCBD>LCBD,max, the UE shall assume no new candidate beams are found for this evaluation period..

In the case of non DRX, since the STMC periodicity of NR-U test cases is 20 ms, the limit for unavailable SMTC occations is LCBD,max=7 during the evaluation period of TEvaluate_CBD_SSB_CCA. In the case of DRX, the configuration 7 is used for the beam failure detection and link recovery, with DRX cycle of 640 ms. That results in LCBD,max= 3 when DRX is used.

LCBD,max = 7 for beam failure detection and link recovery test cases without DRX, and LCBD,max = 3 for beam failure detection and link recovery test cases with DRX

For the beam failure detection and link recovery test cases , configure LCCA_DL=7 for TCs without DRX, LCCA_DL=3 for TCs with DRX and WCCA_DL= TEvaluate_CBD_SSB_CCA.

Conclusion

This paper has presented Nokia’s views on RRM performance requirements of beam failure detection and link recovery on unlicensed bands. From this discussion we have derived the following observations and proposals:

LCBD,max = 7 for beam failure detection and link recovery test cases without DRX, and LCBD,max = 3 for beam failure detection and link recovery test cases with DRX

For the beam failure detection and link recovery test cases , configure LCCA_DL=7 for TCs without DRX, LCCA_DL=3 for TCs with DRX and WCCA_DL= TEvaluate_CBD_SSB_CCA.

References

- 3GPP TS 38.133: “NR; Requirements for support of radio resource management”