3GPP TSG-RAN WG4 Meeting #100-e                                                    	R4- 2113442

Online, 16th August – 27th August, 2021

Source:	Vodafone

Title:	TP for TR 37.717-21-11: DC_28-32_n1

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_28-32_n1.

The proposed TIB,c, RIB and MSD values were derived from those present in 36101 for CA_1-28-32.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_28-32_n1

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_28A-32A_n1A | DC_28A_n1A |

### 5.x.2	Co-existence studies

Table 5.x.2-1 lists the Band 28A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2-1: Band 28 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1496 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2244 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1172 – 1277 |  |  | 2623 – 2728 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 424 – 574 |  |  | 3092 – 3257 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3326 – 3476 |  |  | 4543 – 4708 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 653 – 798 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 129 – 324 |  |  | 5012 – 5237 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 2344 – 2554 |  |  | 5246 – 5456 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4029 – 4224 |  |  | 6463 – 6688 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6932 – 7217 |  |  | 832 – 1072 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4264 – 4534 |  |  | 1596 – 1851 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 – 8668 |  |  | 4732 – 4972 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 – 7436 |  |  | 5949 – 6204 |  |  |

Based on Table 5.x.2-1,

-	2nd order harmonics may fall into Rx frequencies of bands 11, 21, 32, 45, 46, 47, 50, 51, 74, 75, 76, 91, 92, 93 and 94

-	3rd order harmonics may fall into Rx frequencies of band 1, 4, 10, 23, 65, 66 and 77

-	2nd order IMD may fall into Rx frequencies of bands 7, 41 and 90

-	3rd order IMD may fall into Rx frequencies of bands 31, 42, 52, 72, 73, 77, 78, 79, 87 and 88

-	4th order IMD may fall into Rx frequencies of bands 30, 40, 41, 46, 46, 53, 77 and 90

-	5th order IMD may fall into Rx frequencies of bands 3, 5, 6, 8, 9, 18, 19, 26, 27, 35 and 79

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.x.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.x.2-2: 2UL Band 28 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD5 |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD4 |
|  | 2400 | - | 2494 | Yes | Asia | IMD4 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD4 |

The requirements for spurious emission band UE coexistence exist for DC_28_n1 in 38101-3.

### 5.x.3	∆TIB and ∆RIB values

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-32A_n1 | 28 | 0.7 |
|  | n1 | 0.3 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A-32A_n1 | 28 | 0.2 |
|  | 32 | 0 |
|  | n1 | 0 |

### 5.x.4	Reference sensitivity exceptions

No additional MSD required for harmonics compared to DC_1-32_n28.

<End of Text Proposal>