3GPP TSG-RAN WG4 Meeting#100-e      	R4-2113487

E-meeting, 16th – 27th August, 2021

Title:	1024QAM simulation results

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	9.6.2.1	Deployment and link level simulation [NR_DL1024QAM_FR1-Core]

Document for:	Discussion

# 1	Introduction

This contribution relates to a work item “Introduction of DL 1024QAM for NR FR1” [1]. In the latest RAN4#99-e meeting we already presented initial simulation results in [3]. In this paper we show further simulation results based on latest simulations assumption updates agreed during RAN4#99-e meeting.

# 2	Discussion

A way forward was agreed in RAN4#99-e meeting [2] with updated simulations assumptions for the work item. Table 1 contains simulation parameters based on [2].

Table 1. Simulation parameters from [2]

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| CBW | 40 MHz |
| SCS | 15kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, Maximum Doppler frequency: 5Hz (used as Baseline) TDL-D 10ns delay spread, Maximum Doppler frequency: 5Hz |
| MCS | 256QAM: MCS 24 in TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH, and other MCSs are not precluded 1024QAM: MCS 24 in the following Table according to the agreement in RAN1 #104, and other MCSs are not precluded MCS Index; Modulation Order; Target code Rate R x [1024]; Spectral. IMCS; Qm; efficiency. 23; 10; 805.5; 7.8662. 24; 10; 853; 8.3301. 25; 10; 900.5; 8.7939. 26; 10; 948; 9.2578. |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8 HARQ processes, maximum 4 transmissions |
| RV sequence | {0,2,3,1} |
| RANK | One and Two |
| BS antenna configuration | Take 1 and 2 Tx antenna as baseline. Companies also welcome to bring 8TX results if they demonstrate differing trends |
| UE antenna configuration | 4 |
| Antenna correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE-IRC |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | DMRS Type 1, Single symbol, additional DM-RS (dmrs-AdditionalPosition): pos1 |
| txEVM | 2%, 2.5%, 3% |
| rxEVM | 0.5%, 1%, 1.5%, 2%, 3% |

Simulations results are shown in Figure 1 to Figure 6. These simulations are following the parameters shown in Table 1. Results in Figure 1 and Figure 2 are for 1x4 case, Figure 3 and Figure 4 are for 2x4 rank1 and Figure 5 and Figure 6 are for  2x4 rank2.

Simulation results show that 1024QAM achieves better throughput than 256QAM at reasonable SNR. Tx EVM 2.5% seems to perform adequately in all cases. Rx EVM should preferably be less than 2%. This combination gives better performance than 256QAM also in rank2 with LOS channel. Rank2 NLOS does not perform well with the simulation assumptions, however with Rx EVM 1.5% even that can achieve higher throughput with 1024QAM than 256QAM.

Observation 1: Simulation results for 1024QAM show higher throughput compared to 256QAM for higher, but still reasonable SNR conditions and reasonable EVM requirements.

Observation 2: Simulation results for 1024QAM show that 3% Tx EVM reduces performance in many simulation cases, in some even below 256QAM.

Figure 1. Simulation results for 1x4 and TDL-A 10ns.

Figure 2. Simulation results for 1x4 and TDL-D 10ns.

Figure 3. Simulation results for 2x4 rank1 and TDL-A 10ns.

Figure 4. Simulation results for 2x4 rank1 and TDL-D 10ns.

Figure 5. Simulation results for 2x4 rank2 and TDL-A 10ns.

Figure 6. Simulation results for 2x4 rank2 and TDL-D 10ns.

# 3	Conclusion

In this paper we have provided simulation results for 1024QAM. We have also made following observations:

Observation 1: Simulation results for 1024QAM show higher throughput compared to 256QAM for higher, but still reasonable SNR conditions and reasonable EVM requirements.

Observation 2: Simulation results for 1024QAM show that 3% Tx EVM reduces performance in many simulation cases, in some even below 256QAM.

# 4 	References

- RP-202886, “Revised WID on Introduction of DL 1024QAM for NR FR1”, Nokia

- R4-2108077, “WF on Link level simulation assumptions”, CATT

- R4-2110141, “1024QAM simulation assumptions and preliminary results”, Nokia, Nokia Shanghai Bell