3GPP TSG-RAN WG4 Meeting #100-e	R4-2114032

# Electronic, 16th– 27th August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-2 | CR |  | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.176-2: Correction of applicability rules for demodulation performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Applicability rules agreed for IAB performacnce verification are not captured in specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of Applicability rules for IAB-DU Clarrification of applicability rules for IAB-MT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance for IAB node cannot be guaranteed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.3, 8.1.1.3.4, 8.2.2, 8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.174 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF 1st CHANGE

##### 8.1.1.3.3	Applicability of PUCCH performance requirements

8.1.1.3.3.1	Applicability of requirements for different formats

Unless otherwise stated, PUCCH requirement tests shall apply only for each PUCCH format declared to be supported (see [D.102] in table 4.6-1).

8.1.1.3.3.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, PUCCH requirement tests shall apply only for each subcarrier spacing declared to be supported (see D.7 in table 4.6-1). If multiple subcarrier spacings are declared to be supported, each supported PUCCH format can be tested on one subcarrier spacing.

8.1.1.3.3.3	Applicability of requirements for different channel bandwidths

For each subcarrier spacing declared to be supported by the IAB-DU, the test requirements for a specific channel bandwidth shall apply only if the IAB-DU supports it (see D.7 in table 4.6-1).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

8.1.1.3.3.4	Applicability of requirements for different configurations

Unless otherwise stated, PUCCH format 3 requirement tests shall apply only for the additional DM-RS configuration declared to be supported (see [D.104] in table 4.6-1). If both options (without and with additional DM-RS) are declared to be supported, the tests shall be done for either without or with additional DM-RS; the same chosen option shall then be used for all tests.

Unless otherwise stated, PUCCH format 4 requirement tests shall apply only for the additional DM-RS configuration declared to be supported (see [D.105] in table 4.6-1). If both options (without and with additional DM-RS) are declared to be supported, the tests shall be done for either without or with additional DM-RS; the same chosen option shall then be used for all tests.

8.1.1.3.3.5	Applicability of requirements for multi-slot PUCCH

Unless otherwise stated, multi-slot PUCCH requirement tests shall apply only if the IAB-DU supports it (see [D.107] in table 4.6-1).

##### 8.1.1.3.4	Applicability of PRACH performance requirements

8.1.1.3.4.1	Applicability of requirements for different formats

Unless otherwise stated, PRACH requirement tests shall apply only for PRACH format declared to be supported (see D.103 in table 4.6-1).

For IAB-DU declares to support more than one PRACH formats, limit the number of tests to any two cases chosen by the manufacturer. If IAB-DU declares to support more than one PRACH formats where formats for both long and short PRACH sequences are presented, require choosing formats with different sequences (see [D.103] in table 4.6-1).

8.1.1.3.4.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, for each PRACH format with short sequence declared to be supported, for each FR, the tests shall apply only for the smallest supported subcarrier spacing in the FR (see [D.103] in table 4.6-1).

8.1.1.3.4.3	Applicability of requirements for different channel bandwidths

Unless otherwise stated, for the subcarrier spacing to be tested, the test requirements shall apply only for anyone channel bandwidth declared to be supported (see D.7 in table 4.6-1).

END OF 1st CHANGE

START OF 2nd CHANGE

### 8.2.2	Demodulation performance requirements

#### 8.2.2.1	General

##### 8.2.2.1.1	Applicability rule for IAB-MT

8.2.2.1.1.1	General

Unless otherwise stated, for a IAB-MT declared to support more than 2 demodulation branches (for IAB-MT type 1-O and IAB-MT type 2-O), the performance requirement tests for 2 demodulation branches shall apply, and the mapping between connectors and demodulation branches is up to IAB-MT implementation.

The tests requiring more than [20] dB SNR level are set to N/A in the test requirements.

8.2.2.1.1.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, the tests shall apply only for each subcarrier spacing declared to be supported (see D.7 in table 4.6-1).

8.2.2.1.1.3	Applicability of requirements for TDD with different UL-DL patterns

Unless otherwise stated, for each subcarrier spacing declared to be supported, if IAB-MT supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns shall be used for all tests.

8.2.2.1.1.4	Applicability of requirements for optional IAB-MT features

{Placeholder for possible introduction in next releases}

8.2.2.1.1.5	Applicability of requirements for mandatory IAB-MT features with capability signalling

The performance requirements in Table 8.2.2.1.1.5-1 shall apply for IAB-MT which support mandatory IAB-MT features with capability signalling only.

Table 8.2.2.1.1.5-1: Requirements applicability for mandatory features with IAB-MT capability signalling

| IAB-MT feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| 256QAM modulation scheme for PDSCH for FR1 (pdsch- 256QAM-FR1) | FR1 TDD | PDSCH | Clause 8.2.2.2.5.1 (Test 1-1) |  |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR1 TDD FR2 TDD | PDSCH | Clause 8.2.2.2.5.1 (Test 2-1) Clause 8.2.2.2.5.2 (Test 2-1, 2-2, 2-3) | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed IAB-MT PDSCH MIMO layers capability |

END OF 2nd CHANGE

START OF 3rd CHANGE

### 8.2.3	CSI reporting requirements

#### 8.2.3.1	General

##### 8.2.3.1.1	Applicability of requirements

###### 8.2.3.1.1.1	Applicability of requirements for PMI and RI reporting

Unless otherwise stated, the minimum performance requirements in Clause 8.2.3.3 and 8.2.3.4 shall apply only, if PMI/RI reporting is declared to be supported (see D.201 and D.202 in Table 4.6-1).

###### 8.2.3.1.1.2	Applicability of requirements for optional IAB-MT features

{Placeholder for possible introduction in next releases}

###### 8.2.3.1.1.3	Applicability of requirements for mandatory IAB-MT features with capability signalling

The performance requirements in Table 8.2.3.1.1.3-1 shall apply for IAB-MT which support mandatory IAB-MT features with capability signalling only.

Table 8.2.3.1.1.3-1: Requirements applicability for mandatory features with IAB-MT capability signalling

| IAB-MT feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR1 TDD | CQI | Clause 8.2.3.2 | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed IAB-MT PDSCH MIMO layers capability |
|  |  | PMI | Clause 8.2.3.3 |  |
|  |  | RI | Clause 8.2.3.4 |  |
| Supported maximum number of ports across all configured NZP-CSI-RS resources per CC (maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC) | FR1 TDD | PMI | Clause 8.2.3.3 | The requirements apply only in case the number of NZP-CSI-RS ports in the test case satisfies IAB-MT capability on maximum number of NZP-CSI-RS ports |
| Support of 1 port PTRS (onePortPTRS) | FR2 TDD | CQI | Clause 8.2.3.2 |  |
|  |  | PMI | Clause 8.2.3.3 |  |
|  |  | RI | Clause 8.2.3.4 |  |

END OF 3rd CHANGE

START OF 4th CHANGE

Table 8.2.3.3.4.2-1: Test parameters for testing PMI reporting requirements

| Parameter |  | Unit | FR1 | FR2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 | 100 |
| Subcarrier spacing |  | kHz | 30 | 120 |
| Duplex Mode |  |  | TDD | TDD |
| TDD DL-UL configuration |  |  | 7D1S2U, S=6D:4G:4U | 3D1S1U, S=10D:2G:2U |
| Propagation channel |  |  | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) High XP 8 x 2 (N1,N2) = (4,1) | 2 x 2 ULA Low |
| Beamforming Model |  |  | As specified in Annex J.3.1 | As specified in Annex J.3.1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | Test for 4 TX ports: 4 Test for 8 TX ports: 8 | 2 |
|  | CDM Type |  | Test for 4 TX ports: FD-CDM2 Test for 8 TX ports: CDM4 (FD2, TD2) | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  | Test for 4 TX ports: Row 4 (0,-) Test for 8 TX ports: Row 8, (4,6) | Row 3, (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Test for 4 TX ports, 2RX: (13,-) Test for 8 TX ports: (5,-) | (13,-) |
|  | CSI-RS interval and offset | Slot | 10/1 | 8/1 |
| ReportConfigType |  |  | Periodic | Periodic |
| Sub-band Size |  | RB | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | 10/9 | 8/3 |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | Test for 4 TX ports: (2,1) Test for 8 TX ports: (4,1) | NA |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | Test for 4 TX ports: (4,1) Test for 8 TX ports: (4,1) | NA |
|  | CodebookSubsetRestriction |  | Test for 4 TX ports: 11111111 Test for 8 TX ports: 0x FFFF | 001111 |
|  | RI Restriction |  | Test for 4 TX ports: 00000001 Test for 8 TX ports: 00000010 | NA |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| CQI/RI/PMI delay |  | ms | 5.5 | 1.375 |
| Measurement channel |  |  | Test for 4 TX ports: M-FR1-A.3.5-1 Test for 8 TX ports: M-FR1-A.3.5-2 | M-FR2-A.3.5-3 |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms FR1 / 0.125 ms FR2 granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the IAB-MT reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction shall be used as specified in Annex J.2.3.2.3. Note 5: SSB, TRS, CSI-RS and/or other unspecified test parameters with respect to TS 38.101-4 [18] are left up to test implementation, if transmitted or needed. |  |  |  |  |

END OF 4th CHANGE