3GPP TSG-RAN WG4 Meeting #100-e                                         R4-2114516

Electronic Meeting, August 16-27, 2021

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.839: Principles for simultaneous Rx/Tx capability

Agenda Item:	8.43.2

Document for:	Approval

# Background

This contribution provides the TP for TR of simultaneous Rx/Tx capability based on [1-5].

# References

[1] R4-2105377, WF on Simultaneous Rx/Tx, Huawei, HiSilicon.

[2] R4-2107841, WF on applicability and rules for simultaneous RxTx, Huawei, HiSilicon

[3] R4-2107842, WF on release independent for simultaneous Rx/Tx, CHTTL, Huawei

[4] R4-2107843, WF on simultaneous Rx/Tx for the inter-band combinations having more than two bands, Qualcomm

[5] R4-2108007, WF on simultaneous Rx/Tx for synchronous/asynchronous NR-DC, Apple

[6] R4-2107311, TR skeleton for TR38.839 Simultaneous RxTx band combinations

# Text Proposal

<Start of Text Poposal>

# 5	Principles for simultaneous Rx/Tx UE capability

## 5.1	Applicability of simultaneous Rx/Tx capability for similar band combination

The following consensus was reached in RAN4 on applicability of simultaneous Rx/Tx capability for similar band combination.

- Simultaneous Rx/Tx capability for LTE CA band combination can be applied for the corresponding inter-band NR CA band combination.

- Same simultaneous Rx/Tx capability can be applied for the corresponding band combination among NR CA, NR DC and NR EN-DC

## 5.2	Rules to decide simultaneous Rx/Tx capability for a band combination

### 5.2.1	FR1+FR1 FDD-TDD band combination

Editor note: This band combination category includes SUL band combination where SUL band is derived from FDD band.

<To be added>

### 5.2.2	FR1+FR1 TDD-TDD band combination

Editor note: This band combination category includes SUL band combination where SUL band is derived from TDD band.

<To be added>

### 5.2.3	FR1+FR2 FDD-TDD band combination

For FR1+FR2 FDD-TDD band combination, the capability shall be mandatory if FR1 FDD band (<4GHz) is aggregated with FR2 TDD bands.

### 5.2.4	FR2+FR2 TDD-TDD band combination

<To be added>

## 5.3	Release independent for simultaneous Rx/Tx

The following consensus was reached in RAN4 on release independent for simultaneous Rx/Tx capability.

- At least in the Rel-15 RAN4 specifications, the indication of mandatory/optional simultaneous Rx/Tx capability support for each combination should remain unchanged.

- For the FDD-TDD and TDD-TDD band combination introduced in Rel-17 38.101-1, 38.101-2, 38.101-3, the simultaneous Rx/Tx capability affiliated to the band combination is considered as release independent from Rel-15 if the band combination is mandatory to support simultaneous Rx/Tx in Rel-17.

  - For example: A band combination is specified as mandatory support of simultaneous Rx/Tx in Rel-17. If Rel-15 or Rel-16 UE wants to support this band combination with release independent manner, it shall support simultaneous Rx/Tx.

<End of Text Poposal>