3GPP TSG-RAN WG4 Meeting # 100-e	R4-2115000

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei, HiSilicon

Title: 	TP on overlapping UE channel bandwidths (Overlapping CA)

Agenda Item:	10.2.3

Document for:	Approval

1 Introduction

In last RAN meeting, a new SI has been approved on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths. One of the objective is to study the use of overlapping UE channel bandwidths.

Study the use of overlapping UE channel bandwidths (from both UE and network perspective) to cover operator’s license spectrum for both UL and DL, and if new gNB channel bandwidths are needed.

NOTE:	For all considered solutions, new (dedicated) channel filters (e.g. non-integer-multiples of 5MHz) are not considered for the UE and not prioritized for the gNB.

The comparison of the two solutions were discussed in [2].

Solution 1: new channel bandwidth for gNB and legacy channel bandwidth for UE (Overlapping UE CBW in [4])

New channel bandwidth for gNB

UE1: legacy channel bandwidth

UE2: legacy channel bandwidth

Solution 2: Overlapping CA

Overlapping CA for gNB

Overlapping CA for UE3

UE1: legacy channel bandwidth

UE2: legacy channel bandwidth

In this contribution, we provide text proposal for TR 38.844 on overlapping CA approach.

References

RP-202103, New SID: Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, T-Mobile USA, Ericsson

R4-2101507, Consideration for overlapping UE channel bandwidths, Huawei

R4-2103264, Way forward on overlapping CBW method, Nokia

R4-2105419, Way forward on Evaluation of IrregularBW Approaches,

R4-2110662, Evaluation for overlapping UE channel bandwidths

<TP for TR 38.844>

6.2.3	Overlapping CA (two cells)

6.2.3.1      General Aspects

Using the Overlapping CA approach to support irregular spectrum is one of the methods without the need of introduction of new dedicated channel bandwidths for both UE and BS. Figure 6.2.3-1 shows overlapping CA from network perspective, in which BS supports the intra-band overlapping CA, while UEs only supports the single CC with the existing channel bandwidth. And figure 6.2.3-2 shows overlapping CA from UE perspective, in which both BS and UE support the intra-band overlapping CA to use all RBs.

Even two CCs are overlapped in frequency domain, the network can configure different CCs with non-overlapping BWP to avoid the conflict of physical signals or channels.

UE1: legacy channel bandwidth

UE2: legacy channel bandwidth

Overlapping CA for gNB

Figure 6.2.3-1: Overlapping CA from network perspective

Overlapping CA for gNB

UE3: overlapping CA of CC1 and CC2

CC1: BWP#1

CC2: BWP#2

Figure 6.2.3-2: Overlapping CA from UE perspective

In summary,

- No new gNB channel bandwidth is required

- Legacy UE using existing lower regular NR channel bandwidth can operate in either carrier

- Overlapping CA approach needs to have PRB grid alignment between overlapping CCs

- UE perspective, overlapping CA is optional supported in DL only

< Next changed section>

6.3.x	Overlapping CA (two cells)

From network perspective:

For gNB, no new filter for RF channel is needed. The CA implementation can be reused. The only update is to allow the configuration that the two carriers can be partially overlapped by adjusting the channel spacing. The network can prevent collisions between the two component carriers.

For UE, there is no impact and fully backwards compatible.

From UE perspective:

For gNB, there is no further requirements compared to the above description for overlapping CA from network perspective.

For UE, it is optional support in DL only. For the UE supports DL intra-band non-contiguous CA with corresponding channel bandwidth(s), overlapping CA can be considered by support the configuration with partially overlapping carriers.  And in the case no new channel filter for UE is needed.

For overlapping CA solution, the legacy channel bandwidth should be supported, hence the minimum guardband should not be less than the minimum guardband of lower UE channel bandwidth than operator licensed bandwidth. To support legacy UEs, channel raster should be applied for each UE channels. And in order to simplify the resource schedule and make the use of single SSB for CA operation, RB alignment is required. Table 6.3.x-1 show the channel spacing and spectrum utilization for the example channel bandwidths.

Table 6.3.x-1: channel spacing and spectrum utilization

| Channel Bandwidth(MHz) | SCS(kHz) | CA combos | Nominal channel spacing (MHz) | Transmission bandwidth configuration NRB | Spectrum utilization (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 15 | 5+5 | 0.9 | 30 | 90 |
| 7 | 15 | 5+5 | 1.8 | 35 | 90 |
| 11 | 15 | 10+10 | 0.9 | 57 | 93.3 |
| 12 | 15 | 10+10 | 1.8 | 62 | 93 |
| 13 | 15 | 10+10 | 2.7 | 67 | 92.8 |

< Next changed section>

6.7.x	Overlapping CA (two cells)

The CA framework defines the transmitter emission and receiver blocking at the edge of carriers. Limited impacted is expected to enable coexistence with neighbouring channels for this scenario. Overlapping CA channel spacing need to be updated to consider channel raster, minimum guard band and RB alignment.

For 15 KHz SCS and 100 KHz channel raster,

Channel spacing=2*BWChannel-(BWChannel1+BWChannel2)1.80.9 (MHz)

For NR operating bands with 15 kHz channel raster,

Channel spacing=2*BWChannel-(BWChannel1+BWChannel2)0.18*2n+10.18*2n (MHz)

with

n=μ0

6.7.y	Overlapping UE CBW

If the irregular channel bandwidths are explicitly defined in the specification, it will create huge technical specification work. For each channel bandwidth, the guard band size and the transmission bandwidth configuration need to be specified and used as a basis for defining transmitter and receiver requirements. The identified impact to BS core specification for a new channel BW is shown in following Table 6.7.y-1.

Table 6.7.y-1 Analysis on the impact to BS core specification for a new channel BW

| Subject | Clause in 36.104/38.104 | Requirement | Assessment for new channel BW |
| --- | --- | --- | --- |
| General | 5.3.2 | Transmission bandwidth configuration | the Transmission bandwidth configuration NRB for the new CBW need to be defined |
|  | 5.3.3 | Minimum guardband and transmission bandwidth configuration | The minimum guardband for the new CBW need to be defined |
|  | 5.3.5 | BS channel bandwidth per operating band | new CBW need defined per band |
| Transmitter | 6.3.3 | Total power dynamic range | it is a NRB related requirement |
|  | 6.6.2 | Occupied bandwidth | BS channel bandwidth should be defined |
|  | 6.6.3 | ACLR | The filter are set using transmission bandwidth configuration (BWConfig). It need to be defined for testing |
|  | 6.7 | Transmitter intermodulation | the interfering is defined according to BS channel bandwidth |
| Receiver | 7.2 | Reference sensitivity level | for 15 KHz SCS, 25 RB and 106 RB FRC which can be reused |
|  | 7.3 | Dynamic range | interfering signal level is according to BS channel bandwidth |
|  | 7.4 | In-band selectivity and blocking | the position of interfering signal is defined according to BS channel bandwidth/transmission bandwidth configuration |
|  | 7.7 | Receiver intermodulation | the position of interfering signal is defined according to BS channel bandwidth/transmission bandwidth configuration |
|  | 7.8 | In-channel selectivity | it is defined according to BS channel bandwidth |

It is concluded that new dedicated BS channel bandwidths for irregular channel bandwidths are not defined explicitly in the specification.

< The end of TP >