3GPP TSG-RAN WG4 Meeting # 100-e                                                R4-2115024

# Electronic Meeting, August 16-27, 2021

Agenda item:			8.44

Source:	Moderator (CMCC)

Title:             	         Email discussion summary for [100-e][124] NR_DSS_34_39

Document for:	Information

Introduction

In RAN#92E meeting, a new WI on LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 was approved.

The work item aims to specify spectrum sharing requirements for n34 and n39.

•	Introduce UL 7.5kHz frequency shift for 15kHz SCS operation from Rel-15 [RAN4]

•	Study for backward compatibility

•	Study for which release UL shift can be made mandatory

This email discussion includes contributions in agenda 8.44, the targets of email discussion based on companies’ contributions submitted in this e-meeting are as below:

- 1st round:

  - Discuss backward compatibility for which release UL shift can be made mandatory, and provide comments on the CRs and contributions.

- 2nd round:

  - Strive to approve CRs or WF.

Topic #1: Introduction of uplink 7.5KHz frequency shift for n34 and n39

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112192 | CMCC | Proposal 1: There is no backward compatibility issue to introduce UL 7.5KHz frequency shift from Rel-15 for n34 and n39 Proposal 2: it is proposed to mandatory support UL 7.5KHz frequency shift from Rel-15 for n34 and n39. |
| R4-2112346 | Apple | Proposal 1: UL 7.5kHz shift is kept optional in Rel-15 for TDD bands n34 and n39 (not preventing Rel-15 UEs from implementing and supporting it). Proposal 2: UL shift is mandatory for TDD bands n34 and n39 in later releases. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 Uplink 7.5KHz frequency shift for n34 and n39

Issue 1-1-1: Release independent issue

- Proposals

  - Option1:  No backward compatibility issue to introduce UL 7.5KHz frequency shift from Rel-15 for n34 and n39, and it is proposed to mandatory support UL 7.5KHz frequency shift from Rel-15 for n34 and n39.

  - Option2: UL 7.5kHz shift is kept optional in Rel-15 for TDD bands n34 and n39, and UL shift is mandatory for TDD bands n34 and n39 in later releases.

- Recommended WF

  - TBA. Collect companies’ view in 1st round

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| CMCC | Issue 1-1-1: Release independent issue Option 1. Based on our understanding, no UE support n34 and n39 so far, there is no backward compatibility issue to introduce mandatory support of UL 7.5KHz frequency shift from Rel-15. We would like to understand the concern from option2. |
| Huawei | Issue 1-1-1: Release independent issue Option 1. With clarification by operator, we support that UL 7.5kHz shift is mandatory for n34 and n39 from Rel-15. |
| Apple | Rel-15 is a functionally closed release, which means that any changes mandating certain features, which did not even exist in that release, cannot be accepted. We had the same discussion for other TDD bands, such as n38 and n40, and we would strongly insist on respecting the conclusions. As explained and acknowledged by 3GPP, we do not want to create a precedent of having non-compliant Rel-15 UEs just because we think we can change the Rel-15 specification. Nevertheless, we are open to explore a solution, which has been adopted for other TDD bands, according to which we enable a particular feature making it optional in earlier releases. It means that this feature can be supported by new Rel-15 UEs coming to the market as if it were mandatory to them. |
| XXX | Issue 1-1-1: Issue 1-1-2: …. Others: |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112193 | Company A |
|  | Company B |
| R4-2112194 | Company A |
|  | Company B |
| R4-2112195 | Company A |
|  | Company B |
| R4-2114376 | CMCC: CRs can be discussed later when we reach consensus on the release issue. |
|  | Company B |
| R4-2114377 | CMCC: CRs can be discussed later when we reach consensus on the release issue. |
|  | Company B |
| R4-2114378 | CMCC: CRs can be discussed later when we reach consensus on the release issue. |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreement and candidate options: Issue 1-1-1: Release independent issue - Candidate options - Option1: It is proposed to mandatory support UL 7.5KHz frequency shift from Rel-15 for n34 and n39. - Option2: UL 7.5kHz shift is kept optional in Rel-15 for TDD bands n34 and n39, and UL shift is mandatory for TDD bands n34 and n39 in later releases. - Tentative agreements - UL 7.5kHz shift is FFS (optional/mandatory) for n34 and n39 in Rel-15 and Rel-16 - UL 7.5kHz shift is mandatory for n34 and n39 from Rel-17 - No requirements issue for DSS n34 and n39. Recommendations for 2nd round: Based on 1st round of comments collection. The tentative agreements will be captured in WF on DSS for n34 and n39. Moderator suggested that the 2nd round discussion should focus on how to handle the UL 7.5KHz shift release independent issue in Rel-15 and Rel-16. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on DSS for n34 and n39 | CMCC |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2112193 | Return to |
| R4-2112194 | Return to |
| R4-2112195 | Return to |
| R4-2114376 | Return to |
| R4-2114377 | Return to |
| R4-2114378 | Return to |

Discussion on 2nd round (if applicable)

| R4-2114945 | WF on DSS for n34 and n39 |
| --- | --- |

| Company | Comments |
| --- | --- |
| XXX | …. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on DSS for n34 and n39 | CMCC |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2112192 | Introduction of uplink 7.5KHz frequency shift for n34 and n39 | CMCC | Noted |  |
| R4-2112346 | UL 7.5kHz shift for spectrum sharing in band 34/n34 and band 39/n39 | Apple | Noted |  |
| R4-2112193 | Draft CR to TS 38.101-1 on introduction of UL 7.5KHz frequency shift for n34 and n39 | CMCC | Return to |  |
| R4-2112194 | Draft CR to TS 38.104 on introduction of UL 7.5KHz frequency shift for n34 and n39 | CMCC | Return to |  |
| R4-2112195 | Draft CR to TS 38.104 on introduction of UL 7.5KHz frequency shift for n34 and n39 | CMCC | Return to |  |
| R4-2114376 | UL 7.5kHz shift for spectrum sharing in band 34/n34 and band 39/n39 | Apple | Return to |  |
| R4-2114377 | Introduction of the UL 7.5kHz shift for NR TDD band n34 and n39 | Apple | Return to |  |
| R4-2114378 | Introduction of the UL 7.5kHz shift for NR TDD band n34 and n39 | Apple | Return to |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2114945 | WF on DSS for n34 and n39 | CMCC | Agreeable |  |
| R4-2115097 (From R4-2114376) | UL 7.5kHz shift for spectrum sharing in band 34/n34 and band 39/n39 | Apple | Agreeable |  |
| R4-2112193 | Draft CR to TS 38.101-1 on introduction of UL 7.5KHz frequency shift for n34 and n39 | CMCC | Not Pursued |  |
| R4-2112194 | Draft CR to TS 38.104 on introduction of UL 7.5KHz frequency shift for n34 and n39 | CMCC | Not Pursued |  |
| R4-2112195 | Draft CR to TS 38.104 on introduction of UL 7.5KHz frequency shift for n34 and n39 | CMCC | Not Pursued |  |
| R4-2115089 | Introduction of the UL 7.5kHz shift for NR TDD band n34 and n39 in 38.104 R15 | CMCC | Agreeable |  |
| R4-2115090 | Introduction of the UL 7.5kHz shift for NR TDD band n34 and n39 in 38.104 R16 | CMCC | Agreeable |  |
| R4-2115091 | Introduction of the UL 7.5kHz shift for NR TDD band n34 and n39 in 38.104 R17 | CMCC | Agreeable |  |
| R4-2115087 (From R4-2114377) | Introduction of the UL 7.5kHz shift for NR TDD band n34 and n39（R16 CR） | Apple | Agreeable |  |
| R4-2115088 (From R4-2114378) | Introduction of the UL 7.5kHz shift for NR TDD band n34 and n39 (R17 CR) | Apple | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CMCC | Xiaoran ZHANG | zhangxiaoran@chinamobile.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)