3GPP TSG-RAN WG4 Meeting # 100-e	R4-2115643

Electronic Meeting, 16 – 27 August 2021

Agenda Item:	9.16.5

Source: 	Nokia, Nokia Shanghai Bell

Title: 	WF on BS TX RF requirements

Document for:	Approval

# 1.	Background

Contributions on BS RF TX requirements for 52.6 – 71 GHz in RAN4#100-e have a variety of proposals:

[1]	R4-2111749	Discussion on limitation of the measurement interval for the determination of the averaged EVM for FR2-2 and possible test time improvements	ROHDE & SCHWARZ

[2]	R4-2111972	Discussion on BS TX RF requirements for 52 6-71GHz	CATT

[3]	R4-2112278	Proposals on BS transmitter requirements for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

[4]	R4-2110601	R4-2113316	On BS RF transmitter requirements for the frequency range 52 to 71 GHz	Ericsson

[5]	R4-2113857	Discussion on the OBUE mask boundary for 52.6 - 71 GHz	NEC

[6]	R4-2113922	Discussion on BS Tx requirements for 60GHz	ZTE Corporation

The way forward from the discussion of the contributions are summarized as follows.

# 2.	Way forward

2.1	EIRP and TRP requirements

Re-use both current ([3.4] dB in normal conditions and [4.5] dB in extreme conditions) EIRP and [3] dB TRP accuracy requirements from FR2-1 to FR2-2 and re-use [6%] limit to allow [two] EIRP values to be declared.

2.2	Transient times

The current FR2 BS [3] µs transient period should be applicable for NR operation in 52.6 – 71 GHz range.

2.3	Signal quality – EVM

EVM-% from FR2-1 can be re-used for QPSK, 16QAM and 64 QAM.

[50%] EVM window length is used as starting point.

PT-RS for EVM is discussed after RAN1 decision on PT-RS configurations.

2.4	Signal quality – EVM measurement period

Continue discussion on limiting EVM measurement time in next meeting.

2.5	Signal quality – TAE

Continue discussion on TAE requirements in next meeting.

2.6	Emissions – OBUE and ACLR

For OBUE, further discuss opportunities to re-use or adapt either ETSI BRAN or FR2-1 OBUE requirements to be used for FR2-2 in next meeting.

For ACLR, consider TR 38.803 requirements to be used for FR2-2, unless new simulation results show notable impact on the required ACIR comparing to the current ones in TR 38.803.

2.7	Emissions – Spurious emissions

Continue discussion and down-selection of the following options in next meeting.

Option 1: Use FR2 approach with necessary adaptations on step size for spurious emissions.

Option 2: Use ETSI EN 303 722 and/or 303 753 for spurious emissions.