3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200266

# Electronic Meeting, January 17-25, 2022

Agenda item:	6.6.5.3

Source:	Apple Inc.

Title:	Discussion on SDR requirements with 1KQAM

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#100-e SDR requirements with 1024QAM in NR DL were discussed and way forward [1] was agreed.

- Define SDR requirements for 1024 QAM with rank 1 and rank 2. - Add the MCS indexes for SDR test with 1024QAM based on Table 2. - Interested companies are encouraged to propose MCSpracticalj to decide MCSreqj, where - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj)

In this contribution we present simulation results and propose suitable MCS values for SDR requirements with 1024QAM.

# 2. Discussion

In RAN4#100-e it was agreed to add MCS indices based on the methodology agreed in Rel-15 for SDR requirements.

  - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj)

Where MCSupperboundj was agreed as in table below.

Table 1: MCS indexes for SDR test with 1024QAM

| vLayersj | Qmj | fj | MCSupperboundj |
| --- | --- | --- | --- |
| 1 | 10 | 1 | 25 |
| 1 | 10 | 0.8 | 21 |
| 1 | 10 | 0.75 | 19 |
| 1 | 10 | 0.4 | 9 |
| 2 | 10 | 1 | 25 |
| 2 | 10 | 0.8 | 21 |
| 2 | 10 | 0.75 | 19 |
| 2 | 10 | 0.4 | 9 |

The MCSpracticalj is based on simulation results. Using the simulation parameters from TS38.101-4 Table 5.5A-1, we evaluate the practical MCS with 1024QAM modulation order.

Figure 1: SDR results with 1024QAM with 2RX

The summary of simulation results is provided in the table below.

Table 2: Simulation results for SDR with 1KQAM with 2RX

| Num Layers | MCS | SNR @85% Max TP |
| --- | --- | --- |
| 1 | 23 | 27.1 |
|  | 24 | 27.4 |
|  | 25 | 29.4 |
| 2 | 23 | 30.1 |
|  | 24 | 31.8 |
|  | 25 | 34.0 |

With 1 layer, the operating SNR with MCS 23 and 24 are feasible, but with MCS 25 it’s on the higher side. With 2 layers, the operating SNR with all MCS is > 30-dB and doesn’t seem feasible.

Observation #1: The operating SNR with 2 layers and 1024QAM is high for all MCS. The operating SNR with 1 layer is feasible with MCS 23 and MCS 24.

Proposal #1: For SDR requirements with 1 layer with 1024QAM, use MCSpracticalj as 24.

Based on the simulation results, it doesn’t seem feasible to introduce SDR requirements with 1KQAM with 2 layers as the required SNR is very high with 2RX.

Proposal #2: Do not introduce SDR requirements with 1024QAM with 2 layers with 2RX.

# 3. Conclusion

In this paper, we provide we present simulation results and propose suitable MCS values for SDR requirements with 1024QAM. Our observations and proposals are captured below:

Observation #1: The operating SNR with 2 layers and 1024QAM is high for all MCS. The operating SNR with 1 layer is feasible with MCS 23 and MCS 24.

Proposal #1: For SDR requirements with 1 layer with 1024QAM, use MCSpracticalj as 24.

Proposal #2: Do not introduce SDR requirements with 1024QAM with 2 layers with 2RX.

# Reference

- R4-2120699, “WF on DL1024QAM demodulation and CQI reporting requirements”, Ericsson.